3GPP TSG RAN WG1 Meeting #102-e	R1-2005594

eMeeting, August 17 - 28 , 2020

Agenda Item:	7.2.4.5

Source:	FUTUREWEI

Title:	Remaining details on physical procedures for sidelink

Document for:	Discussion and decision

# Introduction

At RAN1#99, RAN1 completed the standardization work for NR V2X. Some details were left for the CR phase. In this contribution, we discuss two small remaining issues:

- The prioritization between the reception of a PSFCH reception and PSFCH transmission when both PSFCHs have the same priority

- The exact location of the slots that contain the PSFCH

# Discussion

## Priority between PSFCH TX and PSFCH RX

This case occurs when on the same PSFCH symbols the UE has to send a PSFCH TX and is expecting a PSFCH RX. There are different cases to consider:

- When the UE is doing unicast:

  - If a NACK needs to be sent, the UE should prioritize listening to the PSFCH. By not sending the NACK, the UE at the other end will automatically assume a NACK and will retransmit the packet anyway

  - If an ACK needs to be sent, the case can be left up to the UE implementation

- When the UE is doing groupcast option 1: the same behaviour can be used

- When the UE is doing groupcast option 2: the only case to consider is when the UE has to send a NACK. In such a case, our preference is to send the NACK and automatically assume that a NACK was sent on the PSFCH the UE was to receive

Proposal 1: When the UE has to send a PSFCH transmission and is expecting a PSFCH reception on the same symbols:

- For unicast and groupcast option 1

  - If the UE needs to send a NACK, the UE prioritizes the PSFCH reception over PSFCH transmission

  - If the UE needs to send an ACK, this case is left up to the UE implementation

- For groupcast option 2, the UE sends a NACK on the PSFCH and assumes reception of a NACK on the PSFCH

## Time location of the PSFCH

This issue was raised in [1]. Per TS38.213 and TS38.331, a UE is provided with two parameters:

- periodPSFCHresource, which indicates (in slots) the PSFCH periodicity

- MinTimeGapPSFCH, which indicates the minimum of slots a UE must wait before sending the PSFCH

However, where to “anchor” the PSFCH is missing. Note that while at RAN1#102, one bit was added for indicating if the PSFCH was present or not, but this does not remove the ambiguity: for instance, there could be two overlapping pools with two different PSFCH periodicities. Or, for a first transmission, the UE would not know where to find the PSFCH.

The solution proposed in [1] suggests linking the PSFCH location to the resource pool periodicity. While any slot would work, the first slot (slot #0) for the PSFCH anchor location is a simple solution.

Proposal 2: slot #0 of a resource pool contains the PSFCH (if present)

# Conclusions

Physical layer procedures for sidelink were discussed. We propose the following:

Proposal 1: When the UE has to send a PSFCH transmission and is expecting a PSFCH reception on the same symbols:

- For unicast and groupcast option 1

  - If the UE needs to send a NACK, the UE prioritizes the PSFCH reception over PSFCH transmission

  - If the UE needs to send an ACK, this case is left up to the UE implementation

- For groupcast option 2, the UE sends a NACK on the PSFCH and assumes reception of a NACK on the PSFCH

Proposal 2: slot #0 of a resource pool contains the PSFCH (if present)

# References

[1] R1-2004387, “Remaining issues on sidelink physical layer procedure,” NTT DOCOMO INC, RAN1#101-e.