3GPP TSG RAN WG1 #102-e        	                                                                                 R1-2005652

e-Meeting, August 17th – 28th, 2020

Title:	[Draft] Draft reply LS to RAN2 on NR SRS carrier switching

Response to: 	R1-2005209 (R2-2006362)

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	CATT

To:	RAN2

CC:

Contact Person:

Name:

E-mail Address:

1. Overall Description:

RAN1 would like to thank RAN2 for the LS R2-2006362 on NR SRS carrier switching.

In R2-2006362, RAN2 asked RAN1 to provide feedback on whether the following 2 cases on NR SRS carrier switching are allowed:

- Case 1: SRS carrier switching to a SUL without PUCCH/PUSCH while PUCCH/PUSCH is configured in NUL;

- Case 2: SRS carrier switching to both NUL and SUL, neither of which are configured with PUCCH/PUSCH.

RAN1 discussed the two cases in RAN1#102-e, and achieved the consensus that in both cases SRS carrier switching to SUL carrier shall NOT be allowed. In Case 2, SRS carrier switching to the NUL shall be allowed.

2   Actions:

To RAN2

ACTION: RAN1 respectfully asks RAN2 to take the above information into account.

3   Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #103-e                                       26 Oct. – 11 Nov. 2020                       e-Meeting