3GPP TSG RAN WG1 #102-e		R1-2005674

e-Meeting, August 17th – 28th, 2020

Source:	CATT

Title:	Remaining issues on PUSCH enhancements

Agenda Item:	7.2.5.3

Document for:	Discussion and Decision

- Introduction

PUSCH transmission enhancement was introduced in Rel-16 as a part of URLLC enhancements. In this contribution, we analyze some FFS issues for PUSCH enhancement and corresponding text proposals are provided.

- Discussions

  - UCI multiplexing on PUSCH repetition type B

Following agreements on UCI multiplexing on PUSCH were made in RAN1#101-e meeting:

Agreement :

The following TP for TS 38.213 Clause 9 is endorsed (note that the TP is based on the TS 38.213 CR endorsed in RAN1#100bis-e):

### TP for TS 38.213 Clause 9 # 9 UE procedure for reporting control information < Unchanged parts are omitted > If a UE transmits a PUSCH with repetition Type B and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in one or more slots, the UE expects an earliest actual repetition of the PUSCH transmission [6, TS 38.214] that would overlap with the PUCCH transmission to fulfill the conditions in Clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, and the UE multiplexes the HARQ-ACK and/or CSI information in the earliest actual PUSCH repetition with a duration of more than 1 symbol that would overlap with the PUCCH transmission. The UE does not expect that all the actual repetitions that would overlap with the PUCCH transmission have a duration of a single symbol. < Unchanged parts are omitted >

For the last sentence of the above TP, we think further clarification is needed for the case that all the overlapping actual repetitions with a single symbol are segmented from nominal repetitions with L>1.

It was agreed that an actual repetition with a single symbol is omitted except for the case of L =1 in RAN1#100e meeting. Therefore, depending on the assumption when UE omits the actual repetition, there may be two different understandings in case all the overlapping actual repetitions with a single symbol are segmented from nominal repetitions with L>1.

- Understanding 1: It is an error case.

- Understanding 2: All the actual PUSCH repetitions are omitted and PUCCH is transmitted.

It is proposed to clarify the intended UE behaviour.

Proposal 1: If all the actual PUSCH repetitions that would overlap with the PUCCH transmission have a duration of a single symbol and all the actual repetitions are segmented from nominal repetitions with L>1, clarify the intended UE behaviour between the two understandings.

- Understanding 1: It is an error case.

- Understanding 2: All the actual PUSCH repetitions are omitted and PUCCH is transmitted.

  - GP handling after SS/PBCH block or Type-0 CSS symbols

A new RRC parameter numberInvallidSymbolsForDL-UL-Switching was agreed to be introduced for GP handling in RAN1#100bis e-meeting. There was discussion on whether to apply numberInvallidSymbolsForDL-UL-Switching to the symbols after SSB and Type-0 CSS in the last meeting but no conclusion was made.

Some companies consider it not mandatory for UE to receive SSB/type-0 PDCCH when transmitting PUSCH repetition type B. Rx-Tx switching time is not needed in this case hence no need to define gap after SSB and type-0 CSS symbols. However, we think GP symbols are used not only for Rx-Tx switching time, but also for DL-UL interference avoidance. Assuming there is no gap from gNB perspective, considering the propagation delay and timing advance, there would be clear UL-DL interference. Moreover, since SSB and type-0 CSS symbols are treated in the same way as semi-static DL symbols, the motivation for introducing GP symbols after the last semi-static DL symbols can also apply to SSB and type-0 CSS symbol cases. Therefore, we propose to reserve configured number of GP symbols for SSB and type-0 CSS symbols like semi-static DL symbol cases.

Proposal 2: Apply numberInvallidSymbolsForDL-UL-Switching to indicate the number of symbols after the last symbol that is indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH block are invalid symbols for PUSCH repetition Type B transmission.

Proposal 3: Apply numberInvallidSymbolsForDL-UL-Switching to indicate the number of symbols after the last symbol that is indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set are invalid symbols for PUSCH repetition Type B transmission.

The following TP is the proposed corresponding to Proposal 2 and Proposal 3 for TS38.214.

---------------------------------------------- Start of text proposal for 38.214 ----------------------------------

### 6.1.2	Resource allocation

#### 6.1.2.1	Resource allocation in time domain

<unchanged part omitted>

For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows:

-	A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, symbols indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH blocks are considered as invalid symbols for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set are considered as invalid symbol(s) for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, if numberInvalidSymbolsForDL-UL-Switching is configured, numberInvalidSymbolsForDL-UL-Switching symbol(s) after the last symbol that is indicated as downlink in each consecutive set of all symbols that are indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated are considered as invalid symbol(s) for PUSCH repetition Type B transmission. The symbol(s) given by numberInvalidSymbolsForDL-UL-Switching are defined using the reference SCS configuration referenceSubcarrierSpacing provided in tdd-UL-DL-ConfigurationCommon.

-	For operation in unpaired spectrum, if numberInvalidSymbolsForDL-UL-Switching is configured, numberInvalidSymbolsForDL-UL-Switching symbol(s) after the last symbol that is indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH blocks are considered as invalid symbol(s) for PUSCH repetition Type B transmission. The symbol(s) given by numberInvalidSymbolsForDL-UL-Switching are defined using the reference SCS configuration referenceSubcarrierSpacing provided in tdd-UL-DL-ConfigurationCommon.

-	For operation in unpaired spectrum, if numberInvalidSymbolsForDL-UL-Switching is configured, numberInvalidSymbolsForDL-UL-Switching symbol(s) after the last symbol that is indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, are considered as invalid symbol(s) for PUSCH repetition Type B transmission. The symbol(s) given by numberInvalidSymbolsForDL-UL-Switching are defined using the reference SCS configuration referenceSubcarrierSpacing provided in tdd-UL-DL-ConfigurationCommon.

<unchanged part omitted>

---------------------------------------------- End of text proposal for 38.214 ----------------------------------

- Conclusion

This contribution discussed remaining issues on PUSCH enhancements and following proposals are proposed.

Proposal 1: If all the actual PUSCH repetitions that would overlap with the PUCCH transmission have a duration of a single symbol and all the actual repetitions are segmented from nominal repetitions with L>1, clarify the intended UE behaviour between the two understandings.

- Understanding 1: It is an error case.

- Understanding 2: All the actual PUSCH repetitions are omitted and PUCCH is transmitted.

Proposal 2: Apply numberInvallidSymbolsForDL-UL-Switching to indicate the number of symbols after the last symbol that is indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH block are invalid symbols for PUSCH repetition Type B transmission.

Proposal 3: Apply numberInvallidSymbolsForDL-UL-Switching to indicate the number of symbols after the last symbol that is indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set are invalid symbols for PUSCH repetition Type B transmission.

The following TP is the proposed corresponding to Proposal 2 and Proposal 3 for TS38.214.

---------------------------------------------- Start of text proposal for 38.214 ---------------------------------- ### 6.1.2 Resource allocation #### 6.1.2.1 Resource allocation in time domain <unchanged part omitted> For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows: - A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol for PUSCH repetition Type B transmission. - For operation in unpaired spectrum, symbols indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH blocks are considered as invalid symbols for PUSCH repetition Type B transmission. - For operation in unpaired spectrum, symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set are considered as invalid symbol(s) for PUSCH repetition Type B transmission. - For operation in unpaired spectrum, if numberInvalidSymbolsForDL-UL-Switching is configured, numberInvalidSymbolsForDL-UL-Switching symbol(s) after the last symbol that is indicated as downlink in each consecutive set of all symbols that are indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated are considered as invalid symbol(s) for PUSCH repetition Type B transmission. The symbol(s) given by numberInvalidSymbolsForDL-UL-Switching are defined using the reference SCS configuration referenceSubcarrierSpacing provided in tdd-UL-DL-ConfigurationCommon. - For operation in unpaired spectrum, if numberInvalidSymbolsForDL-UL-Switching is configured, numberInvalidSymbolsForDL-UL-Switching symbol(s) after the last symbol that is indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH blocks are considered as invalid symbol(s) for PUSCH repetition Type B transmission. The symbol(s) given by numberInvalidSymbolsForDL-UL-Switching are defined using the reference SCS configuration referenceSubcarrierSpacing provided in tdd-UL-DL-ConfigurationCommon. - For operation in unpaired spectrum, if numberInvalidSymbolsForDL-UL-Switching is configured, numberInvalidSymbolsForDL-UL-Switching symbol(s) after the last symbol that is indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, are considered as invalid symbol(s) for PUSCH repetition Type B transmission. The symbol(s) given by numberInvalidSymbolsForDL-UL-Switching are defined using the reference SCS configuration referenceSubcarrierSpacing provided in tdd-UL-DL-ConfigurationCommon. <unchanged part omitted> ---------------------------------------------- End of text proposal for 38.214 ----------------------------------