3GPP TSG RAN WG1 Meeting #102-e                                                                	   R1- 2005723

e-Meeting, August 17th – 28th, 2020

Source:	CATT

Title:	Baseline coverage performance for FR2

Agenda Item:	8.8.2.2

Document for:	Discussion and Decision

Introduction

At RAN#86 meeting, a study item was achieved as a new Rel-17 SID on NR coverage enhancement [1]. For FR2, the related objectives are as follows

- The target scenarios and services include

  - Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2.

  - eMBB service as first priority and VoIP as second priority for FR2.

  - LPWA services and scenarios are not included.

- Identify baseline coverage performance for both DL and UL for the above scenarios and services based on link-level simulation

  - Both DL and UL channels for FR2.

- Identify the performance target for coverage enhancement, and study the potential solutions for coverage enhancements for the above scenarios and services

  - The target channels include at least PUSCH/PUCCH

  - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions)

  - Study the additional enhanced solutions for FR2 if any

  - Evaluate the performance of the potential solutions based on link level simulation.

In this contribution, we provide preliminary thoughts on the baseline per performance and simulation assumptions for FR2.

Evaluation results

In this section, MPL evaluation results for Urban, Indoor and SubUrban scenarios are provided for FR2. Shadow fading margin and penetration margin are calculated based on channel mode B which is listed in Table 1. The calculation method is the same as the method used for FR1 and IMT-2020 evaluation, which can refer to R1-1808415and R1-1808416.  Target MPL corresponding to target ISD for each scenario is calculated based on channel mode B defined in IMT-2020 evaluation. It is assumed that parameters of Urban scenario are reused for SubUrban scenario.

| Test environment | State | Channel Model | Penetration margin[dB] | Shadow fading margin for 95% ACR [dB] | Shadow fading margin for 90% ACR [dB] |
| --- | --- | --- | --- | --- | --- |
| Urban @28GHz | Indoor- LOS | Model B | 26.02 | 9.04 | 5.6 |
|  | Indoor- NLOS | Model B | 26.02 | 9.04 | 5.6 |
|  | Outdoor (in car)-LOS | Model B | 9.00 | 6.11 | 3.36 |
|  | Outdoor (in car) -NLOS | Model B | 9.00 | 8.03 | 4.79 |
| InH @28GHz | Indoor- LOS | Model B | 0.00 | 2.96 | 1.69 |
|  | Indoor- NLOS | Model B | 0.00 | 8.5 | 5.17 |

- Urban

Target MPL corresponding to target ISD of 400m and 500m are compared. NLOS outdoor-to-outdoor case with 30km/h UE speed and outdoor-to-indoor case with 3km/h UE speed are considered in the evaluation. Required SINR for LLS with both TDL-A and TDL-C channel model are evaluated. Detailed simulation assumptions for Urban scenario are listed in Appendix.

Figure 1: Coverage results for Urban scenario with DDDSU configuration

Figure 2: Coverage results for Urban scenario with DDSU configuration

According to figure 1 and figure 2, coverage gap for cases that can’t satisfy target ISD coverage requirements for Urban scenario are summarized in Table 1.

Table 1: coverage gap for cases that can’t satisfy target ISD coverage requirements for Urban scenario

| Case | channel | Gap between target ISD 500m | Gap between target ISD 200m |
| --- | --- | --- | --- |
| 28GHz DDDSU 3km/h, TDL-A | PUCCH PF3/22bits/rep=1 | -0.81 |  |
|  | eMBB PUSCH | -6.35 | -5.00 |
|  | VoIP PUSCH, w/o rep | -10.05 | -8.70 |
|  | Msg3 PUSCH | -3.85 | -2.50 |
| 28GHz DDDSU 3km/h, TDL-C | PUCCH PF3/22bits/rep=1 | -1.76 | -0.41 |
|  | eMBB PUSCH | -6.95 | -5.60 |
|  | VoIP PUSCH, w/o rep | -10.25 | -8.90 |
|  | Msg3 PUSCH | -4.45 | -3.10 |
| 28GHz DDSU 3km/h, TDL-A | PUCCH PF3/22bits/rep=1 | -0.81 |  |
|  | eMBB PUSCH | -5.45 | -4.10 |
|  | VoIP PUSCH, w/o rep | -10.05 | -8.70 |
|  | Msg3 PUSCH | -3.85 | -2.50 |
| 28GHz DDSU 3km/h, TDL-C | PUCCH PF3/22bits/rep=1 | -1.76 | -0.41 |
|  | eMBB PUSCH | -5.45 | -4.10 |
|  | VoIP PUSCH, w/o rep | -10.25 | -8.90 |
|  | Msg3 PUSCH | -4.45 | -3.10 |

Observation 1: For Urban scenario,

- PDCCH/PDSCH/SSB can meet the coverage requirement.

- For PUCCH format 1, coverage requirement can be fulfilled. For PUCCH format 3, the performance gap can be compensated by enabling repetition.

- PUSCH can’t meet coverage requirement in many cases and PUSCH coverage is the bottleneck for Urban scenario.

- Indoor

Target MPL corresponding to target ISD of 20m is assumed. NLOS indoor case with 3km/h UE speed is considered in the evaluation. Required SINR for LLS with TDL-A channel model is evaluated. Detailed simulation assumptions for Indoor scenario are listed in Appendix.

Figure 3: Coverage results for Indoor scenario

According to figure 3, we can have the following observation:

Observation 2: For Indoor scenario, no bottleneck channel is identified.

- SubUrban

For SubUrban performance evaluation, it is assumed that undefined parameters for SubUrban scenario are the same as Urban Scenario. Target MPL corresponding to target ISD of 400m and 500m are assumed. NLOS outdoor-to-outdoor case with 30km/h UE speed and outdoor-to-indoor case with 3km/h UE speed are considered in the evaluation. Required SINR for LLS with both TDL-A and TDL-C channel model are evaluated. Detailed simulation assumptions for SubUrban scenario are listed in Appendix.

Figure 4: Coverage results for SubUrban scenario with DDDSU configuration

Figure 5: Coverage results for SubUrban scenario with DDDSU configuration

According to figure 4 and figure 5, coverage gap for cases that can’t satisfy target ISD coverage requirements for SubUrban scenario are summarized in Table 2.

Table 2: coverage gap for cases that can’t satisfy target ISD coverage requirements for SubUrban scenario

| Case | channel | Gap between target ISD 500m | Gap between target ISD 200m |
| --- | --- | --- | --- |
| 28GHz DDDSU 3km/h, TDL-A | PUCCH PF3/22bits/rep=1 | -1.02 |  |
|  | VoIP PUSCH, w/o rep | -10.05 | -8.70 |
|  | Msg3 PUSCH | -3.85 | -2.50 |
| 28GHz DDDSU 3km/h, TDL-C | PUCCH PF3/22bits/rep=1 | -1.04 |  |
|  | eMBB PUSCH | -0.65 |  |
|  | VoIP PUSCH, w/o rep | -10.25 | -8.90 |
|  | Msg3 PUSCH | -4.45 | -3.10 |
| 28GHz DDSU 3km/h, TDL-A | PUCCH PF3/22bits/rep=1 | -1.02 | 0.33 |
|  | VoIP PUSCH, w/o rep | -10.05 | -8.70 |
|  | Msg3 PUSCH | -3.85 | -2.50 |
| 28GHz DDSU 3km/h, TDL-C | PUCCH PF3/22bits/rep=1 | -1.04 |  |
|  | eMBB PUSCH | -0.65 |  |
|  | VoIP PUSCH, w/o rep | -10.25 | -8.90 |
|  | Msg3 PUSCH | -4.45 | -3.10 |

Observation 3: For SubUrban scenario,

- PDCCH/PDSCH/SSB can meet the coverage requirement.

- For PUCCH format 1, coverage requirement can be fulfilled. For PUCCH format 3, the performance gap can be compensated by enabling repetition.

- PUSCH can’t meet coverage requirement in many cases and PUSCH coverage is the bottleneck for SubUrban scenario.

With observation 1, 2, 3, we have the following proposal:

Proposal 1: PUSCH coverage enhancement should be studied for FR2.

Conclusion

This contribution discussed the baseline performance and simulation assumptions for FR2, we have following observations:

Observation 1: For Urban scenario,

- PDCCH/PDSCH/SSB can meet the coverage requirement.

- For PUCCH format 1, coverage requirement can be fulfilled. For PUCCH format 3, the performance gap can be compensated by enabling repetition in some cases.

- PUSCH can’t meet coverage requirement in many cases and PUSCH coverage is the bottleneck for Urban scenario.

Observation 2: For Indoor scenario, no bottleneck channel is identified.

Observation 3: For SubUrban scenario,

- PDCCH/PDSCH/SSB can meet the coverage requirement.

- For PUCCH format 1, coverage requirement can be fulfilled. For PUCCH format 3, the performance gap can be compensated by enabling repetition in some cases..

- PUSCH can’t meet coverage requirement in many cases and PUSCH coverage is the bottleneck for SubUrban scenario.

With observation 1, 2, 3, we have the following proposal:

Proposal 1: PUSCH coverage enhancement should be studied for FR2.

References

  - RP-193240, New SID on NR coverage enhancement, China Telecom, RAN Meeting #86.

Appendix: LLS assumptions

Table 3: LLS assumptions for PUSCH

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Indoor | Urban | Suburban |
| Carrier | 28GHz | 28GHz | 28GHz |
| Duplex configuration | TDD config 1: DDDSU (S: 10D:2G:2U) TDD confg 2: DDSU (S: 11D:3G:0U) | TDD config 1: DDDSU (S: 10D:2G:2U) TDD confg 2: DDSU (S: 11D:3G:0U) | TDD config 1: DDDSU (S: 10D:2G:2U) TDD confg 2: DDSU (S: 11D:3G:0U) |
| SCS | 120KHz | 120KHz | 120KHz |
| Number of antenna elements for BS | 128, (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) | 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) | 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) |
| Number of TxRUs for BS | 2 | 2 | 2 |
| Number of antenna elements for UE | 8, one panel:(M, N, P) = (2,2,2) | 8, one panel:(M, N, P) = (2,2,2) | 8, one panel:(M, N, P) = (2,2,2) |
| Number of UE Tx chains | 1 | 1 | 1 |
| Duration of PUSCH | 14 OS | 14 OS | 14 OS |
| Number of DMRS | For eMBB/VoIP: 1 OS For Msg 3: 3 OS | For eMBB/VoIP: 3km/h: 1 OS 30km/h: 2 OS For Msg 3: 3 OS | For eMBB/VoIP: 3km/h: 1 OS 30km/h: 2 OS For Msg 3: 3 OS |
| Occupied channel bandwidth | 30 PRBs for eMBB 4 PRBs for VoIP 2 PRBs for Msg 3 | 30 PRBs for eMBB 4 PRBs for VoIP 2 PRBs for Msg 3 | 4 PRBs for eMBB 4 PRBs for VoIP 2 PRBs for Msg 3 |
| Modulation | QPSK | QPSK | QPSK |
| Coding Rate | TDD config 1: eMBB: CR=379/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; TDD config 2: eMBB: CR=308/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; | TDD config 1: 3km/h: eMBB: CR=379/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 30km/h: eMBB: CR=379/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; TDD config 2: 3km/h: eMBB: CR=308/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 30km/h: eMBB: CR=308/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; | TDD config 1: 3km/h: eMBB: CR=30/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 30km/h: eMBB: CR=30/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; TDD config 2: 3km/h: eMBB: CR=30/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 30km/h: eMBB: CR=30/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; |
| Channel Coding | LDPC | LDPC | LDPC |
| Channel Mode | TDL-A, 100ns | TDL-A, 100ns TDL-C, 100ns | TDL-A, 100ns TDL-C, 100ns |
| UE Mobility: | 3km/h | 3km/h for indoor, 30km/h for outdoor | 3km/h for indoor, 30km/h for outdoor |
| Channel estimation | True | True | True |
| Target BLER | 10% BLER for eMBB 2% BLER for VoIP 10% BLER for Msg3 | 10% BLER for eMBB 2% BLER for VoIP 10% BLER for Msg3 | 10% BLER for eMBB 2% BLER for VoIP 10% BLER for Msg3 |
| PUSCH repetition | w/o repetition | w/o repetition | w/o repetition |
| Waveform | DFT-s-OFDM | DFT-s-OFDM | DFT-s-OFDM |

Table 4: LLS assumptions for PUCCH

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Indoor | Urban | Suburban |
| Carrier | 28GHz | 28GHz | 28GHz |
| Format type | PUCCH format 1 with 2bits PUCCH format 3 with 11/22 bits | PUCCH format 1 with 2bits PUCCH format 3 with 11/22 bits | PUCCH format 1 with 2bits PUCCH format 3 with 11/22 bits |
| SCS | 120KHz | 120KHz | 120KHz |
| Number of antenna elements for BS | 128, (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) | 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) | 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) |
| Number of TxRUs for BS | 2 | 2 | 2 |
| Number of UE Tx chains | 1 | 1 | 1 |
| Duration of PUSCH | 14 OS | 14 OS | 14 OS |
| DMRS configuration for PUCCH | w/o additional DMRS | w/o additional DMRS | w/o additional DMRS |
| Occupied channel bandwidth | 1 PRB | 1 PRB | 1 PRB |
| Channel Mode | TDL-A, 30ns | TDL-A, 100ns TDL-C, 100ns | TDL-A, 30ns TDL-C, 30ns |
| UE Mobility: | 3km/h | 3km/h for indoor, 30km/h for outdoor | 3km/h for indoor, 30km/h for outdoor |
| Channel estimation | True | True | True |
| PUCCH repetition | 1/2 | 1/2 | 1/2 |
| Frequency hopping | On | On | On |

Table 5: LLS assumptions for PDSCH

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Indoor | Urban | Suburban |
| Carrier: | 28GHz | 28GHz | 28GHz |
| BWP | 100MHz | 100MHz | 100MHz |
| SCS | 120KHz | 120KHz | 120KHz |
| Number of antenna elements for BS | 128 (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) | 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) Optional: 512, (M,N,P,Mg,Ng) = (8,8,2,2,2) | 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) Optional: 512, (M,N,P,Mg,Ng) = (8,8,2,2,2) |
| Number of TxRUs for BS | 2. | 2 | 2 |
| UE receive chains | 2T2R | 2T2R | 2T2R |
| length of PDCCH | 12 OS | 12 OS | 12 OS |
| Configuration of DMRS | 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. | For 30km/h (optional: 120km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. | For 30km/h (optional: 120km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. |
| Data rate | 25Mbps for eMBB [320] bits with 20ms data arriving for VOIP | 25Mbps for eMBB [320] bits with 20ms data arriving for VOIP | 1Mbps for eMBB [320] bits with 20ms data arriving for VOIP |
| PRBs/TBS/MCS | full bandwidth for eMBB [320] bits with 20ms data arriving for VOIP | full bandwidth for eMBB [320] bits with 20ms data arriving for VOIP | full bandwidth for eMBB [320] bits with 20ms data arriving for VOIP |
| Channel Coding | LDPC | LDPC | LDPC LDPC |
| Channel Model | TDL-A, | TDL- A, TDL-A | TDL- A, TDL-C |
| DS | 30ns | 100ns | 100ns 100ns |
| UE velocity: | 3km/h | 3km/h for indoor, 30km/h, for outdoor. | 3km/h for indoor, 30km/h, for outdoor. |
| Channel estimation | True | True | True |
| Target BLER | 10% BLER for eMBB 2% BLER for VOIP | 10% BLER for eMBB 2% BLER for VOIP | 10% BLER for eMBB 2% BLER for VOIP |
| UL DL configuration if TDD is assumed: | DDDSU (S: 10D:2G:2U) DDSU (S: 11D:3G:0U) | DDDSU (S: 10D:2G:2U) DDSU (S: 11D:3G:0U) | DDDSU (S: 10D:2G:2U) DDSU (S: 11D:3G:0U) |

Table 6: LLS assumptions for PDCCH

| Parameters | Values |
| --- | --- |
| Carrier | 28GHz |
| SCS | 120kHz |
| UE speed | 3kmph, 30kmph |
| Channel model | TDL-A, TDL-C |
| Aggregation level | 16 |
| Payload | 40 bits + 24CRC bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | 1-port precoder cycling |
| BLER for PDCCH | 1% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | 4 |

Table 7: LLS assumptions for SSB

| Parameters | Values |
| --- | --- |
| Carrier | 28GHz |
| Periodicity | 20ms |
| SCS | 120KHz |
| UE speed | 3kmph, 30kmph |
| Channel model | TDL-A, TDL-C |
| BLER for SSB | 1% BLER |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |