3GPP TSG RAN WG1 #102		R1- 2005726

e-Meeting, August 17th – 28th, 2020

Source:	CATT

Title:	Discussion on coverage enhancement for channels other than PUSCH and PUCCH

Agenda Item:	8.8.2.3

Document for:	Discussion and Decision

Introduction

At RAN1#101 e-meeting, the link level simulation assumptions and the link budget method were discussed for FR1, some agreements were made as below.

Agreements:

- Adopt the following target data rates for eMBB performance evaluation for FR1.

- Urban scenario: DL 10Mbps, UL 1Mbps

- Rural scenario: DL 1Mbps, UL 100kbps

- Rural with long distance scenario: DL 1Mbps, UL 100kbps, [30kbps] (optional)

Agreements:

- For link level simulation, adopt following TBS for Msg3 for FR1

- 56 bits

Agreements:

- For link level simulation, for PDSCH of Msg.4 for FR1.

  - Reuse the following simulation assumption for PDSCH

    - Waveform, [PDSCH duration]

  - FFS: Payload size: [3000bits].

  - Other parameters: Reported by companies.

Agreements:

- For link level simulation, for SSB, PDCCH, PDSCH and PDCCH of Msg.2, PDSCH of Msg.4 and PDSCH for FR1.

  - Reuse following simulation assumptions agreed for PUSCH.

    - Scenario and frequency, frame structure, SCS, pathloss model, channel model, delay spread, UE velocity, number of antenna elements and TxRUs for BS.

  - The number of UE receive chains: is 2.

    - 4 for 4GHz/2.6GHz

    - 2 or 4 for 2GHz

    - 2 for 700MHz

  - For PDSCH, reuse DM-RS configuration, BLER, HARQ, Latency requirements for voice agreed for PUSCH.

    - Reuse DM-RS configuration agreed for PUSCH except that 3 DMRS symbols is used for Msg2.

- For link level simulation, for PRACH and Msg.3 for FR1.

  - Reuse following simulation assumptions agreed for PUSCH

    - Scenario and frequency, frame structure, pathloss model, channel model, delay spread, UE velocity, number of antenna elements and TxRUs for BS and Number of UE transmit chains.

  - For Msg.3, reuse SCS, HARQ configuration, frequency hopping agreed for PUSCH.

In this contribution, we provide simulation results of PDCCH/PDSCH/SSB in different scenarios and to discuss the necessity of coverage enhancement for downlink channel.

Discussion

To compare the performance gap of downlink channels, the target performance is the MPL (Maximum Path Loss) at the cell edge corresponding to the target ISD shown in table 1. The detailed link level simulation assumptions can be found in Appendix.

Table 1: the target data rate and ISD requirements for different scenarios

| Frequency band | Scenarios | Target ISD | Target performance |
| --- | --- | --- | --- |
| FR1 | Urban | ISD 1: 400m | 117.97dB for 4GHz; 114.22dB for 2.6GHz |
|  |  | ISD 2: 500m | 121.75dB for 4GHz; 118.01dB for 2.6GHz |
|  | Rural | ISD 1: 1732m | 131.57 dB for 4GHz; 127.83 dB for 2.6GHz 125.55 dB for 2GHz 116.43 dB for 700MHz |
|  | Rural with long distance | ISD1: 12Km | 164.05dB for 4Ghz 148.91dB for 700MHz |
|  |  | ISD2: 30Km | 179.42 dB for 4Ghz 164.29 dB for 700MHz |
| FR2 | Indoor | ISD: 20m | 94dB |
|  | Urban | ISD1: 200m ISD2: 500m | 123.1dB for ISD 200m 121.75dB for ISD 500m |
|  | Suburban | ISD1: 200m ISD2: 500m | 123.1dB for ISD 200m 121.75dB for ISD 500m |

- FR1

The simulation results for FR1 can be found in Figure 1~3. It can be observed that all downlink channels can meet the coverage requirement in urban and rural scenarios. There is no motivation of enhancing the downlink coverage as the current technique is sufficient already. For rural with long distance scenario, the target ISD is as huge as 12km and 30km.If the current throughput at cell edge is pursued under the umbrella of such large ISD, the largest performance gap need to be compensated is about 30dB at 4GHz carrier.  From this perspective, we don’t believe there is any chance to compensate such a large performance gap with physical techniques. It should be carefully handled by deployment, e.g. relay.

Figure 1: the link budget results of Urban

Figure 2: the link budget results of Rural-TDLC-3kmph

Figure 3: the link budget results of Rural with long distance

- FR2

The simulation results for FR2 are shown in Figure 4~6. It can be observed that all downlink channels can meet the coverage requirement in the given scenarios.

Figure 4: the link budget results of Indoor in FR2

Figure 5: the link budget results of Urban in FR2

Figure 6: the link budget results of Suburban in FR2

Based on the link budget results of downlink channel in FR1 and FR2, we can find the rural with long distance scenario in FR1 is the only one scenario that the downlink channel cannot meet the coverage requirement. The performance gap is large with target 30km at 4GHz and it is hard to handle it with any physical techniques.

Observation: Downlink channels are not the bottleneck except for the rural case with long distance, wherein the performance gap is too large to be compensated by physical techniques.

Proposal: The coverage enhancements for downlink channels are not necessary.

Conclusion

This contribution discussed the necessary of enhancements for PDCCH/SSB/PDSCH in FR1 and FR2, we have the following observations and proposals.

Observation: Downlink channels are not the bottleneck except for the rural case with long distance, wherein the performance gap is too large to be compensated by physical techniques.

Proposal: The coverage enhancements for downlink channels are not necessary.

References

  - RP-193240, New SID on NR coverage enhancement, China Telecom, RAN Meeting #86.

Appendix: the LLS assumptions

Table 2: simulation assumptions of PDSCH

| Parameters | Values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | Urban |  |  |  | Rural |  | Rural with long distance |  |
|  | eMBB |  | VoIP |  | eMBB | VoIP | eMBB | VoIP |
| Carrier: | 4GHz (TDD), 2.6GHz (TDD) |  |  |  | 4GHz (TDD), 2.6GHz (TDD) 2GHz (FDD), 700MHz (FDD) |  | 700MHz (FDD), 4GHz (TDD) |  |
| BWP | 100MHz for 4GHz and 2.6GHz. |  |  |  | 100MHz for 4GHz and 2.6GHz. 20MHz for 2GHz 20MHz for 700MHz. |  | 100MHz for 4GHz. 20MHz for 700MHz. |  |
| SCS | 30kHz |  |  |  | 30kHz for TDD, 15kHz for FDD. |  | 30kHz for TDD, 15kHz for FDD. |  |
| Number of antenna elements for BS | 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) |  |  |  | 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |  | 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |  |
| Number of TxRUs for BS | 64TxRUs for 2.6 and 4 GHz. |  |  |  | 2 or 4 TXRUs for 2GHz, 700 MHz 64TxRUs for 2.6 and 4 GHz. Optional: 32 TXRUs at 2 GHz |  | 2 or 4 TXRUs for 700 MHz 64TxRUs for 4 GHz. |  |
| UE receive chains | 4 for 4GHz/2.6GHz |  |  |  | 4 for 4GHz/2.6GHz 4 for 2GHz 2 for 700MHz |  | 4 for 4GHz; 2 for 700MHz |  |
| length of PDSCH | 12 OS |  |  |  | 12 OS |  | 12 OS |  |
| Configuration of DMRS | 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. |  |  |  | 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. |  | For 120km/h, (Optional: 30km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. |  |
| Data rate | 10Mbps | [320] bits with 20ms data arriving |  |  | 1Mbps | [320] bits with 20ms data arriving | 1Mbps | [320] bits with 20ms data arriving |
| PRBs/TBS/MCS [TBD] | full bandwidth | [320] bits with 20ms data arriving |  |  | full bandwidth | [320] bits with 20ms data arriving | full bandwidth | [320] bits with 20ms data arriving |
| Channel Coding | LDPC |  |  |  | LDPC |  | LDPC |  |
| Channel Model | TDL-C for NLOS, |  |  |  | TDL-C for NLOS, TDL-D for LOS. |  | TDL-C for NLOS, TDL-D for LOS. |  |
| DS | 300ns |  |  |  | 300ns |  | 30ns |  |
| UE velocity: | 3km/h |  |  |  | 3km/h |  | 120km/h |  |
| Channel estimation | True |  |  |  | True |  | True |  |
| Target BLER | w/o HARQ, 10% iBLER. |  |  | 2% rBLER. | w/o HARQ, 10% iBLER. | 2% rBLER. | w/o HARQ, 10% iBLER. | 2% rBLER. |
| UL DL configuration if TDD is assumed: | DDDSU (S: 10D:2G:2U) only for 4GHz DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz DDDDDDDSUU (S: 6D:4G:4U) only for 2.6GHz |  |  |  | DDDSU (S: 10D:2G:2U) only for 4GHz DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz DDDDDDDSUU (S: 6D:4G:4U) only for 2.6GHz |  | DDDSU (S: 10D:2G:2U) only for 4GHz DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz DDDDDDDSUU (S: 6D:4G:4U) only for 2.6GHz |  |

Table 3: simulation assumptions of PDCCH

| Parameters | Values |
| --- | --- |
| Carrier | 2GHz/2.6GHz/4GHz/700MHz |
| SCS | 30kHz, 15kHz |
| Aggregation level | 16 |
| Payload | 40 bits + 24CRC bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | 1-port precoder cycling |
| BLER for PDCCH | 1% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | 4 |

Table 4: simulation assumptions of SSB

| Parameters | Values |
| --- | --- |
| Carrier | 2GHz/2.6GHz/4GHz/700MHz |
| Periodicity | 20ms |
| SCS | 15KHz, 30KHz |
| BLER for SSB | 1% BLER |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |