3GPP TSG RAN WG1 #102-e		R1-2005762

e-Meeting, August 17th – 28th, 2020

Agenda Item:	8.11.2.1

Source: 	NEC

Title:	Views on resource allocation for power saving

Document for:	Discussion/Decision

- Introduction

The WI 'NR Sidelink enhancement' regarding Rel.17 sidelink was approved and updated in RP-201385. [1]One of the objectives is to specify radio solutions to reduce power consumption of UEs in terms of resource allocation:

2. Resource allocation enhancement: - Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

In this contribution, we showed our views on the potential issues in introducing Rel.14 random resource selection and partial sensing to Rel.16 NR sidelink resource allocation mode 2.

- Discussion

As per WI, Rel.14 LTE sidelink random resource selection and partial sensing will be used as baseline. Random resource selection and partial sensing were deemed efficient to reduce power for mode 4 UEs in LTE. Reuse them for NR sidelink power saving in Rel.17 is a straightforward way.

Proposal 1: Introduce random resource selection and partial sensing for NR sidelink, reuse the Rel.14 sidelink design as baseline.

  - Random resource selection

In Rel.14 sidelink, as marked with blue color in the illustration below, a UE can choose to perform random resource selection when a selected resource pool is configured with resourceSlectionConfigP2X which includes {partial sensing, random selection} or {random selection} only.

Figure 1 Random resource selection in Rel.14 sidelink

In a resource pool allowing both partial sensing and random selection. LTE random selection UE shall reserve resources for multiple periodic TBs by reservation signaling, which can mitigate interference to partial sensing UEs. While NR V2X support aperiodic traffic, random selection for aperiodic traffic without periodic reservation signaling will lead more resource collisions and interferences to other partial sensing or sensing UEs. How to mitigate interference to sensing/ partial sensing UEs in the same resource pool needs further discussion.

There are some other issues which should also be taken into account to design Rel.17 random resource selection. For instance: How to mitigate resource selection collision between UEs? When to perform random resource selection for multiple TBs? Is any enhancement needed for traffics with stricter requirement?

Proposal 2: Interference and collision issues should be studied firstly for random resource selection.

  - Partial sensing

There are mainly three steps in LTE partial sensing:

- Step 1: determining Y (>= minNumCandidateSF) candidate single-subframe resources within the resource selection window;

- Step 2: monitoring any subframe ty-k×PSL within the sensing window, where tySL is a subframe included in the set Y and k×P is indicated by high layer parameter gapCandidateSensing;

- Step 3: excluding associated resources within the resource selection window by SCI decoding and RSRP measurement.

From our respective, following issues shall be discussed regarding minimum candidate resources, monitoring occasions and excluding resources.

    - Minimum candidate resources

In the discussion of partial sensing for P-UE in LTE sidelink. The following agreements were made:

Agreements of RAN1#87 - When P-UE makes resource selection/reselection decision at TTI m, the possible candidates resources, i.e., Y subframes, are selected in [m+T1, m+T2] - The minimum allowed value of Y is (pre)configured. Selection of Y subframes is up to P-UE implementation.

The minimum allowed value of Y is configured by high layer parameter minNumCandidateSF which takes an integer value from 1 to 13. The value range [1, 13] was determined in the email discussion '[87-15] LS of V2X (RRC parameter list)'. In that stage of Rel.14 sidelink, the minimum selection window is [n+4, n+20] which includes 17 subframes, considering some subframes which the UE cannot monitor, it's feasible and reasonable to set minNumCandidateSF as [1, 13] because even the largest candidate value 13 is less than the minimum resource selection window 17.

In NR sidelink, the resource selection window size depends on subcarrier spacing configuration μ, RRC parameter T2_min, remaining PDB and UE implementation. The minimum window size it’s no longer a fixed value as LTE sidelink does. For example, the minimum resource section window may be 1 slots, 2 slots, 3 slots, 8 slots in some cases and any other possible values that determined by subcarrier spacing configuration μ, RRC parameter T2_min, remaining PDB and UE implementation. One set of minimum allowed value of Y as LTE cannot work properly for NR sidelink.

Proposal 3: Study potential issues regarding minimum candidate resource configuration for partial sensing.

    - Monitoring occasions

Figure 2 monitoring occasions for partial sensing

In LTE sidelink, the reservation information of a candidate resource in subframe Y (green part in Figure 1) may be transmitted in subframes Y - M (cyan parts in Figure 1), M could be [100, 200 … 1000] and UE need to monitor the subframe Y - M by high layer configuration. NR sidelink supports aperiodic traffics (or mixed periodic and aperiodic traffics) and the allowed reservation periods are configured among 0, [1-99], 100, 200 … 1000. Legacy LTE partial sensing design is target on periodic traffics and reservation periods larger than 100ms. We need to consider how to identify occupied resources of aperiodic traffics and reservation periods less than 100ms in NR partial sensing.

The second aspect we noticed is the sensing window size in NR sidelink. LTE only support 1000ms sensing window size while there are two sensing window sizes configured for NR sidelink: 100ms and (1000+100)ms. How to handle the '100ms' in both configurations is an issue we need to consider for NR partial sensing.

Proposal 4: Study potential issues regarding monitoring occasions for partial sensing, at least the following aspects should be considered:

- aperiodic traffics

- reservation periods < 100ms

- sensing windows size

    - Excluding resources

In LTE or NR sensing procedure, the UE shall monitor all the subframes/slots within the sensing window except for those in which its transmissions occurred. In the illustration below, assuming UE had transmissions in the green slot, then the UE cannot detect SCIs from other UEs, the associated resources within the resource selection window becomes 'unknown' slots from the UE's perspective. The specified way is to exclude all the possible resources reserved by all the allowed reservation periods of the resource pool.

Figure 3 excluding 'unknown' slots

In Rel.14 sensing procedure, step 5 was captured to excluding the 'unknown' slots while no solution is specified to handle the 'unknown' slots for partial sensing. In our understating, the 'unknown' slots were indeed taken into account in the email discussion '[87-15] LS of V2X (RRC parameter list)' for minNumCandidateSF value determination. However, the selection of candidate resources within the selection window is up to UE implementation in current specification. As a consequence, UE still has possibility to select 'unknown' slots as the candidate resources and cannot exclude these slots in the subsequent Rel.14 partial sensing procedures.

Proposal 5: For Rel.17 sidelink, introduce solutions regarding the 'unknown' slots which are not handled in Rel.14 partial sensing.

- Conclusion

In this contribution, we discussed some potential issues that exist in reusing Rel.14 sidelink random resource selection and partial sensing, and proposed that:

Proposal 1: Introduce random resource selection and partial sensing for NR sidelink, reuse the Rel.14 sidelink design as baseline.

Proposal 2: Interference and collision issues should be studied firstly for random resource selection.

Proposal 3: Study potential issues regarding minimum candidate resource configuration for partial sensing.

Proposal 4: Study potential issues regarding monitoring occasions for partial sensing, at least the following aspects should be considered:

- aperiodic traffics

- reservation periods < 100ms

- sensing windows size

Proposal 5: For Rel.17 sidelink, introduce solutions regarding the 'unknown' slots which are not handled in Rel.14 partial sensing.

- Reference

[1] RP-201385, WID revision: NR sidelink enhancement, 3GPP TSG RAN Meeting #88e, June 29 - July 3, 2020.