3GPP TSG RAN WG1 Meeting #102e                                                                  	     R1-2005837

e-Meeting, August 17th – August 28th, 2020

Agenda Item:	8.9.1

Source:	Lenovo, Motorola Mobility

Title:	Support 16QAM for NBIoT

Document for:	Discussion and decision

Introduction

In the RAN#88 plenary meeting, an updated work item for Rel.17 eMTC/NBIoT was approved. The objective is to specify the following enhancements to NB-IoT.

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4]

  - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4]

In this contribution, detailed considerations of 16QAM support for NPDSCH and NPUSCH, especially new TBS table and related DL power allocation design are presented.

Discussion

16QAM for NPDSCH

In Rel.13 NBIoT, up to QPSK is supported for DL. In order to increase the data rate, higher modulation of 16QAM is proposed in Rel.17 NBIoT scope. The requirement of 16QAM support for unicast NPDSCH should be optional. UE indicates its 16QAM capability to the network so that the network could decide whether to configure 16QAM to the UE accordingly.

Proposal 1: Introduce UE capability signaling for the support of 16QAM for unicast NPDSCH.

If 16QAM for NPDSCH is configured, the maximum TBS is increased about twice that of the Rel-16 noted in WID, so the TBS table is extended to ITBS= [21] as shown in Table 1. Note: some of the TBS entry of ITBS=21 is larger than the code rate of 0.93, especially for inband case.

Table 1: Transport block size (TBS) table for NPDSCH.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1384 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 1000 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | 2472 | 2984 | 4008 | 4968 |

The modulation order is indicated dynamically by DCI to fast adapt channel condition change. In Rel.13 NBIoT, only QPSK is supported Qm= 2, there is no mapping table for TBS and MCS, and ITBS=IMCS. To support 16QAM, the MCS table should be design as LTE. There are two ways to generate the modulation, TBS and MCS table based on the MCS field indication in DCI format N1.

- Option 1: The easiest way is to introduce an additional bit in DCI (e.g., 5 bit MCS field) to support all the MCS/TBS, which may result in a large number of TBS entries being unusable for NBIoT due to TBS limitation (the maximal TBS entry is [22] if reusing legacy TBS table extension)

- Option 2: Reuse the methodology (e.g., totally 16 MCS entries with 4 bits in DCI) and do a remapping of TBS and MCS in TBS table such that some rows of TBS are omitted, as shown in Table 2.

Table 2 Modulation and TBS table

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 1 |
| 2 | 2 | 3 |
| 3 | 2 | 4 |
| 4 | 2 | 5 |
| 5 | 2 | 7 |
| 6 | 2 | 8 |
| 7 | 2 | 9 |
| 8 | 4 | 11 |
| 9 | 4 | 12 |
| 10 | 4 | 13 |
| 11 | 4 | 14 |
| 12 | 4 | 16 |
| 13 | 4 | 17 |
| 14 | 4 | 18 |
| 15 | 4 | 20 |

Proposal 2: To support 16QAM of NPDSCH, the MCS field in DCI format N1 is enlarged or reinterpreted, which needs further discussion.

For UEs without capability of supporting 16QAM for NPDSCH, a 4-bits MCS field is used. For UEs with the capability of supporting 16QAM, UE should also detect DCI with 4 bits MCS field before the capability indication acquired by the eNB. To save the DCI bits (e.g., Option 1) or enhance the TBS indication granularity (e.g., Option 2), eNB can disable the 16QAM configuration by RRC signaling semi-statically for 16QAM capable UEs when it is found the channel condition or the packet size is static and 16QAM is not suitable.

Proposal 3: The configuration of 16QAM for NPDSCH can be enabled/disabled by eNB through RRC signaling.

In NBIoT, two kinds of UE category are defined in TS36.306 in Rel.13~16, at least one new UE category is needed, the maximum TBS is [4584] and soft buffer size is increased from 6400 accordingly.

# Table 3: Downlink physical layer parameter values set by the field ue-Category-NB

| UE Category | Maximum number of DL-SCH transport block bits received within a TTI | Maximum number of bits of a DL-SCH transport block received within a TTI | Total number of soft channel bits |
| --- | --- | --- | --- |
| Category NB1 | 680 | 680 | 2112 |
| Category NB2 | 2536 | 2536 | 6400 |

DL power allocation

In Rel.13 NBIoT, the UE may assume NRS EPRE is constant across the downlink NB-IoT system bandwidth and constant across all subframes that contain NRS. A UE may assume the ratio of NPDSCH EPRE to NRS EPRE is 0 dB with one NRS antenna port and -3 dB with two NRS antenna ports. In order to support 16QAM for NPDSCH, the ratio of NPDSCH EPRE to NRS EPRE should be configured due to need of 16QAM demodulation by receiver side. LTE DL power allocation solution is the baseline for NBIoT design.

Beside the two types of OFDM symbols in LTE downlink power allocation, there are three types of OFDM symbol for NBIoT (inband case) as shown in Figure 1.

- Type A OFDM: without NRS or CRS, symbol (1),2,4

- Type B OFDM: with NRS, symbol 5,6

- Type C OFDM: with CRS, symbol 0,(1),3

The ratio of PDSCH EPRE (E_A, E_B, E_C) to NRS EPRE (E_NRS) among PDSCH REs for each OFDM symbol is denoted by ρA [dBm],ρB[dBm] and ρC[dBm]separately, e.g., ρA =E_A/E_NRS, ρB=E_B/E_NRS, ρC =E_C/E_NRS. Network should semi-statically configure three types of NPDSCH EPRE separately.

Figure 1 NBIoT downlink power allocation illustration (inband case)

Proposal 4: Network should semi-statically configure three types of NPDSCH EPRE separately.

16QAM support for NPUSCH

Similar to 16QAM support for NPDSCH, UE reports its NPUSCH 16QAM capability to the network so that the network could decide whether to configure 16QAM to the UE accordingly.

Proposal 5: Introduce UE capability signaling for the support of 16QAM for unicast NPUSCH.

Proposal 6: The configuration of 16QAM for NPUSCH can be enabled/disabled by eNB through RRC signaling.

As noted in WID, UE TBS is not intended to be increased with introduction of NPUSCH 16QAM, and the TBS should not be larger than the largest Rel-16 UL TBS.

For the first solution as shown in section 2.1, TBS table is increased to ITBS= 20 [21] as Table 4, but maximal TBS is still smaller than 2536, and then generate modulation, TBS and MCS table similar to NPDSCH with additional 1 bit MCS field in DCI or remapping of TBS and MCS in TBS table.

Note: the state '1110' of “modulation and coding scheme” field is used for PUR ACK/fallback indication for uplink transmission on PUR. To be backward compatible for support PUR and 16QAM NPUSCH simultaneously, the IMCS entry number should be considered.

Another solution is to keep the current Rel.16 TBS table for NPUSCH, and number of resource unit is reinterpreted for modulation order of 16QAM.

Table 4 Transport block size (TBS) table for NPUSCH.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1384 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 1000 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 |  |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 |  |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2472 |  |  |  |

Proposal 7: Support 16QAM for NPUSCH needs further study:

- Option1: Extend TBS table and generate modulation, TBS and MCS table.

- Option2: Reinterpret the number of resource unit for modulation order of 16QAM.

Conclusions

In this contribution, considerations of 16QAM support for NPDSCH and NPUSCH are provided. The following proposals are given.

Proposal 1: Introduce UE capability signaling for the support of 16QAM for unicast NPDSCH.

Proposal 2: To support 16QAM of NPDSCH, the MCS field in DCI format N1 is enlarged or reinterpreted, which needs further discussion.

Proposal 3: The configuration of 16QAM for NPDSCH can be enabled/disabled by eNB through RRC signaling.

Proposal 4: Network should semi-statically configure three types of NPDSCH EPRE separately.

Proposal 5: Introduce UE capability signaling for the support of 16QAM for unicast NPUSCH.

Proposal 6: The configuration of 16QAM for NPUSCH can be enabled/disabled by eNB through RRC signaling.

Proposal 7: Support 16QAM for NPUSCH needs further study:

- Option1: Extend TBS table and generate modulation, TBS and MCS table.

- Option2: Reinterpret the number of resource unit for modulation order of 16QAM.

- References

RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon, RAN#88e