3GPP TSG RAN WG1 #102-e		             R1-2005840

E-meeting, August 17th – August 28th, 2020

Agenda Item:	8.11.2.2

Source:	Lenovo, Motorola Mobility

Title:	Sidelink resource allocation for Reliability enhancement

Document for:	Discussion

Introduction

In RAN#86, a new WI has been approved for supporting NR Sidelink general enhancement in NR Rel. 17 with following objectives on reliability enhancement and latency reduction as captured in RP-201385 which is the revision approved in RAN#88e:

Resource allocation enhancement:

- Study the feasibility and benefit of the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution if deemed feasible and beneficial [RAN1, RAN2]

  - Inter-UE coordination with the following until RAN#90.

    - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission.

  - Note: The study scope after RAN#88 is to be decided in RAN#90.

  - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios.

  - Note: RAN2 work will start after RAN#89.

In this contribution, we provide our views on the sidelink resource allocation procedure for reliability enhancement and latency improvement for V2X.

_____________________________________________________________________________

Sidelink operation for Reliability improvement

In NR Rel-16 Sidelink operation for mode 2 autonomous resource selection operation consists of following procedure sensing, candidate resource selection, candidate resource exclusion, resource re-evaluation and pre-emption. NR Sidelink Rel-16 supports different cast type (Broadcast, Groupcast, Unicast) and various reliability improvements techniques such as resource re-evaluation and pre-emption are introduced. Mode 2b i.e., Inter-UE coordination was briefly discussed during the Rel-16 study item phase for improving the packet reception rate with receiver assistance and Rel-17 resource allocation mechanism should further study on the benefit of supporting it.

Benefit in supporting Inter-UE Coordination

Sidelink reliability enhancement for mode 2 resource allocation should start with the investigation of various reasons associated for the consecutive packet loss. Below are few reasons for consecutive packet loss to be considered:

- Could be due to Half duplex where UE-A and UE-B are transmitting at the same time slot and cannot hear each other transmissions

- Consecutive NACK(s)/DTX reception from RX UE(s) due to bad channel conditions

- Congestion in the resource pool

- Interference at the receiver side due to hidden node

Proposal 1: RAN1 study on the list of reasons associated for the consecutive packet loss like Half duplex, Consecutive NACKs/DTX due to bad radio channel, Congestion in the resource pool and Hidden node problem

- Mitigation of half duplex impact

UE-A can determine the set of resources based on its transmission/reception status; two alternatives may be considered:

Alt.1: The set of resources contains a set of slots at which UE-A will perform transmission. UE-A may have data to be transmitted to other UEs, and it will determine a set of resources to perform PSCCH/PSSCH transmission, in these slot UE-A cannot receive the transmission from UE-B. If UE-A informs these slots to UE-B, the half-duplex issue can be avoided to improve the reliability of the transmissions from UE-B. After receiving the set of slots from UE-A, UE-B should exclude the resources in these slots from its candidate resources.

Alt.2: The set of resources contains a set of slots at which UE-A will perform reception. UE-A may receive PSCCH/PSSCH transmission from other UEs, the other UEs may reserve resources for subsequent transmission. In the reserved slots UE-A will perform reception. If UE-A informs these slots to UE-B, it also helps to improve the reliability of the UE-B’s transmission. After receiving the set of slots from UE-A, UE-B should take these slots into its resource selection procedure, e.g., higher priority of the candidate resources in these slots.

Proposal 2: UE-A can transmit a set of slots to inform UE-B its transmission/reception status to mitigate the half duplex impact.

- Mitigation of hidden node impact

To mitigate the hidden node impact the set of resources can be determined based on the sensing of UE-A. UE-A performs sensing and determines a set of candidate resources as defined in Rel-16 V2X, then UE-A transmits the candidate resources  to UE-B. UE-B should also perform sensing and determine a set of candidate resources. In the resource selection procedure of UE-B it should consider two resource sets, e.g., random selection from the resources which are the intersection resources in two resource sets.

In Rel-16 V2X the resource exclusion is based on the SCI decoding and L1 SL-RSRP measurement, and the L1 SL-RSRP threshold is (pre-)configured for each combination of pi and pj where pi is the priority indication associated with the resource indicated in SCI and pj is the priority of the transmission in the UE selection resources. UE-A should also determine the set of resources based on its SCI decoding and L1 SL-RSRP measurement. To guarantee the comparison of two resource sets at UE-B side, it is better for UE-B to have the knowledge on the L1 SL-RSRP threshold during the candidate resources determination procedure at UE-A side.

Proposal 3: UE-A can transmit a set of resources based on its sensing to UE-B to mitigate the hidden node impact.

- Inter-UE coordination for Groupcast

The reliability issue due to a potential half duplex/hidden node in a Groupcast/Unicast transmission for NR mode-2 can be successfully mitigated with the help of RX-UE(s)/RSU periodically or a-periodically providing feedback arising due to any potential half duplex problem or hidden node problem.

RX-UE(s)/RSU could further check the destination group identifier(s) and the corresponding Sidelink resource selected or reserved for SL data transmission by group member UE(s) from their SCI and if their destination group identifier(s) are identical and the corresponding resource selected by the group member TX UE(s) occupies the same time slot, then there is a potential half duplex problem among them and a potential feedback to the corresponding TX UE(s) could be helpful to avoid problems arising due to half duplex.

Feedback from Rx UE(s) on the scheduled transmission slots considering all destination ids could help Tx UE to avoid scheduling transmission in those slots.  Feedback on the interference condition on subchannels in a resource pool from the receiver side is helpful in selecting and reserving suitable resource in a resource pool at the transmitter side that increases the packet reception ratio. Feedback from receiver can include information about the interference free resource or interfered resource and there is an impact on the resource allocation procedure like candidate resource exclusion and re-evaluation mechanism.

As an enhancement, even if the PSSCH from TX UE(s) are successfully decoded by the group member UE(s), RX UE(s) could report NACK(s) on the feedback resource to implicitly indicate potential half duplex problem to respective TX UE(s) when the half duplex detection conditions are met and then the TX UE(s) could (re)transmit the corresponding transport block. Otherwise, RX UE can also (re)transmit the data to those TX UE(s) and the selection of RX UE in a group to (re)transmit the data can be based on SUE, relay UE etc..

Proposal 4: Half duplex for groupcast transmission can be detected if more than one TX UE(s) transmit SCI with the same destination group id and in the same time slot

- For groupcast option-1, RX UE(s) feedback NACK

- Otherwise, RX UE (re)transmit the data to TX UE(s)

- Signaling of the set of resources

According to the approved WID the set of resource should be transmitted to UE-B, higher layer signaling (e.g., PC5-RRC) and physical layer signaling can be considered.

Proposal 5: Higher layer signaling and physical layer signaling can be considered to transmit the set of resources to UE-B.

_____________________________________________________________________________

Conclusion

In this contribution, we focused on Sidelink reliability enhancement aspects on the resource allocation mechanism for V2X and present our views on the following topics:

Following proposals are made as a result:

Proposal 1: RAN1 study on the list of reasons associated for the consecutive packet loss like Half duplex, Consecutive NACKs/DTX due to bad radio channel, Congestion in the resource pool and Hidden node problem.

Proposal 2: UE-A can transmit a set of slots to inform UE-B its transmission/reception status to mitigate the half duplex impact.

Proposal 3: UE-A can transmit a set of resources based on its sensing to UE-B to mitigate the hidden node impact.

Proposal 4: Half duplex for groupcast transmission can be detected if more than one TX UE(s) transmit SCI with the same destination group id and in the same time slot

- For groupcast option-1, RX UE(s) feedback NACK

- Otherwise, RX UE (re)transmit the data to TX UE(s)

Proposal 5: Higher layer signaling and physical layer signaling can be considered to transmit the set of resources to UE-B.

References

- RP-201385, WID revision: NR sidelink enhancement