3GPP TSG RAN WG1 Meeting #102_e                                                                  	        R1-2005845

e-Meeting, August 17th – 28th, 2020

Agenda Item:		7.2.2.2.3

Source: 	Intel Corporation

Title:	Enhancements to HARQ for NR-unlicensed

Document for: 	Discussion/Decision

# Introduction

In this contribution, we provide our views on the remaining issues on HARQ-ACK transmission for NR-U.

# Handling PDSCH with insufficient process timing in one-shot feedback

In RAN1 #100_e meeting, it was discussed on how to handle a HARQ process in one-shot feedback if a DCI is detected however the scheduled PDSCH by the DCI cannot be decoded with sufficient processing time for HARQ-ACK transmission in the one-shot feedback. Specifically, the scheduled PDSCH may be before, at or after the PUCCH carrying one-shot feedback. This issue is still listed as FFS. Therefore, a related agreement/conclusion is needed for the completeness of specification.

A simplest way to handle this issue is to consider it as an error case. However, it results in loss of peak data rate, as shown in Figure 1. That is, for a detected PDCCH, the corresponding PDSCH cannot be allocated in a window around the PUCCH, which causes low resource efficiency.

Figure 1: No PDSCH scheduling within a window of PDSCH processing time

Therefore, it is preferable to allow such PDSCH allocations and UE report a HARQ-ACK for the HARQ process indicated by a latest detected DCI by which the scheduled PDSCH is not decoded yet. There are two cases depending on the NDI toggling or not in the latest detected DCI.

- Case 1: if the NDI in the latest detected DCI is NOT toggled

In this case, gNB schedules retransmission for a TB of a HARQ process, UE should just report the HARQ-ACK for the same TB of the HARQ process. Specifically, if the UE ever reports an ACK for the same TB in earlier HARQ-ACK transmission, but gNB fails to decode the HARQ-ACK information, gNB may schedule the retransmission of the same TB again. Therefore, UE should retransmit ACK for the HARQ process in one-shot feedback.

- Case 2: if the NDI in the latest detected DCI is toggled

In this case, UE could know that gNB already correctly receive HARQ-ACK transmission for the previous TB of a HARQ process. Therefore, UE should report a NACK. Otherwise, it may result in NACK-to-ACK error if UE report an ACK for the previous TB again. As shown in Figure 2, the reception of a latest DCI with toggled NDI without sufficient time for PDSCH decoding may be due to the missing of an actual initial DCI which schedules the new TB. If UE report ACK for the previous TB, gNB will misunderstand it as ACK for the initial scheduling of the new TB. Therefore, the only correct behaviour in Case 2 is reporting NACK.

Figure 2: No PDSCH scheduling within a window of PDSCH processing time

Proposal 1: For the case that DCI is detected but PDSCH is not decoded with sufficient processing time for one-shot feedback,

- Case 1: if the NDI in the latest detected DCI is NOT toggled, UE report actual HARQ-ACK for the HARQ process;

- Case 2: if the NDI in the latest detected DCI is toggled, UE reports NACK for the HARQ process.

Text proposal for section 9.1.4 in 38.213-g10.

…

if UE has reported HARQ-ACK information for TB t for HARQ process number h on serving cell c, and has not subsequently detected a DCI format scheduling a PDSCH reception with non-toggled NDI, or has not received a SPS PDSCH, with TB t for HARQ process number h on serving cell c

while g<NHARQ-ACK,cCBG/TB,max

j=j+1

g=g+1

end while

else thc

while g<NHARQ-ACK,cCBG/TB,max

= HARQ-ACK information bit for CBG g of TB t for HARQ process number h of serving cell c

j=j+1

g=g+1

end while

end if

…

if UE has reported HARQ-ACK information for TB t for HARQ process number h on serving cell c and has not subsequently detected a DCI format scheduling a PDSCH reception with non-toggled NDI, or has not received a SPS PDSCH, with TB t for HARQ process number h on serving cell c

= NACK

j=j+1

t=t+1

else thc

= HARQ-ACK information bit for TB t for HARQ process h of serving cell c

j=j+1

t=t+1

end if

# Out-of-order issue for NNK1

In [2], an OOO problem due to the adaptation of NNK1 was described. According to current specification, after UE detect a PDSCH scheduled with NNK1, UE tries to find a next DCI which schedules a dynamic PDSCH or directly triggers one-shot feedback. However, HARQ-ACK transmission for SPS PDSCH is not considered in NNK1 operation. As a result, UE will transmit HARQ-ACK for the SPS PDSCH in PUCCH1 and transmit HARQ-ACK for PDSCH1 on PUCCH2 together with the HARQ-ACK for PDSCH3, which is the OOO problem.

Figure 3: Out-of-order issue for NNK1 (from [2])

In the above problem, UE knows that there is a PUCCH1 for the SPS PDSCH. Since a basic principle for NR operation is to avoid OOO problem, UE could know that HARQ-ACK transmission for PDSCH1 must be scheduled no later than PUCCH1. Due to the missing a next PDCCH to PDSCH1 (i.e. PDCCH scheduling PDSCH2 is missed), UE cannot know exact PUCCH timing for PDSCH1. We can consider a virtual UE internal procedure, i.e. UE prepare a codebook for HARQ-ACK transmission on the same timing as PUCCH1 (there actually doesn’t happen since it is useless). Consequently, there is no OOO issue for HARQ-ACK transmission on PUCCH1 for the SPS PDSCH. When HARQ-ACK for PDSCH1 is transmitted on PUCCH2, there is still no OOO issue. The reason is that HARQ-ACK transmission on PUCCH 2 for PDSCH1 is a retransmission.

The HARQ-ACK for PDSCH1 may or may not be in the same codebook as the SPS PDSCH. Several options can be considered,

Option 1: UE skip any HARQ-ACK transmission on PUCCH1. Then, UE will retransmit HARQ-ACK for PDSCH1 on PUCCH 2 when HARQ-ACK for PDSCH3 is reported.

Option 2: UE only transmits HARQ-ACK for the SPS PDSCH on PUCCH1 as a fallback operation. There is no OOO issue since UE knows HARQ-ACK for PDSCH1 is no later than PUCCH1, i.e. from UE point of view, HARQ-ACK for a PDSCH is still known to be no later than a later PDSCH. UE will retransmit HARQ-ACK for PDSCH1 on PUCCH 2 when HARQ-ACK for PDSCH3 is reported.

Option 2 has the better performance and has no impact on gNB complexity. In fact, gNB must do blind detection on two PUCCHs. One is the PUCCH indicated by PRI field in a DL DCI which is correct PUCCH without transmission errors. The other PUCCH is the PUCCH for SPS PDSCH which is for the case if UE misses all dynamically scheduled PDSCHs. Therefore, gNB can correctly get the HARQ-ACK for SPS PDSCH by Option 2.

Proposal 2:

- If there is a previous PDSCH for which HARQ-ACK transmission timing is not determined until UE receives a SPS PDSCH, UE expects the HARQ-ACK timing for the previous PDSCH is no latter than the PUCCH configured for the HARQ-ACK transmission of the SPS PDSCH.

- No TP is needed

# Conclusion

In this contribution, we discussed the remaining issues on HARQ-ACK transmission for operations on unlicensed spectrum, and we derived the following proposals. It is proposed to agree on the related text proposals.

Proposal 1: For the case that DCI is detected but PDSCH is not decoded with sufficient processing time for one-shot feedback,

- Case 1: if the NDI in the latest detected DCI is NOT toggled, UE report actual HARQ-ACK for the HARQ process;

- Case 2: if the NDI in the latest detected DCI is toggled, UE reports NACK for the HARQ process.

Proposal 2:

- If there is a previous PDSCH for which HARQ-ACK transmission timing is not determined until UE receives a SPS PDSCH, UE expects the HARQ-ACK timing for the previous PDSCH is no latter than the PUCCH configured for the HARQ-ACK transmission of the SPS PDSCH.

- No TP is needed

# References

[1] R1-2000726, Enhancements to HARQ for NR-unlicensed, Intel Corporation

[2] R1-2004445, TP for Enhancements to Scheduling and HARQ operation for NR-U, Qualcomm Incorporated