3GPP TSG RAN WG1 #102-e                                                                                      R1-2005890

e-Meeting, August 17th – 28th, 2020

Source: 	Intel Corporation

Title:	Discussion on potential techniques for PUCCH coverage enhancement

Agenda item:	8.8.2.2

Document for:	Discussion and Decision

# Introduction

At the RAN1#101-e meeting, the following agreements were made for NR coverage enhancement [1]:

Agreements:

- Adopt the following target data rates for eMBB performance evaluation for FR1.

  - Urban scenario: DL 10Mbps, UL 1Mbps

  - Rural scenario: DL 1Mbps, UL 100kbps

  - Rural with long distance scenario: DL 1Mbps, UL 100kbps, [30kbps] (optional)

Agreements:

- For VoIP performance evaluation based on link-level simulation for FR1.

  - A packet size of [320] bits with 20ms data arriving interval is adopted.

  - FFSTBD: TBS for SIP invite message. Payload of 1500 bytes can be a starting point.

Agreements:

- For link level simulation, adopt the following table for PUSCH and PUCCH for FR1.

| Parameters | Values |
| --- | --- |
| Scenario and frequency | Urban: 4GHz (TDD), 2.6GHz (TDD) Rural: 4GHz (TDD), 2.6GHz (TDD), 2GHz (FDD), 700MHz (FDD) Rural with long distance: 700MHz (FDD), 4GHz (TDD) |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) only for 4GHz DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz DDDDDDDSUU (S: 6D:4G:4U) only for 2.6GHz Other frame structures can be reported by companies. |
| Pathloss model (select from LoS or NLoS) | Urban: NLoS Rural: NLoS and LoS |
| BWP | 100MHz for 4GHz and 2.6GHz. 20MHz for 2GHz (FDD 20MHz (optional for 10MHz) for 700MHz. (FDD) |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| Channel model for link-level simulation | TDL-C for NLOS, TDL-D for LOS. [CDL] |
| UE velocity | Urban: 3km/h for indoor Rural: 3km/h for indoor, 120km/h (optional 30km/h) for outdoor |
| Frequency hopping | w/ or w/o Intra-slot frequency hopping for PUSCH w/ frequency hopping for PUCCH is enabled. |

- FFS whether there are any additional simulation considerations for the extreme coverage scenarios (e.g., rural)

Agreements

- For link level simulation, adopt the following table for PUCCH for FR1.

| Parameters | Values |
| --- | --- |
| PUCCH format type | Format 1, 2bits UCI. Format 3, [4bits (3 bits A/N + 1 bit SR)]/11/22 bits UCI |
| BLER for PUCCH | For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. For PUCCH format 3: BLER for Ack/Nack, SR: 1% FFS: BLER for CSI (10% or 1%) |
| Number of PRBs for PUCCH | 1 PRB |
| Number of UE transmit chains for PUCCH | 1 |
| Number of repetitions for PUCCH | w/ repetition (optional), w/o repetition for PUCCH. The maximum number of repetitions is 8. |
| PUCCH duration | 14 OS |
| DMRS configuration for PUCCH | FFS: number of DMRS symbols for PUCCH Format 3. |

Agreements:

- For link level simulation, adopt the following table for eMBB data or VoIP on PUSCH and for PUCCH for FR1.

| Parameters | Values |
| --- | --- |
| Number of receive antenna elements for BS | Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) Rural: 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| Number of receive TxRUs for BS | gNB architectures to study for TDL: - 2 or 4 TXRUs for 2GHz, 700 MHz - 64TxRUs for 2.6 and 4 GHz. - Optional: 32 TXRUs at 2 GHz [gNB modeling in LLS for TDL: - Option 1: 2 or 4 gNB receive chains in LLS (as starting point). FFS: correlation - Option 2: Number of gNB receive chains = number of TXRUs in LLS. FFS: correlation.] [gNB architectures to study for CDL: - Urban: 64 receive chains for 2.6 and 4 GHz in LLS - Rural: 8 receive chains for 4GHz and 2.6GHz in LLS - 4 receive chains for 2GHz and 700MHz in LLS.] [gNB modeling in LLS for CDL: Number of gNB receive chains = number of TXRUs in LLS.] |
| Delay spread | Urban: 300ns Rural: 300ns Rural with long distance: 30ns |
| PRBs/TBS/MCS for eMBB for PUSCH | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use 30 PRBs for 1Mbps, 4 PRBs for 100kbps, 1 PRB for 30kbps as a starting point. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP for PUSCH | [4 PRBs] for VoIP as starting point. Other values of PRBs can be reported by companies. QPSK, pi/2 BPSK (optional) |

Note: For TDL models, companies report whether antenna array gain, obtained from mapping antenna elements to TXRU, is included in LLS or link budget template. Array gain calculation method and how channel estimation is accounted for is reported by companies

Agreements:

- For link level simulation, adopt the following table for PUCCH for FR2.

| Parameters | Values |
| --- | --- |
| Format | Format 1, 2bits UCI. Format 3, [4bits (3 bits A/N + 1 bit SR)]/11/22 bits UCI FFS: Format 0, 2 |
| BLER for PUCCH | The same as FR1 |
| Number of PRBs for PUCCH | The same as FR1 |
| Number of UE transmit chains for PUCCH | The same as FR1 |
| Number of repetitions for PUCCH | The same as FR1 |
| PUCCH duration | 14 OFDM symbols FFS: 4 OFDM symbols |
| DMRS configuration for PUCCH | FFS: [4] DMRS symbols for PUCCH Format 3. |

In the contribution, we discuss potential techniques for PUCCH coverage enhancement. Our views on baseline coverage performance for FR1 and FR2 are described in our companion contributions [2] and [3], respectively. In addition, our views on coverage enhancement for PUSCH and other physical channels are described in our companion contributions [4] and [5], respectively.

# Techniques for PUCCH coverage enhancement

Based on link budget analysis as presented in our companion contributions [2][3], it is proposed to focus on coverage enhancement for PUSCH and PUCCH format 3. In this section, we discuss potential techniques and present link level simulation results for PUCCH coverage enhancement, including

- Coverage enhancement in time domain

- Coverage enhancement in frequency domain

- Coverage enhancement in spatial domain

- Potential DMRS enhancement

- Advanced receiver for coverage enhancement

## Coverage enhancement in time domain

In NR Rel-15, multi-slot based transmission for PUCCH was supported, which can be used to improve the coverage. Further, the number of slots for PUCCH repetition can be 2, 4 or 8. To further improve the coverage, one straightforward approach is to increase the number of repetitions for PUCCH transmission, especially for PUCCH format 3 and 4.

Figure 1 illustrates link level simulation results for PUCCH format 3 with different number of repetitions. In the simulations, it is assumed 22 bit UCI payload and intra-slot frequency hopping for PUCCH format 3. In addition, it is assumed 2 DMRS symbols are allocated for each slot. From the figure, it can be observed that link level performance for PUCCH format 3 can be improved by increasing the number of repetitions. Further, ~2dB gain can be observed when doubling the repetition levels for PUCCH format 3.

Figure 1. Simulation results for PUCCH format 3 with repetitions

Observation 1

- ~2dB performance gain can be observed when doubling the repetition levels for PUCCH format 3.

When repetition is employed for the transmission of PUCCH, same time domain resource allocation is applied in each slot in Rel-15. If the number of symbols in each slot is limited, the coverage enhancement target for uplink transmission may not be satisfied even when the indicated repetition level is large. As shown in Figure 2, contiguous time domain resource allocation may be considered by spanning PUCCH in time as long as possible so as to improve the coverage performance for PUCCH.

Figure 2. Continuous uplink transmission in time for PUCCH repetition

Proposal 1

- It is desirable to increase the number of repetitions for PUCCH to further improve the coverage.

- Contiguous uplink transmission in time can be considered for PUCCH coverage enhancement.

## Coverage enhancement in frequency domain

In Rel-15, when inter-slot frequency hopping is enabled, UE transmits the PUCCH in one frequency resource in one slot and switches to another frequency resource in the next available slot. Note that this frequency hopping pattern may need to be further enhanced for PUCCH coverage enhancement.

For coverage limited scenario, channel estimation is typically a bottleneck in terms of link level performance. This may motivate the implementation of cross-slot channel estimation at the receiver to improve the channel estimation performance. To facilitate the cross-slot channel estimation, frequency resource for uplink transmission during the repetitions may remain the same for certain number of slots in order to allow inter-slot interpolation for channel estimation improvement.

Figure 3 illustrates one example of enhanced inter-slot frequency hopping pattern for coverage enhancement. In the example, PUCCH transmission occupies the same frequency resource for two slots before it switches to other frequency resource.

Figure 3. Enhanced inter-slot frequency hopping pattern for coverage enhancement

Figure 4 illustrates link level simulation results for PUCCH format 3 with different frequency hopping (FH) patterns. In the simulations, it is assumed 8 repetitions for PUCCH transmission with 1) intra-slot FH, 2) inter-slot FH and 3) enhanced inter-slot FH pattern with 4 consecutive slots in a same frequency resource. Further, cross-slot channel estimation is employed with a fixed window size of 4 slots.

From the figure, it can be observed that when employing cross-slot channel estimation, substantial performance gain, i.e., ~1.5dB, can be achieved for enhanced inter-slot frequency hopping pattern, compared to Rel-15 intra-slot and inter-slot frequency hopping pattern.

Figure 4. Simulation results for PUCCH format 3 with different frequency hopping patterns

Observation 2

- When employing cross-slot channel estimation, substantial performance gain, i.e., ~1.5dB can be achieved for enhanced inter-slot frequency hopping pattern, compared to Rel-15 intra-slot and inter-slot frequency hopping pattern.

Further, when relatively large number of repetitions, e.g., 8 or 16, is applied for PUCCH transmission, number of hops may be increased from 2 to 4, which can further improve the performance by exploiting the benefit of frequency diversity. Note that the performance gain may be more pronounced when a relatively large bandwidth is allocated for UL BWP.

Figure 5 illustrates link level simulation results for PUCCH format 3 when 2 and 4 frequency hops are employed. In the simulation, it is assumed 22 bit UCI payload and 2 DMRS symbols are allocated in each slot. From the figure, it can be observed that compared to 2 frequency hops, ~2dB performance gain can be achieved when 4 frequency hops are used for PUCCH format 3.

Figure 5. Simulation results for PUCCH format 3 with different number of hops

Observation 3

- ~2dB performance gain can be achieved when 4 frequency hops are employed for PUCCH format 3, compared to 2 frequency hops.

Proposal 2

- Enhanced inter-slot frequency hopping pattern can be considered for PUCCH coverage enhancement.

- For inter-slot frequency hopping, number of hops can be increased to 4 to further improve PUCCH coverage.

  - Coverage enhancement in spatial domain

When UE is equipped with multiple antennas or multiple panels, several options may be considered for PUCCH coverage enhancement by exploiting the benefit of spatial diversity:

- Precoder cycling in time: different precoders may be applied for the transmission of PUCCH during repetition. For this option, certain precoder cycling pattern may need to be defined.

- SORTD: orthogonal resources in time/frequency/code domain may be applied for the transmission of PUCCH in different APs. For instance, when two ports are defined, different frequency resources can be allocated for the PUCCH transmission on different ports. Note that this mechanism can be applied for PUCCH transmission without repetition.

In addition, for FR2, multi-TRP or multi-beam based PUCCH transmission may be employed to improve the coverage performance. As shown in Figure 6, different Tx beams can be applied for a first and second part of repetitions, which can help in reducing blockage and improving reliability and link budget of PUCCH transmission.

Figure 6. Multi-beam based PUCCH transmission

Note that as indicated in the FeMIMO WI [6], one of the objectives is to “identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline”. In our view, it is more appropriate to study multi-TRP/multi-beam based uplink transmission under FeMIMO WI, instead of NR coverage enhancement SI/WI.

Proposal 3

- Multi-TRP/multi-beam based PUCCH transmission for coverage enhancement is studied under FeMIMO WI.

## Potential DMRS enhancement

As mentioned above, for coverage limited scenario, channel estimation is typically a bottleneck in terms of link level performance. When increasing the reference signal density, channel estimation performance can be improved substantially at the cost of higher coding rate. This indicates that an appropriate tradeoff can be achieved between the channel estimation gain and coding loss. In Rel-15, flexible DMRS pattern including high DMRS density, e.g., 4 DMRS symbols for long PUCCH format was defined. For instance, for PUCCH format 3 and 4, when additionalDMRS is configured, 4 DMRS symbols can be associated with PUCCH transmission.

Figure 7 illustrates link level simulation results for PUCCH format 3 with 2 and 4 DMRS symbols. In the simulation, it is assumed 22 bit UCI payload and inter-slot frequency hopping. From the figure, it can be observed that for 8 repetitions, 4 DMRS symbols can achieve slightly better link level performance than 2 DMRS symbols. This is primarily due to the fact that with 8 repetitions, higher DMRS density can help in improving channel estimation performance, which is critical for PUCCH performance at low SNR regime.

Figure 7. Simulation results for PUCCH format 3 with 2 and 4 DMRS symbols

Observation 4

- For 8 repetitions with inter-slot frequency hopping, 4 DMRS symbols can achieve slightly better link level performance than 2 DMRS symbols for PUCCH format 3.

Note that when operating at relatively high SNR regime, it may be possible to consider DMRS-less operation in certain slots during PUCCH repetition. In this case, unused DMRS symbols can be allocated for UCI transmission, which can help reduce UCI code rate and improve PUCCH link budget. Further, advanced receiver including cross-slot channel estimation may need to be implemented in conjunction with DMRS-less scheme to ensure the decoding performance. In our view, careful study is needed to investigate DMRS enhancement, including higher DMRS density and DMRS-less scheme for PUCCH repetition.

Proposal 4

- RAN1 to further study DMRS enhancement including higher DMRS density and DMRS-less scheme.

## Advanced receiver for coverage enhancement

As mentioned above, advanced receiver including cross-slot channel estimation algorithm may help in improving the channel estimation performance, and hence overall link budget of uplink transmission. This is of primary importance when considering coverage enhancement solutions are mainly targeted for low SNR regime where channel estimation is typically performance bottleneck.

Figure 8 illustrates link level simulation results for PUCCH format 3 with and without cross-slot channel estimation. In the simulations, it is assumed 22 bit UCI payload and inter-slot frequency hopping. Further, two algorithms for cross-slot channel estimation are considered in the simulation:

- Sliding window based cross-slot channel estimation: the estimated channels on the DMRS symbols from previous N slots including current slot in a same frequency resource are jointly interpolated by using a 2D-MMSE filter to obtain the channel estimates on the data REs for current slot. In one example, if N = 4, for the first slot, only estimated channels on the DMRS symbols from the first slot are used for channel estimation on the data REs, while for the second slot, estimated channels on the DMRS symbols from the first and second slot are jointly used for channel estimation on the data REs in the second slot, and so on.

- Fixed window based cross-slot channel estimation: The estimated channels on the DMRS symbols from a fixed window size of N slots are all computed and then jointly interpolated by using a 2D-MMSE filter to obtain the channel estimates on the data REs for N slots. In the simulation, it is assumed a fixed window size of 4 slots is used for cross-slot channel estimation.

From the figure, it can be observed that for PUCCH format 3 with 8 repetitions and inter-slot frequency hopping, when employing cross-slot channel estimation algorithm, >1dB performance gain can be achieved compared to without cross-slot channel estimation.

Figure 8. Simulation results for PUCCH format 3 with cross-slot channel estimation

Observation 5

- For PUCCH format 3 with 8 repetitions and inter-slot frequency hopping, when employing cross-slot channel estimation algorithm, >1dB performance gain can be achieved compared to without cross-slot channel estimation.

- Advanced receiver needs to be considered when evaluating performance for different techniques for coverage enhancement.

# Conclusions

In this contribution, we discussed potential techniques for PUCCH coverage enhancement. Further, we summarize the observations and proposals as follows:

Observation 1

- ~2dB performance gain can be observed when doubling the repetition levels for PUCCH format 3.

Observation 2

- When employing cross-slot channel estimation, substantial performance gain can be achieved for enhanced inter-slot frequency hopping pattern, compared to Rel-15 intra-slot and inter-slot frequency hopping pattern.

Observation 3

- ~2dB performance gain can be achieved when 4 frequency hops are employed for PUCCH format 3, compared to 2 frequency hops.

Observation 4

- For 8 repetitions with inter-slot frequency hopping, 4 DMRS symbols can achieve slightly better link level performance than 2 DMRS symbols for PUCCH format 3.

Observation 5

- For PUCCH format 3 with 8 repetitions and inter-slot frequency hopping, when employing cross-slot channel estimation algorithm, >1dB performance gain can be achieved compared to without cross-slot channel estimation.

- Advanced receiver needs to be considered when evaluating performance for different techniques for coverage enhancement.

Proposal 1

- It is desirable to increase the number of repetitions for PUCCH to further improve the coverage.

- Contiguous uplink transmission in time can be considered for PUCCH coverage enhancement.

Proposal 2

- Enhanced inter-slot frequency hopping pattern can be considered for PUCCH coverage enhancement.

- For inter-slot frequency hopping, number of hops can be increased to 4 to further improve PUCCH coverage.

Proposal 3

- Multi-TRP/multi-beam based PUCCH transmission for coverage enhancement is studied under FeMIMO WI.

Proposal 4

- RAN1 to further study DMRS enhancement including higher DMRS density and DMRS-less scheme.

# References

- Chairman’s notes, RAN1 #101 e-Meeting, May 2020

- R1-2005887, “Discussion on baseline coverage performance for FR1”, Intel Corporation, e-Meeting, August 2020

- R1-2005888, “Discussion on baseline coverage performance for FR2”, Intel Corporation, e-Meeting, August 2020

- R1-2005889, “Discussion on potential techniques for PUSCH coverage enhancement”, Intel Corporation, e-Meeting, August 2020

- R1-2005891, “Discussion on NR coverage enhancement for other physical channels”, Intel Corporation, e-Meeting, August 2020

- RP-193133, “New WID: Further enhancements on MIMO for NR”, Samsung, Sitges, Spain, December 2019