3GPP TSG-RAN WG1 Meeting #102-e	Tdoc R1-2005917

e-Meeting, 17th – 28th August, 2020

Agenda Item:	7.2.2.2.4

Source:	Ericsson

Title:	Configured Grant Enhancement

Document for:	Discussion, Decision

# 1	Introduction

This document is for the purposes of NR-U maintenance under the configured grant enhancement agenda item.

# 2	Discussion

In the previous meeting, it was discussed whether the redundancy version of the first repetition in a CG-PUSCH including CG-UCI should be determined by the UE or should be set to 0. This issue was postponed for resolution to this meeting and the corresponding text was captured within [] in Clause 6.1.2.3.1 of TS 38.214.

Our view is that the disputed text should be removed from the specification TS38.214 for the following reasons:

In principle, for TB transmissions using configured grants, a transmission occasion with CG-PUSCH transmission is determined by the UE. For operation on unlicensed, another level of uncertainty for using a transmission occasion is added due to the uncertainty in LBT success.

Hence, extending the RV selection based UE to all the CG-PUSCH transmission is beneficial to simplify UE implementation considering the time needed for preparation of a PUSCH transmission.

Additionally, constraining the first transmission to be based on RV=0, does not provide considerable simplification for the gNB to justify a different behaviour for different transmissions since the gNB has to perform blind detection to detect a CG-PUSCH transmission in a transmission occasion and decode the corresponding CG-UCI irrespective of the potential assumption of RV=0 for first transmission.

Therefore, we propose the following:

- Proposal 1	If cg-RetransmissionTimer is provided, the redundancy version for uplink transmission with a configured grant, including the first transmission is determined by the UE.

- Adopt the following TP1 is Clause 6.1.2.3.1 of TS38.214.

----------------------------------------- Beginning of Text Proposal 1 (TS38.214)------------------------------------------------ 6.1.2.3.1 Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant The procedures described in this clause apply to PUSCH transmissions of PUSCH repetition Type A with a Type 1 or Type 2 configured grant. The higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. If cg-RetransmissionTimer is provided, the redundancy version for uplink transmission with a configured grant is determined by the UE, [except for the redundancy version of the first repetition that is set to 0]. If the parameter repK-RV is not provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, the redundancy version for uplink transmissions with a configured grant shall be set to 0. If the parameter repK-RV is provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, for the nth transmission occasion among K repetitions, n=1, 2, …, K, it is associated with (mod(n-1,4)+1)th value in the configured RV sequence. If a configured grant configuration is configured with Configuredgrantconfig-StartingfromRV0 set to 'off', the initial transmission of a transport block may only start at the first transmission occasion of the K repetitions. Otherwise, the initial transmission of a transport block may start at - the first transmission occasion of the K repetitions if the configured RV sequence is {0,2,3,1}, - any of the transmission occasions of the K repetitions that are associated with RV=0 if the configured RV sequence is {0,3,0,3}, - any of the transmission occasions of the K repetitions if the configured RV sequence is {0,0,0,0}, except the last transmission occasion when K≥8. ---------------------------------------------------- End of Text Proposal 1----------------------------------------------------

# 3	Conclusion

In this contribution, we discussed our view on the redundancy version of the first repetition in a CG-PUSCH and proposed the following:

If cg-RetransmissionTimer is provided, the redundancy version for uplink transmission with a configured grant, including the first transmission is determined by the UE.