3GPP TSG RAN WG1 #102-e Meeting	    			             R1-2006099

e-Meeting, August 17th – 28th, 2020

Agenda item:	7.2.4.1

Source: 	Samsung

Title: 	On Physical Layer Structures for NR Sidelink

Document for:	Discussion and Decision

Introduction

It was declared that the WI for 5G V2X with NR sidelink was completed in RAN#86. However, there are still some remaining issues to be resolved in current version of Rel-16 NR specifications [2]-[5]. In this contribution, we provide views on the remaining issues on sidelink physical layer structure.

Issues

Mapping of 2nd SCI in PSSCH

The number of coded modulation symbols for 2nd SCI is determined as below.

For 2nd-stage SCI transmission on PSSCH with SL-SCH, the number of coded modulation symbols generated for 2nd-stage SCI transmission prior to duplication for the 2nd layer if present, denoted as QSCI2', is determined as follows: QSCI2'=minOSCI2+LSCI2∙βoffsetSCI2QmSCI2∙R, αl=0NsymbolPSSCH-1MscSCI2(l)+γ where - OSCI2 is the number of the 2nd-stage SCI bits - LSCI2 is the number of CRC bits for the 2nd-stage SCI, which is 24 bits. - βoffsetSCI2 is indicated in the corresponding 1st-stage SCI. - MscPSSCH(l) is the scheduled bandwidth of PSSCH transmission, expressed as a number of subcarriers; - MscDMRS(l) is the number of subcarriers in OFDM symbol l that carries DMRS, in the PSSCH transmission. - MscPT-RS(l) is the number of subcarriers in OFDM symbol l that carries PT-RS, in the PSSCH transmission. - MscSCI2(l) is the number of resource elements that can be used for transmission of the 2nd-stage SCI in OFDM symbol l, for l=0,1,2⋯,NsymbolPSSCH-1 and for NsymbolPSSCH=Nsymbsh-NsymbPSFCH, in PSSCH transmission, where NsymbshNsymbslotNsymbslot = sl-lengthSymbols - 2, where sl-lengthSymbols is the number of sidelink symbols within the slot provided by higher layers as defined in [6, TS 38.214]. If higher layer parameter sl-PSFCH-Period = 2 or 4, NsymbPSFCH = 3 if "PSFCH overhead indication" field of SCI format 1-A indicates "1", and NsymbPSFCH = 0 otherwise. If higher layer parameter sl-PSFCH-Period = 0, NsymbPSFCH=0. If higher layer parameter sl-PSFCH-Period is 1, NsymbPSFCH=3. - MscSCI2(l) = MscPSSCH(l) - MscDMRS(l) - MscPT-RS(l) - γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. - R is the coding rate as indicated by "Modulation and coding scheme" field in SCI format 1-A. - α is configured by higher layer parameter sl-Scaling. The input bit sequence to rate matching is d0, d1,d2,d3, ⋯,dN-1, where N is the number of coded bits. Rate matching is performed according to Clause 5.4.1 by setting IBIL=1. The output bit sequence after rate matching is denoted as g0SCI2, g1SCI2, g2SCI2, g3SCI2,⋯, gGSCI2-1SCI2, where GSCI2=QSCI2'∙QmSCI2 and QmSCI2 is modulation order of the 2nd-stage SCI. A UE is not expected to have GSCI2>K.

Since the 2nd SCI is mapped from the first DMRS symbol of the corresponding PSSCH, the starting symbol used for mapping the 2nd SCI could be different from initial transmission and retransmission as shown in the following figure.

With this understanding, the maximum limit could be differently applied to the mapping and also SL-CSI-RS and PT-RS in different symbols could be impact on the mapping differently. Therefore, one possible solution is that the same DMRS symbols for PSSCH are assumed for initial transmission and retransmission(s) when the 2nd SCI mapping is considered for a TBS. For example, when multiple DMRS patterns are configured, one of the patterns could be assumed to use.

Proposal 1: When the 2nd SCI mapping is considered for a TBS, DMRS pattern for PSSCH are assumed to be the same for initial transmission and retransmission(s).

As already explained, the maximum limit could be differently applied to the mapping due to different DMRS patterns for initial transmission and retransmission(s). Similar to UCI mapping enhancement specified for eURLLC in Rel-16, additional maximum limit can be considered in the formula as

QSCI2'=minOSCI2+LSCI2∙βoffsetSCI2∙l=0NsymbolPSSCH-1MscSCI2(l)r=0CSL-SCH-1Kr, αl=0NsymbolPSSCH-1MscSCI2(l), NRE+γ,

where NRE is the number of available REs for the 2nd SCI mapping.

Proposal 2: Update QSCI2' as QSCI2'=minOSCI2+LSCI2∙βoffsetSCI2∙l=0NsymbolPSSCH-1MscSCI2(l)r=0CSL-SCH-1Kr, αl=0NsymbolPSSCH-1MscSCI2(l), NRE+γ, where NRE is the number of available REs for the 2nd SCI mapping.

.

Supported max data rate for NR sidelink

In TS38.306, the supported max data rate is defined for NR Uu. Similarly, the max data rate supported for NR sidelink should be specified for the calculation of layer 2 buffer size. RAN2 sent the LS R1-2003258 to RAN1 to ask to define the max data rate.

For the max data rate for NR sidelink, the formula for the max data rate of NR Uu is reused with the following parameters.

- The number of aggregated component carriers, J, is equal to 1.

- is the maximum number of supported layers for sidelink TX and RX, respectively.

- is the maximum supported modulation order for sidelink TX and RX, respectively.

- is 1.

- is 0.21 for for sidelink TX and RX.

Proposal 3: The max data rate for NR sidelink is specified and the formula to calculate the max data rate of NR Uu is reused with the following parameters

- J: the number of aggregated component carrier (J  = 1),

- is the maximum number of supported layers for sidelink TX and RX, respectively,

- is the maximum supported modulation order for sidelink TX and RX, respectively,

- is 1,

- is 0.21 for for sidelink TX and RX.

Conclusions

In this contribution, we provide our views on the physical layer structures for NR sidelink with the following proposals.

Proposal 1: When the 2nd SCI mapping is considered for a TBS, DMRS pattern for PSSCH are assumed to be the same for initial transmission and retransmission(s).

Proposal 2: Update QSCI2' as QSCI2'=minOSCI2+LSCI2∙βoffsetSCI2∙l=0NsymbolPSSCH-1MscSCI2(l)r=0CSL-SCH-1Kr, αl=0NsymbolPSSCH-1MscSCI2(l), NRE+γ, where NRE is the number of available REs for the 2nd SCI mapping.

Proposal 3: The max data rate for NR sidelink is specified and the formula to calculate the max data rate of NR Uu is reused with the following parameters

- J: the number of aggregated component carrier (J  = 1),

- is the maximum number of supported layers for sidelink TX and RX, respectively,

- is the maximum supported modulation order for sidelink TX and RX, respectively,

- is 1,

- is 0.21 for for sidelink TX and RX.

- References

[1] RP‑191723, “Revised WID: 5G V2X with NR sidelink,” Sept. 2019.

[2] 3GPP TS 38.211 V16.2.0, “NR; Physical channels and modulation,” June 2020.

[3] 3GPP TS 38.212 V16.2.0, “NR; Multiplexing and channel coding,” June 2020.

[4] 3GPP TS 38.213 V16.2.0, “NR; Physical layer procedures for control,” June 2020.

[5] 3GPP TS 38.214 V16.2.0, “NR; Physical layer procedures for data,” June 2020.