3GPP TSG RAN WG1 #102-e	                                                                 R1-2006136

# e-Meeting, August 17th – 28th, 2020

Agenda item:	8.2.1

Source:	Samsung

Title:	Design aspects for extending NR to up to 71 GHz

Document for:	Discussion and Decision

# Introduction

In RAN #86, a new study item for Rel-17 was approved to extend NR to up to 71 GHz [1]. This contribution will discuss the design aspects for extending NR to up to 71 GHz, including the general aspects of the operating bands from 52.6 GHz to 71 GHz, target development scenarios, new numerology and carrier bandwidth to be supported for this carrier frequency range, and identified impacts to NR PHY design assuming same DL and UL waveforms are reused.

# General Aspects for Operating Bands from 52.6 to 71 GHz

TR 38.807 [2] has summarized the current licensing situation for data communication for carrier frequency between 52.6 GHz and 114.25 GHz for various countries under ITU region 1, 2, and 3. Table 1 shows the situation for carrier frequency between 52.6 GHz and 71 GHz, as the focus of this study item.

Table 1 Current licensing situation between frequency 52.6GHz and 71GHz [2].

| Region | Country | Frequency (GHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 52.6-54.25 | 54.25-55.78 | 55.78-56.9 | 56.9-57 | 57-58.2 | 58.2-59 | 59-59.3 | 59.3-64 | 64-65 | 65-66 | 66-711) |
| ITU Region 1 | Europe/CEPT |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Israel |  |  |  |  |  |  |  |  |  |  |  |
|  | South Africa |  |  |  |  | U (Mobile) |  |  |  | U (Mobile) |  |  |
| ITU Region 2 | USA |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Canada |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Brazil |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Mexico |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
| ITU Region 3 | China |  |  |  |  |  |  | U (Mobile) |  |  |  |  |
|  | Japan |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Korea |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | India |  |  |  |  |  |  |  |  |  |  |  |
|  | Taiwan |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Singapore |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Australia |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
| NOTE 1: Access regime currently under discussion in CEPT |  |  |  |  |  |  |  |  |  |  |  |  |

It can be observed that only unlicensed bands are currently identified between 52.6 GHz and 71 GHz, and the identified unlicensed bands are all located between 57 GHz and 71 GHz (i.e., so called 60 GHz unlicensed bands), wherein the carrier frequency range between 57 GHz to 66 GHz is globally unlicensed, while the carrier frequency range between 66 GHz and 71 GHz is only applicable in US at least for now.

It has been clarified in [1] that the design target of the study item includes both licensed and unlicensed bands, however, due to identified licensed situation, it would be more efficient to prioritize unlicensed band (i.e., 60 GHz unlicensed band) in this study item, due to a much clearer situation of the channelization and regulation on that band. Moreover, there could be a chance of licensing licensed band between 66 GHz to 71 GHz, other than US, then there is a potential possibility to have overlapping bandwidth supporting both licensed and unlicensed operations, which should be taken into account when designing the system.

Observation 1: Only unlicensed bands are currently identified between 57 GHz and 71 GHz.

Observation 2: There is a potential to support both licensed and unlicensed operations over the bandwidth between 66 GHz and 71 GHz.

Proposal 1: The study item shall support both licensed and unlicensed operation between 52.6 GHz and 71 GHz, and unlicensed band between 57 GHz and 71 GHz (i.e., 60 GHz unlicensed band) should be prioritized.

# Carrier Bandwidth and Numerology

One of the key aspects for this study item is to identify the carrier bandwidth and numerology for carrier frequency between 52.6 GHz and 71 GHz, assuming same DL and UL waveform of NR are reused. In NR Rel-15 and Rel-16, a supported maximum carrier bandwidth is 100 MHz for FR1 (corresponding to SCS of 30 kHz with FFT size 4096), and 400 MHz for FR2 (corresponding to SCS of 120 kHz with FFT size 4096). However, such bandwidth may not be sufficient for operation between 52.6 GHz and 71 GHz. For example, at least for 60 GHz unlicensed band, a carrier with bandwidth as large as 2.16 GHz should be supported, wherein the channelization is same as the coexistence RAT such as 802.11 ad operating on the same carrier frequency range.

In NR Rel-15 and Rel-16, a supported SCS is scaled from 15 kHz and in the form of 15⋅2μ kHz, and the maximum FFT size is 4096. To simplify the hardware changes, in order to support the carrier frequency between 52.6 GHz and 71 GHz, the same maximum FFT size shall be maintained. According to this design target, at least for 60 GHz unlicensed band with 2.16 GHz channelization, 960 kHz SCS shall be considered as a baseline. A SCS smaller than 960 kHz will result in a FFT size larger than 4096 (as shown in Table 2), and shall not be supported for the channelization of 2.16 GHz; while a SCS larger than 960 kHz (e.g. 1920 kHz) will result in very small symbol duration, so RAN1 shall carefully study its necessity, especially when 960 kHz could be sufficient.

Moreover, in NR Rel-15 and Rel-16, both normal CP length (i.e., 14 symbols in a slot) and extended CP (i.e., 12 symbols in a slot) are supported, wherein the NCP is supported for all the SCS, and ECP is only supported for 60 kHz SCS. ECP is mainly targeted for the scenario with long delay spread, wherein the CP length in NCP may not be sufficient. For the same reasoning, both NCP and ECP shall be studied for the carrier frequency between 52.6 GHz and 71 GHz, taking into consideration the targeted development scenarios clarified by RAN1.

A summary of the FFT size, symbol duration, and CP length with respect to different SCS and CP length is illustrated in Table 2, wherein a carrier with 2.16 GHz bandwidth is assumed.

Table 2 A summary of FFT size, symbol duration, and CP length for 2.16 GHz carrier.

| SCS | FFT size | Symbol duration (NCP) | CP length (NCP) | Symbol duration (ECP) | CP length (ECP) |
| --- | --- | --- | --- | --- | --- |
| 480 kHz | 8192 | 2.23 us | 0.15 us | 2.60 us | 0.52 us |
| 960 kHz | 4096 | 1.11 us | 0.07 us | 1.30 us | 0.26 us |
| 1920 kHz | 2048 | 0.56 us | 0.04 us | 0.65 us | 0.13 us |

Other than a new channelization of 2.16 GHz, at least one of the supported channelization and numerology in FR2 could also be supported for 52.6 GHz to 71 GHz, wherein the same maximum FFT size of 4096 is assumed. More precisely, a carrier bandwidth of 400 MHz with SCS of 120 kHz can be supported, and the supporting of other SCS for 400 MHz carrier bandwidth should be further studied, taking account the potential specification impact and use case. If 400 MHz with 120 kHz SCS is sufficient, other SCS is not needed.

Moreover, when both 2.16 GHz carrier bandwidth with 960 SCS and 400 MHz carrier bandwidth with 120 kHz SCS are supported, one further aspect to be studied is whether mixture of the two SCSs is expected by the UE. Supporting multiplexing of signals/channels with different SCSs (e.g. either TDMed or FDMed) may provide flexibility in implementation, but requires more specification impact (especially for a larger SCS ratio). Hence, RAN1 shall further study the necessity of supporting mixture of numerology for above 52.6 GHz.

Proposal 2: The maximum FFT size for the new carrier frequency range should be kept as 4096.

- The following two combinations of channel bandwidth and numerology shall be supported:

  - Carrier bandwidth of 2.16 GHz with SCS of 960 kHz (further study NCP and/or ECP);

  - Carrier bandwidth of 400 MHz with SCS of 120 kHz and NCP.

- Further study whether the support of other SCS is needed.

- Further study whether mixed numerology is needed.

# Impacts to PHY Design

According to the support of new numerology(ies) and new carrier bandwidth, the physical layer design will be impacted. At least the following aspects are expected to be modified/enhanced from NR FR2, in order to support the operation between 52.6 GHz and 71 GHz.

## SS/PBCH Block

In Rel-15 and Rel-16, SS/PBCH block is only supported for 15 kHz and 30 kHz for FR1, and 120 kHz and 240 kHz for FR2, as shown in Figure 1. For the new carrier frequency range 52.6 to 71 GHz, there could be new numerology supported, and the SS/PBCH block pattern needs to be defined for the new numerology. Scaling from existing SS/PBCH block pattern can be considered as the baseline, but design aspects including the switching time between neighboring SS/PBCH block beams shall be taken into account. For example, if reusing Case D for 960 kHz SCS, the gap between the neighboring SS/PBCH block in a slot is 0 symbol, which implies the beam switching duration of the SS/PBCH blocks has to be restricted within the CP length, which is 0.07 us according to Table 2. This restriction may not be achievable in implementation, so at least one symbol (or even more) should be guaranteed in time domain between the neighboring SS/PBCH blocks in a slot.

Figure 1 Illustration of supported SS/PBCH block pattern in Rel-15 and Rel-16.

Proposal 3: RAN1 shall study the SS/PBCH block pattern for the new numerology, taking into account the beam switching time between neighboring SS/PBCH blocks.

## RACH

PRACH preamble is also not supported for the new numerology, but since the PRACH can have different numerology compared to other UL channels in NR; thus a direct scaling or reuse from existing numerology can be considered. Meanwhile, due to the much larger SCS, the symbol duration is much shorter than NR Rel-15/16.

Given that if LBT is still required before each PRACH transmission, the consecutive allocation of ROs within a PRACH slot could lead to the scenario where LBT for a RO is failed due to a PRACH transmission utilizing the previous RO. In a typical scenario, the duration for LBT can expand over a number of symbols, such that a LBT gap is needed before a RO. The gap duration between neighboring ROs can be achieved by configuring UEs to only utilize the RACH occasions with odd indexes 2n+1 (e.g., the 1-st, 3-rd, 5-th) within the RACH slot for PRACH transmissions, while the RACH occasions with even indexes 2n (e.g., the 0-th, 2-nd, 4-th) can be used for the LBT operation of UEs that will utilize RO with index 2n+1. This approach can directly use the existing PRACH configuration table from Rel-15/16 and thus requires less modification from current spec. An illustration of this approach is illustrated in Figure 2, wherein 6 ROs with PRACH preamble format A1 are configured within the RACH slot, among which 3 ROs are configured as the LBT gap.

Figure 2 An example of creating LBT gap through even/odd RO indexes.

Proposal 4: RAN1 shall study the scaling/reuse of current PRACH SCS with some enhancement including the non-consecutive RO configuration.

## Processing Timing Related Procedure

Processing timing related procedures, e.g. PDSCH to HARQ-ACK processing time, PDCCH to PDSCH or PUSCH scheduling delay, and HARQ process, CSI computation time, and PDCCH monitoring are all sensitive to the time domain unit.

For operation between 52.6 GHz and 71 GHz, if a new numerology is supported, the timing indication granularity and range should be studied to be compatible with the new time domain unit. Meanwhile, keeping trackable overhead in the indication should also be studied, in order to minimize the specification change from Rel-15 and Rel-16.

For PDCCH monitoring, a much shorter duration of a slot with large SCS, e.g. slot duration is 1/64 ms with 960KHz SCS, would lead to very frequent PDCCH detection, if a UE monitors PDCCH in each slot. It undesirably increases UE power consumption as well as increases UE complexity and capability, because the number of PDCCH BDs within a period, e.g. 1 ms would be dramatically increases, which would be out of practical UE capability. On the other hand, to keep similar UE capability as Rel-16, the number of PDCCH BD per slot is dramatically decreased which would seriously impact the scheduling flexibility and increases PDCCH blocking probability.  One possible compromise way is to relax per slot PDCCH BD capability to per reference period PDCCH BD capability. For example, taking the slot duration of 120KHz SCS as the reference duration, the total number of PDCCH BD within 8 slots of 960KHz SCS is limited by the maximum PDCCH BD candidates number defined for one slot of 120KHz SCS. Alternatively, extending the span of PDCCH monitoring to more than one slot for maximum PDCCH BD capability can also relax UE processing timeline.

If PDCCH SS periodicity is larger than one slot, cross-slot scheduling is needed to enable PDSCH/PUSCH transmissions in slots between 2 consecutive PDCCH SSs to fully utilize DL/UL resources.  If one DCI can only schedule one PDSCH/PUSCH, the number of PDCCHs in one PDCCH monitoring occasion increases, which increases UE PDCCH BD burden. It is beneficial to consider multi-PDSCH/PUSCH scheduled by a single DL assignment/UL grant to keep reasonable PDCCH BD complexity. In Rel-16 NR-U, multi-PUSCH scheduled by a single UL grant is already supported. It is simple to extend such scheduling to PDSCH.

Proposal 5: RAN1 shall study the processing timing related procedures for modification/enhancement, taking into consideration of the impact from the new numerology.

- Timing indication (K0/K1/K2)

- HARQ procedure with increased value of K0/K1/K2

- PDCCH monitoring with practical PDCCH BD capability

- Multi-PDSCH/PUSCH scheduling

## Resource Allocation

With large SCS, the slot duration is much smaller, i.e., 1/8 with 960kHz SCS of the slot duration with 120kHz SCS. As discussed in previous section, due to hardware constraint, the processing time will not be scaled down linearly. Moreover, considering potential longer PDCCH SS periodicity, round-trip time of a packet will increase accordingly. With same HARQ process number, even transmission time of PUSCH/PDSCH is within several symbols or within one slot, it will not increase the practical data rate. In addition, with restriction on maximum transmission power, the coverage will be restricted by transmission time. Therefore, slot bundling can be considered to increase the coverage, at least for PUSCH.

Since the slot duration is smaller and we mainly focus on stationary scenario, coherent time may be longer compared with the slot duration.  If slot bundling is considered, DMRS time domain density may be able to further decrease to reduce the overhead.

PSD boosting to a smaller BW can increase SINR at receiver side. The channel estimation performance is better at high SINR and a better BLER performance can be expected. With 960kHz SCS, 1 PRB occupies 11.52MHz bandwidth. If a decent coverage needs to be achieved, PSD boosting can be used to increase SINR at receiver side. 1 PRB or even sub-PRB/subcarrier bundle based frequency domain scheduling can be further studied. On the other hand, for the operation over unlicensed band, PSD boosting may not be achieved by allocating few PRB, because PSD is limited by 1MHz which is almost the same as one RE bandwidth. In such case, to fully utilize the maximum transmission power as well as to meet OCB requirement, it would be desirable to always allocate several PRBs for a UE. Then, a much shorter duration in time domain is expected for unlicensed band compared with the case of single PRB or sub-PRB allocation with multiple slots over licensed band. Therefore, more flexible resource allocation in both time and frequency domain for different scenarios should be studied.

Proposal 6: RAN1 shall study more flexible resource allocation in both time and frequency domain for different scenarios, including slot bundling, subcarrier bundling/sub-PRB.

## Beam Adjustment in Initial Access

The beam related issue has been raised in last meeting for initial access in FR2. Due to the lack of (frequent) beam tracking in the initial access, the practical implementation faces that the beam will be failed during the RACH procedure. Normally, UE selects the PRACH resource associated with the SSB it picked with relatively high RSRP; and by successfully received the correct the feedback for gNB, the UE is able to set-up a working beam pair with gNB. The beam related issues are mainly in two aspects:

- For DL beams, unlike the beam tracking procedure after RRC connection, e.g., SRS or CSI-RS, the UE might experience the loss of preferred beam while not knowing it. For example, for a SSB selected for RACH transmission, UE will start the msg.3 transmission after receiving a correct RAR and also start the contention resolution timer. However, such timer could be as large as 64 ms, the preferred beam might be changed due to the environment change or mobility, thus the msg3 might be failed causing by the DCI is missed as shown in following figure. Then the DL beam adjustment for initial access including the finer beam reference signal measurement and also multi-beam selection/reporting during the procedure could be considered.

Figure 3 Impact of preferred DL beam changed during random access.

- For UL beams, all UE holds the beam correspondence capability in current NR. But some of the UEs need the assistance of gNB. Besides, during the random access procedure, the Tx beam for msg.1 is up to UE implementation. According to the DL measurement, the UE needing no assistance could directly determine which UL Tx beam is good based on DL Rx beam corresponding to the selected SSB.  However,  the UE needing the assistance need to try different UL Tx beam in one procedure, and based on the feedback for gNB (e.g., whether a correct RAR could be received), UE can finally identify the previously used the UL Tx beam is good enough or not. For >52.6GHz case, the beam will be even narrower, and the attempts for the UE to find the matched/working beam might be even longer than before. This is not only adding the access delay in the initial access and as we discussed above, the longer time may also increase the possibility that UE may lose the track of best/good DL Tx beam.

For msg.1 with beam based transmission, the RO bundle could be considered for msg.1 enhancement.  The RO bundle means that a number of RO is grouped together, and UE can transmit the msg.1 per RO bundle instead of only one RO. The benefits of the RO bundle could be in two perspectives:

- Transmitting preambles in multiple UL Tx beams at one RACH attempt as shown in Figure 5 (a). This is particularly beneficial for these UEs needing assistance for beam correspondence. Generally, the UE could identify the matched beam with less re-attempt times, thus save access latency;

- The preambles in different ROs in a RO bundle could be regarded as repetitions, which can provide link budget enhancement for PRACH reception, even for these UEs who already know which Tx beam is the best as shown in Figure 5 (b).

Figure 4 RO bundle for same Tx beam an different Tx beam.

Proposal 7: RAN1 shall study the beam adjustment mechanism in initial access procedure.

## Reference Signals

It is well-known that phase noise can be divided to two parts, CPE (common-phase error) and ICI (inter-carrier interference). Rel-15 PT-RS design focused on CPE compensation, but the effect of phase noise is more severe for higher frequency due to ICI. Hence, for the new carrier frequency range, the PT-RS can be enhanced to address the ICI issue.

In this contribution, we illustrate the ‘chunk’ based pattern for PT-RS. Assuming that the whole RB allocation is divided into Nc chunks (groups) and the PT-RS is located in the middle of each chunk. Each chunk contains NPTRS subcarriers. Figure 5 shows the aforementioned PT-RS pattern. This proposed PT-RS pattern can be considered as a generalization of Rel-15 PT-RS, in the sense that the distribute PT-RS pattern in Rel-15 can be represented as NPTRS=1, and the fully localized PT-RS pattern can be represented as Nc=1.

Figure 5 Illustration of proposed PT-RS pattern.

Proposal 8: RAN1 shall study the enhancement to reference signals (e.g. PT-RS) for the new carrier frequency range, taking into consideration of the impact from the new numerology.

# Conclusion

In this contribution, we have discussed design aspects for extending NR to up to 71 GHz. Observations and proposals of this contribution are summarized as follows.

Observation 1: Only unlicensed bands are currently identified between 57 GHz and 71 GHz.

Observation 2: There is a potential to support both licensed and unlicensed operations over the bandwidth between 66 GHz and 71 GHz.

Proposal 1: The study item shall support both licensed and unlicensed operation between 52.6 GHz and 71 GHz, and unlicensed band between 57 GHz and 71 GHz (i.e., 60 GHz unlicensed band) should be prioritized.

Proposal 2: The maximum FFT size for the new carrier frequency range should be kept as 4096.

- The following two combinations of channel bandwidth and numerology shall be supported:

  - Carrier bandwidth of 2.16 GHz with SCS of 960 kHz (further study NCP and/or ECP);

  - Carrier bandwidth of 400 MHz with SCS of 120 kHz and NCP.

- Further study whether the support of other SCS is needed.

- Further study whether mixed numerology is needed.

Proposal 3: RAN1 shall study the SS/PBCH block pattern for the new numerology, taking into account the beam switching time between neighboring SS/PBCH blocks.

Proposal 4: RAN1 shall study the scaling/reuse of current PRACH SCS with some enhancement including the non-consecutive RO configuration.

Proposal 5: RAN1 shall study the processing timing related procedures for modification/enhancement, taking into consideration of the impact from the new numerology.

- Timing indication (K0/K1/K2)

- HARQ procedure with increased value of K0/K1/K2

- PDCCH monitoring with practical PDCCH BD capability

- Multi-PDSCH/PUSCH scheduling

Proposal 6: RAN1 shall study more flexible resource allocation in both time and frequency domain for different scenarios, including slot bundling, subcarrier bundling/sub-PRB.

Proposal 7: RAN1 shall study the beam adjustment mechanism in initial access procedure.

Proposal 8: RAN1 shall study the enhancement to reference signals (e.g. PT-RS) for the new carrier frequency range, taking into consideration of the impact from the new numerology.

# Reference

[1] RP-193259, New SID: Study on supporting NR from 52.6GHz to 71 GHz, Intel Corporation, 2019.

[2] 3GPP TR 38.807 v16.0.0, Study on requirements for NR beyond 52.6 GHz, 2019.