3GPP TSG RAN WG1 #102e 		                                      R1-2006225

e-Meeting, August 17th – 28th, 2020

Agenda item:	8.8.1.2

Source: 	CMCC

Title: 	Discussion on the baseline performance in FR2

Document for:	Discussion and Decision

Introduction

In the RAN1#101e meeting, significant progresses had been accomplished. Most the evaluation assumptions had been agreed [1]. a few issues remain for further study.

Agreements:

- For link level simulation, adopt the following simulation assumption for eMBB data or VoIP on PUSCH and on PDSCH for FR2.

| Parameters | Values |
| --- | --- |
| Number of UE antenna elements | 8, one panel:(M, N, P) = (2,2,2), FFS: Two panels in link budget, one panel in LLS, 16 for each panel: (M, N, P) = (4,2,2) |

During the email discussion, some proposals are left for this meeting. [2].

Proposal 13:

Identify the target performance and coverage bottlenecks based on target performance metric for FR2.

- FFS: the target performance metric and potential down selection.

- Option 1: The target path loss or relative MPL is considered as the target performance.

  - Alt1: The target path loss is derived from the target ISD.

  - Alt2: Relative MPL.

- Option 2: The target MCL An MCL or MCL based metric is considered as the target performance.

  - Alt1: Derived from the target ISD.

  - Alt2: Fixed target MCL, e.g. 147dB for VoIP [to achieve better performance than other RAT(s)].

  - Alt3: Relative MCL

- If optional SLS is performed, the target performance for SLS is determined by the 5th percentile SINR value in CDF curve for different physical channels

- Other target performance metrics are not precluded.

Agreements:

- The link budget template for FR2 is the same as FR1.

In this contribution, we provide our views for the remaining issues for the evaluation and preliminary evaluation results for FR2.

Discussion

- 2.1 UE antenna configuration

During the email discussion, two panel for FR2 UE was proposed to use in the link budget. Current link budget takes the antenna array gain into the calculation. The different antenna element number could impact the link budget. For the multiple panels, it is not clear to capture the impact of different panels into the link budget. The multiple panels could accumulate the antenna gains if all the panel facing to the base station, which is similar to count the total antenna array gains. If only one panel could connect to the base station, this is same to the current configuration. Besides that, the specification of multiple panel is still unclear. There is no strong need to take the multiple panel assumption into account in the link budget.

Proposal 1:

There is no strong need to take the multiple panel assumption into account in the link budget.

- 2.2 The target performance metric

According to the discussion in the last meeting, multiple concepts, such as maximum coupling loss (MCL), maximum pathloss (MPL) and Maximum isotropic loss (MIL), had been raised, which include various components for calculating the loss during the propagation. Since the definitions of those concepts are not fully aligned among the companies, the discussion could be difficult. Then we propose that,

Proposal 2:

Further clarify and align the definition of MCL, MPL and MIL to facilitate the discussion.

Our understanding of the MCL is same as TR 36.824, which is

- UL MCL = UL Max Tx power - eNB Sensitivity

- DL MCL = DL Max Tx power - UE Sensitivity

The antenna array gain is included in the MCL implicitly. But it is not easy to observe the impact of antenna array from the MCL, since the MCL also contains the pathloss, shadow fading and some other components. The MCL could facilitate the relative comparison. It cannot provide any direct observation or guidance to the deployment, such as tolerable pathloss or coverage distance. Thus, among the down selection of the performance metric, the maximum pathloss (MPL) is preferred. To achieve this, multiple configurations should be assumed or identified, such as the assumptions of antenna array, shadow fading, penetration loss and other enhancement technology gains. But it is more clear and easy to compare with the real deployment parameters.

Also it is the same reason we propose to use the IMT-2020 template. According to this template, we could also derive the MCL, which may fulfil some companies’ requirements.

Proposal 3:

For the target performance metric and coverage bottlenecks, we prefer to use the target pathloss or relative MPL as the target performance (the option 1).

To solve some companies’ concern and facilitate that they can use the results as a reference to the deployments, the definition of MCL could be updated with the explicit inclusion of antenna array gains. To avoid the collision of definition, the updated MCL definition should be captured in the TR. The updated MCL calculation could be as below,

- UL MCL = UL Max Tx power - eNB Sensitivity+ Transmitter array gain + Receiver array gain

- DL MCL = DL Max Tx power - UE Sensitivity+ Transmitter array gain + Receiver array gain

Proposal 4 :

Update the definition of MCL with consideration of antenna array gain explicitly. Capture the new definition of MCL in the TR.

- UL MCL = UL Max Tx power - eNB Sensitivity+ Transmitter array gain + Receiver array gain

- DL MCL = DL Max Tx power - UE Sensitivity+ Transmitter array gain + Receiver array gain

Preliminary coverage analysis for FR2

According to the simulation assumption defined in the last meeting, a preliminary coverage performances have been derived according the IMT-2020 template. The specific simulation assumptions could refer to the Annex I. And the template used for the link budget could refer to Annex II.

For the FR2 link budget, many parameter assumptions are still not stable or aligned, such as transmission ERIP of base station, noise figure, feeder loss, cable, connector, combiner, body loss and etc. And the calculation of shadow fading margin and penetration margin for FR2 should also be aligned. In our link budget analysis, the EIRP of base station is assumed as 65dBm (800MHz) for the outdoor micro cell. And the ERIP of indoor micro cell is assumed as 52dBm (800MHz).

Proposal 5:

The parameter assumptions of link budget of FR2 should be clarified and aligned, such as noise figure, feeder loss, cable, connector, combiner, body loss.

Figure 1 Coverage performance of Dense Urban O2I scenario in MPL

According to the current parameter assumption, the PUSCH with 5Mbps requirement is the most limited channel. And the 2nd limited channel is PUCCH format 3 carrying 22bit. Most downlink channel are not limited.

Considering the pahtloss function from 38.901, 200m and 100m ISD needs to compensate 123dB and 111.3dB respectively. For the 200m ISD, the PUSCH needs 14dB enhancement for 200m ISD and 2dB for 100m ISD.

Observation 1

PUSCH with 5Mbps is the most limited channel within the system.

Observation 2

According to different system centre frequency, the specific enhancement requirement will be different.

Conclusions

In this contribution, we provide our views for the remaining issues for the evaluation and preliminary evaluation results for FR2. The proposal and observations are as follows,

Proposal 1:

There is no strong need to take the multiple panel assumption into account in the link budget.

Proposal 2:

Further clarify and align the definition of MCL, MPL and MIL to facilitate the discussion.

Proposal 3:

For the target performance metric and coverage bottlenecks, we prefer to use the target pathloss or relative MPL as the target performance (the option 1).

Proposal 4 :

Update the definition of MCL with consideration of antenna array gain explicitly. Capture the new definition of MCL in the TR.

- UL MCL = UL Max Tx power - eNB Sensitivity+ Transmitter array gain + Receiver array gain

- DL MCL = DL Max Tx power - UE Sensitivity+ Transmitter array gain + Receiver array gain

Proposal 5:

The parameter assumptions of link budget of FR2 should be clarified and aligned, such as noise figure, feeder loss, cable, connector, combiner, body loss.

Observation 1

PUSCH with 5Mbps is the most limited channel within the system.

Observation 2

According to different system centre frequency, the specific enhancement requirement will be different.

References

- [1] #101-e RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #101-e e-Meeting, May 25th – June 5th, 2020

- [2] R1-2005192, [101-e-Post-NR-Cov-Enh] Email discussion/approval focusing on remaining evaluation assumptions, Moderator (China Telecom), RAN1 #101-e

Annex 1- LLS simulation assumptions

Annex 2- link budget calculation using the modified template of IMT-2020