3GPP TSG RAN WG1#102e	R1-2006555

e-Meeting, August 17th – August 28th, 2020

Source:	Sharp

Title:	Remaining issues and corrections on HARQ enhancement for NR-U

Agenda Item:	7.2.2.2.3

Document for:	Discussion and Decision

- Introduction

The following agreements were achieved at RAN1#98bis meeting [1].

Agreement: For signaling the number of scheduled PUSCHs and TDRA in one DCI scheduling multiple PUSCHs, the TDRA table is extended such that each row indicates multiple PUSCHs (continuous in time-domain) - Alt. 1: Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI. - FFS: Separate k2 - Alt. 2: - A separate SLIV for - the starting slot, - the ending slot - The slots in between have a SLIV of S=0, L=14 - A separate mapping type for - the starting slot, - the ending slot, - the slots in between (a single mapping type for all of them) - The total number of PUSCHs for the row. - FFS: Extension to non-contiguous PUSCHs Agreement: Scheduling multiple PUSCHs is not supported in UL fallback DCI

In this contribution, we provide discussions on remaining issues and corrections for Release 16 NR-U HARQ enhancement.

- Remaining issues on multi-PUSCH scheduling

As agreed previously, multiple PUSCH scheduling doesn’t apply to DCI format 0_0. On the other hand, it is a bit ambiguous which TDRA table the DCI format 0_1 apply when multiple PUSCH scheduling is configured to the UE.

In the last meeting in RAN2 [2], it is agreed that the configuration on the multi-PUSCH scheduling has a separate IE pusch-TimeDomainAllocationListForMultiPUSCH-r16. That’s why the clarification is necessary in TS38.214 to include this.

Thus, the following TP should be adopted to TS38.214 and TS38.212.

Proposal 1: TP#1 for TS38.214 and TP#2 for TS38.212 should be adopted.

Text proposal#1 --------- beginning of text proposal for TS 38.214 ##### 6.1.2.1.1 Determination of the resource allocation table to be used for PUSCH -------- Unchanged contents are omitted Table 6.1.2.1.1-1A: Applicable PUSCH time domain resource allocation for DCI format 0_1 in UE specific search space scrambled with C-RNTI, MCS-C-RNTI, CS-RNTI or SP-CSI-RNTI pusch-ConfigCommon includes pusch-TimeDomainAllocationList; pusch-Config includes pusch-TimeDomainAllocationList; pusch-Config includes pusch-TimeDomainAllocationList -ForDCIformat0_1 or pusch-TimeDomainAllocationListForMultiPUSCH-r16; PUSCH time domain resource allocation to apply. No; No; No; Default A. Yes; No; No; pusch-TimeDomainAllocationList provided in pusch-ConfigCommon. No/Yes; Yes; No; pusch-TimeDomainAllocationList provided in pusch-Config. No/Yes; No/Yes; Yes; pusch-TimeDomainAllocationList-ForDCIformat0_1 or pusch-TimeDomainAllocationListForMultiPUSCH-r16 provided in pusch-Config. -------- Unchanged contents are omitted --------- end of text proposal

Text proposal#2 --------- beginning of text proposal for TS 38.212 ##### 7.3.1.1.2 Format 0_1 -------- Unchanged contents are omitted - Time domain resource assignment – 0, 1, 2, 3, 4, 5, or 6 bits - If the higher layer parameter neither of PUSCH-TimeDomainResourceAllocationList-ForDCIformat0_1 nor pusch-TimeDomainAllocationListForMultiPUSCH-r16 is configured and if the higher layer parameter pusch-TimeDomainAllocationList is configured, 0, 1, 2, 3, or 4 bits as defined in Clause 6.1.2.1 of [6, TS38.214]. The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter pusch-TimeDomainAllocationList; - If the higher layer parameter either of PUSCH-TimeDomainResourceAllocationList-ForDCIformat0_1 or pusch-TimeDomainAllocationListForMultiPUSCH-r16 is configured, 0, 1, 2, 3, 4, 5 or 6 bits as defined in Clause 6.1.2.1 of [6, TS38.214]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter PUSCH-TimeDomainResourceAllocationList-ForDCIformat0_1 or pusch-TimeDomainAllocationListForMultiPUSCH-r16; - otherwise the bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the default table. -------- Unchanged contents are omitted --------- end of text proposal

- Text proposals on enhanced dynamic HARQ-ACK codebook

In this section, the following TPs are provided:

##### TS38.212 ##### 7.3.1.2.2 Format 1_1 - Downlink assignment index – number of bits as defined in the following - 6 bits if more than one serving cell are configured in the DL and the higher layer parameter NFI-TotalDAI-Included-r16 = enable. The 4 MSB bits are the counter DAI and the total DAI for the scheduled PDSCH group, and the 2 LSB bits are the total DAI for the non-scheduled PDSCH group. - 4 bits if only one serving cell are configured in the DL and the higher layer parameter NFI-TotalDAI-Included-r16 = enable. The 2 MSB bits are the counter DAI for the scheduled PDSCH group, and the 2 LSB bits are the total DAI for the non-scheduled PDSCH group; - 4 bits if more than one serving cell are configured in the DL, the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic or pdsch-HARQ-ACK-Codebook-r16 = enhancedDynamic, and NFI-TotalDAI-Included-r16 is not configured, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI; - 4 bits if one serving cell is configured in the DL, and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, and the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, and is provided ACKNACKFeedbackMode = JointFeedback, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI; - 2 bits if only one serving cell is configured in the DL, the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic or pdsch-HARQ-ACK-Codebook-r16 = enhancedDynamic, and NFI-TotalDAI-Included-r16 is not configured, when the UE is not configured with CORESETPoolIndex or the value of CORESETPoolIndex is the same for all CORESETs if CORESETPoolIndex is provided or the UE is not configured with ACKNACKFeedbackMode = JointFeedback, where the 2 bits are the counter DAI; - 0 bits otherwise. - PDSCH group index – 0 or 1 bit. - 1 bit if the higher layer parameter pdsch-HARQ-ACK-Codebook-r16 = enhancedDynamic; - 0 bit otherwise. - New feedback indicator – 0, 1 or 2 bits. - 1 bit if the higher layer parameter pdsch-HARQ-ACK-Codebook-r16 = enhancedDynamic and the higher layer parameter NFI-TotalDAI-Included-r16 is not configured; - 2 bits if the higher layer parameter pdsch-HARQ-ACK-Codebook-r16 = enhancedDynamic and the higher layer parameter NFI-TotalDAI-Included-r16 = enable; the MSB corresponds to the scheduled PDSCH group, and the LSB corresponds to the non-scheduled PDSCH group, as defined in [TS38.213] clause 9.1.3.3 - 0 bit otherwise. - Number of requested PDSCH group(s) – 0 or 1 bit. - 1 bit if the higher layer parameter pdsch-HARQ-ACK-Codebook-r16 = enhancedDynamic; - 0 bit otherwise. …

## TS 38.213 ## 9.1 HARQ-ACK codebook determination … If a UE detects a DCI format 1_1 indicating - SCell dormancy without scheduling a PDSCH reception, as described in Clause 10.3, and - is provided pdsch-HARQ-ACK-Codebook = dynamic or pdsch-HARQ-ACK-Codebook-r16 = enhancedDynamic the UE generates a HARQ-ACK information bit as described in Clause 9.1.3 for a DCI format 1_1 indicating SCell dormancy and the HARQ-ACK information bit value is ACK. ### … ### 9.1.3 Type-2 HARQ-ACK codebook determination This clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic or with pdsch-HARQ-ACK-Codebook-r16 = enhancedDynamic. Unless stated otherwise, a PDSCH-to-HARQ_feedback timing indicator field provides an applicable value. A UE does not expect to multiplex in a Type-2 HARQ-ACK codebook HARQ-ACK information that is in response to a detection of a DCI format that does not include a counter DAI field. If a UE receives a first DCI format that the UE detects in a first PDCCH monitoring occasion and includes a PDSCH-to-HARQ_feedback timing indicator field providing an inapplicable value from dl-DataToUL-ACK, - if the UE detects a second DCI format, the UE multiplexes the corresponding HARQ-ACK information in a PUCCH or PUSCH transmission in a slot that is indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in the second DCI format, where - if the UE is not provided pdsch-HARQ-ACK-Codebook-r16 = enhancedDynamic, the UE detects the second DCI format in any PDCCH monitoring occasion after the first one - if the UE is provided pdsch-HARQ-ACK-Codebook-r16 = enhancedDynamic, the UE detects the second DCI format in any PDCCH monitoring occasion after the first one, and the second DCI format indicates a HARQ-ACK information report for a same PDSCH group index as indicated by the first DCI format as described in Clause 9.1.3.3 - if the UE is provided pdsch-HARQ-ACK-OneShotFeedback-r16, the first DCI format does not indicate SPS PDSCH release or SCell dormancy, the UE detects the second DCI format in any PDCCH monitoring occasion after the first one, and the second DCI format includes a One-shot HARQ-ACK request field with value 1, the UE includes the HARQ-ACK information in a Type-3 HARQ-ACK codebook, as described in Clause 9.1.4. - otherwise, the UE does not multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH transmission. … #### 9.1.3.3 Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission If a UE is provided pdsch-HARQ-ACK-Codebook-r16 = enhancedDynamic, the UE determines HARQ-ACK information for multiplexing in a PUCCH transmission occasion according to the following procedure. …

Proposal 2: Adopt the TPs related to the configuration on enhanced dynamic codebook type in TS38.213 to align with TS38.331 specification.

- Conclusion

In this contribution, we provide the following proposals:

Proposal 1: TP#1 for TS38.214 and TP#2 for TS38.212 should be adopted.

Proposal 2: Adopt the TPs related to the configuration on enhanced dynamic codebook type in TS38.213 to align with TS38.331 specification.

- References

- Chairman’s note at RAN1#100bis e-Meeting, April 2020

- Chairman’s note at RAN2#110 e-Meeting, May 2020