3GPP TSG-RAN WG1 #102-e		R1-2006670

eMeeting August 17th – 28th, 2020

Source: 	Ericsson

Title:      	Work plan for Rel17 WI on NR Dynamic spectrum sharing (DSS)

Agenda item:	8.13

Document for:	Discussion and Decision

# 1 Introduction

In [1], a new Rel17 WI on NR Dynamic spectrum sharing (DSS) was approved with below objectives

This work item is limited to FR1, and includes the following objectives for NR Dynamic Spectrum Sharing (DSS): - PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2] - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI - The number of cells can be scheduled at once is limited to 2 - The increase in DCI size should be minimized - Note: The total PDCCH blind decoding budget should not be changed as a result of this work - Note: These enhancements are not specific to DSS and are generally applicable to cross-carrier scheduling in carrier aggregation

This contribution provides high level RAN1 work plan for the WI taking into account the TU allocation agreed in [1], current RAN1 meeting calendar, and Rel17 timeline based on [2].

# 2. RAN1 Work Plan

| RAN1 meeting (TU allocation) | Work plan |
| --- | --- |
| RAN1#102-e August 2020 (0 TUs) | PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell - Initial exchange of views on high level framework Study on PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI - Initial exchange of views on study for whether or not to support the feature |
| RAN1#103-e, October 2020 (1 TU) | PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell - Discuss and agree on high-level framework (including aspects impacting Stage 2 spec) Study on PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI - Discuss (including any evaluations) whether or not to support the feature |
| RAN1#104 March 2021 (1 TU) | PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell - Finalize any remaining aspects on high-level framework (including aspects impacting Stage 2 spec) - Initial discussion on Stage 3 details Study on PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI - Finalize the study and reach agreement on whether or not to support the feature |
| RAN1#104bis April 2021 (1TU) | PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell - Continue discussion on Stage 3 details Study on PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI - To be updated later based on outcome of the study |
| RAN1#105 May 2021 (1TU) | PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell - Agree on Stage 3 details Study on PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI - To be updated later based on outcome of the study |
| From Q2 2021 until RAN1 freeze for Rel17 (2 TUs) Note: Exact timeline and meeting schedule TBD for now [2] | PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell - Finalize any remaining Stage 3 details - Finalize and agree to the corresponding CRs Study on PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI - To be updated later based on outcome of the study |

# 3. References

- RP-193260 - “New WID on NR Dynamic spectrum sharing (DSS)”, Ericsson, RAN#86, Dec 2019, Sitges, Spain.

- RP-201361 – “TU management for Q3’2020”, TSG RAN, RAN WG1, RAN WG2, RAN WG3, RAN WG4 Chairmen, RAN#88-e, June 2020, eMeeting.

Note : The TU allocation is also used for RAN1 impact from RP-201040 (LTE_NR_DC_enh2)