3GPP TSG RAN WG1 Meeting #102-e	                                                                          R1-2006767

August 17th – 28th, 2020

Agenda item:    7.2.2.2.5

Source:             Qualcomm Incorporated

Title:                  TP for wideband operation for NR-U

Document for:  Discussion and Decision

# Introduction

This paper lists TPs for a few issues identified for agenda item “wideband operation for NR-U”.

# Available RB set bitmap in COT-SI

When the active BWP contains multiple LBT bandwidth, for indication of available LBT bandwidth, a bitmap can be configured in the DCI 2_0 with each bit corresponds to one LBT bandwidth. There are a few issues we identified

- For the COT-SI transmission at the beginning of a COT, since the COT-SI encoding needs processing time, the DCI format 2_0 cannot carry valid available RB set information. A default value is needed to indicate the available RB set information is not yet available. We propose to use all 0 bitmap to indicate that.

- Also need to consider the case that the bitmap is not configured, in which case, the only UE assumption makes sense is assume all RB sets available when the DCI 2_0 is detected.

============TP for 38.213 Section 11.1.1====================================

--Unchanged part omitted------------------------

For each serving cell in the set of serving cells, the UE can be provided:

-	an identity of the serving cell by servingCellId

-	a location of a SFI-index field in DCI format 2_0 by positionInDCI

-	a set of slot format combinations by slotFormatCombinations, where each slot format combination in the set of slot format combinations includes

-	one or more slot formats indicated by a respective slotFormats for the slot format combination, and

-	a mapping for the slot format combination provided by slotFormats to a corresponding SFI-index field value in DCI format 2_0 provided by slotFormatCombinationId

-	for unpaired spectrum operation, a reference SCS configuration  by subcarrierSpacing and, when a supplementary UL carrier is configured for the serving cell, a reference SCS configuration  by subcarrierSpacing2 for the supplementary UL carrier

-	for paired spectrum operation, a reference SCS configuration  for a DL BWP by subcarrierSpacing and a reference SCS configuration  for an UL BWP by subcarrierSpacing2

-	a location of an available RB set indicator field in DCI format 2_0 that is

-  	one bit, if intraCellGuardBandDL-r16 for the serving cell indicates no intra-cell guard-bands are configured, where a value of '1' indicates that the serving cell is available for receptions, a value of '0' indicates that the serving cell is not available for receptions, by availableRB-SetPerCell-r16, and the serving cell remains available or unavailable for reception until the end of the indicated channel occupancy duration

-  	a bitmap having a one-to-one mapping with the RB sets [6, TS 38.214] of the serving cell, if intraCellGuardBandDL-r16 for the serving cell indicates intra-cell guard-bands are configured, where the bitmap includes NRB,set,DL bits and NRB,set,DL is the number of RB sets in the serving cell, a value of '1' indicates that an RB set is available for receptions, a value of '0' indicates that an RB set is not available for receptions, by availableRB-SetPerCell-r16, and an RB set remains available or unavailable for receptions until the end of the indicated channel occupancy duration. When all bits in the bitmap are ‘0’, in the availableRB-SetPerCell-r16, the availability for all RB sets for reception are considered as unknown, till another DCI format 2_0 is received.

-	When availableRB-SetPerCell-r16 is not configured for a serving cell configured with CO-DurationPerCell-r16 or slotFormatCombinations, the UE considered all RB sets available for reception when the DCI format 2_0 is detected

-	a location of a channel occupancy duration field in DCI format 2_0, by CO-DurationPerCell-r16, that indicates a remaining channel occupancy duration for the serving cell starting from a first symbol of a slot where the UE detects the DCI format 2_0 by providing a value from CO-DurationList-r16. The channel occupancy duration field includes maxlog2COdurationListSize,1 bits, where COdurationListSize is the number of values provided by CO-DurationList-r16. If CO-DurationPerCell-r16 is not provided, the remaining channel occupancy duration for the serving cell is a number of slots, starting from the slot where the UE detects the DCI format 2_0, that the SFI-index field value provides corresponding slot formats

-	a location of a search space set group switching field in DCI format 2_0, by SearchSpaceSwitchTrigger-r16, that indicates a group from two groups of search space sets for PDCCH monitoring for scheduling on the serving cell as described in Clause 10.4.

--Unchanged part omitted------------------------

===============================================================