3GPP TSG-RAN WG1 Meeting #102e	R1-2007104

e-meeting August-17th -28th 2020

Agenda Item:	7.2.8

Source:	Moderator (Ericsson)

Title:	Feature lead summary for maintenance of UL SRS and L1 procedures for NR positioning

Document for:	Discussion

# 1	Introduction

This document is based on the feature lead summary for NR positioning maintenance AI 7.2.8[1], and tracks the progress of the discussion for aspects 14-18 and 22 as stated in the chair notes:

[102-e-NR-Pos-02] Email discussion/approval on UL SRS and L1 procedures focusing on aspects 14-18 and 22 in the FL summary until 8/21; if necessary, endorse remaining TPs by 8/27 – Florent (Ericsson)

The following aspects are treated:

- Aspect #14: SRS Configuration

- Aspect #15: AP- SRS Support

- Aspect #16: MAC CE for SP/AP SRS Spatial Relation Indication

- Aspect #17: UE Sounding Procedure - Alignment of Parameter Names

- Aspect #18: Prioritization for Transmission Power Reduction

- Aspect #22: Priority of SRS for Positioning

# 2 	Discussion

## 2.1 	Aspect #14: SRS Configuration

### 2.1.1 summary and proposal

In [[9] Huawei], it is proposed to add BWP and carrier information to NRPPa. It is also proposed to reflect potential agreements in an LS to RAN3.

Proposal 2: Include the carrier information and active BWP information in the SRS configuration. - Carrier information includes one or more UL carriers, each containing - PCI - SFN Initialization time - PointA position - Usable RBs for each subcarrier spacing (Resource grid) - Active BWP information, containing - BWP location and bandwidth - Subcarrier spacing - CP type - Tx DC location - 7.5kHz shift

- Feature lead proposal 1: the following information is added to the SRS configuration IEs sent by the gNB to the LMF:

- Carrier information includes one or more UL carriers, each containing

- - PCI

- - SFN Initialization time

- - PointA position

- - Usable RBs for each subcarrier spacing (Resource grid)

- Active BWP information, containing

- - BWP location and bandwidth

- - Subcarrier spacing

- - CP type

- - Tx DC location

- - 7.5kHz shift

Companies are encouraged to provide their feedback on the feature lead proposal in the comment section below.

| Company | Comment |
| --- | --- |
| Nokia/NSB | Shouldn’t the NRPPa information follow both RRC spec and the higher layer parameter LS that RAN1 sent to both RAN2/RAN3? Or is there additional information in the proposal that RAN3 needs to be informed of? |
| Huawei/HiSilicon | RRC parameters only concern parameters under SRS-Config. The point is SRS-Config is indicated the following ASN.1 structure. ServingCellConfig (in RRCReconfiguration) UplinkConfig (uplinkConfig/supplementaryUplink) BWP-Uplink (uplinkBWP-ToAddModList) BWP-UplinkDedicated (bwp-Dedicated) SRS-Config (srs-Config) There is additional information conveyed by RRCReconfigurationComplete (from UE to gNB) on UL DC indication. The information is hard for RAN3 to extract from the whole RRC spec, which has strong correlation with RAN1. |
| CATT | If all the information mentioned in FL proposal 1 had been included in RRC spec, it is no need to repeat them in NRPPa spec. |
| Huawei/HiSilicon2 | We would like to remind the group that the only WI that has exception in RAN3 is NR positioning, and we should provide assistance in our best effort to help RAN3 finish their work. Clearly we do not need to transfer the entire RRCReconfiguration message in NRPPa, and what information is needed for receiving SRS is RAN1 expertise, e.g. how the first RB of SRS is counted, whether Tx DC location is needed, whether 7.5kHz shift is needed. |
| Qualcomm | We think that Ran3 will benefit from RAN1 input. Similar view with HW, that currently, there is no Tx DC location nor 7.5 KHz shift in the latest NRPPa draft. These 2 would need to be included. Looking at the latest draft, it seems the PointA, offset-To-carrier, BWP-offset, SRS config are added, along with all the remaining parameters. So, at least informing them that Tx DC location and 7.5 Khz shift is needed also, would be beneficial. To HW/HiSi: The remaining information of the above proposal seems already included in the latest NRPPa draft right? |
| Huawei/HiSilicon | To QC: Yes they are; however, we suggest to adopt the fields that are used in RRC to facilitate gNB internal processing and better understand the field without new explanation, which induces the following change - offset-To-carrier Usable RBs for each subcarrier spacing (Resource grid) - BWP-offset BWP location and bandwidth |
| Ericsson | This issue is better suited for a RAN3 discussion. |

### 2.1.2 status at the discussion deadline

Based on the current feedback, there are two companies proposing an agreement to add Tx DC location and 7.5 Khz shift. There is also a proposal to ask RAN3 to modify the BWP offset and offset to carrier fields to be in accordance with their RRC counterparts. However, other companies are saying that RAN3 should have the relevant information already.

Proposal for offline consensus: no agreement needed in RAN1.

Companies are encouraged to provide their feedback on the Proposal for offline consensus in the comment section below.

| Company | Comment |
| --- | --- |
| Huawei/HiSilicon | We have strong concern to make no agreement and leave it entirely to RAN3. Some parameters are strongly correlated with RAN1/physical layer, yet RAN1 did not include the parameters in the list. Some essential parameters may even be neglected by RAN1. |
| Qualcomm | We are also worried that RAN3 would not understand why some of these parameters are needed. Based on current draft, it is obvious that this is the case. We ask companies to reconsider. |
| CATT | Support FL proposal. We also think this issue is better suited for RAN3 discussion. |
| Huawei/HiSilicon2 | Our concern is not properly handled, and according to our knowledge, RAN3 has not resolved this issue. How about the following proposal? - Proposal: - The following information is added to the SRS configuration IEs sent by the gNB to the LMF - - Tx DC location - - 7.5kHz shift - - Note: it is up to RAN3 to decide whether other parameters in NRPPa should be aligned with RRC. - - Send an LS to RAN3 reflecting the agreement |
| Ericsson | Based on our discussion with RAN3 colleagues, it seem the issue can be resolved in RAN3. The parameters seem to already be included in the CR discussion in RAN3, so we propose to wait for RAN3 to complete their work. |

## 2.2 	Aspect #15: AP- SRS Support

### 2.2.1 summary and proposals

In [[9] Huawei], several issues pertaining to aperiodic SRS support are discussed:

Issue with triggering of aperiodic SRS, based on a DCI codepoint sent by the LMF:

- whether the LMF can “order” the gnodeB to send the SRS triggered by the code point in the LMF message.

- whether the gnodeB should trigger all SRS (including SRS mimo and SRS for positioning) associated with the DCI code point

Issue with the understanding of the transmission instant for neighbouring gnodeBs:

- The serving gnodeB should send additional information regarding the delay between the DCI and the actual SRS transmission (slot offset) to the LMF for forwarding to measurement neighboring nodes.

- The offset between DCI and triggered SRS transmission (slot offset) may not be useful for LMF or measurement neighbouring nodes as they do not detect DCI.

- The serving gnodeB should send additional information regarding the actual SRS transmission timing to the LMF for forwarding to measurement neighboring nodes.

The following is proposed:

Proposal 4: Send a reply LS to RAN3 on the problem of providing the following two parameters in NRPPa, and recommend to adopt the SFN and slot number of the AP-SRS to be transmitted instead. - aperiodicSRS-ResourceTriggerList - slotOffset

- Feature lead proposal 2: When using aperiodic SRS in positioning methods:

- - The LMF is informed of the aperiodic SRS configuration by:

- Option 1: the gNodeB transmits a list of code points the LMF can choose from, and the LMF can, at the moment of transmission, request any of the code points to trigger SRS.

- Option 2: the LMF request aperiodic transmission (which may include a proposal for an appropriate code point) and the gNodeB responds by providing the code point used to the LMF. The code point may differ from the LMF recommendation

- - the gNodeB should include slotOffset (including slot number and SFN) as part of the aperiodic SRS information sent to the LMF.

- - Send a reply LS to RAN3 reflecting the agreement

Companies are encouraged to provide their feedback on the feature lead proposal in the comment section below.

| Company | Comment |
| --- | --- |
| Nokia/NSB | Our understanding is that Option 2 is what was intended to be supported and that RAN3 was not deciding that LMF would have radio resource control. Positioning Activation Request should only be considered a recommendation. |
| Huawei/HiSilicon | We suggest to break the discussion into the following two orthogonal issues. Issue #1: Actual SRS transmission timing for TRP measurement. (Example: DCI will be sent in slot#2 of SFN#2, slot offset for AP-SRS triggering is 4 slots, which means that SRS will be transmitted in slot#6 of SFN#2). - Option 1: Based on existing signaling, i.e. slot offset with respect to the triggering DCI. In the example, TRP will be aware of the slot offset 4. - Option 2: Specific slot number and SFN when SRS will be triggered. In the example, TRP will be aware of the slot/SFN number slot 6 of SFN#2. Issue #2: SRS triggering codepoint information between LMF and the serving gNB - Option 1: the gNodeB transmits a list of code points the LMF can choose from, and the LMF can, at the moment of transmission, request any of the code points to trigger SRS. - Option 2: the LMF request aperiodic transmission (which may include a proposal for an appropriate code point) and the gNodeB responds by providing the code point used to the LMF. The code point may differ from the LMF recommendation |
| ZTE | We support both Option 2 from Huawei’s proposals. |
| CATT | First of all, we want to clarify the SRS mentioned in FL’s proposal or above Huawei’s two options refer to only SRS-Pos or both SRS-Pos and SRS-MIMO? |
| LG | We are fine with the FL’s proposal and we think that decision of option 1 or 2 can be up to RAN3. In our understanding, this discussion is only for SRS for positioning. |
| Huawei/HiSilicon2 | In reply to CATT: Our understanding is that it depends on the functionality supported in NRPPa. If NRPPa supports both AP-SRS-MIMO and AP-SRS-Pos, then the two issues should be applicable to both SRS. From our side, the general information on how SRS timing is transferred to the non-serving cell, and on how SRS triggering codepoints are exchanged between LMF and the serving cell should be provided by RAN1. |
| CATT | If the SRS mentioned in FL’s proposal or above Huawei’s two options maybe refer to AP-SRS-MIMO (depends on the functionality supported in NRPPa), we suggest to add Issue #3: Issue #3: For the SRS mentioned in Issue#1 and Issue#2, it refer to: - Option 1: Only SRS-Pos; - Option 2: both SRS-Pos and SRS-MIMO (depends on the functionality supported in NRPPa); |

Based on the first round of feedback, it seems that more details need to be discussed on how the trigger is forwarded to the non-seving gnodeB. The following revised proposal is given:

- Feature lead proposal 2a:

- Issue #1 When using aperiodic SRS in positioning methods:

- - The LMF is informed of the aperiodic SRS configuration by:

- Option 1-1: the gNodeB transmits a list of code points the LMF can choose from, and the LMF can, at the moment of transmission, request any of the code points to trigger SRS.

- Option 1-2: the LMF request aperiodic transmission (which may include a proposal for an appropriate code point) and the gNodeB responds by providing the code point used to the LMF. The code point may differ from the LMF recommendation

- Issue #2: Actual SRS transmission timing for TRP measurement. To notify the non-serving gnodBs of the slot where the aperiodic SRS is transmitted

- - option 2-1: based on existing ignaling, i.e. slot offset with respect to the triggering DCI.  The non-serving node receives the slot/SFN number where DCI is transmitted, and is aware of the slot offset between DCI and SRS transmissions.

- - option 2-2 the ignaling to the non-serving gnodeB includes the specific slot number and SFN when SRS will be triggered

- - Send a reply LS to RAN3 reflecting the agreement

Companies are encouraged to provide their feedback on the feature lead proposal in the comment section below.

| Company | Comment |
| --- | --- |
| Qualcomm | Our understanding of the procedure: - Step 1: Serving gNB sends all codepoints to LMF. Then LMF picks one of those. - In other words we support, Opt. 1.1 - Step 2: Serving gNB sends to LMF the slotoffset part of the SRS config as it is already in in RRC (0..31). LMF sends the codepoint with absolute time (Activation time) of where SRS should be transmitted. - We dont see a proposal for that aspect above - Step 3: LMF sends absolute time (in seconds) to the Neigboring gNB of where SRS is transmitted. - In other words we support, Option 2.2; no need to send slotoffset with respect to the DCI to the neighboring cell. A generic/absolute time is enough. |
| Huawei/HiSilicon | We support Option 1-2 and Option 2-2. Reply to QC: In step 2, we do not think the following absolute time is feasible. - Step 2: Serving gNB sends to LMF the slotoffset part of the SRS config as it is already in in RRC (0..31). LMF sends the codepoint with absolute time (Activation time) of where SRS should be transmitted. - We dont see a proposal for that aspect above LMF cannot order gNB in which slot to send the DCI, simply because it is not in controlled by LMF (LMF does not know the slot format, or the search space configruation, or the DRX state, or the gNB resource assignment, and LMF is in 5GC instead of NG-RAN), and decision when to send the DCI should be up to gNB implemtantion. The so-called absolute time should only be understood as a rough time window recommendation, and gNB should respond an actual time when DCI will be sent or when SRS will be triggered, which will be later forwarded to the neighbouring gNB in the form of absolute time of SRS transmision. |
| CATT | - We would like to add Issue #3 based on Feature lead proposal 2a, as follows, Issue #3: For the AP-SRS mentioned in Issue#1 and Issue#2, it refer to: - Option 1: Only AP-SRS-Pos; - Option 2: both AP-SRS-Pos and AP-SRS-MIMO (depends on the functionality supported in NRPPa); From the 1st round discussion, at least one company(LG) prefer the AP-SRS mentioned in Issue#1 and Issue#2 only refer to SRS-Pos, but Huawei think it includes both AP-SRS-Pos and AP-SRS-MIMO, therefore, we had better to clarify what is the exact meaning of AP-SRS in the FL proposal 2a. In our point of view, maybe we can only include AP-SRS-Pos at current stage, not sure whether there are potential issues when it include both AP-SRS-Pos and AP-SRS-MIMO. For Issue #1, we prefer this issue should be discussed in RAN2. For Issue #2, we prefer Option 2-2, i.e., TRPs should know both slot offset and SFN. |
| Ericsson | - We support option 1-2 and 2-2. Regarding the issue raised by Qualcomm, our understanding ist that the gNodeB will send the SFN and slot offset tot he LMF for forwarding to neighbour nodes. Our view is that we can leave it to RAN3 to figure out the appropriate translation from the slot offset to activation time and RAN1 need not intervene. - We also wonder what is the value in having this agreement this late in the discussion. RAN3 seem to have all the information needed. - Regarding CATT’s question on the SRS resource the codepoint points to, we believe this is the prerogative of the gNodeB, and by implementation the gnodeB can decide what should be triggered. |

### 2.2.2 status at the discussion deadline

- Based on the comments, the following offline consensus is proposed:

- Proposal for offline consensus

- When using aperiodic SRS in positioning methods, LMF request aperiodic transmission (which may include a proposal for an appropriate code point) and the gNodeB responds by providing the code point used to the LMF. The code point may differ from the LMF recommendation.

- For actual SRS transmission timing for TRP measurement: To notify the non-serving gNodBs of the slot where the aperiodic SRS is transmitted, the signaling to the non-serving gNodeB includes the specific slot number and SFN when SRS will be triggered

- - Send a reply LS to RAN3 reflecting the agreement

Companies are encouraged to provide their feedback on the Proposal for offline consensus in the comment section below.

| Company | Comment |
| --- | --- |
| Huawei/HiSilicon | Support. |
| LG | We understand the issue and the potential solution in the proposal. Support in principle, and we also prefer to leave it to RAN3. |
| Qualcomm | Object. The LMF need to know all the codepoints. |
| CATT | For option 1-2 (the first paragraph in above consensus), we prefer this issue should be discussed in RAN2. Then, we would like only mention option 2-2, and not mention option 1-2(i.e., delete the first paragraph) in the LS to RAN3. |
| Huawei/HiSilicon | Based on our understanding, QC is not objecting the second part on AP SRS transmission timing correct? To LGE, according to our understanding, RAN3 is discussing this issue, but some companies are not convinced why slot timing should be there. It will facilitate them to converge and complete support of the AP-SRS feature in Rel-16. How about the following proposal? - Proposal: Actual SRS transmission timing for TRP measurement. - To notify the non-serving gNodBs of the slot where the aperiodic SRS is transmitted, the signaling to the non-serving gNodeB includes the specific slot number and SFN when SRS will be triggered - - Send a reply LS to RAN3 reflecting the agreement |
| Ericsson | We don’t necessarily disagree with the proposal, but we don’t see the need for sending an LS to RAN3 or to have the agreement in RAN1. Our RAN3 colleagues seem to have already agreed to have a SFN and the slot number in the NRPPa messages. |

## 2.3	Aspect #16: MAC CE for SP/AP SRS Spatial Relation Indication

### 2.3.1 summary and proposals

Rel.16 MIMO agenda has introduced a new MAC CE to update the spatial relation info of SP/AP SRS. But there is no conclusion on whether the above agreement can be applicable to positioning SRS. In addition, the MAC CE defined in clause 6.1.3.26 of TS 38.321 doesn’t support to update spatial relation info of SP/AP positioning SRS.  Two companies propose to correct the issues via two similar text proposals:

- in [4] the following TP  for clause 6.2.1 of the TS 38.214 specification propose to make the MAC CE update of the spatial relation only applicable to SRS resources belonging to an SRS-ResourceSet (i.e. a set configured for mimo)

TP 2.3A ========================unchanged parts omitted================================ when a UE receives an spatial relation update command, as described in clause 6.1.3.26 of [10, TS 38.321], for an SRS resource configured by SRS-ResourceSet, and when the HARQ-ACK corresponding to the PDSCH carrying the update command is transmitted in slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions on updating spatial relation for the SRS resource shall be applied for SRS transmission starting from the first slot that is after slot n+3Nslotsubframe,µ. The update command contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the updated SRS resource set. Each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the update command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the update command if present, same serving cell and bandwidth part as the SRS resource set otherwise. When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', the UE shall not expect to be configured with different spatial relations for SRS resources in the same SRS resource set. ==========================unchanged parts omitted==============================

- in [8] the following TP  for clause 6.2.1 of the TS 38.214 specification propose to make the MAC CE update of the spatial relation only applicable to SRS resources configured by SRS-Resource (i.e. a resource configured for mimo).

TP 2.3B ===================== Unchanged parts omitted ========================= For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource-r16 is set to 'aperiodic': ===================== Unchanged parts omitted ========================= - when a UE receives an spatial relation update command, as described in clause 6.1.3.26 of [10, TS 38.321], for an SRS resource configured with the higher layer parameter SRS-Resource, and when the HARQ-ACK corresponding to the PDSCH carrying the update command is transmitted in slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions on updating spatial relation for the SRS resource shall be applied for SRS transmission starting from the first slot that is after slot n+3Nslotsubframe,µ. The update command contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the updated SRS resource set. Each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the update command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the update command if present, same serving cell and bandwidth part as the SRS resource set otherwise. When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', the UE shall not expect to be configured with different spatial relations for SRS resources in the same SRS resource set. ===================== Unchanged parts omitted =========================

Companies are encouraged to provide their preference between TP2.3A, TP2.3B or other changes (if any)  in the comment section below.

| Company | Comment |
| --- | --- |
| vivo | Seems both TPs are addressing the same thing. We’re okay with the intention of either TP |
| OPPO | TP 2.3B is preferred since the wording is more aligned with the configuration of SRS. |
| Huawei/HiSilicon | Support either TP. |
| ZTE | Both TPs are OK. |
| CATT | Support TP 2.3B for better wording. |
| SS | Both are OK. |
| LG | Either TP 2.3A or TP2.3 B is fine |
| QC | Support either TP. |
| MTK | TP2.3B is slightly preferred |

### 2.3.2 status at the discussion deadline

Based on the current feedback, it seems that both TP could be agreed and that TP 2.3B has more explicit support. The following offline consensus is proposed:

Proposal for offline consensus: TP 2.3B is endorsed.

Specification: 	 TS 38.214

Clauses affected:	6.2.1

Reason for Change: 	clarification regarding the Spatial relation assumption in the activation command for semi persistent SRS for positioning.

Summary of Change: 	the correction clarifies that the spatial relation update command for SRS in MAC CE  only applies to SRS resources configured with SRS-Resource

TP 2.3B ===================== Unchanged parts omitted ========================= For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource-r16 is set to 'aperiodic': ===================== Unchanged parts omitted ========================= - when a UE receives an spatial relation update command, as described in clause 6.1.3.26 of [10, TS 38.321], for an SRS resource configured with the higher layer parameter SRS-Resource, and when the HARQ-ACK corresponding to the PDSCH carrying the update command is transmitted in slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions on updating spatial relation for the SRS resource shall be applied for SRS transmission starting from the first slot that is after slot n+3Nslotsubframe,µ. The update command contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the updated SRS resource set. Each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the update command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the update command if present, same serving cell and bandwidth part as the SRS resource set otherwise. When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', the UE shall not expect to be configured with different spatial relations for SRS resources in the same SRS resource set. ===================== Unchanged parts omitted =========================

Companies are encouraged to provide their feedback on the Proposal for offline consensus in the comment section below.

| Company | Comment |
| --- | --- |
| Huawei/HiSilicon | Support. |
| LG | Support |

## 2.4	Aspect #17: UE Sounding Procedure - Alignment of Parameter Names

### 2.4.1 summary and proposals

Two companies provided updates to the UE sounding procedure in 38.214 for the purpose of parameter alignment:

- in  [[5], CATT] the TP (reproduced below as TP 2.4A) aims to align parameter names with RAN2 WG. Another issue in the above descriptions in section 6.2.1 is the ambiguity on the applicability of the higher layer parameter cyclicShift-n8-r16 and combOffset-n8-r16. For the higher layer parameter cyclicShift-n8-r16, in fact, only two higher layer parameters cyclicShift-n2 and cyclicShift-n4 are applicable for the configuration of the cyclic shifts of SRS-MIMO. The higher layer parameter cyclicShift-n8-r16 is only applicable for SRS-Pos but not applicable for SRS-MIMO. However, the descriptions in the current section 6.2.1 in 38.214 mix these parameters together, which maybe cause some ambiguity on the applicability of the higher layer parameter cyclicShift-n8-r16. For the higher layer parameter combOffset-n8-r16, there is the similar issue.

# TP 2.4A # ------------------------------------------Start of Text Proposal for 38.214-------------------------------------------------- # 6.2.1 UE sounding procedure # -------------------------------------------- Unchanged part omitted -------------------------------------------------------- The following SRS parameters are semi-statically configurable by higher layer parameter SRS-Resource or SRS-PosResource-r16. - srs-ResourceId or SRS-PosResourceId-r16 determines SRS resource configuration identity. - Number of SRS ports as defined by the higher layer parameter nrofSRS-Ports and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, nrofSRS-Ports is 1. - Time domain behaviour of SRS resource configuration as indicated by the higher layer parameter resourceType or resourceType-r16, which may be periodic, semi-persistent, aperiodic SRS transmission as defined in Clause 6.4.1.4 of [4, TS 38.211]. - Slot level periodicity and slot level offset as defined by the higher layer parameters periodicityAndOffset-p or periodicityAndOffset-sp for an SRS resource of type periodic or semi-persistent, which configured by SRS-Resource, and periodicityAndOffset-p-r16 or periodicityAndOffset-sp-r16 for an SRS resource of type periodic or semi-persistent, which configured by SRS-PosResource-r16. The UE is not expected to be configured with SRS resources in the same SRS resource set SRS-ResourceSet or SRS-PosResourceSet-r16 with different slot level periodicities. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a slot level offset is defined by the higher layer parameter slotOffset. For an SRS-PosResourceSet-r16 configured with higher layer parameter resourceType-r16 set to 'aperiodic', the slot level offset is defined by the higher layer parameter slotOffset-r16 for each SRS resource. - Number of OFDM symbols in the SRS resource, starting OFDM symbol of the SRS resource within a slot including repetition factor R as defined by the higher layer parameter resourceMapping or resourceMapping-r16 and described in Clause 6.4.1.4 of [4, TS 38.211]. If R is not configured, then R is equal to the number of OFDM symbols in the SRS resource. - SRS bandwidth and , as defined by the higher layer parameter freqHopping or freqHopping-r16 and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then= 0. - Frequency hopping bandwidth, , as defined by the higher layer parameter freqHopping or freqHopping-r16 and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then = 0. - Defining frequency domain position and configurable shift, as defined by the higher layer parameters freqDomainPosition and freqDomainShift, or freqDomainPosition-r16 and freqDomainShift-r16 respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. If freqDomainPosition or freqDomainPosition-r16 is not configured, freqDomainPosition or freqDomainPosition-r16 is zero. - Cyclic shift, as defined by the higher layer parameter cyclicShift-n2 or cyclicShift-n4 for transmission comb value 2 or 4 for an SRS configured by SRS-Resource, respectively, and defined by the higher layer parameter cyclicShift-n2-r16, cyclicShift-n4-r16, or cyclicShift-n8-r16 for transmission comb value 2, 4 or 8 for an SRS configured by SRS-PosResource-r16, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. - Transmission comb value as defined by the higher layer parameter transmissionComb or transmissionComb-r16 described in Clause 6.4.1.4 of [4, TS 38.211]. - Transmission comb offset as defined by the higher layer parameter combOffset-n2 or combOffset-n4 for transmission comb value 2 or 4 for an SRS configured by SRS-Resource, respectively, and defined by the higher layer parameter combOffset-n2-r16, combOffset-n4-r16, or combOffset-n8-r16 for transmission comb value 2, 4, or 8 for an SRS configured by SRS-PosResource-r16, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. - SRS sequence ID as defined by the higher layer parameter sequenceId or sequenceId-r16 in Clause 6.4.1.4 of [4]. - The configuration of the spatial relation between a reference RS and the target SRS, where the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16, if configured, contains the ID of the reference RS. The reference RS may be an SS/PBCH block, CSI-RS configured on serving cell indicated by higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise, or an SRS configured on uplink BWP indicated by the higher layer parameter uplinkBWP or uplinkBWP-r16 , and serving cell indicated by the higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise. When an SRS is configured by the higher layer parameter SRS-PosResourceSet-r16, the reference RS may also be a DL PRS configured on a serving cell, an SS/PBCH block or a DL PRS of a non-serving cell indicated by a higher layer parameter. -------------------------------------------Unchanged part omitted --------------------------------------------------------- ---------------------------------------------End of Text Proposal ----------------------------------------------------------

- In [[9], OPPO], it is proposed to align parameter names for UE sounding procedure Section 6.2.1 of TS 38.214

  - The following TP below is proposed:

TP 2.4B ### 6.2.1 UE sounding procedure *** Unchanged text is omitted *** The following SRS parameters are semi-statically configurable by higher layer parameter SRS-Resource or SRS-PosResource-r16. - srs-ResourceId or srs-PosResourceId-r16 determines SRS resource configuration identity. - Number of SRS ports as defined by the higher layer parameter nrofSRS-Ports and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, nrofSRS-Ports is 1. - Time domain behaviour of SRS resource configuration as indicated by the higher layer parameter resourceType, which may be periodic, semi-persistent, aperiodic SRS transmission as defined in Clause 6.4.1.4 of [4, TS 38.211]. - Slot level periodicity and slot level offset as defined by the higher layer parameters periodicityAndOffset-p or periodicityAndOffset-sp for an SRS resource of type periodic or semi-persistent. The UE is not expected to be configured with SRS resources in the same SRS resource set SRS-ResourceSet or SRS-PosResourceSet-r16 with different slot level periodicities. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a slot level offset is defined by the higher layer parameter slotOffset. For an SRS-PosResourceSet-r16 with higher layer parameter resourceType-r16 set to 'aperiodic-r16', the slot level offset is defined by the higher layer parameter slotOffset-r16 for each SRS resource. - Number of OFDM symbols in the SRS resource, starting OFDM symbol of the SRS resource within a slot including repetition factor R as defined by the higher layer parameter resourceMapping and described in Clause 6.4.1.4 of [4, TS 38.211]. If R is not configured, then R is equal to the number of OFDM symbols in the SRS resource. - SRS bandwidth and , as defined by the higher layer parameter freqHopping and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then= 0. - Frequency hopping bandwidth, , as defined by the higher layer parameter freqHopping and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then = 0. - Defining frequency domain position and configurable shift, as defined by the higher layer parameters freqDomainPosition and freqDomainShift, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. If freqDomainPosition is not configured, freqDomainPosition is zero. - Cyclic shift, as defined by the higher layer parameter cyclicShift-n2, cyclicShift-n4, or cyclicShift-n8 for transmission comb value 2, 4 and 8, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. - Transmission comb value as defined by the higher layer parameter transmissionComb or transmissionComb-r16 described in Clause 6.4.1.4 of [4, TS 38.211]. - Transmission comb offset as defined by the higher layer parameter combOffset-n2, combOffset-n4, or combOffset-n8 for transmission comb value 2, 4, or 8 respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. - SRS sequence ID as defined by the higher layer parameter sequenceId or sequenceId -r16 in Clause 6.4.1.4 of [4]. - The configuration of the spatial relation between a reference RS and the target SRS, where the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16, if configured, contains the ID of the reference RS. The reference RS may be an SS/PBCH block, CSI-RS configured on serving cell indicated by higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise, or an SRS configured on uplink BWP indicated by the higher layer parameter uplinkBWP, and serving cell indicated by the higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise. When SRS is configured by the higher layer parameter SRS-PosResourceSet-r16 the reference RS may also be a DL PRS configured on a serving cell, an SS/PBCH block or a DL PRS of a non-serving cell indicated by a higher layer parameter. *** Unchanged text is omitted *** - if the UE is configured with the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16 containing the ID of a reference 'ssb-Index', 'ssb-IndexServing-r16', or 'ssb-IndexNcell-r16' the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16 contains the ID of a reference 'csi-RS-Index' or 'csi-RS-IndexServing-r16', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, if the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16 contains the ID of a reference 'srs' or 'srs-SpatialRelation-r16', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS or of the reference semi-persistent SRS. When the SRS is configured by the higher layer parameter SRS-PosResourceSet-r16 and if the higher layer parameter spatialRelationInfoPos-r16 contains the ID of a reference 'dl-PRS-ResourceId-r16', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference DL PRS. *** Unchanged text is omitted ***

Companies are encouraged to provide their preference between TP2.4A, TP2.4B or other changes (if any)  in the comment section below.

| Company | Comment |
| --- | --- |
| Nokia/NSB | Support TP2.4A. |
| vivo | support TP2.4A |
| OPPO | Support both A and B |
| Huawei/HiSilicon | From our side, when a field has both versions without suffix and with suffix –r16, and they serve the same functionality, we suggest to keep only the description for the fields without suffix to keep the spec clean, which by the way that is widely used in RAN2 spec. Therefore, we prefer TP2.4B without the changes w.r.t. transmissionComb and sequenceId. |
| ZTE | Both TP2.4A and TP2.4B. |
| CATT | Support TP2.4A and the 2nd part of TP2.4B. The 1st part of TP2.4B is not complete when modifying the contents of this part. For Huawei’s comments: We cannot agree to Huawei’s comments on with suffix and without suffix, the wording in the specs should be clear and unambiguous, so we prefer to add the suffix –r16 for SRS-Pos related parameters. |
| SS | Both are acceptable |
| LG | Support TP2.4A |
| Huawei/HiSilicon | To us, we do not see any ambiguity for those parameter name without suffix, which has been used in LTE and NR, e.g. TS 36.211, TS 38.211. For the sake of progress, we can agree with such a change for this meeting, however we strongly discourage this kind of change by enumerating the same field in different releases with the same meaning in the future meeting, otherwise they will come one meeing after another. For SRS, we have the following description in TS 38.211, and I guess no one considers them as ambiguous. ##### 6.4.1.4.2 Sequence generation The sounding reference signal sequence for an SRS resource shall be generated according to r(pi)n,l'=ru,v(αi,δ)n 0≤n≤Msc,bSRS-1 l'∈0,1,…,NsymbSRS-1 where Msc,bSRS is given by clause 6.4.1.4.3, ru,vα,δ(n) is given by clause 5.2.2 with δ=log2KTC and the transmission comb number KTC∈2,4,8 is contained in the higher-layer parameter transmissionComb. The cyclic shift for antenna port is given as , where is contained in the higher layer parameter transmissionComb. The maximum number of cyclic shifts nSRScs,max are given by Table 6.4.1.4.2-1. The sequence group u=fghns,fμ,l'+nIDSRS mod 30 and the sequence number in clause 5.2.2 depends on the higher-layer parameter groupOrSequenceHopping in the SRS-Resource IE or the SRS-PosResource-r16 IE. The SRS sequence identity is given by the higher layer parameter sequenceId in the SRS-Resource IE, in which case nIDSRS∈0, 1, …, 1023, or the SRS-PosResource-r16 IE, in which case nIDSRS∈0, 1, …, 65535. The quantity l'∈0,1,…,NsymbSRS-1 is the OFDM symbol number within the SRS resource. |
| Qualcomm | We dont agree doing spec changes that involved adding the same parameters with a different release number when the functionality is the same. So, either TP does not work for us. For the same functionality, we should not just repeat the field in RAN1 spec, unless it is really necessary because the functionality changes. So, we cannot accept either TP as they currently stand. |
| CATT | For Huawei’s comments, we remind companies should keep a precise attitude on the wording of specs. Every parameter names in the specs should be accurate and no ambiguity. However, current descirptions in the above sections mixed the names of parameters for SRS-Pos and SRS-MIMO. If we don’t correct such issues, there will be a misconception that SRS-Pos and SRS-MIMO use the same parameters, but in fact, they have different higher-layer parameter names and different candidate values, such as periodicityAndOffset-p and periodicityAndOffset-p-16 have the following different values. If we only mention periodicityAndOffset-p in the specs, the reader maybe only use SRS-PeriodicityAndOffset for both SRS-MIMO and SRS-Pos, but not use SRS-PeriodicityAndOffset-r16 for SRS-Pos. SRS-PeriodicityAndOffset ::= CHOICE { sl1 NULL, sl2 INTEGER(0..1), sl4 INTEGER(0..3), sl5 INTEGER(0..4), sl8 INTEGER(0..7), sl10 INTEGER(0..9), sl16 INTEGER(0..15), sl20 INTEGER(0..19), sl32 INTEGER(0..31), sl40 INTEGER(0..39), sl64 INTEGER(0..63), sl80 INTEGER(0..79), sl160 INTEGER(0..159), sl320 INTEGER(0..319), sl640 INTEGER(0..639), sl1280 INTEGER(0..1279), sl2560 INTEGER(0..2559) } SRS-PeriodicityAndOffset-r16 ::= CHOICE { sl1 NULL, sl2 INTEGER(0..1), sl4 INTEGER(0..3), sl5 INTEGER(0..4), sl8 INTEGER(0..7), sl10 INTEGER(0..9), sl16 INTEGER(0..15), sl20 INTEGER(0..19), sl32 INTEGER(0..31), sl40 INTEGER(0..39), sl64 INTEGER(0..63), sl80 INTEGER(0..79), sl160 INTEGER(0..159), sl320 INTEGER(0..319), sl640 INTEGER(0..639), sl1280 INTEGER(0..1279), sl2560 INTEGER(0..2559), sl5120 INTEGER(0..5119), sl10240 INTEGER(0..10239), sl40960 INTEGER(0..40959), sl81920 INTEGER(0..81919), ... } As mentioned in our tdoc, another issue is the ambiguity on the applicability of the higher layer parameter cyclicShift-n8-r16 and combOffset-n8-r16. For the higher layer parameter cyclicShift-n8-r16, in fact, only two higher layer parameters cyclicShift-n2 and cyclicShift-n4 are applicable for the configuration of the cyclic shifts of SRS-MIMO. The higher layer parameter cyclicShift-n8-r16 is only applicable for SRS-Pos but not applicable for SRS-MIMO. However, the descriptions in the current section 6.2.1 in 38.214 mix these parameters together, which maybe cause some ambiguity on the applicability of the higher layer parameter cyclicShift-n8-r16. For the higher layer parameter combOffset-n8-r16, there is the similar issue. In order to solve above issues, we prefer to adopt the TP2.4A. About Huawei’s example 6.4.1.4.2 in 38.211, we have to say the issues also exist in 38.211, and they had better to be fixed together. We also have one example on 38.211, which fixed the suffix issue in below: ##### 6.4.1.4.4 Sounding reference signal slot configuration For an SRS resource configured as periodic or semi-persistent by the higher-layer parameter resourceType, a periodicity (in slots) and slot offset are configured according to the higher-layer parameter periodicityAndOffset-p or periodicityAndOffset-sp in the SRS-Resource IE, or periodicityAndOffset-p-r16 or periodicityAndOffset-sp-r16 in the SRS-PosResource-r16 IE. Candidate slots in which the configured SRS resource may be used for SRS transmission are the slots satisfying SRS is transmitted as described in clause 11.1 of [5, TS 38.213]. For Qualcomm’s comments, we think the functionality and candidate values of these parameters for SRS-MIMO and SRS-Pos is not the same, the detailed explanation can be found in the above. |
| MTK | It is kind of messy to add suffix. But it seems necessary to be clear. We are fine with CATT |

### 2.4.2 status at the discussion deadline

The majority of comments expressed support for TP2.4A. however, this is also a number of company voicing support for either TP2.4B or no changes. We propose TP2.4A as the candidate for offline consensus, to see if compromise can be reached.

Proposal for offline consensus: TP 2.4A is endorsed.

Specification: 	 TS 38.214

Clauses affected:	6.2.1

Reason for Change: 	clarification of higher layer parameter names with respect to their configuration for SRS for positioning or SRS for MIMO.

Summary of Change: 	the correction clarifies which IE applies to SRS for positioning and which IE applies to SRS for MIMO.

# TP 2.4A # ------------------------------------------Start of Text Proposal for 38.214-------------------------------------------------- # 6.2.1 UE sounding procedure # -------------------------------------------- Unchanged part omitted -------------------------------------------------------- The following SRS parameters are semi-statically configurable by higher layer parameter SRS-Resource or SRS-PosResource-r16. - srs-ResourceId or SRS-PosResourceId-r16 determines SRS resource configuration identity. - Number of SRS ports as defined by the higher layer parameter nrofSRS-Ports and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, nrofSRS-Ports is 1. - Time domain behaviour of SRS resource configuration as indicated by the higher layer parameter resourceType or resourceType-r16, which may be periodic, semi-persistent, aperiodic SRS transmission as defined in Clause 6.4.1.4 of [4, TS 38.211]. - Slot level periodicity and slot level offset as defined by the higher layer parameters periodicityAndOffset-p or periodicityAndOffset-sp for an SRS resource of type periodic or semi-persistent, which configured by SRS-Resource, and periodicityAndOffset-p-r16 or periodicityAndOffset-sp-r16 for an SRS resource of type periodic or semi-persistent, which configured by SRS-PosResource-r16. The UE is not expected to be configured with SRS resources in the same SRS resource set SRS-ResourceSet or SRS-PosResourceSet-r16 with different slot level periodicities. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a slot level offset is defined by the higher layer parameter slotOffset. For an SRS-PosResourceSet-r16 configured with higher layer parameter resourceType-r16 set to 'aperiodic', the slot level offset is defined by the higher layer parameter slotOffset-r16 for each SRS resource. - Number of OFDM symbols in the SRS resource, starting OFDM symbol of the SRS resource within a slot including repetition factor R as defined by the higher layer parameter resourceMapping or resourceMapping-r16 and described in Clause 6.4.1.4 of [4, TS 38.211]. If R is not configured, then R is equal to the number of OFDM symbols in the SRS resource. - SRS bandwidth and , as defined by the higher layer parameter freqHopping or freqHopping-r16 and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then= 0. - Frequency hopping bandwidth, , as defined by the higher layer parameter freqHopping or freqHopping-r16 and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then = 0. - Defining frequency domain position and configurable shift, as defined by the higher layer parameters freqDomainPosition and freqDomainShift, or freqDomainPosition-r16 and freqDomainShift-r16 respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. If freqDomainPosition or freqDomainPosition-r16 is not configured, freqDomainPosition or freqDomainPosition-r16 is zero. - Cyclic shift, as defined by the higher layer parameter cyclicShift-n2 or cyclicShift-n4 for transmission comb value 2 or 4 for an SRS configured by SRS-Resource, respectively, and defined by the higher layer parameter cyclicShift-n2-r16, cyclicShift-n4-r16, or cyclicShift-n8-r16 for transmission comb value 2, 4 or 8 for an SRS configured by SRS-PosResource-r16, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. - Transmission comb value as defined by the higher layer parameter transmissionComb or transmissionComb-r16 described in Clause 6.4.1.4 of [4, TS 38.211]. - Transmission comb offset as defined by the higher layer parameter combOffset-n2 or combOffset-n4 for transmission comb value 2 or 4 for an SRS configured by SRS-Resource, respectively, and defined by the higher layer parameter combOffset-n2-r16, combOffset-n4-r16, or combOffset-n8-r16 for transmission comb value 2, 4, or 8 for an SRS configured by SRS-PosResource-r16, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. - SRS sequence ID as defined by the higher layer parameter sequenceId or sequenceId-r16 in Clause 6.4.1.4 of [4]. - The configuration of the spatial relation between a reference RS and the target SRS, where the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16, if configured, contains the ID of the reference RS. The reference RS may be an SS/PBCH block, CSI-RS configured on serving cell indicated by higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise, or an SRS configured on uplink BWP indicated by the higher layer parameter uplinkBWP or uplinkBWP-r16 , and serving cell indicated by the higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise. When an SRS is configured by the higher layer parameter SRS-PosResourceSet-r16, the reference RS may also be a DL PRS configured on a serving cell, an SS/PBCH block or a DL PRS of a non-serving cell indicated by a higher layer parameter. -------------------------------------------Unchanged part omitted --------------------------------------------------------- ---------------------------------------------End of Text Proposal ----------------------------------------------------------

Companies are encouraged to provide their feedback on the Proposal for offline consensus in the comment section below.

| Company | Comment |
| --- | --- |
| Huawei/HiSilicon | We have concern to agree with the TP. Besides the reason that we mentioned, additional reasons are - In summary of change, two instances of “IE” in “which IE applies to SRS for positioning and which IE applies to SRS for MIMO” should be changed “field”. - The parameter freqDomainPosition-r16 does not exist in ASN.1 - Two is’s are missing after “which” in the following clauses which configured by SRS-Resource, and periodicityAndOffset-p-r16 or periodicityAndOffset-sp-r16 for an SRS resource of type periodic or semi-persistent, which configured by SRS-PosResource-r16 - The description citing TS 38.211 is not aligned with TS 38.211, in which the same field names are not listed twice. We can accept this change for this time if the first three issue listed above are resolved, and we discourage any future change of this kind, and we would like to have the following note in the Chairman’s Notes Huawei/HiSilicon have concern on the TP that creates duplicated fields describing the same functionality. |
| LG | Support for a more clear description. |
| CATT | For Huawei’s comments, we remind companies should keep a precise attitude on the wording of specs. Every parameter names in the specs should be accurate and no ambiguity. However, current descirptions in the above sections mixed the names of parameters for SRS-Pos and SRS-MIMO. If we don’t correct such issues, there will be a misconception that SRS-Pos and SRS-MIMO use the same parameters. We strongly discourage to keep the issues in the specs and not try to fix them. In order to solve above issues, we prefer to adopt the updated TP2.4A to address Huawei’s comments: # TP 2.4A # ------------------------------------------Start of Text Proposal for 38.214-------------------------------------------------- # 6.2.1 UE sounding procedure # -------------------------------------------- Unchanged part omitted -------------------------------------------------------- The following SRS parameters are semi-statically configurable by higher layer parameter SRS-Resource or SRS-PosResource-r16. - srs-ResourceId or SRS-PosResourceId-r16 determines SRS resource configuration identity. - Number of SRS ports as defined by the higher layer parameter nrofSRS-Ports and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, nrofSRS-Ports is 1. - Time domain behaviour of SRS resource configuration as indicated by the higher layer parameter resourceType or resourceType-r16, which may be periodic, semi-persistent, aperiodic SRS transmission as defined in Clause 6.4.1.4 of [4, TS 38.211]. - Slot level periodicity and slot level offset as defined by the higher layer parameters periodicityAndOffset-p or periodicityAndOffset-sp for an SRS resource of type periodic or semi-persistent, which is configured by SRS-Resource, and periodicityAndOffset-p-r16 or periodicityAndOffset-sp-r16 for an SRS resource of type periodic or semi-persistent, which is configured by SRS-PosResource-r16. The UE is not expected to be configured with SRS resources in the same SRS resource set SRS-ResourceSet or SRS-PosResourceSet-r16 with different slot level periodicities. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a slot level offset is defined by the higher layer parameter slotOffset. For an SRS-PosResourceSet-r16 configured with higher layer parameter resourceType-r16 set to 'aperiodic', the slot level offset is defined by the higher layer parameter slotOffset-r16 for each SRS resource. - Number of OFDM symbols in the SRS resource, starting OFDM symbol of the SRS resource within a slot including repetition factor R as defined by the higher layer parameter resourceMapping or resourceMapping-r16 and described in Clause 6.4.1.4 of [4, TS 38.211]. If R is not configured, then R is equal to the number of OFDM symbols in the SRS resource. - SRS bandwidth and , as defined by the higher layer parameter freqHopping or freqHopping-r16 and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then= 0. - Frequency hopping bandwidth, , as defined by the higher layer parameter freqHopping or freqHopping-r16 and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then = 0. - Defining frequency domain position and configurable shift, as defined by the higher layer parameters freqDomainPosition and freqDomainShift or freqDomainShift-r16 respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. If freqDomainPosition is not configured, freqDomainPosition is zero. - Cyclic shift, as defined by the higher layer parameter cyclicShift-n2 or cyclicShift-n4 for transmission comb value 2 or 4 for an SRS configured by SRS-Resource, respectively, and defined by the higher layer parameter cyclicShift-n2-r16, cyclicShift-n4-r16, or cyclicShift-n8-r16 for transmission comb value 2, 4 or 8 for an SRS configured by SRS-PosResource-r16, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. - Transmission comb value as defined by the higher layer parameter transmissionComb or transmissionComb-r16 described in Clause 6.4.1.4 of [4, TS 38.211]. - Transmission comb offset as defined by the higher layer parameter combOffset-n2 or combOffset-n4 for transmission comb value 2 or 4 for an SRS configured by SRS-Resource, respectively, and defined by the higher layer parameter combOffset-n2-r16, combOffset-n4-r16, or combOffset-n8-r16 for transmission comb value 2, 4, or 8 for an SRS configured by SRS-PosResource-r16, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. - SRS sequence ID as defined by the higher layer parameter sequenceId or sequenceId-r16 in Clause 6.4.1.4 of [4]. - The configuration of the spatial relation between a reference RS and the target SRS, where the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16, if configured, contains the ID of the reference RS. The reference RS may be an SS/PBCH block, CSI-RS configured on serving cell indicated by higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise, or an SRS configured on uplink BWP indicated by the higher layer parameter uplinkBWP or uplinkBWP-r16 , and serving cell indicated by the higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise. When an SRS is configured by the higher layer parameter SRS-PosResourceSet-r16, the reference RS may also be a DL PRS configured on a serving cell, an SS/PBCH block or a DL PRS of a non-serving cell indicated by a higher layer parameter. -------------------------------------------Unchanged part omitted --------------------------------------------------------- ---------------------------------------------End of Text Proposal ----------------------------------------------------------. About Huawei’s comments on “the description citing TS 38.211 is not aligned with TS 38.211, in which the same field names are not listed twice.” We think the issues also exist in 38.211, and they had better to be fixed together. But we can fix the issues in TP2.4A in this week, and try to fix the issues in 38.211 next week. We also have one example on 38.211, which fixed the suffix issue in below: ##### 6.4.1.4.4 Sounding reference signal slot configuration For an SRS resource configured as periodic or semi-persistent by the higher-layer parameter resourceType, a periodicity (in slots) and slot offset are configured according to the higher-layer parameter periodicityAndOffset-p or periodicityAndOffset-sp in the SRS-Resource IE, or periodicityAndOffset-p-r16 or periodicityAndOffset-sp-r16 in the SRS-PosResource-r16 IE. Candidate slots in which the configured SRS resource may be used for SRS transmission are the slots satisfying SRS is transmitted as described in clause 11.1 of [5, TS 38.213]. For Qualcomm’s comments, we think the functionality and candidate values of these parameters for SRS-MIMO and SRS-Pos is not the same, the detailed explanation can be found in the above. |

### 2.4.2 update to proposal for offline consensus

The proposal from CATT for TP 2.4A seem to address the concern from Huawei. The following update is then proposed:

Proposal for offline consensus: TP 2.4Abis is endorsed.

Specification: 	 TS 38.214

Clauses affected:	6.2.1

Reason for Change: 	clarification of higher layer parameter names with respect to their configuration for SRS for positioning or SRS for MIMO.

Summary of Change: 	the correction clarifies which field applies to SRS for positioning and which field applies to SRS for MIMO.

# TP 2.4Abis # ------------------------------------------Start of Text Proposal for 38.214-------------------------------------------------- # 6.2.1 UE sounding procedure # -------------------------------------------- Unchanged part omitted -------------------------------------------------------- The following SRS parameters are semi-statically configurable by higher layer parameter SRS-Resource or SRS-PosResource-r16. - srs-ResourceId or SRS-PosResourceId-r16 determines SRS resource configuration identity. - Number of SRS ports as defined by the higher layer parameter nrofSRS-Ports and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, nrofSRS-Ports is 1. - Time domain behaviour of SRS resource configuration as indicated by the higher layer parameter resourceType or resourceType-r16, which may be periodic, semi-persistent, aperiodic SRS transmission as defined in Clause 6.4.1.4 of [4, TS 38.211]. - Slot level periodicity and slot level offset as defined by the higher layer parameters periodicityAndOffset-p or periodicityAndOffset-sp for an SRS resource of type periodic or semi-persistent, which is configured by SRS-Resource, and periodicityAndOffset-p-r16 or periodicityAndOffset-sp-r16 for an SRS resource of type periodic or semi-persistent, which is configured by SRS-PosResource-r16. The UE is not expected to be configured with SRS resources in the same SRS resource set SRS-ResourceSet or SRS-PosResourceSet-r16 with different slot level periodicities. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a slot level offset is defined by the higher layer parameter slotOffset. For an SRS-PosResourceSet-r16 configured with higher layer parameter resourceType-r16 set to 'aperiodic', the slot level offset is defined by the higher layer parameter slotOffset-r16 for each SRS resource. - Number of OFDM symbols in the SRS resource, starting OFDM symbol of the SRS resource within a slot including repetition factor R as defined by the higher layer parameter resourceMapping or resourceMapping-r16 and described in Clause 6.4.1.4 of [4, TS 38.211]. If R is not configured, then R is equal to the number of OFDM symbols in the SRS resource. - SRS bandwidth and , as defined by the higher layer parameter freqHopping or freqHopping-r16 and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then= 0. - Frequency hopping bandwidth, , as defined by the higher layer parameter freqHopping or freqHopping-r16 and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then = 0. - Defining frequency domain position and configurable shift, as defined by the higher layer parameters freqDomainPosition and freqDomainShift or freqDomainShift-r16 respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. If freqDomainPosition is not configured, freqDomainPosition is zero. - Cyclic shift, as defined by the higher layer parameter cyclicShift-n2 or cyclicShift-n4 for transmission comb value 2 or 4 for an SRS configured by SRS-Resource, respectively, and defined by the higher layer parameter cyclicShift-n2-r16, cyclicShift-n4-r16, or cyclicShift-n8-r16 for transmission comb value 2, 4 or 8 for an SRS configured by SRS-PosResource-r16, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. - Transmission comb value as defined by the higher layer parameter transmissionComb or transmissionComb-r16 described in Clause 6.4.1.4 of [4, TS 38.211]. - Transmission comb offset as defined by the higher layer parameter combOffset-n2 or combOffset-n4 for transmission comb value 2 or 4 for an SRS configured by SRS-Resource, respectively, and defined by the higher layer parameter combOffset-n2-r16, combOffset-n4-r16, or combOffset-n8-r16 for transmission comb value 2, 4, or 8 for an SRS configured by SRS-PosResource-r16, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. - SRS sequence ID as defined by the higher layer parameter sequenceId or sequenceId-r16 in Clause 6.4.1.4 of [4]. - The configuration of the spatial relation between a reference RS and the target SRS, where the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16, if configured, contains the ID of the reference RS. The reference RS may be an SS/PBCH block, CSI-RS configured on serving cell indicated by higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise, or an SRS configured on uplink BWP indicated by the higher layer parameter uplinkBWP or uplinkBWP-r16 , and serving cell indicated by the higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise. When an SRS is configured by the higher layer parameter SRS-PosResourceSet-r16, the reference RS may also be a DL PRS configured on a serving cell, an SS/PBCH block or a DL PRS of a non-serving cell indicated by a higher layer parameter. -------------------------------------------Unchanged part omitted --------------------------------------------------------- ---------------------------------------------End of Text Proposal ----------------------------------------------------------

## 2.5	Aspect #18: Prioritization for Transmission Power Reduction

### 2.4.1 summary and proposals

- In  [[13], LGE] a TP (reproduced below as TP2.5A)  is proposed for section 7.5 Prioritizations for transmission power reductions`

TP2.5A ---- Unchanged parts omitted ---- The total UE transmit power in a symbol of a slot is defined as the sum of the linear values of UE transmit powers for PUSCH, PUCCH, PRACH, and SRS in the symbol of the slot. ---- Unchanged parts omitted ---- - SRS transmission, with aperiodic SRS having higher priority than semi-persistent and/or periodic SRS, or PRACH transmission on a serving cell other than the Pcell - SRS transmission, with SRS resource configured by SRS-Resource having higher priority than SRS resource configured by SRS-PosResource-r16 ---- Unchanged parts omitted ----

Companies are encouraged to provide their feedback on TP2.5A  in the comment section below.

| Company | Comment |
| --- | --- |
| Nokia/NSB | Support. |
| Vivo | Not support. In our opinion, the power allocation issues when SRS for positioning is transmitted simultaneously with other signal is bigger than the solution of this proposed TP. For instance, should periodic SRS (for MIMO) always take priority over aperiodic SRS for positioning? There’re many contributions for Rel-17 positioning SI discussing some priority among signals including power allocation. We think this issue in section 2.5 should be left to Rel-17 for a complete discussion. |
| OPPO | Not support There is no motivation to specify that SRS for MIMO has higher priority than SRS for position or the other way. |
| Huawei/HiSilicon | We suggest not to support the enhancement at this stage. Note that below the text, there is additional priority between CCs (CC with PUCCH and without PUCCH, NUL and SUL) than also be used to address simultaneous SRS transmission between CCs. |
| ZTE | Not support. The issue can be discussed in Rel-17, since it’s more relevant to latency/priority enhancement. |
| CATT | Not support. It seems like no sufficient arguments to support SRS-MIMO has higher priority than SRS-Pos. |
| LG | Support. We understand it is difficult to discuss it at this stage, but the Rel-16 UE behavior is ambiguous when a total transmission power determined by the power control rule exceeds the maximum available power for simultaneous transmission of SRS resources. There is a simple motiviation to have high priority of SRS for MIMO since SRS for MIMO can also be resued for positioning purpose, so it has higher priority in case of simultaneous transmission. We are open to discuss further issues such as consideraion of time-domain behaviour, NUL/SUL and so forth to complete the Rel-16 UE-behaviour for transmission of SRS for positioning. |
| Huawei/HiSilicon | In reply to LG, what SRS is in use for positioning depends on what SRS configuration is transferred to LMF and forwarded to the TRPs, which means that UE may transmit MIMO-SRS, but no TRP is listenning to it, instead they are listenining to the simultaneously transmitted positioning SRS. We suggest to the keep the current priority rule and any enhancement can be discussed in Rel-17. |
| Qualcomm | We consider it an enhancment at this stage, so we have preference to not support it. |
| MTK | Discuss this in Rel-17 |

### 2.4.2 status at the discussion deadline

The majority of companies do not support the TP.

## 2.6	Aspect #22: Priority of SRS for Positioning

### 2.4.1 summary and proposals

- In [[5]], CATT] it is proposed to have higher priority for Aperiodic SRS-Pos than for PUSCH. In particular, the following proposals were made:

  - Aperiodic SRS-Pos should have a higher transmission priority than PUSCH, and PUSCH should be dropped in the overlapped symbols when colliding with aperiodic SRS-Pos.

  - Adopt the following text proposal (TP-D) for collision handling between SRS-Pos and PUSCH in section 6.2.1 of 38.214:

TP2.6A ---------------------------------------------Start of Text Proposal for 38.211-------------------------------------------- 6.2.1 UE sounding procedure --------------------------------------------- Unchanged part omitted --------------------------------------------------- For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s), or only L1-SINR report(s). A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK, link recovery request (as defined in clause 9.2.4 of [6, 38.213]) and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s). For PUSCH and SRS on the same carrier, PUSCH shall not be transmitted when aperiodic SRS configured by the higher layer parameter srs-PosResource-r16 is triggered to be transmitted to overlap in the same symbol with PUSCH. ---------------------------------------------- Unchanged part omitted ------------------------------------------------------ -------------------------------------------End of Text Proposal ------------------------------------------------------------

Companies are encouraged to provide their feedback on TP2.6A  in the comment section below.

| Company | Comment |
| --- | --- |
| Nokia/NSB | We consider this an enhancement at this stage and suggest proponents to bring it to Rel-17 for discussion. |
| vivo | We had a similar proposal in previous RAN1 meeting. We support this in principle. However, as we mentioned in section 2.5, we think this whole priority issue should be discussed in details in Rel-17. |
| OPPO | Do not support the TP. That is a new enhancement and we shall not introduce new enhancement at later CR stage. It can be discussed in Rel-17. Furthermore, we do not think we can simply conclude that SRS for positioning always has higher priority than PUSCH. PUSCH has different types: URLLC PUSCH and eMBB PUSCH. Apparently, URLLC PUSCH shall not have lower priority than SRS. |
| Huawei/HiSilicon | Similar view to Nokia/NSB, vivo and OPPO. |
| ZTE | Not support. It’s an enhancement at CR stage. |
| CATT | If most of companies support to discuss this issue in Rel-17, we can accept not to pursue it in Rel-16. |
| SS | Agree with Nokia. |
| LG | Same view with Nokia |
| Qualcomm | Not support |

| MTK | Discuss this in Rel-17 |
| --- | --- |

### 2.4.2 status at the discussion deadline

Most companies do not want to bring the issue in release 16. Therefore, the TP won’t be pursued.

# Conclusion

Aspect #16 and aspect #17 TPs are endorsed as follow:

Aspect #16: MAC CE for SP/AP SRS Spatial Relation Indication

Specification: 	 TS 38.214

Clauses affected:	6.2.1

Reason for Change: 	clarification regarding the Spatial relation assumption in the activation command for semi persistent SRS for positioning.

Summary of Change: 	the correction clarifies that the spatial relation update command for SRS in MAC CE  only applies to SRS resources configured with SRS-Resource

TP 2.3B ===================== Unchanged parts omitted ========================= For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource-r16 is set to 'aperiodic': ===================== Unchanged parts omitted ========================= - when a UE receives an spatial relation update command, as described in clause 6.1.3.26 of [10, TS 38.321], for an SRS resource configured with the higher layer parameter SRS-Resource, and when the HARQ-ACK corresponding to the PDSCH carrying the update command is transmitted in slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions on updating spatial relation for the SRS resource shall be applied for SRS transmission starting from the first slot that is after slot n+3Nslotsubframe,µ. The update command contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the updated SRS resource set. Each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the update command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the update command if present, same serving cell and bandwidth part as the SRS resource set otherwise. When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', the UE shall not expect to be configured with different spatial relations for SRS resources in the same SRS resource set. ===================== Unchanged parts omitted =========================

Aspect #17: UE Sounding Procedure - Alignment of Parameter Names

Specification: 	 TS 38.214

Clauses affected:	6.2.1

Reason for Change: 	clarification of higher layer parameter names with respect to their configuration for SRS for positioning or SRS for MIMO.

Summary of Change: 	the correction clarifies which field applies to SRS for positioning and which field applies to SRS for MIMO.

# TP 2.4Abis # ------------------------------------------Start of Text Proposal for 38.214-------------------------------------------------- # 6.2.1 UE sounding procedure # -------------------------------------------- Unchanged part omitted -------------------------------------------------------- The following SRS parameters are semi-statically configurable by higher layer parameter SRS-Resource or SRS-PosResource-r16. - srs-ResourceId or SRS-PosResourceId-r16 determines SRS resource configuration identity. - Number of SRS ports as defined by the higher layer parameter nrofSRS-Ports and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, nrofSRS-Ports is 1. - Time domain behaviour of SRS resource configuration as indicated by the higher layer parameter resourceType or resourceType-r16, which may be periodic, semi-persistent, aperiodic SRS transmission as defined in Clause 6.4.1.4 of [4, TS 38.211]. - Slot level periodicity and slot level offset as defined by the higher layer parameters periodicityAndOffset-p or periodicityAndOffset-sp for an SRS resource of type periodic or semi-persistent, which is configured by SRS-Resource, and periodicityAndOffset-p-r16 or periodicityAndOffset-sp-r16 for an SRS resource of type periodic or semi-persistent, which is configured by SRS-PosResource-r16. The UE is not expected to be configured with SRS resources in the same SRS resource set SRS-ResourceSet or SRS-PosResourceSet-r16 with different slot level periodicities. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a slot level offset is defined by the higher layer parameter slotOffset. For an SRS-PosResourceSet-r16 configured with higher layer parameter resourceType-r16 set to 'aperiodic', the slot level offset is defined by the higher layer parameter slotOffset-r16 for each SRS resource. - Number of OFDM symbols in the SRS resource, starting OFDM symbol of the SRS resource within a slot including repetition factor R as defined by the higher layer parameter resourceMapping or resourceMapping-r16 and described in Clause 6.4.1.4 of [4, TS 38.211]. If R is not configured, then R is equal to the number of OFDM symbols in the SRS resource. - SRS bandwidth and , as defined by the higher layer parameter freqHopping or freqHopping-r16 and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then= 0. - Frequency hopping bandwidth, , as defined by the higher layer parameter freqHopping or freqHopping-r16 and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then = 0. - Defining frequency domain position and configurable shift, as defined by the higher layer parameters freqDomainPosition and freqDomainShift or freqDomainShift-r16 respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. If freqDomainPosition is not configured, freqDomainPosition is zero. - Cyclic shift, as defined by the higher layer parameter cyclicShift-n2 or cyclicShift-n4 for transmission comb value 2 or 4 for an SRS configured by SRS-Resource, respectively, and defined by the higher layer parameter cyclicShift-n2-r16, cyclicShift-n4-r16, or cyclicShift-n8-r16 for transmission comb value 2, 4 or 8 for an SRS configured by SRS-PosResource-r16, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. - Transmission comb value as defined by the higher layer parameter transmissionComb or transmissionComb-r16 described in Clause 6.4.1.4 of [4, TS 38.211]. - Transmission comb offset as defined by the higher layer parameter combOffset-n2 or combOffset-n4 for transmission comb value 2 or 4 for an SRS configured by SRS-Resource, respectively, and defined by the higher layer parameter combOffset-n2-r16, combOffset-n4-r16, or combOffset-n8-r16 for transmission comb value 2, 4, or 8 for an SRS configured by SRS-PosResource-r16, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. - SRS sequence ID as defined by the higher layer parameter sequenceId or sequenceId-r16 in Clause 6.4.1.4 of [4]. - The configuration of the spatial relation between a reference RS and the target SRS, where the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16, if configured, contains the ID of the reference RS. The reference RS may be an SS/PBCH block, CSI-RS configured on serving cell indicated by higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise, or an SRS configured on uplink BWP indicated by the higher layer parameter uplinkBWP or uplinkBWP-r16 , and serving cell indicated by the higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise. When an SRS is configured by the higher layer parameter SRS-PosResourceSet-r16, the reference RS may also be a DL PRS configured on a serving cell, an SS/PBCH block or a DL PRS of a non-serving cell indicated by a higher layer parameter. -------------------------------------------Unchanged part omitted --------------------------------------------------------- ---------------------------------------------End of Text Proposal ----------------------------------------------------------

# References

R1-2006996, Feature lead summary for NR positioning maintenance AI 7.2.8, Moderator (Intel), Ericsson, CATT, Qualcomm

R1-2005357, Remaining issues on DL RS for NR positioning	vivo

R1-2005358, Remaining issues on physical layer procedure for NR positioning	vivo

R1-2005452, Maintenance of NR positioning	ZTE

R1-2005681, Remaining issues on DL PRS and measurements for NR Positioning	CATT

R1-2005682, Remaining issues on UL SRS and UL procedures for NR Positioning	CATT

R1-2005780, Discussion on QCL for PRS	ZTE

R1-2005795, NR positioning corrections	Huawei, HiSilicon

R1-2005806, RAN1 inputs to RAN3 on SRS support	Huawei, HiSilicon

R1-2005978, Remaining Issues on measurements and procedure for NR Positioning OPPO

R1-2005979, Remaining Issues on RS for Positioning OPPO

R1-2006120, On remaining issues for Rel.16 positioning Samsung

R1-2006199, Remaining issues on DL PRS processing order	CMCC

R1-2006372, Discussion on remaining issues on simultaneous SRS transmission and PRS processing priority for NR positioning	LG Electronics

R1-2006373, Discussion on remaining issues on QCL and spatial relation information for NR positioning		LG Electronics

R1-2006425, Maintenance on measurements for NR positioning	Nokia, Nokia Shanghai Bell

R1-2006426, Priority of Assistance Data	Nokia, Nokia Shanghai Bell

R1-2006784, Maintenance on DL Reference Signals for NR Positioning	Qualcomm Incorporated

R1-2006911, Maintenance of rel16 reference signals for NR positioning	Ericsson

R1-2006912, Maintenance of rel16 Physical-layer procedures to support UE - gNB measurements	Ericsson