3GPP TSG RAN WG1 Meeting #102-e	R1-2007415

eMeeting, August 17 – 28, 2020

Agenda Item:	7.2.4.2.1

Source:	Moderator (Ericsson)

Title:	TPs for 38.212 for the agreements in [102-e-NR-5G_V2X_NRSL-Mode-1-01]

Document for:	Discussion, Decision

# 1	Introduction

This document captures the TPs for 38.213 endorsed in the following e-mail discussion:

[102-e-NR-5G_V2X_NRSL-Mode-1-01] Email discussion/approval covering:

- Remaining issues for configured grant

  - Whether clarifications for the formula determining the granted slots are necessary and whether the issue should be left to RAN2.

  - Clarifications on signalling for number of retransmissions

  - Editorial corrections and clarifications for configured grant (if any).

- DCI aspects

  - Alignment of DCI format 3_0 with other DCI formats.

  - Cells on which the UE monitors DCI formats 3_0 and 3_1, including discussion on PUCCH cell.

  - Editorial corrections and clarifications for DCI (if any).

By 8/20, followed by potential TPs by 8/25 – Ricardo (Ericsson)

The single text proposal in this document captures the following agreements:

Agreements:

- If the DCI size budget is not exceeded, no alignment of DCI format 3_0 / 3_1 with other NR DCI formats is performed.

- If the DCI size budget is exceeded, DCI format 3_0 / 3_1 is zero-padded until the size is equal to that of the next large Uu DCI format (in size).

- The UE does not expect that the following two conditions happen simultaneously:

  - The DCI size budget is exhausted

  - DCI format 3_0 / 3_1 is larger than all other configured DCI formats.

- Note: the DCI size budget is performed for Uu DCI formats first, before the considerations for DCI format 3_0/3_1 as listed in the above bullets

Agreements:

- In the preceding agreement, the DCI size budget refers to the budget of the cell on which the DCI format 3_0 or DCI format 3_1 is received.

# 2	Text Proposals

#### TP 1.2.1-1

The motivation of the TP is as follows:

- Reasons for change: implement agreement on DCI size alignment for SL scheduling.

- Summary of changes:

  - Introduction of a procedure for aligning the DCI sizes when DCI format 3_0 or DCI format 3_1 are monitored on a cell.

- Specs/Sections impacted: TS 38.212 Clause 7.3.1.0.

- Consequences if not approved: the number of DCI sizes monitored by the UE may exceed the budget.

-------------------------- Start of Text Proposal for TS 38.212 -------------------------- <Unchanged parts omitted> #### 7.3.1.0 DCI size alignment <Unchanged parts omitted> ##### 7.3.1.0.1 DCI size alignment for DCI formats for scheduling of sidelink If DCI format 3_0 or DCI format 3_1 are monitored on a cell, DCI size alignment for DCI format 3_0 and DCI format 3_1 is performed as described in this clause after performing the DCI size alignment described in Clause 7.3.1.0. If DCI format 3_0 or DCI format 3_1 are monitored on a cell and the total number of DCI sizes of the DCI formats configured for [scheduling] the cell and DCI format 3_0 or DCI format 3_1 does not fulfill the [first] condition defined in Step 3 of Clause 7.3.1.0, zeros shall be appended to DCI format 3_0, if configured, and DCI format 3_1, if configured, until the payload size equals that of the smallest monitored DCI format after DCI size alignment in 7.3.1.0 for [scheduling] the cell that is larger than DCI format 3_0 or DCI format 3_1 if such a larger format is configured. The UE is not expected to handle a configuration that, [after] applying the above steps, results in - the total number of DCI sizes of the DCI formats configured for [scheduling] the cell and DCI format 3_0 or DCI format 3_1 does not fulfill the [first] condition defined in Step 3 of Clause 7.3.1.0; and - the payload size of DCI format 3_0 or DCI format 3_1 is larger than the payload size of all other DCI formats configured for [scheduling] the cell. ------------------------------------ End of Text Proposal ------------------------------------