# 3GPP TSG RAN WG1 #102-e						R1-200xxxx

# e-Meeting, August 17th – 28th, 2020

Agenda item:	5

Source: 	Moderator (Qualcomm Incorporated)

Title: 	[102-e-LS-AI5-04] Discussion on NR SRS carrier switching

Document for:	Discussion and Decision

- Background

[…]

for typeA RAN2 didn’t achieve consensus on whether the following 2 cases on NR SRS carrier switching are allowed:

- Case 1: SRS carrier switching to a SUL without PUCCH/PUSCH while PUCCH/PUSCH is configured in NUL;

- Case 2: SRS carrier switching to both NUL and SUL, neither of which are configured with PUCCH/PUSCH.

2. Actions:

To RAN1 group.

ACTION: 	RAN2 respectively asks RAN1 to provide feedback on the above

In LS R1-2005209, RAN1 received the following questions from RAN2:

As a result of this LS, the following contributions were submitted to this meeting (7 contributions, 5 different sources):

R1-2005466	Draft reply LS on NR SRS carrier switching	ZTE

R1-2005553	Discussion for NR SRS carrier switching on SUL	ZTE

R1-2005652	Draft reply LS to RAN2 on NR SRS carrier switching	CATT

R1-2005654	Discussion on NR SRS carrier switching on SUL carrier	CATT

R1-2006078	[Draft] Reply LS on NR SRS carrier switching	Samsung

R1-2006757	Discussion on SRS carrier switching	Qualcomm Incorporated

R1-2006945	Discussion on NR SRS carrier switching	Huawei, HiSilicon

- Summary of input contributions

In the following, we summarize the received input:

- Three sources [ZTE, Samsung, Qualcomm] indicate that the two cases from RAN2 are not supported from RAN1 specification point of view.

- One source [CATT] indicates that Case 1 is not supported, and in Case 2 switching to SUL is not supported, but switching to NUL is supported.

- One source [Huawei/HiSi] indicates that both case 1 and Case 2 are supported.

- One source [Qualcomm] further states that SRS carrier switching is not supported for a target serving cell configured with SUL.

Several reasons are included in the contributions for the support / no support of this feature.

- Four sources [ZTE, CATT, Samsung, Qualcomm] mention the following specification snippet (TS 38.214) that indicates that SRS carrier switching is only supported for a “serving cell with slot formats comprised of DL and UL symbols”:

#### 6.2.1.3 UE sounding procedure between component carriers …. A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of srs-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier c2. ….

- Two sources [Qualcomm, ZTE] mention that SRS carrier switching is for DL CSI acquisition (SRS resource set(s) with higher layer parameter usage set to 'antennaSwitching') and, therefore, there is no use case for SUL.

- One source [Huawei/HiSi] mentions the following two agreements (in RAN1#89 and RAN1#91) as justification for supporting both Case 1 and Case 2:

Agreements: - For NR standalone operation for a UE, - NR supports that the UE is allowed to transmit on UL carriers on different frequency ranges but the UE has the capability to only transmit on one of the carriers at a given time in the following case: - case of SRS carrier switching with at least one of the frequency ranges agreed for LTE-NR UL sharing by RAN4 (e.g. refer to R4-1704411)

Agreement: - For a UE configured two ULs in a cell, two TPC bit(s) fields within group common DCI for SRS power control can be configured to the UE, i.e., one for UL and one for SUL;

- Discussion points

Q1:	Is the following case supported form RAN1 point of view?

- Case 1: SRS carrier switching to a SUL without PUCCH/PUSCH while PUCCH/PUSCH is configured in NUL;

| Company | Input |
| --- | --- |
| ZTE | Not supported. As we mentioned in our tdoc, SRS carrier switching is for DL CSI in TDD band. However, SUL has no DL symbols. |

Q2:	Is the following case supported form RAN1 point of view? (NOTE: Sub-case of Case 2)

- Case 2.1: SRS carrier switching to SUL, when neither SUL nor NUL of which are configured with PUCCH/PUSCH.

| Company | Input |
| --- | --- |
| ZTE | Not supported. |

Q3:	Is the following case supported form RAN1 point of view? (NOTE: Sub-case of Case 2)

- Case 2.2: SRS carrier switching to NUL, when neither SUL nor NUL of which are configured with PUCCH/PUSCH.

| Company | Input |
| --- | --- |
| ZTE | Not supported As we explained in our tdoc, we don’t think it is a valid usecase that SUL is configured without PUCCH/PUSCH. |

Q4:	Is SRS carrier switching supported for a target serving cell configured with SUL?

| Company | Input |
| --- | --- |
| ZTE | This question is not related with the LS. However, technically, we don’t think it is an appropriate usecase that SRS carrier switching to a target serving cell which is configured SUL. |

- Conclusion

<To be completed after discussion>

Appendix: Summary of proposals in input contributions

| R1-2005466 / R1-2005553 | ZTE | […] Thus, it is not valid that a SUL without any PUSCH and PUCCH transmission is configured with SRS carrier switching for DL CSI acquisition. Therefore, the above two cases are not allowed from RAN1 perspective. |
| --- | --- | --- |
| R1-2005652 / R1-2005654 | CATT | […] that in both cases SRS carrier switching to SUL carrier shall NOT be allowed. In Case 2, SRS carrier switching to the NUL shall be allowed. |
| R1-2006078 | Samsung | […] Hence, the NR SRS carrier switching are not allowed for both Case 1 and Case 2. |
| R1-2006757 | Qualcomm | Indicate to RAN2 that Case 1 and Case 2 are not supported in RAN1 specifications. […] Include the following conclusion in the LS to RAN2: - SRS carrier switching is not supported for a target serving cell configured with SUL. - SRS carrier switching is supported for a source serving cell configured with SUL. The interrupted carrier is signaled by srs-SwitchFromCarrier. |
| R1-2006945 | Huawei, HiSilicon | Confirm that both Case 1 and Case 2 described in R1-2005209 are supported by Type A SRS carrier switching. |