3GPP TSG RAN WG1 Meeting #102-e	R1-200xxxx

E-meeting, August 17–28, 2020

Agenda Item:	6.2.2

Source:	Moderator (Huawei)

Title:	Text Proposal for Issue#1 in [102-e-LTE-NB_IoTenh3-01]

Document for:	Discussion and Decision

Introduction

This contribution provides the text proposal for Issue#1 of the following email discussion:

[102-e-LTE-NB_IoTenh3-01] Corrections regarding RAN2 LS reply on PUR – Xiang (Huawei)

- Issue#1: L1 adjustment on the NPUSCH repetition number

- Issue#2: PUR collision handling

- Discussions/Agreement by 8/21, TPs by 8/28

| Reason for change: | - For PUR, the UE uses the NPUSCH repetition number from dedicated PUR ACK DCI or RRC configuration, whichever is received latest. And the L1 adjustment on the (N)PUSCH repetition number is intended to update the higher layer (i.e. RRC) configuration. In the current spec, the NPUSCH repetition number cannot be further updated by RRC configuration, and is against the intention of “intended to update the higher layer (i.e. RRC) configuration”. - In TS36.213 section 16.5.1.1, “Subcarrier indication field” and “resource assignment field” are used. However, these are different from the names in PUR-config-NB. |
| --- | --- |
| Summary of change: | - Replace “determined by the NPUSCH repetition adjustment field according to Table 16.5.1.1-3 from the most recent NPDCCH DCI format N0 with CRC scrambled by PUR C-RNTI with the value of "modulation and coding scheme" field () set to '14' if detected, configured by higher layers otherwise.” with “configured by higher layers” - Add the correct name in PUR-config-NB. |
| Consequences if not approved: | - The NPUSCH repetition number cannot be further updated by RRC configuration, and is against the intention of “intended to update the higher layer (i.e. RRC) configuration”. - The parameter names are not aligned between TS 36.213 and TS 36.331. |
| Clauses affected: | 16.5.1.1, 16.6.4 of TS 36.213 |

Text Proposal

------------------------------------- Start of Text Proposal for TS 36.213------------------------------- ------------------------------------------- Unchanged parts omitted -------------------------------------- #### 16.5.1.1 Resource allocation The resource allocation information in uplink DCI format N0 for NPUSCH transmission or configured by higher layers for NPUSCH transmission using preconfigured uplink resource indicates to a scheduled UE - a set of contiguously allocated subcarriers () of a resource unit determined by the Subcarrier indication field, or by the higher layer parameter npusch-SubCarrierSetIndex in PUR-Config-NB - a number of resource units () determined by the resource assignment field according to Table 16.5.1.1-2, or by the higher layer parameter npusch-NumRUsIndex in PUR-Config-NB - a repetition number () determined by the repetition number field according to Table 16.5.1.1-3. For a NPUSCH transmission using preconfigured uplink resource, the UE shall use the repetition number configured by higher layers. … ### 16.6.4 Preconfigured uplink resource ACK/fallback procedure If a UE has initiated a NPUSCH transmission using preconfigured uplink resource on a given serving cell, and upon detection of a NPDCCH with DCI format N0 with CRC scrambled by PUR C-RNTI intended for the UE within the PUR search space window as defined in Subclause 16.6, and the value of "modulation and coding scheme" field () in the corresponding DCI set to '14', the UE shall deliver the PUR ACK/fallback indication and the NPUSCH repetition adjustment, as signaled on the NPDCCH, to the higher layers. -------------------------------------------- Unchanged parts omitted ------------------------------------- ---------------------------------------------- End of Text Proposal ----------------------------------------

- Reference

- R1-2005471	Remaining issues for transmission in preconfigured UL resources for NB-IoT	ZTE

- R1-2005556	PUR maintenance issues for Rel-16 NB-IoT	Ericsson

- R1-2005816	Corrections regarding RAN2 LS reply on PUR	Huawei, HiSilicon

- R1-2006189	Maintenance on PUR	Qualcomm Incorporated

- R1-2006419	Corrections on transmission in preconfigured UL resources for NB-IoT	Huawei, HiSilicon