3GPP TSG RAN WG1 Meeting #102-e	R1-200xxxx

E-meeting, August 17–28, 2020

Agenda Item:	6.2.2

Source:	Moderator (Huawei)

Title:	Text Proposal for Issue#4 in [102-e-LTE-NB_IoTenh3-02]

Document for:	Discussion and Decision

Introduction

This contribution provides the text proposal for Issue#1 of the following email discussion:

[102-e-LTE-NB_IoTenh3-02] Clarification on sequence-group hopping and NPDSCH/NPDCCH interference randomization – Xiang (Huawei)

- Issue#3: Sequence-group hopping

- Issue#4: NPDSCH/NPDCCH interference randomization

- Discussions/Agreement by 8/21, TPs by 8/28

| Reason for change: | - In the discussion of the WI, it was assumed to re-use legacy NPDCCH/NPDSCH transmission schemes. However, some places in the spec are overlooked with the introduction of PUR-RNTI, such as the interference randomization. |
| --- | --- |
| Summary of change: | - Add “PUR-RNTI” in NPDSCH/NPDCCH interference randomization. - Change “or” to “/”, and add some line breaks for better readability |
| Consequences if not approved: | - Interference randomization does not apply to NPDSCH/NPDCCH related to PUR. |
| Clauses affected: | 10.2.3.4, 10.2.5.5 of TS 36.211 |

Text Proposal

------------------------------------- Start of Text Proposal for TS 36.211------------------------------- ------------------------------------------- Unchanged parts omitted -------------------------------------- ### 10.2.3 Narrowband physical downlink shared channel #### 10.2.3.4 Mapping to resource elements … For frame structure type 1, - for NPDSCH associated with C-RNTI when interferenceRandomisationConfig is used according to [11], or - for NPDSCH associated with RA-RNTI, TC-RNTI or P-RNTI and transmitted in an NB-IoT carrier configured by SystemInformationBlockType22-NB, or - for NPDSCH associated with C-RNTI in an NB-IoT carrier configured by SystemInformationBlockType22-NB when RadioResourceConfigDedicted-NB is not configured by higher layer, or - for NPDSCH associated with PUR-RNTI/G-RNTI/SC-RNTI, or for frame structure type 2, - for for NPDSCH not carrying the BCCH, define as the block of complex-valued symbols mapped to subframe number ns/2 and radio frame number . … ### 10.2.5 Narrowband physical downlink control channel #### 10.2.5.5 Mapping to resource elements #### … For frame structure type 1, - for NPDCCH associated with RA-RNTI, TC-RNTI or - for P-RNTI and transmitted in an NB-IoT carrier configured by SystemInformationBlockType22-NB, or - for NPDCCH associated with C-RNTI in an NB-IoT carrier configured by SystemInformationBlockType22-NB when RadioResourceConfigDedicted-NB is not configured by higher layer, or - for NPDCCH associated with PUR-RNTI/G-RNTI/SC-RNTI, or - for NPDCCH associated with C-RNTI when interferenceRandomisationConfig is used according to [11], or for frame structure type 2, each complex-valued symbol , shall be multiplied with ,where -------------------------------------------- Unchanged parts omitted ------------------------------------- ---------------------------------------------- End of Text Proposal ----------------------------------------

- Reference

- R1-2005471	Remaining issues for transmission in preconfigured UL resources for NB-IoT	ZTE

- R1-2005556	PUR maintenance issues for Rel-16 NB-IoT	Ericsson

- R1-2005816	Corrections regarding RAN2 LS reply on PUR	Huawei, HiSilicon

- R1-2006189	Maintenance on PUR	Qualcomm Incorporated

- R1-2006419	Corrections on transmission in preconfigured UL resources for NB-IoT	Huawei, HiSilicon