3GPP TSG-RAN WG1 Meeting #102-e	R1-200xxxx

e-Meeting, 17 – 28 August 2020

Title:	[draft] Reply LS on UE capability

Response to:

Release:	Rel-16

Work Item:	5G_V2X_NRSL

Source:	OPPO [RAN1]

To:	RAN2

Cc:	RAN4

Contact Person:

#### Name:	Kevin Lin

Tel. Number:

E-mail Address:	Kevin.Lin@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN1 thank RAN2’s LS in R2-2008346 and would like to provide our answers to the following questions.

Q1: In the received latest version of RAN1 feature list, there is a following NOTE referring to an undefined table “Table 5.2E-1” in 38.101-1. RAN1 is respectfully asked to provide feedback on how it should be corrected (e.g., to “Table 5.2-1” or “Table 5.2E.1-1”).

Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1

Answer to Q1: According to the latest version of TS 38.101-1 v16.4.0, the correct table number should be “Table 5.2E.1-1”. We will provide updates of this in the next version of RAN1 UE features list for NR and LTE.

Q2: In the received latest version of RAN1 feature list, there is a component defined in FG 15-2 (for NR) / 5-2 (for LTE)

Up to 8 configured grants can be configured for a UE.

Considering RAN2 has agreed in RAN2#109bis that

8: For SL capability report on Uu-RRC, introduce MAC parameters: a) LCP restriction, b) Logical channel SR-delay timer, c) Multiple CGs.

RAN2 understands in FG 15-2 (for NR) / 5-2 (for LTE), the “8 configured grants” is for maximum value that the UE supports, while UE reports the specific number of supported configured grants via the agreed capability at RAN2 as follows. RAN1 is respectfully asked to provide feedback if any concern.

| multipleConfiguredGrantsSidelink-r16 Indicates whether UE supports 8 sidelink configured grant configurations (including both Type 1 and Type 2) in a resource pool. If absent, for each resource pool, the UE only supports one sidelink configured grant configuration. | UE | No | No | No |
| --- | --- | --- | --- | --- |

Proposal for Q2: No reply to this question since it is in line with RAN1’s understanding.

Q3: In the received latest version of RAN1 feature list, there is a component defined in FG 15-16 (for NR) / 5-11 (for LTE), on simultaneous transmission of uplink and sidelink, RAN1 is respectfully asked to provide feedback on whether and how for network to know whether UE support simultaneous reception of downlink and sidelink, on the preferred way between the two following solutions:

Alt-1: a separate capability for simultaneous reception of downlink and sidelink is defined as in LTE V2X;

Alt-2: simultaneous reception is automatically supported if the same Uu-BC/PC5-BC is reported to support simultaneous transmission of uplink and sidelink;

According to TS 36.306, the following is defined for simultaneous reception of uplink and sidelink in LTE V2X.

#### 4.3.5.27 v2x-SupportedTxBandCombListPerBC-r14, v2x-SupportedRxBandCombListPerBC-r14 This field indicates, for a particular band combination of EUTRA, the supported band combination list among v2x-SupportedTxBandCombinationList or v2x-SupportedRxBandCombinationList on which the UE supports simultaneous transmission and reception of EUTRA and V2X sidelink communication respectively.

Company are invited to provide preference and views on which alternative to take.

| Company | Preference | Comments |
| --- | --- | --- |
| OPPO | Alt-1 | It is not certain for us that it can be assumed that a UE automatically support simultaneous reception in the same Uu/PC5 band combination(s) for simultaneous uplink and sidelink transmission as in Alt-2. Since there is not much time to evaluate and discuss this topic, Alt-1 is a safer option at this stage. Also, since Alt-1 is one of the preferred solutions mentioned, in our understanding there would be no concern to introduce a new FG 15-24 (for NR) and FG 15-18 (for LTE) for simultaneous reception of downlink and sidelink in RAN2. The new FG could look something like in the following for NR and LTE, respectively. |

| 15-24 | Simultaneous reception of downlink and sidelink | UE supports simultaneous reception of NR downlink and NR sidelink (in different bands) in a band combination for which the UE indicated simultaneous sidelink and downlink support in a band combination. | 15-1 | Yes | No |  | Per band combination | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 5-18 | Simultaneous reception of downlink and sidelink | UE supports simultaneous reception of LTE downlink and NR sidelink (in different bands) in a band combination for which the UE indicated simultaneous sidelink and downlink support in a band combination. | 5-1 | Yes | N/A |  | Per band combination | N.A. | N.A. |  |  | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Answer to Q3: TBD based on discussion outcome.

Q4: In the received latest version of RAN1 feature list, there is a NOTE defined for 15-2 (for NR) / 5-2 (for LTE) and 15-3 (for NR) / 5-3 (for LTE), there is a NOTE:

Note: the UE supports up max(B, C) as the total number of sidelink HARQ processes across both Mode 1 and Mode 2

Since RAN2 has agreed in RAN2#109-E that no simultaneous mode-1 and mode-2,

10:	Simultaneous operation of NR Sidelink Mode 1 and 2 is not supported for a UE performing transmission of NR sidelink communication.

the NOTE above is not to be captured. RAN1 is respectfully asked to provide feedback if any concern.

Proposal for Q4: No reply to this question since it is in line with RAN1’s understanding.

2. Actions:

To RAN2 group

RAN1 respectfully requests RAN2 to take the above response to Q1 and Q3 into account in their future work.

3. Date of Next TSG-RAN1 Meetings:

RAN1#103-e	26 October - 13 November 2020   	E-Meeting

RAN1#104	1-5 March 2021		Athens