3GPP TSG RAN WG1 #102-e			R1-200xxxx

e-Meeting, August 17th – 28th, 2020

Agenda item:	7.2.2.2.3

Source: 	Moderator (Huawei)

Title: 	TP for NR-U maintenance email thread 102-e-NR-unlic-NRU-HARQ-02

Document for:	Discussion and Decision

- Analysis

| Reasons for change | There is ambiguity on the TDRA table for DCI format 0_1 when multiple PUSCH scheduling is configured to the UE. |
| --- | --- |
| Summary of changes | Correction to include pusch-TimeDomainAllocationListForMultiPUSCH in the applicable PUSCH time domain resource allocation for DCI format 0_1 in UE specific search space, and clarification of the TDRA field in DCI format 0_1. |
| Specs/Sections impacted | TS38.214 v16.2.0 section 6.1.2.1.1, TS38.212 v16.2.0 section 7.3.1.1.2. |
| Consequences if not approved | The UE applies an incorrect TDRA table when it is configured with pusch-TimeDomainAllocationListForMultiPUSCH |

- Text proposal

================== Start of text proposal for TS 38.212 v16.2.0 ===================

7.3.1.1.2	Format 0_1

<Unchanged parts are omitted>

-	Time domain resource assignment – 0, 1, 2, 3, 4, 5, or 6 bits

-	If the higher layer parameter PUSCH-TimeDomainResourceAllocationList-ForDCIformat0_1 is not configured and if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is not configured and if the higher layer parameter pusch-TimeDomainAllocationList is configured, 0, 1, 2, 3, or 4 bits as defined in Clause 6.1.2.1 of [6, TS38.214]. The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter pusch-TimeDomainAllocationList;

-	If the higher layer parameter PUSCH-TimeDomainResourceAllocationList-ForDCIformat0_1 is configured or if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured, 0, 1, 2, 3, 4, 5 or 6 bits as defined in Clause 6.1.2.1 of [6, TS38.214]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter PUSCH-TimeDomainResourceAllocationList-ForDCIformat0_1 or pusch-TimeDomainAllocationListForMultiPUSCH;

-	otherwise the bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the default table.

-	Frequency hopping flag – 0 or 1 bit:

-	0 bit if only resource allocation type 0 is configured, or if the higher layer parameter frequencyHopping is not configured and the higher layer parameter pusch-RepTypeIndicatorForDCI-Format0-1 is not configured to pusch-RepTypeB, or if the higher layer parameter frequencyHoppingForDCI-Format0-1 is not configured and pusch-RepTypeIndicatorForDCI-Format0-1 is configured to pusch-RepTypeB, or if only resource allocation type 2 is configured;

-	1 bit according to Table 7.3.1.1.1-3 otherwise, only applicable to resource allocation type 1, as defined in Clause 6.3 of [6, TS 38.214].

-	Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214]

-	New data indicator – 1 bit if the number of scheduled PUSCH indicated by the Time domain resource assignment field is 1; otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined based on the maximum number of schedulable PUSCH among all entries in the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, where each bit corresponds to one scheduled PUSCH as defined in clause 6.1.4 in [6, TS 38.214].

-	Redundancy version – – number of bits determined by the following:

-	2 bits as defined in Table 7.3.1.1.1-2 if the number of scheduled PUSCH indicated by the Time domain resource assignment field is 1;

-	otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined by the maximum number of schedulable PUSCHs among all entries in the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH, where each bit corresponds to one scheduled PUSCH as defined in clause 6.1.4 in [6, TS 38.214] and redundancy version is determined according to Table 7.3.1.1.2-34.

<Unchanged parts are omitted>

================== End of text proposal for TS 38.212 v16.2.0 ===================

================== Start of text proposal for TS 38.214 v16.2.0 ===================

6.1.2.1.1	Determination of the resource allocation table to be used for PUSCH

<Unchanged parts are omitted>

Table 6.1.2.1.1-1A: Applicable PUSCH time domain resource allocation for DCI format 0_1 in UE specific search space scrambled with C-RNTI, MCS-C-RNTI, CS-RNTI or SP-CSI-RNTI

| pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList -ForDCIformat0_1 | pusch-Config includes pusch-TimeDomainAllocationListForMultiPUSCH | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- |
| No | No | No | No | Default A |
| Yes | No | No | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| No/Yes | Yes | No | No | pusch-TimeDomainAllocationList provided in pusch-Config |
| No/Yes | No/Yes | Yes | No | pusch-TimeDomainAllocationList-ForDCIformat0_1 provided in pusch-Config |
| No/Yes | No/Yes | No | Yes | pusch-TimeDomainAllocationListForMultiPUSCH provided in pusch-Config |

<Unchanged parts are omitted>

================== End of text proposal for TS 38.214 v16.2.0 ===================