[102-e-NR-5G_V2X_NRSL-PHYstructure-01] Email discussion/approval w.r.t. “PSCCH and 2nd SCI related issue including their RS”: 1A, 1B, 9F (as in the summary) by 08/21, with potential TPs by 8/26 – Jeongho (Samsung)

This document has the following questions.

1A. Maximum length of 2nd SCI rate matching

1B. Starting symbol, resource, Q_2ndSCI

-B1

-B2

-B3

9F. Number of PSCCH RBs when sub-channel size is less than 20 PRBs

A. Maximum length of 2nd SCI rate matching

In current specifications (TS38.212), the maximum length (K) of 2nd SCI for rate matching is not defined yet.

- The output bit sequence after rate matching is denoted as g0SCI2, g1SCI2, g2SCI2, g3SCI2,⋯, gGSCI2-1SCI2, where GSCI2=QSCI2'∙QmSCI2 and QmSCI2 is modulation order of the 2nd-stage SCI. A UE is not expected to have GSCI2>K.

In RAN1#102-e, a number of contributions are submitted [Futurewei], [vivo], [CATT], [Huawei, HiSilicon], [Intel], [Ericsson], [Apple], [Qualcomm] with the following alternatives.

- Alt 1. 2048: Futurewei, Huawei, HiSilicon, Apple

- Alt 2. 4096: vivo, CATT, Intel, Ericsson, Qualcomm

- Alt 3. Other value.

Please provide your views and reason.

| Company | Views |
| --- | --- |
| LG Electronics | Alt 3(8192). For compromise, Alt 2 is acceptable. We are not yet fully convinced with the necessity of having further restriction on the value of E for PSCCH. According to TS38.212, in the perspective of polar coding, the number of coded bits is 32, 64, 128, 256, 512 as in PDCCH polar coding. For the other values of E, the UE will perform rate-matching including puncturing, shortening, or circular repetition. In case of circular repetition, the RX UE does not needs to have separate bin for all the E coded bits. LLR (log-likelihood ratio) associated with circular repeated bits will be added to suitable bin, so the large E may not make large memory size. |
| Ericsson | We support Alt.2 |
| Qualcomm | We support Alt 2. Since K is a signaling restriction, using 2048 would exclude some MCS values in the low-SE table from being used. |
| Apple | We support Alt.1. Performance viewpoint: If 2nd SCI rate matching output bits is 4096/2048 bits, then the corresponding coding rate is about 0.017/0.03. Considering the lowest coding rate of 1st SCI is only about 0.03, we do not see the strong motivation that 2nd SCI rate matching output bits is 4096 bits. Implementation viewpoint: Since a UE needs to decode multiple 2nd SCI in a slot, the channel de-interleaver should not be too complex for each 2nd SCI decoding. It is different from Uu link where a UE only make a single channel interleaving operation on UCI, which is upper bounded by 8192 bits. |
| Intel | Agree with Alt 2. According to the current definition of SCI 2-B in 38.212, the current maximum size of any SCI 2 is 72, including 24-bit CRCs. The MCS 0 of Table 5.1.3.1-3 in 38.214 having a code rate of 30/1024. If we assume that the SCI 2 should at least have the same code rate, the resulting number of coded bits would be 2458. |
| Nokia, NSB | Alt 2 |
| Sharp | Alt 2 |
| ZTE, Sanechips | Alt 2 |
| CATT | Alt 2 |
| OPPO | Support Alt 2. |
| vivo | The complexity of the polar coding mainly depends on the mother code bits. According to the minimum code rate defined in polar coding as 1/8, the number of rate matching bits can be 512/ (1/8) = 4096bits, which is good tradeoff between future extension and complexity. |
| MediaTek | Alt 2 |
| NEC | Alt 2 |
| Huawei, HiSilicon | Holding UE complexity to the lowest necessary level is desirable, so Alt. 1 would be possible with what might be a relatively small restriction regarding MCS0 of the URLLC 64-QAM table. However, if companies generally view Alt. 2 (4096 bits) as the really essential level, then we can accept it. To LG: Last meeting already agreed K = either 2048 or 4096. Value of 8192 is reverting the agreement. |
| Samsung | Alt 2 |

Summary

Considering the views from companies, we can agree with Alt 2.

Proposal

In current specifications (TS38.212), for the the maximum length (K) of 2nd SCI for rate matching, the following TP is adopted.

- The output bit sequence after rate matching is denoted as g0SCI2, g1SCI2, g2SCI2, g3SCI2,⋯, gGSCI2-1SCI2, where GSCI2=QSCI2'∙QmSCI2 and QmSCI2 is modulation order of the 2nd-stage SCI. A UE is not expected to have GSCI2>K4096.

B. resource mapping of 2nd SCI

Regarding the resource mapping of the 2nd SCI, the contributions [Futurewei], [vivo], [Panasonic], [Sharp], [Qualcomm] are submitted to RAN1#102-e.

Issue B-1.

The current specifications define the 2nd SCI mapping from the first PSSCH DMRS symbols as found in Appendix 1.

Two contributions [Futurewei], [Panasonic] proposes to map the 2nd SCI around the first PSSCH DMRS, which means the symbols before/after the first PSSCH DMRS are used for 2nd SCI. One contribution proposes to map the 2nd SCI from the first PSSCH DMRS as the spec is.

- Alt 1-1. Map the 2nd SCI from the first PSSCH DMRS (as the current specifications.)

- Alt 1-2. Map the 2nd SCI around the first PSSCH DMRS

- Alt 1-3. Others

The recommendation is not to change the current specifications unless the spec breaks the systems.

Please share your views.

| Company | Views |
| --- | --- |
| NTT DOCOMO | Alt 1-1. Current spec works. No need to update in maintenance phase. |
| LG Electronics | Alt 1-1. According to agreement made in [99-NR-08], Alt 1-1 is already agreement. Agreements: - For determination of the number of coded bits, the determination of coded modulation symbols of HARQ-ACK with UL-SCH in Rel-15 NR is a baseline. - (Working assumption) The number of coded modulation symbols per layer for 2nd SCI is determined as follows. - QSCI2'=minOSCI2+LSCI2∙βoffsetSCI2∙l=0NsymbolPSSCH-1MscSCI2(l)r=0CSL-SCH-1Kr, αl=0NsymbolPSSCH-1MscSCI2(l)+γ - OSCI2 is the number of the 2nd SCI bits - LSCI2 is the number of CRC bits for 2nd SCI, LSCI2 value is FFS - βoffsetSCI2 is indicated by the corresponding 1st SCI. - α is (pre-)configured per resource pool. - NsymbolPSSCH is the number of allocated symbols for the PSSCH except AGC symbol. - MscSCI2(l) is the number of REs that can be used for transmission of the 2nd SCI. - γ is determined to ensure that there is no remaining RE in the RB having the last coded symbol of the SCI 2 after mapping the SCI2. - Kr is the r-th code block size for SL-SCH of the PSSCH transmission. - CSL-SCH is the number of code blocks for SL-SCH of the PSSCH transmission. - The first symbol that can be used for 2nd SCI mapping is the first PSSCH DMRS symbol. - Note: This symbol is not the symbol for AGC training. However, since the first SL symbol in a slot is a copy of the second SL symbol in the slot, the first SL symbol may have REs for the 2nd SCI. In our perspective, we need to clarify that what is the first PSSCH DMRS if the first PSSCH DMRS symbol is punctured by PSCCH. For instance, if the number of PRB of PSCCH is the same as subchannel size, and a single subchannel is used for PSSCH transmission, whether the first PSSCH DMRS symbol is symbol position indicated by 1st SCI or first available PSSCH symbol. For instance, in the following figure, if DMRS symbol in symbol#1 is punctured, when the 2nd SCI mapping starts? If the indicated first DMRS symbol is correct, after rate-matching, 2nd SCI mapping will start from symbol#3. On the other hand, if the first available DMRS symbol is correct, 2nd SCI mapping will start from symbol#5. We prefer that the first PSSCH DMRS position is indicated one regardless of actual transmission. |
| Ericsson | In our understanding, there is no need to change the specification. In regard to that, we had the following agreement which is correctly captured in the spec. Agreement from from [99-NR-08]: - The first symbol that can be used for 2nd SCI mapping is the first PSSCH DMRS symbol. - Note: This symbol is not the symbol for AGC training. However, since the first SL symbol in a slot is a copy of the second SL symbol in the slot, the first SL symbol may have REs for the 2nd SCI. |
| Qualcomm | We agree with Alt 1-1. One issue that was brought up in contributions was what to do when the first signaled DMRS symbol is fully punctured by PSCCH. In this case, our interpretation is that the first available DMRS symbol is used. It would be good to clarify that this is the common understanding. |
| Apple | Alt. 1-1. We do not see strong motivation to change the current agreement and specifications. |
| Intel | OK to go with Alt1-1 Clarification is needed for the cases mentioned by LGE and Qualcomm |
| Nokia, NSB | Alt1-1; support clarifying the issue raised by LGE/QC |
| Sharp | Alt 1-1 |
| ZTE, Sanechips | Alt 1-1. To our understanding, the current specification implies the 2nd SCI is mapped from the first actually transmitted DMRS and this could work under typical setting. |
| CATT | Alt 1-1 Support to clarify the issues raised by LGE and Qualcomm. |
| OPPO | Alt 1-1, we do not see the need to change specification and RAN1 agreement, and it is also our understanding that the 2nd stage SCI starts from the first available DMRS symbol, as clearly captured in current specification. |
| vivo | Alt 1-1. We are OK to clarify the issue raised by LGE. |
| MediaTek | Alt 1-1. This seems a corner case. It can be up to Tx UE implementation to avoid such fully puncturing of PSSCH DMRS so that the first DMRS indicated by 1st SCI is same as the first available DMRS without any ambiguity. |
| Panasonic | Alt1-2. The 2nd stage SCI should be decoded before PSSCH decoding. The required reliability of 2nd stage SCI is higher than reliability of PSSCH with HARQ feedback. Therefore, 2nd stage SCI is useful to be located on near DMRS to improve the channel estimation performance and located on earlier symbols to decode 2nd stage SCI earlier. When the first PSSCH DMRS symbol is fully punctured by PSCCH, we need to clarify 2nd stage SCI is located on near “first available DMRS”. |
| NEC | Alt 1-1 to follow the agreements. Fine to clarify the issue raised by LGE. |
| Huawei, HiSilicon | Alt 1-1, under the condition that the DMRS issue mentioned by LG is fixed (and see below for our view). Or, if Alt 1-1 does not include DMRS issue, then Alt 1-3. The agreement is already that: The first symbol that can be used for 2nd SCI mapping is the first PSSCH DMRS symbol. ([99-NR-08]) However, the current working assumption on PSSCH DMRS leads to ambiguity on what is the first PSSCH DMRS symbol when PSCCH and PSSCH are purely TDM’d, because the first symbol of the current patterns is occupied by PSCCH. For subCH size < 20 PRB, subject to WA on 20PRB subCH size restriction for PSSCH DRMS mapping, the “first PSSCH DMRS symbol” in current 38.211 DMRS table cannot be mapped. On top of this, the result is that the agreement to have 2, 3, or 4 DMRS symbols is violated in such cases due to the loss or absence of the first DMRS symbol. The DMRS patterns need to be fixed by ensuring that the first PSSCH DMRS symbol according to SCI is in fact the first real PSSCH DMRS symbol, including in cases of TDM and in PRBs < 20. This issue may be better handled after 9F. |
| Samsung | We prefer Alt 1-1 as the current specifications in order to simplify not only but also the implementation for Tx and Rx. |

Summary

Considering the views from companies, we can agree with Alt 1-1.

Proposal

The 2nd SCI can be mapped from the first PSSCH DMRS symbol.

- No spec change is needed.

Issue B-2.

One contribution [vivo] proposes to use the symbols before the first symbol of PSSCH DMRS as below.

- If after the last PSSCH symbol there are still 2nd SCI REs left, these remaining REs are mapped from the starting symbol of PSSCH (excluding the AGC symbol).

Please share your views (whether to support or not) and reason.

| Company | Views |
| --- | --- |
| NTT DOCOMO | Not support. It is unclear why UE transmits a TB with parameters which lead to this a bit strange situation. |
| LG Electronics | We do not need such mapping method. In our understanding, such case needs to be avoided by setting suitable value of alpha. In such cases, the number of REs for SL-SCH mapping is much smaller than the number of REs for 2nd SCI. |
| Ericsson | No need to support such mapping. Please see our reply to Issue B-1. |
| Qualcomm | No need to support. UE can select transmission parameters to avoid this case. |
| Apple | Such mapping is not supported. Overall, we think this situation (i.e., 2nd SCI REs left after the last PSSCH symbol) should be avoided, probably by configuring a proper value of alpha. |
| Intel | In our opinion this situation may only happen if the value of α in formula for calculating the 2nd stage resources (8.4.4 of 38.212) is not properly configured. Thus, it does not have an impact on a properly configured system. |
| Nokia, NSB | Not supported |
| Sharp | Not supported |
| ZTE, Sanechips | Not support, refer to our response to issue B-1 |
| CATT | Not supported. This case can be avoided by proper configuration and UE implementation.. |
| OPPO | Not needed, this problem should be avoided by proper (pre-)configuration and parameters selection. |
| vivo | Let’s have some clarification here. We also agree that, in general with proper value of alpha, the 2nd SCI RE will not be mapped beyond the last PSSCH symbol. The problem we found is that, in some case the first PSSCH DMRS cannot transmitted (e.g., the BW of the PSSCH is less than 20PRBs), and according to the spec, the 2nd SCI is mapped from the actual transmitted DMRS. In this case, even a good value of alpha does not help. This example is shown below, where the available number of symbols for PSSCH is 6, and DMRS pattern is indicated as <1, 5>. Only one subchannel is allocated to PSSCH, and the subchannel size is less that 20PRBs. As the 1st DMRS is not transmitted, there is not enough RE for mapping 2nd SCI as the mapping is started from OFDM symbol-5. This problem would be severe if PSFCH is present. We think this issue should be addressed. |
| MediaTek | Not necessary. The proper configuration can avoid such corner case. |
| Panasonic | When Alt 1-2 in issue B-1 is supported to 2nd stage SCI is located on earlier symbols, this issue is not happened. When Alt 1-1 in issue B-1 is supported, we are open to support this. When PSSCH length is 6 symbols, DMRS of PSSCH location is 1st and 5th symbol and 1st DMRS is fully punctured by PSCCH, it might be used. |
| NEC | Not supported, this situation could be avoided by proper (pre-) configuration. Otherwise, RE number for PSSCH will be very limited, especially in the case PSCCH occupy the whole subchannel. |
| Huawei, HiSilicon | Refer to issue B-1. |
| Samsung | We do not think it is necessary. |

Summary

Considering the views from companies, we can agree not to support.

Proposed conclusion

Not support that f after the last PSSCH symbol there are still 2nd SCI REs left, these remaining REs are mapped from the starting symbol of PSSCH (excluding the AGC symbol).

- No spec change is needed.

Issue B-3.

In Appendix 2, the rate matching of 2nd SCI is captured. Two contributions [Qualcomm], [Sharp] propose to consider the number of resources used for PSCCH when calculating the maximum limit MscSCI2(l) of the 2nd SCI. It means to replace MscSCI2(l) = MscPSSCH(l) - MscDMRS(l) - MscPT-RS(l) with MscSCI2(l) = MscPSSCH(l) - MscDMRS(l) - MscPT-RS(l) - MscPSCCH(l), where MscPSCCH(l) denotes the number of subcarriers in OFDM symbol l that carries PSCCH.

Please share your views (whether to support or not) and reason.

| Company | Views |
| --- | --- |
| NTT DOCOMO | Support We prefer to update definition of MscSCI2(l) as ‘the number of subcarriers in OFDM symbol l that is available for the PSSCH transmission’ since we think it is the original intention of the formula. But the above proposal is fine as well. |
| LG Electronics | We support FL’s proposal. |
| Ericsson | We are fine with the proposed change. |
| Qualcomm | We agree with the proposal. |
| Apple | Support |
| Intel | We agree to enable proper calculation of the available 2nd stage PSCCH resources, the REs occupied by the 1st stage PSCCH need to be excluded. |
| Nokia, NSB | No strong view. Actually I had assumed that MscPSSCH(l) excludes the PSCCH subcarriers already, but I agree that it is not really clear. |
| Sharp | We support FL’s proposal. |
| ZTE, Sanechips | Support |
| CATT | Support |
| OPPO | Support the change. |
| vivo | OK |
| MediaTek | Fine for the proposal with the further clarification on MscPSCCH(l) as the number of subcarriers in OFDM symbol l that carries PSCCH and PSCCH-DMRS. Besides, the definition of QmSCI2 is also missing in the spec. |
| Panasonic | We support the proposal. |
| NEC | Support |
| Huawei, HiSilicon | Agree. PSCCH resource is configured per RP and therefore is known to UE before transmitting/receiving SL. |
| Samsung | Agree |

Summary

Considering the views from companies, we can agree to modify the 2nd SCI rate matching by considering the 1st SCI mapping.

Proposal

In calculation of Q_SCI2 of clause 8.4.4 of TS38.212, the following modification is adopted.

- MscSCI2(l) = MscPSSCH(l) - MscDMRS(l) - MscPT-RS(l) - MscPSCCH(l)

- MscPSCCH(l) denotes the number of subcarriers in OFDM symbol l that carries PSCCH.

C. Number of PSCCH RBs when sub-channel size is less than 20 PRBs

The current specifications TS38.214 has the following:

### 8.2.2 PSSCH DM-RS transmission procedure The UE selects the DM-RS time domain pattern out of the patterns configured using the higher layer parameter TimePatternPsschDmrs for the resource pool on which the PSSCH is to be transmitted. If more than one DM-RS time domain pattern is configured, the selected pattern is indicated by the “DMRS pattern” field in the SCI format 0-1 associated with the PSSCH transmission. If PSSCH DMRS and PSCCH are mapped to the same OFDM symbol, then this mapping within a single sub-channel is only supported if higher layer parameter subchannelsize >= 20, i.e. the sub-channel size is at least 20 PRBs.

Regarding the yellow-highlighted part, UE behavior seems unclear in the case of the part.

One contribution [LGE] proposes to remove the above yellow-highlighted part.

One contribution [Qualcomm] proposes to add the restriction on the (pre)-configuration not to allow the case that the number of PRBs for PSCCH is less than the subchannel size if subchannel size < 20.

- Alt 1. Remove the yellow-highlighted part above.

- Alt 2. Introduce a restriction on (pre-)configuration not to allow the case that the number of PRBs for PSCCH is less than the subchannel size if subchannel size < 20.

- Alt 3. Other(s), if any.

Please share your views and reasons.

| Company | Views |
| --- | --- |
| NTT DOCOMO | Alt 3: No spec update. Current spec is clear in our understanding as: - If sub-channel size is less than 20 PRBs, PSCCH and PSSCH DM-RS is not FDMed within the sub-channel (above yellow part) - PSSCH DM-RS is mapped on each PSSCH PRB (38.211 - 8.4.1.1.2 (referring 6.4.1.1.3)) - PSSCH is not mapped on PSCCH PRB (38.213 - 8.1.2.2) To achieve above three bullets, PSCCH size shall be sub-channel size when sub-channel size is less than 20 PRBs. Situation mentioned by LGE shall be avoided by (pre-)configuration. But if clarification as Alt 2 is preferable, we are fine with the direction as well. Note that, if no. of SL symbols is larger than 8 and only 2-symbol DM-RS is (pre-)configured, PSCCH is not overlapped with PSSCH DM-RS in the same symbol. Alt 2 should be updated a bit. |
| LG Electronics | Our 1st preference is Alt 1. 2nd preference is Alt 3 which is that the UE expects that PSSCH DMRS is not transmitted in all the allocated sub-channels during PSCCH symbol duration if the number of PRBs for PSCCH is less than the subchannel size if subchannel size <20. In other words, if subchannel size <20 and if the number of PRBs for PSCCH is less than the subchannel size, DMRS pattern that results FDM between PSCCH and PSSCH DMRS is not used. If subchannel size>20 or if the number of PRBs for PSCCH is the same as subchannel size, DMRS pattern that results FDM between PSCCH and PSSCH DMRS can be used. |
| Ericsson | We don’t think any update is necessary in this regard. So we support Alt. 3 i.e. no update to the spec. |
| Qualcomm | We support Alt 2. As Docomo mentioned, it is the simple way to implement the highlighted text. |
| Apple | From the current specifications, if the sub-channel size is smaller than 20 PRBs and a single sub-channel is scheduled, then the PRBs beyond PSCCH region on a PSCCH symbol are not used for PSSCH DRMS, even if PSSCH DMRS time-domain location is set to that symbol (i.e., Table 8.4.1.1.2-1 of TS 38.2111). Hence, we do not need to update the specifications. |
| Intel | If subchannel size is large e.g. 20PRBs and PSCCH bandwidth is less e.g. 10 PRBs then PSSCH demodulation in symbols of the subchannel carrying PSCCH may be problematic due to lack of DMRS. One way to solve it is to force PSCCH to be 20 PRBs. Another way is to enable FDM of PSCCH and PSSCH DMRS within subchannel. First approach does not bring any benefit for PSSCH. Second approach may help PSSCH demodulation performance and thus Alt.1 is preferred. |
| Nokia, NSB | Alt 3 – no update. We don’t see a critical issue with the spec. |
| Sharp | We support Alt 3. |
| ZTE, Sanechips | Alt 2 or no spec. change. |
| CATT | Ok for Alt 2 or Alt 3 Since PSCCH bandwidth and sub-channel size are resource pool configuration parameters, proper configuration can avoid this issue. We are also fine with clear capture the restriction in spec |
| OPPO | No spec change. PSCCH size<subchannel size<20 can be avoided by (pre-)configuration, even this happens, some PSSCH PRBs may not have DMRS, but DMRS on other PRBs or symbols can be used for channel estimation, there is no need to optimize. |
| vivo | Alt 3 or Alt 2. We are OK to introduce a restriction (Alt 2) but this restriction should be captured in RRC description, not in RAN1. |
| MediaTek | Alt3. It seems clear that such multiplexing with less than 20RBs is not supported, i.e., PSCCH will occupy the whole sub-channel size of the sub-channel in this case. |
| Panasonic | We support Alt2. The PSCCH is (pre-)configured to have the same size as a sub-channel when sub-channel size is smaller than 20 PRBs to reduce channel estimation complexity. |
| NEC | Alt 2. Since current spec sentence was aligned with past agreements, we're fine to add restrictions but not to delete it. |
| Huawei, HiSilicon | Alt 3, but note that some resolution of the underlying issue is needed: As explained in Issue B-1, this 20PRB subchannel size restriction affects DMRS pattern (incl. number of DMRS patterns.), which further affect how 2nd-stage SCI is mapped. It does not make sense to mandate PSCCH PRBs = subchannel size (alt 2), which would always result in TDM’d PSCCH and PSSCH when allocated number of subCH = 1, and this could result in only one DMRS symbol in the system when DMRS = {2} is indicated in SCI 1-A as the first DMRS is lost to PSCCH. This would violate the agreement on having 2, 3, or 4 PSSCH DMRS symbols. The DMRS patterns need to be fixed by ensuring that - The first PSSCH DMRS symbol according to SCI is in fact the first real PSSCH DMRS symbol; and - The number of PSSCH DMRS symbols according to SCI is in fact the number of symbols containing DMRS including in cases of TDM between PSCCH/PSSCH, and in subchannel PRBs < 20. |
| Samsung | We support Alt 1. If it is confusing and no implementation issue exist, then we can remove. |

Summary

Based on the companies’ views, RAN1 needs to clarify the specifications.

If PSSCH DMRS and PSCCH are mapped to the same OFDM symbol, then this mapping within a single sub-channel is only supported if higher layer parameter subchannelsize >= 20, i.e. the sub-channel size is at least 20 PRBs.

There could be interpretations on the above specifications.

- Alt 1. Support Case A, NOT support Case B, and NOT support Case C.

- Alt 2. Support Case A, NOT support Case B, and support Case C.

Proposal

RAN1 discusses what alternative is the common understanding.

# Reference

- R1-2005292	Remaining details on physical layer structure for the sidelink	FUTUREWEI

- R1-2005307	Remaining issues of NR sidelink physical layer structure	ZTE, Sanechips

- R1-2005338	Remaining issues on physical layer structure for NR sidelink	vivo

- R1-2005646	Discussion on sidelink physical layer structure	MediaTek Inc.

- R1-2005667	Remaining issues on physical layer structure for NR sidelink	CATT

- R1-2005740	Discussion on essential corrections in physical layer structure	LG Electronics

- R1-2005761	TP on 1st symbol duplication for AGC	NEC

- R1-2005786	Remaining issues of V2X PHY layer structure	Mitsubishi Electric RCE

- R1-2005796	Remaining details of sidelink physical layer structure	Huawei, HiSilicon

- R1-2005846	Remaining opens for NR-V2X sidelink physical layer structure	Intel Corporation

- R1-2005997	Remaining issues on physical structure for NR sidelink	OPPO

- R1-2006074	Remaining issues on PHY structure for NR V2X	InterDigital, Inc.

- R1-2006099	On Physical Layer Structures for NR Sidelink	Samsung

- R1-2006254	Remaining issues for sidelink physical layer structure	Spreadtrum Communications

- R1-2006433	TPs related to PHY structures	Ericsson

- R1-2006484	On Remaining Issues of Sidelink Physical Layer Structure	Apple

- R1-2006535	Remaining issue on physical layer structure for sidelink in NR V2X	Panasonic Corporation

- R1-2006557	Remaining issues on physical layer structure for NR sidelink	Sharp

- R1-2006584	Remaining issues on sidelink physical layer structure on NR V2X	ASUSTeK

- R1-2006693	Maintenance for sidelink physical layer structure	NTT DOCOMO, INC.

- R1-2006768	Sidelink Physical Layer Structure	Qualcomm Incorporated

# Appendix 1. 2nd SCI mapping in TS38.211

#### 8.3.1.5 Mapping to virtual resource blocks For each of the antenna ports used for transmission of the PSSCH, the block of complex-valued symbols z(p)0, …, z(p)(Msymbap-1) shall be multiplied with the amplitude scaling factor βDMRSPSSCH in order to conform to the transmit power specified in [5, TS 38.213] and mapped to resource elements (k',l)p,μ in the virtual resource blocks assigned for transmission, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission.- The mapping operation shall be done in two steps: - first, the complex-valued symbols corresponding to the bit for the 2nd-stage SCI in increasing order of first the index k' over the assigned virtual resource blocks and then the index l, starting a the first PSSCH symbol carrying an associated DM-RS and meeting all of the following criteria: - the corresponding resource elements in the corresponding physical resource blocks are not used for transmission of the associated DM-RS, PT-RS, or PSCCH; - secondly, the complex-valued modulation symbols not corresponding to the 2nd -stage SCI shall be in in increasing order of first the index k' over the assigned virtual resource blocks, and then the index l with the starting position given by [6, TS 38.214] and meeting all of the following criteria: - the resource elements are not used for 2nd-stage SCI in the first step - the corresponding resource elements in the corresponding physical resource blocks are not used for transmission of the associated DM-RS, PT-RS, CSI-RS, or PSCCH; The resource elements used for the PSSCH in the first OFDM symbol in the mapping operation above shall be duplicated in the OFDM symbol immediately preceding the first OFDM symbol in the mapping.

# Appendix 2. 2nd SCI rate matching length in TS38.212

### 8.4.4 Rate Matching For 2nd-stage SCI transmission on PSSCH with SL-SCH, the number of coded modulation symbols generated for 2nd-stage SCI transmission prior to duplication for the 2nd layer if present, denoted as QSCI2', is determined as follows: QSCI2'=minOSCI2+LSCI2∙βoffsetSCI2QmSCI2∙R, αl=0NsymbolPSSCH-1MscSCI2(l)+γ where - OSCI2 is the number of the 2nd-stage SCI bits - LSCI2 is the number of CRC bits for the 2nd-stage SCI, which is 24 bits. - βoffsetSCI2 is indicated in the corresponding 1st-stage SCI. - MscPSSCH(l) is the scheduled bandwidth of PSSCH transmission, expressed as a number of subcarriers; - MscDMRS(l) is the number of subcarriers in OFDM symbol l that carries DMRS, in the PSSCH transmission. - MscPT-RS(l) is the number of subcarriers in OFDM symbol l that carries PT-RS, in the PSSCH transmission. - MscSCI2(l) is the number of resource elements that can be used for transmission of the 2nd-stage SCI in OFDM symbol l, for l=0,1,2⋯,NsymbolPSSCH-1 and for NsymbolPSSCH=Nsymbsh-NsymbPSFCH, in PSSCH transmission, where NsymbshNsymbslot = sl-lengthSymbols - 2, where sl-lengthSymbols is the number of sidelink symbols within the slot provided by higher layers as defined in [6, TS 38.214]. If higher layer parameter sl-PSFCH-Period = 2 or 4, NsymbPSFCH = 3 if "PSFCH overhead indication" field of SCI format 1-A indicates "1", and NsymbPSFCH = 0 otherwise. If higher layer parameter sl-PSFCH-Period = 0, NsymbPSFCH=0. If higher layer parameter sl-PSFCH-Period is 1, NsymbPSFCH=3. - MscSCI2(l) = MscPSSCH(l) - MscDMRS(l) - MscPT-RS(l) - γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. - R is the coding rate as indicated by "Modulation and coding scheme" field in SCI format 1-A. - α is configured by higher layer parameter sl-Scaling.