3GPP TSG RAN WG1 #102-e Meeting	R1-200xxxx

e-Meeting, August 17th – 28th, 2020

Title:	[Draft] Reply LS on maximum data rate for NR sidelink

Response to:	LS on UE capability (R2-2004087)

Release:	Rel-16

Work Item:	5G_V2X_NRSL

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

Name:	Jeongho Yeo

- E-mail Address:	jeongho7.yeo@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: 	-

1. Overall Description:

In RAN1#102-e, RAN1 reached the following agreements to define the maximum data rate for NR sidelink.

Agreements: - The supported max data rate for SL is based on NR’s formula (based on BW, layer..) in Clause 4.1.2 of TS38.306 - The supported max data rate for SL is dependent on UE capability (which is defined either per band or per band combinations or a combination thereof, to be decided by 08/20). - Defined per band combinations - RAN1 further discusses Qm, f, OH values. Agreements: - For max data rate for SL Tx (or Rx), v_layers is the maximum number of supported layers for SL Tx (or Rx) according to UE capability. Agreements: - For max data rate for SL, Qm is determined: - between 64QAM and 256QAM (based on the existing UE capability) Agreements: - For max data rate for SL, N_PRB^BW is the maximum possible RB allocation in bandwidth BW for PSSCH, where BW is the UE supported maximum bandwidth in the given band or band combination. Agreements: - For max data rate for SL, OH value is determined as [0.23] for FR1 and [0.25] for FR2.

2. Actions:

To RAN2

RAN1 respectfully asks RAN2 to take the above information into account in their further work

3. Date of Next Meetings:

TSG-RAN WG1 Meeting #103-e 	October 26 – November 13, 2020	e-meeting

TSG-RAN WG1 Meeting #104 	March 1 – 5, 2021			Athens, Greece