#3GPP TSG RAN WG1 #102-e						R1-2006957

e-Meeting, August 17th – 28th, 2020

______________________________________________________________________Agenda item: 7.2.4.5

Source: Moderator (LG Electronics)

Title: Feature lead summary#1 for AI 7.2.4.5 Physical layer procedures for sidelink

Document for: Discussion and decision

- Potential issues for email discussions

- Power control

  - Issue 1-1: TP to clarify how to perform open-loop power control for PSSCH

  - Issue 1-2: TP for that total sidelink transmit power is the same in the symbols used for actual PSCCH/PSSCH transmissions in a slot

- Prioritization

  - Issue 2-1: Tie-break of the same priority

  - Issue 2-2: How to handle remaining UL/SL prioritization cases

- HARQ operation

  - Issue 3-1: Details of indicating groupcast HARQ feedback option

  - Issue 3-2: Capturing PSFCH reception behavior in the specifications

  - Issue 3-3: Exact location of PSFCH slots in the time domain

- Sidelink CSI

  - Issue 4-1: Assumptions for the CSI reference resource

Company input on the identification of email discussion topics:

| Company | Prioritized issues | Comments |
| --- | --- | --- |
| LGE | Thread #1 {3-1, 3-2, 3-3} Thread #2 {4-1, 2-2(2nd priority)} | We propose to focus on the HARQ operation and Sidelink CSI where more essential corrections need to be made for the spec completeness. If the load to the proposed Thread #2 is low, some other topics like Issue 2-2 can be discussed with 2nd priority in the same thread. TPs for the clarification in power control (Issue 1-1 and 1-2) may be directly discussed during the TP phase. |

Proposed email discussion topics:

| Company | Comment |
| --- | --- |

- Power control

Agreements made in RAN1#97:

- For the SL open-loop power control, a UE can be configured to use DL pathloss (between TX UE and gNB) only, SL pathloss (between TX UE and RX UE) only, or both DL pathloss and SL pathloss.

- When the SL open-loop power control is configured to use both DL pathloss and SL pathloss.

  - The minimum of the power values given by open-loop power control based on DL pathloss and the open-loop power control based on SL pathloss is taken.

    - (Working assumption) P0 and alpha values are separately (pre-)configured for DL pathloss and SL pathloss.

Working assumption made in RAN1#98bis

- For the power limited case in supporting simultaneous sidelink and uplink transmissions (SL carrier is different from UL carrier),

  - If sidelink transmission is prioritized over uplink transmission, the UE shall adjust the uplink transmission power before the start of the transmission such that its total transmission power does not exceed  on any overlapped portion. In this case, calculation of the adjustment to the uplink transmission power is not specified.

  - If uplink transmission is prioritized over sidelink transmission, the UE shall adjust the sidelink transmission power before the start of the transmission such that its total transmission power does not exceed  on any overlapped portion. In this case, calculation of the adjustment to the sidelink transmission power is not specified.

  - Total sidelink transmit power is the same in the symbols used for actual PSCCH/PSSCH transmissions in a slot in case of simultaneous transmission of sidelink and uplink

    - PSCCH/PSSCH transmissions can be dropped in some symbols when there are uplink transmissions with higher priority and the UE cannot keep the same sidelink transmission power in the symbols.

      - Selection of the dropped symbols is up to UE implementation where the dropped symbols should include the overlapping symbols.

  - If the simultaneous transmission of sidelink and uplink is beyond the UE capability, the one not prioritized can be dropped.

  - FFS: when to prioritize which transmission

  - FFS: how to address UE processing time

  - FFS: whether there is a case of dropping some symbols of uplink transmissions

Whether/how to address RF transient period is up to RAN4.

- Issue 1-1: Whether or how to modify description for open-loop power control for PSSCH in TS38.213

  - Three cases of open-loop power control for PSSCH are not clearly described in the latest version of TS38.213

    - Support: [ZTE,1] [Huawei,6] [OPPO,8]

      - Separate following cases

        - Case 1: Only p0-DL-PSCCHPSSCH is provided

        - Case 2: Only p0-SL-PSCCHPSSCH is provided

        - Case 3: Both p0-DL-PSCCHPSSCH and p0-SL-PSCCHPSSCH are provided

- Issue 1-2: Whether or how to specify that PSCCH/PSSCH transmissions can be dropped in some symbols when the UE cannot keep the same sidelink transmission power in the symbols

  - Specify UE behavior to avoid total sidelink power cannot be kept constant in TS38.213

    - Support [LG,5] [Spreadtrum,11]

    - Rationale: Current description allows total sidelink transmission power can be changed in the middle of PSCCH/PSSCH transmission.

- Others

  - Further consideration on whether TPC for PUCCH carrying SL HARQ-ACK reports is introduced or not in DCI format 3_0 [ETRI,12]

  - Further consideration on which RS resource is used for DL pahtloss estimation when beam failure recover procedure or T310 is running [ASUSTek,17]

- Prioritization

- Issue 2-1: Tie-break of the same priority

  - PSFCH TX and PSFCH RX

    - Up to UE implementation: [LG,5]

    - For groupcast HARQ feedback Option 1, NACK transmission is deprioritized over PSFCH reception [Futurewei,3]

    - For unicast/groupcast HARQ feedback Option 2, NACK transmission is prioritized over PSFCH reception [Futurewei,3]

    - Groupcast with HARQ feedback Option 1 is prioritized: [CATT,4]

    - Based on additional rule by using HARQ status, cast type, and HARQ feedback option): [InterDigital,10]

    - NACK feedback of unicast/groupcast with HARQ feedback Option 2 is deprioritized:

  - For prioritization between PUCCH carrying SL HARQ reporting and SL-TX with the same priority,

    - Up to UE implementation

      - Support: [ZTE,1] [LG,5]

- Issue 2-2: How to handle remaining UL/SL prioritization cases

  - When PUSCH carrying SL HARQ reporting overlaps with SL transmissions,

    - Reuse UL/SL prioritization rule for PUSCH without SL HARQ reporting

      - Support: [LG,5]

    - SL TX is prioritized when the SL TX is prioritized over both PUSCH with SL HARQ reporting and SL HARQ reporting. Otherwise, UL TX is prioritized.

      - Support: [Huawei,6] [Apple,14] [NTT,18]

  - For prioritization between PSCCH/PSSCH and PUCCH/PUSCH without UL-SCH/SRS

    - Reuse UL/SL prioritization rule for PSFCH and UL transmission except for PUCCH carrying SL HARQ reporting

      - Support: [ZTE,1] [LG,5]

  - For prioritization between PSFCH RX or S-SSB RX and UL-TX

    - Reuse UL/SL prioritization rule for PSFCH and UL transmission except for PUCCH carrying SL HARQ reporting

      - Support: [ZTE,1] [LG,5]

  - For prioritization between SL RX other than PSFCH/S-SSB and UL-TX,

    - If priority of SL RX is known to the UE (subject to UE processing time),

      - Reuse the prioritization rule for the collision between PSFCH and UL transmission(s)

        - Support: [LG,5]

    - Else

      - Up to UE implementation

        - Support: [LG,5]

  - For prioritization between PUCCH carrying SL HARQ-ACK reports and PRACH/Msg3,

    - PRACH or Msg3 is prioritized [Samsung,9]

  - Further consideration on how to treat MsgA for UL/SL prioritization

    - Support: [Apple,14]

- Others

  - Consideration on how to support power sharing between UL transmission and SL transmission for dual connectivity [LG,5]

    - Reuse power control for NE-DC and EN-DC for simultaneous transmission of SL transmission and UL transmission on different carriers

    - NR-DC with Semi-static-mode1 or Semi-static-mode2 is supported for NR sidelink

- HARQ operation

- Issue 3-1: Details of indicating groupcast HARQ feedback option if non-distance based groupcast HARQ feedback Option 1 is supported

  - Cast type indicator in SCI format 2-A is used to indicate groupcast HARQ feedback option

    - Support: [ZTE,1] [LG,5] [Intel,7] [OPPO,8] [Ericsson,13] [Panasonic,15] [NTT,18]

  - Additional 1-bit indicator in SCI format 2-A is used to indicate groupcast HARQ feedback option

    - Support: [CATT,4]

  - Communication range requirement field in SCI format 2-B is used to indicate distance-based HARQ-ACK feedback is disabled

    - Support: [Intel,7] [Qualcomm,19]

- Issue 3-2: Capturing PSFCH reception behavior in the specifications

  - Whether or how to capture UE procedure for receiving HARQ-ACK feedback on sidelink in TS38.213

    - Depending on cast type, GC HARQ feedback Options, PHY layer determines HARQ-ACK status to report to MAC layer [LG,5] [InterDigital,10]

      - Rationale:

        - MAC will define the UE behavior such as commencing retransmission based on the SL HARQ status reported from PHY

        - MAC will use the absence of HARQ feedback (DTX) for RLF declaration.

      - For unicast and groupcast with HARQ feedback Option 2, what is reported to MAC layer when UE determines the absence of the expected PSFCH

        - Reporting nothing: [LG,5]

        - Reporting DTX state: [vivo,2] [InterDigital,10]

- Issue 3-3: Exact location of PSFCH slots in the time domain in a given resource pool

  - What is the first PSFCH slot

    - First SL slot in a resource pool

      - Support: [ZTE,1] [vivo,2] [Futurewei,3] [LG,5] [OPPO,8] [Panasonic,15]

    - (periodPSFCHresource - 1)-th slot in a resource pool

      - Support: [Huawei,6] [NTT,18]

    - The first PSFCH slot is determined so that the last slot in a resource pool is a PSFCH slot

      - Support: [Ericsson,13]

  - Whether the number of slots in a resource pool is always a multiple of periodPSFCHresource

    - Support: [LG,5] [Sharp,16] [NTT,18]

    - Not support: [ZTE,1] [Panasonic,15]

- Others

  - Further consideration on PSFCH transmission with a consideration of HARQ RTT or a TB [vivo,2] [Intel,7]

- Sidelink CSI

- Issue 4-1: Assumptions for the CSI reference resource

  - AGC overhead

    - First symbol is occupied by duplicated symbol of 2nd symbol within a SL slot: [Huawei,6]

  - CP length and SCS

    - Same as those of SL BWP: [vivo,2] [LG,5] [Huawei,6] [Intel,7] [Samsung,9] [Apple,14]

  - Redundancy value

    - RV0: [vivo,2] [LG,5] [Huawei,6] [Intel,7] [Apple,14]

    - Actual RV associated with CSI reporting triggering: [Samsung,9]

  - PSCCH overhead

    - Predefined overhead: [vivo,2]

    - (Pre)configured overhead: [LG,5] [Huawei,6] [Intel,7] [Samsung,9] [Apple,14]

  - PSSCH symbol duration

    - Predefined value: [vivo,2]

    - (Pre)configured overhead: [LG,5] [Huawei,6] [Intel,7] [Samsung,9] [Apple,14]

      - PSFCH symbol overhead

        - No PSFCH symbol: [vivo,2] [LG,5]

        - PSFCH overhead if PSFCH resource is configured: [Huawei,6] [Apple,14]

        - Actual PSFCH overhead associated with CSI reporting triggering: [Samsung,9]

  - CSI-RS overhead

    - No CSI-RS overhead: [vivo,2] [LG,5] [Huawei,6] [Intel,7] [Apple,14]

    - Actual overhead associated with CSI reporting triggering: [Samsung,9]

  - 2nd-stage SCI overhead

    - No 2nd-stage SCI overhead is used: [LG,5] [Huawei,6] [Intel,7] [Apple,14]

    - Lowest overhead per (pre)configuration is used: [Intel,7]

    - Actual overhead associated with CSI reporting triggering: [Samsung,9]

  - Assumption on the number of DMRS symbol

    - Actual overhead associated with CSI reporting triggering: [vivo,2] [Huawei,6] [Samsung,9]

    - Lowest density per (pre)configuration: [LG,5] [Intel,7] [Apple,14]

- Other topics

- Further consideration on how to complete the higher layer parameters for Rel-16 sidelink

  - [NTT,18]

    - Clarify that sl-PSFCH-HopID-r16 is used instead of hoppingId for PUCCH.

    - PSFCH sequence is generated as pucch-GroupHopping = neither

- Capture how a UE determines SCI fields in SCI format 1-A [Intel,7]

References

- R1-2005320	Remaining issues in PHY procedures for Rel-16 sidelink	ZTE, Sanechips

- R1-2005344	Remaining issues on physical layer procedure for NR sidelink	vivo

- R1-2005594	Remaining details on physical procedures for sidelink	FUTUREWEI

- R1-2005671	Remaining issues on physical layer procedures for NR V2X	CATT

- R1-2005745	Discussion on essential corrections in physical layer procedure	LG Electronics

- R1-2005801	Remaining details of physical layer procedures for sidelink	Huawei, HiSilicon

- R1-2005849	Remaining opens for NR-V2X sidelink physical layer procedures	Intel Corporation

- R1-2006000	Remaining issues of physical layer procedure for NR-V2X	OPPO

- R1-2006104	On Physical Layer Procedures for NR Sidelink	Samsung

- R1-2006179	Remaining issues on NR SL physical layer procedures	InterDigital, Inc.

- R1-2006265	Remaining issues on sidelink physical layer procedure	Spreadtrum Communications

- R1-2006354	Physical layer procedures for sidelink	ETRI

- R1-2006439	Open issues and corrections for SL Physical layer procedures	Ericsson

- R1-2006486	On Remaining Issues of Sidelink Physical Layer Procedures	Apple

- R1-2006536	Remaining issue on physical layer procedures for sidelink in NR V2X	Panasonic Corporation

- R1-2006561	Remaining issues on physical layer procedures for NR sidelink	Sharp

- R1-2006586	Remaining issues on sidelink physical layer procedure on NR V2X	ASUSTeK

- R1-2006696	Maintenance for on sidelink physical layer procedure	NTT DOCOMO, INC.

- R1-2006773	Physical layer procedures for sidelink	Qualcomm Incorporated