3GPP TSG RAN WG1 #102-e		R1-200xxxx

e-Meeting, Aug. 17th - 28th, 2020

Title: 	Remaining issues on PHY procedures for Rel-16 sidelink

Source: 	ZTE, Sanechips

Agenda item:	7.2.4.5

Document for:	Discussion and decision

  - Text proposal for TS 38.214

===========================<Start of change #x>=======================

- 8	Physical sidelink shared channel related procedures

A UE can be configured by higher layers with one or more sidelink resource pools. A sidelink resource pool can be for transmission of PSSCH, as described in Clause 8.1, or for reception of PSSCH, as described in Clause 8.3 and can be associated with either sidelink resource allocation mode 1 or sidelink resource allocation mode 2.

In the frequency domain, a sidelink resource pool consists of numSubchannel contiguous sub-channels. A sub-channel consists of subchannelsize contiguous PRBs, where numSubchannel and subchannelsize are higher layer parameters.

The set of slots that may belong to a sidelink resource pool is denoted by (t0SL,t1SL,⋯,tTmax-1SL) where

-	0≤tiSL<10240×2μ, 0≤i<Tmax,

-	the slot index is relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell or DFN 0,

-	the set includes all the slots except the following slots,

-	NS_SSB slots in which S-SS/PSBCH block (S-SSB) is configured,

-	NnonSL slots in each of which at least one of Y-th, (Y+1)-th, …, (Y+X-1)-th OFDM symbols are not semi-statically configured as UL as per the higher layer parameter TDD-UL-DL-ConfigCommon, where Y and X are set by the higher layer parameters sl-StartSymbol and sl-LengthSymbols, respectively.

-	The reserved slots which are determined by the following steps.

1)	the remaining slots excluding NS_SSB slots and NnonSL slots from the set of all the Tmax slots are denoted by (l0,l1,⋯,l(10240×2μ-NSSSB-NnonSL-1)) arranged in increasing order of slot index.

2)	a slot lr (0≤r<10240×2μ-NSSSB-NnonSL) belongs to the reserved slots if r=m∙(10240×2μ-NSSSB-NnonSL)Nreserved, here m=0,1,⋯,Nreserved-1 and Nreserved=10240×2μ-NSSSB-NnonSL mod Lbitmap where Lbitmap denotes the length of bitmap configured by higher layers.

-	The slots in the set are arranged in increasing order of slot index.

-	The UE determines the set of slots assigned to a sidelink resource pool, denoted by (t0RP, t1RP, …tM-1RP) , as follows:

-	a bitmap b0,b1,…,bLbitmap-1 associated with the resource pool is used where Lbitmap the length of the bitmap is configured by higher layers.

-	a slot tkSL (0≤k<10240×2μ-NSSSB-NnonSL-Nreserved) belongs to the slot pool if bk'=1 where k'=k mod Lbitmap.

-	The set of slots assigned to the sidelink resource pool are arranged in increasing order of slot index.

The UE determines the set of resource blocks assigned to a sidelink resource pool as follows:

-	The resource block pool consists of NPRB PRBs.

-	The sub-channel m for m=0,1,⋯,numSubchannel-1 consists of a set of nsubCHsize contiguous resource blocks with the physical resource block number nPRB=nsubCHRBstart+m∙nsubCHsize+j for j=0,1,⋯,nsubCHsize-1, where nsubCHRBstart and nsubCHsize are given by higher layer parameters sl_StartRB-Subchannel and sl-SubchannelSize, respectively

A UE is not expected to use the last NPRB mod nsubCHsize PRBs in the resource pool.

============================<End of change #x>=======================

  - Text proposal for TS 38.213

===========================<Start of change #y>=======================

## 16.3	UE procedure for reporting HARQ-ACK on sidelink

<Unchanged part is omitted>

A UE can be provided, by periodPSFCHresource, a number of slots in a resource pool for a period of PSFCH transmission occasion resources. If the number is zero, PSFCH transmissions from the UE in the resource pool are disabled.

A UE expects a slot tkRP (0≤k<M) has PSFCH transmission occasion resource if k mod NPSSCHPSFCH=0, where tkRP is defined in Clause 8 [6, TS 38.214], M is the number of slots which belong to the resource pool within a SFN/DFN period(10240 msec) according to Clause 8 [6, TS 38.214], and NPSSCHPSFCH is provided by periodPSFCHresource.

===========================<end of change #y>=======================