#3GPP TSG RAN WG1 #102-e						R1-20xxxxx

e-Meeting, August 17th – 28th, 2020

______________________________________________________________________Agenda item: 8.11.1

Source: Moderator (LG Electronics)

Title: FL summary#1 for AI 8.11.1 SL evaluation methodology update for power saving

Document for: Discussion and decision

- Email discussion

As per Chairman’s guideline, the following email discussion was allocated for AI 8.11.1. Please provide your view on the questions in Section 1.1/1.2/1.3/1.4 by 8/19 (Wednesday). Based on the collected view, I’ll make a set of proposals that will be discussed and finalized by 8/21. Note that other remaining assumptions, if any, will be discussed/finalized in the second phase of email discussion.

      - [102-e-NR-SL_enh-01] Email discussion/approval using the summary as a starting point, focusing on simulation assumptions – Seungmin (LGE)

- By 8/21 – Simulation assumptions, critical ones

- By 8/27 – Remaining simulation assumptions

# 1.1	Reference configuration for power consumption model

      - Q1: Do you agree that the number of SL symbols in a slot is 14 (including AGC and TX-RX switching period)?

| Company | Answer | Comment |
| --- | --- | --- |

      - Q2: Do you agree that the SCS for FR1 is 30kHz and the SCS for FR2 is 120kHz?

| Company | Answer | Comment |
| --- | --- | --- |

      - Q3: Do you agree that the system BW for power consumption model is 100MHz as in TR38.840?

| Company | Answer | Comment |
| --- | --- | --- |

      - Q4: Do you agree that the number of OFDM symbols for PSCCH (excluding AGC symbol) is 2?

| Company | Answer | Comment |
| --- | --- | --- |

      - Q5: Do you agree that the modulation order for power consumption model is 256QAM as in TR38.840?

| Company | Answer | Comment |
| --- | --- | --- |

      - Q6: Do you agree that the number of TX APs for power consumption model is 1 as in TR38.840?

| Company | Answer | Comment |
| --- | --- | --- |

      - Q7: Do you agree that the number of RX APs for power consumption model is 4 for FR1 or 2 for FR2 as in TR38.840?

| Company | Answer | Comment |
| --- | --- | --- |

      - Q8: Do you agree that TX power for power consumption model is {0dBm, 23dBm} for FR1 as in TR38.840? Do you agree that TX power for power consumption model is 0dBm for FR2?

| Company | Answer | Comment |
| --- | --- | --- |

# 1.2	Power consumption scaling for adaptation

      - Q1: Do you agree to support that X MHz power for RX is (0.4 +0.6*(X-20)/80)*100MHz power for RX?

| Company | Answer | Comment |
| --- | --- | --- |

      - Q2: Do you agree that no scaling is supported for SL BWP adaptation in TX perspective?

| Company | Answer | Comment |
| --- | --- | --- |

      - Q3: For antenna scaling in RX perspective, do you agree that 2Rx power is 0.7*4Rx power for FR1 and 1Rx power is 0.7*2Rx power for FR2?

| Company | Answer | Comment |
| --- | --- | --- |

      - Q4: For antenna scaling in TX perspective for FR1, do you agree that 2Tx power is 1.4*1Tx power at 0dBm and 1.2*1Tx power at 23dBm? Is the antenna scaling in TX perspective necessary for FR2?

| Company | Answer | Comment |
| --- | --- | --- |

      - Q5: Other power consumption scaling for adaptation (please specify)

| Company | Comment |
| --- | --- |

# 1.3	Power consumption level

      - Q1: Do you agree to reuse three states of “Sleep” specified in TR38.840 including transition time and energy consumption?

| Company | Answer | Comment |
| --- | --- | --- |

      - Q2: In “1st SCI/2nd SCI RX” state, a UE tries to decode 1st SCI on PSCCH and 2nd SCI on PSSCH, but the UE does not decode SL-SCH on PSSCH. In this case, what is the UE power consumption level?

- Q2-1: For power consumption level of “1st SCI/2nd SCI RX” in non-PSFCH-slot, which option is used?

  - Option 1: Same as power consumption level of “PDCCH-only” for same-slot scheduling

  - Option 2: Scaling factor Y * power consumption level of “PDCCH-only” for same-slot scheduling

    - Option 2-1: Y = 1.45

    - Option 2-2: Y= 2

  - Option 3: Scaling factor Y * power consumption level of “PSCCH/PSSCH RX”

    - Y is selected from [0.6 0.7 0.8 0.9]

| Company | Preferred option | Comment |
| --- | --- | --- |

- Q2-2: For power consumption level of “1st SCI/2nd SCI RX” in PSFCH-slot, which option is used?

  - Option 1: Same as power consumption level of “PDCCH-only” for same-slot scheduling

  - Option 2: Scaling factor Y * power consumption level of “PDCCH-only” for same-slot scheduling

    - Y = 0.85

  - Option 3: Scaling factor Y * power consumption level of “PSCCH/PSSCH RX”

    - Y is selected from [0.6 0.7 0.8 0.9]

| Company | Preferred option | Comment |
| --- | --- | --- |

      - Q3: In “PSCCH/PSSCH RX” state, a UE tries to decode 1st SCI on PSCCH and 2nd SCI on PSSCH, and SL-SCH on PSSCH. In this case, what is the UE power consumption level?

- Q3-1: For power consumption level of “PSCCH/PSSCH RX” in non-PSFCH-slot, do you agree that the power consumption level is the same as that of “PDCCH+PDSCH”?

| Company | Answer | Comment |
| --- | --- | --- |

- Q3-2: For power consumption level of “PSCCH/PSSCH RX” in PSFCH-slot, do you agree that the power consumption level is 0.9*power consumption level of “PDCCH+PDSCH”?

| Company | Answer | Comment |
| --- | --- | --- |

      - Q4: In “PSCCH/PSSCH TX” state, a UE transmits PSCCH and PSSCH. In this case, what is the UE power consumption level?

- Q4-1: For power consumption level of “PSCCH/PSSCH TX” in non-PSFCH-slot, do you agree that the power consumption level is the same as that of “UL” for long PUCCH or PUSCH?

| Company | Answer | Comment |
| --- | --- | --- |

- Q4-2: For power consumption level of “PSCCH/PSSCH TX” in PSFCH-slot, do you agree that the power consumption level is 0.8*power consumption level of “UL” for long PUCCH or PUSCH?

| Company | Answer | Comment |
| --- | --- | --- |

      - Q5: In “PSFCH TX” or “PSFCH RX” state, a UE transmits or receives PSFCH only in a slot, respectively. In this case, what is the UE power consumption level?

- Q5-1: For power consumption level of “PSFCH TX”, which option is used?

  - Option 1: Same as power consumption level of “UL” for short PUCCH

  - Option 2: Scaling factor Y*power consumption level of “UL” for long PUCCH or PUSCH

    - Option 2-1: Y=0.36

    - Option 2-2: Y= 0.2

  - Option 3: 72

| Company | Preferred option | Comment |
| --- | --- | --- |

- Q5-2: For power consumption level of “PSFCH RX”, which option is used?

  - Option 1: Same as power consumption level of “PDCCH-only” for same-slot scheduling

  - Option 2: Same as power consumption level of “PDCCH-only” for cross-slot scheduling

  - Option 3: 5 for FR1, 30 for FR2

| Company | Preferred option | Comment |
| --- | --- | --- |

      - Q6: In PSFCH-slot, a UE can (1) SCI decoding only, (2) SCI decoding and PSFCH RX, (3) SCI decoding and PSFCH TX, (4) PSCCH/PSSCH decoding, (5) PSSCH/PSSCH decoding and PSFCH RX, (6) PSCCH/PSSCH decoding and PSFCH TX, (7) PSCCH/PSSCH TX and PSFCH RX, or (8) PSSCH/PSSCH TX and PSFCH TX. What is the power consumption level of a combination of PSCCH/PSSCH/PSFCH operation?

- Q6-1: For power consumption level of a combination of “1st SCI/2nd SCI RX” and “PSFCH RX”, do you agree that the power consumption level is the same as that of “1st SCI+2nd SCI RX” in non-PSFCH-slot?

| Company | Answer | Comment |
| --- | --- | --- |

- Q6-2: For power consumption level of a combination of “PSCCH/PSSCH RX” and “PSFCH RX”, do you agree that the power consumption level is the same as that of “PSCCH/PSSCH RX” in non-PSFCH-slot?

| Company | Answer | Comment |
| --- | --- | --- |

- Q6-3: For power consumption level of a combination of “PSCCH/PSSCH TX” and “PSFCH TX”, do you agree that the power consumption level is the same as that of “UL (long PUCCH or PUSCH)”?

| Company | Answer | Comment |
| --- | --- | --- |

- Q6-4: For power consumption level of a combination of “1st SCI/2nd SCI RX” and “PSFCH TX”, do you agree that the power consumption level is a sum of power consumption level of “1st SCI/2nd SCI RX” in PSFCH-slot and power consumption level of “PSFCH TX”?

| Company | Answer | Comment |
| --- | --- | --- |

- Q6-5: For power consumption level of a combination of “PSCCH/PSSCH RX” and “PSFCH TX”, do you agree that the power consumption level is a sum of power consumption level of “PSCCH/PSSCH RX” in PSFCH-slot and power consumption level of “PSFCH TX”?

| Company | Answer | Comment |
| --- | --- | --- |

- Q6-6: For power consumption level of a combination of “PSCCH/PSSCH TX” and “PSFCH RX”, do you agree that the power consumption level is a sum of power consumption level of “PSCCH/PSSCH TX” in PSFCH-slot and power consumption level of “PSFCH RX”?

| Company | Answer | Comment |
| --- | --- | --- |

      - Q7: In “S-SSB RX”, a UE tries to detect S-SSB and decode PSBCH. In “S-SSB TX”, a UE transmits S-SSB in a slot. What is the power consumption level?

- Q7-1: For power consumption level of “S-SSB RX”, which option is used?

  - Option 1: Same as power consumption level of “SSB processing” with 1 SSB in a slot

  - Option 2: Same as power consumption level of “SSB processing” with 2 SSB in a slot

  - Option 3: Same as that of “1st SCI/2nd SCI RX” in non-PSFCH-slot

  - Option 4: 150

  - Option 5: 0.3

  - Option 6: 5

| Company | Preferred option | Comment |
| --- | --- | --- |

- Q7-2: For power consumption level of “S-SSB TX”, do you agree that the power consumption level is the same as power consumption level of “UL” for (long PUCCH or PUSCH)?

| Company | Answer | Comment |
| --- | --- | --- |

      - Q8: Depending on the Synch reference source, a UE performs synchronization process based on GNSS, SSB from gNB, or S-SSB from another UE. What is the power consumption level?

- Q8-1: For power consumption level of “GNSS-processing”, which option is used?

  - Option 1: 5

  - Option 2: 8

  - Option 3: 15

  - Option 4: 0.08*Power consumption level of PSCCH/PSSCH RX

| Company | Preferred option | Comment |
| --- | --- | --- |

- Q8-2: For power consumption level of “SSB-processing”, which option is used?

  - Option 1: Same as power consumption level of “SSB processing”

  - Option 2: 10

| Company | Preferred option | Comment |
| --- | --- | --- |

      - Q9: For power consumption level of “CSI-RS processing”, which option is used?

- Option 1: Same as that of “CSI-RS processing”

- Option 2: 150

- Option 3: 0.4 for FR1, 300 for FR2

- Option 4: No need to specify it

| Company | Preferred option | Comment |
| --- | --- | --- |

# 1.4	Other remaining assumptions (to be discussed/finalized by 8/27)

      - Q1: What additional assumptions (other than those in Section 1.1/1/2/1/3) are needed to discussed/finalized during the second phase of email discussion?

| Company | Comment |
| --- | --- |

- Appendix A - Summary of contributions

      - Power consumption model for Rel-17 NR sidelink

- Reference configuration for power consumption model

  - FR1

    - Number of SL symbols in a slot (including AGC and TX-RX switching periods)

      - 7: [1]

      - 14: [1], [9]

    - SCS

      - 30kHz: [1], [2], [3], [5], [9], [11], [12], [15], [16], [20]

    - System BW

      - 20MHz: [5], [11], [12], [16], [20]

      - 40MHz: [1], [3], [12], [15], [20]

      - 100MHz: [2], [9]

    - PSCCH

      - 2 symbols: [1], [3], [9], [11], [15]

      - 3 symbols: [2], [15], [20]

    - PSSCH

      - Modulation order:

        - 64QAM: [1], [3], [5], [11], [15], [20]

        - 256QAM: [2], [9], [15]

    - Number of TX APs

      - 1: [1], [2], [5], [9], [15], [20]

      - 2: [11], [12]

    - Number of RX APs

      - 2: [1], [5], [9], [15], [20]

      - 4: [11]

    - TX power

      - 0 dBm: [1], [3], [5], [9], [11]

      - 23 dBm: [1], [3], [5], [9], [11], [12], [20]

      - 26 dBm: [12]

  - FR2

    - Number of SL symbols in a slot (including AGC and TX-RX switching periods)

      - 7: [1]

      - 14: [1], [9]

    - SCS

      - 120kHz: [1], [9], [11], [16]

    - System BW

      - 100MHz: [1], [9], [16]

      - 200MHz: [11]

    - PSCCH

      - 2 symbols: [1], [9], [11]

    - PSSCH

      - Modulation order:

        - 64QAM: [1], [11]

        - 256QAM: [9]

    - Number of TX APs

      - 1: [1], [9]

      - 2: [11]

    - Number of RX APs

      - 2: [1], [9], [11]

    - TX power

      - 0 dBm: [1], [9]

      - 23 dBm: [1], [11]

- Power consumption scaling for adaptation

  - SL BWP size adaption

    - For reception: Scaling of X MHz = 0.4 +0.6*(X-20)/80 compared to 100MHz

      - Support: [1], [2], [5], [9], [11]

    - For transmission: No scaling

      - Support: [1], [2], [5], [9]

    - Antenna scaling in RX perspective

      - 2Rx power is 0.7x 4Rx power

        - Support: [1], [9], [11]

      - 1Rx power is 0.7x 2Rx power

        - Support: [1], [5], [9], [11]

    - Antenna scaling in TX perspective

      - 2Tx power is 1.4x 1Tx power at 0dBm. 1.2x at 23dBm FR1 only

        - Support: [5], [9], [11]

    - Additional scaling for adaptation

      - Scaling based on symbol duration

        - 0.6x 14-symbol case for 7-symbol case: [1]

        - Linear interpolation for various symbol duration: [5], [13]

      - Linear interpolation for other TX power: [2], [7]

      - Scaling based on the number of BD: [11]

      - Scaling based on the number of allocated PRBs: [11], [13], [17]

- Power consumption level

  - Reuse three states of “Sleep” specified in TR38.840 including transition time and energy consumption.

    - Support: [1], [2], [3], [5], [7], [9], [11], [15], [16], [17], [20]

  - 1st SCI+2nd SCI RX

    - In non-PSFCH-slot:

      - [0.6 0.7 0.8 0.9] * power consumption level of “PSCCH/PSSCH RX”: [3]

      - Same as that of “PDCCH-only” with same-slot scheduling: [5], [7], [11]

      - Larger than that of “PDCCH-only” with same-slot scheduling: [9]

        - 1.45 * Power consumption level of “PDCCH-only” with same-slot scheduling: [9]

        - 2 * Power consumption level of “PDCCH-only” with same-slot scheduling: [15]

    - In PSFCH-slot:

      - 0.85 * power consumption level of “1st SCI+2nd SCI RX” in non-PSFCH-slot: [9]

  - PSCCH/PSSCH RX

    - In non-PSFCH-slot:

      - Same as that of “PDCCH+PDSCH: [1], [2], [3], [5], [7], [9], [11], [15]

    - In PSFCH-slot:

      - 0.9 * power consumption level of “PSCCH/PSSCH RX” in non-PSFCH-slot: [9]

  - PSCCH/PSSCH TX

    - In non-PSFCH-slot:

      - Same as that of “UL (long PUCCH or PUSCH)”: [1], [2], [3], [5], [7], [9], [11]

      - 300 for FR1, 350 for FR2: [16]

      - 400: [17]

    - In PSFCH-slot:

      - 0.8 * power consumption level of “PSCCH/PSSCH TX” in non-PSFCH-slot: [9]

  - S-SSB RX:

    - Same as that of “SSB processing” with 1 SSB in a slot: [1]

    - Same as that of “SSB processing” with 2 SSBs in a slot: [3], [5], [16]

    - Same as that of “1st SCI+2nd SCI RX” in non-PSFCH-slot: [9]

    - 150: [7]

    - 0.3: [13]

  - S-SSB TX:

    - Same as that of “UL (long PUCCH or PUSCH)”: [1], [3], [5], [9], [11]

  - GNSS-operation:

    - 5: [7]

    - 8: [1]

    - 15: [11]

    - 0.08*Power consumption level of PSCCH/PSSCH RX: [3], [9]

  - Synch gNB

    - 10: [11]

  - Synch SLSS

    - 5: [11]

  - CSI-RS RX

    - Same as that of “CSI-RS processing”: [2]

    - 150: [7]

    - 0.4 for FR1, 300 for FR2: [11]

    - Not necessary to specify it: [9]

  - PSFCH TX

    - Same as that of “UL (short PUCCH)”: [1], [2], [3], [7], [15]

    - Different value compared to “UL (short PUCCH)”: [9], [11]

      - 0.36 for scaling factor: [9]

      - 0.2 for scaling factor: [11]

      - 62: [17]

  - PSFCH RX

    - Same as that of “PDCCH only” for same-slot scheduling: [1], [3], [15], [16]

    - Same as that of “PDCCH only” for cross-slot scheduling: [9]

    - 5 for FR1, 30 for FR2: [11]

  - 1st SCI+2nd SCI RX and PSFCH RX

    - Same as that of “1st SCI+2nd SCI RX” in non-PSFCH-slot: [9]

  - PSCCH/PSSCH RX and PSFCH RX

    - Same as that of “PSCCH/PSSCH RX” in non-PSFCH-slot: [9]

  - PSCCH/PSSCH TX and PSFCH TX

    - Same as that of “UL (long PUCCH or PUSCH)”: [9]

  - 1st SCI+2nd SCI RX and PSFCH TX

    - Sum of power consumption level of “1st SCI+2nd SCI RX” in PSFCH-slot and power consumption level of “PSFCH TX”: [9]

  - PSCCH/PSSCH RX and PSFCH TX

    - Sum of power consumption level of “PSCCH/PSSCH RX” in PSFCH-slot and power consumption level of “PSFCH TX”: [9]

  - PSCCH/PSSCH TX and PSFCH RX

    - Sum of power consumption level of “PSCCH/PSSCH TX” in PSFCH-slot and power consumption level of “PSFCH RX”: [9]

- Performance metric

  - PRR reduction ratio compared to the baseline scheme

    - Support: [1], [11], [12], [13]

  - Power consumption reduction ratio compared to Rel-16 NR sidelink operation

    - Support: [1], [11], [12], [13]

      - Further consideration on evaluation methodology for pedestrian UE: [14], [2], [3], [8], [10], [11], [16], [20]

      - Further consideration on evaluation for other Use cases: [16], [2], [6], [10], [11], [18]

- Public safety scenario: [16], [2], [6]

- Commercial scenario: [2], [3], [11], [18]

- Dynamic ride sharing and tethering via Vehicle: [10]

- Reference

- R1-2005254	Sidelink evaluation methodology update for power saving	Huawei, HiSilicon

- R1-2005309	Analysis on sidelink evaluation methodology	ZTE, Sanechips

- R1-2005402	Discussion on sidelink evaluation methodology	vivo

- R1-2005499	Discussion of sidelink evaluation methodology update for power saving	Nokia, Nokia Shanghai Bell

- R1-2005595	Sidelink evaluation updates for V2X	FUTUREWEI

- R1-2005610	Discussion on sidelink evaluation methodology for power saving	ITRI

- R1-2005641	Evaluation methodology for sidelink power saving	MediaTek Inc.

- R1-2005690	Discussion on evaluation methodology for sidelink enhancement	CATT

- R1-2005747	Discussion on sidelink evaluation methodology update for power saving	LG Electronics

- R1-2005838	Discussion on sidelink evaluation methodology update for power saving	Lenovo, Motorola Mobility

- R1-2005895	Sidelink evaluation methodology update for UE power saving	Intel Corporation

- R1-2006012	Discussion on evaluation methodology for SL power saving	OPPO

- R1-2006168	On Sidelink Evaluation Methodology Updates for Power Saving	Samsung

- R1-2006182	V2X Channel Model Updates for Pedestrians	InterDigital, Inc.

- R1-2006266	Sidelink evaluation methodology for power saving	Spreadtrum Communications

- R1-2006443	Evaluation assumptions and methodology for power saving	Ericsson

- R1-2006506	On Sidelink Power Consumption Model	Apple

- R1-2006624	Discussion on sidelink evaluation methodology update for power Saving	Xiaomi

- R1-2006746	Discussion on sidelink evaluation methodology for power saving	NTT DOCOMO, INC.

- R1-2006827	Evaluation Methodology for Power Saving in Sidelink	Qualcomm Incorporated