#3GPP TSG RAN WG1 #102-e						R1-20xxxxx

e-Meeting, August 17th – 28th, 2020

______________________________________________________________________Agenda item: 8.11.2.2

Source: Moderator (LG Electronics)

Title: FL summary#1 for AI 8.11.2.2 Feasibility and benefits for mode 2 enhancements

Document for: Discussion and decision

- Email discussion

As per Chairman’s guideline, the following email discussion was allocated for AI 8.11.2.2.

      - [102-e-NR-SL_enh-02] Email discussion/approval using the summary as a starting point, focusing on high-level concepts for 8.11.2.2 – Seungmin (LGE)

- To start from 8/21 till 8/27

When considering the relevant objective below, it would be desirable to focus on discussing high-level concepts for “Inter-UE coordination” in this meeting. Based on the outcome of email discussion, in the next meeting, RAN1 can discuss/conclude details and send LS to RAN (subject to decision of September RAN meeting). Please provide your view on the questions in Section 1.1/1.2/1.3/1.4 by 8/26 (Wednesday). Based on the collected view, I’ll make a set of proposals that will be discussed and finalized by 8/27.

2. Resource allocation enhancement: - Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly. - Study the feasibility and benefit of the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution if deemed feasible and beneficial [RAN1, RAN2] - Inter-UE coordination with the following until RAN#90. - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission. - Note: The study scope after RAN#90 is to be decided in RAN#90. - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios. - Note: RAN2 work will start after [RAN#89].

# 1.1	How to define “A set of resources” in Mode 2

      - Q1: Do you agree that the followings can be considered? If you want to add other options, please specify it.

- Resource set recommended by UE-A

  - e.g.,

    - TX resource set which can be used by UE-B

    - Preferred RX resource set of UE-A

| Company | Comment |
| --- | --- |
| Ericsson | In our view, the general procedure for Inter-UE coordination should consider the set of resources which are suitable for the TX of UE-B. Unless “the preferred RX resource set of UE-A” can be understood as “the suitable resource set for UE-B”, we consider that the second alternative (preferred set of resource from UE-A perspective) does not bring advantages compared to the other options and should not be included. Moreover, we think it is important to study the overhead of the coordination message in all casting scenarios, i.e., unicast, groupcast and broadcast, and assess the feasibility of the Inter-UE coordination message as defined in Proposal 1.1. In our view, the potential size of the coordination message containing the set of resources could create issues in groupcast or broadcast scenarios and potentially different options for the inter-UE coordination message may need to be defined. |

# 1.2	When UE-A transmits “A set of resources” to UE-B in Mode 2

      - Q1: Do you agree that the followings can be considered? If you want to add other options, please specify it.

- When requested by UE-B

- When satisfying the pre-defined condition

| Company | Comment |
| --- | --- |
| Ericsson | We agree with both options. We think that the Inter-UE coordination mechanism should be triggered upon request from UE-B or/and whenever some specific defined conditions are met. Nevertheless, some further study must be done before agreeing on the options: - For the first proposed option, the feasibility of the request message should be considered in terms of latency and signaling overhead. Also, if supported, the specific format and the channel on which it is carried should be discussed. - For the second proposed option, the pre-defined conditions should be listed and studied before agreeing on its feasibility. Additionally, we propose to study the periodic transmissions of the coordination message for particular scenarios. |

# 1.3	Which container is used to transmit “A set of resources” to UE-B in Mode 2

      - Q1: Do you agree that the followings can be considered? If you want to add other options, please specify it.

- PSSCH

  - MAC message

  - PC5-RRC signaling

- New 2nd SCI format

- New physical channel

| Company | Comment |
| --- | --- |
| Ericsson | First of all, we think that we should not engage in a major designing of PHY structures. Besides, the limitations in TUs, we must ensure that Rel-17 functionalities can coexist with Rel-16 sidelink. Modifications of PHY structures, if any, should respect these principles (e.g., including the Inter-UE coordination in PSFCH-like transmission or modifications to SCI format) We agree on considering the higher layer signaling (MAC or PC5-RRC signaling) as well as a potential new 2nd SCI format. |

# 1.4	How to use “A set of resources” for UE-B in Mode 2

      - Q1: Do you agree that the followings can be considered? If you want to add other options, please specify it.

- Take it into account to make a candidate TX resource set in the resource selection procedure

- Take it into account to decide selected resources for PSCCH/PSSCH TX

| Company | Comment |
| --- | --- |
| Ericsson | We are supportive of the first option, where UE-B takes into account the suggestion included in the Inter-UE coordination message to create its candidate resource set (i.e., sensing). We would like to ask for clarification regarding the second option included in the proposal since it is not clear for us the intention of it. If the intention is to include the information as part of the resource (re-)selection procedure, we are open to consider it. However, as indicated in the WID, the set of resources indicated in the Inter-UE coordination message cannot be used to “force” the receiving UE to select a specific set of resources. Therefore, if this is the intention of the second option, we do not agree with it. As indicated in Proposal 1.1, some additional options should be considered from the UE-B, e.g., resource re-selection, upon receiving an Inter-UE coordination message. |

# 1.5	Other aspects for “Inter-UE coordination”

      - Q1: What other aspects (other than those in Section 1.1/1.2/1.3/1.4) are needed to be discussed?

| Company | Comment |
| --- | --- |
| Ericsson | In our view, one point that should also be discussed is the different casting types that need to be considered for the Inter-UE coordination mechanism and the associated signaling, including potential restrictions and/or different solutions. |

- Appendix A - Summary of contributions

      - Targeting scenario

- Problems to be considered

  - Hidden-node problem [1] [3] [4] [6] [8] [9] [12] [14] [19] [20] [26] [27]

    - Example of related scenario:

      - UE-A transmits PSCCH/PSSCH to UE-B.

      - UE-C transmits PSCCH/PSSCH to UE-B.

      - UE-A and UE-C cannot recognize each other.

  - Exposed-node problem [1]

    - Example of related scenario:

      - UE-A transmits PSCCH/PSSCH to UE-C.

      - UE-B transmits PSCCH/PSSCH to UE-D.

      - UE-A and UE-B are close each other.

      - UE-C and UE-D cannot recognize each other.

  - Half-duplex problem [1] [3] [4] [5][6] [8] [9] [12] [14] [26] [27]

    - Example of related scenario:

      - UE-A transmits PSCCH/PSSCH to UE-B in slot i.

      - UE-B transmits PSCCH/PSSCH to UE-C in slot i.

  - Persistent collision [2] [4] [9] [12] [20]

    - Example of related scenario:

      - UE-A transmits PSCCH/PSSCH with period of N slots from slot i.

      - UE-B transmits PSCCH/PSSCH with period of N slots from slot i.

  - PSFCH collision problem [3] [25]

    - Example of related scenario:

      - UE-A transmits PSCCH/PSSCH to UE-B and expects to receive the corresponding PSFCH in slot i.

      - UE-A receives PSCCH/PSSCH from UE-C and is expected to transmit the corresponding PSFCH in slot i (and/or UE-A receives PSCCH/PSSCH from UE-D and is expected to transmit the corresponding PSFCH in slot i).

- Cast type to be considered

  - Broadcast: [6] [17] [20]

  - Groupcast: [3] [4] [6] [8] [17] [18] [20] [21]

  - Unicast: [3] [4] [6] [8] [17] [18] [20] [21]

      - How to define “A set of resources”

- e.g.

  - Selected resources of UE-A [2] [5][6] [9] [10] [11] [18] [21] [22]

  - Set of SL resources where UE-A can or cannot perform SL RX [3] [4] [5][6] [8] [9] [12] [26]

  - Set of SL resources suggested by UE-A [3] [5][9] [13] [15] [20] [22] [24] [26] [27] [28]

  - Sensing results of UE-A [5][6] [7] [8] [9] [10] [12] [14] [19] [20] [21] [22] [25] [26]

  - Indication for release of reserved resources [5][8][9]

  - Recommendation of transmission characteristics [9] [26] [27]

  - Resource re-selection triggering for UE-B [3] [8][13] [20] [26]

  - Selected resources for UE-B [1] [3] [7] [8] [23]

      - When UE-A transmits “A set of resources” to UE-B

- e.g.

  - Request of UE-B [1] [6] [27]

  - Event-triggered [1] [6] [27]

      - Which container is used to transmit “A set of resources” to UE-B in Mode 2

- e.g.

  - New 2nd SCI format [2] [9] [12] [16] [24] [27] [28]

  - PSSCH [9] [12] [15] [16] [18] [24] [27] [28]

    - MAC message

    - PC5-RRC signal

  - New physical channel [9] [28]

      - How to use “A set of resources” for UE-B

- e.g.

  - Take “A set of resources” into account to make S_A in Mode 2 [9] [11] [15] [16] [18] [19] [20] [24]

  - Take “A set of resources” into account to decide selected resources for PSCCH/PSSCH transmission [1] [3] [7] [8] [9] [11] [13] [14] [16] [20] [21] [23]

      - Further consideration on mixture of blind retransmission and HARQ-ACK feedback-based retransmission [8] [25] [26]

      - Further consideration on Mode 2 RA enhancement considering sum of multiple interference and TX power level [9]

      - Further consideration on resource selection from the earliest available resources [13] [25] [26]

- Reference

- R1-2005255	Inter-UE coordination in sidelink resource allocation	Huawei, HiSilicon

- R1-2005276	Sidelink resource allocation for Reliability enhancement	Lenovo, Motorola Mobility

- R1-2005296	Views on resource allocation enhancements for sidelink communication	FUTUREWEI

- R1-2005404	Discussion on mode-2 enhancements	vivo

- R1-2005501	Discussion of sidelink resource allocation mode 2 enhancements	Nokia, Nokia Shanghai Bell

- R1-2005537	Resource Allocation Enhancements for Mode 2	Fraunhofer HHI, Fraunhofer IIS

- R1-2005546	Considerations on inter-UE coordination for mode 2 enhancements	Fujitsu

- R1-2005588	High-level concepts for mode 2 enhancements	Sony

- R1-2005612	Considerations on Mode 2 Latency Enhancement	ITRI

- R1-2005645	Discussion on Mode 2 enhancements	MediaTek Inc.

- R1-2005692	Discussion on feasibility and benefits for mode 2 enhancements	CATT

- R1-2005749	Discussion on feasibility and benefits for mode 2 enhancements	LG Electronics

- R1-2005763	Views on feasibility and benefits for mode 2 enhancements	NEC

- R1-2005774	Feasibility and benefits for mode 2 enhancements	TCL Communication Ltd.

- R1-2005840	Sidelink resource allocation for Reliability enhancement	Lenovo, Motorola Mobility

- R1-2005897	On feasibility and benefits of sidelink enhancements targeting Mode 2 reliability and latency	Intel Corporation

- R1-2005903	Inter-UE coordination for enhanced resource allocation	Mitsubishi Electric RCE

- R1-2005961	Inter-UE coordination in mode-2	ZTE, Sanechips

- R1-2006010	Discussion on feasibility and benefits of mode 2 enhancements	OPPO

- R1-2006171	On Feasibility and Benefits for Mode2 Enhancements	Samsung

- R1-2006184	NR SL Mode 2 enhancement for reliability improvement 	InterDigital, Inc.

- R1-2006231	Discussion on reliability and latency enhancements for mode-2 resource  allocation	CMCC

- R1-2006268	Discussion on feasibility and benefit of mode 2 enhancements	Spreadtrum Communications

- R1-2006445	Feasibility and benefits of mode 2 enhancements for inter-UE coordination	Ericsson

- R1-2006508	Mode 2 Resoruce Allocation with Inter-UE Coordination	Apple

- R1-2006537	Mode 2 enhancements in sidelink	Panasonic Corporation

- R1-2006587	Discussion on V2X mode 2 enhancements	ASUSTeK

- R1-2006626	Discussion on Mode 2 enhancement for enhanced reliability and reduced latency	Xiaomi

- R1-2006748	Discussion on sidelink resource allocation for reliability and latency enhancements	NTT DOCOMO, INC.

- R1-2006829	Reliability and Latency Enhancements for Mode 2	Qualcomm Incorporated

- R1-2006876	Sidelink Resource Allocation Enhancements	ROBERT BOSCH GmbH

- R1-2006922	On Resource Allocation Mode 2 Enhancement for NR Sidelink	Convida Wireless