Annex B: Evaluations results(Huawei)

B.1	Link level evaluation results

B.1.1	Evaluation results for PDSCH/PUSCH

Table B.1.1-1: SINR in dB achieving PDSCH/PUSCH BLER of 10% /1% for CP-OFDM

| Tdoc / Source | MCS | Channel | 120 kHz @400 MHz |  | 240 kHz @400 MHz |  | 480 kHz @400 MHz | 960 kHz @400 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PHN | CPE | ICI | CPE | ICI | CPE | CPE |
| R1-2008779 / Huawei, HiSilicon | 7 | CDL-B, 20ns | 2/4.8 | -- | 2/4.8 | -- | 2/4.8 | 2/4.8 |
|  |  | CDL-B, 50ns | 2.9/4.2 | -- | 2.6/4.2 | -- | 2.7/4.2 | 2.6/3.9 |
|  |  | CDL-D, 30ns | -1.4/-1 | -- | -1.2/-1 | -- | -1.6/-1.4 | -1.7/-1.7 |
|  | 16 | CDL-B, 20ns | 10.5/12.5 | -- | 10/12.3 | -- | 11.8/12 | 10.7/11.9 |
|  |  | CDL-B, 50ns | 11.2/12.6 | -- | 11.2/13.4 | -- | 11.6/14.0 | 11.9/13.3 |
|  |  | CDL-D, 30ns | 7/7.2 | -- | 6.8/7.1 | -- | 6.7/6.8 | 6.6/6.8 |
|  | 22 | CDL-B, 20ns | NAN | 16.8/20 | NAN | 15.3/17.2 | 17/19 | 15.7/18 |
|  |  | CDL-B, 50ns | NAN | 16.8/18.1 | NAN | 15.8/18.2 | 16.9/19.1 | 18.4/20.8 |
|  |  | CDL-D, 30ns | NAN | 12.5/12.9 | NAN | 11.8/12.4 | 13.2/14.8 | 12.1/12.5 |
|  | Additional report/notes: - CP type: NCP - antenna configuration for CDL model: (Mg,Ng,M,N,P) = (1,1,4,8,2) BS with (0.5 dv, 0.5 dH) (Mg,Ng,M,N,P) = (1,1,2,2,2) UE with (0.5 dv, 0.5 dH) - waveform in case of PDSCH: CP-OFDM - PTRS configuration: K=2 in frequency domain, L=1 in time domain - DMRS configuration 1/2 in frequency in each port - any optional or other assumption/parameters used not as in the baseline: PN model: Example 2 phase noise model scaling to 60 GHz in 38.803 |  |  |  |  |  |  |  |

Table B.1.1-2: SINR in dB achieving PUSCH BLER of 10% /1% for DFT-s-OFDM

| Tdoc / Source | MCS | Channel | 120 kHz @400 MHz | 240 kHz @400 MHz | 480 kHz @400 MHz | 960 kHz @400 MHz |
| --- | --- | --- | --- | --- | --- | --- |
| R1-2008779 / Huawei, HiSilicon | 7 | CDL-B, 20ns | 3.0/4.3 | 2.4/4.4 | 3.2/5.0 | 2.8/4.7 |
|  |  | CDL-D, 20ns | -0.4/0.2 | -0.2/0.5 | -0.5/0.3 | -0.1/0.4 |
|  |  | CDL-D, 30ns | -0.4/0.2 | -0.2/0.5 | -0.5/0.3 | -0.1/0.4 |
|  | 16 | CDL-B, 20ns | 12.1/13.7 | 11.1/13.8 | 12.2/15.4 | 12.5/15.5 |
|  |  | CDL-D, 20ns | 7.0/7.6 | 7.5/8.1 | 7.3/8.0 | 8.1/8.6 |
|  |  | CDL-D, 30ns | 7.0/7.6 | 7.5/8.1 | 7.3/8.0 | 8.1/8.5 |
|  | 22 | CDL-B, 20ns | 20.9/NAN | 17.4/20.6 | 19.1/22.7 | 18.4/21.4 |
|  |  | CDL-D, 20ns | 14.7/NAN | 13.4/14.4 | 13.2/14.3 | 13.4/13.8 |
|  |  | CDL-D, 30ns | 14.7/NAN | 13.4/14.4 | 13.2/14.3 | 13.4/13.8 |
|  | Additional report/notes: - CP type: NCP - antenna configuration for CDL model: (Mg,Ng,M,N,P) = (1,1,4,8,2) BS with (0.5 dv, 0.5 dH) (Mg,Ng,M,N,P) = (1,1,2,2,2) UE with (0.5 dv, 0.5 dH) - waveform in case of PUSCH: DFT-s-OFDM - PTRS configuration: L=1 in time domain, and NgroupPT-RS*Nsamplegroup are: 8*4 for 120 kHz 8*4 for 240 kHz 4*4 for 480 kHz 4*2 for 960 kHz - DMRS configuration: 1/2 in frequency in each port - any optional or other assumption/parameters used not as in the baseline: PN model: Example 2 phase noise model scaling to 60 GHz in 38.803 |  |  |  |  |  |

B.1.2	Evaluation results for PRACH

Table B.1.2-1: SINR in dB achieving PRACH preamble misdetection probability of 1% and corresponding false alarm probability 1‰

| Tdoc / Source | Format | Channel | 120 kHz | 240 kHz | 480 kHz | 960 kHz |
| --- | --- | --- | --- | --- | --- | --- |
| R1-2008779 / Huawei, HiSilicon | A1 | CDL-B, 20ns | -6.920/1‰ | -7.820/1‰ | -7.829/1‰ | -8.234/1‰ |
|  |  | CDL-B, 50ns | -7.842/1‰ | -8.258/1‰ | -8.652/1‰ | -8.917/1‰ |
|  |  | CDL-D, 20ns | -6.740/1‰ | -6.740/1‰ | -6.740/1‰ | -6.728/1‰ |
|  |  | CDL-D, 30ns | -6.740/1‰ | -6.740/1‰ | -6.740/1‰ | -6.728/1‰ |
|  | A2 | CDL-B, 20ns | -9.836/1‰ | -10.860/1‰ | -10.739/1‰ | -11.030/1‰ |
|  |  | CDL-B, 50ns | -10.777/1‰ | -11.079/1‰ | -11.558/1‰ | -11.955/1‰ |
|  |  | CDL-D, 20ns | -9.529/1‰ | -9.523/1‰ | -9.526/1‰ | -9.526/1‰ |
|  |  | CDL-D, 30ns | -9.529/1‰ | -9.523/1‰ | -9.526/1‰ | -9.526/1‰ |
|  | Additional report/notes: 1. PRACH format: A1/A2 2. values of Ncs: 137 3. antenna configuration for CDL model: (Mg,Ng,M,N,P) = (1,1,4,8,2) BS with (0.5 dv, 0.5 dH) (Mg,Ng,M,N,P) = (1,1,2,2,2) UE with (0.5 dv, 0.5 dH) 4. any optional or other assumption/parameters used not as in the baseline PN model: Example 2 phase noise model scaling to 60 GHz in 38.803 |  |  |  |  |  |

B.2	System level evaluation results

B.2.1 System level evaluation results for indoor scenario A

Table B.2-1. System level evaluation results for indoor scenario A (no-LBT and omni-directional LBT)

| Tdoc / Source | Cases |  | no-LBT |  |  | omni-directional LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| R1-2008779 / Huawei, HiSilicon | DL UPT (Mbps) | 5%ile | 2690.3 | 1015.6 | 222.3 | 2527.9 | 881.7 | 158.8 |
|  |  | 50%ile | 6635.8 | 4431.6 | 3385.4 | 6137.7 | 4068.8 | 3062.5 |
|  |  | 95%ile | 8477.6 | 9469.3 | 9404.3 | 8949.0 | 8837.5 | 8628.6 |
|  |  | mean | 6381.8 | 4819.0 | 3818.9 | 5943.5 | 4404.3 | 3439.1 |
|  | DL delay (s) | 5%ile | 0.0228 | 0.0228 | 0.0229 | 0.0241 | 0.0244 | 0.0250 |
|  |  | 50%ile | 0.0325 | 0.0485 | 0.0634 | 0.0352 | 0.0530 | 0.0698 |
|  |  | 95%ile | 0.0801 | 0.2059 | 0.7516 | 0.0867 | 0.2492 | 1.0180 |
|  |  | mean | 0.0396 | 0.0743 | 0.1751 | 0.0430 | 0.0882 | 0.2161 |
|  | UL UPT (Mbps) | 5%ile | 2817.4 | 1027.9 | 304.2 | 2606.0 | 839.2 | 185.6 |
|  |  | 50%ile | 6949.9 | 4798 | 3802.7 | 6527.9 | 4358.4 | 3372.2 |
|  |  | 95%ile | 10637 | 10118 | 9111.1 | 9982.3 | 9243.3 | 8536.7 |
|  |  | mean | 6771.0 | 5046.4 | 4165.3 | 6320.3 | 4602.4 | 3726.3 |
|  | UL delay (s) | 5%ile | 0.0203 | 0.0211 | 0.0236 | 0.0216 | 0.0225 | 0.0251 |
|  |  | 50%ile | 0.0311 | 0.0448 | 0.0564 | 0.0331 | 0.0493 | 0.0634 |
|  |  | 95%ile | 0.0764 | 0.2034 | 0.5975 | 0.0837 | 0.2500 | 0.8877 |
|  |  | mean | 0.0375 | 0.0728 | 0.1613 | 0.0410 | 0.0867 | 0.2029 |
|  | Arrival rate (files/s) |  | 0.4 | 1.2 | 1.6 | 0.4 | 1.2 | 1.6 |
|  | 𝜌DL |  | 99.64% | 99.49% | 97.37% | 99.69% | 99.44% | 96.69% |
|  | 𝜌UL |  | 99.68% | 99.45% | 97.89% | 99.65% | 99.32% | 96.91% |
|  | BO |  | 12.94% | 42.34% | 57.72% | 13.81% | 45.13% | 60.60% |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm for BBU, -32dBm for UE, CWS: CW_min = CW_max = 127 2. Details of case: 2 operators (scenario A) with the same settings, case1: No LBT; case 2: omni-directional LBT 3. Details of COT sharing if used in evaluation: MCOT = 5ms, No COT sharing for DL/UL data transmission. 4. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz. Rank 1 transmission. BS to UE: InH open office channel, ftp3 file size = 27Mbyte. |  |  |  |  |  |  |  |

Table B.2-2. System level evaluation results for indoor scenario A (directional LBT and receiver assisted LBT)

| Tdoc / Source | Cases |  | directional LBT |  |  | receiver-assisted LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| R1-2008779 / Huawei, HiSilicon | DL UPT (Mbps) | 5%ile | 2507.1 | 874.7 | 154.7 | 2587.9 | 994.5 | 242.7 |
|  |  | 50%ile | 6129.2 | 4038.4 | 3031.0 | 6170.7 | 4181.1 | 3187.9 |
|  |  | 95%ile | 8956.3 | 8827.5 | 8625.3 | 8959.2 | 8852.4 | 8690.1 |
|  |  | mean | 5931.7 | 4382.4 | 3432.0 | 5991.0 | 4516.8 | 3582.2 |
|  | DL delay (s) | 5%ile | 0.0241 | 0.0244 | 0.0250 | 0.0241 | 0.0244 | 0.0248 |
|  |  | 50%ile | 0.0351 | 0.0530 | 0.0707 | 0.0352 | 0.0515 | 0.0673 |
|  |  | 95%ile | 0.0858 | 0.2411 | 0.9853 | 0.0838 | 0.2237 | 0.7022 |
|  |  | mean | 0.0427 | 0.0857 | 0.2087 | 0.0424 | 0.0819 | 0.1971 |
|  | UL UPT (Mbps) | 5%ile | 2606.0 | 836.3 | 181.8 | 2698.8 | 938.8 | 275.2 |
|  |  | 50%ile | 6528.8 | 4351.0 | 3369.3 | 6621.1 | 4490.5 | 3537.3 |
|  |  | 95%ile | 9985.1 | 9250.1 | 8531.0 | 9990.1 | 9469.4 | 8631.4 |
|  |  | mean | 6317.1 | 4594.3 | 3724.6 | 6396.9 | 4750.6 | 3868.7 |
|  | UL delay (s) | 5%ile | 0.0216 | 0.0230 | 0.0252 | 0.0216 | 0.0224 | 0.0248 |
|  |  | 50%ile | 0.0330 | 0.0494 | 0.0634 | 0.0329 | 0.0481 | 0.0605 |
|  |  | 95%ile | 0.0824 | 0.2490 | 0.8331 | 0.0815 | 0.2332 | 0.7032 |
|  |  | mean | 0.0405 | 0.0880 | 0.1989 | 0.0402 | 0.0808 | 0.1831 |
|  | Arrival rate (files/s) |  | 0.4 | 1.2 | 1.6 | 0.4 | 1.2 | 1.6 |
|  | 𝜌DL |  | 99.62% | 99.41% | 96.81% | 99.69% | 99.47% | 96.90% |
|  | 𝜌UL |  | 99.66% | 99.33% | 96.98% | 99.66% | 99.34% | 97.21% |
|  | BO |  | 13.82% | 45.20% | 60.57% | 13.81% | 44.89% | 60.35% |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm for BBU, -32dBm for UE, CWS: CW_min = CW_max = 127 2. Details of case: 2 operators (scenario A) with the same settings, case1: directional LBT; case 2: receiver-assisted LBT 3. Details of COT sharing if used in evaluation: MCOT = 5ms, No COT sharing for DL/UL data transmission. 4. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz. Rank 1 transmission. BS to UE: InH open office channel, ftp3 file size = 27Mbyte. |  |  |  |  |  |  |  |

Table B.2-9. System level evaluation results for indoor scenario A (InH mixed, no-LBT and omni-directional LBT)

| Tdoc / Source | Cases |  | no-LBT |  |  | omni-directional LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| R1-2008779 / Huawei, HiSilicon | DL UPT (Mbps) | 5%ile | 346.9 | 150.2 | 62.9 | 339.6 | 145.6 | 60.7 |
|  |  | 50%ile | 1141.5 | 764.6 | 597.5 | 1077.0 | 726.0 | 578.8 |
|  |  | 95%ile | 1851.6 | 1848.6 | 1841.6 | 1786.7 | 1768.5 | 1751.1 |
|  |  | mean | 1105.3 | 846.8 | 701.5 | 1057.6 | 805.6 | 669.1 |
|  | DL delay (s) | 5%ile | 0.0346 | 0.0353 | 0.0404 | 0.0361 | 0.0371 | 0.0421 |
|  |  | 50%ile | 0.0543 | 0.0811 | 0.1034 | 0.0574 | 0.0858 | 0.1080 |
|  |  | 95%ile | 0.2901 | 0.7812 | 1.7303 | 0.3024 | 0.8198 | 1.7773 |
|  |  | mean | 0.1062 | 0.2269 | 0.3583 | 0.1086 | 0.2320 | 0.3627 |
|  | UL UPT (Mbps) | 5%ile | 279.7 | 123.7 | 46.8 | 271.3 | 108.8 | 37.5 |
|  |  | 50%ile | 1071.6 | 753.1 | 590.5 | 1018 | 697.7 | 554.5 |
|  |  | 95%ile | 1973.5 | 1969.3 | 1943.5 | 1886.5 | 1865.4 | 1825.6 |
|  |  | mean | 1076.8 | 846.3 | 701.9 | 1021.9 | 792.7 | 659.9 |
|  | UL delay (s) | 5%ile | 0.0325 | 0.0346 | 0.0391 | 0.0342 | 0.0366 | 0.0415 |
|  |  | 50%ile | 0.0573 | 0.0823 | 0.1054 | 0.0603 | 0.0897 | 0.1121 |
|  |  | 95%ile | 0.4021 | 1.1329 | 2.2083 | 0.4133 | 1.2521 | 2.4032 |
|  |  | mean | 0.1378 | 0.2777 | 0.4186 | 0.1446 | 0.2981 | 0.4457 |
|  | Arrival rate (files/s) |  | 0.25 | 0.6 | 0.8 | 0.25 | 0.6 | 0.8 |
|  | 𝜌DL |  | 99.37% | 97.46% | 94.91% | 99.33% | 97.43% | 94.77% |
|  | 𝜌UL |  | 98.15% | 96.48% | 93.69% | 98.11% | 96.23% | 93.12% |
|  | BO |  | 20.19% | 45.88% | 58.88% | 21.01% | 47.36% | 59.9% |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm for BBU, -32dBm for UE, CWS: CW_min = CW_max = 127 2. Details of case: 2 operators (scenario A) with the same settings, case1: no-LBT; case 2: omni-directional LBT 3. Details of COT sharing if used in evaluation: MCOT = 5ms, No COT sharing for DL/UL data transmission. 4. Other parameters: Frequency 60GHz, BW = 400MHz, SCS = 120kHz. Rank 1 transmission. BS to UE: InH mixed office channel, ftp3 file size = 8Mbyte. |  |  |  |  |  |  |  |

Table B.2-10. System level evaluation results for indoor scenario A (InH mixed, directional LBT and receiver assisted LBT)

| Tdoc / Source | Cases |  | directional LBT |  |  | receiver-assisted LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| R1-2008779 / Huawei, HiSilicon | DL UPT (Mbps) | 5%ile | 332.5 | 145.6 | 59.7 | 350.1 | 165.6 | 76.05 |
|  |  | 50%ile | 1072.7 | 729.1 | 573.7 | 1067.2 | 753.9 | 624.6 |
|  |  | 95%ile | 1787.5 | 1770.5 | 1751.1 | 1784.7 | 1770.2 | 1759.3 |
|  |  | mean | 1056.1 | 805.5 | 668.31 | 1057.5 | 821.0 | 699.1 |
|  | DL delay (s) | 5%ile | 0.0361 | 0.0370 | 0.0421 | 0.0361 | 0.0369 | 0.0406 |
|  |  | 50%ile | 0.0576 | 0.0856 | 0.1078 | 0.0583 | 0.0814 | 0.1000 |
|  |  | 95%ile | 0.3016 | 0.8349 | 1.7601 | 0.2941 | 0.6925 | 1.4098 |
|  |  | mean | 0.1104 | 0.2334 | 0.3629 | 0.1045 | 0.2196 | 0.3192 |
|  | UL UPT (Mbps) | 5%ile | 261.1 | 108.7 | 38.2 | 273.7 | 114.2 | 44.23 |
|  |  | 50%ile | 1016.5 | 699.4 | 557.0 | 1025.8 | 723.6 | 594.1 |
|  |  | 95%ile | 1885.7 | 1862.6 | 1819.1 | 1887.3 | 1867.5 | 1843.5 |
|  |  | mean | 1021.7 | 790.9 | 659.8 | 1032.6 | 810.4 | 684.9 |
|  | UL delay (s) | 5%ile | 0.0343 | 0.0367 | 0.0416 | 0.0342 | 0.0361 | 0.0404 |
|  |  | 50%ile | 0.0605 | 0.0890 | 0.1118 | 0.0599 | 0.0855 | 0.1039 |
|  |  | 95%ile | 0.4228 | 1.2325 | 2.3873 | 0.4007 | 1.0162 | 2.0620 |
|  |  | mean | 0.1452 | 0.2970 | 0.4438 | 0.1371 | 0.2713 | 0.4056 |
|  | Arrival rate (files/s) |  | 0.25 | 0.6 | 0.8 | 0.25 | 0.6 | 0.8 |
|  | 𝜌DL |  | 99.33% | 97.43% | 94.81% | 99.38% | 97.44% | 95.35% |
|  | 𝜌UL |  | 98.11% | 96.32% | 93.18% | 98.25% | 96.54% | 93.70% |
|  | BO |  | 21.04% | 47.4% | 59.8% | 21.24% | 47.41% | 59.83% |
|  | Additional report/notes: Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm for BBU, -32dBm for UE, CWS: CW_min = CW_max = 127 2. Details of case: 2 operators (scenario A) with the same settings, case1: directional LBT; case 2: receiver-assisted LBT 3. Details of COT sharing if used in evaluation: MCOT = 5ms, No COT sharing for DL/UL data transmission. 4. Other parameters: Frequency 60GHz, BW = 400MHz, SCS = 120kHz. Rank 1 transmission. BS to UE: InH mixed office channel, ftp3 file size = 8Mbyte. |  |  |  |  |  |  |  |

B.2.2 System level evaluation results for indoor scenario C

Table B.2-3. System level evaluation results for indoor scenario C (no-LBT and omni-directional LBT)

| Tdoc / Source | Cases |  | no-LBT |  |  | omni-directional LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| R1-2008779 / Huawei, HiSilicon | DL UPT (Mbps) | 5%ile | 2712.1 | 1271.5 | 585.9 | 2478.7 | 1136.6 | 474.6 |
|  |  | 50%ile | 6542.7 | 4481.1 | 4029.6 | 6091.8 | 4458.6 | 3595.8 |
|  |  | 95%ile | 9478.1 | 9472.7 | 9459.5 | 8954.3 | 8848.7 | 8715.9 |
|  |  | mean | 6383.1 | 5130.7 | 4346.5 | 5945.0 | 4722.5 | 3913.9 |
|  | DL delay (s) | 5%ile | 0.0228 | 0.0228 | 0.0228 | 0.0241 | 0.0244 | 0.0247 |
|  |  | 50%ile | 0.0329 | 0.0442 | 0.0534 | 0.0354 | 0.0481 | 0.0598 |
|  |  | 95%ile | 0.0792 | 0.1646 | 0.3375 | 0.0865 | 0.1870 | 0.4128 |
|  |  | mean | 0.0396 | 0.0636 | 0.1125 | 0.0427 | 0.0706 | 0.1309 |
|  | UL UPT (Mbps) | 5%ile | 3004.4 | 1470.8 | 801.5 | 2748.7 | 1271.5 | 551.6 |
|  |  | 50%ile | 6947.7 | 5450.4 | 4476.4 | 6508.5 | 4898.4 | 3944.6 |
|  |  | 95%ile | 10637 | 10594 | 9402.1 | 10014 | 9792.6 | 8676.5 |
|  |  | mean | 6835.8 | 5534.8 | 4759.1 | 6376.1 | 5058.1 | 4232.4 |
|  | UL delay (s) | 5%ile | 0.0203 | 0.0204 | 0.0227 | 0.0215 | 0.0220 | 0.0248 |
|  |  | 50%ile | 0.0311 | 0.0394 | 0.0481 | 0.0331 | 0.0438 | 0.0544 |
|  |  | 95%ile | 0.0719 | 0.1422 | 0.2501 | 0.0786 | 0.1671 | 0.3530 |
|  |  | mean | 0.0366 | 0.0573 | 0.0951 | 0.0395 | 0.0661 | 0.1228 |
|  | Arrival rate (files/s) |  | 0.4 | 1.2 | 1.6 | 0.4 | 1.2 | 1.6 |
|  | 𝜌DL |  | 99.69% | 99.52% | 98.91% | 99.65% | 99.44% | 98.71% |
|  | 𝜌UL |  | 99.80% | 99.54% | 99.01% | 99.79% | 99.47% | 98.53% |
|  | BO |  | 12.75% | 38.75% | 52% | 13.60% | 41.32% | 55.23% |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm for BBU, -32dBm for UE, CWS: CW_min = CW_max = 127 2. Details of case: 1 operators (scenario C), case1: no-LBT; case 2: omni-directional LBT 3. Details of COT sharing if used in evaluation: MCOT = 5ms, No COT sharing for DL/UL data transmission. 4. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz. Rank 1 transmission. BS to UE: InH open office channel, ftp3 file size = 27Mbyte. |  |  |  |  |  |  |  |

Table B.2-4. System level evaluation results for indoor scenario C (directional LBT and receiver assisted LBT)

| Tdoc / Source | Cases |  | directional LBT |  |  | receiver-assisted LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| R1-2008779 / Huawei, HiSilicon | DL UPT (Mbps) | 5%ile | 2487.3 | 1128.1 | 462.6 | 2558.2 | 1209.5 | 561.7 |
|  |  | 50%ile | 6069.5 | 4452.3 | 3588.1 | 6119.3 | 4564.6 | 3801.1 |
|  |  | 95%ile | 8955.1 | 8838.9 | 8732.4 | 8959.9 | 8860 | 8761.1 |
|  |  | mean | 5931.7 | 4711.4 | 3892.5 | 5995.4 | 4826.2 | 4078.2 |
|  | DL delay (s) | 5%ile | 0.0241 | 0.0244 | 0.0247 | 0.0241 | 0.0243 | 0.0246 |
|  |  | 50%ile | 0.0355 | 0.0482 | 0.0599 | 0.0353 | 0.0471 | 0.0565 |
|  |  | 95%ile | 0.0867 | 0.1884 | 0.4249 | 0.0834 | 0.1738 | 0.3608 |
|  |  | mean | 0.0427 | 0.0705 | 0.1315 | 0.0420 | 0.0669 | 0.1214 |
|  | UL UPT (Mbps) | 5%ile | 2786.3 | 1266.3 | 558.5 | 2864.9 | 1353.5 | 619.5 |
|  |  | 50%ile | 6510.4 | 4914.7 | 3943.5 | 6562.9 | 5096.8 | 4128.0 |
|  |  | 95%ile | 10006 | 9819.6 | 8623.7 | 10020 | 9805.7 | 8768.0 |
|  |  | mean | 6376.3 | 5050.6 | 4229.1 | 6443.4 | 5180.5 | 4374.5 |
|  | UL delay (s) | 5%ile | 0.0216 | 0.0219 | 0.0249 | 0.0215 | 0.0220 | 0.0244 |
|  |  | 50%ile | 0.0331 | 0.0437 | 0.0545 | 0.0329 | 0.0422 | 0.0520 |
|  |  | 95%ile | 0.0775 | 0.1666 | 0.3475 | 0.0754 | 0.1562 | 0.3143 |
|  |  | mean | 0.0395 | 0.0662 | 0.1225 | 0.0388 | 0.0622 | 0.1122 |
|  | Arrival rate (files/s) |  | 0.4 | 1.2 | 1.6 | 0.4 | 1.2 | 1.6 |
|  | 𝜌DL |  | 99.64% | 99.44% | 98.71% | 99.65% | 99.44% | 98.78% |
|  | 𝜌UL |  | 99.79% | 99.48% | 98.55% | 99.79% | 99.49% | 98.72% |
|  | BO |  | 13.60% | 41.31% | 55.25% | 13.64% | 41.33% | 55.20% |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm for BBU, -32dBm for UE, CWS: CW_min = CW_max = 127 2. Details of case: 1 operators (scenario C), case1: directional LBT; case 2: receiver-assisted LBT 3. Details of COT sharing if used in evaluation: MCOT = 5ms, No COT sharing for DL/UL data transmission. 4. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz. Rank 1 transmission. BS to UE: InH open office channel, ftp3 file size = 27Mbyte. |  |  |  |  |  |  |  |

B.2.3 System level evaluation results for outdoor scenario B

Table B.2-5. System level evaluation results for outdoor scenario B (1 site, no-LBT and omni-directional LBT)

| Tdoc / Source | Cases |  | no-LBT |  |  | omni-directional LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| R1-2008779 / Huawei, HiSilicon | DL UPT (Mbps) | 5%ile | 1717.8 | 571.6 | 114.3 | 1650.1 | 465.4 | 82.3 |
|  |  | 50%ile | 4271.6 | 3191.9 | 2307.5 | 3986.2 | 2871.9 | 1985.4 |
|  |  | 95%ile | 7548.5 | 7296.0 | 7053.1 | 7115.3 | 6746.6 | 6462.0 |
|  |  | mean | 4543.2 | 3467.8 | 2746.8 | 4242.6 | 3131.2 | 2434.7 |
|  | DL delay (s) | 5%ile | 0.0584 | 0.0292 | 0.0306 | 0.0301 | 0.0317 | 0.0329 |
|  |  | 50%ile | 0.0506 | 0.0673 | 0.0919 | 0.0540 | 0.0746 | 0.1069 |
|  |  | 95%ile | 0.1236 | 0.3558 | 1.0587 | 0.1294 | 0.4282 | 1.3218 |
|  |  | mean | 0.0589 | 0.1096 | 0.2430 | 0.0631 | 0.1278 | 0.3009 |
|  | UL UPT (Mbps) | 5%ile | 2771.4 | 861.9 | 223.1 | 2582.3 | 694.0 | 115.5 |
|  |  | 50%ile | 6935.2 | 4863.5 | 3648.8 | 6475.1 | 4332.3 | 3097.7 |
|  |  | 95%ile | 10570 | 9542.9 | 9058.7 | 9824.7 | 8894.9 | 8501.6 |
|  |  | mean | 6574.7 | 5028.7 | 4045.6 | 6142.3 | 4546.0 | 3540.6 |
|  | UL delay (s) | 5%ile | 0.0204 | 0.0223 | 0.0238 | 0.0219 | 0.0239 | 0.0252 |
|  |  | 50%ile | 0.0311 | 0.0443 | 0.0586 | 0.0333 | 0.0497 | 0.0692 |
|  |  | 95%ile | 0.0779 | 0.2433 | 0.7830 | 0.0830 | 0.2974 | 1.0580 |
|  |  | mean | 0.0385 | 0.0772 | 0.1719 | 0.0416 | 0.0912 | 0.2261 |
|  | Arrival rate (files/s) |  | 0.4 | 1.2 | 1.6 | 0.4 | 1.2 | 1.6 |
|  | 𝜌DL |  | 99.66% | 99.07% | 96.74% | 99.61% | 98.85% | 95.26% |
|  | 𝜌UL |  | 99.80% | 99.40% | 97.19% | 99.78% | 99.22% | 96.05% |
|  | BO |  | 15.86% | 47.41% | 61.76% | 16.92% | 50.47% | 64.82% |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm for BBU, -32dBm for UE, CWS: CW_min = CW_max = 127 2. Details of case: 2 operators (outdoor scenario B, 1 site with wrapped around) with the same settings, case1: no-LBT; case 2: omni-directional LBT 3. Details of COT sharing if used in evaluation: MCOT = 5ms, No COT sharing for DL/UL data transmission. 4. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz, Rank 1 transmission. ftp3 file size = 27Mbyte. |  |  |  |  |  |  |  |

Table B.2-6. System level evaluation results for outdoor scenario B(1 site, directional LBT and receiver assisted LBT)

| Tdoc / Source | Cases |  | directional LBT |  |  | receiver-assisted LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| R1-2008779 / Huawei, HiSilicon | DL UPT (Mbps) | 5%ile | 1654.3 | 479.9 | 77.5 | 1642.2 | 468.6 | 81.5 |
|  |  | 50%ile | 3980.2 | 2843.3 | 1936.8 | 3978.3 | 2878.8 | 2035.3 |
|  |  | 95%ile | 7110.6 | 6722.2 | 6445.0 | 7060.1 | 6709.4 | 6435.9 |
|  |  | mean | 4227.9 | 3119.4 | 2413.8 | 4207.0 | 3152.7 | 2464.7 |
|  | DL delay (s) | 5%ile | 0.0302 | 0.0318 | 0.0330 | 0.0305 | 0.0318 | 0.0331 |
|  |  | 50%ile | 0.0542 | 0.0752 | 0.1089 | 0.0542 | 0.0742 | 0.1041 |
|  |  | 95%ile | 0.1289 | 0.4167 | 1.3588 | 0.1307 | 0.4221 | 1.3624 |
|  |  | mean | 0.0632 | 0.1275 | 0.3056 | 0.0633 | 0.1284 | 0.2997 |
|  | UL UPT (Mbps) | 5%ile | 2581.5 | 679.19 | 117.2 | 2580.8 | 701.0 | 133.3 |
|  |  | 50%ile | 6478.3 | 4323.7 | 3093.7 | 6486.2 | 4413.9 | 3213.7 |
|  |  | 95%ile | 9838.3 | 8912.6 | 8516.8 | 9880.6 | 8910.9 | 8504.7 |
|  |  | mean | 6138.7 | 4537.8 | 3550.8 | 6165.1 | 4589.3 | 3604.5 |
|  | UL delay (s) | 5%ile | 0.0219 | 0.0240 | 0.0253 | 0.0219 | 0.0236 | 0.0252 |
|  |  | 50%ile | 0.0333 | 0.0498 | 0.0691 | 0.0333 | 0.0488 | 0.0666 |
|  |  | 95%ile | 0.0837 | 0.3032 | 1.0907 | 0.0833 | 0.2950 | 1.0840 |
|  |  | mean | 0.0416 | 0.0923 | 0.2282 | 0.0415 | 0.0890 | 0.2253 |
|  | Arrival rate (files/s) |  | 0.4 | 1.2 | 1.6 | 0.4 | 1.2 | 1.6 |
|  | 𝜌DL |  | 99.61% | 98.86% | 95.08% | 99.61% | 98.84% | 95.25% |
|  | 𝜌UL |  | 99.79% | 99.18% | 96.03% | 99.78% | 99.23% | 96.39% |
|  | BO |  | 16.93% | 50.45% | 64.81% | 16.97% | 50.58% | 64.90% |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm for BBU, -32dBm for UE, CWS: CW_min = CW_max = 127 2. Details of case: 2 operators (outdoor scenario B, 1 site with wrapped around) with the same settings, case1: directional LBT; case 2: receiver-assisted LBT 3. Details of COT sharing if used in evaluation: MCOT = 5ms, No COT sharing for DL/UL data transmission. 4. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz, Rank 1 transmission. ftp3 file size = 27Mbyte. |  |  |  |  |  |  |  |

Table B.2-7. System level evaluation results for outdoor scenario B (7 site, no-LBT and omni-directional LBT)

| Tdoc / Source | Cases |  | no-LBT |  |  | omni-directional LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| R1-2008779 / Huawei, HiSilicon | DL UPT (Mbps) | 5%ile | 1456.9 | 909.7 | 229.5 | 1294.1 | 749.5 | 180.6 |
|  |  | 50%ile | 4252.2 | 3809.4 | 2996.6 | 3962.8 | 3390.4 | 2687.2 |
|  |  | 95%ile | 7472.9 | 7389.9 | 7142.7 | 7027.2 | 6877.8 | 6682.8 |
|  |  | mean | 4460.8 | 3887.5 | 3258.4 | 4144.6 | 3558.5 | 2947.9 |
|  | DL delay (s) | 5%ile | 0.0288 | 0.0293 | 0.0303 | 0.0306 | 0.0314 | 0.0325 |
|  |  | 50%ile | 0.0510 | 0.0588 | 0.0722 | 0.0548 | 0.0654 | 0.0807 |
|  |  | 95%ile | 0.1440 | 0.2349 | 0.5771 | 0.1557 | 0.2664 | 0.6798 |
|  |  | mean | 0.0625 | 0.0904 | 0.1567 | 0.0678 | 0.1022 | 0.1808 |
|  | UL UPT (Mbps) | 5%ile | 2603.7 | 1349.6 | 346.4 | 2288.2 | 1147.3 | 248.2 |
|  |  | 50%ile | 6936.5 | 5628.5 | 4358.9 | 6455.4 | 5204.2 | 3836.5 |
|  |  | 95%ile | 10518 | 9156.1 | 9053.4 | 9688.7 | 8694.4 | 8515.9 |
|  |  | mean | 6452.0 | 5610.1 | 4637.2 | 6013.1 | 5151.1 | 4177.2 |
|  | UL delay (s) | 5%ile | 0.0209 | 0.0235 | 0.0238 | 0.0224 | 0.0248 | 0.0252 |
|  |  | 50%ile | 0.0312 | 0.0385 | 0.0492 | 0.0339 | 0.0421 | 0.0556 |
|  |  | 95%ile | 0.0869 | 0.1573 | 0.4317 | 0.0944 | 0.1796 | 0.5169 |
|  |  | mean | 0.0414 | 0.0597 | 0.1131 | 0.0447 | 0.0675 | 0.1317 |
|  | Arrival rate (files/s) |  | 0.4 | 0.8 | 1.4 | 0.4 | 0.8 | 1.4 |
|  | 𝜌DL |  | 98.79% | 98.59% | 96.37% | 98.68% | 98.39% | 95.55% |
|  | 𝜌UL |  | 99.27% | 99.14% | 96.83% | 99.26% | 98.88% | 95.81% |
|  | BO |  | 16.04% | 35.34% | 57.44% | 17.11% | 35.40% | 60.35% |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm for BBU, -32dBm for UE, CWS: CW_min = CW_max = 127 2. Details of case: 2 operators (outdoor scenario B, 7 site) with the same settings, case1: no-LBT; case 2: omni-directional LBT 3. Details of COT sharing if used in evaluation: MCOT = 5ms, No COT sharing for DL/UL data transmission. 4. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz, Rank 1 transmission. ftp3 file size = 27Mbyte. |  |  |  |  |  |  |  |

Table B.2-8. System level evaluation results for outdoor scenario B (7 site, directional LBT and receiver assisted LBT)

| Tdoc / Source | Cases |  | directional LBT schemes |  |  | receiver-assisted LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| R1-2008779 / Huawei, HiSilicon | DL UPT (Mbps) | 5%ile | 1286.7 | 750.8 | 179.2 | 1385.6 | 743.9 | 198.2 |
|  |  | 50%ile | 3934.1 | 3258.1 | 2643.7 | 3926.1 | 3299.0 | 2705 |
|  |  | 95%ile | 7085.9 | 6762.7 | 6671.5 | 6938.1 | 6748.1 | 6514 |
|  |  | mean | 4109.5 | 3485.6 | 2934.1 | 4110.0 | 3520.0 | 2933.3 |
|  | DL delay (s) | 5%ile | 0.0301 | 0.0315 | 0.0317 | 0.0306 | 0.0316 | 0.0327 |
|  |  | 50%ile | 0.0548 | 0.0656 | 0.0798 | 0.0546 | 0.0643 | 0.0787 |
|  |  | 95%ile | 0.1482 | 0.2616 | 0.6793 | 0.1527 | 0.2618 | 0.6499 |
|  |  | mean | 0.0672 | 0.1009 | 0.1808 | 0.0670 | 0.0996 | 0.1757 |
|  | UL UPT (Mbps) | 5%ile | 2126.9 | 1082.2 | 245.9 | 2285.2 | 1091.7 | 274.1 |
|  |  | 50%ile | 6214.7 | 5064.6 | 3816.0 | 6363.7 | 5151.7 | 3903.9 |
|  |  | 95%ile | 8849.0 | 8607.6 | 8455.1 | 9595.1 | 8640.3 | 8491.6 |
|  |  | mean | 5873.7 | 5085.0 | 4157.8 | 5947.3 | 5124.8 | 4216.9 |
|  | UL delay (s) | 5%ile | 0.0242 | 0.0248 | 0.0263 | 0.0223 | 0.0247 | 0.0252 |
|  |  | 50%ile | 0.0343 | 0.0423 | 0.0576 | 0.0336 | 0.0417 | 0.0544 |
|  |  | 95%ile | 0.0975 | 0.1876 | 0.5191 | 0.0916 | 0.1835 | 0.5068 |
|  |  | mean | 0.0458 | 0.0686 | 0.1359 | 0.0445 | 0.0681 | 0.1290 |
|  | Arrival rate (files/s) |  | 0.4 | 0.8 | 1.4 | 0.4 | 0.8 | 1.4 |
|  | 𝜌DL |  | 98.45% | 98.83% | 95.85% | 98.63% | 98.37% | 95.49% |
|  | 𝜌UL |  | 99.13% | 98.80% | 95.93% | 99.31% | 98.89% | 96.01% |
|  | BO |  | 17.51% | 35.84% | 61.15% | 17.32% | 35.93% | 60.93% |
|  | Additional report/notes: 1. LBT procedure and parameters: LBT based on ETSI EN 302 567 v2.1.20, ED thresholds -47dBm for BBU, -32dBm for UE, CWS: CW_min = CW_max = 127 2. Details of case: 2 operators (outdoor scenario B, 7 site) with the same settings, case1: directional LBT; case 2: receiver-assisted LBT 3. Details of COT sharing if used in evaluation: MCOT = 5ms, No COT sharing for DL/UL data transmission. 4. Other parameters: Frequency 60GHz, BW = 2GHz, SCS = 960kHz, Rank 1 transmission. ftp3 file size = 27Mbyte. |  |  |  |  |  |  |  |

### B.2.4	RSRP distribution

Note: companies are encouraged to also submit RSRP distribution (e.g. serving BS to UE links, BS-to-BS links, UE-to-UE links) for the evaluated scenario in SLS.

Figure 1. Serving link RSRP of Indoor scenario-A/C

Figure 2. BS-to-BS interference link RSRP of Indoor scenario-A/C

Figure 3. UE-to-UE interference link RSRP of Indoor scenario-A/C

Figure 4. Serving link RSRP of outdoor scenario-B

Figure 5. BS-to-BS interference link RSRP of Indoor scenario-B

Figure 6. UE-to-UE interference link RSRP of Indoor scenario-B