# Annex B: Evaluations results

## B.1	Link level evaluation results

### B.1.1	Evaluation results for PDSCH

Table B.1.1-1: LLS template: SNR in dB achieving PDSCH BLER of 10% /1%

| Tdoc / Source | MCS | Channel | 120KHz /400MHz | 240KHz /400MHz | 480KHz /400MHz | 960KHz /400MHz | 960KHz /2GHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Lenovo, Motorola Mobility | 7 | TDL-A, 5ns | 3/ 5.2 | 3.2/ 5.17 | 3.2/5.22 | 3.7/5.9 | 2.4/ 3.9 |
|  |  | TDL-A, 10ns | 2.6/ 4.5 | 2.7/ 4.7 | 2.7/ 4.7 | 3.5/5.5 | 2.4/ 3.9 |
|  |  | TDL-A, 20ns | 2.4/ 3.9 | 2.6/ 4.1 | 3.1/ 4.5 | 4/ 5.9 | 2.4/ 3.8 |
|  |  | TDL-A, 40ns | 2.4/3.9 | 2.6/4.4 | 3.8/5.4 | 5.1/7 |  |
|  |  | CDL-B, 20ns | 4.9/5.9 | 5.2/6.1 | 6/6.8 | 5.2/6.2 | 5/6.2 |
|  |  | CDL-B, 50ns | 5.1/7 | 5.3/6.8 | 7/8.3 | 7.8/8.9 | 7.5/8.7 |
|  | 16 | TDL-A, 5ns | 11.2/ 14.4 | 11.2/ 14.2 | 10.9/ 13.5 | 11 /13.8 | 10.9/ 12.8 |
|  |  | TDL-A, 10ns | 10.8/ 13 | 10.8/ 13 | 10.5/ 12.5 | 11/13.7 | 10.9/ 12.9 |
|  |  | TDL-A, 20ns | 10.7/ 12.6 | 10.7/ 12.6 | 10.7/ 12.6 | 11.5/ 13.8 | 10.5/ 12.3 |
|  |  | TDL-A, 40ns | 10.8/12.7 | 10.9/12.8 | 11/12.8 | 13/15.1 |  |
|  |  | CDL-B, 20ns | 13/14.5 | 12.3/12.9 | 13.1/13.8 | 12/12.7 | 12.3/13.2 |
|  |  | CDL-B, 50ns | 13.2/14.5 | 13.7/15.5 | 14.7/15.6 | 18/19.7 | 18.1/20.9 |
|  | 22 | TDL-A, 5ns | 27/ - *Note | 20/- *Note | 18/33 | 17.8 / 26.8 | 24/- *Note |
|  |  | TDL-A, 10ns | -/- *Note | 25.2/- *Note | 19.1/- *Note | 18.5/ 33 | -/- *Note |
|  |  | TDL-A, 20ns | -/- *Note | -/- *Note | 23.5/- *Note | 20/- *Note | -/- *Note |
|  |  | TDL-A, 40ns | -/- *Note | -/- *Note | 27/- *Note | -/- *Note |  |
|  |  | TDL-A, 40ns (ECP) | -/- *Note | -/- *Note | 24/- *Note | 22.5/- *Note |  |
|  |  | CDL-B, 20ns | -/- *Note | -/- *Note | 21.2/25.8 | 17.4/18.4 | 19.3/21.8 |
|  |  | CDL-B, 50ns | -/- *Note | -/ - *Note | -/- *Note | -/- *Note | -/- *Note |
|  |  | CDL-B, 50ns (ECP) |  |  | 21.4/26.1 | 19.1/21.4 | 21.4/25.3 |
|  | Additional report/notes: - CP type: NCP (all), ECP (TDL-A 40ns, CDL-B 50ns) - antenna configuration for CDL model: Configuration 2 (Mg,Ng,M,N,P) = (1,1,4,8,2) BS with (0.5 dv, 0.5 dH) (Mg,Ng,M,N,P) = (1,1,2,2,2) UE with (0.5 dv, 0.5 dH) - waveform in case of PUSCH: NA - PTRS configuration: L=1, K=4 - DMRS configuration: Type 1, front loaded, no data multiplexing - any optional or other assumption/parameters used not as in the baseline -PN model: Example 2 from 38.803 phase noise model (gNB, UE) scaled to 60 GHz -CPE compensation -No TRS, no CSI-RS *Note: The values are missing since the required SNR for 10%/1% BLER is either very high or the BLER target is not reachable due to error floor. |  |  |  |  |  |  |