3GPP TSG-RAN WG1 Meeting #103-e	R1-2007610

# e-Meeting, October 26 – November 13, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | XXX | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on sidelink PT-RS sequence generation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is unspecified which sidelink DM-RS symbol is used for determination of rm for sidelink PT-RS. In NR Uu, the first DMRS symbol is used for rm determination. However, in NR sidelink, such a definition is ambiguous since the first indicated DMRS symbol can be punctured by PSCCH or not mapped due to the 20 PRBs constraint for FMDed DMRS and PSCCH. In RAN1#102-e, it was clarified that 2nd-stage SCI is mapped from the first transmitted PSSCH DMRS symbol. Since 2nd-stage SCI is always transmitted together with PSSCH, the same can be done for the referencing of PT-RS to the DM-RS symbol transmitted in the first symbol of the 2nd-stage SCI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify that rm for sidelink PT-RS is the sidelink DM-RS sequence in the first transmitted symbol of the 2nd-stage SCI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification for sidelink PT-RS sequence generation. If the change is not approved, a UE does not know which PSSCH DMRS symbol is used to determine PT-RS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

##### 8.4.1.2.1	Sequence generation

The precoded sidelink phase-tracking reference signal for subcarrier k on layer j is given by

r(pj)m=rmif j=j' or j=j"0otherwise

where

-	antenna ports pj' or pj',pj'' associated with PT-RS transmission are given by clause 8.2.3 of [6, TS 38.214];

-	rm is given by clause 8.4.1.1.1 at the position of the DM-RS symbol transmitted in the first symbol of the 2nd-stage SCI as described in clause 8.3.1.5.

<Unchanged parts are omitted>