3GPP TSG RAN WG1 Meeting #103-e	R1-2007636

E-meeting, October 26–November 13, 2020

Agenda Item:	7.2.5.7

Source:	Huawei, HiSilicon

Title:	Corrections on SPS enhancements

Document for:	Discussion and Decision

Introduction

During the RAN1#102-e meeting, remaining issues have been discussed for DL SPS and some of them regarding the HARQ-ACK feedback have not been solved. This contribution focuses on those remaining issues.

HARQ-ACK feedback for SPS PDSCH release and SPS PDSCH receptions

According to the FL’s summary[1]，the first  remaining issue is whether and how to handle the scenario where the SPS release DCI and the SPS PDSCH for the same configuration are received in the same slot and the HARQ-ACK for the SPS release and the SPS reception would map to different PUCCHs.

In a slot, no matter if SPS release DCI is received before or after the end of the SPS PDSCH for the same SPS configuration, it is the gNB’s choice and there is no need to restrict its scheduling. And if the HARQ-ACK for the SPS release and for the SPS PDSCH reception would map to different PUCCHs, then there will be no collision and both the SPS PDSCH and also the SPS release DCI could be received and both corresponding HARQ-ACKs should be generated.

Proposal 1: Support the case that SPS release PDCCH is received in a slot where SPS PDSCH is configured to be received for the same SPS configuration corresponding to the SPS release PDCCH if the HARQ-ACK for the SPS release and the SPS reception mapping to different PUCCHs.

- The UE receives the SPS PDSCH and generates HARQ-ACK information for the SPS release and the SPS reception

- Adopt the following TP accordingly

----------------------------------------------------- Start of text proposal -------------------------------------------- 9.1 HARQ-ACK codebook determination <unchanged text omitted> If a UE is configured to receive SPS PDSCHs in a slot for SPS configurations that are indicated to be released by a DCI format, and if the UE receives the PDCCH providing the DCI format in the slot where the end of a last symbol of the PDCCH reception is not after the end of a last symbol of any of the SPS PDSCH receptions, and if HARQ-ACK information for the SPS PDSCH release and the SPS PDSCH receptions would be multiplexed in a same PUCCH, the UE does not expect to receive the SPS PDSCHs, does not generate HARQ-ACK information for the SPS PDSCH receptions, and generates a HARQ-ACK information bit for the SPS PDSCH release. If a UE is configured to receive SPS PDSCHs in a slot for SPS configurations that are indicated to be released by a DCI format, no matter if the UE receives the PDCCH providing the DCI format in the slot where the end of a last symbol of the PDCCH reception is after or before the end of a last symbol of any of the SPS PDSCH receptions, if HARQ-ACK information for the SPS PDSCH release and the SPS PDSCH receptions would be transmitted in two different PUCCHs, the UE receives the SPS PDSCHs and generates HARQ-ACK information for the SPS PDSCH receptions and a HARQ-ACK information bit for the SPS PDSCH release. ----------------------------------------------------- End of text proposal ---------------------------------------------

HARQ-ACK codebook for SPS PDSCH with PDSCH aggregation

For configured multiple aggregation factors, it was agreed to use the maximum value in order to avoid potential ambiguity in the type-1 HARQ-ACK codebook construction. There is one FFS for the RRC parameter RepNumR16, which is used for the dynamic indication of the number of repetitions.

Considering that RepNumR16 is introduced in the MTRP scenario, in which case the type-1 HARQ-ACK codebook also is used，we think that the maximum value of pdsch-AggregationFactor is established by considering values in SPS-Config or PDSCH-Config and RepNum16 values, if provided.

Proposal 2: If RepNumR16 is provided, N_PDSCH^(repeat,max) is the maximum value among  pdsch-AggregationFactor in SPS-Config or PDSCH-Config and RepNumR16 in PDSCH-TimeDomainResourceAllocation.

Conclusion

According to the discussion, following proposals and observations are provided:

Proposal 1: Support the case that SPS release PDCCH is received in a slot where SPS PDSCH is configured to be received for the same SPS configuration corresponding to the SPS release PDCCH if the HARQ-ACK for the SPS release and the SPS reception mapping to different PUCCHs.

- The UE receives the SPS PDSCH and generates HARQ-ACK information for the SPS release and the SPS reception

- Adopt the following TP accordingly

----------------------------------------------------- Start of text proposal -------------------------------------------- 9.1 HARQ-ACK codebook determination <unchanged text omitted> If a UE is configured to receive SPS PDSCHs in a slot for SPS configurations that are indicated to be released by a DCI format, and if the UE receives the PDCCH providing the DCI format in the slot where the end of a last symbol of the PDCCH reception is not after the end of a last symbol of any of the SPS PDSCH receptions, and if HARQ-ACK information for the SPS PDSCH release and the SPS PDSCH receptions would be multiplexed in a same PUCCH, the UE does not expect to receive the SPS PDSCHs, does not generate HARQ-ACK information for the SPS PDSCH receptions, and generates a HARQ-ACK information bit for the SPS PDSCH release. If a UE is configured to receive SPS PDSCHs in a slot for SPS configurations that are indicated to be released by a DCI format, no matter if the UE receives the PDCCH providing the DCI format in the slot where the end of a last symbol of the PDCCH reception is after or before the end of a last symbol of any of the SPS PDSCH receptions, if HARQ-ACK information for the SPS PDSCH release and the SPS PDSCH receptions would be transmitted in two different PUCCHs, the UE receives the SPS PDSCHs and generates HARQ-ACK information for the SPS PDSCH receptions and a HARQ-ACK information bit for the SPS PDSCH release. ----------------------------------------------------- End of text proposal ---------------------------------------------

Proposal 2: If RepNumR16 is provided, N_PDSCH^(repeat,max) is the maximum value among  pdsch-AggregationFactor in SPS-Config or PDSCH-Config and RepNumR16 in PDSCH-TimeDomainResourceAllocation.

- References

- R1-2007124, Summary #1 of [102-e-NR-L1enh-URLLC-IIoTenh-01],Moderator (LG Electronics)