3GPP TSG RAN WG1 #103-e			R1-2007659

e-Meeting, October 26th – November 13th, 2020

Source: 	vivo

Title:  	Discussion on propagation delay compensation enhancements

Agenda Item:	8.3.4

Document for:	Discussion and Decision

Introduction

In RAN1#102e meeting, propagation delay compensation enhancements are discussed and some agreements are achieved as following.

Agreements:

- Take the following use cases as the representative use cases for further study on propagation delay compensation enhancements in Rel-17.

| User-specific clock synchronicity accuracy level | Number of devices in one Communication group for clock synchronisation | 5GS synchronicity budget requirement (note) | Service area | Scenario |
| --- | --- | --- | --- | --- |
| 2 | Up to 300 UEs | ≤900 ns | ≤ 1000 m x 100 m | - Control-to-control communication for industrial controller |
| 4 | Up to 100 UEs | <1 µs | < 20 km2 | - Smart Grid: synchronicity between PMUs |

Agreements:

- ± 8∙64∙Tc/2μ8*64*Tc/2 as the TA indicating error is assumed in the evaluation.

Agreements:

For 5GS synchronicity budget requirement,

- One Uu interface is assumed for smart grid.

- Two Uu interfaces are assumed for control-to-control.

Agreements:

For BS transmit timing error, further study the following three options:

- Option 1: 65 ns

- Option 2:±130ns for the indoor scenario and ±200ns for the smart grid scenario

- Option 3:82.5 ns

Agreements:

The value defined in Table 7.1.2-1 for initial transmit timing error (Te) in TS 38.133 should be considered for evaluation of the time synchronization.

Agreements:

Asymmetry between downlink and uplink channel for control-to-control scenario is not considered.

Agreements:

100 ns is assumed for BS detecting error.

Agreements:

Timing advance adjustment accuracy defined in Table 7.3.2.2-1 in TS 38.133 is assumed for evaluation of the time synchronization.

Agreements:

Both 15 kHz and 30 kHz are assumed for both control-to-control and smart grid for evaluation of the time synchronization.

Agreements:

The following options for propagation delay compensation are further studied in RAN1

- Option 1: TA-based propagation delay

  - Option 1a: Propagation delay estimation based on legacy Timing advance (potentially with enhanced TA indication granularity).

  - Option 1b: Propagation delay estimation based on timing advanced enhanced for time synchronization (as 1a but with updated RAN4 requirements to TA adjustment error and Te)

  - Option 1c: Propagation delay estimation based on a new dedicated signaling with finer delay compensation granularity (Separated signaling from TA so that TA procedure is not affected)

- Option 2: RTT based delay compensation:

  - Propagation delay estimation based on an RAN managed Rx-Tx procedure intended for time synchronization (FFS to expand or separate procedure/signaling to positioning)

In this contribution, we share our views on propagation delay compensation enhancements.

Parameters for propagation delay compensation evaluation

According to agreement in the last RAN1 meeting, some parameters for propagation delay evaluation are discussed. The discussion on some parameters are still open. In this section, the remaining parameters are discussed.

BS transmit timing error

The maximum gNB transmit time alignment error is defined in 38.104 as shown below.

- 6.5.3.2 Minimum requirement for BS type 1-C and BS type 1-H For MIMO transmission, at each carrier frequency, TAE shall not exceed 65 ns. For intra-band contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 260ns. For intra-band non-contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 3µs. For inter-band carrier aggregation, with or without MIMO, TAE shall not exceed 3µs. The time alignment error requirements for NB-IoT are specified in TS 36.104 [13] clause 6.5.3.

For Control-to-Control use case and smart grid use case, a single cell with MIMO transmission can be baseline. For this case, the maximum error is to be within ±65ns, i.e., TAE=65 ns can be assumed.

Proposal 1: For Control-to-Control use case and smart grid use case, 65 ns can be baseline for BS transmit timing error.

Downlink frame timing error

Downlink frame timing error is related to UE detection error. The achievable synchronization accuracy will be very much dependent on the (bandwidth of the) used reference signal and the possible related time domain resolution in the estimation process [3], e.g. PSS/SSS or PBCH DM-RS. Downlink frame timing error may affect the TA accuracy and should be considered in analysis. In this contribution, downlink frame timing error is assumed as 260ns.

Proposal 2: Downlink frame timing error is considered in propagation delay compensation evaluation.

Options for propagation delay compensation

For propagation delay compensation evaluation, TA-based and RTT-based solutions are provided in the last meeting.

TA has been studied and applied widely in NR and previous system. For RTT-based solution, more detail is needed. RTT-based method is used for positioning procedure, it should be clarified whether all procedure/signalling for positioning are reused for propagation delay compensation. On the other hand, it is also need discuss whether URLLC UEs with propagation delay compensation capability must support positioning capability. For RTT-based method, at least the following aspects should be clarified.

- Exchange of time information

For RTT-based method, gNB does not know the transmitting time and receiving time of reference signal at UE side and vice versa. In order to perform propagation delay compensation at UE or gNB side, the exchange of time information is needed between gNB and UE.

- Resource allocation of reference signal.

For RTT-based method, the large bandwidth of reference signal is used to guarantee accuracy. In IIOT scenario, the larger bandwidth of reference signal is required for all UEs if high accuracy delay compensation is needed. Thus, the overhead of reference signal is larger, especially for UE specific reference signal.

- The signaling overhead

Obtaining the required precision for external clock may need quite frequent time information updates over Uu interface. Thus, the signaling overhead caused by triggering RTT-based delay measurement may be huge in order to guarantee the synchronization error is always less than synchronicity budget requirement.

Observation 1: For RTT-based method, the more detail need clarify, e.g. exchange of time information, resource allocation of reference signal, the signaling overhead and so on.

Proposal 3: TA-based method is baseline for propagation delay compensation.

Timing error evaluation

In the following section, the propagation delay budget is evaluated.

Control-to-Control use case

For Control-to-Control use case, UE-to-UE transmission is illustrated in Figure 1. According to the description of TS 23.501, the DS-TT may not be integrated into UE. For the case that the DS-TT and UE is separate, it is proposed to model the error budget for device part one PTP hop (i.e., ±40ns). Considering the device may have lower sync accuracy ability than NW node, we prefer to leave a more tolerable error budget for the device part. For example, error budget of ±50ns for the device part can be assumed.

Figure 1: UE-to-UE gPTP message transmission

As two UE(s) need to be synchronized with the 5G GM via the reference time provided by the gNB, then the errors added by the two UEs would be doubled compared to the one Uu interface from gNB to UE.

Figure 2: Procedure of reference time provisioning and propagation delay compensation for CF RA

The following inaccuracy analysis is calculated after removing the propagation delay by using NTA/2. As shown in Figure 2, the whole procedure of reference time provisioning to the UE includes two separate procedures:

- Procedure 1: The gNB provides the referenceTime (which is pointed to a specific SFN) to the UE via SIB/DLInfomationTransfer.

- Procedure 2: The UE triggers RACH to obtain an NTA closest to the provided referenceTime, after receiving the propagation compensation indication required by the network.

The structure and error components of the following table are based on [2].

Table 1: Timing error in clock synchronisation

| Factors | Timing error type | Requirement for different SCS (kHz) (unit: ns) |  |
| --- | --- | --- | --- |
|  |  | 15 | 30 |
| 1 | Inaccuracy caused by downlink/uplink frame timing alignment |  |  |
| 1.1 | Time alignment error of gNB transmitter (TETAE) | 65 | 65 |
| 1.2 | Inaccuracy caused by UE detection (TEUE-DL-RX) | 260 | 260 |
| 1.3 | Inaccuracy caused by propagation delay when propagation delay is decided from TA. (1.3.1+1.3.2+1.3.3)/2 | 375 | 245 |
| 1.3.1 | Inaccuracy of UE transmitting (TEUE-DL-to-TX) | 390 | 260 |
| 1.3.2 | Inaccuracy of gNB detection (TEUL-RX) | 100 | 100 |
| 1.3.3 | Inaccuracy caused by TA indication (TETA-G) | 260 | 130 |
| 1.4 | Inaccuracy caused by propagation delay when considering 1000 m x 100 m service area (TEPD) | 105 | 105 |
| 2 | Inaccuracy caused by time reference information delivery from gNB to UE (TERT) | 5 | 5 |
| 3 | Inaccuracy caused by the synchronisation between the gNB and 5G GM | 160 | 160 |
| 4 | Inaccuracy caused by the synchronization between the UE and the DS-TT | 50 | 50 |
| Total error over Uu | Total inaccuracy over Uu for 1000 m x 100 m service area (with propagation compensation) = (2*(1.1+1.2+1.3+2)) | 1410 | 1150 |
| Total error of 5GS E2E | Total inaccuracy for 1000 m x 100 m service area (with propagation compensation) = (2*(1.1+1.2+1.3+2+3+4)) | 1830 | 1570 |

As shown in Table 1, total inaccuracy for 1000 m x 100 m service area (with propagation compensation) = (2*(1.1+1.2+1.3+2+3+4)).

According to the analysis given in the above table, the inaccuracy of 15/30KHz SCS cannot fulfil the TSN clock synchronization requirements of 900ns even after the propagation delay compensation based on TA.

Observation 2: The inaccuracy of 15/30KHz SCS cannot fulfil the TSN clock synchronization requirements of 900ns for UE-to-UE time synchronization use case.

Proposal 4: The method in Table 1 is used for overall timing error evaluation for Control-to-Control use case.

Smart grid

For smart grid use case, one Uu interface is assumed. The structure and error components are shown in Table 2.

Table 2: Timing error in clock synchronisation

| Factors | Timing error type | Requirement for different SCS (kHz) (unit: ns) |  |
| --- | --- | --- | --- |
|  |  | 15 | 30 |
| 1 | Inaccuracy caused by downlink/uplink frame timing alignment |  |  |
| 1.1 | Time alignment error of gNB transmitter (TETAE) | 65 | 65 |
| 1.2 | Inaccuracy caused by UE detection (TEUE-DL-RX) | 260 | 260 |
| 1.3 | Inaccuracy caused by propagation delay when propagation delay is decided from TA. (1.3.1+1.3.2+1.3.3)/2 | 375 | 245 |
| 1.3.1 | Inaccuracy of UE transmitting (TEUE-DL-to-TX) | 390 | 260 |
| 1.3.2 | Inaccuracy of gNB detection (TEUL-RX) | 100 | 100 |
| 1.3.3 | Inaccuracy caused by TA indication (TETA-G). | 260 | 130 |
| 1.4 | Inaccuracy caused by propagation delay when considering 20Km2 service area (TEPD) | 8410 | 8410 |
| 2 | Inaccuracy caused by time reference information delivery from gNB to UE (TERT) | 5 | 5 |
| 3 | Inaccuracy caused by the synchronization between the gNB and TG GM | 100 | 100 |
| 4 | Inaccuracy caused by the synchronization between the UE and the DS-TT | 50 | 50 |
| Total error over Uu | Total inaccuracy for 20 km2 service area (with propagation compensation) = 1.1+1.2+1.3+2 | 705 | 575 |
| Total error of 5GS E2E | Total inaccuracy for 20 km2 service area (with propagation compensation) = 1.1+1.2+1.3+2+3+4 | 855 | 725 |

As shown in Table 2, total inaccuracy for 20 km2 service area (with propagation compensation) = 1.1+1.2+1.3+2+3+4.

According to the analysis given in the above table, the inaccuracy of 15/30KHz SCS can fulfil the TSN clock synchronization requirements of 1000ns for this case.

Observation 3: The inaccuracy of 15/30KHz SCS can fulfil the TSN clock synchronization requirements of 1000ns for smart grid use case.

Proposal 5: The method in Table 2 is used for overall timing error evaluation for smart grid use case.

Considering control-to-control use case, synchronization requirements of 900ns can not be satisfied. The additional enhancements are necessary and following aspects can be considered:

- More accurate DL signal detection (1.2)

- More accurate UL signal transmission (1.3.1)

- More accurate UL signal detection (1.3.2)

- Finer granularity of TA indication (1.3.3)

With above analysis, RAN1 should discuss the potential enhancements for the propagation delay compensation for control-to-control use case.

Proposal 6: RAN1 should discuss the potential enhancements for the propagation delay compensation for control-to-control use case:

- More accurate DL signal detection (1.2)

- More accurate UL signal transmission (1.3.1)

- More accurate UL signal detection (1.3.2)

- Finer granularity of TA indication (1.3.3)

Conclusions

Based on the analysis given above, we have the following Observations and Proposals:

Observation 1: For RTT-based method, the more detail need clarify, e.g. exchange of time information, resource allocation of reference signal, the signaling overhead and so on.

Observation 2: The inaccuracy of 15/30KHz SCS cannot fulfil the TSN clock synchronization requirements of 900ns for UE-to-UE time synchronization use case.

Observation 3: The inaccuracy of 15/30KHz SCS can fulfil the TSN clock synchronization requirements of 1000ns for smart grid use case.

Proposal 1: For Control-to-Control use case and smart grid use case, 65 ns can be baseline for BS transmit timing error.

Proposal 2: Downlink frame timing error is considered in propagation delay compensation evaluation.

Proposal 3: TA based method is baseline for propagation delay compensation.

Proposal 4: The method in Table 1 is used for overall timing error evaluation for Control-to-Control use case.

Proposal 5: The method in Table 2 is used for overall timing error evaluation for smart grid use case.

Proposal 6: RAN1 should discuss the potential enhancements for the propagation delay compensation for control-to-control use case:

- More accurate DL signal detection (1.2)

- More accurate UL signal transmission (1.3.1)

- More accurate UL signal detection (1.3.2)

- Finer granularity of TA indication (1.3.3)

Reference

[1] Chairman notes RAN1#102 e-meeting

[2] R1-2005378, “Other issues for Rel-17 URLLC” vivo, RAN1#102 e-meeting.

[3] R1-1900935, Nokia, “Discussion on the RAN2 LS on TSN requirements evaluation”.