3GPP TSG-RAN1 Meeting #103-e	R1-2007752

# e-Meeting, October 26th-November 13th, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on the definition of nr-DL-PRS-expectedRSTD-r16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The definition of nr-DL-PRS-expectedRSTD-r16 is unclear in the current specification. In previous agreements, nr-DL-PRS-expectedRSTD-r16 actually defines the time difference between the received DL subframe timing that UE is expected to receive DL PRS and the received DL subframe timing that UE is expected to receive DL PRS provided by nr-DL-PRS-ReferenceInfo-r16. In other words, nr-DL-PRS-expectedRSTD-r16 provided to any TRP should follow the same reference timing, which is the received DL subframe timing the UE is expected to receive DL PRS provided by nr-DL-PRS-ReferenceInfo-r16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the definition of nr-DL-PRS-expectedRSTD-r16 as “the time difference between the received DL subframe timing the UE is expected to receive DL PRS and the received DL subframe timing the UE is expected to receive DL PRS provided by nr-DL-PRS-ReferenceInfo-r16”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear UE behavior on how to use the high layer parameter nr-DL-PRS-expectedRSTD-r16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.5 PRS reception procedure |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.1.6.5	PRS reception procedure

================================= unchanged parts omitted==================================

The UE expects to be configured with higher layer parameter nr-DL-PRS-expectedRSTD-r16, which defines the time difference between the received DL subframe timing the UE is expected to receive DL PRS and the received DL subframe timing the UE is expected to receive DL PRS provided by nr-DL-PRS-ReferenceInfo-r16, and DL-PRS-expectedRSTD-uncertainty-r16, which defines a search window around the nr-DL-PRS-expectedRSTD-r16.

================================= unchanged parts omitted================================