3GPP TSG RAN WG1 #103-e                                                           R1-2007945

e-Meeting, October 26th – November 13th, 2020

Source:	Intel Corporation

Title:	Evaluation Results for NR Positioning Performance in I-IoT Scenarios

Agenda item:	8.5.2

Document for:	Discussion and Decision

Introduction

Study item on NR Positioning Enhancements was approved in [1]. The SID document has the following objectives:

- b. Evaluate the achievable positioning accuracy and latency with the Rel-16 positioning solutions in (I)IoT scenarios and identify any performance gaps. [RAN1]

In this document, we provide evaluation results for Rel.16 NR positioning technology and analyze potential enhancements of NR Positioning framework. Our views on other Rel.17 NR positioning design aspects are provided in the companion contributions [2]-[3].

Potential Enhancements of NR Positioning

In this section, we provide a list of potential NR Positioning enhancements evaluated in this document in terms of positioning accuracy and/or physical layer latency. Further details on recommended NR positioning enhancements are provided in our companion contribution [3]. In this document, we analyse accuracy and latency performance of the selected Rel.16 NR positioning solutions and benefits of the following potential design enhancements:

LOS/NLOS classification

Aggregation of NR positioning frequency layers (intra and inter-frequency)

Compensation of network synchronization and gNB/UE TX and RX timing calibration errors

Latency reductions for NR Positioning protocols

NR Positioning support for UEs in RRC_IDLE/RRC_INACTIVE states

# NR Positioning - Evaluation of Positioning Accuracy

Rel-16 Baseline NR Positioning

In this section, we provide a performance analysis for the distance error in horizontal and vertical plane by example of the Rel.16 positioning techniques and agreed RAN1 evaluation scenarios and assumptions. The DL-TDOA, UL-TDOA, and the multi-RTT positioning estimation methods are considered in application to the InF-SH and InF-DH baseline scenarios. The performance results are provided for both FR1 and FR2 frequency bands.

Figure 1 shows performance results for the case of the DL-TDOA positioning technique. Figure on the left shows CDF curve of the horizonal error and figure of the right shows CDF curve for the altitude (vertical) error. The upper row represents the results for the FR1 band, and the bottom row represents the results for the FR2 band.

The solid curves describe the case when both LOS and NLOS links are utilized for the distance estimation. The dashed curves describe the case when the only LOS links are used and the NLOS links are completely discarded. The classification of the LOS and NLOS links is performed based on the ideal knowledge without any practical algorithm consideration.

| DL-TDOA Analysis for FR1 |  |
| --- | --- |
| DL-TDOA Analysis for FR2 |  |

Figure 1: Performance of DL-TDOA based positioning for InF scenarios

As follows from Figure 1, the utilization of the NLOS links in the estimation degrades the algorithm performance significantly. The InF-SH model provides the better percentage of the LOS links compared to the rest of the scenarios, and therefore has better performance characteristics.

Figure 2 shows performance results for the case of the UL-TDOA positioning technique. The same notations are used as for the case shown in Figure 1.

| UL-TDOA Analysis for FR1 |  |
| --- | --- |
| UL-TDOA Analysis for FR2 |  |

Figure 2: Performance of UL-TDOA based positioning for InF scenarios

Similar qualitative conclusions can be done compared to the DL-TDOA results shown in Figure 1.

Figure 3 shows performance results for the case of the Multi-RTT positioning technique. The same notations are used as for the case shown in Figure 1 and Figure 2.

| Multi-RTT Analysis for FR1 |  |
| --- | --- |
| Multi-RTT Analysis for FR2 |  |

Figure 3: Performance of Multi-RTT based positioning for InF scenarios

In general, we have the following key observations based on the presented results for the IIoT scenarios:

  - Performance of the Rel.16 positioning techniques highly depends on the measurement data set used in the estimation

  - Usage of the LOS links only provides better performance compared to the case when both LOS and NLOS links are utilized

  - The target performance can be achieved in InF-SH scenario, with high probability of LOS links

  - The target performance cannot be achieved in InF-DH scenario, with high probability of NLOS links

LOS/NLOS Classification

Figure 4 shows performance results for the case of the DL-TDOA positioning technique. The performance of the practical algorithm for LOS/NLOS classification with hard decision is compared to the outlier rejection RAIM algorithm, the description for these two methods is provided in [3].

| DL-TDOA Analysis for FR1 |  |
| --- | --- |
| DL-TDOA Analysis for FR2 |  |

Figure 4: Performance of DL-TDOA based positioning for InF scenarios

As follows from Figure 4, the algorithm based on the LOS/NLOS classification exhibits better performance compared to the outlier rejection algorithm, known in the literature.

Figure 5 shows performance results for the case of the UL-TDOA positioning technique. The same notations are used as for the case shown in Figure 4.

| UL-TDOA Analysis for FR1 |  |
| --- | --- |
| UL-TDOA Analysis for FR2 |  |

Figure 5: Performance of UL-TDOA based positioning for InF scenarios

Similar qualitative conclusions can be done compared to the DL-TDOA results shown in Figure 4.

Figure 6 shows performance results for the case of the Multi-RTT positioning technique. The same notations are used as for the case shown in Figure 4 and Figure 5

| Multi-RTT Analysis for FR1 |  |
| --- | --- |
| Multi-RTT Analysis for FR2 |  |

Figure 6: Performance of Multi-RTT based positioning for InF scenarios

In general, we have the following key observations based on the presented results for the IIoT scenarios:

  - The algorithm for LOS/NLOS classification provides better performance results compared to the conventional outlier rejection RAIM algorithm known in the literature

Performance of Multi-RTT + AoA (Hybrid Positioning)

In this section, we provide a performance analysis for the distance error in horizontal and vertical plane by example of the Multi-RTT positioning technique. We introduce a vertical Angle of Arrival (vAoA) measurements at the gNB side in addition to the Multi-RTT estimation, which forms a hybrid positioning approach. The practical algorithm for LOS/NLOS links classification is used to improve the baseline performance.

Figure 7 shows the performance results for the case of the Multi-RTT + vAoA positioning technique. Figure on the left shows CDF curve of the horizonal error and figure of the right shows CDF curve for the altitude error. The upper row represents the results for the FR1 band, and the bottom row represents the results for the FR2 band.

| Multi-RTT Analysis for FR1 |  |
| --- | --- |
| Multi-RTT Analysis for FR2 |  |

Figure 7: Performance of Multi-RTT + vertical AoA based positioning for InF scenarios

As follows from Figure 7, the introduction of the AoA measurements at the gNB side in addition to the Multi-RTT approach improves the positioning performance.

We have the following observations based on the presented results for the IIoT scenarios:

  - Combination of the Multi-RTT and vertical AoA measurements further improves positioning performance in the InF scenarios

Aggregation of NR Positioning Frequency Layers

In this section, we provide a performance analysis for the distance error in horizontal and vertical plane by example of the Multi-RTT positioning technique. We introduce an aggregation of the multiple NR positioning frequency layers. An ideal LOS/NLOS links classification is used to improve the baseline performance. In FR1 band, the estimation performance with a 100 MHz bandwidth is compared to the performance with a 400 MHz (aggregated) bandwidth. In case of FR2 band, the estimation performance with a 400 MHz bandwidth is compared to the performance with a 800 MHz (aggregated) bandwidth.

| Multi-RTT Analysis for FR1 |  |
| --- | --- |
| Multi-RTT Analysis for FR2 |  |

Figure 8: Performance of Multi-RTT based positioning for InF scenarios

We have the following observations based on the presented results for the IIoT scenarios:

  - Usage of the larger aggregated bandwidth allows to improve performance significantly and achieve target performance requirements

Compensation of gNB/UE Synchronization and Calibration Errors

In this section, we provide a performance analysis for the distance error in horizontal and vertical plane by example of the Multi-RTT positioning technique. We introduce TX/RX timing errors at the gNB and UE sides. An ideal LOS/NLOS links classification is used to improve the baseline performance. The TX/RX timing errors are compensated applying the method proposed in [3].

In the provided simulation results, the timing errors at the TX and RX sides were modeled as independent identically distributed random variables, using truncated Gaussian distribution in the range [-2×X, +2×X]. At the gNB side, the X value was assigned equal to 5 ns and at the UE side it was equal to 10 ns.

We assume ideal estimation and compensation of the timing error (with no residual error) at the gNB side for the case, when compensation is applied. Therefore, the performance degradation is mainly caused by the residual timing error impact at the UE side.

In case, if no compensation is applied, both gNB and UE timing errors deteriorate the performance.

| Multi-RTT Analysis for FR1 |  |
| --- | --- |
| Multi-RTT Analysis for FR2 |  |

Figure 9: Performance of Multi-RTT based positioning for InF scenarios

We have the following observations based on the presented results for the IIoT scenarios:

  - The TX/RX timing errors can significantly deteriorate the positioning performance

  - Usage of compensation method for TX/RX timing errors substantially improves the performance

  - There is still a margin for improvement between the practical and ideal performance and other timing errors compensation techniques can be potentially considered.

# NR Positioning - Evaluation of Physical Layer Latency

Latency Analysis of Rel.16 NR Positioning Solutions

The following agreement was made by RAN1 WG to perform analysis of the physical layer latency for NR positioning solutions:

- Physical Layer Latency Start and End times are defined as follows: Method; Start; End. UE assisted DL-only & DL-ECID & Multi-RTT; Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message; Successful decoding of the PUSCH carrying the LPP Provide Location Information message. UL-only method & UL ECID & Multi-RTT; Reception by the gNB of the NRPPa measurement request message; The transmission by the gNB of the NRPPa measurement response message. UE-based; Transmission of the PDSCH from the gNB carrying the LPP Request Location Information if applicable, otherwise, - Alt. 1: transmission of the PUSCH carrying the MG Request from the UE. - Alt. 2: Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data - Alt. 3: Start of the Reception of DL PRS Note: Suggest to downselect this at the next meeting. Note: The high layers latency components may be subject to adjustment for different alternatives.; Successful decoding of the PUSCH at gNB carrying the LPP Provide Location Information message if applicable, otherwise Calculation of Location Estimate at the UE.

### Reference System Configuration for Latency Analysis

In order to provide NR positioning physical layer latency analysis, we use the following reference system configuration which is in favor of low latency positioning and were used in NR URLLC studies [7].

- Spectrum – FDD / FR1

- SCS – 30 kHz

- PDCCH

  - Monitoring occasions – 7 per slot [1,0,1,0,1,0,1,0,1,0,1,0,1,0]

  - Duration – 1 symbol

- PDSCH

  - One symbol overlap with PDCCH

  - Duration – 2, 4, 7 symbols (Type B mapping)

  - Slot boundary is respected (i.e. transmission does not cross slot boundary)

- PUSCH

  - Any symbol, subject to slot boundary constraint (i.e. transmission does not cross slot boundary)

  - Duration – 2, 4, 7 symbols (Type B mapping w/ front loaded DMRS)

- PUCCH

  - 7 occasions per slot [1,0,1,0,1,0,1,0,1,0,1,0,1,0] for SR and HARQ feedback

  - Duration – 1 symbol

  - No HARQ – initial transmission is successful

- SRS for positioning

  - Single resource, 1 symbol duration

  - Periodicity – each slot

- DL PRS

  - 18 resources / 4 symbols per resource / 12 Comb-6 symbols per period

  - Periodicity – 20 ms

  - UE DL PRS processing capability – N = 0.5 ms (~12 symbols @30kHz), T = 8 ms

- UE capability for Tproc,1 Tproc,2

  - Capability # 2 (for low latency)

- Measurement gap

  - Type 1: BWP switching = 2 slots @ 30kHz

  - MGL = 5.5 ms, MGRP = 20ms

    - 2 BWP switching times per occasion (from and to active DL BWP) + DL PRS duration => 5 slots

  - Nsample = 4 (RAN4 core measurements requirements)

    - UE is expected to perform measurements on DL PRS resource 4 times (i.e. across 4 periods)

### Latency Analysis of the Rel.16 DL-TDOA / DL-AoD

The latency analysis for the Rel.16 DL-TDOA or DL-AoD positioning methods is summarized in Table 1 and is based on procedure and signalling shown in Figure 14 (please refer to corresponding section in Annex B).

Table 1: Rel.16 DL TDOA/AoD Latency Analysis

| Case 1, InF, FR1, R.16 DL-TDOA/DL-AoD Source NW / Destination NW – (RAN procedure is transparent to source/destination) Positioning technique: DL-TDOA/DL-AoD, type DL only, mode: UE-A, Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  |  |
| Step 1. Location Request | 24.25 symbols 0.8661 ms | Value is provided for L1 components only |
| 1A: PDCCH+PDSCH preparation time | 4.75 symbols 0.1696 ms | Not specified. For analysis, the assumption from [38.824] is used: UE's N2/2 + X for scheduling the initial PDSCH. X = 2/4/8 symbols for SCS = 30/60/120kHz, respectively. N2 = 5.5 symbols@30kHz SCS for UE capability 2 5.5/2+2 = 4.75 |
| 1B: PDCCH+PDSCH alignment time | 7 symbols 0.25 ms | Depends on configuration of PDCCH monitoring occasions, PDSCH transmission duration, time to slot boundary, and TDD configuration Worst case: 7 symbols@30kHz for selected reference system configuration |
| 1C: PDCCH+PDSCH transmission time | 7 symbols 0.25 ms | Depends on PDCCH+PDSCH transmission duration 7 symbols@30kHz for selected reference system configuration |
| 1D: PDCCH+PDSCH processing time | 5.5 symbols 0.1964 ms | Specified as UE PDSCH processing Tproc,1. Tproc,1 = N1 + d11+d2, d11 = 1 symbol, d2=0, N1 = 4.5 for 30 kHz SCS and UE capability 2 5.5 symbols@30kHz for selected reference configuration |
| 1E: [L3 UE LPP processing] | [~280symbols ~10 ms] | Not specified. [The following number is assumed – 5ms DL RRC processing time and 5ms LPP processing time. TBD by RAN WG2] |
| Step 2. Measurement Gap Request | 39.75 symbols 1.4196 ms | Value is provided for L1 components only |
| 2A: [L3 UE RRC processing delay] | 56-140 symbols 2-5ms | [Not defined: 2-5ms UE RRC processing delay is assumed in this analysis TBD by RAN WG2] |
| 2B: SR preparation time | 0 symbols 0 ms | 0ms is assumed since SR can be prepared in advance based on UE implementation |
| 2C: SR alignment time | 2 symbols 0.0714 ms | Depends on configuration of SR monitoring occasions, TDD configuration Worst case: 2 symbols@30kHz for selected reference system configuration |
| 2D: SR transmission time | 1 symbol 0.0357 ms | 1 symbol for selected reference system configuration |
| 2E: SR processing time | 4.5 symbols 0.1607 ms | Not specified. For analysis, the assumption from [38.824] is used. For SR-based PUSCH, gNB's processing time for SR is UE's N1. |
| 2F: PDCCH preparation time | 2.25 symbols 0.0804 ms | Not specified. For analysis, the assumptions from the [38.824] are used to derive PDCCH preparation time equal to N1/2: gNB's PUSCH-to-PDCCH processing time (PDCCH alignment has to be included separately) is UE's N1 + X; gNB's decoding time for the last PUSCH is UE's N1/2 + X; X = 2/4/8 symbols for SCS = 30/60/120KHz, respectively. |
| 2G: PDCCH alignment time | 2 symbols 0.0714 ms | Depends on configuration of PDCCH monitoring occasions and TDD configuration |
| 2H: PDCCH transmission time | 1 symbol 0.0357 ms | 1 symbol for selected reference system configuration |
| 2I: PUSCH preparation time | 6.5 symbols 0.2321 ms | Specified as UE PUSCH preparation time Tproc,2 = N2+d21+d2 (if no BWP switch) N2 = 5.5@30kHz, d21 = 1, d2 = 0. |
| 2J: PUSCH alignment time | 7 symbols 0.2500 ms | Worst case:7 symbols for given reference configuration |
| 2K: PUSCH transmission time | 7 symbols 0.2500 ms | 7 symbols for selected reference system configuration |
| 2L: PUSCH processing time | 6.5 symbols 0.2321 ms | Not specified. For analysis, the assumption from the [38.824] is used. gNB's decoding time for the last PUSCH is UE's N1/2 + X; X = 2/4/8 symbols for SCS = 30/60/120KHz, respectively. |
| 2M: [L3 gNB RRC processing delay] | [~84 symbols ~3 ms] | [Not specified. 3ms are assumed in this analysis for gNB RRC processing delay TBD by RAN WG2] |
| Step 3. Measurement Gap Configuration | 24.25 symbols 0.8661 ms | Value is provided for L1 components only |
| 3A: PDCCH+PDSCH preparation time | 4.75 symbols 0.1696 ms | Same as for 1A |
| 3B: PDCCH+PDSCH alignment time | 7 symbols 0.2500 ms | Same as for 1B |
| 3C: PDCCH+PDSCH transmission time | 7 symbols 0.2500 ms | Same as for 1C |
| 3D: PDCCH+PDSCH processing time | 5.5 symbols 0.1964 ms | Same as for 1D |
| 3E: [L3 UE RRC processing delay] | [~280symbols 10 ms] | [Specified - 10ms TBD by RAN WG2] |
| Step 4. DL PRS Processing | 88,5 ms | Value is provided for L1 components only |
| 4A: DL PRS alignment time | 560symbols 20 ms | Depends on DL PRS periodicity settings Worst case: 20ms for selected reference system configuration |
| 4B: DL PRS processing and report delay | 1918 symbols 68.5 ms | Depends on DL PRS configuration, MG configuration (length and period) and UE DL PRS processing capabilities(N,T). MG should accommodate DL BWP switching. The report delay is defined by 38.133 in Section 9.9.2.5. For considered, reference system configuration it is equal to 3*20+8+0.5 = 68.5ms Note: may be revised by RAN WG4 |
| Step 5. DL PRS Measurement Report | 39.75 symbols 1.4196 ms | Value is provided for L1 components only |
| 5A: [L3: UE RRC + LPP processing delay] | [56-140 symbols 2-5ms] | [Not defined 2-5 ms UE RRC processing time and 5ms UE LPP processing time are assumed TBD by RAN WG2] |
| 5B: SR preparation time | 0 symbols 0 ms | Same as for 2B |
| 5C: SR alignment time | 2 symbols 0.0714 ms | Same as for 2C |
| 5D: SR transmission time | 1 symbol 0.0357 ms | Same as for 2D |
| 5E: SR processing time | 4.5 symbols 0.1607 ms | Same as for 2E |
| 5F: PDCCH preparation time | 2.25 symbols 0.0804 ms | Same as for 2F |
| 5G: PDCCH alignment time | 2 symbols 0.0714 ms | Same as for 2G |
| 5H: PDCCH transmission time | 1 symbol 0.0357 ms | Same as for 2H |
| 5I: PUSCH preparation time | 6.5 symbols 0.2321 ms | Same as for 2I |
| 5J: PUSCH alignment time | 7 symbol 0.2500 ms | Same as for 2J |
| 5K: PUSCH transmission time | 7 symbols 0.2500 ms | Same as for 2K |
| 5L: PUSCH processing time | 6.5 symbols 0.2321 ms | Same as for 2L |
| 5M: [L3 gNB LPP processing time] | [84 symbols 3 ms] | [Not defined. 3 ms are assumed for gNB LPP processing time TBD by RAN WG2] |

Note: All L2/L3 components are under discussion and to be determined by RAN WG2

Based on analysis provided above, we have following observations on DL-TDOA, DL-AoD physical layer latency:

Summary of latency components:

  - Sum of L1 components (except DL PRS alignment time and DL PRS report delay): 4,5714 ms

  - Sum of L2+L3 components: [36 ms TBD by RAN WG2]

  - DL PRS alignment time: 20 ms

  - DL PRS report delay: 68.5 ms

If only L1 latency components are considered (except DL PRS alignment time and DL PRS report delay), it is feasible to meet 10ms target requirements for DL-TDOA/DL-AOD

DL-TDOA / DL-AoD physical layer latency is dominated by the following latency components

  - Higher layer (LPP/RRC) processing/configuration time

  - Multiple DL/UL transactions before actual DL PRS processing, including

    - Location request, measurement gap request/configuration and measurement report

  - DL PRS alignment time

    - Rel.16 Measurement Gap (MG) design has certain limitations in terms of MG periodicity, length, RRC configuration time. Therefore, it is difficult to reduce latency even if DL PRS are allocated at minimum period = 4ms.

    - Periodic DL PRS allocation with low periodicity (min of 4ms). Although it is beneficial for positioning latency, such DL PRS resource utilization is very high

  - DL PRS report delay

    - According to the latest draft of core requirements in TS 38.133 for NR positioning RSTD measurements, UE is expected to measure DL PRS-RSTD on four consecutive DL PRS resource periods, that imposes significant delay and thus need to be reconsidered in Rel.17.

### Latency Analysis of the Rel.16 UL-TDOA / UL-AoA

The latency analysis for the Rel.16 UL-TDOA or UL-AoA positioning methods is summarized in Table 2 and based on procedure and signalling shown in Figure 15 (please refer to corresponding section in Annex B).

Table 2: Rel.16 UL-TDOA/AoA Latency Analysis

| Case 2, InF, FR1, R.16 UL-TDOA/UL-AoA Source NW / Destination NW – (RAN procedure is transparent to source/destination) Positioning technique: UL-TDOA/UL-AoA, type UL only, mode: UE-A, Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Reception by the gNB of the NRPPa measurement request message |
| Step 1. Configuration of SRS for positioning | 24.25 symbols 0.8661 ms | Value is provided for L1 components only |
| 1A: PDCCH+PDSCH preparation time | 4.75 symbols 0.1696 ms | Not specified. For analysis, the assumption from [7] is used: UE's N2/2 + X for scheduling the initial PDSCH. X = 2/4/8 symbols for SCS = 30/60/120kHz, respectively. N2 = 5.5 symbols@30kHz SCS for UE capability 2 5.5/2+2 = 4.75 |
| 1B: PDCCH+PDSCH alignment time | 7 symbols 0.2500 ms | Depends on configuration of PDCCH monitoring occasions, PDSCH transmission duration, time to slot boundary, and TDD configuration Worst case: 7 symbols@30kHz for selected reference system configuration |
| 1C: PDCCH+PDSCH transmission time | 7 symbols 0.2500 ms | Depends on PDCCH+PDSCH transmission duration 7 symbols@30kHz for selected reference system configuration |
| 1D: PDCCH+PDSCH processing time | 5.5 symbols 0.1964 ms | Specified as UE PDSCH processing Tproc,g1. Tproc,1 = N1 + d11+d2, d11 = 1 symbol, d2=0, N1 = 4.5 for 30 kHz SCS and UE capability 2 5.5 symbols@30kHz for selected reference configuration |
| 1E: [L3 UE RRC configuration delay] | [280 symbols 10ms] | [Specified - 10ms TBD by RAN WG2] |
| Step 2 (Alt.1): Activation of SRS for positioning (DCI) | 8.5 symbols 0.3036 ms | Value is provided for L1 components only |
| 2A: PDCCH preparation time | 2.25 symbols 0.0804 ms | Not specified. For analysis, the assumptions from the [7] are used to derive PDCCH preparation time equal to N1/2: gNB's PUSCH-to-PDCCH processing time (PDCCH alignment has to be included separately) is UE's N1 + X; gNB's decoding time for the last PUSCH is UE's N1/2 + X; X = 2/4/8 symbols for SCS = 30/60/120KHz, respectively. |
| 2B: PDCCH alignment time | 2 symbols 0.0714 ms | Depends on configuration of PDCCH monitoring occasions and TDD configuration |
| 2C: PDCCH transmission time | 1 symbol 0.0357 ms | 1 symbol for selected reference system configuration |
| 2D: PDCCH processing time | 3.25 symbols 0.1161 ms | Not specified: For analysis, it is assumed to be equal to ½ of UE PUSCH preparation time Tproc,2 = N2+d21+d2 (if no BWP switch) i.e. Tproc,2/2 N2 = 5.5@30kHz, d21 = 1, d2 = 0. |
| Step 2’(Alt.2): Activation of SRS for positioning (MAC CE) | 24.25 symbols 0.8661 ms | Value is provided for L1 components only |
| 2’A: PDCCH+PDSCH preparation time | 4.75 symbols 0.1696 ms | Same as for 1A |
| 2’B: PDCCH+PDSCH alignment time | 7 symbols 0.2500 ms | Same as for 1B |
| 2’C: PDCCH+PDSCH transmission time | 7 symbols 0.2500 ms | Same as for 1C |
| 2’D: PDCCH+PDSCH processing time | 5.5 symbols 0.1964 ms | Same as for 1D |
| 2’E: [L2/L3 component: UE MAC processing delay] | [28-84 symbols 1-3ms] | [Not defined: 1-3ms are assumed TBD by RAN WG2] |
| Step 3. SRS for positioning Tx & Rx | 29 symbols 1.0357 ms | Value is provided for L1 components only |
| 3A: SRS for positioning preparation time | 0 ms | Not defined. 0ms is assumed for analysis |
| 3B: SRS for positioning alignment time | 14 symbols 0.5ms | Depends on SRS resource allocation. Worst case: 14 symbols for selected reference configuration |
| 3C: SRS for positioning transmission time | 1 symbol 0.0357 | 1 symbol for selected reference configuration |
| 3D: SRS for positioning processing time by gNB | [14 symbols 0.5 ms] | [Not defined: 0.5 ms is assumed. Definition is in scope of RAN WG4] |
| Step 4. gNB Measurement Report | 0 | Value is provided for L1 components only |
| 4A: [L3 gNB NRPPa preparation time] | [84symbols 3 ms] | [Not defined: 3 ms is assumed. TBD by RAN WG2] |
| End trigger |  | The transmission by the gNB of the NRPPa measurement response message |

Note: All L2/L3 components are under discussion and to be determined by RAN WG2

Based on analysis provided above, we have following observations on UL-TDOA, UL-AoA physical layer latency:

Summary of latency components:

  - Sum of L1 components: 2,7678 ms

  - Sum of L2/L3 components: 16 ms

Physical layer latency for UL-TDOA and UL-AoA is dominated by SRS for positioning configuration component (RRC configuration delay) that utilizes 10 out of 16 ms total latency for considered reference system configuration.

### Latency Analysis of the Rel.16 Multi-RTT

The latency analysis for the Rel.16 Multi-RTT positioning method is summarized in Table 3 and is based on procedure and signalling shown in Figure 16 (please refer to corresponding section in Annex B).

The latency for Multi-RTT positioning can be estimated as sum of latency components for UL-TDOA and DL -TDOA positioning methods. In the best case, if DL and UL transactions are executed in parallel the latency of multi-RTT can be estimated as max(DL-TDOA, UL-TDOA) latency components. In the worst case, it can be estimated as a sum of DL-TDOA and sum of UL-TDOA latency components assuming that all transactions are performed consecutively. This analysis is presented in Table 3.

Table 3: Rel.16 Multi-RTT Latency Analysis

| Case 3, InF, FR1, R.16 Multi-RTT Source NW / Destination NW – (RAN procedure is transparent to source/destination) Positioning technique: Multi-RTT, type DL+UL, mode: UE-A, Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Reception by the gNB of the NRPPa measurement request message |
| Step 1. Configuration of SRS for positioning | L1: 0.8661 [L3: 10.0] | L1: gNB signaling to UE to configure SRS for positioning – 0.5088 ms [L3: UE RRC configuration delay – 10ms - TBD by RAN WG2] |
| Step 2: Activation of SRS for positioning (MAC CE) | L1: 0.8661 [L2: 1.0] | L1: gNB signaling to UE to activate SRS for positioning transmission – 0.5088 ms [L2: UE MAC CE processing delay – 1 ms - TBD by RAN WG2] |
| Step 3. SRS for positioning TX & RX | L1: 1.0357 | L1: transmission of SRS for positioning by UE and its reception by gNB – 1.0357 ms |
| Step 4. gNB NRPPa preparation time for report to LMF | [L3: 3.0] | [L3: gNB NRPPa preparation time for report to LMF – 3 ms - TBD by RAN WG2] |
| Step 5. Location information request | L1: 0.8661 [L3: 10.0] | L1: gNB transmits to UE location request - 0.5088 ms [L3: 5 ms DL RRC processing time and 5ms LPP processing time - TBD by RAN WG2] |
| Step 6. MG Request | L1: 1.4196 [L3: 5-8] | L1: UE requests gNB to provide MG – 1.0266 ms [L3: UE RRC processing delay – 2-5 ms and gNB RRC processing delay – 3ms TBD by RAN WG2] |
| Step 7. MG Configuration | L1: 0.8661 [L3: 10.0] | L1: gNB provides MG configuration to UE RRC signaling – 0.5088 ms [L3: UE RRC configuration delay – 10 ms - TBD by RAN WG2] |
| Step 8. DL PRS Processing | L1: 88.5 | DL PRS alignment time and DL PRS processing and report delay |
| Step 9. DL PRS Measurement Report | L1: 1.4196 [L3: 3.0] | L1: UE reports measurement results to gNB [L3: gNB LPP processing time - 3ms; 2-5ms UL RRC processing time; 5ms UE LPP processing time are assumed. TBD by RAN WG2] |
| End trigger |  | The transmission by the gNB of the NRPPa measurement response message |

Note: All L2/L3 components are under discussion and to be determined by RAN WG2

Based on analysis provided above, we have following observations on Multi-RTT physical layer latency:

Summary of latency components:

  - Sum of L1 components (except DL PRS alignment time and DL PRS report delay: 7,3393 ms

  - Sum of L2/L3 components: 45 ms (TBD by RAN WG2)

  - DL PRS alignment time: 20 ms

  - DL PRS report delay: 68.5 ms

Physical layer latency for Multi-RTT is dominated by

  - Higher layer (LPP/RRC) processing/configuration time

  - DL PRS alignment time

  - DL PRS report delay

Latency Reductions for NR Positioning Protocols

The following solutions can be considered for physical layer latency reduction of NR positioning solutions

Based on physical layer latency analysis provided in our companion contribution [2], we have following considerations on enhancements to reduce latency of NR positioning protocols:

The following solutions can be considered for physical layer latency reduction of NR positioning solutions

            - Pre-configuration and minimization of DL/UL transactions

              - DL PRS assistance information can be pre-configured to UE. Multiple DL PRS configurations can be associated with DL PRS configuration ID and activated when necessary

              - SRS for positioning configuration information can be pre-configured to UE. Multiple configurations of SRS for positioning can be associated with SRS for positioning configuration ID and activated when necessary

            - BWP switching consideration

              - Operation w/o BWP switching is beneficial for latency reduction and can be achieved by implementation and proper configuration

            - Measurement gaps (MG) optimizations

              - MG-less operation - UE may operate w/o measurement gaps to process DL PRS

              - Support of semi-persistent a-periodic MGs, their pre-configuration and association with MG configuration ID

            - DL PRS alignment time

              - Support of semi-persistent a-periodic DL PRS transmissions

            - DL PRS report delay

              - Optimization of MG configurations

              - Support of single shot DL PRS processing on resource (No need to mandate 4 measurements per resource)

              - Low latency DL processing capabilities

            - Dynamic activation of DL PRS and SRS for positioning transmissions by gNB

            - Dynamic scheduling request for DL PRS and/or SRS for positioning transmission

            - Support of NR positioning techniques for UEs in RRC_INACTIVE state w/ and w/o transition to RRC_CONNECTED state including the following enhancements of two-or-four step RACH procedures:

              - Transmission of SRS for positioning following PRACH and MSG-A transmission

              - Transmission of SRS for positioning triggered by signaling from gNB

              - Request of DL PRS allocation by UE and DL PRS activation by gNB

              - UE DL PRS measurement

              - UE measurement report

Many of listed above enhancements have overlap in terms of scope with RAN WG2 and thus need to be jointly considered with RAN WG1.

| DL-TDOA | UL-TDOA | Multi-RTT |
| --- | --- | --- |

Figure 10: Low latency procedures for NR UE-assisted positioning (DL-TDOA, UL-TDOA, Multi-RTT)

In order to perform latency analysis for enhanced NR positioning solutions, we use the following assumptions:

- gNB location request: 2 symbols

  - Provides activation of DL PRS, UL SRS transmission, and scheduling information for UE measurement report (e.g. resources for UE measurement report transmission can be associated with resources of SRS for positioning or directly indicated)

  - DL PRS and UL SRS for positioning transmission are performed sequentially

- No MG configuration and BWP switching considerations

- DL PRS resource: 2 symbols, Comb-6, 4 symbols in total per DL PRS Resource Set

- SRS for positioning configuration – same as in reference system configuration

  - DL PRS processing delay – 2 ms (in scope of RAN WG4)

- UE measurement report

  - 7 symbols PUSCH duration (same as in reference system configuration)

- Other assumptions are the same as in reference system configuration

| Case 3, InF, FR1, R17 NR Positioning techniques Source NW / Destination NW – (RAN procedure is transparent to source/destination) Positioning technique: R17 NR Positioning techniques, type DL+UL, mode: UE-A, Initial and Final RRC States: CONNECTED |  |  |
| --- | --- | --- |
| Latency Components | Value Range, ms | Description of Latency Component |
| Start trigger |  | Reception by the gNB measurement request message |
| Step 1. gNB location request | 24.25 symbols 0.8861 ms | Value is provided for L1 components only |
| 1A: PDCCH+PDSCH preparation time | 4.75 symbols 0.1696 ms | Not specified. For analysis, the assumption from [7] is used: UE's N2/2 + X for scheduling the initial PDSCH. X = 2/4/8 symbols for SCS = 30/60/120kHz, respectively. N2 = 5.5 symbols@30kHz SCS for UE capability 2 5.5/2+2 = 4.75 |
| 1B: PDCCH+PDSCH alignment time | 7 symbols 0.2500 ms | Depends on configuration of PDCCH monitoring occasions, PDSCH transmission duration, time to slot boundary, and TDD configuration Worst case: 7 symbols@30kHz for selected reference system configuration |
| 1C: PDCCH+PDSCH transmission time | 7 symbols 0.2500 ms | Depends on PDCCH+PDSCH transmission duration 7 symbols@30kHz for selected reference system configuration |
| 1D: PDCCH+PDSCH processing time | 5.5 symbols 0.1964 ms | Specified as UE PDSCH processing Tproc,1. Tproc,1 = N1 + d11+d2, d11 = 1 symbol, d2=0, N1 = 4.5 for 30 kHz SCS and UE capability 2 5.5 symbols@30kHz for selected reference configuration |
| 1E: [L2 UE MAC CE processing time] | [~28-84 symbols 1-3 ms] | [In this analysis MAC CE based on PDCCH+PDSCH is assumed for gNB location request. Other possible option is to use PDCCH only that can further reduce latency TBD by RAN WG2] |
| Step 2: DL PRS transmission | 64 symbols 2.2858 ms | Value is provided for L1 components only |
| 2A. DL PRS alignment time | 4 symbols 0.1429 ms | Worst case: 4ms for selected reference system configuration to reflect slot boundary |
| 2B. DL PRS transmission time | 4 symbols 0.1429 ms |  |
| 2C. DL PRS processing and report delay | [~56 symbols 2 ms] | [ Not defined. It is assumed that new UE capabilities for low latency NR positioning are to be introduced. Definition is in scope of RAN WG4] |
| Step 3. SRS for positioning transmission | 29 symbols 1.0357ms | Value is provided for L1 components only |
| 3A: SRS for positioning preparation time | 0 ms | Not defined. 0ms is assumed for analysis (can be prepared in advance, e.g. during DL PRS processing or earlier) |
| 3B: SRS for positioning alignment time | 14 symbols 0.5ms | Depends on SRS resource allocation. Worst case: 14 symbols for selected reference configuration |
| 3C: SRS for positioning transmission time | 1 symbol 0.0357 | 1 symbol for selected reference configuration |
| 3D: SRS for positioning processing time | [14 symbols 0.5 ms] | [Not defined: 0.5 ms is assumed. Definition is in scope of RAN WG4] |
| Step 4. UE measurement report | 20.5symbols 0.7321ms | Value is provided for L1 components only |
| 4A: [L2 MAC CE preparation] | [~28 symbols 1 ms] | [In this analysis MAC CE based on PUSCH is assumed. TBD by RAN WG2] |
| 4B: PUSCH alignment time | 7 symbols 0.2500 ms |  |
| 4C: PUSCH transmission time | 7 symbols 0.2500 ms | 7 symbols for selected reference system configuration |
| 4D: PUSCH processing time | 6.5 symbols 0.2321 ms | Not specified. For analysis, the assumption from the [7] is used. gNB's decoding time for the last PUSCH is UE's N1/2 + X; X = 2/4/8 symbols for SCS = 30/60/120KHz, respectively. |
| End trigger |  | Transmission by the gNB measurement response message |

Based on analysis in table above, we have following observations

- DL-TDOA latency of L1 components = Step 1 + Step 2 + Step 4 = 3.8839 ms

  - Estimated higher layer signaling time is up to 4ms

- UL-TDOA latency of L1 components = Step 1 + Step 3 = 1.9018 ms

  - Estimated higher layer signaling time is up to 3ms

- Multi-RTT latency of L1 components = Step 1 + Step 2 + Step 3 = 4.1875 ms

  - It is assumed that Step 4 is done in parallel with Step 3B,3C,3D

  - Estimated higher layer signaling time is up to 4ms

NR Positioning Latency Reduction for UEs in RRC_IDLE / INACTIVE States

Another promising direction for NR positioning latency reduction is support of low latency positioning for UEs in RRC_IDLE / RRC_INACTIVE states.

Considerations for UEs in RRC_IDLE state

In our view, NR positioning support for UEs in RRC_IDLE state is rather challenging due to various reasons. UEs in this state do not have RRC context and thus anyway multiple transactions are needed over Uu air-interface and at network side to acquire RRC context. Another issue is security for NR Positioning of RRC_IDLE UEs. Overall, multiple transactions over Uu air-interface as well within CN (see Figure 11) are needed to support it and thus required latency characteristics may be challenging to achieve.

Figure 11: Transition from RRC_IDLE to RRC_CONNECTED state

Considerations for UEs in RRC_INACTIVE state

UEs in RRC_INACTIVE state has valid RRC context and simplified transition to RRC_CONNECTED state that can simplify integration of NR positioning procedures.

Figure 12: Transition from RRC_INACTIVE to RRC_CONNECTED state

In terms of NR positioning enhancements, the support of the following functionality should be considered:

- Support of NR positioning techniques for UEs in RRC_INACTIVE state w/ and w/o transition to RRC_CONNECTED state including the following enhancements

  - Transmission of SRS for positioning following PRACH and MSG-A transmission

  - Transmission of SRS for positioning triggered by signaling from gNB

  - Request of DL PRS allocation by UE and DL PRS activation by gNB

  - UE DL PRS measurement and report

Figure 13: NR positioning in RRC_INACTIVE state

The latency analysis of transition time from INACTIVE to CONNECTED state was provided in [9]. It is obvious that latency is different for different configurations/deployment:

- The maximum latency = 18.5 ms;

- The minimum latency = 11.3 ms;

The introduction of two step RACH procedure in Rel.16, can further reduce latency on ~2ms relative to 4 step RACH procedure. The support of NR positioning may add additional 2-5 ms delay if transition to CONNECTED state is performed by UE. Otherwise, if UE stays in RRC_INACTIVE state the latency limit in the order of 10ms can be achieved.

# Conclusion

In this contribution, we have provided initial analysis of NR positioning performance for IIoT InF scenarios. Our analysis has shown that if enhancements of Rel.16 solutions such as LOS/NLOS classification, DL PRS frequency layer aggregation, and accurate TX/RX timing error calibration and compensation are used then it is feasible to meet the stringent accuracy target requirements in selected IIoT deployment scenarios as well as meet latency targets of NR positioning protocols are optimized.

# References

- RP-193237, “New SID on NR Positioning Enhancements”, Qualcomm Incorporated, Sitges, Spain, December 9th – 12th, 2019.

- R1-2007944, “Remaining Details of Evaluation Methodology for NR Positioning”, Intel Corporation, October 26 – November 13, 2020.

- R1-2007946, “NR Positioning Design Enhancements”, Intel Corporation, October 26 – November 13, 2020.

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”, V16.0.0 (2019-10)

- RP-200927, “SR for RAN 88e R17 Positioning”, June 29 – July 2, 2020.

- 3GPP TS 38.305 V16.2.0 (2020-09), ‘User Equipment (UE) positioning in NG-RAN (Release 16)

- 3GPP 38.824, “Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)”, v16.0.0

- RAN WG2 email discussion “[Post111-e][625][POS] End-to-end latency analysis” (Intel)

- 3GPP TR 37.910, “Study on self evaluation towards IMT-2020 submission”, 16.1.0

# Annex A – Evaluation Assumptions

In this section, we provide a summary of evaluation assumptions that were used for positioning performance evaluation this contribution. Please refer to Table 1, 2, 3 for evaluation assumptions on InF scenarios, NR DL and UL positioning evaluation assumptions.

Table 4: Indoor factory scenario evaluation assumptions

| Parameter |  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- | --- |
| Carrier frequency, GHz |  | 3.5GHz | 28GHz |
| Bandwidth, MHz |  | 100MHz | 400MHz |
| Channel model |  | InF-SH, InF-DH (baseline, optional 1, optional 2) | InF-SH, InF-DH (baseline, optional 1, optional 2) |
| Layout | Hall size | InF-SH: 300x150 m InF-DH: 120x60 m |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |
|  | Room height | 10m |  |
|  | Penetration loss | 0dB |  |
|  | Number of floors | 1 |  |
| gNB noise figure, dB |  | 5dB | 7dB |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |
| UE noise figure, dB |  | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm |  | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE antenna configuration |  | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU |
| UE antenna radiation pattern |  | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| UE drop procedure |  | 100% indoor, uniformly distributed over the horizontal area |  |
| UE mobility |  | 3km/h |  |
| UE antenna height |  | 1.5m |  |
| Min gNB-UE distance (2D), m |  | 0m |  |
| gNB antenna height |  | Fixed height of 8m |  |
| Clutter density: r |  | InF-SH: 60% InF-DH: - baseline: 40% - optional 1: 40% - optional 2: 60% |  |
| Clutter height: hc |  | InF-SH: 6m InF-DH: - baseline: 2m - optional 1: 3m - optional 2: 6m |  |
| Clutter size: dclutter |  | InF-SH: 2m InF-DH: - baseline: 2m - optional 1: 5m - optional 2: 2m |  |
| Note 1: According to 3GPP TR 38.802 |  |  |  |

Table 5: NR DL positioning evaluation assumptions

| Parameters | FR1/FR2 specific values |
| --- | --- |
| Signal bandwidth | - FR1: 100 MHz - FR2: 400 MHz |
| PRS Signals | Evaluated candidate physical structures for PRS transmission |
| DL PRS Physical Structure | - Single port - Density: Comb-2 - Number of occupied symbols: 2 - Boosting = 0dB - IBE – Not applicable |
| TX/RX Settings | TX/RX Beamforming Assumptions in FR1 and FR2 |
| FR1 | - PRS structure w/ all REs used for transmission - TX: No TX beam sweeping, Single TX Port, Quasi-Omni Pattern - RX: Two X-pol RX Ports, Quasi-Omni Pattern |
| FR2 | - PRS structure w/ all REs used for transmission - TX: TX beam sweeping, 8 TX analogue pre-coder states (one OFDM symbol per TX beam) - RX: Quasi-Omni Pattern |
| BS beam forming | - FR2 – 8 TX analogue beams: [-52.2 : … : 52.5], beam width 15o, DFT vectors to scan in horizontal plane |

Table 6. NR UL positioning evaluation assumptions

| Parameters | FR1/FR2 Specific Values |
| --- | --- |
| Signal bandwidth | FR1: 100 MHz FR2: 400 MHz |
| PRS Signals | Evaluated candidate physical structures for PRS transmission |
| UL PRS Physical Structure (Example) | Single port Density: Comb-2 Number of occupied symbols: 1 Boosting = 0dB IBE – Not applicable |
| TX/RX Settings | TX/RX Beamforming Assumptions in FR1 and FR2 |
| FR1 | PRS structure w/ all REs used for transmission TX: No TX beam sweeping, Single TX Port, Quasi-Omni Pattern RX: Two X-pol RX Ports, Quasi-Omni Pattern |
| FR2 | - PRS structure w/ all REs used for transmission - TX: No TX beam sweeping, Single TX Port, Quasi-Omni Pattern - RX: Rx beam sweeping, 8 RX analogue pre-coder states (one OFDM symbol per RX beam), Two X-pol RX Ports |
| gNB beam forming | - FR2 – 8 TX analogue beams: [-52.2 : … : 52.5], beam width 15o, DFT vectors to scan in horizontal plane |
| RX Processing |  |
| BS synchronization error | - FR1, FR2: 0 ns |
| Measurement selection procedure | Earliest timing measured across TX-RX beam pairs for given link |
| First Arrival Path (FAP) Timing Estimation | - FAP timing estimation based on adaptive threshold for CIR processing |
| Angle Estimation | - Differential based processing based on the detected FAP |

Table 7. NR DL/UL positioning RX processing assumptions

| RX Processing |  |
| --- | --- |
| BS synchronization error | - FR1/ FR2: 0 ns |
| Measurement selection procedure | Earliest timing measured across TX-RX beam pairs for given link |
| First Arrival Path (FAP) Timing Estimation | - FAP timing estimation based on adaptive threshold for CIR processing |
| Positioning algorithm assumptions | - UE altitude value is assumed to be below gNBs altitude value |

# Annex B – Rel.16 NR Positioning Procedures

## DL-TDOA/DL-AOD

Figure 14 shows the possible Rel.16 messaging between the LMF, the gNBs and the UE to perform DL-TDOA/DL-AOD positioning procedure [8].

Figure 14: Rel.16 DL-TDOA positioning procedure (also applicable for DL-AoD)

## UL-TDOA/UL-AOA

Figure 15 shows the messaging between the LMF, the gNBs and the UE to perform UL-TDOA NR positioning procedure [8].

Figure 15: Rel.16 UL-TDOA positioning procedure (also applicable for UL-AoA)

## Multi-RTT

Figure 16 shows the messaging between the LMF, the gNBs and the UE to perform LMF-initiated Location Information Transfer Procedure for Multi-RTT [8].

Figure 16: Rel.16 Multi-RTT NR positioning procedure