3GPP TSG RAN WG1 Meeting #103-e	R1-2008389

e-Meeting, October 26th – November 13th, 2020

Source:	Sharp

Title:	Remaining issues on resource allocation mode 2 for NR sidelink

Agenda Item:	7.2.4

Document for:	Discussion and Decision

Introduction

In this document, we share our view on a further clarification to Step 5 in the sensing procedure for NR sidelink.

Discussion

Clarification on Step 5 in the sensing procedure

In RAN1#99, it was agreed that procedure of mapping of periodic semi-persistent resources into the resource selection window is reused from LTE V2V and the corresponding specs text from sub-clause 8.1.4 in TS38.214 on the “no-monitoring” case (i.e. Step 5) is shown as follows,

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 ### […] 5) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: - the UE has not monitored slot tmSL in Step 2. - for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList and a hypothetical SCI format 1-A received in slot tmSL with "Resource reservation period" field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met. 6) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: a) the UE receives an SCI format 1-A in slot tmSL, and "Resource reservation period" field, if present, and "Priority" field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213]; b) the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX; c) the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 1-A, is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of ms. ### […]

For the “no-monitoring” case as specified in step 5, the UE sets the “Resource reservation period” field in the hypothetical SCI format 1-A as any allowed reservation interval. It was also noticed that the condition c in step 6 is invoked when the UE determines whether to exclude a candidate resource Rx, y from set SA in step 5, and accordingly the UE determines the set of RBs and slots according to clause 8.1.5 based on the hypothetical SCI format 1-A. We hereby abstract the specs text of determining subchannels and slots based on SCI indication, as follows. It is observed that SCI format 1-A indicates the number of allocated sub-channels by FDRA and the number of slots (i.e. number of actual resources) by TDRA. Regarding the set of RBs (i.e. subchannels), it was already captured as “the hypothetical SCI indicating all subchannels in this slot”, while it is ambiguous for the indication of the number of actual resources N (i.e. the set of slots as specified in Step 6c) in the hypothetical SCI format 1-A. To reuse the LTE V2V mechanism, it is straightforward that the hypothetical SCI format 1-A indicates N=1.

### 8.1.5 UE procedure for determining slots and resource blocks for PSSCH transmission associated with an SCI format 1-A The set of slots and resource blocks for PSSCH transmission is determined by the resource used for the PSCCH transmission containing the associated SCI format 1-A, and fields "Frequency resource assignment", "Time resource assignment" of the associated SCI format 1-A as described below. "Time resource assignment" carries logical slot offset indication of N = 1 or 2 actual resources when sl-MaxNumPerReserve is 2, and N = 1 or 2 or 3 actual resources when sl_MaxNumPerReserve is 3, in a form of time RIV (TRIV) field which is determined as follows: if N=1 TRIV=0 elseif N=2 TRIV=t1 else if t2-t1-1≤15 TRIV=30t2-t1-1+t1+31 else TRIV=3031-t2+t1+62-t1 end if end if where the first resource is in the slot where SCI format 1-A was received, and ti denotes i-th resource time offset in logical slots of a resource pool with respect to the first resource where for N = 2, 1≤t1≤31; and for N = 3, 1≤t1≤30, t1<t2≤31. […]

Proposal 1: Further clarify that the hypothetical SCI format 1-A indicates N=1 actual resource in step 5 of sensing procedure.

A TP is provided in section 4 for clause 8.1.4 in TS38.214.

Conclusion

In this contribution, we discuss the need for a further clarification to Step 5 in the sensing procedure and make the following proposal.

Proposal 1: Further clarify that the hypothetical SCI format 1-A indicates N=1 actual resource in step 5 of sensing procedure.

Text proposal

### 8.1.4	UE procedure for determining the subset of resources to be   reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

< Unchanged parts are omitted >

5)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

-	the UE has not monitored slot tmSL in Step 2.

-	for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList and a hypothetical SCI format 1-A received in slot tmSL with "Resource reservation period" field set to that periodicity value, indicating all subchannels of the resource pool in this slot and N=1 actual resource, condition c in step 6 would be met.

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

a)	the UE receives an SCI format 1-A in slot tmSL, and "Resource reservation period" field, if present, and "Priority" field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213];

b)	the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than  ThprioRX;

c)	the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 1-A,  is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of ms.

< Unchanged parts are omitted >