3GPP TSG RAN WG1 #103-e		                   R1-2008413

e-Meeting, October 26th – November 13th, 2020

Agenda item:	8.4.4

Source:	LG Electronics

Title:	Discussions on other aspects of NTN

Document for:	   Discussion and Decision

- Introduction

In the WID [1], the following topics are listed for further discussion and specifying if beneficial and needed.

- Enhancement on the PRACH sequence and/or format and extension of the ra-ResponseWindow duration (in the case of UE with GNSS capability but without pre-compensation of timing and frequency offset capabilities) [RAN1/2].

- Feeder link switch [RAN2,RAN1]

- Beam management and Bandwidth Parts (BWP) operation for NTN with frequency reuse [RAN1/2]

  - Including signalling of polarization mode

Also, in RAN1#102-e meeting, following agreements regarding BM and polarization signaling were made as captured below:

Agreement:

One-beam per cell and multiple-beam per cell are supported in existing NR specifications and are baseline for NR NTN.

- FFS: The need for potential enhancement for beam management

- FFS: The need for potential enhancement on association of SSBs, beams and BWPs

Agreement:

Potential enhancements for support of polarisation signalling in NR NTN can consider at least the following:

- Configuration of DL and UL transmit polarization including Right hand and Left hand circular polarizations (RHCP, LHCP)

- Network broadcast DL and UL transmit polarization configuration

- UE polarization capability (RHCP, LHCP, Linear)

- Dependence of polarisation signaling on deployment scenarios. For example,

  - Resource reuse mode with/without polarization for the beam management enhancement

  - Fixed polarization per cell/beam for polarization reuse and circular polarisation with intra-UE and inter-UE multiplexing (intra-UE and inter-UE) signalling

In this contribution, we discuss on other aspects including PRACH, polarization signaling and BWP operation in NTN.

- Discussion

Enhancement on PRACH

In general, GNSS-equipped UE can obtain its location and reference time through GNSS. In this case, existing Rel-15 PRACH formats and preamble sequences can be reused. However, if GNSS-equipped UE is in a particular situations (e.g., indoor, underground, etc.), it may be hard to obtain accurate the location and the reference time because the accuracy of the GNSS may be reduced. In this situation, existing Rel-15 PRACH formats and preamble sequences cannot be reused. Therefore, following four options can be considered for the enhanced PRACH formats and/or preamble sequences in TR38.821 [2].

- Option 1: A single Zadoff-Chu sequence based on larger SCS, repetition number.

- Option 2: A solution based on multiple Zadoff-Chu sequences with different roots.

- Option 3: Gold/m-sequence as preamble sequence with additional process, e.g., modulation and transform precoding.

- Option 4: A single Zadoff-Chu sequence with combination of scrambling sequence.

Regarding this issue, it is necessary to discuss further in Rel-17 NTN WI phase. If it is agreed that the enhanced PRACH formats and/or preamble sequences are necessary, and if one of the above options should be supported, the Option 1 is preferred over other options due to the minimum impact on specification considering the limited time budget in Rel-17 NTN.

Proposal 1. If enhanced PRACH formats and/or preamble sequences are necessary and supported in Rel-17 NTN, the option with simple modification, such as a single Zadoff-Chu sequence based on larger SCS and repetition number, is preferred.

Polarization mode signaling

In this sub-section, we discuss on the polarization mode signalling. Circular polarization can be consists of two perpendicular electromagnetic plane waves of equal amplitude with 90° phase difference as shown in Figure 1. Depending on phase difference, rotating direction can be determined (e.g. LHCP or RHCP). Compared to linear polarization, circular polarization can provide more robustness in signal degradation caused by atmospheric condition, and thus can provide higher link reliability.

Figure 1. Linear polarization vs circular polarization

Inter-cell interference mitigation is important issue not only for NTN but also for TN. This interference mitigation can be achieved by configuring different polarization from neighboring cells. By applying polarization domain in addition to frequency domain, network can be more flexible on configuring resources, and therefore it is expected to achieve higher spectral efficiency or network throughput gain. So, we think polarization mode (RHCP, LHCP) signaling is beneficial for NTN, and it can be broadcasted via SIB. One consideration point is whether additional polarization domain can be further extended to sequence initialization or RACH procedure in order for more efficient interference management.

Proposal 2. For NTN, support polarization mode (RHCP, LHCP) signaling broadcasted via SIB.

BWP operation in NTN

In NR, BWP switching is based on the DCI, MAC-CE or inactivity timer (bwp-InactivityTimer). For example of timer based operation as shown in Figure 2, when UE receives PDCCH scheduling DL or UL, the inactivity timer (re)starts. If there is no additional scheduling command before the timer expires, UE will perform BWP switching to initial BWP or default BWP. This is the way to resolve ambiguity when gNB and UE have different understanding on BWP switching. If a UE misses PDCCH carrying BWP switching indication, gNB can notice this situation at least after round-trip time + T_timer from the time of a PDCCH transmission. Thus, as RTT increase, this issue becomes non-negligible.

Figure 2. Illustration of timer based BWP switching

In NTN, the maximum RTT can be up to 21ms, 42ms and 542ms for LEO-600km, LEO-1200km and GEO, respectively. Hence, the careful BWP switching needs to be considered in order not to waste scheduling resources. To this end, following enhancement can be considered.

- Enhancement on bwp-InactivityTimer including value range extension and (re)start timing.

- PDSCH and PUSCH transmission after ACK transmission of BWP switching command.

Proposal 3. For NTN, potential enhancement on BWP switching can consider at least following aspects:

- Enhancement on bwp-InactivityTimer including value range extension and (re)start timing,

- PDSCH transmission after transmission of ACK for BWP switching command.

- Conclusion

In this contribution, we discussed on the enhancement on PRACH, polarization signaling, and BWP operation in NTN. Based on the above discussion, we have following observations and proposals:

Proposal 1. If enhanced PRACH formats and/or preamble sequences are necessary and supported in Rel-17 NTN, the option with simple modification, such as a single Zadoff-Chu sequence based on larger SCS and repetition number, is preferred.

Proposal 2. For NTN, support polarization mode (RHCP, LHCP) signaling broadcasted via SIB.

Proposal 3. For NTN, potential enhancement on BWP switching can consider at least following aspects:

- Enhancement on bwp-InactivityTimer including value range extension and (re)start timing,

- PDSCH transmission after transmission of ACK for BWP switching command.

Reference

[1] RP-201256, “Solutions for NR to support non-terrestrial networks (NTN)”, Thales

[2] 3GPP TR 38.821 V1.0.0 (Release 16)