3GPP TSG RAN WG1#103-e		R1-2009429

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	7.2.2

Source:	Moderator (Lenovo)

Title:	Summary of [103-e-NR-NRU-02] email discussion

Document for:	Discussion, Decision

# Introduction

According to the guidance by RAN1 (vice-)chairman, this email discussion is to be finalised by 29 October; if necessary, followed by endorsing the corresponding TPs by 5 November.

# Summary of Discussion and Suggestions

## Proposed Agreement #1

- If neither CO-DurationPerCell-r16 nor slotFormatCombinationId are provided in DCI format 2_0 and if ChannelAccessMode-r16 = semistatic, the UE assumes the COT ends Tz=max(0.05Tx, 100us) before the start of the next fixed frame period.

- For the case that a set of symbols of a slot indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE detects a DCI format 2_0 providing a format for the slot using a slot format value other than 255, if the UE is configured by higher layers to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE receives the PDSCH or the CSI-RS in the set of symbols of the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as downlink, and, if applicable, the set of symbols is within remaining channel occupancy duration

TP#A for 38.213

Reasons for change:

Capture the following agreement:

If neither CO-DurationPerCell-r16 nor slotFormatCombinationId are provided in DCI format 2_0 and if ChannelAccessMode-r16 = semistatic, the UE assumes the COT ends Tz=max(0.05Tx, 100us) before the start of the next fixed frame period.

Summary of changes:

- Add description according to above agreement.

- Simplify terminology, removing "indicated" from "indicated remaining channel occupancy duration".

Specs/clauses affected:

TS 38.213 clause 11.1.1

Consequences if not approved:

Incomplete specification of remaining channel occupancy duration.

11.1.1 UE procedure for determining slot format --------------------------------------------- unchanged sections omitted -------------------------------------------- For each serving cell in the set of serving cells, the UE can be provided: - an identity of the serving cell by servingCellId - a location of a SFI-index field in DCI format 2_0 by positionInDCI - a set of slot format combinations by slotFormatCombinations, where each slot format combination in the set of slot format combinations includes - one or more slot formats indicated by a respective slotFormats for the slot format combination, and - a mapping for the slot format combination provided by slotFormats to a corresponding SFI-index field value in DCI format 2_0 provided by slotFormatCombinationId - for unpaired spectrum operation, a reference SCS configuration μSFI by subcarrierSpacing and, when a supplementary UL carrier is configured for the serving cell, a reference SCS configuration μSFI,SUL by subcarrierSpacing2 for the supplementary UL carrier - for paired spectrum operation, a reference SCS configuration μSFI,DL for a DL BWP by subcarrierSpacing and a reference SCS configuration μSFI,UL for an UL BWP by subcarrierSpacing2 - a location of an available RB set indicator field in DCI format 2_0 that is - one bit, if intraCellGuardBandDL-r16 for the serving cell indicates no intra-cell guard-bands are configured, where a value of '1' indicates that the serving cell is available for receptions, a value of '0' indicates that the serving cell is not available for receptions, by availableRB-SetPerCell-r16, and the serving cell remains available or unavailable for reception until the end of the remaining channel occupancy duration - a bitmap having a one-to-one mapping with the RB sets [6, TS 38.214] of the serving cell, if intraCellGuardBandDL-r16 for the serving cell indicates intra-cell guard-bands are configured, where the bitmap includes NRB,set,DL bits and NRB,set,DL is the number of RB sets in the serving cell, a value of '1' indicates that an RB set is available for receptions, a value of '0' indicates that an RB set is not available for receptions, by availableRB-SetPerCell-r16 and a RB set remains available or unavailable for receptions until the end of the remaining channel occupancy duration - a location of a channel occupancy duration field in DCI format 2_0, by CO-DurationsPerCell-r16, that indicates a remaining channel occupancy duration for the serving cell starting from a first symbol of a slot where the UE detects the DCI format 2_0 by providing a value from CO-DurationList-r16. The channel occupancy duration field includes maxlog2COdurationListSize,1 bits, where COdurationListSize is the number of values provided by CO-DurationList-r16. If CO-DurationsPerCell-r16 is not provided, the remaining channel occupancy duration for the serving cell is a number of slots, starting from the slot where the UE detects the DCI format 2_0, that the SFI-index field value provides corresponding slot formats - a reference SCS configuration for CO-DurationList-r16, by subcarrierSpacing-r16 - a location of a search space set group switching flag field in DCI format 2_0, by SearchSpaceSwitchTrigger-r16, that indicates a group from two groups of search space sets for PDCCH monitoring for scheduling on the serving cell as described in Clause 10.4. If neither CO-DurationPerCell-r16 nor SlotFormatCombinationsPerCell are provided and if ChannelAccessMode-r16 = 'semistatic' is provided, the procedures in this clause apply with assuming a channel occupancy time defined in clause 4.3 of [15, TS 37.213] is the remaining channel occupancy duration if a DL transmission burst(s) is detected within the channel occupancy time. A SFI-index field value in a DCI format 2_0 indicates to a UE a slot format for each slot in a number of slots for each DL BWP or each UL BWP starting from a slot where the UE detects the DCI format 2_0. The number of slots is equal to or larger than a PDCCH monitoring periodicity for DCI format 2_0. The SFI-index field includes bits where maxSFIindex is the maximum value of the values provided by corresponding slotFormatCombinationId. A slot format is identified by a corresponding format index as provided in Table 11.1.1-1 where 'D' denotes a downlink symbol, 'U' denotes an uplink symbol, and 'F' denotes a flexible symbol. --------------------------------------------- unchanged sections omitted -------------------------------------------- For a set of symbols of a slot indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE detects a DCI format 2_0 providing a format for the slot using a slot format value other than 255 - if one or more symbols from the set of symbols are symbols in a CORESET configured to the UE for PDCCH monitoring, the UE receives PDCCH in the CORESET only if an SFI-index field value in DCI format 2_0 indicates that the one or more symbols are downlink symbols - if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as flexible and the UE detects a DCI format indicating to the UE to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE receives PDSCH or CSI-RS in the set of symbols of the slot - if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as flexible and the UE detects a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot the UE transmits the PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot - if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as flexible, and the UE does not detect a DCI format indicating to the UE to receive PDSCH or CSI-RS, or the UE does not detect a DCI format, a RAR UL, fallbackRAR UL grant, or successRAR grant indicating to the UE to transmit PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot, the UE does not transmit or receive in the set of symbols of the slot - if the UE is configured by higher layers to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE receives the PDSCH or the CSI-RS in the set of symbols of the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as downlink and, if applicable, the set of symbols is within remaining channel occupancy duration - if the UE is configured by higher layers to receive DL PRS in the set of symbols of the slot, the UE receives the DL PRS in the set of symbols of the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as downlink or flexible. - if the UE is configured by higher layers to transmit PUCCH, or PUSCH, or PRACH in the set of symbols of the slot, the UE transmits the PUCCH, or the PUSCH, or the PRACH in the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as uplink - if the UE is configured by higher layers to transmit SRS in the set of symbols of the slot, the UE transmits the SRS only in a subset of symbols from the set of symbols of the slot indicated as uplink symbols by an SFI-index field value in DCI format 2_0 - a UE does not expect to detect an SFI-index field value in DCI format 2_0 indicating the set of symbols of the slot as downlink and also detect a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit SRS, PUSCH, PUCCH, or PRACH, in one or more symbols from the set of symbols of the slot - a UE does not expect to detect an SFI-index field value in DCI format 2_0 indicating the set of symbols of the slot as downlink or flexible if the set of symbols of the slot includes symbols corresponding to any repetition of a PUSCH transmission activated by an UL Type 2 grant PDCCH as described in Clause 10.2 - a UE does not expect to detect an SFI-index field value in DCI format 2_0 indicating the set of symbols of the slot as uplink and also detect a DCI format indicating to the UE to receive PDSCH or CSI-RS in one or more symbols from the set of symbols of the slot If a UE is configured by higher layers to receive a CSI-RS or a PDSCH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as uplink or flexible, or the UE detects a DCI format indicating to the UE to transmit PUSCH, PUCCH, SRS, or PRACH in at least one symbol in the set of the symbols, the UE cancels the CSI-RS reception in the set of symbols of the slot or cancels the PDSCH reception in the slot. For operation with shared spectrum channel access, if a UE is configured by higher layers to receive a CSI-RS and the UE is provided CO-DurationPerCell-r16, for a set of symbols of a slot that are indicated as downlink or flexible by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided, the UE cancels the CSI-RS reception in the set of symbols of the slot that are not within the remaining channel occupancy duration. --------------------------------------------- unchanged sections omitted -------------------------------------------- For a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE does not detect a DCI format 2_0 providing a slot format for the slot - the UE receives PDSCH or CSI-RS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format - the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR - the UE receives PDCCH as described in Clause 10.1 - if the UE is configured by higher layers to receive PDSCH in the set of symbols of the slot, the UE does not receive the PDSCH in the set of symbols of the slot - if the UE is configured by higher layers to receive CSI-RS in the set of symbols of the slot, the UE does not receive the CSI-RS in the set of symbols of the slot, except when UE is provided CO-DurationPerCell-r16 and the set of symbols of the slot are within the remaining channel occupancy duration. - if the UE is configured by higher layers to receive DL PRS in the set of symbols of the slot, the UE receives the DL PRS - if the UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in the set of symbols of the slot and the UE is not provided EnableConfiguredUL-r16, the UE - does not transmit the PUCCH, or the PUSCH, or the PRACH in the slot and does not transmit the SRS in symbols from the set of symbols in the slot, if any, starting from a symbol that is after PUSCH preparation time Tproc,2 for the corresponding PUSCH timing capability [6, TS 38.214] assuming d2,1=1 after a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0 and the SCS configuration of the SRS, PUCCH, PUSCH or μr, where μr corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise μr=0 - does not expect to cancel the transmission of the SRS, or the PUCCH, or the PUSCH, or the PRACH in symbols from the set of symbols in the slot, if any, starting before a symbol that is after the PUSCH preparation time Tproc,2 for the corresponding PUSCH timing capability [6, TS 38.214] assuming d2,1=1 after a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0 and the SCS configuration of the SRS, PUCCH, PUSCH or μr, where μr corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise μr=0 - if the UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in the set of symbols of the slot and the UE is provided EnableConfiguredUL-r16, the UE can transmit the SRS, or PUCCH, or PUSCH, or PRACH, respectively. For unpaired spectrum operation for a UE on a cell in a frequency band of FR1, and when the scheduling restrictions due to RRM measurements [10, TS 38.133] are not applicable, if the UE detects a DCI format indicating to the UE to transmit in a set of symbols, the UE is not required to perform RRM measurements [10, TS 38.133] based on a SS/PBCH block or CSI-RS reception on a different cell in the frequency band if the SS/PBCH block or CSI-RS reception includes at least one symbol from the set of symbols.

## Proposed Conclusion #2

If CO-DurationPerCell-r16 or slotFormatCombinationId are provided in DCI format 2_0 and if ChannelAccessMode-r16 = semistatic, the UE does not expect the indicated COT duration, by CO-duration or SFI field,  to end later thanTz=max(0.05Tx, 100us) before the start of the next fixed frame period.

## UE behaviour for deactivation of semi-persistent CSI-RS reporting (DL-G1)

Discussion status:

There has been no convergence towards any of the proposed solution so far, however technical discussion has clarified certain aspects of the timing constraints.

# Discussion

Companies are invited to comment on the questions below.

## COT duration indication/ determination (DL-B6)

| Q1: How should the UE obtain the COT duration in semi-static channel access mode (a.k.a. FBE)? R1-2007607, P3 [Huawei]: In FBE, UE can obtain COT duration from SFI or COT duration indicator in DCI format 2_0. UE can also derive COT duration acquired by gNB from the FFP configuration if neither SFI nor COT duration indicator is configured. The corresponding text proposal is in TP#2 in the appendix. R1-2008126, P2/P3 [Samsung]: For FBE, a UE assumes gNB channel occupancy time equals to FFP as long as the UE detects any DL signals within a FFP. Adopt the following TP for TS 37.213. ======================== Start of TP for TS 37.213 ========================== ## 4.3 Channel access procedures for semi-static channel occupancy Channel assess procedures based on semi-static channel occupancy as described in this Clause, are intended for environments where the absence of other technologies is guaranteed e.g., by level of regulations, private premises policies, etc. If a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic' by SIB1 or dedicated configuration, a periodic channel occupancy can be initiated by the gNB every Tx within every two consecutive radio frames, starting from the even indexed radio frame at i∙Txx∙Tx with a maximum channel occupancy time Ty=0.95Tx, where Tx= period Tx=Periodin ms, is a higher layer parameter provided in SemiStaticChannelAccessConfig and i∈0,1,…,20Tx-1. If a UE detects a DL transmission burst(s) in a Tx period, the UE assumes the channel occupancy time is Ty within the Tx period. ========================= End of TP for TS 37.213 =========================.. R1-2008204, P1 [Nokia]: Adopt the TP for 38.213 sub-clause 11.1.1 to reflect RAN1#102 agreements ## TP for TS38.213 11.1.1 UE procedure for determining slot format <unchanged text omitted> a location of an available RB set indicator field in DCI format 2_0 that is - one bit, if intraCellGuardBandDL-r16 for the serving cell indicates no intra-cell guard-bands are configured, where a value of '1' indicates that the serving cell is available for receptions, a value of '0' indicates that the serving cell is not available for receptions, by availableRB-SetPerCell-r16, and the serving cell remains available or unavailable for reception until the end of the indicated channel occupancy duration, if provided, otherwise, until the end of the 95 percent of remaining channel occupancy period provided by semiStaticChannelAccessConfig-r16. - a bitmap having a one-to-one mapping with the RB sets [6, TS 38.214] of the serving cell, if intraCellGuardBandDL-r16 for the serving cell indicates intra-cell guard-bands are configured, where the bitmap includes NRB,set,DL bits and NRB,set,DL is the number of RB sets in the serving cell, a value of '1' indicates that an RB set is available for receptions, a value of '0' indicates that an RB set is not available for receptions, by availableRB-SetPerCell-r16 and a RB set remains available or unavailable for receptions until the end of the indicated channel occupancy duration, if provided, otherwise, until the end of the 95 percent of remaining channel occupancy period provided by semiStaticChannelAccessConfig-r16. - <unchanged text omitted>. R1-2008384, P1 [Sharp] If neither SFI-index field nor CO duration field is configured, COT for FBE (i.e. COT defined in Clause 4.3 of TS37.213) should be considered as being within “indicated channel occupancy duration” if any DL burst is detected in the COT. Adopt Text proposal #1. Text proposal #1 --------- beginning of text proposal for TS 38.213 ## 11.1 Slot configuration <omitted> If a UE is provided ChannelAccessMode-r16 ='dynamic' and is provided availableRB-SetsToAddModList-r16 and availableRB-SetsToRelease-r16, the UE expects to be provided co-DurationsPerCell ToAddModList-r16 and co-DurationsPerCellToReleaseList-r16 and/or slotFormatCombToAddModList and slotFormatCombToReleaseList. If neither CO-DurationPerCell-r16 nor slotFormatCombinationId is provided and if ChannelAccessMode-r16 = semistatic is provided, the procedures in Clause 11.1.1 apply with assuming a channel occupancy time as being indicated as the remaining channel occupancy duration if a DL transmission burst(s) is detected within the channel occupancy time. <omitted>. R1-2008602, P1-2 [Qualcomm]: When COT duration is not explicitly configured for a semi-static channel access system, the UE assumes the COT ends at the beginning of the idle period in the same fixed frame period. If COT duration is explicitly configured, UE does not expect the COT duration to indicate the COT ends later than the beginning of the idle period in the same fixed frame period that the COT duration information is detected. =====================38.213 11.1.1================= ---------unchanged text omitted------------------ - a location of a channel occupancy duration field in DCI format 2_0, by CO-DurationPerCell-r16, that indicates a remaining channel occupancy duration for the serving cell starting from a first symbol of a slot where the UE detects the DCI format 2_0 by providing a value from CO-DurationList-r16. The channel occupancy duration field includes maxlog2COdurationListSize,1 bits, where COdurationListSize is the number of values provided by CO-DurationList-r16. If CO-DurationPerCell-r16 is not provided, the remaining channel occupancy duration for the serving cell is a number of slots, starting from the slot where the UE detects the DCI format 2_0, that the SFI-index field value provides corresponding slot formats - If a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic', and CO-DurationPerCell-r16 is not configured, the UE assumes the COT duration end at Tz=max0.05Tx , 100us before the start of the next periodic channel occupancy starting position [4.3 of 37.213]. - If a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic', and CO-DurationPerCell-r16 is configured, the UE does not expect the COT duration to end later than Tz=max0.05Tx , 100us before the start of the next periodic channel occupancy starting position [4.3 of 37.213]. ---------unchanged text omitted------------------. Please provide your views on the proposals. |
| --- |
| Company | Comment |
| OPPO | Sharp and Huawei’s proposal seem more reasonable, and Sharp’s TP can be the starting point for further discussion. |
| vivo | Support the principle that “When COT duration is not explicitly configured for a semi-static channel access system, the UE assumes the COT ends at the beginning of the idle period in the same fixed frame period”. We could agree the technique proposal first and then go to TP phase. |
| LG Electronics | Agree with OPPO that Sharp’s TP can be used for starting point. One more aspect to be added is that the COT duration lasts until the end of a maximum COT length defined in 37.213. In this sense, slight modification from Sharp’s TP can be made as follows: If neither CO-DurationPerCell-r16 nor slotFormatCombinationId is provided and if ChannelAccessMode-r16 = semistatic is provided, the procedures in Clause 11.1.1 apply with assuming that remaining channel occupancy duration is indicated until the end of a maximum channel occupancy time, as defined in [4.3 of 37.213] |
| Nokia, NSB | Based on last meeting agreement, CO-duration is not needed for RB-set indication, it should be clarified that if SFI nor CO-duration is present, UE assumes remaining channel occupancy duration based on FFP. Also QC’s restriction on indicted remaining channel occupancy within FFP only makes sense. Therefore, we suggest to agree first on the following proposals and only then discuss the way we capture (i.e. TP) Proposal-1: When CO-duration nor SFI field is present in DCI format 2_0 and a UE is provided higher layer parameter ChannelAccessMode-r16 ='semistatic' , UE assumes remaining channel occupancy ends Tz=max0.05Tx , 100us before the start of the next periodic channel occupancy starting position [4.3 of 37.213]. - TP FFS Proposal-2: When CO-duration or SFI field is present in DCI format 2_0 and a UE is provided higher layer parameter ChannelAccessMode-r16 ='semistatic', , the UE does not expect the indicated COT duration, by CO-duration or SFI field, to end later than Tz=max0.05Tx , 100us before the start of the next periodic channel occupancy starting position [4.3 of 37.213]. - TP FFS |
| Sharp | We would like to ask to companies if the following is aligned with their understanding. SFI-index field; CO duration field; For FBE, “indicated channel occupancy duration” in Clause 11.1.1 of TS38.213 (e.g. for available RB set length, for CSI-RS validation, etc) is identified by:. Case 1; Configured; Configured; CO duration field value (already defined in Clause 11.1.1). “COT for FBE” isn’t used.. Case 2; Configured; Not configured; SFI-index value (already defined in Clause 11.1.1). “COT for FBE” isn’t used.. Case 3; Not configured; Configured; CO duration field value (already defined in Clause 11.1.1). “COT for FBE” isn’t used.. Case 4; Not configured; Not configured; “COT for FBE” if any DL burst is detected in the COT.. Given that the above is the common understanding, we prefer Sharp’s TP. Regarding the point raised by LG, the following modification can be considered. If neither CO-DurationPerCell-r16 nor slotFormatCombinationId is provided and if ChannelAccessMode-r16 = semistatic is provided, the procedures in Clause 11.1.1 apply with assuming a channel occupancy time defined in [4.3 of 37.213] as being indicated as the remaining channel occupancy duration if a DL transmission burst(s) is detected within the channel occupancy time. |
| Samsung | The intention of our TP is also to cover the case when UE does not detect DCI format 2_0 (with/without CO-duration and/or SFI field in 2_0). Because UE assumption of gNB’s channel occupancy is not clearly specified when UE does not detect the DCI 2_0 but detects other DL signal/channel. In such case, we believe UE should assume that gNB occupies channel until the end of Tx duration, which should be clarified. |
| Qualcomm | Our first preference is actually Samsung proposal, i.e., the COT is fixed to be till the end of the FFP, except the idle period. For initial access PRACH transmission, this is already the case. There is no need to have different COT understanding for different transmissions. If this cannot be agreed, we are fine with Nokia proposals too. |
| Ericsson | We agree with Samsung. FBE is different from LBE. All is needed to determine whether a COT is initiated. When that is determined, the Ue assumption on maximum COT for the current FFP before the idle period that can share with gNB. Hence, there is no need to indicate to the UE duration of an initiated COT. We support Samsung TP. It is important to adopt Samsung TP to have a clear understanding. Among other TPs, we think we should adopt Nokia’s TP. The reason is different from Samsung TP. Nokia’s TP covers the case, when COT duration is not indicated, which is the case for FBE as explained above. Hence, both TPS are needed. Summary: Support both Samsung TP and Nokia TP. |
| Sharp | There seems to be 3 cases to be discussed in terms of UE behaviors defined in 11.1.1 in 38.213. Case1: FBE without configuration of SFI or CO-duration field So far, all the companies think a remaining channel occupancy ends right before the idle period of the FFP if the UE detects any DL in the FFP. Case2: FBE with detection of at least one of SFI and CO-duration field Nokia is proposing that “the UE does not expect the indicated COT duration by the detected CO-duration or SFI field to exceed the start of the idle period. In our view this is reasonable. Case3: FBE with configuration of at least one of SFI and CO-duration field but no detection of any of them. Option 1) There is no remaining channel occupancy, regardless of a detection of DL in the FFP. Option 2) A remaining channel occupancy ends right before the idle period of the FFP, if the UE detects any DL in the FFP (as proposed by Samsung). Actually, we do not have a strong preference between Option 1 and 2. But, with Option 2, even if the network configured CO-duration field, it has to always assume that some UEs might fail to detect DCI format 2_0. This assumption leads to the consequence that the gNB scheduling and the UE behavior would be the same as when CO-duration field is not configured. Therefore, we do not see any motivation to configure CO-duration field for FBE. We’d like to know the other companies’ views. |
| ZTE, Sanechips | We agree with the TP modified by Sharp. But the specific TP can be discussed in the draft TP phase. |
| Huawei, HiSilicon | As well as the TP from Sharp, the following changes should be considered in TS38.213 section 11.1.1 because the COT duration is no longer indicated only. - a bitmap having a one-to-one mapping with the RB sets [6, TS 38.214] of the serving cell, if intraCellGuardBandDL-r16 for the serving cell indicates intra-cell guard-bands are configured, where the bitmap includes NRB,set,DL bits and NRB,set,DL is the number of RB sets in the serving cell, a value of '1' indicates that an RB set is available for receptions, a value of '0' indicates that an RB set is not available for receptions, by availableRB-SetPerCell-r16 and a RB set remains available or unavailable for receptions until the end of channel occupancy duration |
| LG Electronics | We totally agree with Sharp’s view. Regarding Samsung’s TP: - For initial access PRACH, it’s already clear initial access UE is allowed to transmit PRACH in case the UE detects any DL signal/channel within the same FFP. - For DCI 2_0 missing case, we should separate two different cases; 1) neither SFI nor CO-duration is provided, 2) SFI or CO-duration is provided. For the first case, it’s OK for UE to assume end-of-COT as FFP-ending. On the other hand, for the second case, gNB may intend to grab the channel occupancy shorter than FFP duration. In that sense, we think Samsung’s TP is so broad that it can be misunderstood that gNB always has to occupy channel until the end of FFP. If we need to capture a UE behavior when UE does not detect DCI 2_0 in case neither of SFI nor CO-duration is provided, it would be better to precisely specify that behavior, instead of Samsung’s TP. To summarize, we are fine with either of Nokia’s or Sharp’s, but have concern with Samsung’s TP. |
| Lenovo, Motorola Mobility | We support proposals by Samsung and Nokia. We would like to clarify that Samsung's TP does not impose any new attempts to detect DL signals. |
| Sharp | For Samsung’s TP, we have the following comments: Currently, the term “channel occupancy” in 37.213 and “channel occupancy duration” in 38.213 are based on different definitions. “Channel occupancy” in 37.213 is defined in Clause 4.0 as “A channel occupancy refers to transmission(s) on channel(s) by eNB/gNB/UE(s) after performing the corresponding channel access procedures in this clause.” In other words, in 37.213 channel occupancy is tied to transmission(s). On the other hand, “channel occupancy duration” in 38.213 is defined by a channel occupancy duration field and SFI-index field in DCI format 2_0, which is not tied to “transmission(s)” but is just an indicated duration. The thing is the specs have not specified any linkage between them yet for FBE (The linkage for LBE is defined in Clause 4.2.1.0.0 of 37.213). What we should do now is to define the linkage for FBE. Looking at Samsung’s TP, their intention is a kind of re-defining of “channel occupancy time”, which does not help to define the linkage. On the contrary, it may cause confusion, because it looks conflicting with the original definition of “channel occupancy time” in 37.213 (i.e. the total time of transmission(s) and gaps <=25us). |

### Second round discussion

| Q1a: Proposal NRU02-1: If neither CO-DurationPerCell-r16 nor slotFormatCombinationId are provided in DCI format 2_0 and if ChannelAccessMode-r16 = semistatic, the UE assumes the COT ends Tz=max0.05Tx , 100us before the start of the next fixed frame period. In addition, there has been discussion to clarifying the following case: FBE with configuration of at least one of SFI and CO-duration field but no detection of any of them. Solution Option 1) There is no remaining channel occupancy, regardless of a detection of DL in the FFP. Solution Option 2) A remaining channel occupancy ends right before the idle period of the FFP, if the UE detects any DL in the FFP (as proposed by Samsung). Including recent opinions exchanged over the email reflector indicate a tendency towards option 1. Can we agree Proposal NRU02-1 and agree/conclude that Option 1 is adopted? Can the following TP be endorsed? --------- beginning of text proposal for TS 38.213 ## 11.1 Slot configuration <omitted> If a UE is provided ChannelAccessMode-r16 ='dynamic' and is provided availableRB-SetsToAddModList-r16 and availableRB-SetsToRelease-r16, the UE expects to be provided co-DurationsPerCell ToAddModList-r16 and co-DurationsPerCellToReleaseList-r16 and/or slotFormatCombToAddModList and slotFormatCombToReleaseList. If neither CO-DurationPerCell-r16 nor slotFormatCombinationId is provided and if ChannelAccessMode-r16 = semistatic is provided, the procedures in Clause 11.1.1 apply with assuming a channel occupancy time defined in [4.3 of 37.213] as being indicated as the remaining channel occupancy duration if a DL transmission burst(s) is detected within the channel occupancy time. <omitted>. |
| --- |
| Company | Comment |
| LG Electronics | Support |
| Nokia, NSB | Small editorial suggestion: If neither CO-DurationPerCell-r16 nor slotFormatCombinationId is provided and if ChannelAccessMode-r16 = semistatic is provided, the procedures in Clause 11.1.1 apply with assuming a channel occupancy time defined in [4.3 of 37.213] as being indicated as is the indicated remaining channel occupancy duration if a DL transmission burst(s) is detected within the channel occupancy time. |
| Qualcomm | We prefer option 2 as that is the behavior already applied when PRACH is transmitted. So we have a consist behavior across UEs for all transmission. If all other companies prefer option 1, we can live with that as well. |
| Sharp | Support. Fine with the Nokia’s modification as well. |
| Samsung | Although our preference is Option 2, we can live with Option 1 |
| vivo | Support Proposal NRU02-1. |
| ZTE, Sanechips | Agree proposal NRU02-1 and support Option 1. |
| OPPO | Support proposal NRU02-1, support option 1 |
| Huawei, HiSilicon | Support proposal NRU02-1. Either option 1 or 2 could be acceptable. Clarification is required in the specification. |

## Reception/Measurement/Validation (DL-D1/DL-B6)

| Q2: R1-2008126, P1 [Samsung] , R1-2008041, P1 [LG]:: Adopt the following TP for TS 38.213. ======================== Start of TP for TS 38.213 ========================= 11.1.1 UE procedure for determining slot format ======================= Unchanged Texts Omitted ========================= For operation with shared spectrum channel access, if a UE is configured by higher layers to receive a CSI-RS and the UE is provided CO-DurationPerCell-r16, for a set of symbols of a slot that are indicated as downlink or flexible by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided, the UE cancels the CSI-RS reception in the set of symbols of the slot that are not within the indicated remaining channel occupancy duration. If the UE detects a DCI format 2_0 providing a downlink or flexible slot format for the set of symbols of the slot that are not within the indicated remaining channel occupancy duration, the UE shall ignore the slot format for the set of symbols of the slot. ======================= Unchanged Texts Omitted ======================== ======================== End of TP for TS 38.213 ========================. R1-2008204, P4 [Nokia]: Adopt the following clarification for TS38.213 for sub-clause 11.1.1 For a set of symbols of a slot indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE detects a DCI format 2_0 providing a format for the slot using a slot format value other than 255 - …. - if the UE is configured by higher layers to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE receives the PDSCH or the CSI-RS in the set of symbols of the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as downlink and, if applicable, the set of symbols is within indicated remaining channel occupancy.. Please provide your views on the proposals. |
| --- |
| Company | Comment |
| OPPO | Agree with Samsung’ TP and fine with Nokia’s TP in principle. |
| Vivo | Agree with Nokia’s TP. |
| LG Electronics | Prefer Samsung’s TP, since it can resolve specification conflict issue and in addition, ignoring flexible symbols outside COT can provide more flexibility to gNB. |
| Nokia, NSB | Either of TPs does the job. |
| Sharp | R1-2008204, P4 [Nokia] should be discussed with R1-2008126, P1 [Samsung] and R1-2008041, P1 [LG]. We are fine with either one. |
| Samsung | Support our TP as proponent. |
| Qualcomm | Prefer Nokia’s TP. For Samsung’s TP, we think it should be “the UE shall ignore the CSI-RS reception for the set of symbols of the slot.” |
| Ericsson | The issue with Samsung/LG TP is that it stated to ignore slot format. We agree with QC that it is better to describe the corresponding functionality. In all three TPs, it is mentioned “indicated” remaining channel occupancy. We should remove “indicated” since for FBE we don’t need to indicate COT. It would be sufficient to mention “remaining channel occupancy”. How the UE determines “the remaining channel occupancy” depends on if LBE or FBE used, etc. Nokia’s TP is preferred with removal of “indicated”. - if the UE is configured by higher layers to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE receives the PDSCH or the CSI-RS in the set of symbols of the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as downlink and, if applicable, the set of symbols is within indicated remaining channel occupancy. |
| ZTE, Sanechips | This issue has been discussed in the last meeting and has no any consensus. But if most companies think this issue needs to be discussed further, we can respect your opinions. For Nokia’s TP, it is acceptable for us with minor modification: Modified TP based on Nokia’s TP For a set of symbols of a slot indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE detects a DCI format 2_0 providing a format for the slot using a slot format value other than 255 - …. - if the UE is configured by higher layers to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE receives the PDSCH or the CSI-RS in the set of symbols of the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as downlink and, if applicable, the set of symbols is within indicated remaining channel occupancy duration. |
| Huawei, HiSilicon | Either TP is fine. Also agree with Ericsson’s comment that the word of “indicated” should be removed from “indicated remaining channel occupancy duration” because COT duration is not indicated in FBE. |
| Lenovo, Motorola Mobility | We share Ericsson’s view. |

### Second round discussion

| Q2a: Is the following Proposal agreeable: Proposal NRU02-2: If CO-DurationPerCell-r16 or slotFormatCombinationId are provided in DCI format 2_0 and if ChannelAccessMode-r16 = semistatic, the UE does not expect the indicated COT duration, by CO-duration or SFI field, to end later than Tz=max0.05Tx , 100us before the start of the next fixed frame period. If above proposal is agreeable, do we need a TP to reflect the agreement, or should the above rather be a conclusion of the issue without the need of a spec change? Note that the following could be a base TP (TS38.213 sub-clause 11.1.1) for inclusion in the specs (based on Nokia’s TP). For a set of symbols of a slot indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE detects a DCI format 2_0 providing a format for the slot using a slot format value other than 255 - …. - if the UE is configured by higher layers to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE receives the PDSCH or the CSI-RS in the set of symbols of the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as downlink and, if applicable, the set of symbols is within remaining channel occupancy.. |
| --- |
| Company | Comment |
| LG Electronics | Support Nokia’s TP. For NRU02-2, we don’t expect any spec change even though it can be agreed as a conclusion. |
| Nokia, NSB | I got bit confused, how is the Nokia TP related to Proposal NRU02-2 In any case, we support our TP with (indicated removed) as well. And we support the proposal 2-2 as well, we are fine with having it as conclusion. |
| Qualcomm | Agree with the proposal. A conclusion may not be enough. We should capture either the UE does not expect the COT duration or SFI indicating the slot in the later FFP to be in COT, or the UE should ignore the COT duration or SFI indication in the later FFP. |
| Sharp | Support Nokia’s TP. Regarding NUR02-2, we are fine with either an agreement or a conclusion (we slightly prefer a conclusion, though). |
| Samsung | Agree with the proposal and fine with either agreement or a conclusion. |
| Vivo | Support Nokia’s TP. Conclusion for NRU02-2 is fine. |
| ZTE, Sanechips | Support Nokia’s TP with minor modification to align with wording elsewhere in specs on “remaining channel occupancy duration”: For a set of symbols of a slot indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE detects a DCI format 2_0 providing a format for the slot using a slot format value other than 255 - …. - if the UE is configured by higher layers to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE receives the PDSCH or the CSI-RS in the set of symbols of the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as downlink and, if applicable, the set of symbols is within remaining channel occupancy duration.. Regarding Proposal NRU02-2, we tend to see it as a conclusion. |
| OPPO | Support the TP and we share same view as QC that conclusion is not enough. |
| Huawei, HiSilicon | Fine with Nokia’s TP and proposal NRU02-2. Not sure whether additional TP is required for NRU02-2 if it is not a conclusion. Seems Nokia’s TP is not related to NRU02-2. |

## UE behaviour for deactivation of semi-persistent CSI-RS reporting (DL-G1)

During RAN4 #95 it was agreed to clarify the UE ehavior in case of receiving the MAC-CE deactivation command for semi-persistent CSI reporting, in case of UL LBT failure for sending the HARQ-ACK.

The following options had been discussed in RAN1 #102-e (cf. R1-2005220, R1-2007260):

Option 1
If UE cannot transmit HARQ-ACK on MAC-CE deactivation due to UL CCA failure, UE continues to be in its previous state, i.e., it should measure and report L1-RSRP until it successfully transmits HARQ-ACK

Option 1bis
If UE cannot transmit HARQ-ACK on MAC-CE deactivation due to UL CCA failure, UE continues to be in its previous state, however it is up to UE implementation whether it continues measure and report L1-RSRP or report stale L1-RSRP until it successfully transmits HARQ-ACK

Option 2
For semi-persistent CSI reporting with PUCCH, if UE cannot transmit HARQ-ACK on the MAC CE deactivation due to the UL LBT failures, UE continues the L1-RSRP measurements but delay the L1-RSRP reporting. If UE does not receive deactivation command during the delay period, UE restarts to transmit L1-RSRP reporting.

Option 3
Delay the L1-RSRP reporting when the HARQ feedback cannot be transmitted after receiving the MAC CE deactivation command. A time limit shall be defined when the L1-RSRP reporting is delayed. When exceeding the time limits, UE shall abandon the stored measurement results, where the time limit is FFS. The UE shall also abandon the measurement results when the HARQ feedback is retransmitted for the deactivation command

Option 4
For semi-persistent CSI reporting with PUCCH, if UE cannot transmit HARQ-ACK on the MAC CE deactivation due to the UL LBT failure, the UE performs deactivation at the original MAC action time.

| Q3: What should be the UE behaviour for the case mentioned by RAN4? R1-2007607, O2 [Huawei]: All three options [FL NOTE: Option 1/2/3] can solve the ambiguity issue between gNB and UE on the CSI-RS-based measurement and reporting. Option 1 has least standard impact from Rel-15. R1-2007979, P2 [Ericsson]: Support Option 4 and send LS reply to RAN4 R1-2008204, P5 [Nokia]: To enable gNB to re-send MAC-CE in case of PUCCH decoding failure or LBT failure occurs, prolong the MAC-CE processing delay from 3ms to X ms (FFS: X) Please share your view on above listed proposals or how to define the UE ehavior. |  |  |  |
| --- | --- | --- | --- |
| Company | Comment |  |  |
| OPPO | Support option 4. |  |  |
| Vivo | Support Option 1 with the following reason: First, I would like to list the possible cases that may happen: Case 1: UE decodes MAC CE successfully + UE sends PUCCH on slot n upon LBT success + gNB decode PUCCH successfully; Case 2: UE decodes MAC CE successfully + UE sends PUCCH on slot n upon LBT success + gNB fails to decode the PUCCH; Case 3: UE decodes MAC CE successfully + UE fails to send PUCCH on slot n due to LBT failure + gNB won’t decode the PUCCH; Case 4: UE fails to decode MAC CE + UE won’t send PUCCH on slot n + gNB won’t decode the PUCCH. Second, the behavior of gNB and UE are listed in the following table for Option 1 and Option 4 respectively. It is clearly observed that Option 1 has ambiguity issue for Case 2 and Option 4 has ambiguity issue for Case 4. Besides, the ambiguity time is almost the same, i.e. from the first slot that is after slot n+3Nslotsubframe,µ until successful reception of HARQ ACK for MAC CE retransmission +3Nslotsubframe,µ. Option 1; Option 4. Case 1; gNB&UE: Deactivation of SP-CSI reporting from the first slot that is after slot n+3Nslotsubframe,µ No ambiguity; gNB&UE: Deactivation of SP-CSI reporting from the first slot that is after slot n+3Nslotsubframe,µ No ambiguity. Case 2; gNB: No deactivation of SP-CSI reporting reception from the first slot that is after slot n+3Nslotsubframe,µ and wait for HARQ ACK retransmission. After a while, there is no any HARQ ACK received, gNB retransmit MAC CE. UE: Deactivation of SP-CSI reporting reception from the first slot that is after slot n+3Nslotsubframe,µ Ambiguity from the first slot that is after slot n+3Nslotsubframe,µ until successful reception of HARQ ACK for MAC CE retransmission +3Nslotsubframe,µ; gNB: Deactivation of SP-CSI reporting reception from the first slot that is after slot n+3Nslotsubframe,µ UE: Deactivation of SP-CSI reporting from the first slot that is after slot n+3Nslotsubframe,µ No ambiguity. Case 3; gNB: No deactivation of SP-CSI reporting reception from the first slot that is after slot n+3Nslotsubframe,µ and wait for HARQ ACK retransmission. UE: No deactivation of SP-CSI reporting from the first slot that is after slot n+3Nslotsubframe,µ and retransmit HARQ ACK in next opportunity. No ambiguity but deactivation delayed; gNB: Deactivation of SP-CSI reporting reception from the first slot that is after slot n+3Nslotsubframe,µ UE: Deactivation of SP-CSI reporting reception from the first slot that is after slot n+3Nslotsubframe,µ No ambiguity. Case 4; gNB: No deactivation of SP-CSI reporting reception from the first slot that is after slot n+3Nslotsubframe,µ and wait for HARQ ACK retransmission. After a while, there is no any HARQ ACK received, gNB retransmit MAC CE. UE: No deactivation of SP-CSI reporting No ambiguity but deactivation delayed; gNB: Deactivation of SP-CSI reporting reception from the first slot that is after slot n+3Nslotsubframe,µ and wait for HARQ ACK retransmission. After a while, there is no any HARQ ACK received, gNB retransmit MAC CE. UE: No deactivation of SP-CSI reporting. Ambiguity from the first slot that is after slot n+3Nslotsubframe,µ until successful reception of HARQ ACK for MAC CE retransmission +3Nslotsubframe,µ. Finally, to make the decision, the question boils down to how often will Case 2 and Case 4 occur? As we know, the target BLER for PUCCH is less than 1% while that for PDSCH carrying MAC CE is around 10%. Therefore, it is clearly Option 1 is better than Option 4 since it could solve more ambiguity cases. |
| Nokia, NSB | We are not OK with Option 4 due to adding complexity at gNB, and our preference is still Option 1. However, we also think that if MAC-CE delay of application is increased to e.g. 10-20 ms for NR-U, at least for Scell deactivation and SP-CSI on PUCCH, then ambiguity can be handled with low complexity on both gNB and UE side. This could be potential compromise. |  |  |
| Qualcomm | We support option 4 for UE complexity reason. |  |  |
| Ericsson |  | We support Option 4 for the following reasons: - Option 4 requires no spec change. - We observe that the Rel-15 spec contains a multitude of other MAC-CE use cases besides activation/deactivation of SP-CSI on PUCCH. 38.321 Section 6.1.3 contains a long list of MAC-CE messages, and 38.214 contains related procedure text in multiple locations regarding the timing of when these messages should be applied at the MAC layer. It would be highly undesirable to have a unique solution just for deactivation of SP-CSI reporting on PUCCH. Why should this use case be treated differently than all of the others? - The relevant section of the 38.214 spec on timing of activation/deactivation of SP-CSI reporting on PUCCH is 5.2.1.5.2 which contains the following paragraph: For semi-persistent reporting on PUCCH, the PUCCH resource used for transmitting the CSI report are configured by reportConfigType. Semi-persistent reporting on PUCCH is activated by an activation command as described in clause 6.1.3.16 of [10, TS 38.321], which selects one of the semi-persistent Reporting Settings for use by the UE on the PUCCH. When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated semi-persistent Reporting Setting should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. While this paragraph says “activation” it actually applies to both activation and deactivation of SP-CSI reporting due to the structure of the MAC-CE message. As shown in the extract from 38.321 below, the MAC-CE message includes a 4-bit field for activating/deactivating SP-CSI reporting on PUCCH for up to 4 configured Report Settings. A ‘1’ indicates activation for a particular Report Setting and a ‘0’ indicates deactivation. - The highlighted wording above is exactly Option 4. “Would transmit a PUCCH” is interpreted as “would transmit a PUCCH if LBT had been successful” and the remaining highlighted text refers to the “original MAC-CE action time” stated in Option 4. - Option 4 simplifies UE processing - This option allows a separation of PHY and MAC at the UE, thus allowing MAC processing to behave as in in Rel-15 and not be dependent on LBT outcome (performed in radio hardware). - Option 4 does not place additional processing burden on the gNB relative to Option 1 - Fundamentally, the gNB implementation must be able to take into account uncertainty on whether the UE has deactivated SP-CSI for both Option 1 and Option 4. This can happen even if the UE passes LBT and is able to transmit PUCCH (e.g., PUCCH decoding failure ag the gNB). To account for this uncertainty, the gNB will need to try two hypotheses for reception of PUCCH resources in the future (after slot n) to account for the fact that CSI may or may not be multiplexed (the gNB doesn’t know). This is needed so the gNB doesn’t fail to decode other important HARQ-ACK/NACKs that may be multiplexed on PUCCH. This must happen until the gNB is able to confirm that the UE deactivated SP-CSI, e.g., by re-transmitting another MAC-CE deactivation and eventually successfully receiving another ACK. So, if the gNB needs to account for the uncertainty anyway, it doesn’t matter whether or not the UE uses Option 1 or Option 4. Hence, if Option 4 is simpler from a UE perspective, it makes sense to adopt such a solution. Question to proponents of Option 1: It is not clear how the UE reports L1-RSRP when LBT is failing for PUCCH. Question to vivo: In all of cases 2, 3, and 4, the gNB fails to decode PUCCH. It does not know if it is because of LBT failure at the UE, PDSCH decoding failure of MAC-CE, or just a missed detection of PUCCH. It is not clear that the gNB action would be any different for any of these cases, and it is not clear that Option 1 would result in any different processing at the gNB compared to Option 4. It feels like an optimization to make a spec change for this one case of MAC-CE compared to the multitude of other MAC-CE use cases. |
| ZTE, Sanechips |  | In our view, Option 1 and Option4 are acceptable for us. But if “at the original MAC action time” in Option 4 can be understood as the time corresponding to the first slot that is after slot n+3Nslotsubframe,µ and n is the slot corresponding to PUCCH transmission if LBT has been successful , then we tend to support option 4. |
| Huawei, HiSilicon |  | We support option 1 as it follows Rel-15 design. The implementation at gNB can be maintained. The compromised solution from Nokia is also acceptable. |
| Lenovo, Motorola Mobility |  | We prefer the UE to continue sending the reports (like Option 1). The point of the acknowledgement is to establish that a) the UE is not transmitting the report any longer, and b) the corresponding resource can be reassigned for other purposes by the gNB. However we acknowledge that if the UE received a deactivation command, obviously the gNB does not see a need for updating the measurement any longer. So in case the UE cannot transmit the acknowledgement due to LBT failure, it should still be okay to stop the measurement procedure right away and report one of the abundant "not valid" values as already defined in 38.133 Table 10.1.6.1-1. An additional benefit is that this can be understood by the gNB that the deactivation command was received but acknowledgement could not be sent. In summary: If UE cannot transmit HARQ-ACK on MAC-CE deactivation due to UL CCA failure, UE stops measuring L-RSRP but continues to report L1-RSRP (using the existing "not valid" content) until it successfully transmits HARQ-ACK. |
| Ericsson 2 |  | There seems to be conflicting views on what is the "no spec change" option. In our understanding, Option 4 is the no-spec change option, not Option 1. Consider Rel-15 (licensed operation) where vivo's Case 1, 2, or 4 apply. The relevant paragraph from 38.214 is in Section 5.2.1.5.2: When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated semi-persistent Reporting Setting should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. Note that this paragraph applies to both activation and deactivation as we describe in our previous comment above, and in more detail in our contribution. According to this paragraph of the spec, for Case 1 and 2, the UE continues to be in its previous state, thus will measure/report SP-CSI up until slot n + 3 ms. For Case 4, the UE continues to measure/report up to slot n1 + 3 ms, where n1 > n is the slot in which the UE would transmit PUCCH with HARQ-ACK corresponding to a successfully received re-transmission of the MAC-CE deactivation message on PDSCH. So, in all three cases, the UE continues to be in its previous state, and thus measure/report up to slot n + 3 ms. It does not stop measuring/reporting at slot n (or n1). Now if we consider unlicensed operation, vivo's Case 3 is the new case to consider (LBT failure in slot n where the UE would transmit PUCCH). According to Option 4, "if UE cannot transmit HARQ-ACK on the MAC CE deactivation due to the UL LBT failure, the UE performs deactivation at the original MAC action time." Our understanding of this wording is that the UE continues to report up to slot n + 3ms, and stops reporting in the first slot after that. So, in answer to ZTE's question, yes, the original MAC action times is indeed the first slot that is after slot n+3Nslotsubframe,µ. In contrast, Option 1 says "If UE cannot transmit HARQ-ACK on MAC-CE deactivation due to UL CCA failure, UE continues to be in its previous state, i.e., it should measure and report L1-RSRP until it successfully transmits HARQ-ACK." With Option 1, this means the UE stops measuring/reporting at slot n, not slot n + 3ms as the current spec says. In summary, Option 4 is the "no spec change" option, not Option 1. I feel we at least need to achieve common understanding on this point before debating the various options. |
| Vivo2 |  | According to Ericsson’s 2nd round comments, it seems that there is need to have some clarification for Option 1 and Option 4 first. In our understanding, the key difference between them is how to define the slot n instead of whether to have +3ms: Option 1: From UE side, the slot n is the slot where UE transmits HARQ-ACK on MAC-CE deactivation successfully; From gNB side, the slot n is the slot where gNB receives HARQ-ACK on MAC-CE deactivation successfully; Option 4: From UE side, the slot n is the first slot where UE plans to transmit HARQ-ACK on MAC-CE deactivation; From gNB side, the slot n is the slot where gNB receives HARQ-ACK on MAC-CE deactivation successfully or doesn’t receive anything on PUCCH resource corresponding to the sending MAC-CE deactivation PDSCH. Whether Option 1 or Option 4, the indicated semi-persistent Reporting Setting should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where is the SCS configuration for the PUCCH. This understanding seems different with what Ericsson mentioned above (i.e “the UE stops measuring/reporting at slot n, not slot n + 3ms as the current spec says.”). Based on the above understanding, I would say Option 4 is not aligned with current spec/gNB implementation at least for Case 4, i.e. UE fails to decode MAC CE and doesn’t send PUCCH. In licensed case, gNB will consider this DTX state as NACK and assume UE will not stop measuring and reporting. However, for Option 4, gNB will consider DTX state as ACK and assume UE will stop measuring and reporting. In contrast, Option 1 is aligned with current spec that the deactivation will be only applied when HARQ-ACK information is transmitted/received. In answer to Ericsson’s question below: In all of cases 2, 3, and 4, the gNB fails to decode PUCCH. It does not know if it is because of LBT failure at the UE, PDSCH decoding failure of MAC-CE, or just a missed detection of PUCCH. It is not clear that the gNB action would be any different for any of these cases, and it is not clear that Option 1 would result in any different processing at the gNB compared to Option 4. gNB follows the defined behavior and doesn’t need to distinguish case 2, 3 and 4. As you see in the table in our 1st round comment, the behavior for case 2, 3 and 4 at gNB side is the same, i.e. no deactivation. Regarding gNB processing, I am not sure that current gNB implementation will take any uncertainty into account as Ericsson mentions (i.e. the gNB will need to try two hypotheses for reception of PUCCH resources in the future (after slot n) to account for the fact that CSI may or may not be multiplexed (the gNB doesn’t know)) since the ambiguity happens for Case 2 is rare and it could be quite small. This could be also maintained by Option 1. However, if we go for Option 4, the ratio of Case 4 is around 10%, gNB must take this into account and try two hypotheses for future PUCCH reception and some PUSCH reception. Question to ZTE: For if “at the original MAC action time” in Option 4 can be understood as the time corresponding to the first slot that is after slot n+3Nslotsubframe,µ and n is the slot corresponding to PUCCH transmission if LBT has been successful, should this be exactly Option 1 if the above clarification is correct? In summary, the understanding of Option 1 and Option 4 should be clarified (especially the key difference). If our understanding is correct, Option 1 is more aligned with the behavior in NR Rel-15. |
| Nokia NSB |  | We disagree with Ericsson that the spec is clear. The spec can be interpreted either way Option 1 or Option 4. This will not move us anywhere. Again, we proposed to relax the timeline of application from n+3ms to n+Xms to facilitate low complex handling of LBT failure at gNB and UE. Any issues with this proposal? And if we cannot converge, then RAN1 can just reply to RAN4 that RAN1 specification is not clear when considering LBT failure and there is no consensus on how to clarify it in RAN1 either. |
| Ericsson 3 |  | I would like to respond to one thing specifically in vivo's reply before discussing further: Vivo: Whether Option 1 or Option 4, the indicated semi-persistent Reporting Setting should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where is the SCS configuration for the PUCCH. This understanding seems different with what Ericsson mentioned above (i.e “the UE stops measuring/reporting at slot n, not slot n + 3ms as the current spec says.”). This is a misunderstanding of what we provided in our response. We made this statement about Option 1. Please see clarifying text in red in our previous "Ericsson 2" response above. |
| Qualcomm2 |  | Basically share the same view as Ericsson. For spec impact, our understanding is Option 4 has no spec impact. Slot n is read as the time when the transmission intended to happen. As Ericsson noted, there are many places with this n+3Nslotsubframe,µ behavior is applied. We don’t think it is reasonable to has special processing only for the use case identified in the RAN4 LS. On the other hand, it might to be too late to introduce a complete solution for this problem. Another issue as we mentioned in last meeting is, for RTT time reset for DRX, RAN2 already agreed to use intended A/N transmission time instead of the actually A/N transmission time. We believe this is similar issue and we should not introduce different solutions. |
| Vivo3 |  | Thanks for Ericsson’s response. This is exact point we have different understanding for Option 1. My understanding is Option 1 still use n+3ms. The key difference between Option 1 and 4 is how to define slot n. |
| Ericsson 4 |  | Responding to vivo's comment Vivo3: Option 4: Option 4 uses the current spec definition of slot n. "Would transmit a PUCCH" is interpreted as in other places in the spec as "Would transmit a PUCCH if LBT was successful" We had a similar debate + conclusion last meeting in the context of UCI multiplexing on PUCCH. When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated semi-persistent Reporting Setting should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. Hence, option 4 does not require any change to the definition of n or the time at which the deactivation is applied (n + 3 ms). Option 1: Option 1 changes the definition of n to a new value n' where n' is later than n, so a spec change is needed. I disagree with vivo's claim that Option 1 still uses n + 3ms. Option 1 changes the definition of when the deactivation is applied – it will be applied in slot n' (not n' + 3ms). Option 1 If UE cannot transmit HARQ-ACK on MAC-CE deactivation due to UL CCA failure, UE continues to be in its previous state, i.e., it should measure and report L1-RSRP until it successfully transmits HARQ-ACK |
| ZTE, Sanechips |  | Response to vivo’s question: For option1, I think the slot n specified in the current spec is just a slot that HARQ-ACK tried to be sent but failed to sent due to LBT failure. In this case, the location where the HARQ-ACK is successful sent has changed from slot n to slot n1. n1>n. from this point of view, I tend to support Option 4 with a assumption that slot n is a slot corresponding to PUCCH transmission if LBT has been successful. Besides, I also agree Nokia’s suggestion, that is, if it is difficult for us to focus on an acceptable option at the moment, I also tend to support reply a LS to RAN4 first, that is, RAN1 specification is not clear when considering LBT failure and there is no consensus on how to clarify it in RAN1 either. |
| Ericsson |  | Question to ZTE: As we commented in Ericsson 4, the spec is clear on how slot n is interpreted: the current behavior is that slot n is the slot where the HARQ-ACK tried to be sent, but LBT failed. It seems ZTE agrees that this is the current behavior. But the highlighted part in ZTE's response below seems contradictory, i.e., the following two in the highlighted section conflict with each other: (1) Option 1 is current spec behavior, and (2) Option 1 changes the definition of slot n to slot n1 (n1 > n). Was the intention to say that Option 4 aligns with the current spec behavior? For option1, I think the slot n specified in the current spec is just a slot that HARQ-ACK tried to be sent but failed to sent due to LBT failure. In this case, the location where the HARQ-ACK is successful sent has changed from slot n to slot n1. n1>n. from this point of view, I tend to support Option 4 with a assumption that slot n is a slot corresponding to PUCCH transmission if LBT has been successful. |