3GPP TSG RAN WG1 #103e-E                                                                                                R1-2009657

e-Meeting, October 26th – November 13th, 2020

Source:	Moderator (ZTE)

Title:	Summary#2 of AI 8.4.3 for HARQ for NTN

Agenda Item:	8.4.3

Document for:     Discussion and Decision

# Introduction

In RAN1#102e meeting, agreements w.r.t the extension of HARQ process number and enabling/disabling on HARQ feedback for downlink transmission have been made. In this meeting, companies’ views to refine the details and on other remaining issues are summarized with corresponding observations/proposals on following aspects with detailed proposals from each company listed in appendix.

- Enhanced HARQ process ID indication

- HARQ codebook enhancements

- Restriction on HARQ feedback disabling

- Performance enhancements

- PDSCH/PUSCH scheduling restriction

# Enhanced HARQ process ID indication

According to the agreement in last meeting, the maximal supported HARQ process number is extended to 32. Detailed solutions are proposed in [Huawei, CAICT, Apple, vivo, CMCC, Intel, Xiaomi, CATT, Panasonic, Ericsson, QC, Sony, Lenovo, Spreadtrum, LG, ZTE, OPPO] with following options:

- Option 1: Slot index as the MSB [QC,HW, Sony, Lenovo]

In this option, the slot index is interpreted as MSB for HARQ ID calculation along with the indicated HARQ ID in existing DCI field.

- Option 1-a: Slot index as the LSB [CMCC, ZTE]

In this option, the slot index is interpreted as LSB for HARQ ID calculation along with the indicated HARQ ID in existing DCI field.

- Option 2: Reusing one bit from other bit field [CATT, CMCC, Apple, VIVO, ZTE]

In this way, one bit from other bit field is interpreted for HARQ ID calculation along with the indicated HARQ ID in existing DCI field.

- Option 3: Extending the HARQ process ID field up to 5 bits [Ericsson, Panasonic, CAICT]

In this option, the existing HARQ process ID field is extended to 5 directly.

- Option 4: CCE index associated [LG]

In this option, the determination of HARQ process ID will be coupled with the index of CCE, which carrying the scheduling information for transmission.

- Option 5: Additional scrambling for scheduling grant [Spreadtrum]

In this option, the determination of HARQ process ID will be up to additional scrambling on the scheduling grant.

- Option 6: DM-RS index associated [HW]

In this option, the determination of HARQ process ID will be coupled with the DM-RS index, which is associated to the transmission.

Moreover, [Intel] is supportive to associate the HARQ ID determination with slot index, but without detailed proposals. [OPPO, Xiaomi] highlights that the solution with lower DCI overhead, e.g., unchanged size is preferred. Meanwhile, as mentioned in [CMCC], extension of HARQ process ID field up to 5 is only used for DCI 0-2/1-2.

According to the above summary, the following proposals are listed as majority views:

[Initial Proposal 1]: Enhanced HRAQ process ID indication is supported by at least one of options:

- Option 1: Slot index as the MSB

- Option 1-a: Slot index as the LSB

- Option 2: Reusing one bit from other bit field

- Option 3: Extending the HARQ process ID field up to 5 bits

| Company | Comments and Views |
| --- | --- |
| MediaTek | Option 3 |
| CATT | Option 2 and option 3 are both ok for us. |
| LG | Maybe we first decide whether it is based on explicit (option 3) or implicit (other options) indication. Regarding implicit indication, we still think it is beneficial to consider CCE based indication which is already used in NR for PUCCH resource allocation. |
| OPPO | Option 1 |
| Panasonic | We support option 3 considering only 1 bit needs to be added. The extension of HARQ process ID field should be applied to DCI 0-1, 0-2 and 1-1, 1-2, i.e. not to add DCI 0_0 and 1_0. If adding even 1 bit is undesired, option 1 (or 1-a) would be also acceptable. |
| Ericsson | We support Option 3 for non-fallback DCI (0_1/1_1 and 0_2/1_2). For fallback DCI (0_0/1_0) it is acceptable to limit the number of HARQ processes to 16. The design rationale of non-fallback DCI is to support adding new features with configurable fields, so it is natural to add one bit to the HARQ process ID field in non-fallback DCI if 32 HARQ processes are configured. The fundamental NR HARQ design is asynchronous and thus flexible. Slot index-based method is not aligned with this principle and re-introduces unnecessary scheduling restriction from LTE. This is against the agreement that the solution should be “minimizing the impacts on specification and scheduling”. |
| Intel | Option 1 or Option 1-a |
| Apple | We support the proposal. Among the options, our preference is Option 2. |
| Sharp | Option 2 or Option 3 |
| Qualcomm | Support Option 1 or 1-a. |
| Xiaomi | We prefer to have unchanged size of HARQ process ID field, option 1/1a is fine to us. |
| Huawei | We agree with LGE that the indication should rather be represented by two options: the first is explicit indication, the second is implicit indication. Option 3 is not preferred as it cannot be applied to all types of DCI formats. Option 1 is an implicit method but it may pose some scheduling restrictions. Option 2 is more flexible than Option 1, for enabled HARQ processes, there may not be extra bits available in the DCI to be used for this purpose Our proposal for rewording the draft agreement is: Initial Proposal 1]: Enhanced HARQ process ID indication is supported by at least one of options: Option1: Explicit Indication - CCE index associated - DM-RS index associated - Extending the HARQ process ID field up to 5 bits - Reusing one bit from other bit field Option 2: Implicit Indication - Slot index as the MSB - Slot index as the LSB |
| ZTE | Option-2 is preferred since it can be used for all DCIs formats. W.r.t Option-3, it can be considered for DCI X-2 only following the existing NR style. Option-1a is preferred over Option-1 with less restriction. |
| Samsung | Support Option 3 (same principle as for DCI format 1_2 in Rel-16) |
| Lenovo, MotoM | We support the proposal, our first preference is option 1, and second preference is option 3. |
| Sony | Support initial proposal 1. From minimizing spec. impact perspective, we prefer Option 1, Option 1-a and Option 2. |
| APT | Option 3 because of - Without changing a DCI format size for non-fallback DCI - No scheduling restriction in the time domain like Option 1 - No strange combination in fields or damaging other features like Option 2 |
| ETRI | We prefer Option 1/1-a/2. For minimizing the spec. impact, the methods of maintaining DCI size might be advantageous. Especially, for option 2, 1 bit could be taken from rv field, which means that 1 bit rv could be used instead of 2 bit rv as already defined in the legacy NR. |
| CMCC | Option 3 is first preferred. Extension of HARQ process ID field up to 5 for DCI 0-2/1-2 can be considered as a starting point. We are also fine with Option 2 and Option 1-a. |
| Nokia, Nokia Shanghai Bell | According to our understanding, the feature lead (and several other companies) have overinterpreted the agreement from last meeting. The agreement states: “The extension of maximal HARQ process number can be considered with following assumptions:”. So it was agreed to consider to extend the maximum number of HARQ processes, but formally, there has not been any agreement for extending the amount of HARQ processes. Hence, this discussion point is not suitable as it is built on an erroneous assumption/interpretation. Our opinion is that disabling HARQ for selected processes and using slot aggregation would be sufficient. If the maximum number of supported HARQ processes is to be increased to 32, we would have a preference for option 3, as variants of option 1 would limit gNB scheduling flexibility, and option 2 would limit link adaptation flexibility. |
| Thales | We prefer option 3. An implicit indication (option 1 and Option 1a) can be an alternative solution if the HARQ process ID field cannot be extended. |
| vivo | Considering to the compatibility and minimum spec impact, we prefer Option 2. |
| CAICT | Option 3. Minimizing the impacts on specification and scheduling should be considered for the extension of maximal HARQ process number should consider |
| China Telecom | Opt2 or Opt3 |

In addition, w.r.t the supports for larger HARQ process number, [Samsung, CMCC, OPPO, Apple, MTK, Xiaomi, Asia Pacific Telecom, IDC] prefer to define it as optional UE capability. And [Ericsson, ZTE] highlight that larger HARQ process number is only enabled via the RRC configuration.

According to the above summary, the following proposals are listed as majority views:

[Initial Proposal 2] Support on the maximal HARQ process number is up to UE capability.

| Company | Comments and Views |
| --- | --- |
| MediaTek | Agree |
| CATT | This proposal needs more clarification, no sure if it is only corresponding to 32 HARQ processes, or other number less than 32 is up to UE capability. In addition, if only single codeword is supported for NTN UE, no special UE capability definition is needed. Since 16 HARQ processes with two codewords have been supported in NR Rel-15, the new UE capability definition seems not needed. |
| LG | Agree |
| OPPO | Agree |
| Panasonic | Up to 32 HARQ processes can be supported without too much complexity considering the number of MIMO layers and modulation order would be restrictive in NTN. Our preference is mandatory feature for NTN UE. |
| Ericsson | Regardless of UE capability, as a general principle, a new feature should be accompanied with RRC parameter(s) for configurability. This should be applied to the configuration of up to 32 HARQ processes as well. |
| Intel | We support the proposal |
| Apple | Agree. Some low-capable UE does not have to support up to 32 HARQ processes. |
| Sharp | Agree |
| Qualcomm | Agree in principle and suggest to modify the text as: Support on the maximal HARQ process number more than 16 is up to UE capability |
| Xiaomi | Agree |
| Huawei | Agree with Panasonic, the agreed NTN 32 HARQ process is relative a conservative value and can be support by UE without much challenge. On the other hand, it is a bit early to discuss UE capability for NTN. |
| ZTE | Fine to make agreement now or postponed during the UE feature discussion (same as other topic) |
| Samsung | We think maximum HARQ process number should be considered jointly with maximum TBS. In R1-2008166 we propose the following two options: Option 1. gNB broadcasts the maximum TBS to be configured for the cell and UE reports its capability for a number of HARQ processes. Option 2. UE reports its capability for a number of pairs of {maximum number of HARQ processes, maximum TBS constraint}. |
| Spreadtrum | Agree |
| Lenovo, MotoM | We support the feature is optional to UE, and up to UE capability |
| Sony | Support initial proposal 2. |
| APT | Agree Initial Proposal 2 |
| ETRI | Agree |
| CMCC | Agree |
| Nokia, Nokia Shanghai Bell | Do not support. When discussing this aspect last meeting, companies were targeting at addressing the HARQ stalling problem. Has this problem disappeared, or has the opinion of companies changed? If larger number of HARQ processes are to be supported, there should also be commitment for this, and hence this should be mandatory feature. |
| Thales | We support the proposal |
| vivo | Agree. |
| CAICT | Agree with CATT, more clarifications are needed. |
| China Telecom | Agree |

# HARQ codebook enhancements

In RAN1#102e, preliminary discussion on the potential enhancements for HARQ codebook are conducted. In this meeting, views from companies are provided on this aspect. More specifically:

- W.r.t the Type-1 Codebook (semi-static codebook):

The enhancement for Type-1 codebook is highlighted by [HW, VIVO, ZTE Samsung, Sony, Ericsson]. More specifically, the codebook size will be reduced by skipping the HARA-ACK feedback for disabled HARQ process. For example, the skipping can be conducted according to the indication (e.g., bitmap) from gNB [Samsung] or if all transmissions within same slot [ZTE] or all slots [vivo] are scheduled with disabled HARQ feedback. Also, improvement on the feedback overhead may also be achieved via different TDRA configuration as mentioned by [Sony].

However, as highlighted in [OPPO, CATT, Apple], enhancements on Type-1 is unnecessary.

- W.r.t the Type-2 Codebook (Dynamic codebook):

The enhancement for Type-2 codebook is highlighted by [CATT, Samsung, OPPO, Sony, Apple, Intel, Asia Pacific Telecom, CAICT, QC]. More specifically, for example, the codebook size will be reduced by directly without counting DAI for the scheduling via HARQ process with disabled feedback [Ericsson]. But, to ensure the error correction capability [Asia Pacific Telecom, ZTE], one additional parameter (e.g., to count the number of scheduling with enabled feedback) can be considered to construct the codebook [ZTE].

- W.r.t the enhanced Type-2 (Enhanced Dynamic Type-2) and Type-3 Codebook (Oneshot-feedback):

For these two cases, enhancements are not preferred by [CATT, Samsung, Apple, Intel, ZTE, and CAICT] since such functionalities are introduced only for unlicensed spectrum (i.e., shared spectrum) [Samsung, ZTE]. But, [Ericsson] highlights that it is also beneficial to improve it.

According to the above summary, the following proposals are listed as majority views:

[Initial Proposal 3]: Enhancement on Type-1 and Type-2 HARQ codebook is supported.

| Company | Comments and Views |
| --- | --- |
| CATT | Type I codebook is semi-statically configured, so we don’t think it needs special optimization to reduce codebook size. |
| LG | Agree with FL’s proposal. Regarding Type-3 HARQ codebook, following agreements were made during RAN1#103-e NR-U maintenance session. Agreements: - The FG10-15/16 are also applicable to licensed bands - The FG10-20a is also applicable to licensed bands - Note: this agreement should not cause any specification impact Where 10-16; One-shot HARQ ACK feedback; - Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_1 scheduling a PDSCH - Support feedback of type 3 HARQ-ACK codebook , triggered by a DCI 1_1 without scheduling a PDSCH using a reserved FDRA value. According to above agreement, Type-3 codebook can be applicable to L-bands. Thus, we think it is worthwhile to consider Type-3 codebook for NTN as well. |
| OPPO | For the ‘enhancement’ of type 1 codebook, some companies proposed to replace HARQ-ACK information with NACK for disabled HARQ, some other companies proposed to reduce the type 1 codebook size. Which direction are we referring with ‘enhancement’? From our point of view, if the number of HARQ-ACK bit is not reduced, it does not make sense to force UE reporting NACK for disabled HARQ. If we are heading towards type 1 CB size reduction, we need to first discuss different enhancement alternatives and the respective spec impact, before making firm decision on supporting type-1 codebook enhancement. We fear that the spec effort may be unaffordable. |
| Panasonic | HARQ codebook is beneficial for TDD and/or CA especially large number of CCs, which would not be main focus in NTN. Therefore, optimization would not be needed. |
| Ericsson | We support this proposal. For Type-3, they can be FFS. |
| Intel | We can accept enhancement of Type I HARQ codebook only if it is determined based on semi-static configuration. We agree that enhancement for Type 2 HARQ codebook is needed. |
| Apple | Since type-1 HARQ-ACK codebook is semi-static, UE does not reduce the HARQ-ACK codebook size for HARQ processes with disabled HARQ feedback. It could either send NACK or send ACK/NACK for the HARQ process with disabled feedback. For type-2 HARQ-ACK codebook, UE does not expect to receive increased counter DAI for HARQ processes with disabled HARQ feedback. |
| Sharp | We support this proposal. |
| Qualcomm | The proposal is ambiguous. It’s not clear what is considered as an enhancement given that some changes are needed. |
| Huawei | The Feature Lead initial proposal is somewhat too generic, and there is not a lot of progress we would actually do there. We could instead pick some of the enhancements actually proposed in the input documents: [Initial Proposal 3]: Enhancement on Type-1 and Type-2 HARQ codebook is supported from following options. For Type-1: - Reduce codebook size - Report NACK on disabled processes For Type-2: DAI is not incremented on disabled processes |
| ZTE | Support for the proposal for only enhance the Type-1 and Type-2. Details can be done later if there is sufficient time in RAN1#103e. |
| Samsung | Support Proposal 3 in principle. Suggest to change ‘supported’ to ‘considered’. |
| Spreadtrum | We shared the same views with Samsung. |
| Lenovo, MotoM | Before answer the question, we should focus on what kinds of enhancement will be supported. We should do either study the enhancement of type 1 and type 2 HARQ-ACK codebook or support a detail enhancement on the table if necessary. |
| Sony | Support initial proposal 3. For Type-1 Codebook, the overhead reduction is necessary. This is because the UE assumes that all PDSCH occasions, i.e., all HARQ processes need HARQ feedback and so constructs the codebook based on pre-configured parameters such as TDRA table, serving cell number and K1 value. For example, if a UE is configured with 32 HARQ processes in the serving cell, then assuming HARQ process ID#1 is configured for feedback enabled and the other HARQ processes are configured for feedback disabled. In that sense, around 95% of the bits reserved are redundancy which would cause large overhead. For Type-2 Codebook, DAI is not incremented for a PDCCH which is scheduled on a HARQ process with HARQ-ACK disabled. However, the HARQ feedback for the SPS release/activation for HARQ process with HARQ-ACK disabled is necessary. And the DAI should be incremented for SPS activation/release correspondingly. Type-3 Codebook enhancement is FFS. |
| APT | Support Initial Proposal 3 |
| Nokia, Nokia Shanghai Bell | Before engaging into this discussion it would be nice to see very specific proposals of how the Type-1 and Type-2 HARQ codebook would be enhanced. Especially the focus for such potential solutions should clarify how the potential error cases would be addressed while maintaining no or low additional overhead. |
| Thales | Agree. Enhancements on HARQ codebook (Type-1 and Type-2) will be needed . But need to provide more details about these enhancements. |
| vivo | Support the proposal. For example, for Type-1 codebook, the codebook size or feedback overhead could be reduced by skipping the HARQ-ACK feedback for disabled HARQ process. On another hand, with ignoring codebook size or overhead, feedback for disabled HARQ process could be considered as assistant information and beneficial for further scheduling or link adaptation. Thus, enhancements on Type-1 and Type-2 HARQ codebook should be supported. |
| CAICT | Share the opinion by Samsung, Spreadtrum. For Type-2 HARQ codebook, DAI is not included in the DCI for the scheduling of HARQ process with disabled HARQ-ACK feedback. Besides, other bit fields about HARQ-ACK feedback like “PDSCH-to-HARQ_feedback timing indicator”, “PUCCH resource indicator”, “Downlink assignment index”, and etc are not needed in the DCI for scheduling the HARQ process which HARQ is disabled. Further enhancements could be discussed. |

# Restriction on HARQ feedback disabling

In addition to the agreement that disabling of HARQ feedback is done per UE per HARQ process via RRC, for ensuring the efficiency and reliability of transmission carrying the signalling, e.g., RRC configuration/MAC CE command, as highlighted in [CATT, Sony, MTK, ZTE, Ericsson], at least one HARQ process with feedback should be kept. Meanwhile, w.r.t the SPS release, the corresponding feedback should also be kept [Sony]. W.r.t the corresponding specification impact, down selection between following two options are proposed in [Ericsson]:

- Option 1: UE expects that at least one HARQ process is configured with UL HARQ feedback;

- Option 2: UE does not expect a MAC CE activation/deactivation command, which would become effective 3 msec after the UE would transmit the corresponding HARQ-ACK, to be scheduled on a downlink HARQ process with disabled feedback.

Meanwhile, LS with RAN1’s recommendation to inform RAN2 is proposed by [ZTE].

According to the above summary, the following proposals are listed as majority views:

[Initial Proposal 4]: UE expects that at least one HARQ process is configured with HARQ feedback.

| Company | Comments and Views |
| --- | --- |
| MediaTek | Agree |
| CATT | Agree |
| OPPO | We would like to understand what is the motivation of this proposal? If at least one HARQ process is configured with HARQ feedback, how does the UE expect the gNB to use this HARQ process? If it is completely up to gNB implementation, we don’t see the necessity to have this constraint, as the configuration should naturally be up to gNB implementation as well. |
| Panasonic | The basic assumption of above two options might be that MAC CE is always transmitted with HARQ enabled process. Before discussing the two options, we think it should be determined first whether to mandate MAC CE transmission with HARQ enabled process or to leave it up to network implementation. Our position is up to network implementation, but this should be determined in RAN2. Handling of MAC CE transmitted in DL and UL can be different as what information is to be sent in DL is usually up to network operation but UL may be specified. |
| Ericsson | We support this proposal. |
| Intel | Support the proposal |
| Apple | Agree |
| Sharp | The restriction is not necessary. It could be up to network implementation without any spec effort. |
| Qualcomm | Better to be decided by RAN2. |
| Xiaomi | Agree |
| Huawei | Support the FL proposal. |
| ZTE | Support |
| Samsung | Support Proposal 4 |
| Spreadtrum | Support |
| Lenovo, MotoM | Support the proposal, It could be up to network implementation without any spec effort and the proposal may be a conclusion. |
| Sony | Support initial proposal 4, and support sending LS to RAN2. Some specific DCI also needs HARQ-ACK feedback. - The HARQ-ACK response for SPS release should be kept even though the HARQ-ACK is disabled for SPS PDSCH. - The HARQ-ACK response for SPS activation should be supported when the HARQ-ACK is disabled for SPS PDSCH. |
| APT | Support Initial Proposal 4 |
| ETRI | Same view as Qualcomm |
| CMCC | Support |
| Nokia, Nokia Shanghai Bell | Do not agree. This should be left to gNB implementation or alternatively to RAN2. Companies have agreed that each HARQ process should have possibility to be configured on/off operation for the HARQ feedback. At present we do not see any need to limit the gNB flexibility. |
| Thales | We support the proposal |
| vivo | Agree with the proposal. |
| CAICT | Agree |
| China Telecom | Agree |

# Performance enhancements

For enhancing the performance of transmission, in last meeting, different solutions including potential parameters configurations are proposed by companies. In this meeting, following aspects are categorized into following aspects according to the views from each company:

- Enhancements on aggregated transmission (including repetition)

For this aspect, view from 12 companies are elaborated with corresponding enhancements mainly includes:

  - Value of aggregation factor:

As highlighted by [vivo, CATT, CMCC, MTK, ETRI, ZTE, Spreadtrum, Lenovo], supports of the larger aggregation factor is beneficial for NTN.

  - Indication of aggregation factor:

[CMCC, Lenovo, Ericsson, ETRI] prefer to introduce different configurations for the transmission via HARQ process with enabled or disabled feedback. But, the unified parameters configuration is preferred by [CATT].

Meanwhile, DCI based indication for the repetition/aggregation related information is mentioned by [HW, Samsung]. More specifically, enhancement under the Rel-16 framework (e.g., numberOfRepetitions-r16 included in TDRA configuration)) is preferred by [Samsung].

  - Transmission scheme:

In case of supports on larger aggregation factor, the time-interleaved transmission is mentioned in [CATT, CMCC, vivo]. Moreover, as results shown in [ZTE], performance gain can be achieved for the transmission with reduced DM-RS density. [Panasonic] proposes to introduce the scaling factor for TBS determination.

- Enhancements on MCS (including CQI report)

For this aspect, view from 8 companies are elaborated with corresponding enhancements mainly includes:

  - Different MCS table configuration:

[vivo, Ericsson] prefer to introduce different configurations of MCS table in TBS determination for transmission via HARQ process with enabled or disabled feedback. But, the unified parameters configuration is preferred by [CATT, Lenovo, ZTE].

  - CQI report:

A new CQI table with larger BLER e.g., 1% [IDC] is proposed by [Thales, IDC, Qualcomm]. But this part is not preferred by [ZTE] since similar performance can be achieved by implementation of scheduling.

- Blind retransmission

As highlighted by [Nomor Research GmbH, Thales, IDC, Spreadtrum, Apple,], supports on blind PDSCH (re)transmission of the same packet by MAC scheduling without waiting for the transmission of the HARQ feedback can be considered. But as highlighted by [CATT], the gain for blind retransmission is not clear and also from [ZTE] perspective, this solution can be conducted by implementation once the time domain scheduling restriction for PDSCH with same HARQ process is updated.

- UCI

As highlighted in [Xiaomi, Qualcomm, ETRI, LG], in case of scheduling with disabled HARQ feedback, additional new UCI feedback, e.g., to report the decoding statistic or reporting DL transmission disruption and/or requesting DL scheduling changes, can be considered to improve the scheduling configuration from gNB side.

- UE assistance information

As mentioned in [Samsung, Huawei], report for the assist information from UE side, e.g., the buffer situation in the DL HARQ procedure [Samsung] via reserved resource [Huawei], is beneficial for scheduling the decision for HARQ scheduling with enabled/disabled feedback.

In addition, other solutions, e.g., long CSI periodicity [Thales, Nomor Research GmbH], RV limitation for scheduling [QC] is proposed.

Based on the above analysis, following proposal is provided according to majority view:

[Initial Proposal 5]: Study on following enhancements is prioritized:

- Enhancements on aggregated transmission (including repetition)

- Enhancements on MCS (including CQI report)

| Company | Comments and Views |
| --- | --- |
| Ligado | Per https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_103-e/Docs/R1-2008802.zip we have demonstrated significant performance gains arising from adaptive blind repetitions (ABR) based on CQI feedback, specifically in NTN where channel blockage is a larger concern than multipath fading. Ligado commends this analysis to companies. The assertion [CATT] that the gain for blind repetition is not clear does not consider the adaptive aspect. ABR should at least be considered for further study. |
| MediaTek | Agree |
| CATT | Aggregation transmission can be enhanced due to lower SINR scenario in NTN. For MCS enhancement, we need to consider it on top of URLLC feature with BLER 10^-1 and BLER^-5, two MCS tables. In addition, in most of cases, NTN channel is similar to AWGN channel due to strong LOS presence. So small fluctuation of SINR will have significantly impact to CQI estimation, but larger feedback delay in NTN scenario will eat the gain of finer CQI reporting since it is impossible to get very accurate CQI. Hence, we think the MCS/CQI enhancement is not motivated. |
| LG | Agree for first sub bullet. Regarding MCS enhancement, we share view with CATT. |
| OPPO | We agree to discuss the aggregated transmission. However, as blind retransmission is proposed by many companies too, it would be also meaningful to have some places for discussing blind retransmission. At least to us, it is very clear why the blind retransmission is ruled out. |
| Panasonic | We support proposal 5. Enhancement on aggregated transmission (including repetition) can be beneficial for throughput improvement with a limited number of HARQ processes as well as reliability improvement. As the enhancement, we propose to introduce TB scaling factor with repetition (aggregated transmission) in order to use the repetition to improve the user throughput. User throughput for 32 HARQ processes is still restrictive especially for Ka-band (i.e. larger SCS) because the number of slots that can be scheduled within RTT is too low (e.g. 32 out of 335 slots in case of SCS 120kHz). By using scaling factor (> 1) for transport block size determination to gather with repetition, user throughput can be improved. For example, 4 times larger TB size is used for 4 times repetitions. In this way, the number of slots used for the transmission and amount of data within RTT can be increased by 4 times, thus throughput is improved with a limited number of HARQ processes like 32. Note that because scaling factor for transport block size determination (≤1) is already used for PDSCH scheduled by RA-RNTI and P-RNTI, additional complexity would be trivial. |
| Ericsson | Prioritization of which enhancements to study can be left to each contributing company. The necessity of proposed enhancements should be justified with at least simulation results. Then companies could check the results and discuss whether a specific enhancement is needed. Note that we do not propose larger aggregation factor but configuration of different aggregation factors for enabled/disabled HARQ processes. So we would appreciate if FL could move our company name from the first sub-bullet in (1) to the second sub-bullet in (1). |
| Intel | Agree with the proposal |
| Apple | Agree with the proposal on the first bullet, but we think blind retransmission should also be included. Although blind retransmission has higher PDCCH signaling overhead than that in aggregated retransmission, it has the flexibility in selecting retransmission resources, as well as in controlling the number of retransmissions. |
| Sharp | Agree |
| Qualcomm | Support |
| Xiaomi | Fine with the first bullet. For the second bullet, we want a more generic description, not limited to CQI report. |
| Huawei | Support the FL proposal for first bullet. The necessity of second bullet is not clear to us. In addition, the proposal should not preclude other enhancements of UCI and UE assistance information, in general, the contents of new UCI described above is also a kind of assistant information. gNB have to schedule new time/frequency resources for this kind of information. Our proposal for rewording the draft agreement is: [Initial Proposal 5]: Study on following enhancements is prioritized: - Enhancements on aggregated transmission (including repetition) - Enhancements on MCS (including CQI report) - Enhancements on new time/frequency resources for UE assistant information |
| ZTE | Support, fine to prioritize these two items as subbullet. |
| Samsung | In addition to what is included in Proposal 5, we propose to add UE assistance information. |
| Spreadtrum | Agree |
| Lenovo, MotoM | Agree the proposal. We only see the benefit from separate configuration of aggregated transmission (including repetition) |
| Sony | Support initial proposal 5. |
| APT | Agree Initial Proposal 5 |
| ETRI | Agree with the initial proposal 5 on the first bullet (“Enhancements on aggregated transmission (including repetition)”) Based on our simulation in R1-2009017, if the slot aggregation is used, it would be important to control the aggregation factor properly for getting better adaptation. In our view, UCI (for controlling aggregation factor) and different parameter configurations (for providing different aggregation factors) might be beneficial for enhancing adaptation performance of the aggregated transmission. Moreover, regarding UCI, regardless of HARQ feedback availability (enable/disable), it seems that gNB cannot distinguish the difference between too reliable aggregated transmission and just optimal aggregated transmission because ACK (if enabled) might be fed back to gNB in both cases. By controlling aggregation factor via UCI, this problem could be also solved. Thus, UCI and different parameter configurations should be not excluded and considered in combination with the aggregated transmission. to Moderator) we also propose separate aggregation factors depending on HARQ feedback/RNTI/MCS as described in proposal 3 of R1-2009017. So we would appreciate if our company name could be added to the first sentence of the bullet (“Indication of aggregation factor”) : ([CMCC, Lenovo] prefer to introduce … [CMCC, Lenovo, ETRI] prefer to introduce …) |
| CMCC | Agree for first sub bullet. The necessity of second bullet is not clear to us. |
| Nokia, Nokia Shanghai Bell | From our point of view, the priority here should be on feasibility of aggregated transmission through existing mechanisms. That is, considering to increase the number of slots that can be aggregated beyond the current maximum value of 8 slots. This might be useful for the GEO use case where the RTT and potential link budget restrictions may be the limiting factors. At present, we do not see a need to consider enhancements to the MCS signalling or the CQI/CSI reporting mechanisms. Current MCS signalling and CQI/CSI reporting mechanisms seem appropriate. |
| Thales | We support the proposal In case of disabled HARQ feedback and therefore HARQ retransmission, a lower residual BLER target is important to avoid latency intensive RLC retransmissions. Aggregation transmission is one possibility to reduce the residual BLER. Maybe some enhancement on the aggregation factor /repetition will be needed. w.r.t CQI report: According conducted simulations, applying an CQI feedback with 1% PHY BLER target performs better in terms of throughput than applying an CQI feedback with 10% PHY BLER target and an additional offset |
| vivo | Agree to study the enhancements on aggregated transmission. However, we think enhancements on MCS should be deprioritized. Actually, in current specification, there are several MCS tables with low and high spectral efficiency to satisfy various NR scenarios and cases. In our understanding, these MCS levels are enough to cover NTN scenarios and cases by network implementation. Thus, enhancements on MCS should be deprioritized. |
| CAICT | Agree |

# PDSCH/PUSCH scheduling restriction

PUSCH

The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0, 0_1 or 0_2 scrambled by C-RNTI or MCS-C-RNTI for a given HARQ process until after the end of the expected transmission of the last PUSCH for that HARQ process.

PDSCH

The UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process...

In existing specification following restrictions are considered for the PUSCH and PDSCH scheduling with same HARQ process ID, respectively:

To enable the scheduling flexibility via the HARQ with disabled feedback, views on the PDSCH/PUSCH scheduling are elaborated below:

- PDSCH:

[Ericsson] propose to update the restriction as: For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH for the given HARQ process until after the end of the expected reception of the last PDSCH for that HARQ process.

[OPPO] propose to update the restriction with consideration a new scheduling constraint to ensure enough PDSCH processing time between two PDSCH receptions with same HARQ ID.

- PUSCH:

[Qualcomm, OPPO] propose to define a minimum gap between two PUSCHs of a HARQ process, for example, T_pro is defined in [OPPO].  However, as proposed by [Ericsson], the existing scheduling rule should be kept for PUSCH.

Based on the above analysis, following proposal is provided according to majority view:

[Initial Proposal 6]: Following principle is supported for the PDSCH scheduling via same HARQ process with disabled feedback:

- For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH for the given HARQ process until after the end of the expected reception of the last PDSCH for that HARQ process.

| Company | Comments and Views |
| --- | --- |
| MediaTek | Agree |
| CATT | Agree |
| LG | Agree |
| OPPO | We don’t agree with this proposal as is. First of all, the proposal is not very clear to us. Is the ‘another PDSCH’ is a new transmission of a given HARQ or a retransmission? Note that PDSCH repetition is also another PDSCH. Secondly, we think we should first discuss the issues on both PDSCH and PUSCH scheduling restriction. Then based on the discussion outcome to make possible agreement. For PDSCH repetition, the network does not know if the UE needs to decode all the PDSCH repetitions to get an ACK or part of them. Thus, the network should give enough PDSCH processing time. For PUSCH, we have a comment on Ericsson’s proposal for the case illustrated their Tdoc and copied below In this case, we have strong concern that if there is no preparation time between PUSCH1 and PUSCH2, the UE cannot prepare the PUSCH2 in the buffer that was used for preparing PUSCH1 previously. In summary, it would be good to have some discussions among companies to have a clear understanding on the concerns before scrambling to have some quick agreements. |
| Panasonic | We support proposal 6. |
| Ericsson | We support the proposal. |
| Intel | Agree with the proposal |
| Apple | We support the proposal |
| Sharp | We are OK to support. |
| Qualcomm | Not support. Current UE implementation always expects a sufficient time gap between two PDSCHs. This should be kept in order not to change UE processing timeline. An explicit nonzero gap is needed. |
| Huawei | Agree with the FL proposal. HARQ ID for two different PDSCH cannot be identified by UE without DCI for scheduling the other one. |
| ZTE | Agree |
| Samsung | We think this constraint is not needed. Instead, the Rel-16 restriction on PDSCH reception could be applied only for enabled HARQ feedback. |
| Spreadtrum | Agree |
| Lenovo, MotoM | Support the proposal |
| Sony | Support Initial Proposal 6. |
| APT | Support Initial Proposal 6. No sure why the time gap between PDSCHs is needed. |
| CMCC | Agree |
| Nokia, Nokia Shanghai Bell | Do not agree to this proposal. The delay between the DL transmissions for the same HARQ processes should follow existing principles of UE processing. That is, the DL HARQ process may be used again for the same UE when a time has been reached by which the UE would otherwise have sent its HARQ-ACK indication in the uplink. |
| Thales | We support the proposal |
| vivo | Agree. |
| CAICT | Agree |

# Summary for the 1st round discussion

# #Issues-1: Enhanced HARQ process ID indication

W.r.t the Initial Proposal-1 in section 2, in the 1st round discussion, 21 companies are provided views:

- Option-1:

  - Supportive: 9 companies (OPPO, Panasonic, Intel, Qualcomm, Xiaomi, Lenovo, Sony, ETRI, Thales)

    - [Panasonic, Thales]prefers to take it as 2nd priority

  - Negative:  5 companies (Ericsson, Huawei, ATP, Nokia, ZTE)

- Option 1a:

  - Supportive: 9 companies (Panasonic, Intel, Qualcomm, Xiaomi, Sony, ETRI, ZTE, CMCC, Thales)

    - [Panasonic, CMCC, ZTE, Thales] prefer to take it as 2nd priority

  - Negative:  2 company (Ericsson, Nokia)

- Option 2:

  - Supportive: 7 companies (CATT, Apple, Sharp, ZTE, Sony, ETRI, CMCC, vivo)

    - [CMCC] prefers to take it as 2nd priority

  - Negative: 2 company (APT, Nokia)

- Option 3:

  - Supportive: 12 companies (MediaTek, CATT, Panasonic, Ericsson, Sharp, Samsung, Lenovo, APT, CMCC, Nokia, Thales, CAICT)

    - [CMCC] prefers to support this option for DCI format X-2 as starting point.

    - [Panasonic] highlights that no bits should be added for DCI 0-0/1-0.

  - Negative: 2 company (Xiaomi, Huawei, ZTE)

In addition, one company (Ericsson) is negative to enhance the DCI 0-0/1-0.

From moderator perspective, it seems that Option-3 is preferred by most of companies at least for DCI format X-2, which is also aligned with motivation for current design. In addition, Option 1a and Option-2 is preferred by companies with less negative views. Then, the following updated proposal is used for the 2nd round discussion.

[Initial Proposal 1]: Enhanced HRAQ process ID indication is supported with following options:

- For DCI 0-2/1-2:

  - Option 3: Extending the HARQ process ID field up to 5 bits

- For DCI 0-1/1-1:

  - Option 1-a: Slot index as the LSB

  - Option 2: Reusing one bit from other bit field

  - Option 3: Extending the HARQ process ID field up to 5 bits

  - Note: Down-selection is needed and only one option will be supported;

- For DCI 0-0/1-0

  - Further discussion on the necessity for enhancement with consideration on the UE status.

| Company | Comments and Views |
| --- | --- |
| OPPO | agree |
| Samsung | Basically we don't see the need to use different solution according to DCI format. We support Option 3 also for DCI 0-1/1-1, which is the simplest one. This would makes consistency among DCI formats. |
| Panasonic | Support proposal 1. We share the view with Samsung. |
| Qualcomm | Same solution is preferred. As such, option 1-a should also be added for DCI 0-2/1-2 when more than 16 HARQ processes are configured, i.e., - For DCI 0-2/1-2: - Option 1-a: Slot index as the LSB when more than 16 HARQ processes are configured - Option 3: Extending the HARQ process ID field up to 5 bits |
| LG | We also thinks same solution is preferred among DCI formats. |
| China Telecom | We also agree to have a same solution for all formats, chosen from opt.2 or opt.3. |
| Apple | We also prefer a unified solution for all DCI formats. |
| APT | Support Initial Proposal 1. If down-selection can be done in this meeting, here is our preference: Extending the HARQ process ID field up to 5 bits for non-fallback DCI formats. No change for fallback DCI formats. |
| CATT | No need many options. We prefer same method for all DCI formats and use 5 bits for HARQ ID indication. |
| Huawei | We support the idea of having one unified solution for both DCI format X-1 and X-2. We think Option 1/1a or Option 2 can also work for both DCI format X-1 and X-2. Hence we would like to add Option 1a and Option 2 back for DCI format X-2. One clarification to the FL summary is that we are not against Solution 1/1a. Our first round comment was related to splitting the solutions based on whether they are explicit or implicit. So please remove Huawei from the list of companies negative to Option 1. |
| Thales | We support option 3 for DCI 0-2/1-2 Extending the HARQ process ID field up to 5 bits is to be considered also for DCI 0-1/1-1 |
| ZTE | Agree to take current proposal as starting point for further discussion and for achieving the unified design (if necessary), all DCI format (DCI X-0/1/2) should be considered. |
| Sony | We prefer a unified solution for all DCI formats. Increasing the HARQ process number without enlarging the HARQ number field is our first preference, e.g., Option 1/1-a, but Option 3 for all DCI formats can be a compromise if adding 1 bit is desired. |
| Xiaomi | We share QC’s view |
| MediaTek | Support proposal 1. Down selection is needed. Preference for Option 3 |
| ETRI | Support Initial Proposal 1 for further discussion. Additionally, we also prefer a same solution for all DCI formats. |
| CMCC | Support Proposal 1. If down-selection can be done in this meeting, we prefer to at least support Option 3 for both DCI X-2 and X-1. We are open to support Option 3 for DCI X-0. |
| Ericsson | For DCI format 0_2/1_2, we support the proposal. For DCI format 0_1/1_1, we want to emphasize that there are many fields in these DCI formats with sizes that depend on the configuration (which is the design intention since Rel-15 that non-fallback DCI format supports ease of adding new features in NR for forward compatibility), so Option 3 is in line with common practice. Further, adding one bit has negligible performance impact (typically size of DCI format 0_1/1_1 is ~80 bits. Adding 1 bit only affects coverage by 10*log10(81/80) = 0.05 dB). Therefore, we don’t think Option 1-a and Option 2 need to be listed, considering their negative impact on scheduling flexibility and link adaptation flexibility (depending on what other bit field that is reused), respectively. For DCI format 0_0/1_0, it is acceptable to limit the number of HARQ processes to 16 but we do not oppose the proposed text. |
| CAICT | Support Option 3 for DCI 0-1/1-1. Prefer same solution. |
| vivo | We also think it’s better to have a unified solution for all the DCI formats, which can be chosen from option2 and option3. |
| Nokia, Nokia Shanghai Bell | First, we would like to re-iterate our response from first round: According to our understanding, the feature lead (and several other companies) have overinterpreted the agreement from last meeting. The agreement states: “The extension of maximal HARQ process number can be considered with following assumptions:”. So it was agreed to consider to extend the maximum number of HARQ processes, but formally, there has not been any agreement for extending the amount of HARQ processes. Hence, this discussion point is not suitable as it is built on an erroneous assumption/interpretation. Our opinion is that the HARQ stalling problem for both UL and DL can be addressed either by the agreed possibility of disabling HARQ for selected processes or by and using slot aggregation, and hence it should be possible to reach continuous scheduling for a single UE. Second, we have not yet had any discussion as to whether or not the considerations discussed above would lead to increase of number of HARQ processes for both UL and DL. According to our understanding, the control of the UL HARQ is completely in the hands of the gNB, so no need to even consider increasing the number of HARQ processes for this operation. Again, slot aggregation for the UL transmissions would ensure that we can have continuous UL transmissions for a single UE. Hence, any enhancements for DCI formats 0_0, 0_1, and 0_2 should be excluded, while discussions for the DL DCI formats should only be considered if/when it has been decided to use up to 32 HARQ processes for the DL. Third, if it is decided to increase the number of HARQ processes to 32, we should consider and respect the agreement from last meeting, which explicitly states that focus should be on minimizing impacts on specifications and scheduling. Such minimization would happen when using a direct indication of the HARQ process to use using explicit indication. Hence, extension of the HARQ process ID field up to 5 bits would be the only logical approach for DL DCI formats which we may identify as needing support for the the potential additional HARQ processes. |
| Intel | Our initial preference is Option 1-a. However, unified solution for different DCI formats is preferable, so we are fine to compromise and support Option 3. |

W.r.t the Initial Proposal-2 in section 2, in the 1st round discussion, 23 companies are provided views:

- Up to 17 companies (MediaTek, LG, OPPO, Intel, Apple, Sharp, QC, Xiaomi, Spreadtrum, Lenovo, Sony, APT,ETRI, ZTE,CMCC, Thales, vivo) are fine with the proposal;

  - [QC] proposed to update the wording to highlighting the case beyond 16.

- [CATT, CAICT] prefer more clarification on the proposal.

- [Panasonic, Huawei, Nokia] prefer to keep to take it, i.e., supports on up 32 HARQ processes, as mandatory feature.

- [Ericsson] highlights that the newly introduced feature will be enabled by RRC parameters from gNB scheduling.

- [Samsung] prefer to joint consider it with maximum TBS.

From moderator perspective, it seems that such topic is highly related to the UE implementation, and it’s recommended to treat it later during the UE feature discussion.

# #Issues-2: HARQ codebook enhancements

W.r.t the Initial Proposal-3 in section 3, in the 1st round discussion, 19 companies are provided views:

- Up to 11 companies (LG, Ericsson, Intel, Sharp, Samsung, Spreadtrum, Sony, APT, Thales, vivo,CAICT) are general with the proposal:

  - Supports on the Type-3 codebook is mentioned by [LG] and it’s can be FFS for [Ericsson].

  - [Samsung, Spreadtrum, CAICT] prefer to change the wording by replacing the “support” with “considered”;

- Up to 6 companies (OPPO, Apple, Qualcomm, Lenovo, Huawei, Nokia) prefer to capture more detailed options for discussion.

- [CATT] is negative for Type-1 codebook and [Panasonic] is not supportive for all types.

From moderator perspective, it’s fine to further discussion it with more detailed options. Companies are encouraged to justify the pro and con for each solution.

With consideration on the majority views and also to address the concerns, following updated proposal is used for the 2nd round discussion.

[Initial Proposal 3]: Discussion on HARQ codebook enhancement with following options is considered:

- Type-1 HARQ codebook:

  - Option-1: No enhancement;

  - Option-2: Report NACK on disabled process

  - Option-3: Reduce codebook size with criteria

- Type-2 HARQ codebook:

  - Option-1: No enhancement;

  - Option-2: Reduce codebook size with only considering the scheduling via HARQ process with enabled feedback

| Company | Comments and Views |
| --- | --- |
| OPPO | We understand no enhancement is by default supported, i.e. even if an enhanced CB is supported, it does not preclude R15 codebooks. |
| Samsung | We are open to discuss further with options, but we support Option 3 for Type-1 and Option 2 for Type-2. We need to consider low SINR environment for NTN and need to reduce codebook size. It seems better to make it clear that RAN1 will choose only one option. |
| Panasonic | Agree to discuss these options. We see general view is to support type-1 and type-2 HARQ codebook in NTN. For Type-1, option 2 would be preferable because it requires the minimum essential correction to support type-1. We don't see the need of optimization like option 3. If the codebook size reduction is necessary, it is achieved by type 2. For type 2, option 2 is ok as it is straightforward. |
| Qualcomm | Suggest to clarify “no enhancement”. Does “no enhancement” mean HARQ processes with feedback disabled is treated the same way as HARQ processes with feedback enabled in terms of HARQ reporting and DAI counter? |
| LG | Fine with proposal. We suggest to add “FFS on Type-3 HARQ codebook” in third main bullet. |
| Apple | Agree to discuss these options. For type-1 HARQ codebook, we are open to Option 1 and Option 2. Since type-1 HARQ codebook is semi-static, we assume UE does not reduce the HARQ codebook size resulting from HARQ processes with disabled HARQ feedback. For type-2 HARQ codebook, we prefer Option 1 and think network can handle this case. Specifically, UE is not expected to receive increased counter DAI for HARQ processes with disabled HARQ feedback. |
| APT | Support Initial Proposal 3. If down-selection can be done in this meeting, here are our preference: Option 2 for Type-1 and Type-2. Also, we believe for the no enhancement options, some minor corrections cannot be avoided. |
| CATT | We are fine to current proposal. If need down-selection, for type 1 codebook, option-1 is supported, and option-2 is supported in type 2 codebook. |
| Spreadtrum | We are fine to current proposal. |
| Huawei | First of all, we are not sure about the interpretation of “no enhancement” for both Type-1 and Type-2 HARQ-ACK codebook. Does it mean for disabled HARQ processes that: - the gNB ignores the ACK/NACK field and the UE can set it freely in Option 1 or - the UE should still generate ACK/NACK based on the PDSCH decoding result? For further down selection, we support Option 3 for Type-1 HARQ-ACK codebook since it will reduce unnecessary overhead for the disabled HARQ process. For Type-2 HARQ-ACK codebook, we support Option 2, i.e. DAI is only incremented for enabled HARQ processes. |
| Thales | We support option3 for Type-1 HARQ CB and option 2 for Type-2 HARQ CB |
| ZTE | For both type-1 and type-2, enhancement to reduce the codebook size is preferred. So, Option-3 for type-1 and Option-2 for type-2 is prioritized. If no enhancement is agreed for any type, same behaviour for UE according to current spec is expected, i.e., the UE should still generate ACK/NACK based on the PDSCH decoding result. But it’s contradict to the definition of “disabled” HARQ process. |
| Sony | We support the proposal. For Type-1 HARQ codebook, we would like to clarify that in RAN2#105 meeting, it was agreed that there is no UL feedback for DL transmissions if the HARQ is disabled. If the option 1 which is no enhancement means UE decode the PDSCH and then report the ACK/NACK based on the decoding results to the gNB, we think this option may not necessary. For further down-selection, we support Option 3. For Type-2 HARQ codebook, we think the Option 2 is supported naturally. And we suggest adding enhancement on DAI as many companies mentioned in the 1st round discussion. And due to the large overhead of Type-1 HARQ codebook, we think Type-2 HARQ codebook should be default HARQ codebook in NTN. Agreements: 1. Retransmissions at one or several layers shall be supported for NTN and configurable by the network. 2. The network should be able to configure the UE whether the HARQ is “turned off”. There is no UL feedback for DL transmission in the if HARQ is turned off. FFS the impact on other procedures and how to configure.. |
| MediaTek | Support proposal 3. Agree to discuss these options |
| CMCC | Agree to discuss these options. For type-1 HARQ-ACK codebook, we support Option 1, more specifically, the UE still generate ACK/NACK based on the PDSCH decoding result, even if it is corresponding to disabled HARQ process. W.r.t. Option 3: - If Type 1 HARQ-ACK codebook is determined based on semi-static configuration, the scheduling flexibility is significant restricted. - If Type 1 HARQ-ACK codebook is determined based on dynamic scheduling, error cases from miss detection of DCI scheduling disabled HARQ process may results in mis-understanding between the gNB and the UE both on the bit size of Type-1 HARQ-ACK codebook and the HARQ-ACK bit mapping relationship as shown below. W.r.t. Option 2: Option 1 is more preferred than Option 2 since it requires no specification modification. For type-2 HARQ-ACK codebook, we support Option 2. Furthermore, further enhancement on error correction capability for disabled HARQ process seems unnecessary, since in both cases of UE detect NACK for disabled HARQ process (the SN for RLC PDU can NOT be decoded) and the DCI scheduling disabled HARQ process is miss-detected by the UE, the RLC feedback process can NOT be trigged. So it seems no need for the UE to recognize the DCI miss detection event for disabled HARQ process and to inform the event to its high level. |
| Ericsson | For Type-1 HARQ codebook, we would like to add a clarification for Option-3 that the codebook size should remain semi-static. Also it should be clarified that only one option will be chosen. For Type-2 HARQ codebook there seems to be strong support and no strong concerns for codebook size reduction using the method proposed by several companies, i.e., that DAI is not incremented on disabled processes. Therefore, we propose to remove Option 1 and add more details to Option 2. [Initial Proposal 3]: Discussion on HARQ codebook enhancement with following options is considered: - Type-1 HARQ codebook: - Option-1: No enhancement; - Option-2: Report NACK on disabled process - Option-3: Reduce codebook size with criteria while keeping the codebook size semi-static - Note: Down-selection is needed and only one option will be supported; - Type-2 HARQ codebook: - Option 1: No enhancement - Option 2: Reduce codebook size as follows: - with only considering the scheduling via Increment DAI only for HARQ process with enabled feedback in the DCI, and include only these processes in the HARQ codebook. FFS: Other HARQ codebook types |
| CAICT | Agree with the proposal. For Type-2 HARQ codebook, DAI is not included in the DCI for the scheduling of HARQ process with disabled HARQ-ACK feedback. |
| vivo | Fine to discuss these options, and we prefer Option-3 for Type-1 and Option-2 for Type-2. For Type-1, if all the HARQ processes within the occasions are disabled, this HARQ-ACK feedback can be omitted. If both enabled and disabled HARQ processes within the occasions are transmitted, insert ACK or NACK depends on the decoding results of PDSCHs in the positions corresponding to PDSCHs associated with the disabled HARQ processes. In other word, R15 HARQ codebook is reused for this situation. For Type-2, Option 2 is straightforward. |
| Nokia, Nokia Shanghai Bell | Type-1 HARQ codebook: Initial opinion: Option 1 Type-2 HARQ codebook: Initial opinion: Option 1 Both responses are provided with the assumption of minimizing specification impact. |
| Intel | We prefer option 1 for Type-1 HARQ codebook and option 2 for Type-2 HARQ codebook |

# #Issues-3: Restriction on HARQ feedback disabling

W.r.t the Initial Proposal-4 in section 4, in the 1st round discussion, 24 companies are provided views:

- Up to 17companies (MediaTek, CATT, Ericsson, Intel, Apple, Xiaomi, Huawei, ZTE, Samsung, Spreadtrum, Lenovo, Sony, APT, CMCC, Thales, vivo, CAICT) are fine with the proposal.

- Up to 4 companies (OPPO, Panasonic, Sharp, Nokia) prefer to keep it as gNB implementation.

- Up to 3 companies (QC, Sony, ETRI) prefer to handle this issue by RAN2.

From moderator perspective, this topic have been discussed for two meeting, and most of companies prefer to take it with corresponding conclusion/agreements to at least resolve the MCE CE issue. With consideration on the comments that for RAN2 part, maybe a LS is needed to RAN2 with potential solution/recommendation in RAN1.

With consideration on the majority views and also to address the concerns, following updated proposal is used for the 2nd round discussion.

[Initial Proposal 4]: A LS to RAN2 with capturing following options is needed to resolve the issue related to MAC CE activation/deactivation command:

- Option 1: UE expects that at least one HARQ process for DL scheduling is configured with UL HARQ feedback;

- Option 2: Up to gNB’s implementation for scheduling

| Company | Comments and Views |
| --- | --- |
| OPPO | We think the motivation is not clear. We would propose the following rewording to make the motivation clearer. Option 1: UE expects that at least one HARQ process for DL scheduling is configured with UL HARQ feedback and MAC-CE without UL HARQ feedback is not supported Option 2: Up to gNB’s implementation for scheduling |
| Samsung | We would like to be clarified the purpose of sending the LS. We think RAN1 can decide one option and make progress based on that. |
| Panasonic | Support proposal 4. We agree with OPPO’s refinement. |
| LG | Agree with Samsung. |
| APT | Support Initial Proposal 4. Agree with OPPO. UE does not expect to receive any MAC CE activation/deactivation command without scheduling the corresponding HARQ-ACK feedback. |
| CATT | We are fine for this proposal. Since this issue had been discussed in RAN1 with several meetings but without final conclusion, then LS to RAN2 is needed. OPPO’s refinement seems good. |
| Spreadtrum | We are fine for this proposal. |
| Huawei | Agree with Samsung. |
| Thales | We support option 1 |
| ZTE | Agree with this proposal, But if RAN1 conclusion on this topic can be made. The LS for RAN2 may not be necessary since the original intention is to consult RAN2’s views on the MAC CE related scheduling. |
| Sony | Agree with ZTE |
| Xiaomi | Not sure about the necessity of sending the LS |
| MediaTek | Support proposal 4. Preference for Option1. |
| ETRI | We are fine for Initial Proposal 4. And agree with OPPO |
| CMCC | We are fine for this proposal. |
| Ericsson | We support the proposal. |
| CAICT | Agree |
| vivo | We share the same views with OPPO. |
| Nokia, Nokia Shanghai Bell | Ok to send LS to RAN2, but we would like to re-iterate that configuration of the system is normally left to the gNB. |
| Intel | We prefer option 1 with modification from OPPO |

# #Issues-4: Performance enhancements

W.r.t the Initial Proposal-5 in section 5, in the 1st round discussion, 25 companies are provided views:

- Up to 10 companies (MTK, Intel, Sharp, QC, ZTE, Spreadtrum, Sony, APT, CAICT, Thales) are fine with the proposal without additional comments.

- Up to 12 companies (CATT, LG, OPPO, Apple, HW, Samsung, Lenovo, ETRI, CMCC, Nokia, Xiaomi, vivo) are fine with the 1st sub-bullet of proposal.

  - And up to 6 companies (CATT, LG, Lenovo, CMCC, Nokia, vivo) are negative for the 2nd sub-bullet of proposal.

- Up to 7 companies (Ligado, OPPO, Panasonic, Apple, Huawei, Samsung, ETRI) propose to add other solutions per company’s interest.

In addition, Ericsson points out the necessity of proposed enhancements should be justified with at least simulation results.

From moderator perspective, this topic have been discussed for two meeting with a lot of candidate solutions. With consideration on the progress, it’s better to identify some prioritized aspects for discussion (according to majority view). Meanwhile, such prioritization does not imply that the solution will be definitely supported and corresponding evaluation is preferred.

With consideration on the majority views and also to address the concerns, following updated proposal is used for the 2nd round discussion.

[Initial Proposal 5]: Study on following enhancements is prioritized:

- Enhancements on aggregated transmission (including repetition)

- Enhancements on MCS (including CQI report)

- Note-1: Supports on the detailed solution should be justified.

- Note-2: For other solutions, companies are encouraged to provide the results to justify the gain comparing to these two in sub-bullets.

| Company | Comments and Views |
| --- | --- |
| OPPO | Blind retransmission bring similar benefit as aggregated transmission. It is not meaningful to support both. Therefore, we propose to discuss either support aggregated transmission or blind retransmission. Secondly, the enhancement on MCS table does not seem to reach consensus. Therefore, we propose the following updated proposal. [Initial Proposal 5]: Decide to support enhancement on either aggregated transmission (including repetition) or blind retransmission. - Note-1: Supports on the detailed solution should be justified. - Note-2: For other solutions, companies are encouraged to provide the results to justify the gain comparing to these two in sub-bullets. |
| Samsung | Support Proposal 5 |
| Panasonic | Support Proposal 5. We agree with Moderator’s comment that the proponents should provide evaluation results. As an enhancement on aggregated transmission (including repetition), we propose transport block size scaling for aggregated transmission (repetition) that allows to increase the number of slots within RTT with a limited number HARQ processes. Simulation results are provided in our contribution R1-2009050, and a significant user throughput gain is seen. We believe this enhancement is worth to consider. |
| Qualcomm | Support |
| LG | Partially agree. As commented earlier, we are not sure on the second sub bullet (enhancement on MCS). |
| Apple | We share the view of OPPO to study the enhancement of blind retransmission together with aggregated transmission. |
| APT | Support Initial Proposal 5 |
| CATT | For the second bullet, - Enhancements on MCS (including CQI report) The benefit is unclear for NTN case, since we need study it on top of Rel-15 and Rel-16 features. |
| Spreadtrum | Support Proposal 5 |
| Huawei | Support Proposal 5 |
| Thales | We support Initial Proposal 5 |
| ZTE | Support to take it as starting point for discussion |
| Sony | Support the proposal. |
| Xiaomi | For the second sub-bullet, prefer to have a more generic description in the bracket, e.g, enhanced UE reporting |
| MediaTek | Support proposal 5. |
| ETRI | Support Initial Proposal 5 as a start point for further discussion. |
| CMCC | Partially agree. We share the view of LG. The benefit of the second sub bullet is unclear. |
| Ericsson | We don’t think there is sufficient justification to include or exclude any particular performance enhancement proposal at this stage. |
| CAICT | Agree |
| vivo | We support proposal 5 with the 1st sub-bullet. As commented earlier, existing MCS levels in current specifications are enough to cover NTN scenarios and cases by network implementation. Thus, enhancements on MCS should be deprioritized. |
| Nokia, Nokia Shanghai Bell | As a starting point we suggest that we keep the specification work as simple as possible, and hence primarily focus on enhancements on aggregated transmissions, while further enhancements could be considered only if companies can show a substantial gain of such additions (including commitment to implement such additions). |

# #Issues-5: PDSCH/PUSCH scheduling restriction

W.r.t the Initial Proposal-6 in section 6, in the 1st round discussion, 22 companies are provided views:

- Up to 16 companies (MediaTek,CATT,LG,Panasonic,Ericsson,Intel,Apple,Sharp,Huawei,ZTE,Spreadtrum, Lenovo, Sony, APT, CMCC, Thales, vivo, CAICT) are fine with the proposal

- Up to 4 companies (OPPO, QC, Samsung, Nokia) has concerns on it.

For addressing the concerns from these companies, following views are provided by moderator:

  - W.r.t the necessity of this proposal: Rel-16 restriction (cited in section 6) can only be used for the scheduling with enabled HARQ feedback, which is not suitable for the case with disabled feedback. So additional (new) rule is needed.

  - W.r.t the terminology “another PDSCH”: This description is same as current specification (cited in section 6) and refer to the new PDSCH scheduled by new DCI (which can be either new transmission or re-transmission). From moderator perspective, there is no ambiguity. In order to further clarify, “the reception of PDSCHs by different scheduling” is updated in the main bullet.

  - W.r.t the PDSCH process time: In current specification, the PDSCH process timing (defined in section 5.3 in TS 38.214) is mainly for timeline with HARQ-ACK feedback/CSI/Cross carriers scheduling, etc. For example, the time gap, e.g., Tproc,1 is defined between the transmission of ACK and the end of the last symbol of the PDSCH carrying the TB being acknowledged.

For the HARQ disabled case, the required time for the PDSCH processing can be assumed same as legacy behavior. But the detailed value for required gap should be further discussed since not HARQ-ACK information is needed. For addressing the concerns, the gap X is introduced in the proposal and detailed value is FFS.

With consideration on the majority views and also to address the concerns, following updated proposal is considered as offline consensus:

[Proposal 6]: Following principle is supported for the reception of PDSCHs by different scheduling via same HARQ process with disabled feedback:

  - For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH for the given HARQ process until [X] after the end of the reception of the last PDSCH for that HARQ process.

    - FFS: value of X, e.g., X= Tproc,1;

For the PUSCH related issue as pointed by OPPO, according to the current specification (section 6.4 in TS 38.214), the processing time Tproc,2 is only defined to handle the timeline between the transmission of PUSCH and reception of PDCCH along with the restriction cited in section 6. For NTN, since there is no other modification for the UL scheduling, additional changes seems not to be needed. W.r.t the case for successive transmission of two PUSCH by different scheduling, it can be handled by gNB implementation.

Then, companies are encouraged to express the views on above analysis to identify the necessity for enhancement.

| Company | Comments and Views |
| --- | --- |
| OPPO | We don’t agree that the issue can completely be handled by gNB implementation. It is a PUSCH preparation time issue illustrated in the figure, unless we restrict the gNB to avoid this contiguous PUSCH with the same HARQ process number case, otherwise, the gNB implementation won't help here. |
| Samsung | We are fine in principle. It seems better to use "TB" rather than only PDSCH. So we suggest the following modification. Following principle is supported for the reception of PDSCHs by different scheduling via same HARQ process with disabled feedback:  For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH a PDSCH of another TB for with the given HARQ process until [X] after the end of the reception of the last PDSCH of a TB with for that HARQ process.  FFS: value of X, e.g., X= T_Proc,1; |
| Panasonic | Support proposal 6. |
| Qualcomm | The first sentence is not very clear. We propose to change the proposal as follows. [Proposal 6]: Following principle is supported for the reception of PDSCHs by different scheduling via associated with the same HARQ process and with disabled feedback: - For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH for the given HARQ process until [X] after the end of the reception of the last PDSCH for that HARQ process. FFS: value of X, e.g., X= Tproc,1; X=max(Tproc,1, k*Tslot ) with k being the minimum of dl-DataToUL-ACK if configured and 0 otherwise. |
| LG | We support most updated version by FL as below:  For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH for the given HARQ process until [X] after the end of the reception of the last PDSCH for that HARQ process.  FFS: value of X |
| APT | Support Proposal 6. Agree with LG’s wording. |
| CATT | Support proposal 6. |
| Spreadtrum | Support proposal 6. |
| Thales | We support the proposal |
| ZTE | Support with updates in the email reflector. |
| Sony | Support the proposal. |
| MediaTek | Support proposal 6 |
| CMCC | Support the proposal. |
| Ericsson | We agree with the above analysis for PUSCH. Scheduling of consecutive PUSCH for the same HARQ process without a time gap is supported in the current specification, despite there may be 2 interpretations of the spec text, as analysed in our contribution R1-2009093. |
| vivo | Support proposal 6. We agree the value of X is Tproc,1 as a start point. If no HARQ-ACK is needed for the given HARQ process, which is related to the solution of #issues-2, the value of X can be smaller than Tproc,1. |
| Nokia, Nokia Shanghai Bell | Comments on this already provided in the online discussion thread, but response is put here as well: Thanks for this proposal. From our side we have a few comments related to this proposal: - The preamble is in principle not needed, as the text in the bullet is able to stand by itself, so proposal could simply start with the text in the bullet. - Our preference would be to lean on the existing text for data reception for the same HARQ process, where the minimum delay between two transmissions is defined by the parameter dl-DataToUL-ACK, which is upper-bounded by a value of 15 according to RRC specifications. Hence, our proposal for update would be: [Proposal 6]: Following principle is supported for the reception of PDSCHs by different scheduling via same HARQ process with disabled feedback:  For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH for the given HARQ process until [X] slots after the end of the reception of the last PDSCH for that HARQ process. - X is configured by network and within the range of [1:15]  FFS: value of X, e.g., ; |
| Intel | Support proposal 6 |

# Summary for the 2st round discussion

# #Issues-1: Enhanced HARQ process ID indication

For the Initial Proposal-1 in section 7.1, in the 2st round discussion, 22 companies are provided views:

- Up to 7 companies [OPPO, Panasonic, APT, ZTE, MediaTek, ETRI, CMCC] are supportive for this proposal.

- Up to 12 companies (Samsung, Panasonic, Qualcomm, Xiaomi, LG, China Telecom, Apple, Huawei, Sony, ETRI, CAICT, vivo) prefer to define a unified solution for all DCI formats

  - Option-3 is supported as the candidate by 12 companies as [Samsung, Panasonic, China Telecom, CATT, Thales, MTK, CMCC, Ericsson, vivo, Nokia, Intel]

  - Option-1a is supported as the candidate by 5 companies a [Qualcomm, Xiaomi, Huawei, Sony, intel]

  - Opiton-2 is supported as candidate by 4 companies [China Telecom, ZTE, vivo]

- W.r.t DCI 0-0/1-0:

  - APT is negative to extend the bit field. But [ZTE, Sony] prefer same solution. [CMCC, Ericsson] are open for discussion.

From moderator perspective, it’s clear that unified solution is preferred by companies with majority view. W.r.t the detailed solution, Option-3 is preferred by most of companies. Then, the following updated proposal is used as offline proposal for GTW discussion. Companies are encouraged provide the comment directly over the reflector directly.

[Updated Proposal 1]: Enhanced HRAQ process ID indication is supported for DCI 0-2/1-2 and DCI 0-1/1-1 by:

  - Option 3: Extending the HARQ process ID field up to 5 bits

  - FFS: DCI 0-0/1-0

# #Issues-2: HARQ codebook enhancements

For the Initial Proposal-3 in section 7.2, in the 2st round discussion, 20 companies are provided views:

- Up to 7 companies [Spreadtrum, CATT, APT, LG, ZTE, Sony, MediaTek, CMCC, CAICT] are supportive for this proposal.

- W.r.t the Type-1 codebook

  - Option-1 (without enhancement) is supported by 5 companies as [CATT,OPPO,CMCC, Nokia, Intel];

  - Option-2 is supported by [APT, Panasonic]

  - Option-3 is supported by 6 companies [Huawei,Samsung,Thales,ZTE,Sony,vivo];

    - [Ericsson] highlights that the codebook size for Type-1 should be kept as semi-static way.

    - [CMCC] highlights that potential error case will occurs for the Option-3 for Type-1 codebook.

- W.r.t the Type-2 codebook

  - Option-1 (without enhancement) is supported by 3 companies as [OPPO, Apple, Nokia].

  - Option-2 is supported by 14 companies as [Huawei,CATT,APT,Samsung,Panasonic,Thales,ZTE,Sony,CMCC,Ericsson,vivo, CAICT,Intel]. And [Sony, Ericsson, CAICT] prefer to add the details for this solution.

In additional, [Qualcomm] asked for the clarification on Option-1. From moderator perspective, Option-1 (no enhancement) means that the same behavior is same behavior for UE according to current spec is expected, i.e., the UE should still generate ACK/NACK based on the PDSCH decoding result. But it’s contradict to the definition of “disabled” HARQ process.

From moderator perspective, it’s clear that majority views on the Type-2 enhancement is observed, but more discussion is needed for Type-1. Then, the following updated proposal is used as offline consensus. Companies are encouraged provide the comment directly over the reflector directly.

[Updated Proposal 3]: HARQ codebook enhancement is supported as:

- For Type-2 HARQ codebook:

  - Option-2: Reduce codebook size with DAI increment for HARQ process with enabled feedback in the DCI

- For Type-1 HARQ codebook, further discuss is needed with down selection among following options:

  - Option-1: No enhancement;

  - Option-2: Report NACK on disabled process

  - Option-3: Reduce codebook size with criteria

# #Issues-3: Restriction on HARQ feedback disabling

For the Initial Proposal-4 in section 7.3, in the 2st round discussion, 19 companies are provided views:

- Up to 13 companies as (Panasonic,APT,CATT,Spreadtrum,Thales,ZTE,MediaTek,ETRI,CMCC,Ericsson,CAICT,Nokia, Sony) are fine with the proposal.

  - [Nokia] prefers to to re-iterate that configuration of the system is normally left to the gNB.

  - [OPPO,vivo, ETRI, Panasonic, Intel] prefer to the updated wording by adding the “and MAC-CE without UL HARQ feedback is not supported”.

- Up to [Samsung, LG, Huawei] prefer to address this issue in RAN1 and ask for the clarification on the purpose of this LS. [Xiaomi] thinks that this LS is not necessary.

From moderator perspective, this topic has been discussed in RAN1 in two meetings without any conclusion (some companies argue that the MAC CE related issue should be addressed in RAN2). So, sending a LS along with potential candidate discussed in RAN1 to RAN2 will be helpful for cross-checking. And the recommendation from RAN2 is expected to useful to resolve this issue with more comprehensive understanding. Then, the following updated proposal is used for GTW discussion. Companies are encouraged provide the comment over the reflector directly if there is any updates on the views.

[Updated Proposal 4]: A LS to RAN2 with capturing following options is needed to resolve the issue related to MAC CE activation/deactivation command:

- Option 1: UE expects that at least one HARQ process for DL scheduling is configured for the scheduling of MAC-CE.

- Option 2: Up to gNB’s implementation for scheduling

# #Issues-4: Performance enhancements

For the Initial Proposal-5 in section 7.4, in the 2st round discussion, 20 companies are provided views:

- Up to 13 companies as (Samsung, Panasonic,Qualcomm,APT,Spreadtrum,Huawei,Thales,ZTE,Sony,MediaTek,ETRI,CAICT) are fine with the proposal.

  - W.r.t the second sub-bullet (i.e., for MCS), [OPPO, LG, CATT, CMCC, vivo, Nokia] express the strong concerns on the necessity and gain.

- [Xiaomi] prefer to modify the second sub-bullet with more general wording.

- [Ericsson] thinks that it’s not mature to include or exclude any proposal without sufficient justification.

From moderator perspective,

- W.r.t the second sub-bullet, the main intention is to check whether additional MCS table or CQI with new BLER is needed. So, the terminology, “e.g., enhanced UE reporting” is too general and no aligned with intention of majority view in previous discussion;

Meanwhile, in the original proposal, it’s already claimed that Supports on the detailed solution should be justified. So, companies, who have concerns on the 2nd sub-bullet is encouraged to provide the corresponding analysis and results.

- W.r.t the prioritization: simulation results are surely preferred to justify the performance gain of certain solution, but reasonable analysis can still be useful to provide the general guidance for future works. The intention of this proposal is just to provide general views for future works. Otherwise, inefficient discussion will be expected in upcoming meetings.

According to the above analysis, Companies are encouraged to take the following proposal as general guidance for future discussion with more solid results in RAN1#103e. The decision on whether/which enhancement will be supported will be done by RAN1#104e.

[Initial Proposal 5]: Study on following enhancements is prioritized:

- Enhancements on aggregated transmission (including repetition)

- Enhancements on MCS (including CQI report)

- Note-1: Supports on the detailed solution should be justified.

- Note-2: For other solutions, companies are encouraged to provide the results to justify the gain comparing to these two in sub-bullets.

# #Issues-5: PDSCH/PUSCH scheduling restriction

For the Proposal-6 in section 7.5, in the 2st round discussion, 17 companies are provided views. W.r.t the concerns from companies, corresponding clarification is done via the email reflector including:

- W.r.t the terminology "another PDSCH", it is same as current specification (cited as "The UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process...") and refers to the new PDSCH scheduled by new DCI (which can be either new transmission or re-transmission).   This kind of clarification is actually captured by the main bullet in original proposal (i.e., Following principle is supported for the reception of PDSCHs by different scheduling via same HARQ process with disabled feedback), but it's removed since it seems to be redundant for other companies according to the replies.

#==

It seems that there is still strong concerns from [Samsung] on this issue, further updates by adding “by different scheduling” is added.

- From scheduling perspective, another PDSCH with same HARQ process ID can also be the potential re-transmission for same TB, so, I prefer to keep the original wording with PDSCH instead of TB.

And it seems that the following updated version is stable as offline consensus:

[Updated Proposal 6]:

For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH by different scheduling for the given HARQ process until [X] after the end of the reception of the last PDSCH for that HARQ process.

- FFS: value of X.

For the PUSCH related issue, according to two rounds discussion, the views from companies are still diverged and the necessity for corresponding enhancement is not justified. Then, from moderator perspective, proponents are encouraged to have more offline discussions with other companies. And we can come back to this issue in RAN1#104e.

# Conclusion

In this summary, following proposals are made according to the contribution submitted in AI 8.4.3:

# Appendix

| Contribution | Observation/Proposals |
| --- | --- |
| R1-2007571 Huawei | Observation 1: HARQ ID can be implicitly indicated via slot index at the cost of scheduling flexibility. Observation 2: HARQ ID can be implicitly indicated via DMRS at the cost of false alarm ratio loss. Proposal 1: The following mechanisms for implicit indication of HARQ process ID can be considered. - Option 1: Associate HARQ process ID with slot index; - Option 2: Associate HARQ process ID with DMRS sequence; Proposal 2: The Type-1 HARQ-ACK codebook size can be reduced by skipping the HARQ-ACK feedback for disabled HARQ processes. Proposal 3: Transmission parameters shall be configurable and indicated via DCI. Proposal 4: Reinterpreting idle bits in DCI for indicating transmission parameters shall be considered. Proposal 5: UE assistance information reporting in reserved resource can be considered for NTN. |
| R1-2007662 vivo | Observation 1: Considering different target BLER MCS tables for different use cases is benefit to NTN. Observation 2: Consider time interleaved slot aggregation for NTN. - the slot interval depends on the traffic packet size and transmission scheme. Observation 3: Considering different MCS tables and different aggregation factors for different NTN scenarios is benefit to NTN. Proposal 1: Support re-interpreting existing DCI field to indicate the extension of HARQ process number for NTN. Proposal 2: Support different MCS tables and different aggregation factors for different NTN use cases and scenarios. Proposal 3: At least one enabled HARQ process should be configured for NTN. Proposal 4: Support to report HARQ-ACK for disabled DL HARQ processes if enabled HARQ process(es) is within the occasions for type-1 HARQ-ACK codebook. |
| R1-2007856 CATT | - At most 32 HARQ processes can be supported without UE capability report if only single layer transmission is assumed in NTN. - Specify supported HARQ process number for fallback case. - Re-interprete DCI field to indicate 32 HARQ indexes. - Keep at least one HARQ process with feedback if UE specific disabling is configured. - A unified solution of configuration parameter is preferred for with and without HARQ . - Support more than 8 repetitions in slot-aggreation transmission. - No optimization is needed for type 1 HARQ-ACK codebook and no touch on type 3 HARQ-ACK codebook. - Type 2 HARQ-ACK codebook can be optimized for NTN from saving overhead perspective. - Blind retransmission is not supported in NTN case. - Slot aggregation factor can be extended to 16 for very low SINR case. - Support time interleaved slot aggregation to improve transmission reliability. - No need to define a minimum time gap for without HARQ-ACK feedback case if blind retransmission is not supported. - Blind retransmission doesn’t show clear benefits over slot aggregation but with redundant DCI indication. - Define a minimum time gap between two PDSCHs of a HARQ process without HARQ-ACK feedbacks，which is necessary only for the blind retransmission. - There is no need for the enhancement on HARQ feedback. |
| R1-2008012 CMCC | Proposal 1: Support DCI format 0_2 and 1_2 with up to 5 bit HARQ process number field as configured by higher layer parameter. Proposal 2: The following solutions for up to 32 HARQ process ID indication can be further studied - Re-interpretation of existing DCI field - Slot/SFN-based solution with LSB value for HARQ ID determined by time information. Proposal 3: Support different configuration on aggregation factor for HARQ process with/without feedback. Proposal 4: Support larger aggregation factor in NTN. Proposal 5: Support on the maximal HARQ process number is up to UE capability. |
| R1-2008166 Samsung | Proposal 1: Enhanced Type-2 and Type-3 HARQ-ACK codebooks are not supported for NTN. Proposal 2: Enable a gNB to avoid HARQ-ACK information in a Type-1 HARQ-ACK codebook for HARQ processes with disabled HARQ-ACK information by configuring a bitmap that indicates slots where the UE should generate HARQ-ACK information. Proposal 3: When HARQ-ACK information for a HARQ process with disabled HARQ-ACK information is included in a HARQ-ACK codebook, the UE reports - A predetermined HARQ-ACK information value, such as a NACK, for the HARQ process with disabled HARQ-ACK information when the UCI payload size is no more than 11 bits. - HARQ-ACK information based on a reception outcome of a corresponding TB for the HARQ process with disabled HARQ-ACK information when the UCI payload size is more than 11 bits. Proposal 4: A Type-2 HARQ-ACK codebook only includes HARQ-ACK information for HARQ processes with enabled HARQ-ACK information report. - DAI values change only when a TB in a corresponding PDSCH is associated with HARQ process with enabled HARQ-ACK information report. Proposal 5: Support a larger number of repetitions for NTN - The number of repetitions is indicated by the DCI format as in Rel-16. - Extend the “Time domain resource assignment” field to indicate the number of repetitions. Proposal 6: For the maximum number of HARQ processes, consider the following options. - Option 1. gNB broadcasts the maximum TBS to be configured for the cell and UE reports its capability for a number of HARQ processes. - Option 2. UE reports its capability for a number of pairs of {maximum number of HARQ processes, maximum TBS constraint}. Proposal 7: UE assistance information for HARQ should be studied for NTN. |
| R1-2008255 OPPO | Proposal 1: The enabling/disabling of HARQ processes for both DL and UL scheduling via RRC or DCI should be supported. Proposal 2: HARQ-ACK information for disabled DL HARQ processes should be reported at least in Type-1 HARQ-ACK codebook and FFS in Type-2 HARQ-ACK codebook. Proposal 3: Support on the maximal HARQ process number is up to UE capability. Proposal 4: Low DCI overhead methods should be considered if the number of HARQ processes is 32. Proposal 5: Enhancements to PDSCH/PUSCH with disabled HARQ process to achieve a higher reliability should be considered. Proposal 6: PUSCH processing time should be updated in NTN. Proposal 7: PDSCH reception constraint for a given enabled DL HARQ process should be enhanced in NTN. Proposal 8: PDSCH reception constraint for a given disabled DL HARQ process should be considered in NTN. Proposal 9: PUSCH transmission constraint for a given enabled UL HARQ process should be enhanced in NTN. Proposal 10: PUSCH transmission constraint for a given disabled UL HARQ process should be considered in NTN. |
| R1-2008361 Sony | Observation 1: The beam switching is a time-sensitive behavior due to the movement of satellite. Waiting for the HARQ feedback for PDSCH carrying MAC CE for beam switching may miss the favorable time. Observation 2: The redundant feedback of Type-1 / semi-static HARQ codebook would be large based on current HARQ codebook design. Proposal 1: Support slot-based HARQ process ID indication when maximal supported HARQ process number is extended to 32. Proposal 2: Support at least one HARQ process with HARQ feedback enabled. Proposal 3: When the MAC CE for beam switching is carried by PDSCH without HARQ feedback. UE applies the corresponding action with the reference to slots of the end of PDSCH transmission. Proposal 4: At least adjustment for HARQ codebook Type-1 and Type-2 should be considered. - Enhancement on Type-1/ Semi-static HARQ codebook to reduce the redundant feedback should be studied. Following method can be considered: - Separate TDRA table configuration for HARQ process with HARQ-ACK disabling and HARQ-ACK enabling. - DAI is not incremented for a PDCCH which is scheduling a HARQ process with HARQ-ACK disabling. Proposal 5: When the HARQ process of SPS PDSCH is HARQ feedback disabled, UE reports HARQ feedback information for the SPS PDSCH activation. |
| R1-2008412 LG | Proposal 1: The maximum number of HARQ process is up to UE capability. Proposal 2: Consider CCE index based HARQ process id identification for NTN. Proposal 3: FFS on the use case and clear benefit of dynamic HARQ enabling/disabling. Proposal 4. Consider recommended repetition factor, PDSCH decoding results or probability as new CSI contents. Proposal 5: Consider HARQ-ACK codebook enhancement when HARQ feedback is disabled. |
| R1-2008467 Apple | Proposal 1: Supporting the maximal HARQ process number of 32 is up to UE capability. Proposal 2: Consider always applying LBRM for UE operating with 32 HARQ processes. Proposal 3: The HARQ process number field in DCI is remained to be 4 bits, and DCI fields are re-interpreted to indicate up to 32 HARQ process numbers. Proposal 4: Support to have different configurations for HARQ processes with or without HARQ feedback. Proposal 5: Support blind PDSCH and PUSCH retransmissions for NTN. Proposal 6: In type-1 HARQ-ACK codebook construction, UE does not reduce the HARQ-ACK codebook size for HARQ processes with disabled HARQ feedback. Proposal 7: In type-2 HARQ-ACK codebook construction, UE does not expect to receive increased counter DAI for HARQ processes with disabled HARQ feedback. |
| R1-2008810 MTK | Proposal 1: Support of 32 HARQ processes in the device is a UE capability in NR NTN. Observation 1: To improve reliability in case UL HARQ feedback is disabled via configuration, faster ARQ re-transmissions over the RLC layer could be configured – e.g. every 5 ms, 10 ms, 15 ms, 20 ms, ... Observation 2: Reliability of Message 3 in RACH procedure cannot be achieved via RLC ARQ as RLC AM is not possible before contention resolution has completed. Proposal 2: UL HARQ retransmissions is not disabled for Message 3 transmission in RACH procedure. Proposal 3: Whether UE should expect that at least one HARQ process is configured with UL HARQ feedback for MAC CE activation / de-activation is specified or up to network configuration. Proposal 4: The HARQ parameters for HARQ processes with enabling/disabling on HARQ feedback for downlink transmission can be configured differently to ensure adequate reliability – i.e. Block error rate target, MCS table, aggregation factor, Time Domain and Frequency Domain resource allocation, PRB bundling, etc. Proposal 5: The network can configure one HARQ process pool with UL HARQ feedback enabled and one HARQ process pool with UL HARQ feedback disabled. Proposal 6: Whether HARQ process IDs with UL HARQ feedback disabled via RRC can do HARQ soft combining is a UE capability. Observation 3: In NR specifications, the MCS selection, time domain allocation, and frequency resource allocation type 0 and type 1 can be done first as in the specifications. Then, repetitions with values 2, 4, or 8 to increase the reliability of each transmissions as in URLLC can be done based on pdsch-AggregationFactor for DL or repK for UL based on RRC configuration. Observation 4: An SNR gain in the order of 1 dB is required to achieve a BLER target of e-3 compare to BLER target of e-1 in TDL-D channel profile in NR NTN. This represents less than a CQI step for the gNB scheduler. Proposal 7: Higher level of slot aggregation / repetitions than 8 is FFS |
| R1-2008852 ZTE | Proposal 1: Re-interpretation of bits in DCI should be considered as the baseline to support the HARQ process indication with extended maximum HARQ process number. Proposal 2: LS to RAN2 is recommend to capture the identified impacts (along with RAN1’s suggestions) on scheduling due to disabled HARQ feedback. Proposal 3: Enhancement on the Type-1/2 codebook can be considered. Proposal 4: Enlarged aggregation factor and reduced DM-RS density should be supported to improve the performance for NTN. Proposal 5: Following enhancements are not needed to be supported. - Blind transmission: - CQI with new BLER target - UCI including DL decoding Infor/MCS request - Different parameters configuration - MCS table - Time domain resource allocation table - Frequency resource allocation type 0 and type 1 - Block error rate target - Physical resource block (PRB) bundling configuration - PDSCH mapping type A and type B |
| R1-2008881 Nomor Research GmbH, Thales | Observation 1: Considering 15UEs per cell, the DL UE throughput is similar for 16 and 32 HARQ processes if a restriction to schedule less than 4UEs per TTI is applied. Observation 2: Considering 15UEs per cell with a restriction to schedule up to 4UEs per TTI, the 50%-tile DL UE throughput is 7% higher, if up to 32 instead of 16 HARQ processes can be configured per UE. Observation 3: Increasing the number of UEs per cell from 15 to 20, the difference of the DL UE throughput between systems where up to 16 or 32 HARQ processes per UE can be configured disappears. Observation 4: Considering 15UEs per cell with a restriction to schedule up to 3UEs per TTI, the 50%-tile UL UE throughput is 11% higher, if up to 32 instead of 16 HARQ processes per UE are used. Observation 5: Considering 20UEs per cell with a restriction to schedule up to 3UEs per TTI, the 50%-tile UL UE throughput is 9% higher, if up to 32 instead of 16 HARQ processes can be configured per UE. Observation 6: Considering 30UEs per cell the with a restriction to schedule up to 3UEs per TTI, the CDF of the UL UE throughput is similar if up to 32 or 16 HARQ processes can be configured per UE. Observation 7: If the propagation delay decreases, e.g. for a system using a lower orbit, the round trip time decreases and 16 HARQ processes per UE will be sufficient. Observation 8: The main purpose of NTN is to provide coverage everywhere and to support high mobility. In real NTN scenarios, there is no need to schedule a UE in each TTI. Observation 9: Considering a GEO Ka-Band scenario with FR3 and 10 LOS UEs per cell, the TP per UE is on average 30% lower (8.8Mbit/s vs. 12.6Mbit/s) if link adaptation is performed based on the instantaneous channel state. Observation 10: Considering a GEO Ka-Band scenario with FR3 and 10 LOS UEs per cell, the probability of an RLC error rate per UE larger than 3% is 4% if link adaptation is performed based on the instantaneous channel state and BLER offset, while it is 0.6% if link adaptation is performed based on initial channel state measurement and BLER offset. Observation 11: Considering a GEO Ka-Band scenario with FR3 and 10 LOS UEs per cell, it is useful to do an averaging in terms of BLER rather than taking into account the instantaneous channel state due to the expiration of the channel state information upon receiving the CQI at the gNB because of the large transmission delay. Observation 12: Applying a CQI feedback with 1% PHY BLER target performs better in terms of TP than applying a CQI feedback with 10% PHY BLER target and an additional offset. Observation 13: The probability of an RLC error rate per UE larger than 2% is 0.8% if a PHY BLER target of 1% is applied and 0.5% if an averaged SINR in dB and an offset of -4.5dB is used for link adaptation. In the other considered cases of averaged SINR the RLC error rate is significantly larger. Observation 14: Considering a GEO Ka-Band scenario with FR3 and 10 LOS UEs per cell, the mean TP per UE increases from 13.2Mbit/s to 14.0Mbit/s if a PHY BLER target of 2% instead of 1% is applied. Observation 15: Considering a GEO Ka-Band scenario with FR3 and 10 LOS UEs per cell, the 5%-tile of the TP per UE increases from 8.4Mbit/s to 9.2Mbit/s if a PHY BLER target of 2% instead of 1% is applied. Observation 16: Considering a GEO Ka-Band scenario with FR3 and 10 LOS UEs per cell, the mean RLC error rate per UE increases from 1.1% to 2.2% if a PHY BLER target of 2% instead of 1% is applied. Observation 17: Considering a GEO Ka-Band scenario with FR3 and 10 LOS UEs per cell, the 5%-tile of the RLC error rate per UE increases from 0.8% to 1.8% if a PHY BLER target of 2% instead of 1% is applied. Observation 18: The specified 5QI match either packet error rate or delay of a GEO scenario but not both. Proposal 1: The support of 32 HARQ processes is up to UE capability. Proposal 2: Allow to send blind PDSCH (re)transmission of the same packet by MAC scheduling without waiting for the transmission of the HARQ feedback. Proposal 3: Introduce larger CSI-Report periodicity values in TS 38.331 to avoid unnecessary overhead in scenarios with large transmission delay. Proposal 4: Introduce a target BLER for CQI-Reporting to support NTN scenarios with HARQ disabled. Proposal 5: RAN1 to discuss reasonable assumptions for operator defined 5QI requirements to support GEO satellite communication in NR. |
| R1-2008924, Lenovo, Motorola Mobility | Proposal 1: The HARQ process number is tied to SFN/slot index of PDCCH/PUSCH/PDSCH. Proposal 2: Different numbers of HARQ processes is configured based on UE capability. Proposal 3: UE assumes the HARQ feedback disabling where HARQ ID belongs to the RRC configured HARQ process disabling subset. Proposal 4: The multiple transmissions of same TBs in consecutive or interlaced slots can be considered when HARQ is disabled Proposal 5: Repetition transmission number and interlace transmission interval can be indicated in corresponding DCI when HARQ process disabling. Proposal 6: A unified configuration should be considered for each HARQ process with/without feedback except aggregation factor if benefit identified. |
| R1-2008991 Intel | Proposal 1: - Support Rel. 15 Type I HARQ codebook for the case where HARQ feedback is disabled for a subset of HARQ processes - For Type II HARQ codebook, PDSCH with disabled HARQ feedback is not counted for DAI and the corresponding ACK/NACK is not transmitted by the UE Proposal 2: - If 32 parallel HARQ processes are supported for NTN, - HARQ process ID is determined based on DCI indication and slot index of the corresponding transmission - 4 bits are used for HARQ process ID indication in DCI |
| R1-2009017 ETRI | Observation 1 : The worst scenarios for transmission correspond to the cases having both GEO and handheld. - For SC5, the DL geometry SINR range might be from 1.5 dB (5%) to 5 dB (95%). - For other cases (SC4,19,20), the DL geometry SINR range might be from -6 dB (5%) to 1 dB (95%). Observation 2 : Under the worst NTN scenario, S(I)NR might be narrower and lower than TN. - For all cases(SC4,5,19,20), 90 % (between 5% and 95%) of the links are concentrated within 4 dB - For SC5, 95 % of the links are below 5 dB. - For other cases (SC4,19,20), 95 % of the links are below 1 dB. Observation 3 : Within low S(I)NR region, the slot aggregation could enhance BLER and SE simultaneously. Observation 4 : The slot aggregation might have shorter latency than HARQ retransmission. Observation 5 : The slot aggregation has no specification impact. Observation 6 : If aggregation factor is determined properly, the optimal channel adaptation between BLER and SE might be possible. - The transmission parameter should be determined properly. - Too reliable parameter : throughput loss - Proper parameter : optimal adaptation - Too un-reliable parameter : reliability/latency loss (might be unable to communicate) Observation 7 : The appropriate aggregation factor value might be varying depending on the channel condition and the target performance Observation 8 : In NR, if the slot aggregation is used, gNB cannot distinguish between just proper parameter and too reliable parameter. - 0 CRC OK in a bundle (too un-reliable parameter) : NACK - only 1 CRC OK in a bundle (proper parameter) : ACK - multiple(>1) CRC OK in a bundle (too reliable parameter) : ACK Observation 9 : In NR, If the slot aggregation is used, gNB cannot optimally react to some cases. - toward better reliability : possible (reaction for receiving NACK quite consistently) - maintain : possible (reaction for receiving ACKs quite consistently) - toward better throughput : (seems to be )impossible Observation 10 : In NR, there is no feedback mechanism to guide AF into lower value for better throughput - Once the AF value gets larger, it may be impossible to be reduced again Observation 11 : If all the HARQ feedback are disabled, gNB might have no HARQ feedback. Observation 12 : If all the HARQ feedback are disabled, gNB cannot optimally react to all cases - toward better reliability : (seems to be )impossible - maintain : (seems to be )impossible - toward better throughput : (seems to be )impossible Observation 13 : UL feedback via MAC-CE/RRC might be preferred rather than UL feedback via UCI. - specification impact would be minimized - soft combinable retransmission mechanism on PUSCH might be beneficial for compensating in low S(I)NR under NTN Observation 14 : Each parameter (especially IMCS) has its own optimal aggregation factor value. Observation 15 : For optimal adaptation, different aggregation factor should be applied depending on the parameter (especially IMCS) Observation 16 : The optimal aggregation factor might be different, depending on the target performance. Observation 17 : For optimal adaptation, different aggregation factor should be applied depending on the target performance. Observation 18 : In NR, various kinds of transport channels are multiplexed into PDSCH/PUSCH. - Each transport port channel might have its own transmission purpose (target performance). - Transport channel might be distinguishable by checking the RNTI type of PDSCH/PUSCH - PDSCH related RNTI : {P,SI,RA,MSGB,TC,C,MCS-C,CS}-RNTI - PUSCH related RNTI : {TC,C,MCS-C,CS}-RNTI Observation 19 : In NTN, different target performance might be defined by the HARQ feedback availability. Proposal 1 : Consider the slot aggregation (“larger aggregation factor”) as the solution for “#Issue 4: Enhancement on the transmission” in [5]. Proposal 2 : Consider the UCI for scheduling change request as the solution for both “6.4 #Issue 4: Enhancement on the transmission” in [5] and the adaptation feasibility issue described in this paper. - UCI can include information such as - request for increasing/decreasing pdsch-AggregationFactor - request for increasing/decreasing MCS - DL decoding statistics - combinations of the above - MAC-CE/RRC might be also acceptable, instead of UCI. - for minimizing specification impact. - for compensating low S(I)NR in NTN by using soft combinable retransmissions on PUSCH Proposal 3 : Consider “Introducing the separate aggregation factors depending on followings” as the solution for “6.2 #Issue 2: Mechanism for disabling/enabling HARQ feedback” in [5]. - (a group of) MCS index - (a group of) RNTI type (or search space) - PDSCH related RNTI : {P,SI,RA,MSGB,TC,C,MCS-C,CS}-RNTI - PUSCH related RNTI : {TC,C,MCS-C,CS}-RNTI - HARQ feedback availability (enabled/disabled) - (a group of) HARQ process number - combinations of the above - subsets of the above |
| R1-2009034 Xiaomi | Proposal 1: The number of supported HARQ processes is subject to the UE’s capability. Proposal 2: The length of the HARQ process indicator bit-field in the scheduling grant should be kept unchanged. Proposal 3: Dynamic HARQ enabling/disabling is not supported. Proposal 4: Enhancement on the UCI reporting such as the data decoding statistics should be introduced. |
| R1-2009050 Panasonic | Proposal 1: The maximum number of HARQ processes for NTN is 32. Proposal 2: 1 bit is added for HARQ process ID indication in DCI format 0_1, 1_1 and 0_2, 1_2. Proposal 3: Enhancement of PDSCH/PUSCH transmission to improve user throughput without further increasing the number of HARQ processes should be discussed. Proposal 4: Transport block size scaling in case of repetition should be supported to improve user throughput with a limited number of HARQ processes. |
| R1-2009059 Asia Pacific Telecom | Proposal 1 Support on the maximal HARQ process number is up to UE capability. Proposal 2 More than 16 HARQ shall only be supported by VAST and mounted devices. Proposal 3 RAN1 impact on HARQ UL retransmission enabling/disabling shall be discussed. Proposal 4 Timing enhancement on the Type-1 HARQ-ACK codebook shall be needed. Proposal 5 HARQ-ACK disabling shall not change the Type-1 HARQ-ACK codebook size. Proposal 6 Timing enhancement on the Type-2 HARQ-ACK codebook shall be needed. Proposal 7 DAI enhancement on the Type-2 HARQ-ACK codebook shall ensure the error correction capability. |
| R1-2009078 CAICT | Proposal 1: Support up to 32 HARQ processes indication with one additional bit field in the DCI. Proposal 2: Configure two subsets of HARQ processes for enabled HARQ processes and disabled HARQ processes respectively via RRC signaling. To decide the HARQ disable/enable state with HARQ process ID indication in the scheduling DCI. Proposal 3: HARQ-ACK for disabled HARQ is not included in the dynamic HARQ-ACK codebook. Proposal 4: Consider enhancements on DCI formats and corresponding PDCCH detection when DL HARQ process with feedback disabled and enabled are simultaneously supported for one UE. Proposal 5: Enabling/disabling of HARQ feedback for DL SPS/UL CG is configured per configuration. Proposal 6: Provide higher priority order for the HARQ disabled transmission than the priority order for HARQ enabled transmission. |
| R1-2009093 Ericsson | Observation 1 It is not necessary to schedule 32 HARQ processes using fallback DCI 0_0/1_0. Observation 2 Adding an additional configurable bit in DCI 0_1/1_1 or 0_2/2_2 to support 32 HARQ processes minimizes the impacts on specification and scheduling. Observation 3 Time window-based method (and its variations) for supporting 32 HARQ processes is not aligned with asynchronous NR HARQ design principle, introduces unnecessary scheduling restriction, and thus is against the RAN1 agreement of “minimizing the impacts on specification and scheduling.” Observation 4 The intention of disabling HARQ feedback for a downlink transmission is not to send HARQ feedback for the downlink transmission. Observation 5 If the network schedules a PDSCH on a HARQ process with feedback disabled, it is clear that the network is not interested in receiving the feedback. Observation 6 Sending feedback from UE for a HARQ process with disabled feedback leads to waste of radio resource and UE power consumption, as well as increased interference. Observation 7 The uplink HARQ process reuse rule that “the UE is not expected to be scheduled to transmit another PUSCH for a given HARQ process until after the end of the expected transmission of the last PUSCH for that HARQ process” does not impose restriction that would lead to throughput reduction in the presence of large RTT, regardless of whether the DL and UL frame timings are aligned or not at the gNB/UE. Observation 8 The downlink HARQ process reuse rule that “the UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process” is not applicable to a HARQ process with feedback disabled. Proposal 1 Whether 32 HARQ processes are used or not in the uplink can be configured by RRC. Proposal 2 Whether 32 HARQ processes are used or not in the downlink can be configured by RRC. Proposal 3 If 32 HARQ processes are configured, an additional bit is present in DCI 0_1/1_1. This additional bit, together with the existing 4 bits in the HARQ process ID field, can be used to indicate 32 HARQ processes. Proposal 4 If 32 HARQ processes are configured, the size of the HARQ process ID field in DCI 0_2/2_2 is 5 bits. Proposal 5 RAN1 to down select one option: Option 1 – UE expects that at least one HARQ process is configured with UL HARQ feedback; Option 2 – UE does not expect a MAC CE activation/deactivation command, which would become effective 3 msec after the UE would transmit the corresponding HARQ-ACK, to be scheduled on a downlink HARQ process with disabled feedback. Proposal 6 RAN1 to discuss what parameters need to be configured differently for HARQ processes with feedback and HARQ processes without feedback. Example parameters include MCS table and aggregation factor. Proposal 7 When HARQ processes are enabled/disabled on a per HARQ process basis, in the case of the NR Type-1 HARQ codebook, the UE inserts NACKs in positions corresponding to PDSCHs associated with feedback disabled HARQ processes. Proposal 8 When HARQ processes are enabled/disabled on a per HARQ process basis, in the case of the NR Type-2 HARQ codebook, the UE ignores counter DAI from a PDCCH that is associated with a feedback disabled HARQ process and counter DAI is not incremented for such a PDCCH. Proposal 9 When HARQ processes are enabled/disabled on a per HARQ process basis, in the case of the NR Type-2 HARQ codebook, the total DAI (if present) indicates the sum of all the scheduled PDCCHs associated with feedback enabled HARQ process. Proposal 10 When HARQ processes are enabled/disabled on a per HARQ process basis, in the case of the NR Type-3 HARQ codebook, the codebook size is dimensioned to include ACK/NACK information only for HARQ processes that are enabled. Proposal 11 Keep the existing uplink HARQ process reuse rule. Proposal 12 For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH for the given HARQ process until after the end of the expected reception of the last PDSCH for that HARQ process. |
| R1-2009118 IDC | Observation-1: lowering target BLER for PDSCH when HARQ feedback is disabled is beneficial in terms of resource utilization and latency as it can reduce the number of retransmissions in higher layer Observation-2: use of a CQI table with a lower BLER target (e.g., 1%) could provide a better link adaptation with lower PDSCH BLER target when HARQ feedback is disabled Observation-3: increasing the maximum HARQ process number could provide benefits in terms of higher throughput, low latency, better resource utilization over HARQ feedback disabling scheme when channel/interference changes dynamically Observation-4: increasing the maximum HARQ process number is desirable only for some type of UEs (e.g., VSAT) Proposal-1: a CQI table with a new target BLER (e.g., 1%) is considered when HARQ feedback is disabled Proposal-2: blind retransmission is considered when HARQ feedback is disabled Proposal-3: support maximum 32 HARQ process number as UE optional feature |
| R1- 2009154 Spreadtrum | Proposal 1: Additional scrambling for scheduling grant should be considered for enhancement on HARQ process number. Proposal 2: Blind retransmission and larger aggregation/repetition factor can be considered for enhancing the performance of transmission. |
| R1-2009244 Nokia | Observation 1: According to [4], disabling HARQ may achieve similar throughput compared to increasing the number of HARQ processes. Observation 2: According to [5], using 16 or 32 DL HARQ processes will give similar UE experience in most scenarios Observation 3: Using slot aggregation at low effective code rates will provide efficiency gains for cases with 2 or 4 slots being aggregated. Observation 4: Slot aggregation allows for supporting larger RTT delays while not compromising the HARQ performance. Proposal 1: Limit the maximum of HARQ processes for NTN to 16 to ensure minimum specification changes Proposal 2: For operating NTN systems, mechanisms like HARQ process disabling and slot aggregation are sufficient means for addressing the HARQ stalling problem. |
| R1-2009264 QC | Proposal 1: For NTN, UE reports the capability on the number of HARQ processes. Proposal 2: For NTN, more than 16 HARQ processes can be configured. Proposal 3: For NTN, support slot number based HARQ process identification when more than 16 HARQ processes are configured to a UE. Proposal 4: Define a minimum time gap between two PDSCHs of a HARQ process without feedbacks - Different numerologies may have different time gaps. - FFS to introduce virtual k1. Proposal 5: Consider new CQI BLER targets for HARQ processes without feedbacks. Proposal 6: Support a new UCI feedback for reporting DL transmission disruption and/or requesting DL scheduling changes when HARQ feedback is disabled. - To study the new UCI format and associated resource allocation. Proposal 7: For DL HARQ processes with HARQ feedback disabled, initial transmissions shall use RV 0 and retransmissions shall not use RV 0. Proposal 8: For Type-2 HARQ codebook, only PDSCHs of HARQ processes with feedback enabled are considered in DAIs in DCI. Proposal 9: Support search space configuration per HARQ process type. Proposal 10: Support different transmit parameters and/or configurations per HARQ process or per HARQ process type (retransmissions is enabled/disabled), including - Power control - MCS table - UCI multiplexing parameters - FFS other parameters Proposal 11: For NTN, UE may receive a DCI scheduling a PUSCH of a given HARQ process before the end of the transmission of another PUSCH of that HARQ process. Proposal 12: Define a minimum time gap between two PUSCHs of a HARQ process. |