3GPP TSG RAN WG1 #103-e																																R1-2009812

e-Meeting, October 26th – November 13th, 2020

Source:	Moderator (Qualcomm, vivo)

Title:	FL Summary of RAN1 103-e agreements and email discussions on Rel-17 SI on XR evaluations for NR

Agenda Item:	8.14.1

Document for:	Information

# Introduction

For SI on XR evaluations for NR [1], RAN1 #103-e conducted email discussions as follows.

[103-e-NR-XR-02] Email discussion/approval for applications, traffic model and evaluation methodology – Eddy (Qualcomm) and Xiaohang (vivo)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

This contribution presents the outcome of RAN1 #103-e as well as a summary of the above email discussions. In Section 2, we present the agreements of RAN1 #103-e.  The next sections provide summary of the email discussions.

# Agreements of RAN1 #103-e

This section captures the agreements of RAN1 #103-e.

Agreement 1. XR applications

RAN1 confirms that diverse applications of VR1/2, AR1/2, CG are of interest for study. Potential prioritization/down selection of these applications for evaluation is to be discussed after detailed traffic models and relevant evaluation assumptions are stable.

- FFS: other applications, e.g., XR conferencing

Agreement 2. Traffic model

Traffic model for DL and UL should reflect various aspects, e.g., various bit rates, variable frame/packet (definition of frame/packet to be clarified with traffic model as necessary) size, and periodicity (how to model jitter is FFS).  RAN1 will strive to conclude on detailed traffic models in the next RAN1 meeting (104-e) where SA4 outcome on traffic model is expected to be available.

- Statistical model is preferred.

- It is preferred traffic model for both UL and DL have a certain degree of variability so thatand the total number of traffic models can be reduced.

- Note: Taking into account the fact that the decision on traffic models may hold many other crucial decisions, discussion on traffic model in the next RAN1 meeting is prioritized from the beginning.

Agreement 3: Adopt the following deployment for XR/CG evaluations

- Indoor hotspot: FR1 and FR2

  - Detailed definition of Indoor hotspot refers to TR 38.913.

  - Channel model: InH. Detailed definition of InH refers to TR 38.901.

- Dense urban: FR1 and FR2

  - Detailed deployment refers to TR 38.913, where single layer with Marco layer is assumed.

  - Channel model: UMi. Detailed definition of UMi refers to TR 38.901.

FFS: Whether to prioritize FR1 for evaluation.

Note 1: When selecting the deployment and evaluation assumptions for XR/CG evaluations, it is up to company to evaluate FR1 or FR2 or both for the frequency range.

Note 2: It does not mean that all applications are evaluated for all the deployment scenarios.

Agreement 4: Urban Macro can be reported for XR/CG evaluations only for FR1.

- FFS: whether Uma is optional or not

- Following parameters can be assumed.

| Parameter | Proposed value |
| --- | --- |
|  | Urban Macro (FR1) |
| Layout | 21cells with wraparound ISD = 500 m |
| BS Tx power | FR1: 49 dBm/20 MHz |

Agreement 5: It is to be further discussed how to prioritize the combinations of deployment scenarios and applications after traffic models for each application are stable.

Agreement 6: System capacity is defined as the maximum number of users per cell with at least X % of UEs being satisfied.

- X=90 (baseline) or 95 (optional)

- Other values of X can also be evaluated optionally

Note: The exact ‘satisfied’ requirements will be discussed separately

FFS: how to calculate the percentage of satisfied users across multiple drops of simulations

Agreement 7: Adopt the simulation assumptions in table 1 as below

Table 1: Simulation assumptions for XR evaluation (Part 1)

| Parameter | Proposed value |  |
| --- | --- | --- |
|  | Indoor hotspot FR1/FR2 | Dense urban FR1/FR2 |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 | 21cells with wraparound ISD: 200m |
| Carrier frequency | FR1: 4 GHz FR2: 30 GHz |  |
| Subcarrier spacing | FR1: 30 kHz FR2: 120 kHz |  |
| BS height | 3m | 25m |
| UE height | hUT=1.5 m |  |
| BS noise figure | FR1: 5 dB FR2: 7 dB |  |
| UE noise figure | FR1: 9 dB FR2: 13 dB |  |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Realistic FFS:Ideal(optional) |  |
| UE speed | 3 km/h |  |
| MCS | Up to 256QAM |  |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi | 3-sector antenna radiation pattern, 8 dBi |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, FR2: UE antenna radiation pattern model 1, 5dBi |  |

Agreement 8: Adopt the following UE distribution for XR/CG evaluation for outdoor scenario

- For outdoor scenario:

  - FR1: 80% indoor, 20% outdoor

  - FR2: 100% outdoor

Other UE distribution can be evaluated optionally.

Agreement 9: Adopt the following TDD configuration for XR/CG evaluation

- FR1:

  - Option 1: DDDSU

  - Option 2: DDDUU

- FR2:

  - Option 1: DDDSU

FFS detailed S slot format

Note: Other TDD configuration or FDD can be optionally evaluated.

Agreement 10:  Adopt the following BS antenna parameters for indoor scenario for XR/CG evaluation

- FR1:

  - 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4)

  - (dH, dV) = (0.5, 0.5)λ

- FR2:

  - Option 2: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (16, 8, 2,1,1;1,1)

  - (dH, dV) = (0.5, 0.5)λ

Other BS antenna parameters can be optionally evaluated

Agreement 11:  For XR/CG evaluation, adopt the following assumptions for downtilt

·         For XR/CG evaluation, adopt the following assumptions for downtilt

- Dense Urban

  - FFS: 6 or 12 degree

- Indoor hotspot

  - 90° (pointing to the ground)

Other downtilt can be optionally evaluated

Agreement 12:  Adopt the simulation assumptions in table 3 as below

Table 3: Simulation assumptions for XR evaluation (Part 3)

| Power control parameter | Companies should report |
| --- | --- |
| Transmission scheme | Companies should report |
| Scheduler | SU/MU-MIMO PF scheduler (company to report SU or MU), other scheduler (e.g., delay aware scheduler) is up to companies report |
| CSI acquisition | Realistic Both CSI feedback and SRS are considered Companies should report • CSI feedback delay, CSI report periodicity, whether using CSI quantization, CSI error model or not, • Assumptions on SRS: periodicity, processing gain, processing delay, etc • and etc. |
| PHY processing delay | Baseline: UE PDSCH processing Capability #1 Optional: UE PDSCH processing Capability #2 Companies should report gNB processing delay, e.g. DL NACK to retransmission delay, UL previous transmission to current transmission delay and etc. |
| PDCCH overhead | Companies should report |
| DMRS overhead | Companies should report |
| Target BLER | Companies should report |
| Max HARQ transmission | Companies should report |

Agreement 13: The following aspects are to be discussed after traffic model is stable.

- For the system capacity definition, how to determine whether a UE is satisfied or not is to be deferred until the exact traffic model along with how to measure E2E user experience is available.  Additional metrics to be collected will be further discussed after traffic model is stable.

- Various options for traffic arrival offset among UEs per cell were proposed by companies, e.g., even offset, random offset, no offset. It will be discussed after traffic model is determined.

Agreement 14: System bandwidth for XR/CG evaluations are as follows.

- For FR1,

  - Baseline: 100 MHz

  - Optional: 20/40 MHz (FFS: 200 MHz)

- FFS FR2

Agreement 15: For outdoor scenarios, the BS antenna parameters are as follows.

- FFS FR1,

  - Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8)

  - Option 2: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2)

  - Option 3: 32TxRUs (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1,4,4)

(dH, dV) = (0.5λ, 0.5λ)

- FR2:

  - 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1)

(dH, dV) = (0.5λ, 0.5λ)

Other configurations can be optionally evaluated.

Agreement 16: UE antenna parameters for XR/CG evaluations are as follows

- FR1:

  - Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ

  - Optional: 4T/4R, 1T/2R, 2T2R

- FFS FR2: down-selection between the next two options. Please indicate if you have preference.

  - Option 1 (Follow Rel-17 evaluation methodology for FeMIMO in R1-2007151)

    - (M, N, P)=(1, 4, 2), 3 panels (left, right, top)

    - (Mp, Np) is up to company. Need to be reported with simulation result.

  - Option 2 (from TR 38.802 – developed in Rel-14)

    - 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, the polarization angles are 0° and 90°

Agreement 17: BS Tx power for XR/CG evaluations are as follows

- For Indoor hotspot:

  - FR1:

    - 24 dBm per 20 MHz

  - FR2:

    - 23 dBm per 80 MHz. EIRP should not exceed 58 dBm

- For Dense urban:

  - FR1:

    - 44 dBm per 20 MHz

  - FR2:

    - 40 dBm per 80 MHz. EIRP should not exceed 73 dBm

For system BW larger than above, Tx power scales up accordingly.

Agreement 18: UE max Tx power for XR/CG evaluations are as follows

- FR1: 23 dBm

- FR2: 23 dBm, maximum EIRP 43 dBm

Agreement 19. Baseline power evaluation methodology

TR38.840 is the baseline methodology potentially with some modifications if necessary.  RAN1 aim to minimize modeling effort.

Agreement 20.

- RAN1 continues to discuss evaluation methodologies for UE power consumption and system capacity.

- RAN1 is to discuss whether/how to study/evaluate mobility and coverage at a later stage, e.g., starting from Q1 2021.

# Summary of round2 email discussion for Capacity Evaluation

Q1 (round #1): For UMi and Dense urban scenarios, whether both of them need to be separately evaluated, or only UMi is to be evaluated for the sake of reducing the number of evaluation scenarios? Companies views Umi - ZTE, FUTUREWEI, LG, DCM, QC, MTK, Xiaomi, Nokia, CATT, FB, Samsung, Apple Dense Urban - MTK, CMCC, E///, Huawei, CATT, AT&T, FB, Samsung, Apple Huawei, Intel: separate propagation channel models (InH, UMi, UMa; see TR 38.901) from deployment scenarios (Dense Urban, Urban Macro; see TR 38.913). Both parts need to be agreed

Summary of Q1 in Round 1 discussion:

According to 38.913, Dense urban is one of the deployment scenarios.

According to 38.901, UMi is one of the channel model including large-scale channel parameters and fast fading parameters

For XR/CG evaluation, dense urban is adopted as the deployment scenario and UMi is adopted as the channel model.

Q2 (round #1): The deployment scenarios for evaluation may be applied to each of XR/CG applications of interest. However, if all the XR/CG applications are considered, there will be too many combinations of deployment scenarios and XR/CG applications, which could lead to numerous simulation work. Therefore, it may be desirable to consider prioritization of combinations of deployment scenarios and XR/CG applications, e.g., Companies views Deployment and applications - Good to have priority - FR1 is prioritized - ZTE, FUTUREWEI, LG, DCM, CMCC, Xiaomi, Huawei, CATT, Intel (for simulation purpose), MTK, Samsung, Apple - FR2 is also prioritized - QC, vivo, AT&T - Agree to reduce the combinations of scenario and application - FUTUREWEI - MTK - Not need - E/// - How to reduce the combinations - FFS - Support the proposal for combinations of scenario and application - FUTUREWEI (FR1) - LG, FR1, without UMa - DCM, FR1 - QC - Other proposal - IDC: also consider FR1, UMa: AR and CG - E/// - CG in FR1: Urban macro and dense urban - AR in FR1: Urban macro and dense urban - Huawei: too early to do prioritization. Traffic model should be discussed first - AT&T: AR should be also evaluated in indoor scenarios - FB: AR for indoor

Summary of Q2 in Round 1 discussion

Some companies want to prioritize FR1 over FR2.

It is also mentioned by companies that FR1 over FR2 only for evaluation purpose.

Some companies want to prioritize both FR1 and FR2

It is also mentioned by companies that the prioritization for the combination between deployment scenarios and applications should be discussed after determining traffic model and the application of interest.

If all the combinations are to be considered for evaluation, it will result in huge simulation work.

Given that the applications of interest haven’t been decided yet and they will be discussed with traffic model, it may be difficult to determine the priority for the combination now. So, it is suggested to further discuss the prioritization after deciding the traffic model for XR/CG applications.

Questions for Round #2 discussion

#### Proposal: Adopt the following deployment for XR/CG evaluations

- Indoor hotspot: FR1 and FR2

  - Detailed definition of Indoor hotspot refers to TR 38.913.

  - Channel model: InH. Detailed definition of InH refers to TR 38.901.

- Dense urban: FR1 and FR2

  - Detailed deployment refers to TR 38.913, where single layer with Marco layer is assumed.

  - Channel model: UMi. Detailed definition of UMi refers to TR 38.901.

- Please share your views on the above Proposal.

| Company | Comment |
| --- | --- |
| AT&T | Agree with the proposal |
| CATT | Agree with moderator’s proposal |
| vivo | Agree with the proposal |
| MTK | Agree with the proposal |
| ZTE,Sanechips | We agree to have this as evaluation for FR1 while the evaluations related to FR2 had better be optional according to majority view in the first round feedback. |
| Sony | Agree with the proposal |
| InterDigital | We support the proposal for Indoor hotspot and Dense urban both for FR1 and FR2 |
| Ericsson | Propose to add a note: “Note that this does not mean that all applications are evaluated for all the deployment scenarios.” |
| QC | We support the proposal. |
| Huawei, HiSilicon | Considering the majority view to prioritize FR1 and to avoid too huge simulation workload, we suggest to remove FR2. FR2 can be optional, and if companies have concerns here, maybe we can add a Note like “Note: FR2 is not precluded, and interested companies can report the evaluation results for FR2”. Q1 in the 1st round was about UMi vs. Dense Urban. Indoor hotspot was not mentioned in Q1. And it seems there was no discussion on choosing Indoor hotspot as an evaluation scenario during this meeting. We have some concern that whether Indoor hotspot can be agreed without any discussion on it. |
| Nokia, NSB | We agree with the FL proposal of adopting only indoor hotspot and dense urban as deployment scenarios with InH and single Macro layer UMi as respective channel models. |
| FUTUREWEI | Suggest focussing the evaluation on FR1. Note that without modelling beam management aspects we do not see much extra value evaluating also FR2 which will take much longer time for simulation due to its much wider bandwidth, large number of antennas, and more users in the system. Also suggest to further reduce the combination of scenarios and applications. Make dense urban as mandatory while indoor as optional can also help to reduce the simulation load. |
| Facebook | Agree with the proposal |
| DOCOMO | Agree with the proposal. |

##### Summary of question 1

- Agree with the proposal

  - AT&T, CATT, vivo, MTK, Sony, InterDigital, QC, Nokia, Facebook, DOCOMO

- FR1 is mandatory. FR2 is optional

  - ZTE, Huawei

- Dense Urban is mandatory. Indoor hotspot is optional

  - Huawei, FUTUREWEI

During Round 1, only a few companies addressed Urban Macro. Since there is no sufficient input from companies yet, it may need more discussions on whether and how to adopt UMa for XR/CG evaluation. So the following questions are to be discussed in the round #2 discussion.

#### Question 2: Please share your view on whether Urban Marco needs to be evaluated (only for FR1) in addition to indoor hotspot and Dense urban, where the definition of Urban Marco refers to TR 38.913 and channel model is UMa which refers to TR 38.901.

| Company |  | Comment |  |
| --- | --- | --- | --- |
| AT&T |  | We believe Urban Macro/UMa should be evaluated at least for FR1 |  |
| CATT |  | Urban Macro could be optional |  |
| vivo |  | Urban Macro could be optional |  |
| MTK |  | Urban Macro could be optional |  |
| ZTE,Sanechips | We prefer not to have this additional scenario. |  |
| Sony |  | Urban Macro could be optional |  |
| InterDigital |  | We think UMa should be included in XR/CG evaluation in order to include some relevant scenarios which might be apparent after traffic model for XR/CG applications is decided. |  |
| Ericsson |  | Urban Macro could be optional |  |
| QC |  | We think UMa could be also evaluated together with other scenarios. It is relevant to low rate AR case which is most relevant use case in near term market. |  |
| Huawei, HiSilicon |  | Support Urban Macro in addition to Dense Urban. Ok with UMa for Urban Macro. Both Dense Urban and Urban Macro scenarios with FR1 are the typical commercial deployment of NR networks. We suggest to perform evaluation and provide valuable studies at least for the already deployed NR networks. As commented under Question 1, it seems there was no discussion on choosing Indoor hotspot as an evaluation scenario during this meeting. We have some concern that whether Indoor hotspot can be agreed without any discussion on it. |  |
| Nokia, NSB |  | In order to maintain the number of simulation scenarios to a reasonable level, we think that Urban Marco deployment scenario with UMa channel model does not need to be evaluated. |  |
| FUTUREWEI |  | Urban Macro is not needed considering the work load. |  |
| Facebook |  | We think urban macro should be evaluated as well. |  |
| DOCOMO |  | Urban Macro could be optional. |  |
| LG |  | We think urban macro should be enough. But urban macro can be considered as optional scenario which companies can choose to evaluate. |  |

##### Summary of question 10

- Urban Macro should be evaluated

  - AT&T, InterDigital, QC, Huawei, Facebook

- Urban Macro is optionally evaluated

  - CATT, vivo, Sony, Ericsson, DOCOMO, LG (?)

- Not consider Urban Macro

  - ZTE, FUTUREWEI, Nokia

#### Proposal: It is to be further discussed how to prioritize the combinations of deployment scenarios and applications after traffic models for each application are decided.

- Please share your views on this.

| Company |  | Comment |  |
| --- | --- | --- | --- |
| AT&T |  | Agree, but we hope that any prioritization does not result in too limited of a study |  |
| CATT |  | OK |  |
| vivo |  | Agree with FL’s suggestion. |  |
| MTK |  | OK |  |
| ZTE,Sanechips | Agreed |  |
| Sony |  | Support the proposal |  |
| InterDigital |  | We agree that the discussion on prioritization of the combinations of deployment scenarios and application should be held after decision on traffic models by SA4. |  |
| Ericsson |  | Agree |  |
| QC |  | Agreed. |  |
| Huawei, HiSilicon |  | As explained under Question 1, we suggest to prioritize FR1. Other combinations can be further discussed after the traffic models for each application are decided. |  |
| Nokia, NSB |  | We agree with the proposal. We think that the decision on the combinations of deployment scenarios and applications should be deferred after the definition of the appropriate traffic models for the applications. For instance, we think that CG and VR applications are more reasonable for indoor deployment (Indoor Hotspot), while AR applications better fit outdoor deployment (Dense Urban). |  |
| FUTUREWEI |  | Agree. Also see our answers to Question 1. |  |
| Facebook |  | Agree |  |
| DOCOMO |  | Agree. |  |
| LG |  | Agree |  |

##### Summary of question 2

- Agree to further discuss

  - AT&T, CATT, vivo, MTK, ZTE, Sony, InterDigital, Ericsson, QC, Huawei, Nokia, FUTUREWEI, Facebook, DOCOMO, LG

## Evaluation methodology and assumptions for capacity

### System capacity definition

Q3 (round #1): System capacity is defined as the maximum number of users per cell with at least X % of UEs being satisfied (i.e., meeting a set of requirements). The exact requirements will be defined separately. Companies views Agree - FUTUREWEI, LG, ZTE, DCM(?), QC, E///, CMCC, Xiaomi, vivo, Huawei, CATT, AT&T, Intel (?), FB, Samsung, Apple Other proposal: - MTK: choose 3 user numbers, say 10, 15, 20, and defined the capacity to be the averaged satisfied user number. Further discuss exact requirement - ZTE: under a given {PDB,PER} requirement in which X% is satisfied - IDC: to include different traffic requirements (e.g. DL/UL throughput, RTT latency, max UL/DL PDB, reliability) for different applications (e.g. VR, AR and CG) - DCM: consider eMBB+URLLC multiplexing case - Nokia: UE satisfaction should be defined in terms of network-level requirements like PER, PDB and data-rate. Different set of requirements can be defined for UL and DL according to the traffic models and applications.

Summary of Q3 in Round 1 discussion :

Most of companies agree with the high-level system capacity definition. Details on the requirements UEs to be satisfied should be discussed.

Q4 (round #1): For the system capacity definition in Q3, the X value needs to be determined, e.g., X=90. In addition, it may be useful to collect results (i.e., # UEs per cell being satisfied or meeting the requirements) for multiple values of X, e.g., X = 70, 80, 90, 95 to see the trend of # UEs per cell meeting the requirements as the number of UEs per cell increases. Companies views X=90 - ZTE, FUTUREWEI, DCM, LG, IDC, QC, E///, CMCC, XIAOMI, vivo, Nokia, AT&T, FB, Samsung X=95 - ZTE, DCM, IDC, E///, MTK, CMCC, Xiaomi, Nokia, CATT, X=70, optionally - QC proposed to collect additional data for 70% X=80 - QC proposed to collect additional data for 80% FFS later: Intel, Apple

Questions for Round #2 discussion

#### Proposal: System capacity is defined as the maximum number of users per cell with at least X % of UEs being satisfied.

- Companies report results for both X=90 and 95

- Note: The exact ‘satisfied’ requirements will be discussed separately (e.g., meeting a certain set of requirements)

- Please share your views on this proposal.

| Company |  | Comment |  |
| --- | --- | --- | --- |
| AT&T |  | Agree, but no concern if companies optionally provide other values for X |  |
| CATT |  | It depends on traffic model. The number of UEs is different in some traffic model (e.g., FTP-1). |  |
| vivo |  | Agree with the proposal |  |
| MTK |  | A smaller value of X would increase the accommodated maximum UE number and also increase the simulation effort (large number of UE each cell large simulation effort). We suggest to only keep one value with X = 95. Given the multiple parameters to evaluate, if we keep more than one value for each parameter, the simulation effort grows exponentially. Hence, we should only keep multiple values when companies really see the need. |  |
| ZTE,Sanechips | Agreed |  |
| Sony |  | Agree with the proposal |  |
| InterDigital |  | We agree with the definition and the proposal to report results for X=90 and 95 |  |
| Ericsson |  | Agree |  |
| QC |  | Support X=90 and 95. |  |
| Huawei, HiSilicon |  | We are generally fine with this proposal. Since many issues are unclear, such as the application, traffic model, requirement, etc., we think it’s somehow premature to decide what proper values are for X. So we suggest X=90 and 95 can be starting point, and other values of X are not precluded. Since the exact ‘satisfied’ requirement is not clear yet and will be discussed separately, we suggest to remove the example to have a more general description at this stage. In summary, we suggest the following changes: #### Proposal: System capacity is defined as the maximum number of users per cell with at least X % of UEs being satisfied. - Companies are encouraged to report results for both X=90 and 95, other values for X are not precluded - Note: The exact ‘satisfied’ requirements will be discussed separately (e.g., meeting a certain set of requirements) |  |
| Nokia, NSB |  | We agree with the FL proposal. |  |
| FUTUREWEI |  | Suggest to only use one value of X, 90 or 95, as mandatory. |  |
| Facebook |  | We are fine with the proposal |  |
| DOCOMO |  | Agree with the proposal but would prefer to also consider eMBB+URLLC coexistence case as an optional. Definition in TR38.824 can be reused. |  |
| LG |  | It may depend on the application and traffic model. Therefore it can be decided at later stage. |  |

##### Summary of question 3

- Agree

  - AT&T, vivo, ZTE, Sony, InterDigital, Ericsson, QC, Huawei, Nokia, Facebook, DOCOMO

    - Huawei: X=90 and 95 can be starting point, and other values of X are not precluded

- Only one X value as mandatory

  - MTK (X = 95), FUTRUEWEI (X = 90 or 95)

- Other comment

  - CATT: X depends on traffic model.   The number of UEs is different in some traffic model (e.g., FTP-1)

  - DOCOMO: also consider eMBB+URLLC coexistence case as an optional

  - LG: decided later

Q5 (round #1): For the system capacity definition, how to determine whether a UE is satisfied or not is to be deferred until the exact traffic model along with how to measure E2E user experience is available. Companies views Most of companies agree to defer the discussion Some high level principle can be given as example ZTE: {PDB,PER} requirement IDC: traffic parameters (e.g. PDB, PER) for VR/CG QC: discuss together with traffic model E///: may need to simulation load Huawei: in principle, RAN1 needs to identify a KPI that can reflect the user experience in XR and CG services.

Summary of Q5 in Round 1 discussion

The requirements UE satisfied are related to the traffic model, use case and the user experience.

It is good to discuss the requirements when traffic model is determined.

Some companies proposed some high-level principles can be given as example

Q6 (round #1): On the XR/CG evaluation, other performance metrics (in addition to # of UEs per cell being satisfied) can be reported, e.g., Companies views - PER (file dropping rate) - vivo - UPT - ZTE, MTK, vivo, AT&T - File transfer delay - IDC (RTT delay for CG), Xiaomi (RTT delay), vivo, AT&T - RU - ZTE, MTK, CMCC, vivo, AT&T - Spectrum efficiency - Others - QC: as another UL metric for XR, we could also measure pose related metric such as age of pose (AOP). An AOP is defined as time duration X-Y, where - X is the time a frame Z is generated at XR server - Y is the time that a pose is generated at XR device which is used to render the frame Z - Whether and how to report AOP and/or user interaction delay for CG can be further discussed together with traffic model. Discuss later after traffic model and how to measure user experience - FUTUREWEI, LG, Nokia, CATT(?), FB, Samsung Companies can report any metrics - LG, E/// QC: how to report those metrics w/ capacity result should be further discussed, e.g., averaged over entire UEs or multiple data points (e.g., 10%, 50%, 90%) in CDF of per UE metrics.

#### Observation 1: For the system capacity definition, how to determine whether a UE is satisfied or not is related to the exact traffic model along with how to measure E2E user experience.

### Traffic arrival offset

Q7 (round #1): Whether and how to evaluate XR capacity under various assumptions on traffic arrival offset among UEs (e.g., random offsets, uniform offsets)?Companies views Even offset among UEs per cell - ZTE, IDC, QC Random offset - IDC, QC, E///, MTK, CMCC, Xiaomi, vivo, Samsung, Apple No offset - QC Discuss later after traffic model and metrics are determined - LG, FUTUREWEI, Huawei, Nokia, AT&T, Intel, FB

Summary of Q7 in Round 1 discussion :

It seems the traffic arrival offset assumptions are related to traffic model. So it can be further discuss how to model it when traffic model is decided.

#### Observation 2: Various options for traffic arrival offset among UEs per cell may be considered (e.g., even offset, random offset, no offset).  It may be further discussed after traffic model is determined.

### Evaluation assumptions

Q8 (round #1). Please share your comments on the proposal 1. Proposal 1: Adopt the simulation assumptions in Table 1 for XR evaluation Companies views Majority companies agree the table 1 in general. Other comments FUTUREWEI: target BLER should be the initial Tx target BLER LG: no need for target BLER. Can be discussed later. QC: For FR2, EIRP < 31 dBm is deemed practical and preferred. UE EIRP of 43 dBm may lead to overly optimistic evaluation results E///: why not use 4GHz and 30GHz as in 38.802? - Max number of HARQ retransmissions does not need to be specified. MTK: we suggest to consider 100MHz as baseline for FR1/FR2 and companies can report CA settings or linear scaling the throughput if companies deem a larger BW result is preferred. For FR1 CA, we prefer 2CC CA for FR1 as follows: Total BW = 2 CCs x 100 MHz - CC1 = Lower band (licensed): 3.5 GHz (DDDSU DDSUU) - CC2 = Higher band1 (licensed): 4.9 GHz (SUUDD) or Higher band2 (un-licensed): 5GHz (DDDDD) We also think max number of HARQ retransmissions does not need to be specified. CMCC: 4 or 4.9GHz can be used as carrier frequency for FR1. Huawei: defer the discussion CATT: 4 GHz is widely used for evaluation in NR Nokia: We propose 40 MHz for FR1 and 80 MHz for FR2 as simulation bandwidth. The value of 80 MHz for FR2 has been discussed and agreed in another SI (R1-2007151) and could serve as a base for the discussion in this study. For FR2, we propose up to 256QAM for DL and up to 64QAM for UL. Intel: similar concept was adopted for IMT-2020 eval as well. However, this would become more clear once we have a picture of traffic model and PER/PDB requirements.

Summary of Q8 in Round 1 discussion

Most of the simulation assumptions in Table 1 are based on TR 38.802. They are also for some other 3GPP studies and evaluations. These assumptions in Table 1 can be also used for XR/CG evaluations.

For carrier frequency, as in TR 38.802, 4GHz FR1 and 30GHz for FR2 are adopted.

For bandwidth, most companies agree with 100 MHz for FR1 and 400 MHz for FR2. So it is proposed to adopt the bandwidth assumptions in Table 1 for XR/CG evaluation.

For target BLER, it may be dependent on the requirements and traffic model. So it can be reported by companies.

For max number of HARQ transmissions, it may be dependent on the requirements and traffic model. So it can be reported by companies.

Questions for Round #2 discussion

#### Proposal: Adopt the updated table 1 as below

- Update carrier frequency to 4 GHz FR1 and 30 GHz for FR2

- It is up to companies report for target initial BLER and max number of HARQ transmissions

Table 1: Simulation assumptions for XR evaluation (Part 1) (updated)

| Parameter | Proposed value |  |
| --- | --- | --- |
|  | Indoor hotspot FR1/FR2 | Dense urban FR1/FR2 |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 | 21cells with wraparound ISD: 200m |
| Carrier frequency | FR1: 4 GHz FR2: 30 GHz |  |
| System bandwidth | FR1: 100 MHz FR2: 400 MHz |  |
| Subcarrier spacing | FR1: 30 kHz FR2: 120 kHz |  |
| BS height | 3m | 25m |
| UE height | hUT=1.5 m |  |
| UE power | FR1: 23 dBm FR2: Maximum EIRP 43 dBm |  |
| BS noise figure | FR1: 5 dB FR2: 7 dB |  |
| UE noise figure | FR1: 9 dB FR2: 13 dB |  |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Realistic |  |
| UE speed | 3 km/h |  |
| MCS | Up to 256QAM |  |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi | 3-sector antenna radiation pattern, 8 dBi |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, FR2: UE antenna radiation pattern model 1, 5dBi |  |

- Please share your views on this Proposal.

| Company |  | Comment |  |
| --- | --- | --- | --- |
| AT&T |  | BS height should be 10m for Dense Urban? |  |
| CATT |  | Target BLER should be part of performance target for each XR service |  |
| vivo |  | Agree with the proposal |  |
| MTK |  | We can not accept FR2 system bandwidth = 400MHz. Current UE seldom supports one carrier > 100MHz and SLS simulation for a > 100MHz carrier would be very time consuming. We can assume 100MHz for both FR1/FR2 as in 38.840. For UE power, we prefer FR1: 23 dBm FR2: 37 dBm. |  |
| ZTE,Sanechips | Prefer to add a note regarding the prioritization of FR1 over FR2 simulations. Prefer to set the system BW as 40MHz or 100MHz. |  |
| InterDigital |  | We agree with the stated assumptions in the updated table 1. |  |
| Ericsson |  | OK with proposal |  |
| QC |  | Support the Proposal. |  |
| Huawei, HiSilicon |  | As commented previously, we suggest to prioritize FR1 over FR2. For channel estimation, suggest “ideal” for simplicity. “Realistic” can be up to companies report. |  |
| Nokia, NSB |  | We think that 100 MHz (FR1) + 400 MHz (FR2) may be too large. We prefer to have 40MHz for FR1 and 100MHz for FR2. |  |
| FUTUREWEI |  | Suggest to prioritize FR1 for evaluation purpose as we elaborated in answer to Q1. |  |
| Facebook |  | We are fine with the proposal. |  |
| DOCOMO |  | As commented in 1st round, it is suggested to prioritize FR1 over FR2. |  |
| Apple |  | The channel bandwidth for FR1 is too large for use cases, 20 MHz for FR1 can be considered also. FR1 should be prioritized to avoid too large a scope, e.g. link robustness issues, etc. Simulation time should be considered also in choosing those parameters. |  |
| LG |  | We are fine with the most of the proposal. More discussion may be needed regarding prioritization between FR1 and FR2. |  |

##### Summary of question 4

- Agree the assumptions in the Table 1

  - Vivo, IDC, Ericsson, Facebook, LG, QC

- Comments

  - FR1 is prioritized over FR2

    - ZTE, Huawei, DCM, Apple, FUTUREWEI

  - System bandwidth

    - ZTE: Prefer to set the system BW as 40MHz or 100MHz

    - Nokia: prefer to have 40MHz for FR1 and 100MHz for FR2

    - Apple: 20 MHz for FR1

  - Channel estimation

    - Huawei: suggest “ideal” for simplicity. “Realistic” can be up to companies report.

  - Target BLER should be part of performance target

    - CATT

  - BS height = 10 m for Dense Urban

    - AT&T

#### Proposal: If Urban Macro is adopted to evaluate, the following parameters can be assumed.

| Parameter | Proposed value |
| --- | --- |
|  | Urban Macro (FR1/FR2) |
| Layout | 21cells with wraparound ISD = 500 m |
| BS Tx power | FR1: 49 dBm/20 MHz FR2: 43 dBm/80 MHz |

- Please share your views on this Proposal.  Also please discuss other Uma related parameters if any.

| Company |  | Comment |  |
| --- | --- | --- | --- |
| AT&T |  | FR2 Tx Pwr/BW should be scaled to 400MHz |  |
| CATT |  | Optional |  |
| vivo |  | Agree if Urban Macro is supported for evaluation |  |
| MTK |  | Optional, no strong view |  |
| ZTE, Sanechips | We prefer not the have Uma scenario as evaluation thus this proposal is not needed. |  |
| Sony |  | Optional, no strong view. |  |
| InterDigital |  | We support the stated parameter assumptions for Urban Macro |  |
| Ericsson |  | Optional |  |
| QC |  | Support the Proposal with following modification. FR2 requires EIRP limit of 73dBm. |  |
| Huawei, HiSilicon |  | Agree with FR1 part. |  |
| Nokia, NSB |  | We think Urban Macro does not need to be evaluated. We think that indoor hotspot and dense urban would provide enough insights for this study. |  |
| FUTUREWEI |  | We do not see the need to include Urban Macro case. |  |
| DOCOMO |  | Optional |  |
| Apple |  | We support the inclusion of Urban Macro, whether Uma is optional or dense urban is optional can be decided after discussion on use cases. |  |

##### Summary of question 5

- Support if Uma is used

  - CATT, vivo, MTK, Sony, IDC, Ericsson, QC, Apple, DCM

  - FR 1 only

    - Huawei

  - BS Tx power

    - AT&T: Tx Pwr/BW should be scaled to 400MHz

    - QC: FR2, EIRP limit of 73dBm

- No support for Uma

  - ZTE, Nokia, FUTRUEWEI

Q9 (round #1). Please share your comments on the proposal 2. Proposal 2: Regarding the UE distribution for outdoor scenario, down-select from the following options for XR evaluation. - For outdoor scenario: - FR1: - Option 1: 80% indoor, 20% outdoor - Option 2: 20% indoor, 80% outdoor - Option 3: 100% outdoor - FR2: - 100% outdoor Companies views Majority of companies agree the option 1 for FR1 Other comments Nokia: he decision on the UE distribution should be deferred after the agreement on the prioritization of XR applications and deployments discussed on Q2. If only AR applications are evaluated in the outdoor deployments, we propose to consider the following options for UE distribution: - FR1: 100% outdoor (Option 3) FR2: 100% outdoor

Majority of companies agree with the option 1 for FR1 for outdoor scenario.

Proposals for Round #2 discussion

#### Proposal: Adopt the following UE distribution for XR/CG evaluation for outdoor scenario

- For outdoor scenario:

  - FR1:

    - Option 1: 80% indoor, 20% outdoor

  - FR2:

    - 100% outdoor

- Please share your views on this Proposal.

| Company | Comment |  |
| --- | --- | --- |
| AT&T | Agree |  |
| CATT | OK |  |
| vivo | Agree |  |
| MTK | For outdoor scenario in FR1, wouldn't 100% outdoor make more sense? |  |
| ZTE, Sanechips | Agreed for FR1. Prefer to de-prioritize FR2 evaluations. |
| Sony | Support |  |
| InterDigital | We agree with the proposal |  |
| Ericsson | OK |  |
| QC | Support the proposal. |  |
| Huawei, HiSilicon | Agree with FR1 part. |  |
| Nokia, NSB | Agree for FR2. For FR1 we prefer 100% outdoor. The commonality between FR1 and FR2 here may also facilitate the direct comparison of FR1 and FR2 results. |  |
| FUTUREWEI | Ok for FR1 part. |  |
| Facebook | Fine with the proposal |  |
| DOCOMO | Agree |  |
| Apple | 20% indoor, 80 % outdoor can be also considered. In general, it is better to wait for finalizing use cases. 100% outdoor as suggested by Nokia is also fine. |  |
| LG | Agree |  |

##### Summary of question 6

- For outdoor scenario:

  - FR1: Option 1: 80% indoor, 20% outdoor

  - FR2: 100% outdoor

- Agree:

  - AT&T, CATT, vivo, ZTE (FR1 only), Sony, IDC, Ericson, QC, Huawei (FR1 only), Nokia (FR1 part), FUTUREWEI (FR1 only), Facebook, DCM, LG

- Comments:

  - 100% outdoor for FR1

    - MTK, Nokia, Apple

  - 20% indoor, 80% outdoor

    - Apple

Q10 (round #1). Please share your comments on the proposal 3. Proposal 3: Regarding the frame structure, down-select from the following options of FR1 and FR2 for XR evaluation. Companies views - FR1: - Option1: DDDSU - ZTE, FUTUREWEI, DCM, IDC, E///, QC, CMCC, Xiaomi, vivo, Huawei, Samsung, Apple - Option2: DSUUD - IDC, vivo - Option3: DDDSUDDSUU - DCM (2nd), IDC - Option4: SUUDD - Option5: DDDUU - CMCC, QC, Samsung, Apple - Option6: DU - CMCC - Option7: FDD - Nokia - FR2: - Option 1: DDDSU - DCM, E///, QC, MTK, vivo, Nokia - Option 2: DSUUD Note: S is 10:2:2 Other comments: LG: no need to decide in this meeting Nokia: a decision on TDD frame structure should be deferred after decisions (i) on the traffic models for XR applications and (ii) on the PHY processing delay. MTK: For FR1, we suggest 2CC CA for FR1 as follows: Total BW = 2 CCs x 100 MHz; - CC1 = Lower band (licensed): 3.5 GHz (DDDSU DDSUU, Option 3) CC2 = Higher band1 (licensed): 4.9 GHz (SUUDD, Option 4) or Higher band2 (un-licensed): 5GHz (DDDDD)

For FR1, Option 1 is supported by majority companies including operators. So option 1 is suggested for FR1.

For FR2, Option 1 is supported by majority companies including operators. So option 1 is suggested for FR2.

Other TDD configurations or FDD can also be evaluated by companies.

#### Proposal: Adopt the following TDD configuration for XR/CG evaluation

- FR1:

  - Option1: DDDSU

- FR2:

  - Option 1: DDDSU

Other TDD configuration or FDD can be optionally evaluated.

- Please share your views on this Proposal.

| Company |  | Comment |  |
| --- | --- | --- | --- |
| AT&T |  | Agree |  |
| CATT |  | OK |  |
| vivo |  | Agree |  |
| MTK |  | Fine |  |
| ZTE,Sanechips | Agreed for FR1 . Prefer to de-prioritize FR2 evaluations. |  |
| Sony |  | Support |  |
| InterDigital |  | We support the proposal |  |
| Ericsson |  | OK |  |
| QC |  | Support the proposal. |  |
| Huawei, HiSilicon |  | Agree with FR1 part. |  |
| Nokia, NSB |  | In general, we are fine with the proposal. but would like to extend it a bit. but would like to extend it a bit. We propose not to close the door for other options at later stages. For instance, the current proposal may limit the analysis of the scenarios characterized by heavy UL traffic. In such a case (if we want to simulate high data rate in uplink), 'DU' may be a better frame structure for this case. The choice of the suitable frame structure may also depend on the PHY processing delay. Option1: DDDSU, but do not preclude other TDD configurations |  |
| FUTUREWEI |  | Ok for FR1 part. |  |
| Facebook |  | We agree with option 1. However, in first round, we also indicate the support of Option 5 (DDDUU). We think it’s important to include the scenario with more uplink traffic some XR applications. |  |
| DOCOMO |  | Agree |  |
| Apple |  | Agree with Facebook on Option 5 (DDDUU), it is important to consider uplink traffic. |  |
| LG |  | Maybe ok. But we are not sure if we should make decision on this detail before knowing the traffic models |  |

##### Summary of question 7

- Agree

  - AT&T, CATT, vivo, MTK, ZTE, Sony, IDC, Ericsson, QC, Huawei, Nokia, FUTUREWEI, Facebook, DCM, LG(maybe)

- Comments:

  - Option 5 (DDDUU)

    - Facebook, Apple

Q11 (round #1). Please share your comments on the proposal 4. Proposal 4: Regarding the BS antennas, further discuss the assumptions and down-select from the following options for XR evaluation. Companies views - For indoor scenario: - FR1: - 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4) - (dH, dV) = (0.5, 0.5)λ - ZTE, vivo, Intel - FR2: - Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) - vivo - Option 2: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (16, 8, 2,1,1;1,1) - LG, IDC, QC, E///, MTK, AT&T - (dH, dV) = (0.5, 0.5)λ - For outdoor scenario: - FR1: - Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (12,8,2,1,1;4,8) - CMCC, vivo, Huawei - Option 2: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,4,2,1,1;8,4) - IDC, Intel - Option 3: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) - FUTUREWEI, DCM, QC, AT&T, Samsung - MTK (modified to (8,8,2,1,1;2,8)) - Option 4: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (16,8,2,1,1;4,8) - CATT - Option 5: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,1,1,2;4,4) - IDC - Option 6: TxRU, (M, N, P, Mg, Ng; Mp, Np) = (2, 8, 2, 1, 1;2,8) - IDC, E/// (32 TxRU) (dH, dV) = (0.5λ, 0.8λ) - FR2: - Option 1: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1) - IDC, E///, MTK, vivo, AT&T - Option 2: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (32,8,2,1,1;1,1) - IDC, QC (dH, dV) = (0.5λ, 0.5λ) Other proposal Nokia: We think that for the evaluation in FR1 we should use a relatively small antenna array panel and a larger one for FR2. Furthermore, for indoor scenario (InH) we could further down-select the possible options, since UEs are expected to have good channel conditions. - Indoor scenario: - FR1: Option 1) 4 Tx/4 Rx, (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 2), (dH, dV) = (0.5, 0.5)λ, Option 2) 8 Tx/8 Rx, (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 4), (dH, dV) = (0.5, 0.5)λ - FR2: Option1) 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8), (dH, dV) = (0.5, 0.5)λ – Option 2) 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4), (dH, dV) = (0.5, 0.5)λ - Outdoor scenario: - FR1: Option 1) 4 Tx/4 Rx, (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 2), (dH, dV) = (0.5, 0.5)λ, Option 2) 8 Tx/8 Rx, (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 4), (dH, dV) = (0.5, 0.5)λss - FR2: Option1) 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8), (dH, dV) = (0.5, 0.5)λ – Option 2) 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4) Intel: We need to understand how much TXRU configuration is playing a role in user-satisfaction – this is not clear yet. We provide tentative view below: for FR1 outdoor: - massive MIMO case: Option 2 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,4,2,1,1;8,4) - current typical deployment case: 4 TXRU if it makes sense and user-satisfaction is acceptable for FR1 indoor: - 32 TXRU square array for FR2, we can use 2 TXRU/single-panel for both outdoor (tall) and indoor (square)

For indoor scenario, there are clear majority views on the BS antenna. So following proposal is suggested.

#### Proposal: Adopt the following BS antenna parameters for indoor scenario for XR/CG evaluation

- For indoor scenario:

  - FR1:

    - 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4)

    - (dH, dV) = (0.5, 0.5)λ

  - FR2:

    - Option 2: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (16, 8, 2,1,1;1,1)

    - (dH, dV) = (0.5, 0.5)λ

- Please share your views on this Proposal.

| Company |  | Comment |  |
| --- | --- | --- | --- |
| AT&T |  | Agree |  |
| CATT |  | Agree with moderator’s proposal |  |
| vivo |  | Agree |  |
| MTK |  | M=16 is too large for FR2 simulation. We suggest to set M=4 for FR2 as in 38.802 Table 7.1.4-1 for indoor hotspot. |  |
| ZTE, Sanechips | Agreed for FR1. FR2 evaluations should be de-prioritized |  |
| InterDigtial |  | We agree with the proposal |  |
| Ericsson |  | Agree |  |
| QC |  | We support the proposal. |  |
| Huawei, HiSilicon |  | Agree with FR1 part. |  |
| Nokia, NSB |  | Would be good to clarify that these are specifically for 4GHz and 30 GHz to note that if other frequency bands are used at some point in time, the antenna configurations would need to be revisited. We may provide additional comments later. |  |
| FUTUREWEI |  | Ok for FR1 part. |  |
| DOCOMO |  | Agree |  |
| Apple |  | In general, for indoor scenario, the antenna number is on the large side from the proposal. |  |
| LG |  | Agree |  |

##### Summary of question 8

- Agree:

  - AT&T, CATT, vivo, ZTE, IDC, Ericsson, QC, Huawei, Nokia (for 4GHz and 30GHz), FUTUREWEI, DOCOMO, LG

- Comment:

  - MTK: M=16 is too large for FR2 simulation. We suggest to set M=4 for FR2 as in 38.802 Table 7.1.4-1 for indoor hotspot.

  - Apple: for indoor scenario, the antenna number is on the large side from the proposal.

For outdoor scenario, the number of supported companies for the following options are quite close. Considering the potential simulation work load would be high for XR/CG evaluation, it is good to further down-select the options for BS antenna. \

#### Proposal: Down-select from following options for outdoor scenario for XR/CG evaluations

- For outdoor scenario:

  - For FR1,

    - Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (12,8,2,1,1;4,8)

    - Option 3: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8)

(dH, dV) = (0.5λ, 0.8λ)

  - FR2:

    - Option 1: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1)

    - Option 2: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (32,8,2,1,1;1,1)

(dH, dV) = (0.5λ, 0.5λ)

- The aim is trying to down-select in 2nd round of discussion. Please share your preference among options.

| Company |  | Comment |  |
| --- | --- | --- | --- |
| AT&T |  | FR1: Option 3, FR2: Option 1 |  |
| CATT |  | Why do we not to use antenna configuration for dense urban in IMT-2020 submission? |  |
| vivo |  | For FR1, option 1 or option 3 are fine. For FR2, option 1. |  |
| MTK |  | Why do we not to use antenna configuration for dense urban in IMT-2020 submission? If really to choose, FR1: Option 3, FR2: Option 1. |  |
| ZTE,Sanechips | FR1: Option 1. FR2 evaluations should be de-prioritized |  |
| InterDigtial |  | We think for FR1 option 3 is good and for FR2 Option 1 can be prioritized. |  |
| Ericsson |  | FR1: Option 3, FR2: Option 1 |  |
| QC |  | For FR1, Option 3 is preferred. For FR2, Option 2 is preferred. |  |
| Huawei, HiSilicon |  | FR1: Option 1. |  |
| Nokia, NSB |  | Would be good to clarify that these are specifically for 4GHz and 30 GHz to note that if other frequency bands are used at some point in time, the antenna configurations would need to be revisited. We may provide additional comments later. |  |
| FUTUREWEI |  | For FR1, option 3 is preferred. |  |
| DOCOMO |  | FR1: Optoin 3. FR2 can be deprioritized. |  |
| Apple |  | As we comment on the email thread, we have concern on the large number of antennas: 1) the need to study antenna virtualization, 2) long simulation time. The number of antennas needs to come down, e.g. to 16 TRXUs. |  |
| LG |  | As we replied on the email thread, we want to clarify whether MU-MIMO by 64 TxRU is essential in this study. |  |

##### Summary of question 9

- For outdoor scenario:

  - For FR1,

    - Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (12,8,2,1,1;4,8)

      - Vivo, Huawei

    - Option 3: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8)

      - AT&T, vivo, MTK, IDC, Ericsson, QC, FUTUREWEI, DCM

  - FR2:

    - Option 1: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1)

      - Vivo, MTK, IDC

    - Option 2: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (32,8,2,1,1;1,1)

      - QC

- Comment:

  - CATT, MTK: use antenna configuration for dense urban in IMT-2020 submission

  - Nokia: Would be good to clarify that these are specifically for 4GHz and 30 GHz to note

  - Apple: concern on the large number of antennas: 1) the need to study antenna virtualization, 2) long simulation time. The number of antennas needs to come down, e.g. to 16 TRXUs.

  - LG: need to clarify whether MU-MIMO by 64 TxRU is essential

Q12 (round #1). Please share your comments on the proposal 5. Proposal 5: Regarding the UE antennas, adopt the following assumption for FR1 and down-select from the following options for FR2 for XR evaluation. Companies views - FR1: - 2 or 4Tx/2 or 4Rx, (M, N, P, Mg, Ng; Mp, Np) = (1,2,1/2,1,1;1,2) - 4T/4R: ZTE, FUTUREWEI, vivo, Huawei, Nokia - 2T/2R: LG - 2T/4R: QC, CMCC, vivo, Nokia, CATT, Intel, FB - 1T, 2Rx: E///, MTK, FB, Samsung, Apple - AT&T: (1,2,2,1,1;1,2) (dH, dV) = (0.5, N/A)λ - FR2: 4 Tx/4Rx, - Option 1: (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,2;1,2) - MTK - Option 2: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) - Vivo, Nokia, AT&T - Option 3: {2, 2, 2} per panel. Number/location of panels: 3 panels (left, right, and top) - QC, AT&T, FB - 1T/2R: E///, FB - 2T/2R: Intel (dH,dV) = (0.5, 0.5)λ The polarization angles are 0° and 90°

For FR1, 2T/4R is the typical configuration for UE antennas and it is supported by majority companies. So it is suggested as baseline for FR1 in XR/CG evaluation.

4T/4R and 1T/2R can also be optionally evaluated.

For FR2, 4 Tx/4Rx is used in TR 38.802. So it is suggested as baseline for FR2 in XR/CG evaluation. Other option can be optionally evaluated.

#### Proposal: Adopt the following UE antenna parameters for XR/CG evaluations

- FR1:

  - Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ

  - Optional: 4T/4R, 1T/2R

- FR2:

  - Baseline: 4 Tx/4Rx, (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, the polarization angles are 0° and 90°

    - Based on 38.802 Table A.2.1-4

  - Optional: 4 Tx/4Rx, {2, 2, 2} per panel. Number/location of panels: 3 panels (left, right, and top)

- Please share your views on the Proposal.

| Company |  | Comment |  |
| --- | --- | --- | --- |
| AT&T |  | agree |  |
| CATT |  | OK |  |
| vivo |  | Agree with the proposal |  |
| MTK |  | Fine for FR1. For FR2, in 38.802 Table A.2.1-4, Mg=1 and Ng=1, so we suggest to check the setting in FR2 further. |  |
| ZTE,Sanechips | Agreed for FR1. But it should be clarified that according to this configuration, the baseline should be 4T/4R. |  |
| Sony |  | Support the proposal |  |
| InterDigtial |  | We agree with the proposal |  |
| Ericsson |  | FR1: OK, although 2TX is somewhat futuristic. F2: Don’t support. 4Tx/4Rx. After considerable discussion in RAN1, there was an agreement in the FeMIMO WI to support a 3-panel UE antenna configuration with one active panel (2Tx/Rx), as described in R1-2007151. Propose to use that. |  |
| QC |  | For FR1, we support the proposal. For FR2, - We prefers 2T/2R: (M, N, P; Mp, Np)=(2, 2, 2; 1, 1), the polarization angles are 0° and 90°, 3 panels (left, right, top). - In order to capture more realistic UE panel placement, we prefer to specify the placement of panels (e.g., left, right, top, back) directly as done in [R1-2007151] instead of using rectangular model based on Mg and Ng. Optionally support baseline: 4Tx/4Rx, (M, N, P; Mp, Np) = (2,4,2;1,2), (dH,dV) = (0.5, 0.5)λ, the polarization angles are 0° and 90° with 3 panels placed at left, right, and top. |  |
| Huawei, HiSilicon |  | Support FR1 part. |  |
| Nokia, NSB |  | We agree with the proposal. Would be good to clarify that these are specifically for 4GHz and 30 GHz to note that if other frequency bands are used at some point in time, the antenna configurations would need to be revisited. |  |
| FUTUREWEI |  | Support FR1 with 4T/4R |  |
| Facebook |  | For FR1, agree with the baseline although we think 1T/2R is also important. For FR2, we agree with Ericsson and QCOM that we can reuse R1-2007151 as the baseline. |  |
| DOCOMO |  | Support the proposal |  |
| Apple |  | For FR1, 1Tx and 2R for UE is important, power consumption/thermal issues for XR devices are key issues, starting with more realistic setup is preferred. |  |
| LG |  | For FR1, we think 2TX2R or 1TX2R should be prioritized. |  |

##### Summary of question 10

- FR1:

  - Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ

  - Optional: 4T/4R, 1T/2R

- FR2:

  - Baseline: 4 Tx/4Rx, (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, the polarization angles are 0° and 90°

    - Based on 38.802 Table A.2.1-4

  - Optional: 4 Tx/4Rx, {2, 2, 2} per panel. Number/location of panels: 3 panels (left, right, and top)

- Agree

  - AT&T, CATT, vivo, Sony, IDC, Nokia, DCM

- FR1 part only

    - MTK, ZTE, Ericsson, QC, Huawei

- FR1 with 4T/4R

  - ZTE, FUTUREWEI

- FR1 with 1T/2R

  - FB, Apple, LG

- Comment on FR2:

  - MTK: For FR2, in 38.802 Table A.2.1-4, Mg=1 and Ng=1

  - E///: Don’t support. 4Tx/4Rx. After considerable discussion in RAN1, there was an agreement in the FeMIMO WI to support a 3-panel UE antenna configuration with one active panel (2Tx/Rx), as described in  R1-2007151. Propose to use that.

  - QC: For FR2,

    - We prefers 2T/2R: (M, N, P; Mp, Np)=(2, 2, 2; 1, 1), the polarization angles are 0° and 90°, 3 panels (left, right, top).

    - In order to capture more realistic UE panel placement, we prefer to specify the placement of panels (e.g., left, right, top, back) directly as done in [R1-2007151] instead of using rectangular model based on Mg and Ng.

    - Optionally support baseline: 4Tx/4Rx, (M, N, P; Mp, Np) = (2,4,2;1,2), (dH,dV) = (0.5, 0.5)λ, the polarization angles are 0° and 90° with 3 panels placed at left, right, and top.

  - FB: For FR2, we agree with Ericsson and QCOM that we can reuse R1-2007151 as the baseline.

Q13 (round #1). Please share your comments on the proposal 6. Proposal 6: Regarding the downtilt, down-select from the following options for FR1 and FR2 for XR evaluation. Companies views - Outdoor: - Option 1: 6 degree - ZTE, FUTUREWEI, QC, vivo - Option 2: 14 degree - IDC, MTK - Option 3: 100 - Option 4: 90° in GCS (pointing to horizontal direction) - IDC - Other: - E///, AT&T: appropriate tilt would depend on the scenario - Nokia: The decision on the antenna downtilt should be deferred after the decisions on the deployment scenarios and on the antenna configuration - Indoor: - Option 1: 0 degree - IDC, MTK, vivo - Option 2: 180° in GCS (pointing to the ground) - IDC - 90° (pointing to the ground) - ZTE, QC (For maximum coverage, for outdoors a vertical panel (0 degree) is preferred and for indoors, the panel is expected to be horizontal (90 degrees) (i.e. pointing to the ground).) - E/// - AT&T

#### Proposal: For XR/CG evaluation, adopt the following assumptions for downtilt

- For XR/CG evaluation, adopt the following assumptions for downtilt

  - Dense Urban

    - 6 degree

  - Indoor hotspot

    - 90° (pointing to the ground)

- Please share your views on the Proposal.

| Company |  | Comment |  |
| --- | --- | --- | --- |
| AT&T |  | Ok for indoor hotspot. For outdoor, this should be determined after antenna configuration is decided |  |
| CATT |  | These parameters do not make sense in real system deployment. |  |
| vivo |  | Agree with the proposal |  |
| MTK |  | The downtilt should be determined after antenna configuration is decided |  |
| ZTE,Sanechips | Okay for FR1. Deprioritize the FR2 evaluations. |  |
| InterDigital |  | We agree with the proposal |  |
| Ericsson |  | OK |  |
| QC |  | Support the proposal. |  |
| Ericsson |  | OK |  |
| Nokia, NSB |  | We agree with the proposed downtilt |  |
| FUTUREWEI |  | Ok for FR1 |  |
| DOCOMO |  | OK |  |
| LG |  | OK |  |

##### Summary of question 11

- Agree

  - AT&T (InH only), vivo, IDC, E///, QC, ZTE, Nokia, FUTUREWEI, DCM, LG

- Comment:

  - The downtilt should be determined after antenna configuration is decided

    - AT&T, MTK

  - CATT: not make sense in real system deployment

Q14 (round #1). Please share your comments on the proposal 7. Proposal 7: Regarding the BS Tx power, down-select from the following options for XR evaluation. Companies views - For indoor scenario: - FR1: - Alt1: 24dBm/20MHz - ZTE, DCM, IDC, MTK, Nokia - Alt2: 30dBm - ZTE , IDC - 24dBm/100MHz - QC, CMCC, vivo, CATT, AT&T - FR2: - Alt1: Maximum EIRP 58dBm - QC, vivo, AT&T - Alt2: 23dBm - QC, MTK, Nokia - For outdoor scenario: - FR1: - Alt1: 46dBm - CATT: 46 dBm for 100MHz - Alt2: 49dBm - IDC - QC (Uma) - AT&T (Uma) - Alt3: 44dBm/20MHz - FUTUREWEI, DCM, MTK, Huawei - QC (UMi) - AT&T: UMi - Alt4: 53dBm - IDC, CMCC, vivo - Other: - Nokia: 43 dBm / 20 MHz - FR2: - Alt1: Maximum EIRP 73dBm - IDC, vivo - QC (UMi/UMa) - Alt2: 37dBm - MTK - Alt3: 28dBm - QC (UMi/UMa) - Other: - Nokia: 43 dBm - Other comments: - E///: Follow 38.802, Table A.2.1-1

Summary of Q14 in Round 1 discussion :

It seems there is no converged view from companies for now. In TR 38.802, assumptions on BS Tx power for evaluation are defined in Table A.2.1-1. The BS Tx power can be reused for XR/CG evaluation.

#### Proposal: Reuse the BS Tx power assumptions as defined in 38.802 Table A.2.1-1 for XR/CG evaluation.

- For Indoor hotspot:

  - FR1:

    - 24 dBm per 20 MHz

  - FR2:

    - 23 dBm per 80 MHz. EIRP should not exceed 58 dBm

- For Dense urban:

  - FR1:

    - 44 dBm per 20 MHz

  - FR2:

    - 40 dBm per 80 MHz. EIRP should not exceed 73 dBm

- Please share your views on the Proposal.

| Company |  | Comment |  |
| --- | --- | --- | --- |
| AT&T |  | Can agree with the proposal following TR38.802 |  |
| CATT |  | We don’t agree that only 20 MHz for FR1. |  |
| vivo |  | Agree with the proposal |  |
| MTK |  | Fine with the proposal |  |
| ZTE,Sanechips | Agreed for FR1. Deprioritize the evaluations for FR2. |  |
| Sony |  | Agree to ues TR 38.802 as reference/baseline |  |
| InterDigtial |  | We support adopting the proposed BS Tx power assumptions for both Indoor FR1 & FR2 as well as Dense urban FR1 & FR2 |  |
| Ericsson |  | Support |  |
| QC |  | We suggest to define tx power numbers for 100(400)MHz for FR1(FR2). Our understanding of 38.802 is that 24(23)dBm is defined for system bandwidth which is, in our case, 100(400)MHz for FR1(FR2). According to 38.802, in case companies want to have system bandwidth larger than the simulation bandwidth of 20MHz (in order to reduce simulation runtime), the BS tx power should be scaled down accordingly. We think the idea of “simulation bandwidth” in 38.802 was introduced to allow reduced scale simulation and correct tx power scale down when system bw > simulation bw. We are fine with EIRP limits for FR2. |  |
| Huawei, HiSilicon |  | Agree with FR1 part. |  |
| Nokia, NSB |  | We agree with the proposal. |  |
| FUTUREWEI |  | Ok with FR1 part. |  |
| DOCOMO |  | Agree |  |
| Apple |  | Fine for FR1, but the urban macro one is missing |  |

##### Summary of question 12

- Agree to use the assumptions in TR 38.802

  - AT&T, vivo, MTK, ZTE(FR1 only), Sony, IDC, E///, Huawei (FR1 only), Nokia, FUTUREWEI(FR1 only), DCM, Apple(FR1 only)

- Comments:

  - CATT: don’t agree with only 20 MHz for FR1

  - QC: suggest to define tx power numbers for 100(400)MHz for FR1(FR2).

Q15 (round #1). Please share your comments on the proposal 8. For the following assumptions in Table 2, they are important for the XR evaluation and may be related to the implementation/configuration. Hence, they should be reported by company with detailed assumptions for the evaluation. Proposal 8: Adopt the following simulation assumptions in Table 2 for XR evaluation. Power control parameter; Companies should report. Transmission scheme; Companies should report, such as Type I/II codebook, rank assumption. Scheduler; MU-MIMO PF scheduler, other scheduler (e.g., delay aware scheduler) is up to companies report. CSI Feedback; Realistic Companies should report CSI feedback delay, CSI report periodicity, whether using CSI quantization, CSI error model or not, and etc.. PHY processing delay; UE Capability #1 Companies should report gNB processing delay, e.g. DL NACK to retransmission delay, UL previous transmission to current transmission delay and etc.. PDCCH overhead; Companies should report. DMRS overhead; Companies should report. SRS; Companies should report.

#### Proposal: Adopt the updated table 3 as below

Table 3: Simulation assumptions for XR evaluation (Part 3)

| Power control parameter | Companies should report |
| --- | --- |
| Transmission scheme | Companies should report, such as Type I/II codebook, rank assumption |
| Scheduler | SU/MU-MIMO PF scheduler (company to report SU or MU), other scheduler (e.g., delay aware scheduler) is up to companies report |
| CSI acquisition | Realistic Companies should report CSI feedback delay, CSI report periodicity, whether using CSI quantization, CSI error model or not, and etc. Both CSI feedback and SRS are considered |
| PHY processing delay | UE PDSCH processing Capability #1 Companies should report gNB processing delay, e.g. DL NACK to retransmission delay, UL previous transmission to current transmission delay and etc. |
| PDCCH overhead | Companies should report |
| DMRS overhead | Companies should report |
| Target bler | Companies should report |
| Max HARQ transmission | Companies should report |

- Please share your views on the Proposal.

| Company |  | Comment |  |
| --- | --- | --- | --- |
| AT&T |  | Agree |  |
| CATT |  | OK |  |
| vivo |  | Agree |  |
| MTK |  | Fine |  |
| ZTE,Sanechips | Agreed. |  |
| Sony |  | Agree |  |
| InterDigtial |  | We support adopting Table 3 for XR simulation |  |
| Ericsson |  | OK |  |
| QC |  | Support the proposal with following addition – Assumptions on SRS: periodicity, processing gain, processing delay, etc |  |
| Huawei, HiSilicon |  | For channel estimation, suggest “ideal” for simplicity. “Realistic” can be up to companies report. |  |
| Nokia, NSB |  | We agree with the proposal except for the PHY processing delay. For some delay-constrained applications it may be better to consider the Capability # 2 (URLLC) instead of Capability # 1 (eMBB). We suggest thus reformulating this part as “Companies should report”. |  |
| FUTUREWEI |  | Ok. Also ok to use ideal channel estimation for simplicity. And agree with QC to have assumptions for SRS |  |
| DOCOMO |  | OK |  |
| Apple |  | Assume “ideal channel estimation” can be also considered. |  |
| LG |  | Agree |  |

##### Summary of question 13

- Agree

  - AT&T, CATT, vivo, MTK, ZTE, Sony, IDC, E///, QC, Nokia, Huawei

- Comment:

  - QC: Assumptions on SRS: periodicity, processing gain, processing delay, etc

  - For channel estimation, suggest “ideal” for simplicity. “Realistic” can be up to companies report.

    - Huawei, FUTUREWEI, Apple

  - Nokia: For some delay-constrained applications it may be better to consider the Capability # 2 (URLLC) instead of Capability # 1 (eMBB). We suggest thus reformulating this part as “Companies should report”.

### Aspects related to Traffic model

In summary, for XR/CG capacity evaluation, there are some aspects on evaluation methodology and evaluation assumptions that are related to traffic model. So they will be discussed after traffic model is determined.

Proposal: The following aspects are to be discussed after traffic model is determined.

- For the system capacity definition, how to determine whether a UE is satisfied or not is to be deferred until the exact traffic model along with how to measure E2E user experience is available.  Additional metrics to be collected will be further discussed after traffic model is stable.

- Various options for traffic arrival offset among UEs per call were proposed by companies, e.g., even offset, random offset, no offset. It will be discussed after traffic model is determined.

- Whether/how to model transport protocol(s) can be discussed when traffic model is determined.

- Please share your views on the Proposal.

| Company |  | Comment |  |
| --- | --- | --- | --- |
| AT&T |  | Ok with the proposal |  |
| CATT |  | We don’t think the proposals are appropriate. The traffic model is a specific characteristic of the stochastic process. We never evaluate the service with higher layer protocol in RAN1 |  |
| vivo |  | Agree with the suggestion |  |
| MTK |  | Fine with the first two bullets. The third seems not necessary as suggested by CATT. |  |
| ZTE,Sanechips | Okay with the proposal. |  |
| Sony |  | Support the proposal |  |
| InterDigital |  | We agree with the proposal |  |
| Ericsson |  | Support. It would seem appropriate to investigate if the transport protocols affect performance. If they do, they should be included in the evaluations. |  |
| QC |  | Support proposal. |  |
| Huawei, HiSilicon |  | Generally fine except the last sub-bullet. Regarding the last sub-bullet, could FL explains what does “transport protocol(s)” refer to? Like TCP/UDP? And why it is related to RAN1 evaluation? |  |
| Nokia, NSB |  | We agree with the proposal. |  |
| FUTUREWEI |  | Ok with the proposal. Similar to Huawei’s view, we should focus on RAN1 perspective for evaluation. |  |
| DOCOMO |  | Agree |  |
| Apple |  | Fine with the proposal |  |
| LG |  | Agree |  |

##### Summary of question 14

- Agree

  - AT&T, vivo, ZTE, Sony, IDC, E///, QC, Nokia, DCM, Apple, LG

  - Not include the last bullet

    - CATT, MTK, Huawei, FUTUREWEI,

## Other aspects

Q16 (round #1). Please share additional comments if any on Table 2. Company; Comment. AT&T; Any restrictions on PDSCH mapping type A/B?. MTK; After all the parameters are determined, calibration for some simple scenarios (Ex. FTP model 3) is necessary. For example, companies should report similar throughput, latency, RU, … under the same setting, so the following study can be more aligned between companies..

On AT&T’s comment

It was commented by AT&T that whether and how to consider the PDSCH mapping type, e.g. mapping type A or B, in the evaluations. There are differences for PDSCH mapping type A and type B such as PDSCH starting symbol and DMRS symbol position. Whether type A or B is used in the simulation can be reported by companies when submitting the results.

FL suggestion: Company can report whether type A or B is used in their simulation when submitting results.

Q17 (round #1): In addition to the assumptions in Table 1 and Table 2, are there any assumptions which are necessary to define for XR evaluation? Please share your comments on the Q17. Company; Comment. InterDigital; We think that channel model assumptions might be missing. TDL-A/B/C/D/E or CDL-A/B/C/D. QC; In general, the simulation assumptions between capacity evaluation (which does not require power evaluation at all) and power evaluation (where capacity should be evaluated subject to a capacity constraint) need to be the same. In case different parameters need to be considered, they should be reported with the results.. Nokia, NSB; The assumptions regarding the transport protocol(s) used should be decided at some point, as this will affect the simulation results..

On InterDigital’s comment:

It was proposed by InterDigital that channel model assumptions TDL-A/B/C/D/E or CDL-A/B/C/D are also needed. TDL-A/B/C/D/E or CDL-A/B/C/D channel models are used for LLS.

FL suggestion: According to input from companies, most of companies are focusing SLS for XR/CG capacity evaluation. So it may not need to consider these LLS channel model for XR/CG capacity evaluation

On Nokia’s comment:

FL suggestion: Whether/how to model transport protocol(s) can be discussed when traffic model is determined.

Q18 (round #1): Whether or not to consider the following simulation assumptions for XR evaluation? - Beam related operation, such as beam update mechanism, beam activation delay, beam metric - Others, e.g. RLC, network layer setting, core network delay Q19 (round #1). Please share any other comments if any on capacity evaluation for XR and CG. Company; Comment. MTK; Why not just align to IMT 2020 settings (dense urban, InH …) of System level simulation assumptions as in 38.840? Discussing the various simulation parameter possibilities can be time consuming.. Intel; Agree with MTK – we can start from IMT and see what changes are necessary. Another question is whether calibration campaign is planned for this SID?.

Summary of Q18/Q19 in Round 1 discussion

For the assumptions that may be related to traffic model, they can be discussed with traffic model after there is more input from SA4.

For those simulation assumptions that are not mentioned in the summary, they can be referred to TR 38.802. Most of simulation assumptions in TR 38.802 are similar to those in TR 37.910 for IMT 2020 evaluation.

Companies can also report additional simulation assumptions if any.

# Summary of Round 1 email discussion for Capacity Evaluation

## Deployment

Use cases and deployment scenarios of XR/CG applications proposed by companies [2][3][4][8][11][12][13][15][18] are summarized as below.

| Companies/scenarios | VR | AR | Cloud gaming |
| --- | --- | --- | --- |
| Huawei | Dense urban (FR1) Urban Macro (FR1) | Dense urban (FR1) Urban Macro (FR1) | Dense urban (FR1) Urban Macro (FR1) |
| vivo | Indoor hotspot Dense urban | Indoor hotspot Dense urban | Indoor hotspot Dense urban |
| CATT | Indoor hotspot Dense urban | Indoor hotspot Dense urban | Indoor hotspot Dense urban |
| LG | Indoor with low mobility | AR1: Indoor/outdoor with low mobility AR2: Indoor/outdoor with low/high mobility | Indoor/outdoor with low/high mobility |
| MediaTek | Indoor hotspot (FR1/FR2) | AR1: UMi(indoor & outdoor) (FR1/FR2), HST(FR1) AR2: UMi(indoor & outdoor) (FR1/FR2) | UMi(indoor & outdoor) (FR1) Rural(indoor & outdoor) (FR1) High speed train (FR1) |
| Xiaomi |  | Indoor/outdoor (FR1/FR2) | Indoor/outdoor (FR1/FR2) |
| Qualcomm | Indoor hotspot (open office) (FR1/FR2) UMi mixed (FR1) | UMi mixed (FR1) Indoor hotspot (open office) (FR1/FR2) UMi (outdoor) (FR2) | UMi mixed (FR1) Indoor hotspot (open office) (FR1/FR2) UMi (outdoor) (FR2) |
| AT&T | UMa(indoor &outdoor) (FR1) UMi(outdoor) (FR2) Indoor hotspot (FR1/FR2) | UMa(indoor & outdoor) (FR1) UMi(outdoor) (FR2) Indoor hotspot (FR1/FR2) | UMa(indoor & outdoor) (FR1) UMi(outdoor) (FR2) Indoor hotspot (FR1/FR2) |
| Nokia | Indoor hotspot Urban Macro (UMa) | Indoor hotspot Urban Macro (UMa) | Indoor hotspot Urban Macro (UMa) |

The use cases of XR and CG applications can occur in the indoor or outdoor scenarios. Based on the contributions from companies, the deployment scenarios proposed include InH, UMi, Dense Urban and UMa.

Q1: For UMi and Dense urban scenarios, whether both of them need to be separately evaluated, or only UMi is to be evaluated for the sake of reducing the number of evaluation scenarios?

Please share your views on Q1.

| Company | Comment |
| --- | --- |
| ZTE,Sanechips | UMi scenario should be considered for the evaluation of XR and CG applications. |
| FUTUREWEI | UMi scenario should be used. Overall, we’d like to prioritize FR1 in this SI. |
| LG | We may focus only on InH snd UMi in this study. |
| DOCOMO | UMi scenario should be used. |
| InterDigital | We think UMi (indoor and outdoor) is sufficient for reducing the number of evaluation scenarios |
| QC | Given that Dense urban and UMi are very similar scenario, we propose to evaluate with UMi only. This will help reducing the workload in RAN1. |
| MTK | We think we can choose one of them to reduce evaluation scenarios. Either UMi or dense urban is fine for us. |
| CMCC | We think either UMi or Dense urban can be considered. |
| Ericsson | It is not clear to us what deployment scenario UMi corresponds to. We would suggest using the scenario definitions in 38.913. We propose to use Dense Urban with only the macro layer. |
| Xiaomi | To reduce the evaluation workload, we are fine to only evaluate UMi scenario. |
| vivo | UMi is to evaluated for reducing the number of evaluation scenarios |
| Huawei, HiSilicon | The question needs to separate propagation channel models (InH, UMi, UMa; see TR 38.901) from deployment scenarios (Dense Urban, Urban Macro; see TR 38.913). Both parts need to be agreed. For deployment scenario, we support Dense Urban with FR1 mainly because they are the typical commercial deployment of NR networks. We suggest to perform evaluation and provide valuable studies at least for the already deployed NR networks. For the channel model, it can be further discussed. Note: we added “(FR1)” to Huawei’s proposals in the table above. |
| Nokia, NSB | We think that the number of scenarios to be evaluated should be kept to a reasonable level. To this end, we think that we should select only UMi scenario. |
| CATT | Either Umi or dense urban is fine. |
| AT&T | Agree with Ericsson. Assuming 38.913 we prefer dense urban scenario for outdoor evaluations with highest priority |
| Intel | Agree with Huawei that we have 2 components – deployment scenario and channel model. Between UMi and DU we think one common scenario/channel-model combination is sufficient. |
| Facebook | We think either is fine. |
| Samsung | Either UMi or DU (one only – FR1). |
| Apple | Either one fine for FR1. |
| Sony | We think UMi or Dense urban to cover to both outdoor and indoor scenarios. |

Q2: The deployment scenarios for evaluation may be applied to each of XR/CG applications of interest. However, if all the XR/CG applications are considered, there will be too many combinations of deployment scenarios and XR/CG applications, which could lead to numerous simulation work. Therefore, it may be desirable to consider prioritization of combinations of deployment scenarios and XR/CG applications, e.g.,

- FR 1:

  - InH: CG and VR are prioritized.

  - Umi: AR and CG are prioritized.

  - Uma: AR (e.g., low rate AR)

- FR 2:

  - InH: CG and VR are prioritized.

  - Umi: AR and CG are prioritized.

  - Uma: N/A

Please note that with such prioritization, companies can still submit evaluation results for de-prioritized scenarios.

Please share your views on Q2 including whether such prioritization is needed or not.

| Company | Comment |
| --- | --- |
| ZTE,Sanechips | We think from simulation work load perspective, it would be good to prioritize FR1 over FR2. In terms of the applications within FR1, clarification is needed regarding which one or both of {AR1,AR2}, {VR1, VR2} would be evaluated. The following note under the proposal would serve the purpose: Note: Depending on the outcome of the further discussion, one or both of {AR1,AR2}, {VR1, VR2} are to be evaluated. |
| FUTUREWEI | Prioritize FR1 over FR2. Agree to reduce the number of combinations for evaluation. |
| LG | We are fine with the proposal without Uma. We also think it is better to focus on FR1. |
| DCOCOMO | Fine with the proposal. We also think FR1 should be prioritized for this work. |
| InterDigital | We think the following scenarios should be prioritized: - FR 1: InH: CG and VR, Umi: AR and CG and Uma: AR and CG - FR 2: InH: CG and VR, Umi: AR and CG |
| QC | We support the above prioritization. Low rate AR (e.g., low rate streaming, text notification, etc.) is very interesting to study as it may be more relevant in the near term market. It is expected that such use case may be widely used in both indoor/outdoor scenarios. |
| Ericsson | Not sure if such priority categorization based on use cases is needed, since CG and VR may have very similar traffic models. Proposed prioritization: - CG in FR1: Urban macro and dense urban - AR in FR1: Urban macro and dense urban |
| MTK | We think such prioritization is needed. We suggest to prioritize one scenario for FR1 and one scenario for FR2 and Umi is our preference. |
| CMCC | We think FR1 should be prioritized. |
| Xiaomi | We think InH is also an important scenario for AR applications, e.g. in smart home or office, and thus all the three use cases VR/AR/CG should be prioritized for InH scenario. We are fine to FL’s proposal on other scenarios. To reduce the evaluation workload, we are fine to prioritize FR1 over FR2, and support to select only a subset within AR1/AR2, VR1/VR2 to evaluate. |
| Vivo | Prioritization on some combinations of deployment scenarios and applications is needed. For each application, it can be evaluated in a deployment scenario. - FR 1: - InH: CG and VR are prioritized. - Umi: AR is prioritized. - Uma: low rate AR - FR 2: - InH: CG and VR are prioritized. - Umi: AR is prioritized. |
| Huawei, HiSilicon | We think FR1 should be prioritized over FR2. As for applications, since the traffic model of each application is unclear, we think it’s premature to prioritize among them at this stage. We suggest to discuss the traffic model first so that companies can have better understanding of each application, and then we can pick some representative cases for further evaluation. As for “low rate AR”, some company [18] mentioned it is currently being studied in SA4. According to the following Note in chairman notes, RAN1 will not discuss this application in this meeting. - NOTE: SA4 has ongoing work in the XR area, RAN1 will not address these SA4 aspects but will wait for SA4’s outcome |
| Nokia, NSB | We agree that there should be prioritization in order to reduce the total number of simulation scenarios and keep the simulation effort to a reasonable level. We think that CG and VR applications are more reasonable for indoor deployment (InH), while AR applications fit outdoor deployment (either Uma or Umi). Furthermore, we think that low-rate AR applications are less interesting for capacity evaluation and should not be considered in the simulation scenarios. |
| CATT | We would like to prioritize FR1 over FR2 |
| AT&T | We do not agree to any prioritization of FR1 or FR2 for this study. Both are important for different XR applications. We are OK with the mapping of applications to scenario given by the FL in Q2 except we believe AR should also be added for indoor evaluations in FR1 and FR2. |
| Intel | We think prioritization of FR1 over FR2 is okay from simulation load perspective. We think prioritization across use-cases is not desirable, so should equally cover AR/VR/CG use-cases. We can try this way VR InH, UMi,... AR --> UMi, UMa,... CG --> InH, UMi,... On the other hand, we also agree with Huawei that may be this prioritization exercise is pre-mature and we can wait for requirements, KPIs and traffic model discussions to progress further. |
| Facebook | We think AR should be added for both InH and Umi/Dense Urban as AR is envisioned to be use in all difference scenario. We also think that low rate AR is an important use case addressing the near term market need. |
| Samsung | Prioritize FR1 over FR2. Prioritize as AR > CG > VR. Support to consider low rate AR for UMa. Basically, prefer initial focus of simulation to be on FR1 and on AR for UMi/DU and UMa. |
| Apple | Prioritize FR1, and also focus both both downlink heavy traffic and uplink heavy traffic. |
| Sony | We support to start with FR1, and both downlink and uplink heavy traffic. |

## Evaluation methodology and assumptions

### Methodology

For evaluation of XR/CG applications, the definition of system capacity needs to be determined. In general, similar to the previous 3GPP study e.g. URLLC, the system capacity is defined as the maximum number of users per cell satisfying a certain set of requirements. Hence, for XR/CG evaluation, the system capacity can be defined as the following.

Q3: System capacity is defined as the maximum number of users per cell with at least X % of UEs being satisfied (i.e., meeting a set of requirements).  The exact requirements will be defined separately.

Please share your views on Q3. Companies can also present other definition of system capacity that they believe is appropriate.

| Company | Comment |
| --- | --- |
| ZTE,Sanechips | Agree that number of Ues under a given {PDB,PER} requirement in which X% is satisfied should be evaluated. |
| FUTUREWEI | Agree |
| LG | We are fine with FL’s proposal |
| DOCOMO | Agree in principle but we prefer to consider eMBB+URLLC multiplexing case. There would be the case where only URLLC devices are accommodated in a cell and both URLLC and eMBB devices are accommodated in a cell considering the commercial use cases. For example, the former can target e-sports event, and the latter can target AR conference or VR game on high speed train, where other eMBB devices are also located for other purposes, e.g. internet service. Besides, users who play AR/CG/VR would have mobile phone for eMBB in addition to AR/CG/VR devices for URLLC. |
| InterDigital | We are ok with the definition for system capacity. For the requirements we prefer to include different traffic requirements (e.g. DL/UL throughput, RTT latency, max UL/DL PDB, reliability) for different applications (e.g. VR, AR and CG) |
| QC | We think the above definition of XR capacity is reasonable. |
| Ericsson | Agree |
| MTK | The proposed definition is reasonable but may require a large simulation effort (since we have to sweep every user number). A possible alternative is to choose 3 user numbers, say 10, 15, 20, and defined the capacity to be the averaged satisfied user number. |
| CMCC | Support FL proposal. |
| Xiaomi | Agree |
| vivo | We agree with the FL’s proposal. |
| Huawei, HiSilicon | We are generally fine with the proposal. However, at this stage, it’s unclear to use “a set of requirements” or just “one requirement”. So we suggest to use a more general expression as follows: System capacity is defined as the maximum number of users per cell with at least X % of Ues being satisfied (i.e., meeting a set of requirements). How to define the traffic requirement of a XR/CG user is satisfied The exact ‘satisfied’ requirements will be defineddiscussed separately. |
| Nokia, NSB | We agree with the system capacity definition proposed in Q3. However, while the exact set of requirements and values used to decide whether a UE is fully satisfied can be defined in a later stage, we think that UE satisfaction should be defined in terms of network-level requirements like PER, PDB and data-rate. Different set of requirements can be defined for UL and DL according to the traffic models and applications. |
| CATT | We agree with FL’s proposal |
| AT&T | Agree |
| Intel | At a high-level it make sense but this will get clearer once we know “requirements” better, as an example should we support user differentiation for services for e.g. Ues with geometry > threshold is considered for a particular service. MTK concern is also valid that whether it introduces too much simulation burden that will be clear once we know the range of PER. |
| Facebook | Agree |
| Samsung | Agree – the main issue is to define the requirements/metrics. |
| Apple | Agree |
| Sony | Agree |

Q4: For the system capacity definition in Q3, the X value needs to be determined, e.g., X=90. In addition, it may be useful to collect results (i.e., # UEs per cell being satisfied or meeting the requirements) for multiple values of X, e.g., X = 70, 80, 90, 95 to see the trend of # Ues per cell meeting the requirements as the number of Ues per cell increases.

Please share your views on Q4.

| Company | Comment |
| --- | --- |
| ZTE,Sanechips | X= 90 or 95 should be considered. |
| FUTUREWEI | X = 90 is a reasonable start. |
| LG | We are fine with FL’s proposal |
| DOCOMO | X= 90 or 95 should be considered. |
| InterDigital | We agree with collecting results for different values of X (e.g. 50,..,90, 95) |
| QC | We support reporting multiple data points for X=90, 80, 70. |
| Ericsson | X=90 or 95 should be considered |
| MTK | We prefer X=95. |
| CMCC | X= 90 or 95 should be considered. |
| Xiaomi | X=90 or 95 |
| vivo | X=90 is adopted for the capacity evaluation. X=70,80 can be optionally assumed for the evaluation. |
| Huawei, HiSilicon | X=90 is fine. Other values of X can be FFS if necessary. |
| Nokia, NSB | We agree with the use of few X values for the definition of the system capacity in order to evaluate the trend as number of Ues per cell increase. However, the number of X values should be limited to two values like 90% and 95%. |
| CATT | X=95, which is 2-time standard deviation of normal distribution |
| AT&T | Ok with feature lead proposal |
| Intel | we are fine with the proposal, the exact values of X we can decide later |
| Facebook | Fine with the proposal. |
| Samsung | X=90 is fine to start with. Importance/relative impact of other values (probably of ones > 90) can be discussed as the SI progresses. |
| Apple | X can be decided later. |
| Sony | X=90 or 95, but can be decided later |

Q5: For the system capacity definition, how to determine whether a UE is satisfied or not is to be deferred until the exact traffic model along with how to measure E2E user experience is available.

Please share your comment on Q5.

| Company | Comment |
| --- | --- |
| ZTE,Sanechips | Some high-level principle had better be settled such as the {PDB,PER} requirement. |
| FUTUREWEI | Agree to defer after traffic model and metric(s) for user experience are agreed |
| LG | We are fine with FL’s proposal |
| DOCOMO | Agree with the suggestion. |
| InterDigital | While we understand that using the exact traffic model per-application (i.e. from SA4) and measuring the QoE is important for determining capacity, we think in the evaluations the traffic parameters (e.g. PDB, PER) currently available from TR 26.928 for VR and CG can be used as baseline. The exact traffic model and parameters for AR can be included once available |
| QC | We support Q5. RAN1 is expected to discuss how to measure e2e user experience together with traffic model. |
| Ericsson | Agree. However, it is clear that some definitions of UE satisfaction will provide huge challenges for the foreseen evaluations, e.g., extremely high reliability requirements. |
| MTK | We are fine with the proposal. |
| CMCC | Agree ZTE’s view. A basic principle can be considered. |
| Xiaomi | Agree |
| vivo | Support to defer the discussion on how to determine whether a UE is satisfied. |
| Huawei, HiSilicon | We agree with FL that for the system capacity definition, how to measure E2E user experience is critical when determining whether a UE is satisfied or not. However, we suggest not to entirely defer the discussion in this meeting. In the SID, one objective is “3. Identify evaluation methodology to assess XR and CG performance along with identification of KPIs of interest for relevant deployment scenarios”. So RAN1 needs to identify proper KPIs to reflect XR/CG performance. So for progress, we suggest that in this meeting, companies can discuss and agree some general principles on KPIs, i.e., the principles on how to define the traffic requirement of a XR/CG user is satisfied or not. As mentioned in Q5, we think one principle is that RAN1 needs to identify a KPI that can reflect the user experience in XR and CG services. |
| Nokia, NSB | We agree that the definition whether a UE is satisfied or not should be deferred until the traffic models have been decided. However, we think that the set of constraints and values used to decide whether UE is satisfied should be defined in terms of network performance metrics like data-rate, PER, and PDB. |
| CATT | OK with the proposal |
| AT&T | Agree with InterDigital |
| Intel | We can discuss some principles but unless requirements/traffic-model are clear in terms of PDB, PER we cannot progress much in terms of developing criteria for user-satisfaction because we need to consider simulation complexity and time. So FL proposal is okay for us |
| Facebook | In general fine with the suggestions, but early discussion some system parameters maybe help per some companies suggested. |
| Samsung | OK with the FL proposal. |
| Apple | The latency requirement for a packet can be built in the traffic model itself. |
| Sony | Agree with Interdigital. It is the user of the UE that shall be satisfied and SA4, as the end-to-end performance group, is in their study defining criteria based on human perception of multimedia. |

Q6: On the XR/CG evaluation, other performance metrics (in addition to # of UEs per cell being satisfied) can be reported, e.g.,

- PER (file dropping rate)

- UPT

- File transfer delay

- RU

- Spectrum efficiency

- Etc.

Please share your comments on Q6. Please feel free to suggest additional metrics that you believe are useful to collect.

| Company | Comment |
| --- | --- |
| ZTE,Sanechips | Prefer to include only RU/UPT in this section given they are closely related to capacity. Compared with UPT, the additional information offered by PER is marginal. File Transfer delay is more related to latency. Spectrum efficiency metric may need some further clarification as to how or why it should be done given RU is already captured. |
| FUTUREWEI | Let’s wait until the traffic model and metric(s) for user experience are agreed. |
| LG | In principle, no agreement is necessary on this point since companies can report any performance results. For recommending specific additional metrics, further discussion is necessary based on further SA4 outcome |
| InterDigital | We think the E2E file transfer delay and RTT delay (e.g. for CG) can also be considered for capacity evaluations |
| QC | We think the above metrics are useful. Details of how to report those metrics w/ capacity result should be further discussed, e.g., averaged over entire Ues or multiple data points (e.g., 10%, 50%, 90%) in CDF of per UE metrics. In addition, as another UL metric for XR, we could also measure pose related metric such as age of pose (AOP). An AOP is defined as time duration X-Y, where - X is the time a frame Z is generated at XR server - Y is the time that a pose is generated at XR device which is used to render the frame Z In XR user experience, the motion-to-render-to-photon (M2R2P) delay is one of important metrics measuring user experience. Lower value is required to make user feel “presence”. AOP is one part of M2R2P, so lower AOP is preferred. For CG, similarly user interaction delay could be considered [26.928]. They are measured in similar way but different requirement could be used. Whether and how to report AOP and/or user interaction delay for CG can be further discussed together with traffic model. |
| Ericsson | Companies are free to present other results. However, the benefits are unclear. |
| MTK | UPT and RU can be useful. |
| CMCC | We think RU can be reported for capacity evaluation. |
| Xiaomi | We think file transfer delay should be considered as round trip delay would have great impact on user experience especially for CG. |
| Vivo | To Performance metrics can be additionally reported, including - PER (file dropping rate) - UPT: UPT for each user can be collected and drawn in a UPT CDF curve, to show the throughput distribution among all users involved in the simulation - File transfer delay: CDF of file transfer delay - RU Given that UPT and RU are reported and they are both related to the SE, it seems no need to report SE for the evaluation. |
| Huawei, HiSilicon | The relevant performance metrics should reflect user experience in XR/CG services. However, if Q5 is not solved, then we are unclear about how to determine whether a UE is satisfied or not. Thus, we think it’s hard to determine which performance metrics are relevant. So we suggest to discuss Q5 first, i.e., RAN1 can try to discuss and agree some general principles on KPIs first. We think there is no need to discuss Q6 in this meeting, companies are free to report the performance metrics. |
| Nokia, NSB | We think that is premature to discuss other performance metrics that may be reported. The discussion of other metrics should be deferred after the discussion on traffic models and the set of requirements used to decide whether the UE is satisfied. |
| CATT | UPT and latency are bi-product. We will define the KPI. |
| AT&T | At least UPT/RU/Delay should be reported |
| Intel | We need some clarifications here – PER, PDB, UPT are requirements – correct ? but RU/SE can be reported ? – is this the right understanding ? |
| Facebook | The proposed metrics are fine. However, more importantly we should further define some metrics/KPI capable of reflecting the user experiences depending on the use cases and applications. |
| Samsung | Can be further discussed. Somewhat premature to do so before Q5 is resolved. |
| Sony | Can be further discussed |

It is proposed in [18] that XR capacity could highly depend on the arrival time offset of XR traffics among UEs. It may be useful to study XR capacity under various assumptions on traffic arrival offset among Ues.

Q7:  Whether and how to evaluate XR capacity under various assumptions on traffic arrival offset among Ues (e.g., random offsets, uniform offsets)?

Please share your comments on the Q7.

| Company | Comment |
| --- | --- |
| ZTE,Sanechips | Traffic arrival offset among Ues should be unified for calibration purpose. It would be beneficial to have some cross verification if uniform offset is assumed for evaluation purpose. |
| FUTUREWEI | Let’s wait until the traffic model and metric(s) for user experience are agreed. |
| LG | This aspect should be discussed based on further SA4 outcome on the traffic model |
| InterDigital | For the evaluations of capacity, both random offset (e.g. offset is uniformly distributed) and uniform offset can be considered. The case for using different offsets for traffic arrival may be useful for determining the tradeoff between capacity and UE power savings |
| QC | We think evaluation of different options of traffic arrival offset among UE’s is very useful. It can potentially motivate tight coordination/collaboration between gNB and application server if beneficial in terms of system capacity. We think following three cases can be evaluated for traffic arrival offset. - Case 1: traffic arrival offset is the same for all Ues. This is the worst case in terms of capacity. - Case 2: UE’s traffic arrival offset is randomly distributed among UE’s following uniform distribution in [0, P], where P is the DL frame arrival periodicity. - Case 3: UE’s traffic arrival offsets among Ues within a cell are evenly spaced within [0, P] where P is the DL frame arrival periodicity so that the minimum of traffic arrival offsets among Ues within a cell is maximized To reduce simulation effort, those options may be simulated only for a limited number of scenarios. The exact scenarios to be evaluated can be further discussed. |
| Ericsson | Random traffic arrival should be assumed as baseline. |
| MTK | This depends on NW implementation. Random offsets seems to be more reasonable. |
| CMCC | We think random offset (uniformly distributed) can be considered. |
| Xiaomi | Random/ Uniform distributed offset could be the baseline. But agree that evaluation with different assumptions on traffic arrival offset would be useful for potential collaboration between RAN and core network. |
| Vivo | To simplify the evaluation, random offsets for the traffic arrival time among different Ues are assumed. Other configuration for the traffic arrival offset can also be evaluated. |
| Huawei, HiSilicon | Premature to discuss. We think this is related to traffic model. So we suggest to discuss traffic model first. |
| Nokia, NSB | The offset among UE traffic arrivals can affect in a transitory period but given the likely high data rate requirements of the XR services, the impact of this offset should fade away. The decision of this parameter should be deferred after the decision of the traffic models appropriate to simulate XR and CG services. |
| CATT | The question of traffic arrival offset is not clear. For a stochastic process, it is statistic traffic inter-arrival among users. |
| AT&T | This will depend on the traffic model details |
| Intel | Our thinking is that this should be part of the traffic model discussion (in RAN1) |
| Facebook | Agree with AT&T. Once we finalize the traffic model details, we should be concluded the arrival offsets. |
| Samsung | Random offset with uniform distribution – although it can have a UE-common and a UE-dedicated component, it suffices to only consider a UE-dedicated component. |
| Apple | Traffic arrival at 120 Hz, 90 Hz, eg. Should be studied, random traffic arrival can be assumed. |
| Sony | The arrival process can be related to traffic model, and needs further discussion for down selection. |

### Evaluation assumptions

The evaluation assumptions are provided and discussed in [2][3][4][5][8][10][11][12][13][14][15][16][18]. To facilitate the evaluation and comparison of XR performance, it would be better to align as many assumptions as possible among companies.

According to the input, the evaluation assumptions are listed in Table 1 and Table 2.

Table 1 illustrates the simulation assumptions that are necessary for XR evaluation and for which there is a majority view among companies. So it is recommended to take the simulation assumptions in Table 1 for XR evaluation.

Table 1: Simulation assumptions for XR evaluation (Part 1)

| Parameter | Proposed value |  |
| --- | --- | --- |
|  | Indoor FR1/FR2 | Outdoor FR1/FR2 |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 | 21cells with wraparound |
| Carrier frequency | FR1: 3.5 GHz FR2: 28 GHz |  |
| Bandwidth | FR1: 100 MHz FR2: 400 MHz |  |
| Subcarrier spacing | FR1: 30 kHz FR2: 120 kHz |  |
| BS height | 3m | 25m |
| UE height | hUT=1.5 m |  |
| UE power | FR1: 23 dBm FR2: Maximum EIRP 43 dBm |  |
| BS noise figure | FR1: 5 dB FR2: 7 dB |  |
| UE noise figure | FR1: 9 dB FR2: 13 dB |  |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Realistic |  |
| UE speed | 3 km/h |  |
| MCS | Up to 256QAM |  |
| Target BLER | 10% |  |
| Max number of HARQ transmissions | 4 |  |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi | 3-sector antenna radiation pattern, 8 dBi |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, FR2: UE antenna radiation pattern model 1, 5dBi |  |

Proposal 1: Adopt the simulation assumptions in Table 1 for XR evaluation

Q8. Please share your comments on the proposal 1.

| Company | Comment |
| --- | --- |
| ZTE,Sanechips | We agree with the proposal 1. |
| FUTUREWEI | Should Target BLER be First Transmission Target BLER? |
| LG | Assumption on target BLER may not need to be fixed at this stage but can be discussed after requirements for each XR application are settled down. |
| DOCOMO | Share same view as LG. |
| InterDigital | We agree with the simulation assumptions in Table 1 |
| QC | We generally agree with the parameters in Table 1. - For FR2, EIRP < 31 dBm is deemed practical and preferred. UE EIRP of 43 dBm may lead to overly optimistic evaluation results. |
| Ericsson | The parameters are essentially OK, but why not use 4GHz and 30GHz as in 38.802? Max number of HARQ retransmissions does not need to be specified. |
| MTK | Current UE seldom supports one carrier > 100MHz. Also, SLS simulation for a > 100MHz carrier would be very time consuming. Therefore, we suggest to consider 100MHz as baseline for FR1/FR2 and companies can report CA settings or linear scaling the throughput if companies deem a larger BW result is preferred. For FR1 CA, we prefer 2CC CA for FR1 as follows: Total BW = 2 CCs x 100 MHz - CC1 = Lower band (licensed): 3.5 GHz (DDDSU DDSUU) - CC2 = Higher band1 (licensed): 4.9 GHz (SUUDD) or Higher band2 (un-licensed): 5GHz (DDDDD) We also think max number of HARQ retransmissions does not need to be specified. |
| CMCC | We think 4 or 4.9GHz can be used as carrier frequency for FR1. |
| Xiaomi | Agree |
| vivo | Support the proposal 1. |
| Huawei, HiSilicon | Since Q2 is discussing the deployment scenarios, we suggest to discuss Q2 first, and Q8 can be deferred. |
| Nokia, NSB | Considering the large number of simulation scenarios, simulation bandwidth should be reduced to limit the overall simulation time. The bandwidth should be large enough to capture system limits while keeping simulation time to a reasonable level. This would allow to scale up the results to larger bandwidth by an appropriate factor. We propose 40 MHz for FR1 and 80 MHz for FR2 as simulation bandwidth. The value of 80 MHz for FR2 has been discussed and agreed in another SI (R1-2007151) and could serve as a base for the discussion in this study. For FR2, we propose up to 256QAM for DL and up to 64QAM for UL. |
| CATT | 4 GHz is widely used for evaluation in NR and should be considered here. |
| AT&T | Agree with Ericsson. 4GHz and 30GHz should be assumed for FR1 and FR2 to align with other evaluation campaigns in RAN1 |
| Intel | Same view as Nokia in terms of actual simulated BW – and similar concept was adopted for IMT-2020 eval as well. However, this would become more clear once we have a picture of traffic model and PER/PDB requirements. |
| Samsung | Fine with the proposal and the suggestion by Nokia. Prefer to prioritize FR1. |
| Apple | Focus on FR1, and 20 MHz for bandwidth, 1Tx, 2Rx should also be considered. |

Table 2 illustrates the simulation parameters that are necessary for XR evaluation and are not converged yet. For these assumptions, options proposed by companies are given in the table. To reduce the simulation work, it is recommended for companies to consider to down-select from the options for the assumptions in Table 2. Furthermore, since power control, transmission scheme, PDCCH/DMRS overhead, CSI feedback mechanism and processing delay would affect the capacity performance, these assumptions need to be reported by companies

Table 2: Simulation assumptions for XR evaluation (Part 2)

| Parameter | Proposed value |  |
| --- | --- | --- |
|  | Indoor FR1/FR2 | Outdoor FR1/FR2 |
| UE distribution | 100% indoor | Option 1: 80% indoor, 20% outdoor (HW, vivo, CATT, ZTE, QC-FR1) Option 2: 20% indoor, 80% outdoor (vivo, Intel) Option 3: 100% outdoor (MTK, AT&T-FR2, QC-FR2) |
| Frame structure | FR1: Option1: DDDSU (HW, vivo, E///) Option2: DSUUD (CATT) Option3: DDDSUDDSUU (vivo, MTK) Option4: SUUDD (MTK) Option5: DDDUU (CMCC) Option6: DU (CMCC) Option7: FDD (MTK, IDC, Nokia) FR2: Option 1: DDDSU (vivo, MTK) Option 2: DSUUD (CATT) Note: S is 10:2:2 |  |
| BS antennas | FR1: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4) (vivo, CATT) (dH, dV) = (0.5, 0.5)λ FR2: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) (vivo) 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (16, 8, 2,1,1;1,1) (QC) (dH, dV) = (0.5, 0.5)λ | FR1: Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (12,8,2,1,1;4,8) (HW, vivo) Option 2: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,4,2,1,1;8,4) (ZTE) Option 3: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) (QC) Option 4: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (16,8,2,1,1;4,8) (CATT) Option 5: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,1,1,2;4,4) (MTK) Option 6: TxRU, (M, N, P, Mg, Ng; Mp, Np) = (2, 8, 2, 1, 1;2,8) (E///) (dH, dV) = (0.5λ, 0.8λ) FR2: Option 1: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1) (vivo) Option 2: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (32,8,2,1,1;1,1) (QC) (dH, dV) = (0.5λ, 0.5λ) |
| UE antennas | FR1: 2 or 4Tx/2 or 4Rx, (M, N, P, Mg, Ng; Mp, Np) = (1,2,1/2,1,1;1,2) (dH, dV) = (0.5, N/A)λ FR2: 4 Tx/4Rx, Option 1: (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,2;1,2) (MTK) Option 2: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) (vivo) Option 3: {2, 2, 2} per panel. Number/location of panels: 3 panels (left, right, and top) (QC) (dH,dV) = (0.5, 0.5)λ The polarization angles are 0° and 90° |  |
| Downtilt | Outdoor: Option 1: 6 degree (ZTE, QC) Option 2: 14 degree (MTK, E///) Option 3: 100 (Intel) Option 4: 90° in GCS (pointing to horizontal direction) (vivo) Indoor: Option 1: 0 degree (MTK) Option 2: 180° in GCS (pointing to the ground) (vivo) |  |
| BS power | FR1: Alt1: 24dBm/20MHz (vivo, CATT, QC) Alt2: 30dBm (ZTE) FR2: Alt1: Maximum EIRP 58dBm (vivo) Alt2: 23dBm (QC) | FR1: Alt1: 46dBm (IDC) Alt2: 49dBm (E///) Alt3: 44dBm/20MHz (HW, CATT, ZTE, MTK, Intel, QC) Alt4: 53dBm (vivo) FR2: Alt1: Maximum EIRP 73dBm (vivo) Alt2: 37dBm (MTK) Alt3: 28dBm (QC) |
| Power control parameter | Companies should report |  |
| Transmission scheme | Companies should report, such as Type I/II codebook, rank assumption |  |
| Scheduler | MU-MIMO PF scheduler, other scheduler is up to companies report |  |
| CSI Feedback | Realistic Companies should report CSI feedback delay, CSI report periodicity, whether using CSI quantization, CSI error model or not, and etc. |  |
| PHY processing delay | UE Capability #1 Companies should report gNB processing delay, e.g. DL NACK to retransmission delay, UL previous transmission to current transmission delay and etc. |  |
| PDCCH overhead | Companies should report |  |
| DMRS overhead | Companies should report |  |

Proposal 2: Regarding the UE distribution for outdoor scenario, down-select from the following options for XR evaluation.

- For outdoor scenario:

  - FR1:

    - Option 1: 80% indoor, 20% outdoor

    - Option 2: 20% indoor, 80% outdoor

    - Option 3: 100% outdoor

  - FR2:

    - 100% outdoor

Q9. Please share your comments on the proposal 2.

| Company | Comment |
| --- | --- |
| ZTE,Sanechips | We prefer 80% indoor and 20% outdoor of FR1 in case of outdoor scenario. |
| FUTUREWEI | Prioritize FR1. At least Option 1 is simulated. |
| LG | For FR1, option1 can be prioritized. |
| DOCOMO | Option 1 for FR1 should be simulated. |
| InterDigital | We think i) FR1 Option 1 and Option 2 and ii) FR2 100% indoor and outdoor should be prioritized for evaluations |
| QC | We support option 1. |
| Ericsson | Support option 1 |
| MTK | We prefer Option 3. Also prefer to prioritize FR1 . |
| CMCC | Support option 1 for FR1. |
| Xiaomi | We support option 1 for FR1 |
| vivo | For outdoor scenario, we prefer the following option - FR1: - Option 1: 80% indoor, 20% outdoor |
| Huawei, HiSilicon | Support FR1 Option 1. |
| Nokia, NSB | The decision on the UE distribution should be deferred after the agreement on the prioritization of XR applications and deployments discussed on Q2. If only AR applications are evaluated in the outdoor deployments, we propose to consider the following options for UE distribution: - FR1: 100% outdoor (Option 3) - FR2: 100% outdoor |
| CATT | Option 1 for FR1 |
| AT&T | Option 1 for FR1 is OK. Again we do not agree to any prioritization of FR1 over FR2 |
| Intel | for FR2 it makes sense to use 100% outdoor for FR1 Option 1 is typical assumption – we can keep it as default |
| Facebook | Support option 1 |
| Samsung | Prioritize FR1, option 1. |
| Apple | Both Option 1 and Option 2 can be considered. |

Proposal 3: Regarding the frame structure, down-select from the following options of FR1 and FR2 for XR evaluation.

- FR1:

  - Option1: DDDSU

  - Option2: DSUUD

  - Option3: DDDSUDDSUU

  - Option4: SUUDD

  - Option5: DDDUU

  - Option6: DU

  - Option7: FDD

- FR2:

  - Option 1: DDDSU

  - Option 2: DSUUD

Note: S is 10:2:2

Q10. Please share your comments on the proposal 3.

| Company | Comment |
| --- | --- |
| ZTE,Sanechips | Option1 and option3 of FR1 should be prioritized for the evaluation of XR and CG applications. |
| FUTUREWEI | Prioritize FR1. Focus on Option 1. In terms of SLS performance, Option 2 and Option 4 should be the same and at most one may be considered. |
| LG | For FR1, two options can be chosen to consider different DL-UL ratios, but may not need to be decided in this meeting. |
| DOCOMO | FR1 should be prioritized. Option1 (1st priority) or Option 3 for FR1. Regarding FR2, Option1 should be considered. |
| InterDigital | We think i) FR1 Option 1, Option 2, Option 3 and ii) FR2 Option 1, Option 2 should be prioritized for evaluations. |
| QC | For FR1, we are okay with option 1 or 5. For FR2, option 1 is preferred. |
| Ericsson | Option 1 is preferred for FR1 and FR2. |
| MTK | For FR2, we prefer Option 1. For FR1, we suggest 2CC CA for FR1 as follows: Total BW = 2 CCs x 100 MHz; - CC1 = Lower band (licensed): 3.5 GHz (DDDSU DDSUU, Option 3) CC2 = Higher band1 (licensed): 4.9 GHz (SUUDD, Option 4) or Higher band2 (un-licensed): 5GHz (DDDDD) |
| CMCC | For FR1, We support option 1,5,6. We think Option 6 can be change to DS. |
| Xiaomi | Agree to prioritize FR1, and support option 1 for FR1. |
| vivo | It is necessary to limit the number of options for frame structure so that the simulation work load can be controllable. Following options for FR1 and FR2 can be adopted - FR1 - Option1: DDDSU (S: 10D:2G:2U) - Option2: DDDSUDDSUU (S: 10D:2G:2U) - FR2 - DDDSU (S: 10D:2G:2U) |
| Huawei, HiSilicon | Support FR1 Option 1. |
| Nokia, NSB | We think that FDD (option 7) for both UL and DL can be used for FR1, whereas TDD Option 1 (DDDSU) should be considered for FR2. However, we underline that a decision on TDD frame structure should be deferred after decisions (i) on the traffic models for XR applications and (ii) on the PHY processing delay. Indeed, XR performance can be affected by the delay introduced by TDD frame design (e.g., higher RTT that decreases XR performance). Furthermore, the PHY processing delay affects the design of the frame structure. |
| CATT | Option 1 or 5 for FR1 |
| AT&T | Option 1 for both FR1/FR2 |
| Intel | We also feel its better to understand delay requirements and traffic model first |
| Facebook | For FR1, we think both option 1 and option 5 should be included. The reason option 5 is mainly due to the increasing uplink traffic in lots of AR sharing applications as in SA4 TR 26.928. FR2, Option 1 |
| Samsung | FR1, option 1 or option 5. |
| Apple | Option 1 for FR1, Option 5 for FR1 |

Proposal 4: Regarding the BS antennas, further discuss the assumptions and down-select from the following options for XR evaluation.

- For indoor scenario:

  - FR1:

    - 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4)

    - (dH, dV) = (0.5, 0.5)λ

  - FR2:

    - 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8)

    - 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (16, 8, 2,1,1;1,1)

    - (dH, dV) = (0.5, 0.5)λ

- For outdoor scenario:

  - FR1:

    - Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (12,8,2,1,1;4,8)

    - Option 2: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,4,2,1,1;8,4)

    - Option 3: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8)

    - Option 4: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (16,8,2,1,1;4,8)

    - Option 5: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,1,1,2;4,4)

    - Option 6: TxRU, (M, N, P, Mg, Ng; Mp, Np) = (2, 8, 2, 1, 1;2,8)

(dH, dV) = (0.5λ, 0.8λ)

  - FR2:

    - Option 1: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1)

    - Option 2: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (32,8,2,1,1;1,1)

(dH, dV) = (0.5λ, 0.5λ)

Q11. Please share your comments on the proposal 4.

| Company | Comment |
| --- | --- |
| ZTE,Sanechips | 32TxRU for indoor scenario and 64TxRU for outdoor scenario in FR1 will be considered is preferred the evaluation. |
| FUTUREWEI | Prioritize FR1 and focus on FR1 Option 3 outdoor. |
| LG | For indoor FR2, 2 TxRU can be more general assumption. |
| DOCOMO | FR1 should be prioritized and Option 3 is preferred for FR1 outdoor. |
| InterDigital | For Indoor scenario, we think the given antenna configuration for FR1 is good while for FR2, it would be good to prioritize the configuration 2 TxRU. For Outdoor scenario, we think i) For FR1, Option 2, Option 5 and Option 6 should be prioritized. ii) For FR2 both option 1 and option 2 can be prioritized. |
| QC | For indoor scenario, - In the summary text for indoor - FR2 case, Option 1 and Option 2 were missing - For FR2, we prefer Option 2: 2TxRU, (M, N, P, Mg, Ng; Mp, Np) = (16, 8, 2,1,1;1,1) (dH, dV) = (0.5, 0.5)λ For outdoor scenario, - FR1: option 3 is preferred. - FR2: option 2 is preferred. |
| Ericsson | Larger antennas provide better performance. However, the focus of the XR work should not be to investigate advanced MIMO schemes. Therefore, we propose a conservative antenna layout: Indoor FR2: 2TxRU Outdoor FR1: Option 6 (32TxRU) Outdoor FR2: Option 1 |
| MTK | For FR1, we prefer (8,8,2,1,1;2,8) which is most close to Option 3. For FR2, we prefer Option 1 or (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,1,1;1,1). For FR2, it would be good to prioritize the configuration 2 TxRU. |
| CMCC | For outdoor FR1: Option 1 is preferred. |
| vivo | We prefer the following option - For indoor scenario: - FR1: - 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4) - (dH, dV) = (0.5, 0.5)λ - FR2: - 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) - For outdoor scenario: - FR1: - Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (12,8,2,1,1;4,8) (dH, dV) = (0.5λ, 0.8λ) - FR2: - Option 1: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1) (dH, dV) = (0.5λ, 0.5λ) |
| Huawei, HiSilicon | Fine to FR1 outdoor scenario, Option 1 |
| Nokia, NSB | We think that for the evaluation in FR1 we should use a relatively small antenna array panel and a larger one for FR2. Furthermore, for indoor scenario (InH) we could further down-select the possible options, since UEs are expected to have good channel conditions. We propose to consider the following options for BS antenna configurations and open to discuss a down selection of these options. - Indoor scenario: - FR1: Option 1) 4 Tx/4 Rx, (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 2), (dH, dV) = (0.5, 0.5)λ, Option 2) 8 Tx/8 Rx, (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 4), (dH, dV) = (0.5, 0.5)λ - FR2: Option1) 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8), (dH, dV) = (0.5, 0.5)λ – Option 2) 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4), (dH, dV) = (0.5, 0.5)λ - Outdoor scenario: - FR1: Option 1) 4 Tx/4 Rx, (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 2), (dH, dV) = (0.5, 0.5)λ, Option 2) 8 Tx/8 Rx, (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 4), (dH, dV) = (0.5, 0.5)λss - FR2: Option1) 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8), (dH, dV) = (0.5, 0.5)λ – Option 2) 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4) |
| CATT | Option 4 for FR1 outdoor. |
| AT&T | No prioritization of FR1 over FR2. Both should be considered. For FR1: Option 3 for outdoor For FR2: 2TxRU configuration: Option 2 for indoor, Option 1 for outdoor |
| Intel | We need to understand how much TXRU configuration is playing a role in user-satisfaction – this is not clear yet. We provide tentative view below: for FR1 outdoor: - massive MIMO case: Option 2 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,4,2,1,1;8,4) - current typical deployment case: 4 TXRU if it makes sense and user-satisfaction is acceptable for FR1 indoor: - 32 TXRU square array for FR2, we can use 2 TXRU/single-panel for both outdoor (tall) and indoor (square) |
| Samsung | Option 3 for FR1 outdoor. |
| Apple | Considering for XR evaluation, as packet arrivals are not so frequent, the simulation time can be long in order to generate reliable statistics. Increasing the TxRU number can substantially prolong the simulation time. For indoor, 2Tx from gNB, and for outdoor cases, a relatively small number, e.g. 8Tx or even 4Tx can be considered. |

Proposal 5: Regarding the UE antennas, adopt the following assumption for FR1 and down-select from the following options for FR2 for XR evaluation.

- FR1:

  - 2 or 4Tx/2 or 4Rx, (M, N, P, Mg, Ng; Mp, Np) = (1,2,1/2,1,1;1,2)

(dH, dV) = (0.5, N/A)λ

- FR2: 4 Tx/4Rx,

  - Option 1: (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,2;1,2)

  - Option 2: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2)

  - Option 3: {2, 2, 2} per panel. Number/location of panels: 3 panels (left, right, and top)

(dH,dV) = (0.5, 0.5)λ

The polarization angles are 0° and 90°

Q12. Please share your comments on the proposal 5.

| Company | Comment |
| --- | --- |
| ZTE,Sanechips | For FR1, 4Tx/4Rx, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2) should be considered. |
| FUTUREWEI | Prioritize FR1. And focus on FR1 4T4R. |
| LG | For FR1, 2TX should be included. For FR2, option 3 may not be appropriate as general assumption. |
| DOCOMO | FR1 should be prioritized. |
| InterDigital | We think i) the proposed configuration for FR1 is good. ii) For FR2, we think Option 1 and Option 2 should be prioritized. |
| QC | For FR1, - We prefer 2Tx/4Rx, (M, N, P, Mg, Ng; Mp, Np) = (1,2,1/2,1,1;1,2). Both P=1 or 2 should be supported depending on the number of antennas. For FR2, - In the above text for FR2, prefer Option 3. Option 3 is preferred. |
| Ericsson | More UE antennas provide better performance. However, the focus of the XR work should not be to investigate benefits of more UE antennas. There we propose conservative numbers: For FR1: 1Tx and 2Rx (4Rx is also OK) For FR2: 1Tx and 2Rx |
| MTK | Agree with Ericsson, for FR1: 1Tx and 2Rx seems better. For FR2, we prefer Option 1. Also prefer to prioritize FR1. |
| CMCC | We propose to prioritize FR1 2T4R. |
| Xiaomi | We agree FL proposal on FR1 and open for FR2. |
| vivo | - FR1: - 2 or 4Tx/2 or 4Rx, (M, N, P, Mg, Ng; Mp, Np) = (1,2,1/2,1,1;1,2) (dH, dV) = (0.5, N/A)λ - FR2: 4 Tx/4Rx, - Option 2: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) The polarization angles are 0° and 90° |
| Huawei, HiSilicon | Support FR1, 4T4R |
| Nokia, NSB |  FR1: 2 or 4Tx/4Rx, (M, N, P, Mg, Ng; Mp, Np) = (1,2,1/2,1,1;1,2) (dH, dV) = (0.5, N/A)λ - 2 Rx can be considered if lower frequency bands where 2Rx is applicable are used.  FR2: Option 2: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) |
| CATT | Support FR1 2T4R |
| AT&T | No prioritization of FR1 over FR2. FR1: (1,2,2,1,1;1,2) FR2: option 2 or option 3 (need to align it with other evaluation campaigns in NR) |
| Intel | FR1 2T4R and FR2 2T2R |
| Facebook | FR1: 2Tx/4Rx or 1Tx/2Rx per many companies suggested FR2: ok with more conservative antenna numbers per Ericsson suggestion. However, option 3 should be included as a practical scenario as well. |
| Samsung | FR1: 2Rx and 4Rx – 1Tx with higher priority than 2Tx. |
| Apple | 1Tx and 2Rx should also be considered, they are more practically for XR. |

Proposal 6: Regarding the downtilt, down-select from the following options for FR1 and FR2 for XR evaluation.

- Outdoor:

  - Option 1: 6 degree

  - Option 2: 14 degree

  - Option 3: 100

  - Option 4: 90° in GCS (pointing to horizontal direction)

- Indoor:

  - Option 1: 0 degree

  - Option 2: 180° in GCS (pointing to the ground)

Q13. Please share your comments on the proposal 6.

| Company | Comment |
| --- | --- |
| ZTE,Sanechips | FR1: 90 degrees for indoor and 6 degree for outdoor |
| FUTUREWEI | Prioritize FR1. And focus on FR1 Option 1. |
| LG | This should depends on other scenarios such as antenna heights. |
| InterDigital | - For FR1, we think Option 2 & Option 4 can be prioritized - For FR2, we think Both Options 1 & Options 2 need to be evaluated |
| QC | For FR1 - Prefer option 1. For FR2, For maximum coverage, for outdoors a vertical panel (0 degree) is preferred and for indoors, the panel is expected to be horizontal (90 degrees) (i.e. pointing to the ground). |
| Ericsson | For outdoor, the appropriate tilt would depend on the scenario. For indoor hotspot, 90 degree tilt would be appropriate |
| MTK | For FR1, we prefer Option 2. For FR2, we prefer Option 1. |
| vivo | - Outdoor: - Option 1: 6 degree - Indoor: - Option 1: 0 degree |
| Huawei, HiSilicon | Fine to FR1. Detailed options may depend on scenarios. |
| Nokia, NSB | The decision on the antenna downtilt should be deferred after the decisions on the deployment scenarios and on the antenna configuration, since this parameter can be optimized based on the deployment scenario. Furthermore, we think that a downtilt should depend also on the deployment scenario, since ISD for InH and UMa is notably different (20m and 200m, respectively). |
| CATT | Antenna down tilt is a parameter for coverage tuning in the field deployment. |
| AT&T | Agree with Ericsson that the tilt will depend on the scenario for outdoor. For indoor, pointing to the ground. |
| Intel | Outdoor it depends on ISD and antenna configuration, indoor we should aim for ceiling mounted (pointing to ground) |

Proposal 7: Regarding the BS Tx power, down-select from the following options for XR evaluation.

- For indoor scenario:

  - FR1:

    - Alt1: 24dBm/20MHz

    - Alt2: 30dBm

  - FR2:

    - Alt1: Maximum EIRP 58dBm

    - Alt2: 23dBm

- For outdoor scenario:

  - FR1:

    - Alt1: 46dBm

    - Alt2: 49dBm

    - Alt3: 44dBm/20MHz

    - Alt4: 53dBm

  - FR2:

    - Alt1: Maximum EIRP 73dBm

    - Alt2: 37dBm

    - Alt3: 28dBm

Q14. Please share your comments on the proposal 7.

| Company | Comment |
| --- | --- |
| ZTE,Sanechips | For indoor scenario, consider Alt 1/Alt 2 depending on the system bandwidth |
| FUTUREWEI | Prioritize FR1. Support at least outdoor FR1 Alt3. |
| DOCOMO | FR1 should be prioritized. Alt.1 for FR1 indoor and Alt.3 for FR1 outdoor would be preferable. |
| InterDigital | We think that for indoor scenario, FR1-Alt 1 and FR2-Alt 1 should be prioritized. For outdoor scenario, we think that FR1 – Alt2 & Alt4 and FR2-Alt1 can be prioritized. |
| QC | For indoor scenario: - FR1: 24dBm for 100MHz - FR2: Alt2 is preferred, Alt1 is acceptable. For outdoor scenario: the BS tx power depends on scenarios; UMi and UMa. For FR1 - For UMi: 44dBm for 100MHz - For UMa: 49dBm for 100MHz For FR2 - For UMi: Alt3 is preferred, Alt1 is acceptable For UMa: Alt3 is preferred, Alt1 is acceptable |
| Ericsson | Follow 38.802, Table A.2.1-1 |
| MTK | For indoor FR1, we prefer Alt 1. For indoor FR2, we prefer Alt 2. For outdoor FR1, we prefer Alt3. For outdoor FR2, we prefer Alt2. |
| CMCC | For indoor scenario: - FR1: 24dBm for 100MHz For outdoor scenario: - FR1: 53dBm for 100MHz |
| vivo | - For indoor scenario: - FR1: - 24dBm for 100MHz - FR2: - Alt1: Maximum EIRP 58dBm - For outdoor scenario: - FR1: - Alt4: 53dBm - FR2: - Alt1: Maximum EIRP 73dBm |
| Huawei, HiSilicon | FR1 outdoor scenario, Alt3. |
| Nokia, NSB | Regarding the BS transmission power, we consider the following options: - Indoor scenario: - FR1: 24 dBm / 20 MHz - FR2: 23 dBm - Outdoor scenario: - FR1: 43 dBm / 20 MHz - FR2: 43 dBm |
| CATT | For indoor scenario: - FR1: 24dBm for 100MHz For outdoor scenario: - FR1: 46 dBm for 100MHz |
| AT&T | No prioritization of FR1 over FR2. For Indoor: FR1: 24dBm/100MHz, FR2: Alt. 1. For outdoor: depends on scenario (e.g. Umi or Uma): For Umi: 44dBm/100MHz, for Uma: 49dBm/100MHz. |
| Samsung | Prioritize FR1. OK with corresponding values suggested by QC. |

For the following assumptions in Table 2, they are important for the XR evaluation and may be related to the implementation/configuration. Hence, they should be reported by company with detailed assumptions for the evaluation.

Proposal 8: Adopt the following simulation assumptions in Table 2 for XR evaluation.

| Power control parameter | Companies should report |
| --- | --- |
| Transmission scheme | Companies should report, such as Type I/II codebook, rank assumption |
| Scheduler | MU-MIMO PF scheduler, other scheduler (e.g., delay aware scheduler) is up to companies report |
| CSI Feedback | Realistic Companies should report CSI feedback delay, CSI report periodicity, whether using CSI quantization, CSI error model or not, and etc. |
| PHY processing delay | UE Capability #1 Companies should report gNB processing delay, e.g. DL NACK to retransmission delay, UL previous transmission to current transmission delay and etc. |
| PDCCH overhead | Companies should report |
| DMRS overhead | Companies should report |
| SRS | Companies should report |

Q15. Please share your comments on the proposal 8.

| Company | Comment |
| --- | --- |
| ZTE,Sanechips | We agree with the proposal 8. |
| FUTUREWEI | “CSI feedback” may be changed to “CSI acquisition”, and CSI feedback and/or SRS may be used. |
| LG | Scheduler assumption can be fully up to each companies report. |
| InterDigital | We support the simulation assumptions in the proposal |
| QC | In scheduler, we think delay aware needs to be evaluated as one option in capacity evaluation. The XR traffic typically has tight delay budget and therefore the role of scheduling algorithm may be critical. Although scheduling algorithm is up to implementation, a study on delay aware scheduler can be very useful which can potentially motivate tighter collaboration/coordination between gNB and application/edge server, where some enhancements to specifications may be needed. |
| Ericsson | Support. Question for clarification: UE capability #1, does that mean PDSCH processing capability 1? |
| MTK | For PDCCH overhead, we suggest to consider PDCCH region of 1/3 symbols at beginning of a slot. |
| CMCC | Support the proposal. |
| vivo | Support the proposal 8. |
| Huawei, HiSilicon | SU/MU-MIMO PF scheduler should be the baseline in order to provide a performance reference for different companies. Otherwise, the performance may be difficult to be compared due to different schedulers. |
| Nokia, NSB | We propose the following parameters for proposal 8: Power control parameter; Indoor: Open Loop, α=1, P0= -61 dBm Outdoor: Open Loop, α=1, P0= -103 dBm. Transmission scheme; Indoor: - DL: dynamic rank adaptation (up to 2 streams per UE) - UL: rank 1 Outdoor: - DL: dynamic rank adaptation (up to 4 streams per UE) - UL: rank 1. Scheduler; Latency and frequency aware scheduler. CSI Feedback; Realistic CQI feedback delay: 2ms CQI report periodicity: 2ms CQI quantization step: 1 dB, CQI error model: no error. PHY processing delay; UE Capability #2 (URLLC) Companies should report gNB processing delay, e.g. DL NACK to retransmission delay, UL previous transmission to current transmission delay and etc.. PDCCH overhead; Companies should report PDCCH overhead. DMRS overhead; Companies should report DMRS overhead. SRS; Companies should report SRS settings. |
| CATT | OK with the proposal |
| AT&T | Support the proposal |
| Intel | OK |
| Samsung | Support the proposal |

Q16. Please share additional comments if any on Table 2.

| Company | Comment |
| --- | --- |
| AT&T | Any restrictions on PDSCH mapping type A/B? |
| Samsung | It is understood that, if requested, more assumptions may need to be reported. |

Q17: In addition to the assumptions in Table 1 and Table 2, are there any assumptions which are necessary to define for XR evaluation?

Please share your comments on the Q17.

| Company | Comment |
| --- | --- |
| InterDigital | We think that channel model assumptions might be missing. TDL-A/B/C/D/E or CDL-A/B/C/D |
| QC | In general, the simulation assumptions between capacity evaluation (which does not require power evaluation at all) and power evaluation (where capacity should be evaluated subject to a capacity constraint) need to be the same. In case different parameters need to be considered, they should be reported with the results. |
| Nokia, NSB | The assumptions regarding the transport protocol(s) used should be decided at some point, as this will affect the simulation results. |

The following simulation assumptions are proposed by one or only a few companies for XR evaluation. More clarifications on whether and how to consider these simulation assumptions for the XR evaluation are needed.

- Beam related operation, such as beam update mechanism, beam activation delay, beam metric

- Others, e.g. RLC, network layer setting, core network delay

FL’s comment: For the assumptions that may be related to traffic model, they can be discussed with traffic model after there is more input from SA4.

Q18: Whether or not to consider the following simulation assumptions for XR evaluation?

- Beam related operation, such as beam update mechanism, beam activation delay, beam metric

- Others, e.g. RLC, network layer setting, core network delay

Please share your comments on the Q18.

| Company | Comment |
| --- | --- |
| ZTE,Sanechips | Prefer not to consider beam related operation. In terms of RLC, network delay consideration, this had better be discussed with traffic model and thus it's suggested this discussion, if needed, take place during next meeting when SA4 outcome is supposed to be available. |
| FUTUREWEI | Prefer not to consider these aspects with limited TU for this SI |
| LG | This details can be up to further discussion and may be up to each company’s report in the end. |
| DOCOMO | Share the same view as ZTE and FUTUREWEI. |
| InterDigital | We think the additional assumptions related to UP and CP delay in CN (e.g. between edge function/server and RAN) that affects the end-to-end performance (e.g. user experience and capacity) may be considered in the evaluations |
| QC | In our view, RLC, network layer setting, and core network delay are not explicitly evaluated. Rather, it can be captured in latency requirements for RAN transmission. |
| Ericsson | Beam layout would be sufficient, but dynamic mechanisms are unnecessary. RLC mode needs to be specified – we assume RLC UM. Delays in transport network and CN needs to be stated, perhaps assumed to be 0. Note that quality requirements from SA4 will consider total delay, including delays in transport and core network. |
| MTK | No. Companies can report the used values if necessary. |
| Xiaomi | Agree with ZTE view. NW delay can be considered in traffic model discussion. |
| vivo | No need to consider these simulation assumptions for evaluations. |
| Huawei, HiSilicon | No need to consider beam related operation. |
| Nokia, NSB | Decision related to beam related operations, such as beam update mechanism, beam activation delay, beam metric should be deferred after the decision on the deployment scenarios and antenna configurations. L2 (MAC, RLC, PDCP) and core network parameters affects the performance of TCP-like connections which are used for CG and on-demand VR applications. The decision L2 and core network settings (e.g., core network delay) should be deferred after the decision on traffic models for XR applications. |
| Intel | It can be left to companies to report |
| Samsung | Agree in principle with previous comments to not consider such aspects. Companies can, of course, still report. |

Q19. Please share any other comments if any on capacity evaluation for XR and CG.

| Company | Comment |
| --- | --- |
| MTK | Why not just align to IMT 2020 settings (dense urban, InH …) of System level simulation assumptions as in 38.840? Discussing the various simulation parameter possibilities can be time consuming. |
| Intel | Agree with MTK – we can start from IMT and see what changes are necessary. Another question is whether calibration campaign is planned for this SID ? |

# Summary of Round 1 Email Discussion for Others

## XR Applications

Table 1 captured the views on XR applications of interest from different sources.

Table 1 Views on XR Applications

| Source | View |
| --- | --- |
| [1] | Proposal 1: FS_NR_XR_eval considers selecting one application (e.g., VR2: “Split Rendering: Viewport rendering with Time Warp in device”, or CG: Cloud Gaming) as the focus of study, at least in the initial stage of the study item. |
| [2] | Proposal 1: For VR1, VR2 and CG applications, downlink traffic is studied and evaluated with higher priority. |
| [3] | Proposal 1: RAN1 studies VR2, AR1 and CG applications as high priority. |
| [5] | Proposal 1: The applications of interest of the study item include at least the following listed in RP-193241. - VR1: Viewport dependent streaming - VR2: Split Rendering: Viewport rendering with Time Warp in device - AR1: XR Distributed Computing - AR2: XR Conversational - CG: Cloud Gaming. - Up to one single (Rate, PDB, PER) requirement is evaluated for each application of interest - For delivery of CG applications, generalized split rendering architecture and its relevant traffic characteristics are considered. |
| [7] | Proposal 1: Prioritize the AR applications/use cases from [1] for the XR SI. Also consider cloud gaming. |
| [8] | Proposal 1: Confirm the XR and cloud gaming applications in the SI for XR evaluations |
| [12] | Proposal: Classify AR1 and AR2 as essential applications in the RAN1 Rel-17 study item for evaluation of XR for NR. |
| [13] | Proposal 1: Prioritize the evaluation of CG over AR/VR considering the highest business value of CG among VR/AR/CG. Proposal 2: Prioritize the evaluation of AR over VR considering the higher popularity of AR service. If VR is considered, prioritize indoor deployment scenario, e.g. Indoor Hotspot (InH). Proposal 3: Adopt the following study priorities on applications and deployment scenarios for R17 XR/CG: |
| [16] | Proposal 1 RAN1 to confirm that the VR1/VR2/AR1/AR2/Cloud gaming applications are of interest for Rel-17 study on XR evaluation for 5G NR and determine which of the use cases would need to be prioritized for the evaluation study. Proposal 2 In the XR evaluation SI, RAN1 to treat cloud gaming with first priority, AR use cases with second priority, and VR use cases with third priority. |
| [17] | Proposal 1: - CG and VR can be discussed in the RAN1#103-e meeting considering the availability of traffic models in SA4 TR. As for AR, it needs to wait for SA4 progress. |
| [18] | Proposal 1: RAN1 considers all the XR applications described in the RAN XR SID for evaluation (i.e., VR, AR, and CG). More details, e.g., prioritization and/or down-selection of them can be further discussed once the outcome of SA4 study on XR traffic model becomes available. |
| [9] | Consider XR applications including studies ITT4RT and 5GSTAR, and XR_Traffic |

Summary

Companies have shown their views on application of interests. Following table captures the indicated priorities of applications of interest from sources. “1” = highest priority, “2”=second highest priority, etc.

|  | VR1 | VR2 | AR1 | AR2 | CG |
| --- | --- | --- | --- | --- | --- |
| [3] | 2 | 1 | 1 | 2 | 1 |
| [13] |  |  | 3 | 2 | 1 |
| [1] |  | 1 |  |  | 1 |
| [16] | 3 | 3 | 2 | 2 | 1 |
| [17] | 1 | 1 | 2 | 2 | 1 |
| [7] |  |  | 1 | 1 | 1 |
| [12] |  |  | 1 | 1 |  |

FL Proposal 1: RAN1 confirms that diverse applications of VR1/2, AR1/2 (including low rate AR), CG [TR26.928] are of interest for evaluation. Prioritization/down selection of these applications for evaluation, if needed, is to be discussed after detailed traffic model and evaluation assumptions are determined.

Question 1. Please share your comments on Proposal 1 if any.

| Company | View |
| --- | --- |
| QC | All VR1/VR2/AR1/AR2(including low rate AR)/CG are interesting to us. In particular, low rate AR is interesting application to study because it may be one of the most relevant applications in the near term market. Any decision related to the prioritization or down-selection of applications for evaluation should be made after we have a good understanding, in particular on traffic model. |
| MTK | We are fine with the FL proposal. However, given that CG is marked as “1” for the most companies, we prefer to further prioritize CG, and choose 1 or 2 applications from AR1/2, VR1/2 later. |
| ZTE, Sanechips | We don't think low rate AR should be explicitly captured here given the traffic related discussion is still pending at this moment. Moreover, given the prioritization is still open in the proposal, it's suggested changing evaluation to study to avoid confusion. The preferred change to the proposal would be: FL Proposal 1: RAN1 confirms that diverse applications of VR1/2, AR1/2 (including low rate AR), CG [TR26.928] are of interest for evaluation study. Prioritization/down selection of these applications for evaluation, if needed, is to be discussed after detailed traffic model and evaluation assumptions are determined. |
| Nokia, NSB | We agree that the selection of the applications should be done after the understanding of the appropriate traffic models. |
| FUTUREWEI | We also think prioritization and down-selection should wait after better understanding of traffic model is reached. |
| Apple | We think prioritization and down-selection should be done later. Recall studies from SA4 are still ongoing. |
| DOCOMO | We are fine with the FL proposal. Down-selection of applications should be done after we have better understanding of the traffic models. |
| LG | We are fine with FL’s proposal. |
| InterDigital | We think all VR1/2, AR1/2 and CG applications should be considered for the evaluations. Whether to prioritize/deprioritize any of the applications may be discussed once the details on traffic models are available from SA4. |
| Intel | We think VR1/2, AR1/2, CG should be considered in this study. We think the second sentence is not necessary FL Proposal 1: RAN1 confirms that diverse applications of VR1/2, AR1/2 (including low rate AR), CG [TR26.928] are of interest for evaluation study. Prioritization/down selection of these applications for evaluation, if needed, is to be discussed after detailed traffic model and evaluation assumptions are determined. |
| CMCC | Support FL proposal. There is no need for down-selection at this point. |
| Ericsson | We are fine with deferring prioritization/down selection to a later stage. |
| vivo | We agree with FL’s suggestion. |
| Xiaomi | We are fine with the FL proposal, although we think VR1 can be deprioritized. |
| Huawei, HiSilicon | We think that the five XR applications described in the SID (i.e., VR1/VR2/AR1/AR2/CG) are of interest. As for “low rate AR”, some company [18] mentioned it is currently being studied in SA4. According to the following Note in chairman notes, RAN1 will not discuss this application in this meeting. - NOTE: SA4 has ongoing work in the XR area, RAN1 will not address these SA4 aspects but will wait for SA4’s outcome For the prioritization of five applications, since the traffic model of each application is unclear, it is premature to prioritize among them. We suggest to study the traffic models for all 5 applications so that RAN1 can have better understanding of each application. After that, we may pick some representative cases for performance evaluation considering the simulation workload. So we suggest the following changes to Proposal 1: Proposal 1: RAN1 confirms that diverse applications of VR1/2, AR1/2 (including low rate AR), CG [TR26.928] are of interest for evaluationstudy. Prioritization/down selection of these applications for evaluation, if needed, is to be discussed after detailed traffic model and evaluation assumptions are determined. |
| Nokia, NSB2 | We agree that the selection of the applications should be done after the understanding of the appropriate traffic models. If the choice is to be made straightaway, our 1st preference if GC, the 2nd preference is VR, and the 3rd is AR. Furthermore, we think that low-rate AR is less interesting than high-rate AR and it could be excluded from the study. |
| CATT | VR1/2, AR1/2 and CG are interested applications. Since all these applications could apply to different field, RAN1 should target generic modelling of these 5 traffic types for Uu evaluation. |
| Facebook | In general, we agree with the FL’s proposal. We also want to echo the importance of low rate AR for near term market. Although not specifically mentioned in the SID, we don’t see there is a restriction on this particular use case. This is just a subset of AR and it’s just normal procedure for working group to further discuss the detail. |
| Samsung | Fine with proposal. Realistically, there has to be prioritization. AR (including ‘low rate’) is preferred. |

## Traffic Model

### DL Traffic Model

Table 2 captures the views from different sources on DL traffic model.

Table 2 View on DL Traffic Model

| Source | View |
| --- | --- |
| [1] | Proposal 2: RAN1 notifies SA4 that RAN1 prefers a statistical XR traffic model in FS_NR_XR_eval studies and kindly asks SA4 to confirm that such a statistical XR traffic model can be provided as an outcome of SA4’s related study item on XR. |
| [2] | Proposal 3: For VR and CG performance evaluation, periodic traffic with frame arrival interval 1/F seconds is considered as a starting point, where F is frame rate in FPS. |
| [3] | Proposal 23: For XR and Cloud Gaming, the following two traffic source types can be considered for evaluation, assuming frame rate is X FPS. - Traffic source type 1: every 1/X s, the packets of both eyes arrive at the same time for each frame. - Traffic source type 2: every 1/(2*X) s, the packets of left eye and right eye arrive in turn, e.g. the packet of left eye arrives at odd frames, while the packet of right eye arrives at even frames. Proposal 24: For XR and Cloud Gaming, following options for packet modelling can be considered, - Option 1: an application level packet is modelled as a packet during simulation, i.e. one frame consisting of one or more IP level packets ≈ one packet in simulation. - Option 2: an IP level packet is modelled as a packet during simulation, i.e. one IP level packet ≈ one packet in simulation. Proposal 25: For DL, traffic models in Table 7 and Table 8 are considered as the starting point for XR and Cloud Gaming evaluation, respectively. Proposal 26: For UL, traffic model in Table 9 is considered as the starting point for XR and Cloud Gaming evaluation. |
| [4] | Proposal 9: The XR traffic model could consider the packet size, including fixed value and random distribution, and the packet arrival time, including periodic and non- periodic. |
| [5] | Proposal 2: The following 2 categorizations of XR representative services for DL services as the starting point - 25Mbps,10-6, 300, fixed periodicity, fixed packet size - 100Mbps, 10-3, [5-30] fixed periodicity and fixed packet size/statistical model for fluctuating traffic - FFS periodicity values and the modeling of variant inter arrival rate - FFS statistical model for fluctuating traffic, target to down-select in RAN1#103-e between Gaussian distribution and Pareto distribution Proposal 3: Adopt the three-step methodology to derive the traffic models for Gaussian/Pareto distribution of file size. Step 1: Generate the mean packet size according to the packet arrival rate and the data rate requirement. Step 2: Determine the minimum file size or the standard derivation parameter based on the relationship between the mean data rate and the minimum/maximum (i.e. truncated) data rate. Step 3: Determine the remaining parameter, if any, by jointly considering the mean file size and the parameter obtained from step 2. |
| [6] | Proposal 1: The frame size of DL follows a random distribution, while the frame size of UL can be assumed as two different values, one is for pose and control information, and the other is for scene update information. |
| [7] | Proposal 2: Down-select between the Gaussian and Pareto distributions for modelling packet arrivals and jitter and select a small set of packet sizes, jitter values, and packet arrival rates for evaluation. |
| [9] | Proposal 1: - In the traffic model for XR, multiple data streams (e.g. for audio and video) for each direction (DL or UL) are generated for a UE; - Each data stream can be configured separately with - Periodicity - Packet size distribution (e.g. fixed or following a distribution) Data flow specific latency and reliability requirements |
| [10] | Proposal 1: For evaluations, RAN1 to use a generalized/parametric XR traffic model with configurable parameters that can represent any of the XR traffic. The configurable parameters in the generalized XR traffic model are: - UL: - Traffic arrival distribution: [Quasi-periodic with configurable inter-packet arrival rate] (e.g. 60 to 500Hz) - Traffic file distribution: [Uniform distribution with configurable packet size] (e.g. 30 to 250B) - Number of data streams: [Configurable number of streams, configurable traffic parameters common to all streams] (e.g. single/multiple streams with bounded latency) - Traffic parameters of each data stream: [Configurable data rate, latency and reliability] (e.g. 500kbps, 10ms, 10E-04 PER) - DL: - Traffic arrival distribution: [Quasi-periodic with configurable inter-packet arrival time duration] (e.g. FTP3, inter-packet arrival proportional to 1/frame-rate) - Traffic file size distribution: [Truncated Gaussian distribution or Parero distribution with configurable mean, σ, min, max] (e.g. mean: 1200B) - Number of data streams: [Configurable number of streams, configurable traffic parameters common to all streams] (e.g. isochronous multi-stream with bounded latency) - Traffic parameters of each data stream: [Configurable data rate, latency and reliability] (e.g. 100Mbps, 10ms, 10E-04 PER) |
| [11] | VR1 DL Traffic Model; UL Traffic Model. Traffic model; FTP Model 3; Periodic traffic. Rate; HEVC @ 60 fps: - 4K: 43 Mbit/s; Period = 1/X, X=100-200ms. PDB; 10ms; 10ms. PER; 1e-4; 1e-4. Packet size; 1500 byte; 100 bytes. Packet size distribution; Constant; Constant. Transport protocol; TCP (DASH/HTTP); TCP (DASH/HTTP). VR2 DL Traffic Model; UL Traffic Model. Traffic model; FTP Model 3; CBR. Rate; 50-100 Mbps; Several 100 kbps. PDB; 20 ms; < 10 ms. PER; 1e-4; 1e-4. Packet size; 1500 byte; 100 bytes. Packet size distribution; Constant; Constant. AR1 DL Traffic Model; UL Traffic Model. Traffic model; FTP Model 3 (option 2), CBR (option 3); FTP Model 3 (option 2,3). Rate; Option 2: HEVC @ 60 fps: - 4K: 43 Mbit/s Option 3: - 10 kbit (small object) every 5s - 10 Mbit (large object) every 5s; Option 2,3: HEVC @ 60 fps: - 720p: 10 Mbit/s - 1080p: 29 Mbit/s - 4K: 43 Mbit/s. PDB; 10ms; 10ms. PER; 1e-6; 1e-6. Packet size; 1500 byte; 1500 byte. Packet size distribution; Constant; Constant. Transport protocol; UDP (RTP); UDP (RTP). CG DL Traffic Model; UL Traffic Model. Traffic model; FTP Model 3; CBR. Rate; HEVC @ 60 fps: - 720p: 10 Mbit/s - 1080p: 29 Mbit/s - 4K: 43-45 Mbit/s; 0.2-0.7 Mbit/s. PDB; 10ms; 10ms. PER; 1e-4; 1e-4. Packet size (interval); 1200 byte; 100 byte. Packet size distribution; Constant; Constant. Transport protocol; UDP (GQUIC); UDP (GQUIC). |
| [12] | Proposal: RAN1 should develop relevant over-the-air traffic model for AR1 and AR2 applications as well as VR1, VR2 and CG applications based on the output of SA4 study item during the RAN1 Rel-17 study item for evaluation of XR for NR. |
| [13] | Proposal 4: Adopt the proposed traffic model for cloud gaming traffic. Proposal 5: The jitter should be modelled as a parameter in the traffic model. Proposal 6: For the XR traffic, the Cloud Gaming traffic model could be used as a baseline and extended as needed. |
| [14] | Proposal-1: A simplified baseline DL model (media) is to consider periodic traffic (arriving at framerate) together with a truncated distribution for packet size. The addition of randomness to this periodic traffic burst due to jitter might be required. |
| [15] | Proposal 3: Periodic traffic can be assumed for the DL and UL traffic of the VR service |
| [16] | Proposal 3 The frame size for the video traffic may include a variance, e.g., Gaussian distribution, in time to be more realistic. Proposal 4 The frame arrival time to RAN for the video traffic may be approximated to be periodic and equal to the inverse of a frame refresh rate. Proposal 5 RAN1 should decide the exact video traffic parameters further when SA WG4 XR study is finalized [3]. The parameters can include a frame size in terms of mean, variance, the maximum and the minimum value at least for the minimal acceptable encoding rate and the frame generation interval. Proposal 6 Both DL and UL should be studied to understand the impact of a wide-spread deployment of XR services in a cellular network. |
| [18] | Proposal 2: RAN1 defers its conclusion on XR traffic model to be used for performance evaluations until the outcome of SA4 study on XR traffic model is available. |

Summary

Companies have shown their initial view on following aspect of DL traffic model.

- DL file inter arrival: this could be roughly inverse of frame rate (Fps). Depending on jitter modeling, it could be periodic   (i.e., w/o jitter) or random w/ jitter

  - Periodic (w/o jitter): [2] [3] [4] [5] [9] [14] [15] [16]

  - Random (w/ jitter): [4] [7] [10] [11] [13] [14]

- File size distribution: this is also related to whether to model each file as IP packet or frame.

  - Fixed: [4] [5] [11]

  - Random: [3] [4] [6] [7] [9] [10] [13] [16]

    - Ex : truncated Gaussian, Pareto, etc

- Multiple data streams to model e.g., video, audio, etc.

  - [9] [10]

- Views to take into account SA4 outcome in traffic model discussion given that SA4 is working on traffic model

  - [1][3][12][16][18]

### UL Traffic Model

Table 3 captures the views on UL traffic model from different sources. Some of views commented with DL are already captured in Table 2 are not captured here.

Table 3 Views on UL Traffic Model

| Source | View |
| --- | --- |
| [3] | Table 9. UL traffic model for XR/Cloud Gaming Traffic model; Model 1; Model 2; Model 3; Model 4. Packet size distribution; Fixed, 100Bytes; Fixed, 100Bytes; Fixed, 100Bytes; Fixed, 100Bytes. Packet arrival interval (ms); 1; 2; 5; 20. Packet delay budget (ms); 10; 10; 10; 10. Proposal 26: For UL, traffic model in Table 9 is considered as the starting point for XR and Cloud Gaming evaluation. |
| [5] | Proposal 4: The following 2 categorizations of XR representative services for UL services as the starting point. - 28kbps,10-6, 300, fixed periodicity, fixed packet size - 2.7-50Mbps, 10-3, [5-30] fixed periodicity and fixed packet size/statistical model for fluctuating traffic - FFS periodicity values and the modeling of variant inter arrival rate - FFS statistical model for fluctuating traffic, target to down-select in RAN1#103-e between Gaussian distribution and Pareto distribution |
| [9] | UL traffic |

Summary

Companies have provided views on UL traffic model in following aspects.

- UL file inter arrival

  - Periodic: [3] [5] [15] [11] [10][9]

  - Random: [11] [10]

- UL file size

  - Fixed: [3] [5] [10] [6] [11]

  - Random: [5] [10][9]

- Multiple streams: [9] [10]

- Views to take into account SA4 outcome in traffic model discussion given that SA4 is working on traffic model

  - [1][3][12][16][18]

FL Proposal 2: Traffic model for DL and UL should reflect various bit rates, variable frame/slice/file/packet size, and periodicity with jitter, where statistical model is preferred.  Conclusion on detailed traffic model is deferred to the next RAN1 meeting (RAN1 104-e) where SA4 outcome on traffic model is expected to be available.

Question 2. Please share your comments on Proposal 2 if any.

| Company | View |
| --- | --- |
| QC | Decisions on traffic model should be made after SA4 provides its outcome to RAN1. |
| MTK | We can accept to defer the conclusion on detailed traffic model to the next RAN1 meeting (RAN1 104-e) where SA4 outcome on traffic model is expected to be available. |
| ZTE, Sanechips | It would be better to clarify that it's not mandatory to consider the variable frame frame/slice/file/packet size or periodicity with jitter for all the applications of interest as confirmed in Q1 given at least for VR case, it seems jitter-free periodic traffic with fixed packet size for both UL/DL is sufficient per SA4 TR26.928. Alternatively, we are also fine to initiate the traffic model related discussion in RAN1#104-e A modified proposal would be, Traffic model for DL and UL for certain application(s) of interest should reflect various bit rates, variable frame/slice/file/packet size, and periodicity with jitter, where statistical model is preferred. Conclusion on detailed traffic model is deferred to the next RAN1 meeting (RAN1 104-e) where SA4 outcome on traffic model is expected to be available. |
| Nokia, NSB | We agree that the RAN1 study conclusions should be in line with the SA4 outcome. At the same time, we believe it is important to start discussing (at least discussing, not mandatory concluding) this topic straightaway, as it holds many other important discussions. We can at least start collecting the companies’ view on how many traffic models do we need in RAN1 (e.g., 5 as the use cases, less models, where one model is representative to several use cases, more than one model per use case), etc. Note that reference [11] was missing from UL traffic size inputs (added with highlight) |
| FUTUREWEI | Our understanding is that almost all the companies are proposing a statistical traffic model with near-periodic packet arrival (with jitter as proposed by some) and certain packet size distribution. We should try to reach some high-level consensus towards that direction while waiting for inputs from SA4. |
| APPLE | In our view, the same traffic model can be applied for both downlink and uplink, from SA4’s studies on ITT4RT and 5GSTAR, and some use cases from XR_Traffic, uplink traffic can have the same characteristics as downlink’s, reference from [9] is added with highlight. |
| DOCOMO | Agree with the proposal. It should be better to wait for SA4 outcome for alignment between SA4 and RAN1. |
| LG | We support FL’s proposal. RAN1 discussion before SA4 outcome may cause potential confusion and unnecessary RAN1 overhead. |
| InterDigital | Although in principle we agree with the proposal, we think the use of a configurable traffic model for DL and UL as proposed in [9] and [10] can be considered as baseline. The parameters (e.g. bit rates, periodicity, PDB), to be applied in the configurable traffic model can be updated based on SA4 outcome |
| Intel | We are ok to wait for SA4 outcome for better alignment. |
| CMCC | Support FL proposal in principle. We also prefer to discuss the baseline configurable traffic model since the traffic model is relevant to all the evaluations. |
| Ericsson | We are mostly fine with the proposal. However, we do not see that jitter is part of the traffic model, and SA4 will not provide modelling of the jitter. Jitter is the result of varying delays in the transport and core networks and can be mitigated by appropriate network design and dimensioning. Therefore, we suggest the following reformulation: FL Proposal 2: Traffic model for DL and UL should reflect various bit rates, variable frame/slice/file/packet size, and periodicity, where statistical model is preferred. Conclusion on detailed traffic model is deferred to the next RAN1 meeting (RAN1 104-e) where SA4 outcome on traffic model is expected to be available. How to model jitter is FFS. |
| vivo | We agree with FL’s suggestion. |
| Xiaomi | We are fine to defer conclusion until RAN4 makes decision. However, we are not sure whether jitter or variable packet size should be considered for UL traffic of VR and CG use cases. |
| Huawei, HiSilicon | Please could FL explain what does “slice, file” mean? And why they are relevant to RAN1? And packet refers to IP packet, PDCP packet, or else? Since RAN1 will wait for SA4 outcome, maybe just mentioning “frame” is enough at this stage. Periodicity without jitter should also be considered. The final traffic model may depend on SA4 outcome. But for the sake of progress, we suggest in this meeting, RAN1 can also have some discussions on traffic model, such as frame size distribution, frame arrival interval, etc. This can help companies in RAN1 have a better understanding of each application. In summary, we suggest to revise Proposal 2 as follows: FL Proposal 2: Traffic model for DL and UL should reflect various bit rates, variable frame/slice/file/packet size, and periodicity without or with jitter, where statistical model is preferred. Conclusion on detailed traffic model is deferred to the next RAN1 meeting (RAN1 104-e) where SA4 outcome on traffic model is expected to be available. |
| CATT | In principle, we agree with FL’s intention and summary of traffic model. However, we need to define the underline theory and assumption of these configured parameters, such as packet size, interarrival time, for traffic modelling. We could define the traffic arrival process of XR services based on statistical/stochastic or deterministic process in queueing theory, such as M/M/1, M/M/N, G/M/1, D/D/1 process, to capture the characteristics of different XR applications. |
| Facebook | We agree with the proposal that we can wait for SA4 outcome to make decision on the traffic model, although we also think that some initial discussion in RAN1 would be helpful. |
| Samsung | Agree with the proposal from FL. |

## Evaluation of UE Power Consumption for XR

### Evaluation Methodology for UE power consumption for XR

Companies’ view on evaluation of UE power consumption for XR is collected in Table 4.

Table 4 Companies’ Views on evaluation of UE power consumption for XR

| Source | View |
| --- | --- |
| [3] | Both the power saving gain and the capacity performance loss need to be considered. For power consumption evaluation, both the power saving gain and the capacity performance loss need to be considered. During the evaluation, power consumptions for different cases can be collected for subsequent comparisons as depicted in Figure 3, including: - Case 1: No power saving mechanism is introduced. It is the performance baseline to show the consumed power and corresponding capacity performance. - Case 2: The DRX mechanism for connected mode in NR is introduced which can be a starting point. Each UE may be configured with different DRX offset For XR power consumption evaluation, - Power consumption performance is evaluated by using power consumption model in TR 38.840. - Capacity performance is evaluated by considering different DRX configurations. - Details of DRX configuration are reported by companies |
| [4] | For XR service evaluation, the power consumption evaluation methodology and metric in TR38.840 could be reused. |
| [6] | Besides, the trade-off between power consumption and other performance should be considered, e.g. 60fps with lower power consumption or up to 90fps performance boost with higher power consumption, can be chosen in the game settings. The power consumption model in TR 38.840 can be reused to evaluate the power consumption performance of XR and Cloud Gaming |
| [7] | Rel-16/17 UE power savings mechanisms and potential enhancements are considered subject to minimal/no additional scheduling latency and minimal/no increase in PDCCH blocking probability. The power consumption models developed in TR 38.840 and Rel-16 and Rel-17 mechanisms for power savings can be a starting point subject to considerations for minimal/no additional scheduling latency and minimal/no increase in PDCCH blocking probability. |
| [13] | Coverage evaluation should be done in the agenda of R17 coverage enhancement considering that coverage is a trade-off with capacity. Reuse the evaluation assumption agreed in R17 power saving as baseline with necessary modifications - R1-2007419: LS on evaluation for connected mode UE power saving |
| [15] | The power consumption model and performance metrics in TR 38.840 can be reused for the evaluation of power consumption. |
| [16] | Evaluations for existing connected mode UE power savings techniques so far have shown that achieving power savings generally comes with a throughput/latency trade-off. Baseline XR performance should be evaluated assuming that the UE is always available for scheduling (i.e., DRX or other power saving techniques are not considered) and any studies on power savings techniques should consider latency/throughput impact compared to the baseline. |
| [17] | The followings can be considered for KPIs for XR evaluations: - Capacity: TR38.824 can be baseline for both URLLC independent case and eMBB/URLLC coexistence case - Mobility: up to 300 km/h or 500 km/h should be taken into account - Power: TR38.840 can be starting point - Coverage: Rel-17 coverage enhancement study can be starting point |
| [18] | Power and capacity has trade-off relation. RAN1 performs system level simulation method for power evaluation, especially to accurately evaluate the capacity-power tradeoff. In case power saving gain of power saving techniques is quantified, the gain is evaluated, compared, and captured subject to a given capacity constraint. The grey point is the baseline scheme w/o power saving scheme: UE in always ON state (i.e., no DRX enabled). For XR power evaluation, RAN1 consider various power saving schemes including R15/R16/R17 power saving techniques and various assumptions having high impact on UE power consumption. |
| [9] | TR 38.840 and additional modelling details from the ongoing power saving work item and RedCap study item from Rel-17 can be used as a starting point for XR power consumption evaluation. Focus the evaluation on capacity and UE power consumption. The considering on mobility and coverage can be built into the evaluation methodology itself: e.g. 3 km/hr, indoor deployment and/or outdoor deployment, etc. Then separate efforts to evaluate enhancements for mobility and coverage are not needed. |

System level evaluation

It is discussed in [18] that UE power consumption for XR in reality is affected by various aspects including UE geometry, link adaptation, user selection by gNB scheduler in case of multi-UEs in the network, parameter configuration of applied power saving techniques (e.g., CDRX parameters), etc.  It is therefore proposed in [18] that system level simulations should be conducted to accurately evaluate effect of those aspects since link level simulations have limitations to capture those aspects in evaluation for UE power consumption.

Question 3. Please share your view on whether system level simulation should be the baseline for evaluation of UE power consumption for XR.

| Company | View |
| --- | --- |
| QC | We support the system level simulation study for power evaluation. The operations of dynamic power saving schemes highly depend on PS (power saving) control signal reception (via DCI), and DRX timer values (determining Active Time/non Active Time), and/or scheduling decision across multiple UEs. Thus, we think that system level study should be used to accurately capture the impact of these aspects on UE power consumption. |
| MTK | We support to use system level simulator, while RAN1 should choose a reasonable amount of parameter combinations to avoid a too large simulation effort. |
| ZTE, Sanechips | As expressed in our response to the email thread under 8.14 TR skeleton, it would be good to have a discussion on prioritization among the potential evaluations related to XR given the TU allocated and the work load. We believe power saving related evaluations could be deferred a bit (e.g., to 2021 Q2) compared to capacity and coverage. |
| Nokia, NSB | We should first establish whether we will engage in power consumption evaluation in the first place. It is premature to have this level of detail at this stage. The questionnaire already assumes, not just that a power consumption evaluation is to be made but also has a very specific split of work built into it. What we need to discuss is whether we do power evaluation, and then what is the appropriate level of work. It is also important to avoid work duplication over what is done in the power saving enhancements WID. Notably power consumption evaluation is not part of the SID objectives and we should first start by agreeing if UE power consumption is a relevant KPI to measure the performance. 3. Identify evaluation methodology to assess XR and CG performance along with identification of KPIs of interest for relevant deployment scenarios |
| FUTUREWEI | We also think traffic model and identification of KPIs should be done before this discussion as directed in the SID. |
| Apple | We agree power consumption is key for XR. How to perform power consumption evaluation should be discussed, including whether system level simulation is used, and what studies to be conducted, etc. |
| DOCOMO | We agree that system level simulation should be baseline for UE power consumption evaluation but the decision should be deferred. We think it needs to be discussed first whether evaluation for power consumption is done or not. Considering work load, it would be better to prioritize capacity evaluation. |
| LG | Considering the comments so far, we may need to discussion priority of power consumption analysis in this study first. If we do power consumption analysis, we think link-level approach would be basic one while system level evaluation can be additionally done. |
| InterDigital | For the purpose of evaluating the capacity vs power consumption tradeoff in XR, we agree that system level simulations can be considered for UE power consumption evaluation. |
| Intel | We think we can come back to this after we have some converged views on KPIs, requirements and basic EVM |
| Ericsson | We think it’s not necessary to agree on this at this early stage. It’s up to companies to contribute. |
| vivo | We support FL’s proposal to use SLS for power consumption evaluation, since both the power saving gain and the capacity performance loss need to be considered. |
| Huawei, HiSilicon | So far, many issues are unclear, such as the traffic models, evaluation methodology and KPIs, etc. We think it’s premature to decide whether SLS is needed or not. There is no need to discuss this issue in this meeting. And companies are free to submit simulation results. In addition, the simulation workload should also be considered due to the limited TU of the study item. |
| CATT | The power model for UE power consumption defined in TR38.840 are the relative value to the deep sleep. The UE power consumption for any traffic arrival does not require system level simulation. The UE power consumption and UE perceived throughput (latency) could be evaluated together as those evaluated in Rel-16 UE power saving in TR38.840 |
| Facebook | We think power consumption is critical for XR use case. Hence, we support the proposal on adopting the system level simulation capturing the impacts of different features and system parameters. |
| Samsung | We agree that UE power consumption is one of the key metrics. But the primary one are the performance requirements (e.g. PER, PDB) and UE power consumption can be a complementary metric under the performance requirements. A concern with SLS is that, especially under the current setting of meetings, it may difficult to calibrate or decipher reasons for potential large discrepancies. Despite their limitations and required abstractions, analytical approaches based on well-calibrated LLS should also be complementary. |

Summary

- Is UE power consumption evaluation part of the study, i.e., is UE power consumption a KPI?

  - Yes (9): QC, MTK, Apple, InterDigital, vivo, Xiaomi, CATT, Facebook, Samsung

  - No (2): Nokia, HW

  - Need further discussion (7): Nokia, LGE, ZTE, FutureWei, DOCOMO, Intel, HW

  - It’s up to companies to contribute.

    - Ericsson, HW

- If UE power consumption is evaluated, which is more appropriate between system level evaluation vs. link level simulation

  - System level (5): QC, MTK, InterDigital, vivo, Facebook

  - Link level (3): LG, CATT, Samsung

  - Up to company (2): Ericsson, HW

Baseline UE power consumption: UE power consumption w/ no power saving scheme

Multiple companies emphasize the importance of evaluation of UE power consumption for XR, including evaluation of power saving gain from various power saving techniques that have been developed since Rel-15.  The power saving gain can be evaluated compared to a baseline UE consumption result.  Multiple companies propose to define the baseline UE power consumption to be the power consumption for the case when the UE is always available for scheduling, i.e., the UE never goes into a sleep state (in others words, no power saving technique is applied).

Question 4. Please share your view on the above definition of baseline UE power consumption to be used for evaluation of the gain of a UE power saving technique.

| Company | View |
| --- | --- |
| QC | The baseline power consumption scheme is always ON scheme (w/o any power saving scheme, e.g., no CDRX, no BWP switching, no WUS, etc). The power saving gain of other power saving schemes can be compared w.r.t this baseline. This will allow the comparison of different schemes with the same reference point. |
| MTK | Considering the power-capacity trade-off, we can accept to use the FL proposed baseline for first wave evaluation. If RAN1 found that some existing power saving techniques introduce negligible capacity loss, a new baseline can be applied. |
| ZTE, Sanechips | We agree that no power saving scheme should be considered as the baseline. The impact of power saving schemes on the KPIs, such as capacity, coverage, mobility, should be evaluated. |
| Nokia, NSB | Should the UE power consumption be agreed as a relevant XR performance KPI, there is no reason to exclude DRX or other power saving techniques specified in Rel-15 – Rel-17 if they are applicable with a given traffic model. It is, however, premature to discuss how to evaluate the baseline before we have agreed to do the work at all. If the answer is yes, then at that point we should look at what power saving tools Rel-15 – Rel-17 that are applicable with the traffic models applicable for XR services. |
| FUTUREWEI | When discussing the power consumption (if identified as a KPI) later, at least the relevant features developed in R16 should be included. |
| Apple | The FL proposal is fine to us as a baseline scheme needs to be simple. However, exactly how that is conducted, e.g. with SLS or LLS should be discussed. |
| DOCOMO | Firstly, we think this should be discussed after we make a conclusion to do evaluations for power saving. However, if needed, we should clarify what power saving schemes are applicable for XR traffic models. The schemes should include at least Rel-15/16. |
| LG | While it is fine to perform analysis with no power saving schemes, Rel-16 power saving techniques should be also included when applicable to avoid introducing unnecessary enhancements (which is the case for Rel-17 power saving WI) |
| InterDigital | We think different baseline cases can be considered for XR, e.g. without any power savings and with basic power saving (e.g. assuming generic DRX configuration for eMBB/URLLC traffic) |
| Intel | Similar view as DOCOMO and Nokia, we think identification of KPIs and basic EVM is a priority– we can check this after there is a conclusion to pursue power savings eval. |
| Ericsson | We agree the baseline should be that the UE is always available for scheduling. |
| vivo | We agree with FL’s suggestion that the baseline scheme for power consumption evaluation is no power saving technique is adopted |
| Xiaomi | We agree with FL’s proposal. |
| Huawei, HiSilicon | According to RAN plenary decision (RP-202111, copied below), it’s clear that power saving related issues are not within the scope of this RAN1 meeting. So this question is premature and should not be discussed. - Study on XR Evaluations for NR (RAN1) - Applications, Traffic Model and Evaluation Methodology Focus on applications and evaluation methodology including identification of KPIs of interest for relevant deployment scenarios - Others - NOTE: SA4 has ongoing work in the XR area, RAN1 will not address these SA4 aspects but will wait for SA4’s outcome |
| CATT | Rel-16 UE power saving study and work had clear definition of baseline assumption in TR38.840. Those baseline assumptions could be used for XR evaluation. |
| Facebook | We agree with FL’s proposal. |
| Samsung | Agree with the FL’s proposal. Also agree with evaluating impacts of UE PS mechanisms on KPIs. |

Summary

- Agree (12): QC, MTK, ZTE, Apple, LG, InterDigital, Ericsson, vivo, Xiaomi, CATT, Facebook, Samsung

- Rel-16 power saving scheme should be evaluated:

  - HW, DOCOMO, LG (FL note: this seems to be assumed by all companies although many companies do not explicitly address this point).

- Premature: Nokia, Intel, DOCOMO

Upper bound of UE power saving gain: Genie UE power consumption

As another useful reference for evaluation of UE power consumption for XR, “Genie” UE power consumption is proposed in [18], defined as UE power consumption for the case when the UE is assumed to enter a sleep state in all the slots with neither DL reception nor UL transmission.  Which sleep state (micro, light, or deep) the UE can enter is determined based on the duration of consecutive slots with no DL reception and no UL transmission.  The Genie result can serve as the upper bound of the power saving gain of a power saving technique, which may potentially motivate development of new power saving techniques that can approach the Genie performance.

Question 5. Please share your comment if any on evaluation of the “Genie” UE power consumption as a benchmark.

| Company | View |
| --- | --- |
| QC | We support the evaluation of Genie scheme in XR power evaluation. Identifying gap between any considered scheme and upper bound could be an important outcome of XR power study in the sense that it can identify potential issues and solutions. |
| MTK | We can accept to include the evaluation of the “Genie” UE power consumption as a benchmark. However, it should be noted that the upper bound can not address the power saving schemes that applied BW scaling, MIMO layer adaptation, or UL/DL transmission timing relaxation. |
| ZTE, Sanechips | We think the “Genie” UE power saving consumption can be informative but not mandated. |
| Nokia, NSB | Premature to discuss, no agreement even on whether the UE power consumption is a KPI of interest for assessing the XR and CG performance |
| FUTUREWEI | Not sure this is the right topic for discussion at this point. Also how to define a “Genie” scheme and what is the purpose of such a benchmark for further study? |
| Apple | The evaluation of a genie scheme can be informative. |
| DOCOMO | We understand the “Genie” UE power consumption result can be informative but this should be discussed later. First, we should discuss whether to do evaluations for power consumption. |
| LG | No objection to see Genie UE power consumption reference, but it doesn’t need to be mandatory analysis. |
| InterDigital | We think that if the intention of the Genie UE power consumption is to characterize the best case or upper bound performance, then possibly other methods of identifying when the UE can enter into micro, light or deep sleep can be formulated rather than based on duration of Tx/Rx inactivity. For example, the alternative method can include the use of different sleep/on profiles to serve as upper bound, where these sleep/on profiles are aligned/optimized with known (benchmark) traffic profiles for VR, XR and CG applications |
| Intel | Premature to discuss |
| Ericsson | Too early to even discuss. |
| vivo | We agree with FL’s suggestion. Genie scheme for power consumption evaluation can be evaluated to investigate the upper bound of power saving gain without the capacity performance loss. To clarify the genie scheme, we think a UE with genie scheme can enter the power saving mode when there is no DL reception and UL transmission, and can wake up immediately for DL reception and UL transmission, where the Rel-16 defined power model should be followed. |
| Xiaomi | We agree with other companies that genie scheme can be optional. |
| Huawei, HiSilicon | Premature to discuss. As replied under Question 4, power saving related issues are not within the scope of this RAN1 meeting. This study item should focus on some general issues first, i.e., applications, traffic models, KPIs and evaluation methodology. |
| CATT | The power consumption modelling is very clearly specified in TR38.840. We could refer to those in TR38.840 for the power saving study in XR |
| Facebook | We support the development of upper bound on the power saving/UE power consumption. As mentioned previously, power consumption is critical to the XR use cases. Hence, the evaluation provides a good indication on how far we are from ideal scenario. |
| Samsung | The proposal from the FL is reasonable – together with the one from Q4, it can establish lower bounds and upper bounds for UE PS mechanisms in order to better understand potential ‘deltas’. |

Summary

- Agree (12): QC, MTK, ZTE, Apple, LG, InterDigital, Ericsson, vivo, Xiaomi, CATT, Facebook, Samsung

- Rel-16 power saving scheme should be evaluated:

  - HW, DOCOMO, LG (FL note: this seems to be assumed by all companies although many companies do not explicitly address this point).

- Premature: Nokia, Intel, DOCOMO

Evaluation of UE power saving gain of power saving schemes

It is highly beneficial to study the power saving gain of various power saving schemes, e.g.,

- Rel-15/16/17 UE power saving techniques, e.g., CDRX, BWP switching, cross-slot scheduling, etc.

- Potential future enhancements (e.g., CDRX enhancements if needed, etc.).

In addition, it will be useful to explicitly study and better understand the impact of certain critical features/configurations on UE power consumption, e.g.,

- DL rx and UL tx alignment: In case DL and UL transmissions to/from a UE are consecutive, UE can stay longer in a deeper sleep state.  It will be useful to see the power performance difference between DL/UL alignment vs. misalignment.

- Impact of jitter in UE power consumption: Effect of certain power saving techniques, e.g., CDRX can be substantially reduced by jitter (as packet arrival times at gNB vary over time which are not aligned with the beginning of the configured CDRX on duration).  Although jitter may be part of traffic model, it would be highly beneficial to explicitly evaluate UE power consumption with and without jitter.  The study outcome may potentially motivate development of a new power saving technique that is more effective with jitter.

Question 6. Please share your view on which power saving techniques among various candidates discussed above are to be evaluated with high priority in terms of power saving gain over the baseline discussed above.

| Company | View |
| --- | --- |
| QC | It is very important to evaluate existing power saving schemes to understand power consumption aspect in XR support. However, due to the limited time, it would not be doable to evaluate all the combinations of the power saving schemes defined in R15/R16 as well as potentially in R17. To reduce workload without missing any potentially important study results, RAN1 should focus on PS schemes which have the highest impact on UE power consumption. We think at least following scheme should be evaluated in this SI. - CDRX In addition to the specific PS schemes, we could also consider evaluation of configurations/assumptions with high impact on power consumption. We agree that UL/DL alignment can reduce power consumption by reducing UE’s wake up and extend potential sleep duration. Jitter is another critical aspect to study especially when DRX is used. With jitter, the expected delay for packet transmission can increase which potentially have negative impact on power and capacity as well. |
| MTK | Since RAN1 has developed various power saving schemes in R15/R16/R17(on-going) power saving agenda, to avoid duplicated effort in RAN1 discussion, we prefer to first evaluate existing power schemes in R15/R16 power saving agenda, ex., “CDRX, BWP switching, cross-slot scheduling, SCell dormancy”, to observe the capacity-power trade-off for existing schemes. After this, if it is observed that a significant power saving gain gap exists, new power saving schemes in R17 power saving agenda or other new schemes can be evaluated. |
| ZTE, Sanechips | To have a better understanding of the impact and potential problems of the legacy power saving solutions, we think the schemes including C-DRX, BWP switch, and cross-slot scheduling specified in Rel-15/16, and Rel-17 candidate enhancements (e.g., dynamic PDCCH reduction) needs to be evaluated. |
| Nokia, NSB | Premature to discuss, no agreement even on whether the UE power consumption is a KPI of interest for assessing the XR and CG performance. Notably this SI is not a study of what power saving gains the already specified power saving features offer over no power saving features applied. These components should end up as part of the baseline if such evaluation is decided to be needed. |
| FUTUREWEI | As stated in our answers above, features related to power consumption developed in R16 (and even in R17) should be included if such evaluation is agreed to carry out in this SI. |
| APPLE | We can investigate power saving techniques first then come to the candidate solutions. |
| DOCOMO | Same comment as previous questions. It should be discussed later. |
| LG | This point may not need to be discussed at this stage since companies proposals on potential enhancements should be discussed later stage of the SI in a more structured manner. |
| InterDigital | We think power savings schemes such as CDRX and other techniques are not agnostic to the traffic profile/model, including the DL/UL alignment and jitter-aware techniques, can be considered in the evaluations. The details of power consumption techniques can be deferred for later discussion |
| Intel | Premature to discuss |
| Ericsson | We think it’s not necessary to do prioritization exercise at this early stage. It’s up to companies to contribute. |
| Vivo | Rel-15/16/17 UE power saving techniques should be evaluated with high priority. The DRX configurations and parameters for evaluation need further discussions. Since the power saving gain would be highly depending on the traffic mode, it is necessary to determine the appropriate DRX configuration/parameter for a given application and traffic model. Potential future enhancements can also be considered for power evaluation. We are open to further evaluate DL-UL alignment and impact of jitter as optional features. |
| Xiaomi | We agree with other companies that it is premature to discuss this issue. |
| Huawei, HiSilicon | Premature to discuss. As replied under Question 4, power saving related issues are not within the scope of this RAN1 meeting. At this stage, it’s up to companies. No need to make agreements to prioritize or down-prioritize some techniques. |
| CATT | Rel-16/17 Power saving techniques could be used for XR. However, jitter buffering and other aspects are mostly UE implementation and can’t be captured in UE power consumption evaluation completely. |
| Facebook | We agree that Rel. 15/16/17 power saving techniques can serve as a starting point for evaluation. We are open to the discussion of power saving schemes and potential enhancements at the early stage given the importance of the features. |
| Samsung | Consideration of the suggested configuration aspects for UE PS mechanisms is reasonable but of course discussion is needed for conclusions. Agree that comprehensive evaluation is not possible and a reasonable down-scoping should be discussed for UE PS mechanisms. Considerations on CDRX and jitter-aware mechanisms are justified. |

Summary

- R15/16 power saving techniques potentially with down-selection

  - MTK, QC, ZTE, FutureWei, Apple, InterDigital, CATT, Facebook, Samsung

- Additional techniques

  - QC, Apple, InterDigital

- Premature

  - Nokia, DOCOMO, LG, Intel, Xiaomi, Huawei,

- Up to company to contribute

  - Ericsson

Power saving gain taking into account capacity-power tradeoff

As captured in Table 4, multiple companies have observed and pointed out that there exist tradeoff relations among different performance aspects: capacity, power, coverage. For example, [3][6][16][18] have observed the tradeoff between capacity and power. [13] has observed the tradeoff between coverage and capacity. One good example is tradeoff between power saving gain and capacity.  In network/UE operation, applying a power saving scheme may result in delayed packet scheduling (as the UE is supposed to be in a sleep state for a certain duration depending the applied power saving scheme), which can increase the chance of violating packet delay budget and consequently lead to lower capacity.

Understanding the tradeoff of the evaluated power saving technique is important, e.g., for fair comparison of schemes/assumptions since one scheme may have higher hit on one performance metric than other schemes (e.g., scheme A={CDRX with cycle=50ms} has higher power saving gain than scheme B={CDRX with cycle=5ms}, but A would have lower capacity than B). Thus, when the gain of a power saving technique is evaluated, the capacity loss compared to the baseline scheme (i.e., no power saving scheme) needs to be evaluated together.

FL proposal: The UE power saving gain of a power saving technique is defined to be the reduction (%) in UE power consumption compared to the baseline UE power consumption (i.e., UE power consumption with no power saving scheme), subject to at most Y% (e.g., Y=5) system capacity reduction induced by the applied power saving technique.  When the power saving gain of a power saving technique is submitted, the following should be reported together: (i) the amount of capacity loss caused by the evaluated power saving technique and (ii) how far is from the “Genie” UE power consumption (i.e., additional power consumption in % compared to the Genie power consumption).

Question 7. Please share your view on the above proposal and appropriate Y value.

| Company | View |
| --- | --- |
| QC | Putting a constraint on capacity loss in power number evaluation/comparison is required to make it as a fair evaluation/comparison across different power saving schemes. We think Y=5 would be a reasonable value. |
| MTK | We think the FL proposal is reasonable and Y=5 or 10 can be considered. We also think one value for Y is enough. |
| ZTE, Sanechips | The user experience is strongly relevant to the system capacity, we think Y=2.5% is reasonable. |
| Nokia, NSB | Premature to discuss, no agreement even on whether the UE power consumption is a KPI of interest for assessing the XR and CG performance. |
| FUTUREWEI | Too early to discuss this. |
| Apple | UE power consumption and capacity are important issues, whether trade-offs and how trade-offs can be made can be discussed later. Now it is rather premature to start with that. |
| DOCOMO | Same comment as previous questions. It should be discussed later. |
| LG | We think this is somewhat detail and has to be discussed later. |
| InterDigital | We are in principle supportive of the methodology provided in the proposal for i) evaluating the power saving gain with respect to a benchmark power saving technique and ii) the tradeoff in terms of the loss in capacity. While a specific Y value may not be needed as a constraint in the evaluations, 5% may be used as starting point. The evaluation results should show the power savings achieved for different values of capacity so that the tradeoff curve can be derived. We also think that the details of power consumption evaluations can be discussed later |
| Intel | Premature to discuss |
| Ericsson | This proposal can be deferred, as some aspects are not yet clear/should be sorted out first, such as a proper definition of upper bound. |
| vivo | We agree to evaluate the tradeoff between power saving gain and system capacity. We would like to further clarify how to evaluate power consumption performance considering the definition of system capacity in the [XR-02-Capacity] discussion. It is proposed to define the system capacity as “maximum number of users per cell with at least X % of UEs being satisfied”. In addition to simulating the power consumption performance in the case of system capacity, we think the power consumption evaluation should be also provided with different cell loads (in terms of different RUs or different number of UEs per cell which are less than system capacity), e.g. low load, medium load, high load, etc. For a given cell load, the capacity performance metrics (e.g. UPT) should be reported together with power consumption performance. |
| Huawei, HiSilicon | Premature to discuss. As replied under Question 4, power saving related issues are not within the scope of this RAN1 meeting. |
| CATT | The power saving techniques in Rel-16 and Rel-17 do not have any tradeoff in system capacity. Some power saving techniques, such as cross-slot scheduling, might have degradation on User Perceived throughput. However, most of power saving techniques, such as DRX adaptation, Maximum MIMO layer adaptation, and SCell dormancy, do not have impact on the system capacity. |
| Facebook | We agree that this is one area to be further looked into but agree can be discussed at later stage. |
| Samsung | The proposal is reasonable but detailed discussion/conclusion at this stage is not. Good for further consideration. One side note is that nearly all questions/issues so far focused on UE power consumption. While that is undoubtedly an important metric, it is not the only one for XR and probably it is not the most important. Mechanisms for meeting the (challenging) KPIs under challenging conditions without vacating the network from all other services should have at least a same (if not higher) level of priority in the discussions. |

Summary

- X = 2.5%

- X = 5%

  - QC, InterDigital

- X = 5 or 10%

  - MTK

- Premature

  - Nokia, FutureWei, DOCOMO, Apple, Intel, Ericsson, vivo, HW, Facebook, Samsung

### Other Evaluation Assumptions

Following sources captured in Table 5 have provided their views.

Table 5 Views on Evaluation Methodology on Power

| Source | View |
| --- | --- |
| [3] | Proposal 19: For XR power consumption evaluation, • power consumption performance is evaluated by using power consumption model in TR 38.840. • capacity performance is evaluated by considering different DRX configurations. - details of DRX configuration are reported by companies. |
| [4] | Proposal 5: For XR service evaluation, the power consumption evaluation methodology and metric in TR38.840 could be reused. . |
| [6] | Proposal 2: The power consumption model in TR 38.840 can be reused to evaluate the power consumption performance of XR and Cloud Gaming. |
| [7] | The power consumption models developed in TR 38.840 [4] and Rel-16 and Rel-17 mechanisms for power savings can be a starting point subject to considerations for minimal/no additional scheduling latency and minimal/no increase in PDCCH blocking probability. |
| [13] | Proposal 9: Reuse the evaluation assumption agreed in R17 power saving as baseline with necessary modifications - R1-2007419: LS on evaluation for connected mode UE power saving |
| [15] | Power consumption The power consumption model and performance metrics in TR 38.840 can be reused for the evaluation of power consumption. |
| [17] | Proposal 4: - The followings can be considered for KPIs for XR evaluations: - Capacity: TR38.824 can be baseline for both URLLC independent case and eMBB/URLLC coexistence case - Mobility: up to 300 km/h or 500 km/h should be taken into account - Power: TR38.840 can be starting point - Coverage: Rel-17 coverage enhancement study can be starting point |

Given that R16 UE PS SI provides a good starting point power evaluation methodology, many companies have suggested it as a starting point. There was also a view [17] that R17 UE PS evaluation method should be the baseline with necessary modification.

FL Proposal. For XR UE power evaluation, use TR38.840 as baseline power evaluation methodology with necessary modifications if necessary.

Question 8. Please share your comments on the above proposal if any.

| Company | View |
| --- | --- |
| QC | Agree to use 38.840 as the baseline power evaluation methodology. We think it need further enhancement in UL power modeling to better capture increased power contribution from UL activities of XR devices. Our view on such enhancements is presented below by answering relevant questions. |
| MTK | We think R17 UE PS evaluation method should be the baseline with necessary modification. However, we can accept to use 38.840 since it also includes various R16 power saving techniques for evaluation. |
| ZTE, Sanechips | We agree to take TR38.840 as the baseline. And further enhancements (see our reply to Q9, Q10, Q11, and Q14) are also needed. |
| Nokia, NSB | Premature to discuss, no agreement even on whether the UE power consumption is a KPI of interest for assessing the XR and CG performance. |
| FUTUREWEI | Too early to discuss this. |
| Apple | Consider 38.840 and modifications from RedCap and Rel-17 power saving as starting point |
| DOCOMO | Same comment as previous questions. It should be discussed later. |
| LG | We are fine with FL’s proposal. |
| InterDigital | We support the proposal for using TR 38.840 as baseline with enhancements for power evaluations |
| Intel | Premature to discuss |
| Ericsson | We are fine with this proposal. |
| vivo | We agree with FL’s proposal. |
| Xiaomi | We agree with FL’s proposal. |
| Huawei, HiSilicon | Premature to discuss. We should first discuss whether power consumption is a relevant KPI. |
| CATT | Yes. TR38.840 should be used for UE power consumption related study. |
| Facebook | Fine with the proposal. |
| Samsung | Agree with MTK. |

Summary

- Agree potentially with necessary modifications (11)

  - QC, MTK, Apple, LG, InterDigital, Ericsson, vivo, Xiaomi, CATT, Facebook, Samsung

- Premature (5)

  - Nokia, FutureWei, DOCOMO, Intel, HW

Linear Interpolation based UL Power Consumption Estimate for different Tx Power

In some XR applications, UL data transmissions may include pose/control/scene upload. Their data rates range from e.g., 1Mbps ~ 10Mbps with short interval (2ms ~ 100ms) between two consecutive transmissions. This makes UL power consumption contribution to total power consumption increase significantly. Thus, it is critical to evaluate UE power consumption from UL transmissions.  The power model defined in TR 38.840 is available only for two values of UE tx power: 0dBm and 23dBm, not available for other power levels. Thus, in order to effectively capture different tx power contributions from UEs with different tx power in various locations in the cell (also depending on various interference levels), UE power consumption model for power levels other than 0dBm and 23dBm is needed.  For instance, linear interpolation model (that is already used in TR 38.840 to estimate the power consumption for different number of blind decoding) is proposed in [18].

Question 9. Please share your view on how to model UE power consumption for UE tx power other than 0dBm and 23dBm.

| Company | View |
| --- | --- |
| QC | We think it is quite reasonable to use linear interpolation technique to estimate intermediate power consumption values for tx power other than 0dBm and 23dBm. Based on our study, the linear interpolation in linear power domain could provide quite a good estimate which is close to actual measurements. This is because the scaled power is mostly due to power consumed at power amplifier (PA) and its power consumption scaling w.r.t output tx power is close to linear in the regime of interest. This should be applied to not only the UL slots in 38.840 but also other potential UL slots to be added if necessary. |
| MTK | We are open to model UE power consumption for UE tx power other than 0dBm and 23dBm. The exact method may need to be further discussed and aligned to provide consistent assumptions between companies. |
| ZTE, Sanechips | We think the UE power consumption model for other UE transmission power are necessary. The following issues need discussion/clarification towards linear interpolation, 1)PA output seems to be linear in real domain thus there could be need for a conversion from log domain to real domain 2) We are not quite sure whether linear alone would be sufficient for the whole range of interest including that larger than 23dBm Furthermore, the UL transmission power for FR2 in38.840 (copied as below) is unclear and should be decided before the discussion of scaling rule for FR2 transmission power adaptation. Power State; Characteristics; Relative Power. FR1; FR2. UL; Long PUCCH or PUSCH.; 250 (0 dBm) 700 (23 dBm); 350 (FFS Tx power level). |
| Nokia, NSB | Premature to discuss, no agreement even on whether the UE power consumption is a KPI of interest for assessing the XR and CG performance. |
| FUTUREWEI | Too early to discuss this. |
| DOCOMO | Same comment as previous questions. It should be discussed later. |
| LG | We tend to feel this can be discussed later, but open to discussion. |
| InterDigital | We are open for using the linear interpolation model from TR 38.840 for modelling UE Tx power. However, the details can be discussed later |
| Intel | Premature to discuss |
| Ericsson | It is not clear to us if new power consumption models are in-scope of the SI. If it is in-scope, more discussions are needed to define a proper model. |
| Vivo | Linear interpolation can be a starting point |
| Huawei, HiSilicon | Premature to discuss. |
| CATT | During Rel-16 UE power saving study, the interpolation of UE power consumption should be performed after subtract the minimum power consumption of micro sleep. |
| Facebook | Can be the starting point for further discussion. |
| Samsung | Linear interpolation can be a starting point. |

Summary

- Linear interpolation as a starting point (7)

  - QC, ZTE, vivo, CATT, Facebook, InterDigital, Samsung

- Agree with the necessity of developing model for UE tx power other than 0dBm and 23dBm.  FFS details (1)

  - MTK

- Premature, to be discussed later (6)

  - Nokia, FutureWei, DOCOMO, Intel, HW, LG

Additional UL Slots for Power Evaluation

The UL power model in 38.840 needs to be further improved for UL power evaluation of XR applications. The power model in TR 38.840 has power state for UL (or PUSCH or long-PUCCH) or short-PUCCH (or SRS). However, these two states are not enough to capture different power consumption values generated from various UL activities such as pose/control, scene upload, ACK/NACK for PDSCH, SR, CSI, or SRS especially across different Ues in system level study. In system level study, since multiple Ues’ UL transmissions can be multiplexed in the same slot, additional UL slot modelling is required. For example, in a UL slot with PUSCH+PUCCH+SRS, two Ues could be scheduled: PUSCH for the first UE, and PUCCH+SRS for the second UE. Since the power model in TR 38.840 does not have power state for e.g., PUSCH + gap, or gap+PUCCH+SRS, etc, the current model needs to be improved to capture more diverse slots to more accurately evaluate power consumptions in such cases.

Question 10. Please provide your view on additional UL slot power modeling discussed above.

| Company | View |
| --- | --- |
| QC | The R16 UE PS study was mostly focused on DL side, and UL modelling got less attention than DL. In order to better study UE power consumption for XR, additional UL power modelling is required. We think a new slot of PUSCH(12)+PUCCH(1)+SRS(1) can be considered which supports multi-UE simulation in system level to support PUSCH, PUCCH, and SRS in the same slot. Note that the number inside parenthesis is the # of OFDM symbols for the corresponding channel/signal. Depending on the UL activity of Ues, there are various UL transmissions in a UL slot, e.g., PUSCH only, PUCCH and SRS, etc. The derivatives of the slot (which are generated depending on UE’s activity) are captured in the following table. The basic power numbers for 0dBm and 23dBm are based on existing UL slots in TR 38.840. The baseline slots #1 and #5 are based on TR 38.840. Other slots #2~#4 are estimated based on linear interpolation based on # of active UL symbols considered in each case. #; Power state; Power number; Note. 1; PUSCH(12)+PUCCH(1)+SRS(1); [250] (0dBm) [700] (23dBm); Same as UL power state. 2; PUSCH(12) + PUCCH(1) + Gap(1) PUSCH(12) + Gap(1) + SRS(1); [235] (0dBm) [660] (23dBm); Applied linear interpolation method to get power number for 13 UL symbols case with two end points A and B where A=power number for PUSCH(14) slot, B= power number short-PUCCH(1). 3; PUSCH(12) + Gap(2); [225] (0dBm) [625] (23dBm); Applied linear interpolation method to get power number for 12 UL symbols case with two end points A and B where A=power number for PUSCH(14) slot, B= power number short-PUCCH(1). 4; MS(12) + PUCCH(1) + SRS(1); [90] (0dBm) [248] (23dBm); Applied linear interpolation method to get power number for 2 UL symbols case with two end points A and B where A=power number for PUSCH(14) slot, B= power number short-PUCCH(1) . Assumed that PUCCH and SRS power are the same.. 5; MS(13) + SRS(1); [75] (0dBm) [210] (23dBm); As defined in 38.840, same as short-PUCCH. * PUSCH(X): X symbols of PUSCH * PUCCH(Y): Y symbols of PUCCH * SRS(Z): Z symbols of SRS * MS(N): N symbols of Micro Sleep * Gap(M): M symbols of gap * This is the case for UE’s number of Tx antenna = 1. |
| MTK | We do not really feel the necessity of introducing additional UL slot power modelling since the power value of #1, #2, #3 and #4, #5 are actually close in QC’s suggested table. (#1 is close to #2, #3, and #4 is close to #5) |
| ZTE, Sanechips | We also observe that the UL states in TR38.840 are limited to PUSCH, Long PUCCH, short PUCCH and SRS. We agree to consider more UL power states to precisely simulate the UL UE behavior. And the combinations of UL transmission could be further discussed. |
| Nokia, NSB | Premature to discuss, no agreement even on whether the UE power consumption is a KPI of interest for assessing the XR and CG performance. |
| FUTUREWEI | Too early to discuss this. |
| DOCOMO | Same comment as previous questions. It should be discussed later. |
| LG | We tend to feel this can be discussed later, but open to discussion. |
| InterDigital | We agree that the UL power model needs updating in order to take into account the different types of traffic that can be multiplexed within an UL slot. However, the details can be discussed later |
| Intel | premature to discuss |
| Ericsson | It is not clear to us if new power consumption models are in-scope of the SI. If it is in-scope, more discussions are needed to check the necessity. |
| Vivo | UL slots including multiple types of UL transmissions from one or multiple Ues, e.g. PUSCH+PUCCH, PUSCH+SRS, etc., can be considered for the power evaluation. |
| Huawei, HiSilicon | Premature to discuss. |
| CATT | The principle of power scaling in TR38.840 could be reused although there is no specification on certain channel combination. In TR38.840, the power consumption is based on the average power of one slot. For one symbol transmission/reception, the UE power consumption is one symbol power consumption of specific model averaged with 13 symbols of micro sleep power consumption. |
| Samsung | Deprioritize – the issue is much smaller in relative importance and unclear why it requires additional consideration over TR 38.840 (i.e. what sort of decisive ‘delta’ is expected). |

Summary

- Most of companies think that it is premature, while some indicate the necessity.

Special (S) Slot Power Consumption

The power model in TR 38.840 for special(S) slots is available only for one specific configuration, “PDCCH+PDSCH+PUCCH” assumed to have the same power number as PDCCH+PDSCH state, which may hold only for 0dBm case. The reasoning based on this approximation is that PUCCH tx power at 0dBm is not significantly different from power consumption required for DL reception.  For other tx power level, e.g., 23dBm tx power, this reasoning may not hold any more.  Once S slot’s 23dBm power number is available, then power number for other tx power levels may be estimated by linear interpolation between power number for 0dBm and power number for 23dBm.

Question 11. Please share your view on power consumption model for S slot for XR power evaluation.

| Company | View |
| --- | --- |
| QC | The modelling of S slot is useful in case the TDD frame structure to be evaluated includes S slot. The first step is to define power number for 0dBm and 23dBm. PDCCH+PDSCH+PUCCH+SRS = [300] for 0dBm PDCCH+PDSCH+PUCCH+SRS = [450] for 23dBm Power for 0dBm case is based on PDCCH+PDSCH. Power for 23dBm is based on estimation and PA power measurement data points. Power consumption for different tx power could be computed based on linear interpolation in linear power domain. |
| MTK | We are open to discuss S slot’s 23dBm power number. At the same time, we are curious how to model the SRS periodicity since this depends on NW implementation. |
| ZTE, Sanechips | To reduce the transmission delay, the special slot can be considered in the simulation to enable UE to feedback the HAQR-ACK/pose/position,etc. as soon as possible. We think it is necessary to discuss the power consumption of the special slot. And the combinations of different number of DL and UL symbols should be further discussed. We also agree the power consumption of special slot with different UL transmission power is needed. |
| Nokia, NSB | Premature to discuss, no agreement even on whether the UE power consumption is a KPI of interest for assessing the XR and CG performance. |
| FUTUREWEI | Too early to discuss this. |
| DOCOMO | Same comment as previous questions. It should be discussed later. |
| LG | We tend to feel this can be discussed later, but open to discussion. |
| Interdigital | We are fine for employing linear interpolation between power numbers for 0dBm and 23dBm to approximate the Tx power number for other TX powers. We also think this can be discussed later |
| Intel | Premature to discuss |
| Ericsson | It is not clear to us if new power consumption models are in-scope of the SI. If it is in-scope, more discussions are needed to check the necessity. |
| Vivo | Power model for the S slot could be considered for power evaluation. If introduced, limited models of slot format for the S slot are assumed. |
| Huawei, HiSilicon | Premature to discuss. |
| CATT | Power consumption for S slot should not be much different to normal slot since UE does not have sufficient time to perform micro sleep. This issue was discussed during power model discussion in Rel-16 UE power saving study. |
| Samsung | Similar to Q10. Deprioritize – compared to other aspects, that is a marginal issue. |

Summary

- Most of companies think that it is premature, while some indicate the necessity.

## Evaluation Methodology for Coverage

Table 8 captures the views from different sources on evaluation methodology for coverage evaluation.

Table 8 Views on Evaluation Methodology for Coverage

| Source | View |
| --- | --- |
| [2] | Proposal 6: The CDF of XQI is used to evaluate the coverage performance of the network. |
| [3] | Proposal 15: For XR coverage evaluation, link budget can be adopted as the evaluation methodology, and max isotropic loss (MIL) can be used as the performance metrics. |
| [5] | Proposal 5: Consider the capacity metric and coverage enhancement metric in XR evaluation. - Capacity metric should include both the number/percentage of Ues satisfying the PDB/PER/Rate requirement and the statistical metrics regarding the CDF of UPT. - Coverage metric should use the service based metric i.e. calculate MPL and compare it with the ISD. For the link level simulations used to generate the MPL, the following discussion points should be settled. - For DL, the number of PRBs should be generated by considering the full bandwidth dedicated. TBS should be determined by an agreed MCS. - For UL, the PRBs/TBS/MCS should be set in accordance with the UL traffic. - Target BLER should be set in accordance with the traffic, e.g. 10-3 or 10-6. |
| [6] | Proposal 3: The capacity and coverage can be evaluated by the number of Ues and X% Ues that meet the requirement. |

To better manage RAN1 workload for XR evaluations for NR, it is proposed by the rapporteur in [19] to defer detailed discussion on evaluation methodology for XR mobility to 2021 Q2.  The delayed start will also benefit from stable evaluation methodology and assumptions available for capacity and power evaluations.

Question 12. Please share your view on delay of discussion on evaluation methodology for XR mobility to 2021 Q2.  Also please feel free to comment on any other aspects w.r.t. XR mobility coverage evaluation.

| Company | View |
| --- | --- |
| QC | Defer mobility discussion to 2021 Q2. |
| MTK | Defer mobility discussion to 2021 Q1 or Q2 seems fine. |
| ZTE, Sanechips | There seems to be a typo in the question. We think coverage related discussion should be prioritized and we prefer to re-use the coverage related metrics/methodologies in the NR coverage enhancement study item such as MPL and ISD comparison. This is of interest of XR services given a large number of companies expressed interest towards scenarios such as Umi and Uma. Considering the high data rate requirement of typical XR services, coverage aspect is quite challenging and vital for the smooth delivery. |
| Nokia, NSB | Should the question be about coverage as the text above seems to belong to section 5 on mobility? Mobility evaluation need and methodology discussion can be deferred to a later meeting. Coverage evaluation may not necessitate its own evaluation, as the capacity evaluation can produce a percentage of the users served in a given deployment scenario that is indicative of the service coverage. Anyway, we can defer discussion on whether a specific coverage evaluation is needed to a later meeting (after we have established the capacity evaluation methodology) Notably neither mobility nor coverage are mentioned in the SID objectives and we should first start by agreeing if one of the two or both quality as a relevant KPI to measure the XR and CG performance. 3. Identify evaluation methodology to assess XR and CG performance along with identification of KPIs of interest for relevant deployment scenarios |
| FUTUREWEI | Defer the discussion to Q2 after KPI and traffic model discussions. |
| Apple | Coverage is automatically included in the evaluation assumption already, e.g. indoor, Umi, no separate evaluation is necessary. |
| DOCOMO | Agree with the proposal. |
| LG | We are fine with delaying both mobility aspect and coverage aspect (there seems to be mismatch between section and proposal in the FL paper. Please check. ) Especially regarding mobility aspect, we are not sure whether/how RAN1 should perform evaluation in the SI. |
| InterDigital | We support delaying the discussion on methodology for XR coverage evaluation to 2021 Q1 |
| Intel | yes, we can defer |
| Ericsson | We agree mobility evaluation can be deferred. Contents of section 4 and 5 seem to be mixed up. |
| Vivo | Fine to defer the discussion of XR coverage evaluation to keep a reasonable evaluation workload. |
| Xiaomi | Agree |
| Huawei, HiSilicon | Ok to defer mobility discussion. (PS: it seems Section 4 refers to coverage) |
| CATT | We prefer to defer mobility discussion. |
| Facebook | Agree can be deferred. |
| Samsung | Fine to defer mobility discussion to 2021 Q2. |

## Evaluation Methodology for Mobility

Table 9 shows the captured views from different sources on evaluation methodology on mobility evaluation.

Table 9 Views on Evaluation Methodology for Mobility

| Source | View |
| --- | --- |
| [3] | Proposal 16: For XR mobility evaluation, performance metrics should be identified considering impacts on XR performance due to mobility, such as interruption delay, handover failure rate and cell-edge transmission performance. |
| [6] | Proposal 4: Different UE speeds for VR, AR and Cloud Gaming users can be assumed when evaluate the mobility. |

To better manage RAN1 workload for XR evaluations for NR, it is proposed by the rapporteur in [19] to defer detailed discussion on evaluation methodology for XR coverage to 2021 Q1.  The delayed start will also benefit from stable evaluation methodology and assumptions available for capacity and power evaluations as well as available outcome of Rel-17 coverage enhancement study.

Question 13. Please share your view on delay of discussion on evaluation methodology for XR coverage to 2021 Q1.  Also please feel free to comment on any other aspects w.r.t. XR coverage evaluation.

| Company | View |  |
| --- | --- | --- |
| QC | Defer coverage discussion to 2021 Q1. |  |
| MTK | Since there are related discussion of coverage enhancement in R17 RedCap UE and R17 coverage enhancement, we think coverage can be further deferred after the discussion of mobility. |  |
| ZTE, Sanechips | There seems to be a typo in the question. We think mobility related discussion should be prioritized and we prefer to initiate the study by considering different UE speeds in the system level simulation. The capacity metrics could be re-used to reflect the impact in terms of mobility. |  |
| Nokia, NSB | Should the question be about mobility as the text above seems to belong to section 4 on coverage? Mobility evaluation need and methodology discussion can be deferred to a later meeting. Coverage evaluation may not necessitate its own evaluation, as the capacity evaluation can produce a percentage of the users served in a given deployment scenario that is indicative of the service coverage. Anyway, we can defer discussion on whether a specific coverage evaluation is needed to a later meeting (after we have established the capacity evaluation methodology) Notably neither mobility nor coverage are mentioned in the SID objectives and we should first start by agreeing if one of the two or both quality as a relevant KPI to measure the XR and CG performance. 3. Identify evaluation methodology to assess XR and CG performance along with identification of KPIs of interest for relevant deployment scenarios |  |
| FUTUREWEI |  | Defer the discussion to Q2 after KPI and traffic model discussions. |
| Apple | Mobility consideration can be included in the UE speed assumption already, e.g. 3 km/hr. |  |
| DOCOMO | Agree with MTK. |  |
| LG | We are fine with delaying both mobility aspect and coverage aspect (there seems to be mismatch between section and proposal in the FL paper. Please check. ) Especially regarding mobility aspect, we are not sure whether/how RAN1 should perform evaluation in the SI. |  |
| InterDigital | We support delaying the discussion on methodology for XR mobility evaluation to 2021 Q1 |  |
| Intel | We are fine to defer |  |
| Ericsson | We agree coverage evaluation can be deferred. Contents of section 4 and 5 seem to be mixed up. |  |
| vivo | Fine to defer the discussion of XR mobility evaluation to keep a reasonable evaluation workload. |  |
| Xiaomi | Agree |  |
| Huawei, HiSilicon | Ok to defer coverage discussion. |  |
| CATT | Agree to defer coverage discussion |  |
| Facebook | Agree with the proposal |  |
| Samsung | Fine to defer – should be for later than 2021 Q1 considering the meeting schedule and progress in relevant R17 WIs. |  |

Summary

- Most of companies agree to defer discussion on mobility and coverage to a later stage.

Question 14. Please feel free to comment on any other aspects w.r.t. XR evaluations for NR that are not addressed above.

| Company | View |
| --- | --- |
| MTK | There could be intensive simulation effort for XR agenda due to the various user number, performance requirement, and system aspects. RAN1 should strive to limit the simulation efforts to a reasonable level. |
| ZTE, Sanechips | In addition to the power consumption methodology discussed in section 3, the following aspects should also be considered. - For FR2, the system bandwidth is 100MHz for both DL and UL in 38.840, while in the XR simulation, the bandwidth for FR2 is assumed to be 400MHz. Since there is no scaling rule for FR2 BWP adaptation, we need to discuss it as well. - The number of transmission antenna is 1 in 38.840, while in XR simulation assumption, the Tx configuration is 2Tx or 4Tx. The scaling factor for Tx adaptation in TR38.840 is not sufficient and should be discussed. Reference Configuration for FR1(in 38.840): Uplink: TDD - Tx antenna configuration: 1TX, - Power levels: 0dBm and 23dBm Reference configuration for FR2 (in 38.840): Downlink: TDD, FR2 -.... - System Bandwidth: 100 MHz .... - Uplink: TDD, FR2 - .... - Number of carrier = 1CC - System Bandwidth = 100MHz - Tx antenna configuration: 1TX chain. |
| Nokia, NSB | In order to keep the simulation effort to a reasonable level, RAN1 should identify the smallest subset of traffic models, deployment scenarios, systems aspects and KPIs that closely capture the essential aspects of XR and CG applications and avoid work duplication with other SID/WID. |
| Samsung | As previously commented, the questions so far were heavily focused on UE power consumption. Evaluations of mechanisms that enable achieving the KPIs, such as enhancements to CG-PUSCH, M-TRP, DAPS, ICIC, power control, … should be considered even if/when corresponding evaluations are to be deferred. |

Summary

- Strive to keep the simulation effort to a reasonable level, e.g., identifying a reasonable number of simulation configurations (traffic models, deployment scenarios, and so forth)

- For FR2, the system bandwidth is 100MHz for both DL and UL in 38.840, while in the XR simulation, the bandwidth for FR2 is assumed to be 400MHz. Since there is no scaling rule for FR2 BWP adaptation, we need to discuss it as well.

- Evaluations of mechanisms that enable achieving the KPIs, such as enhancements to CG-PUSCH, M-TRP, DAPS, ICIC, power control, … should be considered even if/when corresponding evaluations are to be deferred.

# Round 2 Email Discussion for Others

## XR applications and traffic model

Based on the majority view from the 1st round of email discussion that is summarized in Section 3, the following proposals are made.

Proposal on XR applications

RAN1 confirms that diverse applications of VR1/2, AR1/2, CG [TR26.928] are of interest for study. Prioritization/down selection of these applications for evaluation, if needed, is to be discussed after detailed traffic model and evaluation assumptions are determined.

Round 2 Question 1: Please share if you have any comments on this proposal.

| Company | View |  |
| --- | --- | --- |
| QC | Agreed with the proposal |  |
| AT&T | Agree |  |
| Apple | ITT4RT and 5GSTAR should also be included. |  |
| CATT | Agree with moderator’s proposal |  |
| MTK | Conditionally agree by removing “if needed” because there is no clear criterion to justify “if needed” |  |
| LG | Agree. In addition, we think “if needed” is necessary. Otherwise, “Potential” prioritization/down selection… would be also relevant. |  |
| ZTE,Sanechips |  | OK |  |
| Somy | Agree |  |
| InterDigital | We agree with the proposal |  |
| Nokia, NSB | In general, we agree with the proposal. We propose to slightly modify the sentence: “Prioritization/down selection of these applications for evaluation, if needed, is to be discussed after detailed traffic models and evaluation assumptions are determined.” (Note: “evaluation assumptions” may be too general here, we don’t see a strong need to define all the evaluation assumptions before the prioritization). |  |
| Ericsson | Fine with this proposal. |  |
| Huawei, HiSilicon | Agree |  |
| Facebook | Agree |  |
| FUTUREWEI | Agree |  |
| DOCOMO | Agree |  |
| vivo | Agree with moderator’s proposal |  |
| Samsung | OK with proposal and agree with MTK on the ‘if needed’. |  |

Proposal on Traffic model

Traffic model for DL and UL should reflect various bit rates, variable frame/slice/file/packet (definition of frame/slice/file/packet to be clarified with traffic model as necessary) size, and periodicity (how to model jitter is FFS), where statistical model is preferred.  Conclusion on detailed traffic model is deferred to the next RAN1 meeting (RAN1 104-e) where SA4 outcome on traffic model is expected to be available.

Round 2 Question 2: Please share if you have any comments on this proposed agreement.

| Company | View |  |
| --- | --- | --- |
| QC | Agreed with the proposal |  |
| AT&T | Agree |  |
| Apple | If the SA4 traffic model is expected to become available the next year, wonder a little bit whether we have time to conclude the traffic model so soon. Maybe we can say “strive to conclude the detailed traffic model at the next RAN1 meeting (RAN1 #104-e) …” |  |
| CATT | Traffic model defined in SA4 is an end-to-end traffic model, which can not be directly used in RAN1 evaluation. In general, we agree the data rate, delay, and jitter should be taken into account in the traffic model. That is where the Queueing theory was used in general (e.g. FTP traffic model in RAN1 evaluation). |  |
| MTK | Agree with the proposal |  |
| LG | We agree with the general concept. On the other hand, details can be considered as example, and the target date for RAN1 decision may not be promising. Therefore, we may say, e.g.: “Traffic model for DL and UL should reflect various aspects, e.g., bit rates, variable frame/slice/file/packet (definition of frame/slice/file/packet to be clarified with traffic model as necessary) size, and periodicity (how to model jitter is FFS), where statistical model is preferred. Conclusion on detailed traffic model is deferred to the next RAN1 meetings (RAN1 104-e) where SA4 outcome on traffic model is expected to be available” |  |
| ZTE,Sanechips |  | OK |  |
| Sony | Agree .Further discuss in the coming meeting(s) based on SA4 input. |  |
| InterDigital | We are ok with the proposal. While the conclusion on the detailed traffic model can be deferred to the next RAN1 meeting, given its flexibility we also think the traffic model for DL and UL, as proposed by FL, with variable frame/packet size, variable periodicity, etc. can be considered as baseline. |  |
| Nokia, NSB | We suggest prioritizing the detailed discussions and possible decisions on traffic models at the beginning of the following RAN1 meeting (104-e), regardless if the complete SA4 input is available by that time or not. The rationale here is that: (i) the decision on traffic models holds many other crucial decisions; and (ii) we share the Apple's concern that, regardless of how hard we aim for conclusions, it may take longer than one RAN1 e-meeting to conclude on the traffic models. Therefore, the discussion on this important topic should start in the beginning of the next meeting, while we also agree with Apple that there should not be a mandatory requirement to conclude on the traffic models within a single meeting. We also agree that the traffic model for both UL and DL should reflect certain degree of variability, so it adjusts to the different applications and scenarios. In order to keep variability to a reasonable level, there should be an aim to reduce the total number of traffic models. If applicable, we should aim at sharing traffic models across different applications (e.g., video traffic can potentially be characterized similarly in different setups). |  |
| Ericsson | Fine with this proposal. |  |
| Huawei, HiSilicon | So far, we think frame and packet may be relevant. And agree the definition of frame/packet may need further discussion. But we are still unclear about the meaning of slice/file, and are not sure whether they are relevant to RAN1 or not. Since RAN1 will wait for SA4 outcome, maybe just mentioning “frame/frame” is enough at this stage. So we suggest the following changes: Proposal on Traffic model Traffic model for DL and UL should reflect various bit rates, variable frame/slice/file/packet (definition of frame/slice/file/packet to be clarified with traffic model as necessary) size, and periodicity (how to model jitter is FFS), where statistical model is preferred. Conclusion on detailed traffic model is deferred to the next RAN1 meeting (RAN1 104-e) where SA4 outcome on traffic model is expected to be available. And we also suggest to prioritize the discussion of traffic models in RAN1#104-e since other aspects like evaluation methodology depends on this. |  |
| Facebook | Fine with the proposal |  |
| FUTUREWEI | Prefer to revise the proposal based on the changes from LG and Huawei. |  |
| DOCOMO | Agree with the proposal in principle but share same concern with Apple. Fine with the suggested description from Apple as it would be difficult to conclude at the next meeting. We may need time to identify RAN1 level traffic model from the SA4 E2E traffic model. |  |
| vivo | Agreed with the proposal |  |
| Samsung | Fine with the proposal. |  |

## KPIs for evaluation of XR performance over NR

The following four key performance aspects of XR evaluations for NR are presented in the SID on XR evaluations for NR (RP-201145)

- Power consumption

- Capacity

- Mobility

- Coverage

The objectives of the SI include identification of KPIs and evaluations towards characterization of identified KPIs.  While detailed KPIs and corresponding evaluation methodology related to “capacity” are being separately discussed in Track 2, Track 1 focuses on KPIs and evaluation methodology for power consumption, mobility, and coverage.

The questionnaire in the 1st round of Track 1 email discussion asked several questions regarding evaluation methodology for power consumption under the assumption that “power consumption” is one of the KPIs for XR performance over NR according to the SID.  However, from the 1st round of Track 1 email discussion, there seem to be different views on whether UE power consumption is a KPI to be evaluated (see Section 3 for more detail):

- Is UE power consumption evaluation part of the study, i.e., is UE power consumption a KPI?

  - Yes (9): QC, MTK, Apple, InterDigital, vivo, Xiaomi, CATT, Facebook, Samsung

  - No (2): Nokia, HW

  - Need further discussion (7): Nokia, LGE, ZTE, FutureWei, DOCOMO, Intel, HW

  - It’s up to companies to contribute.

    - Ericsson, HW

Thus, it is desirable to confirm whether power consumption evaluation is part of the study, i.e., UE power consumption is a KPI.

Round 2 Question 3:

Can we confirm that XR UE power consumption is part of the study as a KPI for XR evaluations for NR?

  - Yes

  - No

Please share your view on Question 3.

| Company | View |  |
| --- | --- | --- |
| QC | Yes |  |
| AT&T | It is important to understand impact of XR on UE power consumptions. However, we believe more discussion is needed to define it as a KPI. |  |
| Apple | Yes |  |
| CATT | Yes. UE power consumption should be one key performance index of XR study |  |
| MTK | Yes |  |
| ZTE,Sanechips |  | Yes |  |
| Sony | Support, FFS the details |  |
| InterDigital | Yes |  |
| Nokia, NSB | No. It is unclear at this stage whether the power consumption should be included as a KPI for this SI. Even though, we observe some support to consider power consumption as an interesting metric to look at, we also see and share some doubts from the companies on if an ultimate decision is too early to make. In case there is a decision to consider this as a KPI, we believe it is absolutely premature to start discussing the detailed aspects of power consumption at this stage. The discussion on the power-related assumptions/evaluation methodology/etc. will take a considerable amount of time and effort. Therefore, it is more beneficial to defer this decision to later meetings, e.g., when some more critical agreements (traffic models, scenarios, capacity evaluation assumptions) are made and then explore how much time/resources this SI has left for the UE power consumption. We thus suggest rephrasing the proposal as "RAN1 is to discuss whether/how to study/evaluate UE power consumption at a later stage, e.g., starting from Q2 2021 after some maturity on scenarios and services has been achieved". |  |
| Ericsson | Our view remains the same: it’s up to companies to contribute. |  |
| Huawei, HiSilicon | We think the importance of UE power consumption may depend on the applications, traffic model, etc. With such things unclear, we think it’s premature to decide power consumption is a KPI or not. So we suggest to wait for the discussion on applications, traffic model, etc. |  |
| Facebook | Yes, and we agree that this is a KPI for XR study. |  |
| FUTUREWEI | We prefer to discuss this at later stage after traffic model and performance metric is clear. If evaluation for power consumption is carried out, it should include relevant features developed in R17 as part of baseline. |  |
| DOCOMO | Agree with HW/HiSi’s comments. It may be better to wait for the discussion on applications and traffic models. |  |
| vivo | Yes |  |
| Samsung | Yes, UE power consumption should be a KPI for the study. However, ensuring the main KPIs, as defined by the corresponding XR models, should be prioritized. Discussion would be needed to balance trade-offs for multiple KPIs. |  |

Regarding mobility and coverage, from the 1st round of Track 1 email discussion, most of companies agree with the following proposal.

Proposal on mobility and coverage: RAN1 is to discuss whether/how to study/evaluate mobility and coverage at a later stage, e.g., starting from Q1 2021.

Round 2 Question 4: Please comment on this proposal if any.

| Company | View |
| --- | --- |
| QC | Agree with the proposal. |
| AT&T | Mobility and coverage are important parts of the study in our view, but we are OK to defer discussion to Q1 2021. |
| Apple | Agree with the proposal |
| CATT | Agree with moderator’s proposal |
| MTK | Agree with the proposal. |
| LG | Agree with the proposal. |
| ZTE, Sanechips | OK for progress |
| Sony | Agree with the proposal |
| InterDigital | We agree to defer the discussion to Q1 2021 |
| Nokia, NSB | We agree with this proposal. |
| Ericsson | Fine with this proposal. |
| Huawei, HiSilicon | We think mobility and coverage are also important for XR study. However, we are fine to defer mobility/coverage discussion. |
| Facebook | We agree with this proposal |
| FUTUREWEI | Agree to defer the discussion |
| DOCOMO | Agree with the proposal |
| vivo | Agreed with the proposal |
| Samsung | OK to start in Q1 but the suggestion to start “at a later stage” seems to indicate otherwise. |

## Evaluation methodology for power consumption

This section presents follow-up questions on evaluation methodology for power consumption based on companies’ views collected from 1st round discussion.

Regarding baseline power evaluation methodology, the following proposal is made based on the majority view from the 1st round, summarized in Section 3.

Proposal on baseline power evaluation methodology

If UE power consumption is agreed as a KPI for evaluation of XR performance over NR, TR38.840 is the baseline methodology potentially with some modifications if necessary.

- FFS whether/how to model UE power consumption for UE tx power other than 0dBm and 23dBm

- FFS whether/how to model UE power consumption for UL slots that are not defined in TR38.840

- FFS whether/how to model UE power consumption for ‘S’ slot

- FFS whether/how to model UE power consumption for 400MHz in FR2 including scaling rule for FR2 BWP adaption.

Round 2 Question 5: Please share if you have any comments on this proposed agreement.

| Company | View |
| --- | --- |
| QC | Agree with the proposal. TR38.840 can be the baseline with necessary modification. We are fine with further discussion on the above bullets although we have already presented our view on the detailed evaluation methodology for the first three bullets. |
| AT&T | We have concerns about a very complicated modeling effort on the ability to complete the SI and compare results |
| Apple | The proposal is fine, but our understanding is there may be other modifications necessary, hence the FFS list may be further discussed in future RAN1 meetings. |
| CATT | In general, the power model for those FFS points were discussed during Rel-16 UE power saving study but considered not much related. We don’t see the need to have FFS points. |
| MTK | We are fine with the proposal. For the last FFS, according to 38.840 Table 21 … 4CC is 3.4x1CC |
| LG | We agree with the main bullet point. On the other hand, we may not need to define such detailed FFS points in this meeting unless we have sufficient discussion on those points. |
| ZTE, Sanechips | Prefer to add the following bullets to the proposal: Proposal on baseline power evaluation methodology If UE power consumption is agreed as a KPI for evaluation of XR performance over NR, TR38.840 is the baseline methodology potentially with some modifications if necessary. - FFS whether/how to model UE power consumption for UE tx power other than 0dBm and 23dBm - FFS whether/how to model UE power consumption for UL slots that are not defined in TR38.840 - FFS whether/how to model UE power consumption for ‘S’ slot - FFS whether/how to model UE power consumption for 400MHz in FR2 including scaling rule for FR2 BWP adaption, the evaluations for FR2 is less prioritized than those in FR1 - FFS whether/how to model UE consumption for the corresponding number of Tx antennas - FFS whether/how to model the UE power consumption for UE tx power under FR2, the evaluations for FR2 is less prioritized than those in FR1 The reasons are : 1) We feel the power consumption evaluations should be aligned with that in capacity. In capacity, we believe the majority view is to have a clear prioritization of FR1 w.r.t FR2. This should be consistent here. 2)The FL proposal for # of Tx antennas is 4, whose power consumption has not been modeled. Prefer to discuss adaptations here. 3) The power consumption under FR2 has not taken into account the actual UE transmitting power. We believe adaptation according to UE transmitting power is necessary in terms of discussion of FR2 power model. |
| Sony | Further discussions needed at a later stage |
| InterDigital | We agree with the proposal |
| Nokia, NSB | If power consumption is defined as a KPI, then the TR.38840 could be used as a baseline. We should, however, be careful not to overcomplicate the model by adding too many modifications since this might significantly increase the effort required for the KPI evaluation. We also share the AT&T’s concern about a very complicated modeling effort on the ability to complete the SI and compare results. If many modifications are required, then perhaps we should not focus on power consumption as a KPI at this stage. We also share the other companies' opinion that FFS should be removed at this stage. |
| Ericsson | Fine with this proposal. |
| Huawei, HiSilicon | Whether to take up RAN1 time on this depends on the answer to Question 3, i.e. whether power saving is in scope or not. Until then, this proposal is jumping ahead of the situation. We share similar view with other companies that we need to avoid an overcomplicated model considering the limited TU. And it seems there are not enough discussions on whether the FFS points are really relevant or not. |
| Facebook | We are fine with the proposal. |
| FUTUREWEI | Suggest to defer the discussion to later stage after the group agree to carry out power consumption evaluation in this SI. |
| DOCOMO | Fine with the proposal |
| vivo | Agree with TR 38.840 as the baseline methodology with modification if necessary. In addition to the FFSs, other modifications are not precluded. |
| Samsung | Fine with the proposal. |

It would not be feasible for RAN1 to evaluate the absolute amount of XR UE power consumption, e.g., in mW or mA.  Please be reminded that the power evaluation methodology in TR 38.840 defines only relative numbers of power consumption compared to ‘deep sleep state’ for different slot states (e.g., PDCCH decoding only, decoding PDCCH and PDSCH, etc.).  Therefore, when it comes to XR power consumption, only “relative comparison” can be evaluated, e.g., evaluation of XR power saving (PS) gain that a certain power saving technique can provide, compared to the power consumption of a reference scheme/scenario.  Therefore, in order to evaluate XR PS gain of certain PS techniques, it is critical to clearly define “reference” scenarios or schemes.  In this regard, several questions were asked in the 1st round discussion for which companies’ views are summarized in Section 3.   Based on the majority view, the following proposal is made.

Proposal. Power Saving Evaluation Scenarios/Techniques

If UE power consumption is agreed as a KPI for evaluation of XR performance over NR, the following scenarios are to be evaluated.  Please note Scenario 3 is not mandatory, but optional.

- Scenario 1 (Baseline/reference): No PS technique is applied, i.e., UE assumed to be always available for DL/UL scheduling/transmission, i.e., not in a sleep state.  UE power consumption in this scenario is the reference to evaluate gain of any power saving technique.

  - Note: Results of baseline/reference power consumption can be collected from capacity simulations, i.e., no separate system level simulation would be needed from capacity simulation.

- Scenario 2 (Evaluation of PS Gain): With a certain PS technique applied, evaluate the PS gain of the PS technique over the baseline/reference evaluated in Scenario 1.

  - FFS what power saving techniques are to be evaluated/prioritized.

    - Rel-15/16 schemes are the baseline (FFS down-selection/prioritization)

    - It is up to company to evaluate other PS schemes beyond Rel-15/16 schemes.

  - FFS the exact definition of power saving gain subject to a constraint on capacity degradation from the applied PS scheme(s).

- Scenario 3 (Genie Power Consumption): “Genie” UE power consumption, defined as defined as UE power consumption for the case when the UE is assumed to enter a sleep state in all the slots with neither DL reception nor UL transmission.  The Genie result is informative, not mandated, and can serve as a benchmark of the power saving gain.

  - Note: Results of Genie power consumption can be collected from capacity simulations, i.e., no separate system level simulation would be needed from capacity simulation.

Round 2 Question 6: Please share your view on this proposal.

| Company | View |  |
| --- | --- | --- |
| QC | We agree that a clear definition of the reference scheme/scenario is needed to evaluate the power saving gain of power saving techniques and/or to compare effectiveness of different PS schemes. In addition, we think the “Genie” result will be very informative as it can present an upper bound for many power saving techniques. |  |
| AT&T | Prefer Scenario 1, although companies can of course optionally provide results with certain Rel-15/16 schemes implemented for reference as well |  |
| Apple | Whether system level simulation is used for power saving or not should be further discussed. The notes to scenario 1 & scenario 3 should be removed, whether capacity and power saving are studied jointly should be further studied. |  |
| CATT | We are OK to have scenarios 1 and 2. Scenario 3 is the upper bound and should be optional |  |
| MTK | As there are various power saving methods in R15/R16, some of them may also be applicable to XR with negligible capacity impact. Therefore, it would be better to include those existing R15/R16 power saving techniques as baseline for this agenda. To achieve this, for scenario 1 we suggest to replace the word “baseline/reference” with “No power saving”. For scenario 2, we suggest to modify it as: - Scenario 2 (Evaluation of PS Gain): With a certain PS technique applied, evaluate the PS gain of the PS technique over the baseline/reference no power saving scenario evaluated in Scenario 1. - FFS what power saving techniques are to be evaluated/prioritized. - Rel-15/16 schemes are the baseline (FFS down selection/prioritizationcombinations to be studied) - It is up to company to evaluate other PS schemes beyond Rel-15/16 schemes - FFS: the baseline for calculating power saving gain to include Rel-15/16 PS schemes applicable to XR/CG. - FFS the exact definition of power saving gain subject to a constraint on capacity degradation from the applied PS scheme(s). |  |
| LG | In general, methodology for power consumption evaluation can be discussed after we conclude if power consumption should be KPI of the study item. Regarding methodology itself, we think Rel-15/16 power saving techniques can be included as reference to avoid confusion in interpretation of companies’ results on the proposals beyond Rel-15/16 schemes. |  |
| ZTE,Sanechips |  | Partially agreed. The metric for scenario 2 may cause huge simulation burden by coupling the capacity and power consumption gain as a metric in the first place. Given a threshold for capacity degradation, e.g. 10%, a wide range of system level simulations would need to be performed by tuning the configurations of the evaluated power saving scheme such that the power saving gain is reported under such a constraint. We can accept to evaluate different PS schemes as in scenario 2 but we would prefer less burden on simulations by simply reporting the capacity degradation and the power saving gain at the same time. The threshold on capacity could be discussed once all the results on {capacity degradation, power consumption gain} is ready. |  |
| InterDigital | We are generally ok with the proposal. In the interest of reducing workload, it is beneficial to strive for using the results from capacity simulations for power consumption evaluations. This also implies taking into consideration of the possible tradeoff between capacity and power consumption when applying the baseline technique for power consumption. |  |
| Nokia, NSB | Premature to discuss. |  |
| Ericsson | On Scenario 2, evaluation of PS gain: the evaluation of new techniques should be compared with existing power saving techniques. In other words, even if a new technique shows gain vs. Scenario 1, it may not be needed, as the gain may be realized by existing power saving techniques already. So, we suggest the following update in red: - Scenario 2 (Evaluation of PS Gain): With a certain PS technique applied, evaluate the PS gain of the PS technique over the baseline/reference evaluated in Scenario 1. - FFS what power saving techniques are to be evaluated/prioritized. - Rel-15/16 schemes are the baseline (FFS down-selection/prioritization) - It is up to company to evaluate other PS schemes beyond Rel-15/16 schemes. If PS scheme beyond Rel-15/16 is evaluated, then evaluations include both PS gain over baseline/reference evaluated in Scenario 1, and PS gain over Rel15/16 scheme. - FFS the exact definition of power saving gain subject to a constraint on capacity degradation from the applied PS scheme(s). |  |
| Huawei, HiSilicon | We think this is premature to discuss. For power evaluation, whether to perform system level simulation (SLS) or link level simulation (LLS) needs further discussion. For example, maybe LLS is enough to evaluate the trade-off between power consumption and user experience. This may depend on the discussion on evaluation methodology, traffic requirement, etc. |  |
| Facebook | In general, we support the proposal and agree with the definition of the 3 scenarios. We consider it’s important to develop both the upper bound and lower bound on the power consumption performance. |  |
| FUTUREWEI | Too early to discuss. Also, PS related features developed in R16/17 should all be considered as part of baseline if evaluation is to be done. |  |
| DOCOMO | We are fine with the proposal but it seems premature to discuss at this stage. |  |
| vivo | Scenario 1 and scenario 2 are necessary. Scenario 3 is optional but it can be a reference. |  |
| Samsung | The proposal is reasonable to consider for UE power savings evaluations. |  |

Round 2 Question 7:

If UE power consumption is a KPI and system level simulation is the baseline or an optional methodology for evaluation of UE power consumption for XR over NR, system level simulation assumptions for power evaluation are the same as in XR capacity evaluation that are being developed in the Track 2 email discussion (moderated by vivo).

Please share your view on Question 7.

| Company | View |  |
| --- | --- | --- |
| QC | We agree with using the same simulation parameters for capacity evaluation and power evaluation. |  |
| AT&T | Agree |  |
| Apple | System level simulation is an optional methodology for power saving evaluation. |  |
| CATT | UE power consumption could be calculated during system level simulation. We need to agree whether the UE power consumption is captured with average of all UE power consumption or the distribution associated with geometry of UE location. |  |
| MTK | Agree with the proposal |  |
| LG | We want to see more companies’ opinions on the KPI itself. Anyhow, we agree with the proposal under the condition on the KPI as such. |  |
| ZTE,Sanechips |  | Agreed. But this agreeing to this would require the discussion of the power model for 4 Tx antennas as in our response to Q5. |  |
| InterDigital | We agree with the proposal |  |
| Nokia, NSB | Premature to decide. We should first define whether power consumption will be considered as a KPI. If UE power consumption is agreed as a KPI, we would also like to first decide on the simulation assumptions for capacity evaluations and only then decide on what kind of additional parameters or modifications (if any) are needed to also capture the UE power consumption. For instance, there might be a case, where the UE power consumption evaluation is important only for a subset of considered scenarios/applications. |  |
| Ericsson | We think this proposal is not needed for now. It is conditional on too many factors. This discussion can be postponed. |  |
| Huawei, HiSilicon | We think this is premature to discuss. For power evaluation, whether to perform system level simulation (SLS) or link level simulation (LLS) needs further discussion. For example, maybe LLS is enough to evaluate the trade-off between power consumption and user experience. This may depend on the discussion on evaluation methodology, traffic requirement, etc. |  |
| Facebook | Agree |  |
| FUTUREWEI | Too early to discuss. |  |
| DOCOMO | Agree in principle. |  |
| vivo | Agree that the system-level simulation assumptions for capacity evaluation and power consumption evaluation need to be aligned. |  |
| Samsung | The proposal is reasonable but is too early. In general, although UE PS is important, there seems to be too much focus on it. There are several other issues to consider in meeting the core KPIs. |  |

# Reference

- RP-201145		Revised SI on XR Evaluations for XR