3GPP TSG RAN WG1 Meeting #103-e		R1-200XXXX

e-Meeting, October 26th – November 13th, 2020

Title:	Session notes for 8.1 (Further enhancements on MIMO for NR)

Source:	Ad-hoc chair (Samsung)

## Further enhancements on MIMO for NR

Please refer to RP-202024 for detailed scope of the WI

### Enhancements on Multi-beam Operation

Mainly targeting FR2 while also applicable to FR1

R1-2008147	Moderator summary for multi-beam enhancement	Moderator (Samsung)

R1-2007546	Enhancement on multi-beam operation	FUTUREWEI

R1-2007586	Enhancements on multi-beam operation in Rel-17	Huawei, HiSilicon

R1-2007626	Discussions on Multi-beam Enhancement	InterDigital, Inc.

R1-2007644	Further discussion on multi beam enhancement	vivo

R1-2007763	Enhancements on Multi-beam Operation	ZTE

R1-2007824	Discussion on enhancement on multi-beam operation	CATT

R1-2008000	Enhancements on multi-beam operation	CMCC

R1-2008148	Multi-Beam Enhancements	Samsung

R1-2008217	Enhancements on Multi-beam Operation	OPPO

R1-2008308	Enhancements on NR multi-beam operation	AT&T

R1-2008346	Considerations on the enhancement of multi-beam operation	Sony

R1-2008438	Views on Rel-17 Beam Management enhancement	Apple

R1-2008573	Enhancements on Multi-beam Operation	LG Electronics

R1-2008899	Enhancements on multi-beam operation	Fraunhofer IIS, Fraunhofer HHI

R1-2008903	Enhancements on Multi-beam Operation	Nokia, Nokia Shanghai Bell

R1-2008910	Enhancements on Multi-beam Operation	Lenovo, Motorola Mobility

R1-2008943	Discussion on multi-beam operation	NEC

R1-2008956	Enhancement on multi-beam operation	MediaTek Inc.

R1-2008977	Enhancements to Multi-Beam Operations	Intel Corporation

R1-2009027	Enhancements on multi-beam operation	Xiaomi

R1-2009060	Discussion on Enhancements for Multi-beam Operation	Asia Pacific Telecom co. Ltd

R1-2009129	Enhancements on multi-beam pperation	Sharp

R1-2009141	Enhancements on Multi-beam Operation	Spreadtrum Communications

R1-2009155	Discussion on multi-beam operation	ASUSTeK

R1-2009158	Multi-beam Enhancements	Convida Wireless

R1-2009174	Discussion on multi-beam operation	NTT DOCOMO, INC.

R1-2009250	Enhancements on Multi-beam Operation	Qualcomm Incorporated

R1-2009288	Enhancements on Multi-beam Operation	Ericsson

[103-e-NR-feMIMO-01] Email discussion on enhancements on multi-beam operation – Eko (Samsung)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

R1-2008147	Moderator summary for multi-beam enhancement	Moderator (Samsung)

Agreement

On beam indication signaling medium to support joint or separate DL/UL beam indication in Rel.17 unified TCI framework:

- Support L1-based beam indication using at least UE-specific (unicast) DCI to indicate joint or separate DL/UL beam indication from the active TCI states

  - The existing DCI formats 1_1 and 1_2 are reused for beam indication

  - Support a mechanism for UE to acknowledge successful decoding of beam indication

    - The ACK/NAK of the PDSCH scheduled by the DCI carrying the beam indication can be used as an ACK also for the DCI

    - FFS: Whether any additional specification support is needed

- Support activation of one or more TCI states via MAC CE analogous to Rel.15/16:

  - At least for the single activated TCI state, the activated TCI state is applied

  - The content for the MAC CE is determined based on the outcome of issue 1

  - FFS: If supported, default TCI state when more than one TCI states are activated by MAC CE

  - Note: There is no implications on the support of single TRP or multi-TRP

- Support a UE capability for the minimum beam indication delay

  - FFS: Whether to measure beam indication delay from DCI reception or from acknowledgment of DCI

  - FFS: The exact supported values e.g. {0.5ms, 2ms, 3ms}

- FFS: Additional enhancement such as L1-based beam indication with group-common DCI

- FFS: Whether the Rel.17 beam indication can also apply to beam indication for single channel (e.g. PDSCH only, single CORESET) or a subset of channels

- FFS: Additional details on extending the support of L1-based beam indication when separate UL (from DL) common beam indication is configured

Continue email discussion on the yellow part.

Agreement

On Rel-17 unified TCI framework, to accommodate the case of separate beam indication for UL and DL:

- Utilize two separate TCI states, one for DL and one for UL.

  - FFS: Contents of separate UL TCI state

  - Note: For FR1, UE does not expect UL TCI to provide a reference for determining common UL TX spatial filter(s), if UL TCI is supported for FR1

- For the separate DL TCI:

  - The source reference signal(s) in M TCIs provide QCL information at least for UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC

- For the separate UL TCI:

  - The source reference signal(s) in N TCIs provide a reference for determining common UL TX spatial filter(s) at least for dynamic-grant/configured-grant based PUSCH, all or subset of dedicated PUCCH resources in a CC

  - Optionally, this UL TX spatial filter can also apply to all SRS resources in resource set(s) configured for antenna switching/codebook-based/non-codebook-based UL transmissions

- FFS: Whether the UL TCI state is taken from a common/same or separate TCI state pool from DL TCI state

  - Note that TCI state pool for joint DL and UL beam indication is still FFS

- FFS: Whether Rel.17 supports TCI configured for single channel (e.g. PDSCH only, single CORESET)

- Note: This does not preclude the type of UE supporting only 1 beam tracking loop, i.e. UE reports value of 1 in UE FG 2-62.

Agreement

On Rel-17 enhancements to enable L1/L2-centric inter-cell mobility:

- The following use cases are assumed:

  - Network architecture:

    - NSA, i.e. LTE PCell and NR-PSCell

    - SA

  - Intra-band CA

    - FFS: If inter-band CA is also included

  - Intra- RAT (excluding inter-RAT)

  - Intra-frequency scenario:

    - The SSBs of non-serving cells have the same center frequency and SCS as the SSBs of the serving cell

    - An SSB of a non-serving cell is associated with a PCI different from the PCI of the serving cell

    - FFS: Support for inter-frequency scenario

  - FFS: Whether to support intra-DU only operation, or whether inter-DU is also allowed

- The following enhancement scope is assumed:

  - Facilitating measurement and reporting of non-serving RSs via incorporating non-serving cell info with some TCI(s), along with the necessary measurement and reporting scheme(s)

    - FFS: Detailed/exact method(s)

    - FFS: Whether this also implies the support of beam indication (TCI state update along with the necessary TCI state activation) for TCI(s) associated with non-serving cell RS(s)

    - FFS: Metric for the measurement and reporting, e.g. L1-RSRP or L3-RSRP or time- or spatial-domain-filtered L1-RSRP

    - FFS: Beam-level event-driven mechanism, using serving cell RS and/or non-serving cell RS

  - Facilitate serving cell to provide configurations for non-serving cell SSBs via RRC

    - FFS: details for the configurations, e.g. time/frequency location, transmission power, etc.

    - FFS: other information needed for inter-cell mobility

  - Note: In RAN1's understanding, non-serving cell SSB and non-serving cell RS can be part of the serving cell configuration

- FFS: The following enhancement scope is assumed by RAN1:

  - Whether RRC reconfiguration signaling is needed or not when a TCI associated with non-serving cell RS is indicated

    - A non-serving cell RS is an RS that is or has an SSB of a non-serving cell as direct or indirect QCL source

    - This implies no C-RNTI update when UE receives DL channel RS associated to non-serving cell RS as QCL source.

    - FFS whether TCI associated with non-serving cell can be indicated to or are applicable for all channels.

  - Whether some RRC parameters need to be updated without additional RRC signaling, e.g. some RRC parameters are pre-configured, which are associated with TCI states with neighbor cell RS as QCL source

  - Whether UE needs/can change serving cell during L1/L2-centric inter-cell mobility.

  - The above assumption to be verified by RAN2

Conclusion

There is no consensus in RAN1 to include the following as part of RAN1 agreement for AI 8.1.1 in RAN1 #103e:

- FFS beam indication for the TCI state assumption/update for the following cases:

  - The beam indication UE-specific DCI (i.e. the CORESETs with the DCI received by UE), the scheduled PDSCH by the DCI and the associated PUCCH for the acknowledgment of the beam indication DCI

  - Non-UE-specific CORESETs and PUSCH/PDSCH scheduled/activated and PUCCH transmission triggered by non-UE-specific CORESETs

Agreement

In Rel-17 enhancement for facilitating fast uplink panel selection, the following use cases are assumed:

- MPE mitigation

- UE power saving

- UL interference management

- Support different configurations across panels

- UL mTRP

Agreement

In Rel-17 enhancement on MP-UE to facilitate fast UL panel selection and MPE mitigation, UL Tx panel(s) are assumed to be a same set or subset of DL Rx panel(s)

Agreement

In Rel.17 enhancement for facilitating fast uplink panel selection, UE-initiated UL panel selection/activation are supported:

- FFS: Whether NW-initiated panel selection/activation is also supported

- FFS: Whether specification support for this feature is necessary and if so the details of such spec support.

Agreement

On UE reporting for MPE mitigation for Rel-17, investigate and, if needed, specify the following:

- Reporting of P-MPR report based on Rel.16 framework.

  - FFS: Whether panel/beam level based P-MPR report is supported

  - FFS: Maximum reported number of panels, e.g. single or multiple

- Reporting SSBRI(s)/CRI(s) and/or indication of panel selection for the purpose of indicating:

  - Alt1: alternative UE panel(s) or TX beam(s) for UL transmission

  - Alt2: feasible UE panel(s) or TX beam(s) for UL transmission taking the MPE effect into account

  - FFS: indication of panel selection details (e.g. explicit/implicit)

- Any additional reporting content: down-select from the following in RAN1#104-e

  - Alt0: no additional reporting content

  - Alt1: Additional reporting content is included (for example P-MPR + L1-RSRP, virtual PHR + L1-RSRP, L1-RSRP/SINR with and without MPE effect, virtual PHR, P-MPR or virtual PHR + CRI/SSBRI, estimated max UL RSRP)

    - Note: Other options are not precluded

    - FFS: Whether the above reporting is triggered by UE or configured by NW

Agreement

On Rel-17 unified TCI framework, support common TCI state ID update and activation to provide common QCL information and/or common UL TX spatial filter(s) across a set of configured CCs:

- The above applies to intra-band CA

- The above applies to joint DL/UL and separate DL/UL beam indications

- Just as Rel.16, the RS in the TCI state that provides QCL-TypeA [or QCL-TypeB] shall be in the same CC as the target channel or RS

- The common TCI state ID implies that the same/single RS determined according to the TCI state(s) indicated by a common TCI state ID is used to provide QCL Type-D indication and to determine UL TX spatial filter across the set of configured CCs

- FFS: The above also applies to inter-band CA

- FFS: TCI state pool for CA

  - Opt-1: sharing a single RRC TCI state pool for the set of configured CCs, e.g., cell-group TCI state pool, or reuse TCI state pool for PDSCH in a reference cell; A CC ID for QCL-Type A RS is absent in a TCI state, and the CC ID for QCL-Type A RS is determined according to a target CC of the TCI state.

    - FFS: Whether it is possible that a single TCI state in the pool includes all source RSs from different CCs

  - Opt-2: configuring RRC TCI state pool per individual CC

- FFS: Whether the Rel-17 common beam update across multiple CCs applies to beam indication for single channel (e.g. PDSCH only, single CORESET), a subset of channels, or all channels

Agreement

On Rel-17 unified TCI framework:

- A pool of joint DL/UL TCI state is used for joint DL/UL TCI state update (beam indication).

- FFS: The pool for separate DL and UL TCI state update (beam indication)

- Note: Here, TCI state pool refers to a pool configured via higher-layer (RRC) signaling

- FFS: Whether joint TCI may include UL specific parameter(s) such as UL PC/timing parameters, PL RS, panel-related indication,etc. and if it is included, it is used only for UL transmission of the DL and UL transmissions to which the joint TCI is applied

9715

Agreement

In RAN1#104-e, on the Rel.17 L1-based TCI state update (beam indication) for the unified TCI framework, interested companies are to provide the following:

- How to use DCI formats 1_1 and 1_2 for UL-only (in case of separate DL/UL) TCI state update (beam indication)

  - Note: The agreement implies that DCI formats 1_1 and 1_2 can be used for UL-only TCI state update beam indication).

  - FFS: Using DCI format 1_1 and 1_2 without DL assignment, and with a new acknowledgment mechanism directly in response to decoding DCI format 1_1 and 1_2, e.g., analogous to SPS PDSCH release

- Whether/how to support at least one additional DCI format dedicated for UL-only beam indication (in case of separate DL/UL), including:

  - Whether the format can also be used for DL-only beam indication (in case of separate DL/UL) and joint DL/UL beam indication

  - Whether it is a “brand new” format or based on some extension of the existing DCI formats other than 1_1 and 1_2 (e.g. 1_0, 0_0, 0_1, or 0_2)

    - If UL-related DCI is used, whether it is accompanied with UL grant or not

  - Acknowledgment mechanism

Agreement

On Rel.17 DCI-based beam indication:

- Regarding application time of the beam indication: if beam indication is received, down-select from the following:

  - Alt1: the first slot that is at least X ms or Y symbols after the DCI with the joint or separate DL/UL beam indication

  - Alt2: the first slot that is at least X ms or Y symbols after the acknowledgment of the joint or separate DL/UL beam indication

  - FFS: whether any existing timing defined for DCI based TCI/spatial relation update can be used for X/Y

- FFS: When to apply the minimum indication delay (e.g., when the newly indicated beam is different with the previously indicated beam)

### Enhancements for Multi-TRP Deployment

#### Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH

R1-2007540	Multi-TRP/panel for non-PDSCH	FUTUREWEI

R1-2007587	Enhancements on multi-TRP for reliability and robustness in Rel-17	Huawei, HiSilicon

R1-2007627	Reliability Enhancements for PDCCH, PUCCH, and PUSCH	InterDigital, Inc.

R1-2007645	Further discussion on enhancement of MTRP operation	vivo

R1-2007764	Multi-TRP enhancements for PDCCH, PUCCH and PUSCH	ZTE

R1-2007783	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Fujitsu

R1-2007793	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	TCL Communication Ltd.

R1-2007825	Discussion on enhancements on multi-TRP/panel for PDCCH, PUCCH and PUSCH	CATT

R1-2008001	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	CMCC

R1-2008149	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Samsung

R1-2008218	Enhancements on Multi-TRP based enhancement for PDCCH, PUCCH and PUSCH	OPPO

R1-2008347	Considerations on Multi-TRP for PDCCH, PUCCH, PUSCH	Sony

R1-2008439	Views on Rel-17 multi-TRP reliability enhancement	Apple

R1-2008574	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	LG Electronics

R1-2008898	On multi-TRP enhancements for PDCCH and PUSCH	Fraunhofer IIS, Fraunhofer HHI

R1-2008904	Enhancements for Multi-TRP URLLC schemes	Nokia, Nokia Shanghai Bell

R1-2008911	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Lenovo, Motorola Mobility

R1-2008944	Discussion on multi-TRP for PDCCH, PUCCH and PUSCH	NEC

R1-2008958	Enhancements on Multi-TRP for PDCCH, PUSCH and PUCCH	MediaTek Inc.

R1-2008978	Multi-TRP enhancements for PDCCH, PUCCH and PUSCH	Intel Corporation

R1-2009028	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Xiaomi

R1-2009054	Discussion on enhancements on multi-TRP for uplink channels	Asia Pacific Telecom co. Ltd

R1-2009130	Enhancements on multi-TRP for PUSCH	Sharp

R1-2009142	Discussion on enhancements on Multi-TRP for PDCCHPUCCH and PUSCH	Spreadtrum Communications

R1-2009159	Multi-TRP Enhancements for PDCCH, PUCCH and PUSCH	Convida Wireless

R1-2009175	Discussion on MTRP for reliability	NTT DOCOMO, INC.

R1-2009223	On PDCCH, PUCCH and PUSCH enhancements with multiple TRPs	Ericsson

R1-2009251	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Qualcomm Incorporated

R1-2009284	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Qualcomm Incorporated

Withdrawn

[103-e-NR-feMIMO-02] Email discussion on enhancements on multi-TRP for PDCCH – Mostafa (Qualcomm)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Possible Agreement

For non-SFN schemes, at least support Option 2. Further study other options.

Possible Agreement

For PDCCH reliability enhancements,

- Combination A: SFN scheme + Alt 1-1 is supported

- Combination C1: [FDM / TDM] schemes + Alts 1-2 / 1-3 / 2 / 3 + Option 2 + Case 1

  - Down-select one from Alts 1-2 / 1-3 / 2 / 3

Agreement

For PDCCH reliability enhancements, support SFN scheme + Alt 1-1.

- FFS: TCI state activation for CORESET, impact on default beam, BFD resource for BFR

Agreement

For PDCCH reliability enhancements with non-SFN schemes, support at least Option 2 + Case 1.

- Maximum number of linked PDCCH candidates is two

- FFS: Details including how the two PDCCH candidates are counted toward the BD limits and impact on overbooking, if any

- Down-select at least one Alt from Alts 1-2 / 1-3 / 2 / 3

- FFS: Linking options such as a fixed rule based on the same PDCCH candidate index, based on start CCE, based on configuration, etc.

  - FFS: additional restriction to facilitate soft combining

- FFS: implicit PUCCH resource determination for >8 PUCCH resources in the resource set, scheduling offset for “timeDurationForQCL”, Out-of-order / in-order definition for PDCCH-to-PDSCH and PDCCH-to-PUSCH, DAI for Type-2 codebook, Slot offset  for scheduling the same PDSCH/PUSCH/CSI-RS/SRS, rate matching PDSCH around the scheduling DCI.

- FFS: whether and how to support for DCI format 2_x

R1-2009683	Summary of email discussions [103-e-NR-feMIMO-02] for mTRP PDCCH enhancements	Moderator (Qualcomm)

Working Assumption

For PDCCH reliability enhancements with non-SFN schemes and Option 2 + Case 1, support Alt3 (two SS sets associated with corresponding CORESETs).

Agreement

For PDCCH reliability enhancements with non-SFN schemes and Option 2 + Case 1, CCEs of the two PDCCH candidates are counted separately following Rel. 15/16 procedures. Further study the BD limit by considering the following

- With respect to the complexity associated with RE de-mapping / demodulation, 2 units are required

- With respect to the complexity associated with decoding, the following assumptions can be further discussed:

  - Assumption 1: UE only decodes the combined candidate without decoding individual PDCCH candidates

  - Assumption 2: UE decodes individual PDCCH candidates

  - Assumption 3: UE decodes the first PDCCH candidate and the combined candidate

  - Assumption 4: UE decodes each PDCCH candidate individually, and also decodes the combined candidate

- Note 1: The Assumptions 1-4 are for discussion purpose only, and they may or may not have specification impact.

  - FFS: The relationship between UE capability, RRC configuration, and the BD limit, and whether the Assumptions 1-4 are relevant for this purpose.

- Note 2: the BD /CCE limit here is counted based on the configuration of PDCCH monitoring capability (e.g. per slot or per span).

Conclusion

Group-common DCI formats (DCI formats 2_x) are not precluded for multi-TRP PDCCH reliability enhancements and can be discussed with a lower priority compared to UE-specific DCI formats.

Note: Enhancements required for DCI formats 2_x, if any, can be discussed case-by-case.

[103-e-NR-feMIMO-03] Email discussion on enhancements on multi-TRP for PUSCH, PUCCH – Keeth (Nokia)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

R1-2009480	Summary of Multi-TRP URLLC for PUCCH and PUSCH	Moderator (Nokia, Nokia Shanghai Bell)

Agreement

For multi-TRP PUCCH transmission schemes.

- Support multi-TRP inter-slot repetition (Scheme 1)

  - One PUCCH resource carries UCI, another PUCCH resource or the same PUCCH resource in another one or more slots carries a repetition of the UCI.

  - FFS: Number of repetitions

- Further study the support (one or both) of the following schemes

  - Multi-TRP intra-slot beam hopping (Scheme 2)

    - UCI is transmitted in one PUCCH resource in which different sets of symbols within the PUCCH resource have different beams.

    - FFS: More than 2 beam hopping instances per PUCCH resource.

  - Multi-TRP intra-slot repetition (Scheme 3)

    - One PUCCH resource carries UCI, another PUCCH resource or the same PUCCH resource in another one or more sub-slots within a slot carries a repetition of the UCI.

- Note1: whether to support two PUCCH resources or the same PUCCH resource with different beams for Scheme 1 and 3 to be discussed separately.

Agreement

For multi-TRP PUCCH transmission schemes,

- For Scheme 1, at least PUCCH format 1/3/4 can be used.

- FFS: Support of PUCCH format 0/2 for Scheme 1

- FFS: Support of PUCCH formats for Scheme 2 and/or Scheme 3 (if schemes are agreed).

Agreement

For single DCI based M-TRP PUSCH repetition schemes, support codebook based PUSCH transmission with following enhancements.

- Support the indication of two SRIs.

  - Alt1: Bit field of SRI shall be enhanced.

  - Alt2: No changes on SRI field

- Support the indication of two TPMIs.

  - The same number of layers are applied for both TPMIs if two TPMIs are indicated

  - The number of SRS ports between two TRPs should be same.

  - FFS: Details on indicating two TPMIs (e.g, one TPMI field or two TPMI fields)

- Increase the maximum number of SRS resource sets to two

- FFS: configuration details of each SRS resource set (e.g., number of SRS resources in a resource set)

Agreement

For single DCI based M-TRP PUSCH repetition schemes, support non-codebook based PUSCH transmission with following considerations.

- Increase the maximum number of SRS resource sets to two, and associated CSI-RS resource can be configured per SRS resource set.

- FFS: Enhancements on SRI field in DCI to indicate the two beams for repetitions

Agreement

For PUCCH multi-TRP enhancements in FR2,

- Support separate power control parameters for different TRP via associating power control parameters via PUCCH spatial relation info.

  - Note: No spec impact.

- For per TRP closed-loop power control for PUCCH, further study the following alternatives considering TPC command when the “closedLoopIndex” values associated with the two PUCCH spatial relation info’s are not the same.

  - Option.1: A single TPC field is used in DCI formats 1_1 / 1_2, and the TPC value applied for both PUCCH beams

  - Option.2: A single TPC field is used in DCI formats 1_1 / 1_2, and the TPC value applied for one of two PUCCH beams at a slot. The TPC value may be applied for the other PUCCH beam at an another slot.

  - Option 3: A second TPC field is added in DCI formats 1_1 / 1_2.

  - Option 4: A single TPC field is used in DCI formats 1_1 / 1_2, and indicates two TPC values applied to two PUCCH beams, respectively.

- FFS: Transition period for beam / power / frequency change.

- FFS: Required power control enhancements for FR1

Agreement

For configuration/indication of the number of PUCCH repetitions for Scheme 1, there is no restriction on using Rel-15 framework on configuring the number of repetitions.

- Rel-17 feMIMO may additionally consider supporting the dynamic indication of the number of repetitions in RAN1 #104 meeting.

Agreement

For single DCI based M-TRP PUSCH repetition Type B, at least nominal repetitions are used to map beams

- Further study details and applicability of each mapping method

- Further study the slot based beam mapping in the cases of nominal repetition across slot boundaries

Agreement

For PUSCH multi-TRP enhancements,

- For per TRP closed-loop power control for PUSCH, further study the following alternatives when the “closedLoopIndex” values are different.

  - Option.1: A single TPC field is used in DCI formats 0_1 / 0_2, and the TPC value applied for both PUSCH beams

  - Option.2: A single TPC field is used in DCI formats 0_1 / 0_2, and the TPC value applied for one of two PUSCH beams at a slot.

  - Option 3: A second TPC field is added in DCI formats 0_1 / 0_2.

  - Option 4: A single TPC field is used in DCI formats 0_1 / 0_2, and indicates two TPC values applied to two PUSCH beams, respectively.

- FFS: Transition period for beam / power / frequency change.

Agreement

Support both type 1 and type 2 CG PUSCH transmission towards MTRP. Further study the following alternatives,

- Alt.1 : single CG configuration

  - Repetitions of a TB transmitted towards MTPR on multiple PUSCH transmission occasions of single CG configuration.

  - At least for codebook-based CG PUSCH, support configuring 2 SRIs/TPMIs.

- Alt.2 : multiple CG configurations

  - Repetitions of a TB transmitted towards MTRP on more than one PUSCH transmission occasions, where one or more transmission occasions are from one CG configuration and another one or more PUSCH transmission occasions are from another CG configuration.

  - 1 SRI/TPMI is configured/indicated for each CG configuration.

- Further study required beam mapping principals, low overhead mechanisms for beam selection, and other enhancements for Alt.1 and Alt.2.

Agreement

For multi-TRP TDM-ed PUCCH transmission schemes,

- Support the use of a single PUCCH resource

- Up to two spatial relation info’s can be activated per PUCCH resource via MAC CE

- FFS: Required enhancements for FR1

FFS: Use of multiple PUCCH resources.

Agreement

For M-TRP PUSCH reliability enhancement, further discuss multi-DCI based PUSCH transmission/repetition scheme(s) considering the following aspects.

- The same TB is repeated towards multiple TRPs with different beams, where one or more PUSCH repetitions are scheduled by one DCI and another one or more PUSCH repetitions are scheduled by another DCI.

- FFS: Details related to timeline restrictions and beam mapping

- Changes on Rel-15/16 MCS, TBS determination, and UL resource allocation are not expected from this scheme.

- The scheme is considered to be supported only if there are gains over single DCI based PUSCH repetition schemes and a similar scheme is not supported by m-TRP PDCCH (e.g. Option 3).

Companies are encouraged to provide simulation results to decide the support of the scheme in next RAN1 meetings

The support of multi-DCI based PUSCH transmission/repetition scheme(s) in Rel-17 will be decided in RAN1#104-e

Possible Working Assumption

For single DCI based M-TRP PUSCH repetition Type A and B, allow the possibility to configure either cyclic mapping or sequential mapping of UL beams.

- The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2.

- FFS: For PUSCH Type A, UL beams mapping applies to (one or both from Alt.1/2 may be selected),

    - Alt.1: different slots (slot-based beam mapping)

    - Alt.2: different frequency hops (mainly considering intra-slot frequency hopping. Refer as beam hopping)

    - Note: For PUSCH Type B, it was agreed that at least nominal repetitions are used to map beams

- FFS: Support of half-half mapping.

- FFS: Use/configuration of a beam mapping pattern if multiple patterns are introduced.

- FFS: Additional considerations Impact on the mapping patterns due to beam hopping (if beam hopping agreed) and/or (including required beam switching gaps)

- Companies are encouraged to provide further simulation results to decide details.

#### Enhancements on Multi-TRP inter-cell operation

R1-2007541	Inter-cell multi-TRP operation	FUTUREWEI

R1-2007588	Enhancements on inter-cell multi-TRP operations in Rel-17	Huawei, HiSilicon

R1-2007628	Synchronization Analysis for M-TRP Inter-cell Operation	InterDigital, Inc.

R1-2007646	Further discussion on inter-cell MTRP operation	vivo

R1-2007765	Discussion on Multi-TRP inter-cell operation	ZTE

R1-2007826	Discussion on multi-TRP/panel inter-cell operation	CATT

R1-2008002	Enhancements on Multi-TRP inter-cell operation	CMCC

R1-2008150	Enhancements on Multi-TRP inter-cell operation	Samsung

R1-2008219	Enhancement on inter-cell multi-TRP operation	OPPO

R1-2008348	Considerations on inter-cell operation	Sony

R1-2008440	Views on Rel-17 Inter-cell multi-TRP operation	Apple

R1-2008575	Enhancements on Multi-TRP inter-cell operation	LG Electronics

R1-2008905	Enhancements to enable inter-cell multi-TRP operations	Nokia, Nokia Shanghai Bell

R1-2008912	Enhancements on Multi-TRP inter-cell operation	Lenovo, Motorola Mobility

R1-2008945	Discussion on multi-TRP inter-cell operation	NEC

R1-2008979	Multi-TRP enhancements for inter-cell operation	Intel Corporation

R1-2009029	Enhancement on Inter-cell Multi-TRP operations	Xiaomi

R1-2009070	Enhancement on Multi-TRP inter-cell operation	Ericsson

R1-2009143	Discussion on enhancement on multi-TRP inter-cell operation	Spreadtrum Communications

R1-2009176	Discussion on inter-cell multi-TRP operations	NTT DOCOMO, INC.

R1-2009252	Enhancements on Multi-TRP inter-cell operation	Qualcomm Incorporated

[103-e-NR-feMIMO-04] Email discussion on enhancements on multi-TRP intercell operation – Rakesh (vivo)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Possible Agreement

Non-serving cell information for inter-cell MTRP operation to be associated with TCI state/QCL-info includes at least non-serving cell PCI at least when SSB is used as the reference signal

- The information provided by SSB-Configuration-r16 or MeasObject can be starting point for providing non-serving cell information

  - FFS: Other information

Possible Agreement (new starting point for email discussions)

For QCL/TCI related enhancement for enhanced inter-cell/multi-TRP operations, support configuration of non-serving cell information with one of the following alternatives

- Alt 0: Non-serving cell information can be associated with the TCI state and/or QCL-info at least when “neighbor cell SSB” is used as “QCL referenceSignal”

  - FFS: Whether TCI state enhancement is needed in addition to QCL-info enhancement

  - FFS: Whether the association is explicit or implicit

- Alt 2: Group TCI state and associate non-serving cell information with each group

  - Which RS/channels is (are) supported is separate discussion

Agreement

For QCL /TCI related enhancement for enhanced inter-cell multi-TRP operations, support RRC configuration of non-serving cell information

- Non-serving cell information can be associated with the TCI state and/or QCL -info at least when “neighbor cell SSB” is used as “QCL referenceSignal ”

  - FFS : Whether beam indication enhancement is needed in addition to QCL -info enhancement

  - FFS : Whether the association is explicit or implicit

Agreement

The information provided by SSB-Configuration-r16/ssb-InfoNcell-r16 and/or MeasObject can be starting point for providing non-serving cell information

9731

For future meetings

Consider rate matching behavior related to non-serving cell SSB.

#### nhancements on beam management for multi-TRP

R1-2007542	Beam management for simultaneous multi-TRP transmission with multi-panel reception				FUTUREWEI

R1-2007589	Discussion on multi-TRP for multi-panel reception in Rel-17	Huawei, HiSilicon

R1-2007629	Beam Management Enhancements for Multi-TRP	InterDigital, Inc.

R1-2007647	Further discussion on MTRP multibeam enhancement	vivo

R1-2007766	Enhancements on beam management for Multi-TRP	ZTE

R1-2007784	Enhancements on beam management for multi-TRP	Fujitsu

R1-2007827	Beam management enhancement for multi-TRP simultaneous reception	CATT

R1-2008003	Enhancements on beam management for multi-TRP	CMCC

R1-2008151	Enhancements on beam management for multi-TRP	Samsung

R1-2008220	Enhancements on beam management for multi-TRP	OPPO

R1-2008309	Enhancements on beam management for multi-TRP	AT&T

R1-2008349	Considerations on beam management for multi-TRP	Sony

R1-2008441	Views on Rel-17 multi-TRP BM enhancement	Apple

R1-2008576	Enhancements on beam management for multi-TRP	LG Electronics

R1-2008906	Enhancements on Beam Management for Multi-TRP/Panel Transmission	Nokia, Nokia Shanghai Bell

R1-2008913	Enhancements on beam management for multi-TRP	Lenovo, Motorola Mobility

R1-2008946	Discussion on beam management for multi-TRP	NEC

R1-2008957	Enhancements on beam management for multi-TRP	MediaTek Inc.

R1-2008980	Multi-TRP enhancements for beam management	Intel Corporation

R1-2009030	Enhancement on beam management for Multi-TRP	Xiaomi

R1-2009052	Discussion of enhancements on beam management for multi-TRP	Asia Pacific Telecom co. Ltd

R1-2009144	Discussion on enhancements on beam management for multi-TRP	Spreadtrum Communications

R1-2009156	Discussion on beam management for multi-TRP	ASUSTeK

R1-2009160	On Per-TRP Beam Failure Recovery	Convida Wireless

R1-2009177	Discussion on beam management for MTRP	NTT DOCOMO, INC.

R1-2009213	Discussion on beam management for multi-TRP	ITRI

R1-2009225	On beam management enhancements for simultaneous multi-TRP transmission with multi-panel reception	Ericsson

R1-2009253	Enhancements on beam management for multi-TRP	Qualcomm Incorporated

[103-e-NR-feMIMO-05] Email discussion on enhancements on beam management for multi-TRP – Runhua (CATT)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

R1-2009500	Summary on beam management for simultaneous multi-TRP transmission with multiple Rx panels	Moderator (CATT)

Agreement

- For M-TRP beam failure detection, support independent BFD-RS configuration per-TRP, where each TRP is associated with a BFD-RS set.

  - FFS: The number of BFD RSs per BFD-RS set, the number of BFD-RS sets, and number of BFD RSs across all BFD-RS sets per DL BWP

  - Support at least one of explicit and implicit BFD-RS configuration

    - With explicit BFD-RS configuration, each BFD-RS set is explicitly configured

      - FFS: Further study QCL relationship between BFD-RS and CORESET

    - FFS: How to determine implicit BFD-RS configuration, if supported

- For M-TRP new beam identification

  - Support independent configuration of new beam identification RS (NBI-RS) set per TRP if NBI-RS set per TRP is configured

    - FFS: detail on association of BFD-RS and NBI-RS

    - Support the same new beam identification and configuration criteria as Rel.16, including  L1-RSRP, threshold

Agreement

Support TRP-specific BFD counter and timer in the MAC procedure

- The term TRP is used only for the purposes of discussions in RAN1 and whether/how to capture this is FFS

Agreement

- Support a BFRQ framework based on Rel.16 SCell BFR BFRQ

  - In RAN1#104-e, select one from the following options

    - Option 1: Up to one dedicated PUCCH-SR resource in a cell group

      - A cell group refers to either MCG, SCG, or PUCCH cell group

      - FFS: number of spatial filters associated with the PUCCH-SR resources

      - FFS: How the SR configuration is done

    - Option 2:  Up to two (or more) dedicated PUCCH-SR resources in a cell group

      - A cell group refers to either MCG, SCG, or PUCCH cell group

      - FFS: whether each PUCCH-SR resource is restricted to be associated to one spatial filter

      - FFS: How the SR configuration is done

  - FFS: Whether no dedicated PUCCH-SR resource can be supported in addition to Option 1 or Option 2

- Study whether and how to provide the following information in BFRQ MAC-CE

  - Index information of failed TRP(s)

  - CC index (if applicable)

  - New candidate beam index (if found)

  - Indication whether new beam(s) is found

  - FFS: whether/how to incorporate multi-TRP failure

Agreement

Down-select at least one of the following options for beam measurement/reporting enhancement to facilitate inter-TRP beam pairing in RAN1 #104-e

- Option 1: In a CSI-report, UE can report N>1 pair/groups and M>=1 beams per pair/group

  - Different beams in different pairs/groups can be received simultaneously

  - FFS: whether M is equal or can be different across different pair/group

- Option 2: In a CSI-report, UE can report N(N>=1) pairs/groups and M (M>1) beams per pair/group

  - Different beams within a pair/group can be received simultaneously

- Option 3: UE report M(M>=1) beams in N (N>1) CSI-reports corresponding to N report setting

  - Different beams in different CSI-reports can be received simultaneously

  - FFS: whether/how to introduce an association between different CSI-reports

  - FFS: whether/how to differentiate reported measurements for beams that are received simultaneously vs. beams that are not received simultaneously

    - Whether/how to introduce an indication along with the CSI-reports to indicate whether the beams in different CSI-reports can be received simultaneously

- FFS: value of N and M in each option

- FFS: Association between different beams in above options and different TRP/UE panels

- FFS: Identify new use cases per option compared with R16 (including backhaul)

- FFS: whether different beams in different pairs/groups/reports can be received by same spatial filter per option

#### Enhancements on HST-SFN deployment

R1-2007543	Enhancement to support HST-SFN deployment scenario	FUTUREWEI

R1-2007590	Discussion on multi-TRP for high speed train in Rel-17	Huawei, HiSilicon

R1-2007630	Enhancements for M-TRP to Support HST-SFN Deployment	InterDigital, Inc.

R1-2007648	Further discussion and evaluation on HST-SFN schemes	vivo

R1-2007767	Discussion on Multi-TRP HST enhancements	ZTE

R1-2007828	On enhancements on HST-SFN deployment	CATT

R1-2008004	Enhancements on HST-SFN deployment	CMCC

R1-2008152	Enhancements on HST-SFN	Samsung

R1-2008221	Enhancements on HST-SFN deployment	OPPO

R1-2008350	Considerations on HST-SFN operation for multi-TRP	Sony

R1-2008442	Views on Rel-17 HST enhancement	Apple

R1-2008577	Enhancements on HST-SFN deployment	LG Electronics

R1-2008907	Enhancements for HST-SFN deployment	Nokia, Nokia Shanghai Bell

R1-2008947	Discussion on HST-SFN deployment	NEC

R1-2008981	Enhancements to HST-SFN deployments	Intel Corporation

R1-2009069	Enhancement on HST-SFN deployment	Ericsson

R1-2009099	Enhancements for HST-SFN deployment	Lenovo, Motorola Mobility

R1-2009145	Discussion on enhancements on HST-SFN deployment	Spreadtrum Communications

R1-2009178	Discussion on HST-SFN deployment	NTT DOCOMO, INC.

R1-2009254	Enhancements on HST-SFN deployment	Qualcomm Incorporated

[103-e-NR-feMIMO-06] Email discussion on enhancements on HST-SFN deployment – Alexei (Intel)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

R1-2009476	Summary#1 of AI: 8.1.2.4 Enhancements on HST-SFN deployment	Intel Corporation

Agreement

Support at least the following configuration for HST scenario in Rel-17

- The same DMRS port(s) can associate with multiple TCI states

  - Note: DMRS and [PDCCH]/PDSCH from different TRPs are transmitted in SFN manner

  - FFS other details

Agreement

At most two QCL/TCI states are supported for HST scenario in Rel-17

- FFS: Whether to support more than two TCI states for FR2

- FFS configuration/signalling details of the TCI states

- Note: DMRS and [PDCCH]/PDSCH from different TRPs are transmitted in SFN manner

R1-2009646	Summary#2 of AI: 8.1.2.4 Enhancements on HST-SFN deployment	Intel Corporation

Agreement

Support at least the following configuration for HST scenario in Rel-17

- The same DMRS port(s) can associate with multiple TCI states

  - Note: DMRS and PDCCH/PDSCH from different TRPs are transmitted in SFN manner

  - FFS other details

Note: Strive for a unified solution for PDCCH in agenda item 8.1.2.1 and 8.1.2.4

Agreement

At most two TCI states are supported for HST scenario in Rel-17

- FFS: Whether to support more than two TCI states for FR2

- FFS configuration/signalling details of the TCI states

Note: DMRS and PDCCH/PDSCH from different TRPs are transmitted in SFN manner

Note: Strive for a unified solution for PDCCH in agenda item 8.1.2.1 and 8.1.2.4

Updated proposal 5-1:

- When the same DMRS port(s) are associated with two QCL/TCI states containing TRS as source reference signal, at least one variant is supported for Rel-17 HST-SFN scenario

- Variant A: One of QCL/TCI state can be associated with {average delay, delay spread}  and another QCL/TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

- Variant B: One of the QCL/TCI state can be associated with {average delay, delay spread} and another QCL/TCI state with {Doppler shift, Doppler spread} (i.e., ‘QCL-TypeB’)

- Variant C: One of the QCL/TCI state can be associated with {delay spread}  and another QCL/TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

- Variant E: Both QCL/TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

- FFS: Indication method to apply QCL, e.g., via new QCL-type, or reuse existing QCL-type while UE to ignore certain QCL properties

- Note 1: Each TCI state in the above variants may be additionally associated with {Spatial Rx parameter} (i.e., QCL-TypeD)

- Note 2: Companies are encouraged to provide evaluation results for the above variants based on agreed EVM from RAN1#102e meeting

- Note 3: DMRS and PDCCH/PDSCH from different TRPs are transmitted in SFN manner

### Enhancements on SRS flexibility, coverage and capacity

R1-2007544	Enhancements on SRS flexibility, coverage and capacity	FUTUREWEI

R1-2007591	Discussion on SRS enhancements for Rel-17	Huawei, HiSilicon

R1-2007631	Discussion on SRS Enhancements	InterDigital, Inc.

R1-2007649	Further discussion on SRS enhancement	vivo

R1-2007768	Enhancements on SRS flexibility, coverage and capacity	ZTE

R1-2007829	On enhancements on SRS  flexibility, coverage and capacity	CATT

R1-2008005	Enhancements on SRS flexibility, coverage and capacity	CMCC

R1-2008153	Enhancements on SRS	Samsung

R1-2008222	Enhancements on SRS flexibility, coverage and capacity	OPPO

R1-2008351	Considerations on SRS flexibility, coverage and capacity	Sony

R1-2008443	Views on Rel-17 SRS enhancement	Apple

R1-2008578	Enhancements on SRS flexibility, coverage and capacity	LG Electronics

R1-2008900	Enhancements on SRS for coverage and capacity	Fraunhofer IIS, Fraunhofer HHI

R1-2008908	Enhancements on SRS flexibility, coverage and capacity	Nokia, Nokia Shanghai Bell

R1-2008914	Enhancements on SRS	Lenovo, Motorola Mobility

R1-2008948	Discussion on SRS enhancement	NEC

R1-2008959	Enhancements on SRS flexibility, coverage and capacity	MediaTek Inc.

R1-2008982	Discussion on SRS enhancements	Intel Corporation

R1-2009031	Discussion on SRS enhancements	Xiaomi

R1-2009131	Enhancements on SRS	Sharp

R1-2009146	Considerations on SRS enhancement	Spreadtrum Communications

R1-2009179	Discussion on SRS enhancement	NTT DOCOMO, INC.

R1-2009211	SRS Performance and Potential Enhancements	Ericsson LM

R1-2009255	Enhancements on SRS flexibility, coverage and capacity	Qualcomm Incorporated

R1-2009286	Discussion on enhancement of SRS in Rel. 17 further enhanced MIMO 	CEWiT

[103-e-NR-feMIMO-07] Email discussion on SRS flexibility, coverage, and capacity – Hao (ZTE)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

R1-2009384	FL summary #1 on SRS enhancements	Moderator (ZTE)

Agreement

A given aperiodic SRS resource set is transmitted in the (t+1)-th available slot counting from a reference slot, where t is indicated from DCI, or RRC (if only one value of t is configured in RRC), and the candidate values of t at least include 0. Adopt at least one of the following options for the reference slot.

- Opt. 1: Reference slot is the slot with the triggering DCI.

- Opt. 2: Reference slot is the slot indicated by the legacy triggering offset.

- FFS the detailed definition of “available slot” considering UE processing complexity and timeline to determine available slot, potential co-existence with collision handling, etc., e.g.,

  - Based on only RRC configuration, “available slot” is the slot satisfying: there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set

- FFS explicit or implicit indication of t

- FFS whether updating candidate triggering offsets in MAC CE may be beneficial

Agreement

Support at least DCI 0_1 and 0_2 to trigger aperiodic SRS without data and without CSI.

- FFS whether/how to re-purpose the unused fields, e.g., the triggering offset(s) and the frequency resources for triggering A-SRS on one or more component carriers, SFI-index, etc.

- FFS UL/DL DCI with data for aperiodic SRS

- FFS group common DCI

Agreement

In Rel-17 SRS coverage and capacity enhancement, support at least one scheme from Class 2 and Class 3, and deprioritize Class 1.

- Note: Extensions of Rel-15/16 frequency hopping are included in Classes 2 and 3, e.g. where UE hops once per symbol within a Rel-17 SRS resource.

Possible Agreement

For antenna switching up to 8Rx, support SRS resource configurations for {1T6R, 1T8R, 2T6R, 2T8R, 4T6R, 4T8R}.

- Note: companies are encouraged to evaluate directional UE antennas

R1-2009650	FL summary #2 on SRS enhancements	Moderator (ZTE)

Agreement

Candidate schemes for Class 2:

- Scheme 2-0: Increase the number of repetition symbols in one slot

- Scheme 2-1: Inter-slot repetition on consecutive symbols or non-consecutive symbols across slots

- Scheme 2-2: Repetition with TD-OCC

- Scheme 2-3: Repetition with CS hopping

Candidate schemes for Class 3:

- Scheme 3-1: RB-level partial frequency sounding

- Scheme 3-2: Subcarrier-level partial frequency sounding

- Scheme 3-3: Subband-level partial frequency sounding

- Scheme 3-4: Partial-frequency sounding schemes assisted with CSI-RS, where SRS is transmitted in a subset of RBs of the original SRS frequency resource

- Scheme 3-5: Dynamic change of SRS bandwidth with RB-level subband size scaling

- Note: Consider issues like gNB receiver complexity,  PAPR, etc., with above schemes

- Note: Joint operation between Class 2 and Class 3 schemes can be considered

Proposed conclusion:

For the purpose of Rel-17 SRS enhancement, the following is assumed:

A Rel-15/16 UE that supports nT=nR antenna switching with n={1,2,4} can be configured with an n port SRS resource that is in both an SRS resource set with usage=’codebook’ and another SRS resource set with usage=’antennaSwitching’, provided that the SRS resource sets have the same time domain behavior.

- The existing conclusions/agreements for Rel-15/16 is unchanged.

Agreement

For antenna switching up to 8Rx, support SRS resource configurations for {1T6R, 1T8R, 2T6R, 2T8R, [4T6R], 4T8R}.

### CSI enhancements: MTRP and FR1 FDD reciprocity

R1-2007545	CSI enhancement for multi-TRP and FDD	FUTUREWEI

R1-2007592	Discussion on CSI Enhancements for Rel-17	Huawei, HiSilicon

R1-2007632	CSI Enhancements for the Support of MTRP and FDD Reciprocity	InterDigital, Inc.

R1-2007650	Further discussion and evaluation on MTRP CSI and Partial reciprocity	vivo

R1-2007769	CSI enhancements for Multi-TRP and FR1 FDD reciprocity	ZTE

R1-2007830	CSI enhancements for MTRP and FR1 FDD with partial reciprocity	CATT

R1-2008006	Enhancements on CSI reporting for Multi-TRP	CMCC

R1-2008154	Views on Rel. 17 CSI enhancements	Samsung

R1-2008223	CSI enhancement for M-TRP and FDD reciprocity	OPPO

R1-2008352	Considerations on CSI enhancements	Sony

R1-2008444	Views on Rel-17 CSI enhancement	Apple

R1-2008579	CSI enhancements for Rel-17	LG Electronics

R1-2008901	CSI enhancements on Type II PS codebook and multi-TRP	Fraunhofer IIS, Fraunhofer HHI

R1-2008909	Enhancement on CSI measurement and reporting	Nokia, Nokia Shanghai Bell

R1-2008949	Discussion on CSI enhancement for multi-TRP	NEC

R1-2008960	CSI enhancement for NCJT	MediaTek Inc.

R1-2008983	On CSI enhancements for MTRP and FDD reciprocity	Intel Corporation

R1-2009100	CSI enhancements for mTRP and FDD reciprocity	Lenovo, Motorola Mobility

R1-2009147	Discussion on CSI enhancement for M-TRP transmission and FR1 FDD reciprocity	Spreadtrum Communications

R1-2009180	Discussion on CSI enhancements	NTT DOCOMO, INC.

R1-2009224	On CSI enhancements in Rel-17 feMIMO	Ericsson

R1-2009256	CSI enhancements - MTRP and FR1 FDD reciprocity	Qualcomm Incorporated

[103-e-NR-feMIMO-08] Email discussion on CSI enhancements: MTRP and FR1 FDD reciprocity – Min (Huawei)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

R1-2009529	Summary of CSI enhancements for MTRP and FDD	Huawei, HiSilicon

Agreement

Port selection codebook enhancements utilizing DL/UL reciprocity of angle and/or delay is supported in Rel-17.

Agreement

Rel-17 CSI measurement and reporting for DL multi-TRP and/or multi-panel transmission shall be enhanced to support and enable more dynamic channel/interference hypotheses for NCJT.

Possible Agreement

- For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, [at least for multi-DCI based and single-DCI based schemes (scheme 1a)], NZP CSI-RS resources for channel measurement are associated to different TRPs/TCI states at resource level

  - FFS: whether CMRs corresponding to different TRPs respectively shall be configured within the same resource set (i.e. scheme 1-2) or different resource sets (i.e. scheme 1-3).

Note that an implicit/explicit set of reporting settings CSI-ReportConfigs, which are associated to different TRPs/TCI states, is not excluded and can be discussed further.

Agreement

9530

Agreement

For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, [at least for multi-DCI based and single-DCI based schemes (scheme 1a)], NZP CSI-RS resources for channel measurement are associated to different TRPs/TCI states at resource level

- CMRs corresponding to different TRPs respectively shall be configured within the same resource set (i.e. scheme 1-2) and have the same number of ports among CMRs.

- At least ‘typeI-SinglePanel’ codebook is supported

  - FFS: Other codebook types

- Note that RAN1 shall strive to finalize NCJT CSI enhancement with single reporting setting firstly.

- The support of larger than 32 ports across two CMRs is optional for a UE supporting Rel. 17 mTRP CSI

Working Assumption

For CSI measurement for multi-DCI based NCJT, down select one of following two options:

- Option 1 (Explicit): CMRs corresponding to different TRPs can be associated with different reporting settings respectively, with the same configurations between two settings except for PUCCH/PUSCH resources and CMR/IMR resources setting(s)

- Option 2 (Implicit): a single CSI reporting setting associated with each TRP where a NZP CSI-RS is configured for interference measurement from another TRP

- FFS:  how interference from CMR in the linked reporting settings in option 1 or from the NZP CSI-RS configured as IMR in option 2 is considered in CQI calculation

Ffollowing restrictions apply to both options:

- At least ‘typeI-SinglePanel’ codebook is supported

  - FFS: Other codebook types

- Only ‘periodic’ and ‘semiPersistentOnPUCCH’ cases are supported;

- The number of ports of two CMRs associated to two reporting settings for NCJT CSI measurement are the same;

- The support of larger than 32 ports across two CMRs is optional for a UE supporting Rel. 17 mTRP CSI

### Others

R1-2007547	Sounding enhancement for interference probing in TDD massive MIMO	FUTUREWEI

R1-2007633	Updated Results on Multi-TRP MIMO Deployments with Multi-Panel UEs	InterDigital, Inc.

R1-2007651	Discussion on higher layer aspects for multi-TRP enhancement	vivo

R1-2007770	Further details on Multi-beam and Multi-TRP operation	ZTE

R1-2007831	Evaluation of multi-TRP enhancement	CATT

R1-2008155	Simulation results for multi-beam enhancements	Samsung

R1-2008323	Discussion on field measurement and evaluation assumptions for FDD CSI enhancements in Rel-17		Huawei, HiSilicon

R1-2008755	Analysis of Control Signaling for multi-beam operation	Dongguan OPPO Precision Elec.

R1-2008902	On FDD channel reciprocity in real-world scenarios	Fraunhofer IIS, Fraunhofer HHI

R1-2008915	HARQ feedback of SPS PDSCH reception in multi-DCI based multiple TRPs	Lenovo, Motorola Mobility

R1-2009010	Evaluation of multi-TRP transmission with multi-panel reception	ETRI

R1-2009132	Other enhancements for beam management.	Sharp

R1-2009181	Discussion on UL dense deployment	NTT DOCOMO, INC.

R1-2009290	Additional simulation results on multi-beam operation	Ericsson