3GPP TSG RAN WG1 Meeting #103-e		R1-200XXXX

e-Meeting, October 26th – November 13th, 2020

Title:	Session notes for 8.10

(Enhancements to Integrated Access and Backhaul)

Source:	Ad-hoc chair (Samsung)

## Enhancements to Integrated Access and Backhaul

Please refer to RP-201293 for detailed scope of the WI

### Enhancements to resource multiplexing between child and parent links of an IAB node

R1-2008858	Enhancements to the IAB resource multiplexing	ZTE, Sanechips

R1-2009269	Resource management for enhanced duplexing	Qualcomm Incorporated

R1-2009301	Resource multiplexing and DC in enhanced IAB	Ericsson

R1-2007594	Resource multiplexing between backhaul and access for IAB duplexing enhancements	Huawei, HiSilicon

R1-2007684	Enhancement to resource multiplexing between child and parent links	vivo

R1-2008029	Discussion on resource multiplexing in the simultaneous operation	CMCC

R1-2008184	Enhancements to Resource Multiplexing for NR IAB	Samsung

R1-2008312	Enhancements for IAB resource multiplexing	AT&T

R1-2008406	Discussions on IAB resource multiplexing enhancements	LG Electronics

R1-2008863	Enhancements for resource multiplexing among IAB backhaul and access links	Nokia, Nokia Shanghai Bell

R1-2008995	Enhancements to Resource Multiplexing between Child and Parent Links of an IAB Node			Intel Corporation

R1-2009018	Enhancements to resource multiplexing between child and parent links of an IAB node	ETRI

Withdrawn

R1-2009108	Enhancements to resource multiplexing for IAB	Lenovo, Motorola Mobility

R1-2009190	Resource multiplexing between child and parent links of an IAB node	NTT DOCOMO, INC.

R1-2009220	Discussion on IAB resource multiplexing enhancements	ETRI

R1-2009221	Discussions on enhancements to resource multiplexing between child and parent links of an IAB node		CEWiT

[103-e-NR-eIAB-01] Email discussion on enhancements to resource multiplexing between child and parent links of an IAB node – Thomas (AT&T)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

R1-2009604	Summary #1 of [103-e-NR-eIAB-01]	Moderator (AT&T)

Agreement

The Rel-16 IAB-DU resource types (Soft/Hard/NA) are the starting point for supporting resource multiplexing for simultaneous operation cases in Rel-17.

- FFS: Whether resource type definitions need to be extended to frequency domain resources

- FFS: Coexistence of simultaneous operation resources and TDM resources

- FFS: Whether new rules governing cell-specific/semi-static signals and channels at the IAB-DU and/or IAB-MT in case of simultaneous operation are necessary

Agreement

Further consider different applicability restrictions/conditions for simultaneous operation multiplexing cases:

- FFS: Whether a given case is only applicable for certain resource types or combinations: e.g. DL access, DL backhaul, UL access, UL backhaul

- FFS: Network (including parent node) awareness of a child IAB node’s ability to support simultaneous operation due to short-term and long-term factors including panel selection, interference, timing, transmit power, capability indication etc.

- FFS: Necessary differentiation for paired spectrum vs. unpaired spectrum

- FFS: Whether specific enhancements are defined for full-duplex cases vs. being left to implementation (as in Rel-16)

- Note: There should not be any impact on legacy UE behavior

Agreement

The Rel-16 explicit indication of soft resources by DCI Format 2_5 is supported for simultaneous operation cases in Rel-17.

FFS: Whether/how to extend DCI Format 2_5 to frequency domain resources and/or paired spectrum

FFS: Coexistence of simultaneous operation resources and TDM resources

Agreement

From a RAN1 perspective, at least intra-donor multi-parent operation is supported in Rel-17

FFS: Inter-donor operation pending additional input from RAN2/RAN3

Agreement

The explicit indication of soft resources by DCI Format 2_5 is supported for multi-parent scenarios in Rel-17.

- FFS: Whether additional enhancements over the Rel-16 solution are needed

Agreement

From a RAN1 perspective, resource multiplexing and coordination is supported for the following DC scenarios in Rel-17:

- Inter-carrier, inter-band

- Inter-carrier, intra-band is additionally supported at least for FR2

  - At least to the extent it reuses solutions for supporting Inter-carrier, inter-band

  - FFS: whether specific enhancements for inter-carrier, intra-band DC are introduced in Rel-17

9734

### Other enhancements for simultaneous operation of IAB-node’s child and parent links

Including IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed

R1-2007595	Enhancements for simultaneous operation of MT and DU	Huawei, HiSilicon

R1-2008407	Discussions on other enhancements for simultaneous operation	LG Electronics

R1-2009109	Enhancements for simultaneous operation in IAB systems	Lenovo, Motorola Mobility

R1-2007685	Other enhancements for simultaneous operation of child and parent links	vivo

R1-2007786	Other enhancements for simultaneous operation of IAB-node’s child and parent links	Fujitsu

R1-2008030	Discussion on the enhancement for simultaneous operation	CMCC

R1-2008185	Enhancements to Timing, Power Control and CLI for NR IAB	Samsung

R1-2008313	Enhancements for IAB interference management	AT&T

R1-2008816	Discussion on simultaneous operation of IAB-node’s child and parent links	CEWiT

R1-2008859	Enhancements for simultaneous operation of child and parent links	ZTE, Sanechips

R1-2008864	Other enhancements for simultaneous operation of IAB Parent and Child	Nokia, Nokia Shanghai Bell

R1-2008996	Power Control Enhancements for Simultaneous Operations	Intel Corporation

R1-2009019	Discussion on simultaneous operation enhancements	ETRI

R1-2009137	Power Control: Subsequent Contribution	Sharp

R1-2009191	Other enhancements for simultaneous operation of IAB-node’s child and parent links	NTT DOCOMO, INC.

R1-2009270	On enhancements for simultaneous operation of IAB-node's child and parent links	Qualcomm Incorporated

R1-2009302	Timing, CLI and power control in enhanced IAB	Ericsson

[103-e-NR-eIAB-02] Email discussion non other enhancements for simultaneous operation of IAB-node’s child and parent links – Luca (Qualcomm)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

R1-2009567	Summary #1 of [103-e-NR-eIAB-02]	Moderator (Qualcomm Incorporated)

Agreement

Select one or both of the following modes of operation for Case 7 timing in RAN1#104-e:

- symbol level alignment without slot level alignment

- slot level alignment

Agreement

Interference management for the following IAB interference scenarios should be discussed:

Inter-IAB scenarios, including:

  - MT to MT, DU to DU, DU to MT, and MT to DU.

Interference to non-IAB nodes, including:

- IAB-DU to non-IAB-DU

- IAB-MT to non-IAB-DU

Intra-IAB-node (self-interference) scenarios (Interference between a DU and MT of an IAB-node).

This agreement does not necessarily mean that specification support is needed for any of the scenarios.

Agreement

Consider resource and beam coordination techniques to mitigate/avoid interference, including (not an exhaustive list):

- FFS: whether or not to support IAB‐node (MT) transmissions in DL access slots

  - FFS: if this has RAN1 impact or it can be handled by implementation.

  - FFS: network coordination impact

- FFS: whether Rel-16 resource management framework is sufficient.

Agreement

An IAB-node can rely on an OTA timing synchronization mechanism to enable/maintain Case 6 timing mode

- FFS whether the Rel-16 OTA synchronization mechanism is sufficient or enhancements are required

  - If required, details of enhancements including the uplink timing(s) required to support different timing alignment cases

FL Proposal 2 (continue email discussion)

An IAB-node, when operating in Case 7 timing mode, supports an OTA timing synchronization mechanism to enable a child node to set its DL Tx timing.

- FFS whether the Rel-16 OTA synchronization mechanism is sufficient or enhancements are required

  - FFS details of enhancements, if required

### Others

R1-2007686	Other enhancements to Rel-17 IAB nodes	vivo

R1-2008314	Performance of simultaneous operation of IAB-node’s child and parent links	AT&T

R1-2008327	Other enhancements for IAB	Huawei, HiSilicon

R1-2008408	Discussion on RACH procedure for IAB enhancement	LG Electronics

R1-2008860	Discussion on multiplexing capability for IAB	ZTE, Sanechips

R1-2009020	Other enhancements to IAB	ETRI

R1-2009303	Refined beam calibration in enhanced IAB	Ericsson