3GPP TSG RAN WG1 Meeting #103-e		R1-200XXX

e-Meeting, October 26th – November 13th, 2020

Title:	Session notes for 8.9 (Rel-17 enhancements for NB-IoT and LTE-MTC)

Source:	Ad-hoc chair (Samsung)

## Rel-17 enhancements for NB-IoT and LTE-MTC

Please refer to RP-201306 for detailed scope of the WI

### Support of 16-QAM for unicast in UL and DL for NB-IoT

R1-2007618	Support of 16QAM for unicast in UL and DL in NB-IoT	Huawei, HiSilicon

R1-2008073	Support of 16-QAM for NB-IoT	Nokia, Nokia Shanghai Bell

R1-2008697	Discussion on UL and DL 16QAM for NB-IoT	ZTE

R1-2008920	Support 16QAM for NBIoT	Lenovo, Motorola Mobility

R1-2008930	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

R1-2008969	Further considerations on support of 16QAM for NB-IOT	MediaTek Inc.

R1-2009112	Support of 16-QAM for NB-IoT	Qualcomm Incorporated

R1-2009125	Design considerations to support 16-QAM for NB-IOT	Sierra Wireless, S.A.

[103-e-LTE-Rel17_NB_IoT_eMTC-01] Email discussion on support of 16-QAM for unicast in UL and DL for NB-IoT – Yubo (Huawei)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

R1-2009477	Feature lead summary #1 on [103-e-LTE-Rel17_NB_IoT_eMTC-01]	Moderator (Huawei)

Agreement

At least for standalone and guard-band deployments, the maximum TBS to support 16-QAM for unicast in DL is 4968 bits with ISF=7.

Agreement

For inband deployment, the maximum TBS to support 16-QAM for unicast in DL is 3624 bits (ISF=7).

Agreement

Different breaking points (QPSK16QAM) are used for standalone/guardband and inband deployments.

- FFS the details of the breaking point.

Agreement

Explicit or implicit signaling of power ratios of NPDSCH EPRE to NRS EPRE for the following cases is supported.

- NPDSCH in symbols without NRS and CRS

- NPDSCH in symbols with CRS (only for “In-band” deployment)

- NPDSCH in symbols with NRS

Agreement

For 16-QAM in NB-IoT, separate optional UE capabilities for UL and DL are supported:

- The support of 16QAM in DL is indicated by an optional UE capability signaling.

- The support of 16QAM in UL is indicated by an optional UE capability signaling.

Agreement

For 16-QAM in NB-IoT, separate UE-specific RRC signaling for UL and DL are supported:

- 16QAM for UL is configured by UE-specific RRC signaling.

- 16QAM for DL is configured by UE-specific RRC signaling.

Possible Working Assumption

To support 16QAM for NB-IoT DL, the following TBS indices are introduced

| I_TBS | I_SF |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | [552, 536] | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | [328, 296] | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | [2472, 2536] | 2984 | 4008 | 4968 |

- FFS: Support of legacy TBS indices with 16-QAM at least for some deployment modes.

- FFS: Mapping of (a subset of) TBS entries to modulation schemes for different deployment modes.

- FFS for I_SF > 7

Possible Working Assumption

To support 16QAM for NB-IoT UL, at least the following TBS indices are introduced

| I_TBS | I_RU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | [2472, 2536] |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | [2536] |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | [2536] |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | [2472,2536] |  |  |  |

### Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC

R1-2007619	Support of 14-HARQ processes in DL for HD-FDD MTC UEs	Huawei, HiSilicon

R1-2008074	Support of 14-HARQ processes in DL for eMTC	Nokia, Nokia Shanghai Bell

R1-2008698	Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC		ZTE

R1-2008931	Support of 14 HARQ processes in DL in LTE-MTC	Ericsson, Telefónica, Verizon, SoftBank, AT&T, Telstra

R1-2009113	Support of 14 HARQ processes and scheduling delay	Qualcomm Incorporated

R1-2009124	Design considerations to support 14-HARQ Feature for LTE-M	Sierra Wireless, S.A.

[103-e-LTE-Rel17_NB_IoT_eMTC-02] Email discussion on support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC – Gerardo (Ericsson)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

R1-2009513	Feature Lead Summary [103-e-LTE-Rel17_NB_IoT_eMTC-02]	Moderator (Ericsson)

Agreement

The following working assumption is confirmed

Introduce a new optional UE capability to support 14 HARQ processes

Agreement

The design of the 14 HARQ processes feature accounts for the presence of non-BL/CE UL and DL subframes in the PUCCH non-repetition case.

- FFS: PDSCH scheduling delays

- FFS: HARQ-ACK delays

- FFS: Configurable/dynamic set of PDSCH delays/HARQ-ACK delays

For future meetings:

Companies to further study on the impact of measurement gaps on the 14 HARQ processes feature.

### Others

R1-2007620	Channel quality reporting in NB-IoT to support 16QAM	Huawei, HiSilicon

R1-2008699	DL TBS increase for eMTC	ZTE

R1-2008932	On the support of a maximum DL TBS of 1736 bits in LTE-MTC	Ericsson