# 3GPP TSG RAN WG1 #103-e							R1-200xxxx

# e-Meeting, October 26th – November 13th, 2020

Agenda item:	6.2.1

Source: 	Qualcomm Incorporated

Title: 	TP for Support of implicit HARQ-ACK feedback

Document for:	Discussion and Decision

# Background

The TP in this contribution implements the necessary changes to support implicit HARQ-ACK feedback, where the new PUSCH DCI can provide HARQ-ACK for all the ongoing PUSCHs.

# TP

<TP, TS 36.213>

## 8.0	UE procedure for transmitting the physical uplink shared channel

<Unchanged parts are omitted>

For a BL/CE UE configured with mpdcch-UL-HARQ-ACK-FeedbackConfig, if the UE detects a first MPDCCH with DCI format 6-0A/6-0B intended for the UE scheduling PUSCH in subframes {s0,…,sN-1}, and if the UE detects a second MPDCCH with DCI format 6-0A/6-0B intended for the UE scheduling PUSCH in subframes {q0,…,qL-1} with M≤q0≤sN-1, where

-	for each HARQ ID i indicated in the first MPDCCH,

- either:

  - the HARQ ID is not indicated in the second MPDCCH, or,

  - the NDI indicated in the second MPDCCH for the corresponding HARQ ID is toggled with respect to the NDI indicated in the first MPDCCH

-	and, the first subframe M in which the second MPDCCH is transmitted meets M≥s0,i+k, where  s0,i is the first subframe in which the HARQ ID i is transmitted

-	for FDD, k = 4,

-	for TDD the value of k is given in Table 8-2 for the corresponding TDD UL/DL configuration; If the value of k is not given in Table 8-2 for subframe n, denote subframe n' as the first downlink/special subframe which has a value in Table 8-2 after subframe n, and substitute n with n' in the above procedure

the UE shall

-	drop the remaining PUSCH transmission(s) of the transport block(s) scheduled by the first MPDCCH starting from subframe K, where M<K≤q0, and

-	deliver HARQ-ACK feedback corresponding to the transport block(s) scheduled by the first MPDCCH to higher layers, and

-	transmit the PUSCH scheduled by the second MPDCCH in subframes {q0,…,qL-1}

<Unchanged parts are omitted>

</TP>