# 3GPP TSG RAN WG1 #103-e							R1-200xxxx

# e-Meeting, October 26th – November 13th, 2020

Agenda item:	6.6

Source: 	Moderator (Qualcomm Incorporated)

Title: 	Summary of email discussion [103-e-LTE-6.6CRs]

Document for:	Discussion and Decision

# Background

In RAN1#103-e, the following two issues were submitted for Rel-16 corrections to DAPS:

| R1-2008524 | Power control for DAPS | Qualcomm Incorporated |
| --- | --- | --- |
| R1-2008799 | Draft CR on power sharing for LTE DAPS | Huawei, HiSilicon |

Both contributions try to capture the agreement made in the previous meeting regarding power sharing for LTE DAPS.

| 8524 (QC) | 8799 (HW/HiSi) |
| --- | --- |
| ### 5.1.4a(new) Power allocation for dual active protocol stack If a UE indicates a capability for dual active protocol stack based handover (DAPS HO), the UE can be provided with a source MCG and a target MCG. If a UE is configured with a target MCG and a source MCG in different bands, and the UE is configured with DAPS-PowerCoordinationInfo, the UE shall apply the procedures described in clause 5.1.4 with the following modifications - Consider the target MCG as the MCG and the source MCG as the SCG. - Replace p-MeNB and p-SeNB by p-DAPS-Target and p-DAPS-Source, respectively. - Replace “(a)synchronous dual connectivity” by “(a)synchronous DAPS”. | # 17 Dual active protocol stack based handover If a UE indicates a capability for dual active protocol stack based handover (DAPS HO), the UE can be provided with a source MCG and a target MCG. If a UE is configured with a target MCG and a source MCG, the UE is indidated the guaranteed power γMCG for transmissions on the target MCG by p-DAPS-Target-r16 and the guaranteed power γSCG for transmissions on the source MCG by p-DAPS-Source-r16 and with an inter-CG power sharing mode by uplinkPowerSharingDAPS-Mode-r16. If the UE indicates support of power control mode 1 and is provided uplinkPowerSharingDAPS-Mode-r16 = 1, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 5.1.4.1 by replacing the MCG with the target MCG and the SCG with the source MCG and the UE assumes only Pcell exists in each MCG. If the UE indicates support for power control mode 2 and is provided uplinkPowerSharingDAPS-Mode-r16 = 2, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 5.1.4.2 by replacing the MCG with the target MCG and the SCG with the source MCG and the UE assumes only Pcell exists in each MCG. If UE does not indicate a capability for power sharing between source and target MCGs in DAPS handover, the UE does not expect the transmissions on the target and source cell overlapped in time domain. |

# Merged text

Based on the input received in the two contributions, the moderator tried to merge both TPs based on the following principles:

- It is preferred to keep this section under power control.

- The first paragraph is identical in both TPs.

- There are several conditions in legacy DC text that are not captured in the Huawei TP, e.g. the following

if the UE supports synchronous dual connectivity but does not support asynchronous dual connectivity, or if the UE supports both synchronous dual connectivity and asynchronous dual connectivity and if the higher layer parameter powerControlMode indicates dual connectivity power control mode 1

-	if the maximum uplink timing difference between transmitted signals to different serving cells including serving cells belonging to different CGs is equal to or less than the minimum requirement for maximum transmission timing difference for synchronous dual connectivity defined in [10].

thus, it is preferred to completely reuse the DC section and refer to 5.1.4 instead of 5.1.4.1/2. At the same time, the Huawei TP includes the correct reference to the DAPS power control parameter – this is added.

- The TP from Huawei includes the additional information about the UE behavior when the UE does not support power sharing. This information is added. Added also that for “intra-band DAPS” the same behavior is expected.

### 5.1.4a(new)	Power allocation for dual active protocol stack

If a UE indicates a capability for dual active protocol stack based handover (DAPS HO), the UE can be provided with a source MCG and a target MCG.

If a UE is configured with a target MCG and a source MCG in different bands, and the UE is configured with DAPS-PowerCoordinationInfo, the UE shall apply the procedures described in clause 5.1.4 with the following modifications

- Consider the target MCG as the MCG and the source MCG as the SCG.

- Replace p-MeNB and p-SeNB by p-DAPS-Target and p-DAPS-Source, respectively.

- Replace “(a)synchronous dual connectivity” by “(a)synchronous DAPS”.

- “Dual connectivity power control mode” is replaced by “DAPS power control mode”, and is given by higher layer parameter uplinkPowerSharingDAPS-Mode”.

If UE does not indicate a capability for power sharing between source and target MCGs in DAPS handover, or if a UE is configured with a target MCG and a source MCG in the same band, the UE does not expect the transmissions on the target and source cell overlapped in time domain.

Companies are encouraged to provide feedback on the above merged TP in the table below.

| Company | Comment |
| --- | --- |

# Conclusion

<To be completed>