3GPP TSG RAN WG1 #103e	 R1-200xxxx

e-Meeting, October 26th–November 13th, 2020

Source: 	Moderator (ZTE)

Title: 	Summary of Email Discussion [103-e-NR-7.1CRs-01]

Agenda item:	7.1

Document for:	Discussion/Decision

# Introduction

In RAN1#103e meeting, the following email discussion is assigned by Chairman to discuss the ambiguity issue for Rel-15 UE features in case of cross-carrier operation.

[103-e-NR-7.1CRs-01] Ambiguity Issues for UE Features with Cross-Carrier Operation (Rel-15) – Xingguang (ZTE)

- Discussion and decision by 10/29, TPs by 11/5

This document is used for collecting companies’ input and summarizing the email discussion.

# Background

For some per-band or per-FS UE features, if these UE features are related to more than one carrier/band, it is not clear for companies how to interpret these UE features in case of cross-carrier operation. Take the Rel-15 UE capability aperiodicTRS as an example. UE capability aperiodicTRS is a “per Band” signaling, which is to indicate the network whether the UE supports DCI triggering aperiodic TRS associated with periodic TRS. If UE indicates support of aperiodicTRS for Band A and NOT support of aperiodicTRS for Band B, without clarification, it is not clear whether UE supports triggering TRS for Band B from Band A. Based on the clarification made in RAN1#102e [1], support of aperiodicTRS in case of cross-carrier operation is only determined by the triggered cell (i.e., the cell transmitting the TRS). Thus, in this example, UE doesn’t support triggering TRS for Band B from Band A.

During RAN1#102e meeting, companies discussed the ambiguity issue for some of the Rel-15 UE features and reached the following conclusion [1] together with an LS to RAN2 to clarify this issue [2]. However, there are still several Rel-15 UE features that companies didn’t reach consensus on how to interpret them in case of cross-carrier operation, e.g., ue-SpecificUL-DL-Assignment and bwp-DiffNumerology / bwp-SameNumerology [3].

Conclusion - Regarding the interpretation of UE capabilities in case of cross-carrier operation, RAN1 clarifies that support of the following UE capability is based on the support of this capability for the band of the scheduled/triggered/indicated cell only. - aperiodicTRS - beamSwitchTiming - Regarding the interpretation of UE capabilities in case of cross-carrier operation, RAN1 clarifies that support of the following UE capability is based on both the support of this capability for the band of the scheduled/triggered/indicated cell and the support of this capability for the band of the scheduling/triggering/indicating cell. - crossCarrierScheduling-SameSCS

# Discussion

As also summarized in the moderator’s summary [3], basically, there are three different interpretations as shown below to interpret the UE capabilities with such ambiguity issue. In the following discussion in this document, we use the Interpretation1, Interpretation2 and Interpretation3 to clarify this ambiguity issue.

Interpretation1: Support of this UE capability is based on the support of this capability for the band of the scheduled/triggered/indicated cell only. Interpretation2: Support of this UE capability is based on the support of this capability for the band of the scheduling/triggering/indicating cell only. Interpretation3: Support of this UE capability is based on the support of this capability for both the band of the scheduled/triggered/indicated cell and the band of the scheduling/triggering/indicating cell.

During RAN1#103e, four contributions are submitted under AI.1 to discuss this ambiguity issue, i.e., [4, ZTE], [5, Apple], [6, vivo] and [7, Huawei/HiSilicon]. The proposals of these four contributions are summarized in Appendix. Among them, four UE features are discussed, i.e., ue-SpecificUL-DL-Assignment, bwp-DiffNumerology/bwp-SameNumerology and pdcch-MonitoringAnyOccasionsWithSpanGap. In this Section, we discuss these UE features raised by companies.

## FG: ue-SpecificUL-DL-Assignment

The description of ue-SpecificUL-DL-Assignment from TS38.306 is as below.

| ue-SpecificUL-DL-Assignment Indicates whether the UE supports dynamic determination of UL and DL link direction and slot format based on Layer 1 scheduling DCI and higher layer configured parameter UL-DL-configuration-dedicated as specified in TS 38.213 [11]. | FS | No | N/A | N/A |
| --- | --- | --- | --- | --- |

As clarified in [4, ZTE] and [6, vivo], UE feature ue-SpecificUL-DL-Assignment is not the FG for SFI. The UE feature for SFI should be dynamicSFI. ue-SpecificUL-DL-Assignment contains the following two parts.

- Determination of UL and DL link direction and slot format based on Layer 1 scheduling DCI. From moderator’s perspective, the corresponding spec for this part is in Section 11.1 of TS 38.213 (copied below for your convenience).

Spec from Section 11.1 of TS38.213 If a UE is not configured to monitor PDCCH for DCI format 2_0, for a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE - the UE receives PDSCH or CSI-RS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format 1_0, DCI format 1_1, or DCI format 0_1 - the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, DCI format 2_3, or a RAR UL grant

- Determination of UL and DL link direction and slot format based on higher layer configured parameter UL-DL-configuration-dedicated. Note: There is a typo in 38.306. UL-DL-configuration-dedicated should be TDD-UL-DL-ConfigDedicated instead.

Based on the previous email discussion, it seems companies have different understanding on whether UE is allowed to report ue-SpecificUL-DL-Assignment for FDD band. This may potentially affect the understanding of which Interpretation should be adopted for ue-SpecificUL-DL-Assignment. For example, as discussed in [4, ZTE], if UE is not allowed to report for ue-SpecificUL-DL-Assignment FDD band, Interpretation3 will preclude the use case of FDD cross-carrier scheduling TDD for this UE capability. However, if we go with Interpretation1 for ue-SpecificUL-DL-Assignment, this kind of scenario can be supported.

Based on the above introduction, companies are encouraged to answer the following questions and provide the corresponding technical justification.

1st phase: Question 1

Is UE allowed to report ue-SpecificUL-DL-Assignment for FDD band?

Which interpretation should be adopted for ue-SpecificUL-DL-Assignment?

Please provide your answers with corresponding technical justification. Comparison between different interpretations are encouraged.

| Company | Answers and Comments |
| --- | --- |
| ZTE | Our understanding is Interpretation1. From our perspective, ue-SpecificUL-DL-Assignment is not applicable to FDD band because it is used to indicate whether UE supports UE-specific DL/UL configuration via TDD-UL-DL-ConfigDedicated and TDD-UL-DL-ConfigDedicated is only allowed to be configured for TDD band based on TS38.331. ServingCellConfig ::= SEQUENCE { tdd-UL-DL-ConfigurationDedicated TDD-UL-DL-ConfigDedicated OPTIONAL, -- Cond TDD. As ue-SpecificUL-DL-Assignment is not applicable to FDD band, Interpretation3 will preclude the use case of FDD band cross-carrier scheduling/triggering/indicating TDD band because Interpretation3 requires UE to indicate support of this capability for both the scheduling/triggering/indicating cell and scheduled/triggered/indicated cell. If companies believe that ue-SpecificUL-DL-Assignment is applicable to FDD band, then some further RAN1/RAN2 CRs are needed to clarify what the corresponding UE behaviors for FDD are. Another part of UE behavior of ue-SpecificUL-DL-Assignment is that, UE supports determination of UL and DL link direction and slot format based on Layer 1 scheduling DCI. The Layer 1 scheduling DCI refers to DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1 and etc., all the Layer 1 scheduling DCIs are regular scheduling DCI, it doesn’t impose any implementation burden on UE. Thus, from this perspective, Interpretation1 is more appropriate as the implementation of this UE feature is more about how to realize the slot formation determination for the scheduled/triggered/indicated cell instead of the DCI receiving for the scheduling/triggering/indicating cell. |
| Intel | We share same view as ZTE and support Interpretation #1. |
| vivo | Our understanding is Interpretation1. Regarding the reporting, our understanding is that according to the RRC spec, TDD-UL-DL-ConfigDedicated can only be configured for a TDD cell (shown below), which means that the ue-SpecificUL-DL-Assignment capability can be reported for TDD cell only. This also implies that Interpretation1 is reasonable, because either Interpretation2 or Interpretation3 requires the ue-SpecificUL-DL-Assignment capability to be reported for the scheduling cell, which precludes the scenario of FDD cell being the scheduling cell. TDD; This field is optionally present, Need R, for TDD cells. It is absent otherwise.. |
| Nokia | - Not applicable for an FDD band - Interpretation #1 |
| Qualcomm | Our understanding is the support is based on Interpretation 3, i.e., on both bands of the cross-carrier operation. This feature has a requirement on both control channel processing on the scheduling band and the data channel or reference signal processing. It is hard to isolate one from the other. For example, this requires UE to be able to support the relatively tight timeline due to dynamic determination of L1 transmission or reception. In case a support of FDD/TDD scheduling relationship needs to be specified, it is understood that his feature should be reported for both bands. |
| Apple | Our understanding is interpretation 3 According to 38.306, ue-SpecificUL-DL-Assignment “Indicates whether the UE supports dynamic determination of UL and DL link direction and slot format based on Layer 1 scheduling DCI and higher layer configured parameter UL-DL-configuration-dedicated as specified in TS 38.213 [11]. Indicates whether the UE supports dynamic determination of UL and DL link direction and slot format based on Layer 1 scheduling DCI and higher layer configured parameter UL-DL-configuration-dedicated as specified in TS 38.213 [11]. “ This capability contains two parts (1) The L1 scheduling DCI (2) The UL-DL-configuration-dedicated. We do agree that tdd-UL-DL-ConfigurationDedicated (which we may need to update 38.306 to align the RRC IE) can only be configured in TDD band. But we need to discuss the L1 scheduling DCI part which we believe interpretation 3 may be beneficial for UE capability reporting. |
| OPPO | We share the same view as QC/Apple, i.e., interpretation 3. Cross-carrier scheduling involves both the scheduling carrier and the scheduled carrier. Thus, we should consider the capabilities of both carriers. |
| Huawei | - We think ue-SpecificUL-DL-Assignment cannot be applied for FDD. - Our understanding is that Interpretation1 should be applied. According the definition of the FG, the determination of UL and DL link direction and slot format is based on Layer 1 scheduling DCI for the scheduled cell and does not have relevance to the scheduling cell carrying the Layer 1 scheduling DCI. Any feature involved in case of cross-carrier operation requires changes of PDCCH implementation on the scheduling cell. It is not clear why aperiodicTRS and beamSwitchTiming can follow Interpretation1 but not ue-SpecificUL-DL-Assignment. In addition, interpretation 1 will provide UE some implementation flexibilities, i.e. a UE does not need to support this UE capacity on both cells in case of cross-carrier operation. |
| Samsung | It is not applicable for an FDD band Our understanding is Interpretation 3. This feature affects to DCI monitoring on the scheduling cell and transmission/reception for the scheduled data on the scheduled cell. |

### Summary of 1st phase discussion

Company position for FG: ue-SpecificUL-DL-Assignment

This FG is NOT applicable to FDD band (7): ZTE, Intel, vivo, Nokia, Huawei/Hisilicon, Samsung

This FG is applicable to FDD band (3): Qualcomm, [Apple], [OPPO]

Adopt Interpretation1 for this FG (6): ZTE, Intel, vivo, Nokia, Huawei/Hisilicon

1) Interpretation3 precludes the use case of FDD band cross-carrier scheduling TDD band.

2) This FG by definition does not have relevance to the scheduling cell carrying the Layer 1 scheduling DCI. The Layer 1 scheduling DCI is regular DCI, which doesn’t add any additional implementation complexity for UE.

3) Interpretation 1 provides UE some implementation flexibilities, i.e. a UE does not need to support this UE capacity on both cells in case of cross-carrier operation.

Adopt Interpretation3 for this FG (4): Qualcomm, Apple, OPPO, Samsung

1) This feature has a requirement on both control channel processing on the scheduling band and the data channel or reference signal processing. It is hard to isolate one from the other. For example, this requires UE to be able to support the relatively tight timeline due to dynamic determination of L1 transmission or reception.

Moderator suggestion

Whether this FG is applicable to FDD band?

7 companies believe this FG is NOT applicable to FDD band and 3 companies believe this FG is applicable to FDD band.

Naturally speaking, this FG is not applicable to FDD band because, as pointed out by several companies, the corresponding RRC parameter tdd-UL-DL-ConfigurationDedicated is only applicable to TDD. Furthermore, if companies propose to make this FG applicable to FDD band, some further RAN1/RAN2 specification changes may be needed to clarify the UE behaviors for FDD.

From moderator’s perspective, we propose the following

Moderator proposal 1-v0: FG ue-SpecificUL-DL-Assignment is NOT applicable to FDD band.

Note: We don’t need to reflect this proposal in the chairman note or in the spec, this is just to help the further discussion.

Any additional requirements on the scheduling cell?

It seems that companies hold totally different view on whether this FG adds any additional implementation burden on the scheduling cell/band and requires tighter timeline requirement. Some argued that this FG requires UE to be able to support the relatively tight timeline. However, some other companies argued that all the L1 scheduling command are regular scheduling DCI for PDSCH/PUSCH/CSI-RS/SRS etc., it doesn’t add any additional implementation burden and doesn’t require tighter timeline requirement. Furthermore, the cross-carrier operation of ue-SpecificUL-DL-Assignment happens only when cross-carrier scheduling is configured. In this case, the carrier to receive scheduling DCI can already be controlled by the UE feature for cross-carrier scheduling (crossCarrierScheduling-SameSCS).

If cross-carrier operation of ue-SpecificUL-DL-Assignment adds some additional requirements on the scheduling cell/band for UE, then it seems Interpretation3 is more appropriate. If not, then it seems Interpretation1 is more appropriate.

From moderator’s perspective, we propose the following

Moderator proposal 2-v0: Further discuss the following issues

1) Whether cross-carrier operation of ue-SpecificUL-DL-Assignment adds any additional requirements on the scheduling cell/band if cross-carrier scheduling has already been supported by the UE?

2) Considering that UE feature for cross-carrier scheduling (crossCarrierScheduling-SameSCS) can already control the scheduling cell and the scheduled cell, do you think we still need interpretation3 for ue-SpecificUL-DL-Assignment to control the scheduling cell and the scheduled cell?

### 2nd phase: Question 1-2

Companies are encouraged to check/update the following positions and answer following questions.

Adopt Interpretation1 for ue-SpecificUL-DL-Assignment Support: ZTE, Intel, vivo, Nokia, Huawei/Hisilicon Object: Adopt Interpretation3 for ue-SpecificUL-DL-Assignment: Support: Qualcomm, Apple, OPPO, Samsung Object:

1) Whether cross-carrier operation of ue-SpecificUL-DL-Assignment adds any additional requirements on the scheduling cell/band if cross-carrier scheduling has already been supported by the UE?

2) Considering that UE feature for cross-carrier scheduling (crossCarrierScheduling-SameSCS) can already control the scheduling cell and the scheduled cell, do you think we still need interpretation3 for ue-SpecificUL-DL-Assignment to control the scheduling cell and the scheduled cell?

| Company | Answers and Comments |
| --- | --- |
| ZTE | 1) No. If UE supports cross-carrier scheduling between band A and band B, there is no additional implementation requirements on the scheduling cell because the Layer 1 scheduling DCIs for ue-SpecificUL-DL-Assignment are regular scheduling DCIs. Also, these regular scheduling DCIs don’t add any additional timeline requirement for UE on top of the existing scheduling timeline. 2) No. Based on last RAN1 meeting’s conclusion, interpretation3 is adopted for crossCarrierScheduling-SameSCS. Thus, UE feature for cross-carrier scheduling can already control the band pairs for cross-carrier scheduling. In other words, UE feature for cross-carrier scheduling can already control the band pair used for ue-SpecificUL-DL-Assignment. |
| Apple | - Yes. ue-SpecificUL-DL-Assignment contains the Layer 1 scheduling DCI which may requires the UE Cross Carrier Scheduling (CCS) support. Dynamic change of the slot format has tight UE timing line impact, especially in terms of the cancellation of the DL reception and/or UL transmission. In fact, there is an ongoing discussion in Rel-16 UE feature for partial cancellation which reflects the UE impact and it is well acknowledged in that agenda that there is UE implementation impact. - Yes. Two things. (a) supporting crossCarrierScheduling-SameSCS does not mean UE has to support CCS for slot format change, which has additional UE implementation impact, for example, on the UL transmission and DL reception cancellation (b) We also support CCS with different CCS, the same FG is also applicable to Rel-16 |

## FG: bwp-DiffNumerology / bwp-SameNumerology

The description of bwp-DiffNumerology / bwp-SameNumerology from TS38.306 is as below.

| Bwp-DiffNumerology Indicates whether the UE supports BWP adaptation up to 4 BWPs with the different numerologies, via DCI and timer. For the UE capable of this feature, the bandwidth of a UE-specific RRC configured DL BWP includes the bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB for Pcell and PSCell (if configured). For Scell(s), the bandwidth of the UE-specific RRC configured DL BWP includes SSB, if there is SSB on Scell(s). | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

| bwp-SameNumerology Defines type A/B BWP adaptation (up to 2/4 BWPs) with the same numerology, via DCI and timer. For the UE capable of this feature, the bandwidth of a UE-specific RRC configured DL BWP includes the bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB for Pcell and PSCell (if configured). For Scell(s), the bandwidth of the UE-specific RRC configured DL BWP includes SSB, if there is SSB on Scell(s). | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

These two capabilities are used to indicate whether the UE supports BWP adaptation up to 2/4 BWPs with different/same numerologies, via DCI and timer. The cell with DCI triggering this BWP switch and the target cell can be in different bands.

These two UE capabilities contain two parts (1) support of up to 2/4 BWPs (2) DCI-based or timer-based BWP switch. As discussed in [4, ZTE] and [6, vivo], if we take Interpretation 3 for this UE capability, in order to support cross-carrier BWP switching, it requires UE to support up to 2/4 BWPs in both scheduling cell and scheduled cell. This may not be reasonable. With Interpretation 1, a carrier with 1 BWP can still be used to cross-carrier switch the BWP of a carrier with up to 2/4 BWPs as long as cross-carrier scheduling is supported. [7, Huawei/HiSilicon] also proposes to adopt Interpretation1 for these two UE capabilities.

Based on the above introduction, companies are encouraged to answer the following question and provide the corresponding justification.

1st phase: Question 2

Which interpretation should be adopted for bwp-DiffNumerology / bwp-SameNumerology?

Please provide your answer with corresponding technical justification. Comparison between different interpretations are encouraged.

| Company | Answers and Comments |
| --- | --- |
| ZTE | Our understanding is Interpretation1. As pointed out by our contribution, Interpretation3 requires both the scheduling cell and the scheduled cell to support up to 2/4 BWPs and Interpretation1 precludes the use case of a carrier with 1 BWP cross-carrier scheduling another carrier with up to 2/4 BWPs. Interpretation1 can remove the above mentioned restrictions. Furthermore, these two UE capabilities bwp-DiffNumerology / bwp-SameNumerology are about supporting up to 2/4 BWPs in the scheduled cell. The DCIs received in the scheduling cell are regular scheduling DCIs, which doesn’t put any additional implementation burden on top of the regular cross-carrier scheduling if UE has already indicated support of cross-carrier scheduling. Thus, Interpretation1 is more appropriate from this perspective. |
| Intel | We share same view as ZTE and support Interpretation #1. |
| Vivo | Our understanding is Interpretation1. Given that these two capabilities are RF-related capabilities, i.e., they are dependent on the target band being BWP adaptation, Interpretation2 is not applicable. On the other hand, if Interpretation3 is accepted, both the scheduling and scheduled cells should support up to 2 or 4 BWPs, which is not desirable from UE implementation perspective. |
| Nokia | Interpretation #1, this is in our view clearly a property of the scheduled/triggered/indicated cell. |
| Qualcomm | Interpretation 3 is our understanding. Similar reason as to question 1. Interpretation 3 allows UE to have a better control of the support for the cross-carrier operation. With interpretation 1 and 2, if UE supports the feature on one band, it is forced to support the feature in all cross-carrier operation with the band where UE supports the feature involved. This is even more undesirable for UE implementation. |
| Apple | Our understanding is interpretation 3 Dynamic BWP switch involves cross carrier scheduling, which in general, impact UE implementation and the related UE capability is not adequate. |
| OPPO | Our understanding is interpretation 3. Similar reasons as Question 1. |
| Huawei | Our understanding is that Interpretation1 should be applied. Refer to reason for Question 1. |
| Samsung | Our understanding is interpretation 3. |

### Summary of 1st phase discussion

Company position for FG: bwp-DiffNumerology / bwp-SameNumerology

Adopt Interpretation1 for these two FGs (6): ZTE, Intel, vivo, Nokia, Huawei/Hisilicon

1) This is a property of the scheduled/triggered/indicated cell.

2) These FGs don’t add any additional implementation burden on the scheduling cell/band.

3) Interpretation 1 provides UE some implementation flexibilities, i.e. a UE does not need to support this UE capacity on both cells in case of cross-carrier operation.

Adopt Interpretation3 for these two FGs (4): Qualcomm, Apple, OPPO, Samsung

1) Interpretation 3 allows UE to have a better control of the support for the cross-carrier operation.

2) Dynamic BWP switch involves cross carrier scheduling, which in general, impact UE implementation and the related UE capability is not adequate.

Moderator suggestion

Any additional requirements on the scheduling cell?

Some companies believe that these two FGs are property of the scheduled/triggered/indicated cell and don’t add any additional implementation burden on the scheduling cell/band. Besides, cross-carrier operation for bwp-DiffNumerology / bwp-SameNumerology only happens when cross-carrier scheduling is configured and cross-carrier scheduling can already be controlled by the FG for cross-carrier scheduling (crossCarrierScheduling-SameSCS). However, some other companies don’t think so and they tend to believe that Interpretation3 allows UE to have a better control of the support for the cross-carrier operation.

It seems the key issue here is whether cross-carrier operation of bwp-DiffNumerology / bwp-SameNumerology adds any additional requirements on the scheduling cell/band. If yes, then it seems appropriate to adopt Interpretation3 here. Otherwise, Interpretation1 is more appropriate.

From moderator’s perspective, we propose the following

Moderator proposal 3-v0: Further discuss the following issues

1) Whether cross-carrier operation of bwp-DiffNumerology / bwp-SameNumerology adds any additional requirements on the scheduling cell/band if cross-carrier scheduling has already been supported by the UE?

2) Considering that UE feature for cross-carrier scheduling (crossCarrierScheduling-SameSCS) can already control the scheduling cell and the scheduled cell, do you think we still need interpretation3 for bwp-DiffNumerology / bwp-SameNumerology to control the scheduling cell and the scheduled cell?

### 2nd phase: Question 2-2

Companies are encouraged to check/update the following positions and answer following questions.

Adopt Interpretation1 for bwp-DiffNumerology / bwp-SameNumerology: Support: ZTE, Intel, vivo, Nokia, Huawei/Hisilicon Object: Adopt Interpretation3 for bwp-DiffNumerology / bwp-SameNumerology: Support: Qualcomm, Apple, OPPO, Samsung Object:

1) Whether cross-carrier operation of bwp-DiffNumerology / bwp-SameNumerology adds any additional requirements on the scheduling cell/band if cross-carrier scheduling has already been supported by the UE?

2) Considering that UE feature for cross-carrier scheduling (crossCarrierScheduling-SameSCS) can already control the scheduling cell and the scheduled cell, do you think we still need interpretation3 for bwp-DiffNumerology / bwp-SameNumerology to control the scheduling cell and the scheduled cell?

| Company | Answers and Comments |
| --- | --- |
| ZTE | 1) No. If UE supports cross-carrier scheduling between band A and band B, there is no additional implementation requirements on the scheduling cell because BWP switching DCIs for bwp-DiffNumerology / bwp-SameNumerology are regular PDSCH/PUSCH scheduling DCIs. Also, these regular scheduling DCIs don’t add any additional timeline requirement for UE top of the existing scheduling timeline. 2) No. Based on last RAN1 meeting’s conclusion, interpretation3 is adopted for crossCarrierScheduling-SameSCS. Thus, UE feature for cross-carrier scheduling can already control the band pairs for cross-carrier scheduling. In other words, UE feature for cross-carrier scheduling can already control the band pair used for bwp-DiffNumerology / bwp-SameNumerology. |
| Apple | - Yes. Supporting Cross Carrier Scheduling (CCS) does not necessarily mean that a UE needs to support multiple BWP, and/or, BWP switch with the same or the different numerology. In fact, BWP switch requires additional switching delay, and in this meeting, there is a separate email thread [03-e-AI5-LS-02] for BWP switch related issues which may not directly related to this discussion, but demonstrates the impact of BWP on the UE implementation. CCS does not imply multiple BWP support and BWP switch support which is also the reason why there are separate FGs even in Rel-15 - Yes. two things. (a) crossCarrierScheduling-SameSCS does not mean UE has to support more than 1 BWP and BWP switch (b) we now also allow CCS with different SCS which is not per FS or per band, it is defined per BC and there is no way to clarify based on interpretation 1/2/3 |

## FG: pdcch-MonitoringAnyOccasionsWithSpanGap

The description of pdcch-MonitoringAnyOccasionsWithSpanGap from TS38.306 is as below.

| pdcch-MonitoringAnyOccasionsWithSpanGap Indicates whether the UE supports PDCCH search space monitoring occasions in any symbol of the slot with minimum time separation between two consecutive transmissions of PDCCH with span up to two OFDM symbols for two OFDM symbols or span up to three OFDM symbols for four and seven OFDM symbols. Value set1 indicates the supported value set (X,Y) is (7,3), value set2 indicates the supported value set (X,Y) is (4,3) and (7,3) and value set 3 indicates the supported value set (X,Y) is (2,2), (4,3) and (7,3). | FS | No | N/A | N/A |
| --- | --- | --- | --- | --- |

The corresponding UE feature description is as below.

| 3-5b | All PDCCH monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a span gap | PDCCH monitoring occasions of FG-3-1, plus additional PDCCH monitoring occasion(s) can be any OFDM symbol(s) of a slot for Case 2, and for any two PDCCH monitoring occasions belonging to different spans, where at least one of them is not the monitoring occasions of FG-3-1, in same or different search spaces, there is a minimum time separation of X OFDM symbols (including the cross-slot boundary case) between the start of two spans, where each span is of length up to Y consecutive OFDM symbols of a slot. Spans do not overlap. Every span is contained in a single slot. The same span pattern repeats in every slot. The separation between consecutive spans within and across slots may be unequal but the same (X, Y) limit must be satisfied by all spans. Every monitoring occasion is fully contained in one span. In order to determine a suitable span pattern, first a bitmap b(l), 0<=l<=13 is generated, where b(l)=1 if symbol l of any slot is part of a monitoring occasion, b(l)=0 otherwise. The first span in the span pattern begins at the smallest l for which b(l)=1. The next span in the span pattern begins at the smallest l not included in the previous span(s) for which b(l)=1. The span duration is max{maximum value of all CORESET durations, minimum value of Y in the UE reported candidate value} except possibly the last span in a slot which can be of shorter duration. A particular PDCCH monitoring configuration meets the UE capability limitation if the span arrangement satisfies the gap separation for at least one (X, Y) in the UE reported candidate value set in every slot, including cross slot boundary. For the set of monitoring occasions which are within the same span: - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for FDD - Processing one unicast DCI scheduling DL and two unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD - Processing two unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD The number of different start symbol indices of spans for all PDCCH monitoring occasions per slot, including PDCCH monitoring occasions of FG-3-1, is no more than floor(14/X) (X is minimum among values reported by UE). The number of different start symbol indices of PDCCH monitoring occasions per slot including PDCCH monitoring occasions of FG-3-1, is no more than 7. The number of different start symbol indices of PDCCH monitoring occasions per half-slot including PDCCH monitoring occasions of FG-3-1 is no more than 4 in SCell. |
| --- | --- | --- |

Apple raises the following example in [5, Apple].

Using a 3 band, FR1 + FR2, deployment with 3 SCS as an example, namely - Sub-1GHz FR1 NR FDD band with 15kHz SCS - ~3GHz FR1 NR TDD band with 30KHz SCS - FR2 NR TDD band with 120KHz SCS One of the reasonable UE implementation is that UE supports CCS with different SCS, i.e., “Scheduling cell of lower SCS and scheduled cell of higher SC” in the corresponding BC, but - UE only supports FG3-5b for FR1 scheduling FR1 - UE does not support FG3-5b for FR1 scheduling FR2 As of now, it is unclear whether this capability is allowed by the specification or not, depending on the interpretation of per FS FG3-5b, under the context of CCS. To resolve this issue, we propose to interpret FG3-5b based on the support of this feature for both scheduling/triggering/indicating cell and scheduled/triggered/indicated cell.

From moderator’s perspective, UE feature pdcch-MonitoringAnyOccasionsWithSpanGap is only related to PDCCH search space monitoring occasions and is not related to the scheduled/triggered/indicated cell. Thus, Interpretation3 may not be a reasonable interpretation from this perspective.

Based on the above analysis, companies are encouraged to answer the following question and provide the corresponding justification.

1st phase: Question 3

Which interpretation should be adopted for pdcch-MonitoringAnyOccasionsWithSpanGap?

Please provide your answer with corresponding technical justification. Comparison between different interpretations are encouraged.

| Company | Answers and Comments |
| --- | --- |
| ZTE | Our understanding is Interpretation2. Our understanding is that UE capability is not ambiguous because all the related UE behaviors are only about PDCCH monitoring and PDCCH reception and it seems to have nothing related to the scheduled/triggered/indicated cell. Thus, technically speaking, Interpretation2 is the appropriate interpretation for pdcch-MonitoringAnyOccasionsWithSpanGap. |
| Intel | We support Interpretation 2. As commented by Xinguang, this feature focuses on the PDCCH transmission/reception. We think it can be independent to the target cell where the UL/DL data/control transmission happens. |
| vivo | Our understanding is Interpretation2. There is no PDCCH monitoring occasion in any cross-carrier scheduled cell, which means that this FG is only relevant to the scheduling cell. Regarding the example considered by Apple, the feature of X-CC scheduling with mix SCSs is not supported in Rel-15, thus is not relevant to this email thread (which is for Rel-15 maintenance). |
| Nokia | Support interpretation #2, this is a property of the scheduling cell. |
| Qualcomm | We support Interpretation 3. Similar reason as question 1 and 2. This feature has an impact on both control channel and data channel for the mini slot-based scheduling and data transmission and reception. These two aspects cannot be isolated from each other. |
| Apple | Our understanding is interpretation 3 FG3-5b involves both the scheduling cell and scheduled cell. In terms of the UE implementation, it is different to support 15kHz cell to schedule 30kHz cell, compared to support 15kHz cell to schedule 120kHz cell. The UE capability needs to address and differentiate different levels of UE implementation complexity, which not only depends on the scheduled cell, but also depends on the scheduling cell. |
| OPPO | Our understanding is interpretation 3. Similar reasons as Question 1/2. |
| Huawei | Our understanding is Interpretation2. |
| Samsung | Our understanding is interpretation 3. |

### Summary of 1st phase discussion

Company position for FG: pdcch-MonitoringAnyOccasionsWithSpanGap

Adopt Interpretation2 for these two FGs (6): ZTE, Intel, vivo, Nokia, Huawei/Hisilicon

1) This FG is about UE capability on PDCCH monitoring and PDCCH reception, which is a property of the scheduling cell

2) Interpretation 1 provides UE some implementation flexibilities, i.e. a UE does not need to support this UE capacity on both cells in case of cross-carrier operation.

Adopt Interpretation3 for these two FGs (4): Qualcomm, Apple, OPPO, Samsung

1) This FG involves both the scheduling cell and scheduled cell. In terms of the UE implementation, it is different to support 15kHz cell to schedule 30kHz cell, compared to support 15kHz cell to schedule 120kHz cell.

Moderator suggestion

It seems the issue is similar here. Some companies argued that this FG is about UE capability on PDCCH monitoring and PDCCH reception and has nothing to do with the scheduled cell. Besides, cross-carrier operation for pdcch-MonitoringAnyOccasionsWithSpanGap only happens when cross-carrier scheduling is configured. UE can already control the scheduling cell and scheduled cell by the FG for cross-carrier scheduling (crossCarrierScheduling-SameSCS). However, some companies argued that this FG involves both the scheduling cell and the scheduled cell and Interpretation3 can better control the scheduling cell and scheduled cell.

From moderator’s perspective, we propose the following

Moderator proposal 4-v0: Further discuss the following issues

1) Whether cross-carrier operation of pdcch-MonitoringAnyOccasionsWithSpanGap adds any additional requirements on the scheduled cell/band if cross-carrier scheduling has already been supported by the UE?

2) Considering that UE feature for cross-carrier scheduling (crossCarrierScheduling-SameSCS) can already control the scheduling cell and the scheduled cell, do you think we still need interpretation3 for pdcch-MonitoringAnyOccasionsWithSpanGap to control the scheduling cell and the scheduled cell?

### 2nd phase: Question 3-2

Companies are encouraged to check/update the following positions and answer following questions.

Adopt Interpretation1 for pdcch-MonitoringAnyOccasionsWithSpanGap: Support: ZTE, Intel, vivo, Nokia, Huawei/Hisilicon Object: Adopt Interpretation3 for pdcch-MonitoringAnyOccasionsWithSpanGap: Support: Qualcomm, Apple, OPPO, Samsung Object:

1) Whether cross-carrier operation of pdcch-MonitoringAnyOccasionsWithSpanGap adds any additional requirements on the scheduled cell/band if cross-carrier scheduling has already been supported by the UE?

2) Considering that UE feature for cross-carrier scheduling (crossCarrierScheduling-SameSCS) can already control the scheduling cell and the scheduled cell, do you think we still need interpretation3 for pdcch-MonitoringAnyOccasionsWithSpanGap to control the scheduling cell and the scheduled cell?

| Company | Answers and Comments |
| --- | --- |
| ZTE | 1) No. If UE supports cross-carrier scheduling between band A and band B, there is no additional implementation requirements on the scheduled cell because this FG is pure about PDCCH reception/monitoring. The scheduled PDSCH/PUSCH ect are all regular PDSCH/PUSCH reception, which is the same as cross-carrier scheduling. Also, these regular scheduling DCIs don’t add any additional timeline requirement for UE top of the existing scheduling timeline. 2) No. Based on last RAN1 meeting’s conclusion, interpretation3 is adopted for crossCarrierScheduling-SameSCS. Thus, UE feature for cross-carrier scheduling can already control the band pairs for cross-carrier scheduling. In other words, UE feature for cross-carrier scheduling can already control the band pair used for pdcch-MonitoringAnyOccasionsWithSpanGap. |
| Apple | - Yes. In the current UE capability design, for cross carrier scheduling, we do not differentiate the detailed PDCCH monitoring capability, for example the basic FG3-1, more advanced FG3-2, FG3-5b, FG3-5a, etc. First of all, supporting FG3-5b or supporting FG3-1 is completely different for the UE. Secondly, for any scheduling cell, supporting different number of scheduled cells also impacts the PDCCH monitoring memory and complexity requirement. We also need to consider the Cross Carrier Scheduling (CCS) with different SCS since we did not replicate FG3-5b for Rel-16 CCS with different SCS, and currently, FG3-5b applies to both the CCS with the same SCS and with different SCS - Yes. There are two main issues there (1) crossCarrierScheduling-SameSCS is agreed to be both scheduling and scheduled cell. But it does not differentiate the PDCCH minoring capability, i.e., FG3-1, FG3-5b, FG3-2, FG3-5a. It is unreasonable to assume that it is the same complexity for UE to support different PDCCH monitoring capability (2) We also support CCS with different SCS, in which the UE capability is defined per BC. We did not introduce new FG3-5b for CCS with different SCS. As results, we also need to consider that impact unless we agree to replicate FG3-5b specifically for CCS with different |

## Any other issue

If you figure out any other issue or any other UE capabilities with the same ambiguity issue that may need further clarification, please input your comments below.

### Question 4:

Any other issue?

| Company | Answers and Comments |
| --- | --- |

# Conclusion

TBD

# Reference

RAN1#102e chairman note.

R1-2007334, LS on Interpretation of UE Features in Case of Cross-Carrier Operation, RAN1#102e.

R1-2007333, Email Discussion Summary of [102-e-NR-7.1CRs-07], Moderator (ZTE), RAN1#102e.

R1-2007722,	Discussion on Ambiguity Issues for UE Features with Cross-Carrier Operation, ZTE, RAN1#103e.

R1-2008423,	Clarification of FG3-5b with Cross Carrier Operation, Apple, RAN1#103e.

R1-2008653,	Discussion on UE FG in case of cross-carrier operation, vivo, RAN1#103e.

R1-2008787,	Clarifications on UE features in case of cross-carrier operation, Huawei, HiSilicon, RAN1#103e.

# Appendix: Proposal Summary

| Contribution | Proposals |
| --- | --- |
| R1-2007722 ZTE | Observation 1: If ue-SpecificUL-DL-Assignment can NOT be reported for FDD band, Interpretation3 will preclude the use of this UE capability in case of FDD-TDD cross-carrier operation, i.e., UE is not expected to be scheduled/triggered PDSCH, CSI-RS, PUSCH, PUCCH, PRACH or SRS in the flexible symbols in TDD band from a FDD band based on the current specification in Section 11.1 of TS 38.213. Observation 2: If UE is allowed to report ue-SpecificUL-DL-Assignment for FDD band, RAN1 and/or RAN2 spec changes are needed to specify the corresponding UE behaviors on interpreting ue-SpecificUL-DL-Assignment reported for FDD bands. Observation 3: The market drive and implementation concern on supporting Interpretation3 instead of Interpretation1 for ue-SpecificUL-DL-Assignment is not clear. Observation 4: With Interpretation3 for bwp-DiffNumerology / bwp-SameNumerology, in order to support cross-carrier BWP switching, it requires UE to support up to 2/4 BWPs in both scheduling cell/band and scheduled cell/band. Proposal 1: Regarding the interpretation of UE capabilities in case of cross-carrier operation, the support of the following UE capability is based on the support of this capability for the band of the scheduled/triggered/indicated cell only. - ue-SpecificUL-DL-Assignment - bwp-DiffNumerology / bwp-SameNumerology |
| R1-2008423 Apple | Proposal 1: Clarify that the support of the following feature is based on the support of this feature for both scheduling/triggering/indicating cell and scheduled/triggered/indicated cell. - FG3-5b, i.e., pdcch-MonitoringAnyOccasionsWithSpanGap |
| R1-2008653 vivo | Proposal 1: In case of cross-carrier operation, the support of UE capability ue-SpecificUL-DL-Assignment is reported for the band of the scheduled/triggered/indicated cell only (i.e., Interpretation1). Proposal 2: In case of cross-carrier operation, the support of UE capability bwp-DiffNumerology / bwp-SameNumerology is reported for the band of the scheduled/triggered/indicated cell only (i.e., Interpretation1). |
| R1-2008787 Huawei, HiSilicon | Proposal: In case of cross-carrier operation, the support of the following UE capabilities is based on the support of a capability for the band of the scheduled/triggered/indicated cell only. - ue-SpecificUL-DL-Assignment - bwp-DiffNumerology/bwp-SameNumerology |