3GPP TSG RAN WG1 #103	R1-200xxxx

e-Meeting, October 26th – November 13th, 2020

Title:	[Draft] LS on HARQ-ACK codebook configuration for secondary PUCCH group

Response to:	N/A

Release:	Rel-16

Work Item:	LTE_NR_DC_CA_enh-Core

Source:	Nokia [RAN1]

To:	RAN2

Contact Person:

Name:	Karri Ranta-aho

E-mail Address:	Karri-Ranta-aho@nokia.com

Attachments: N/A

1. Overall Description:

RAN1 #103 discussed the issue that according to the current specification, both pdsch-HARQ-ACK-CodebookList-r16 and pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 can be configured simultaneously. In this case, it is not clear how to determine the HARQ-ACK codebook type for the two HARQ-ACK codebooks for the secondary PUCCH group. If pdsch-HARQ-ACK-CodebookList-r16 is followed, HARQ-ACK codebook type for the secondary PUCCH group cannot be separately configured from the primary PUCCH group. Otherwise if pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 is followed, only one HARQ-ACK codebook can be configured for the secondary PUCCH group which is not aligned with the intention in URLLC.

RAN1 agreed to resolve the issue with the following solution:

- The same RRC configuration pdsch-HARQ-ACK-CodebookList-r16 is applied to both primary PUCCH group and secondary PUCCH group if two PUCCH groups are configured.

2. Actions:

To RAN2 group

ACTION: 	RAN1 respectfully asks RAN2 to take the above into account and update the TS38.331 accordingly

3. Date of Next TSG-RAN1 Meetings:

3GPPRAN1#104-e	25 Jan – 5 Feb 2021	E-meeting

3GPPRAN1#105-e	12 - 20 April 2021	E-meeting