  - Issue 1 (unified TCI framework)

Proposal 1.A: On Rel.17 unified TCI framework, support common TCI state ID update and activation to provide common QCL information and/or common UL TX spatial filter(s) across a set of configured CCs:

- The above applies to intra-band CA

- The above applies to joint DL/UL and separate DL/UL beam indications

- FFS: Just as Rel.16, the UE will find the corresponding TCI state in the corresponding CC and apply the corresponding TypeA and TypeD QCL assumption from the TCI state ID

- FFS: The above also applies to inter-band CA

- FFS: sharing a single RRC TCI state pool for the set of configured CCs

Proposal 1.B: On Rel.17 unified TCI framework, to accommodate the case of separate beam indication for UL and DL:

- Utilize two separate TCI states, one for DL and one for UL.

  - FFS: Contents of separate UL TCI state

  - Note: For FR1, UE does not expect UL TCI to provide a reference for determining common UL TX spatial filter(s), if UL TCI is supported for FR1

- For the separate DL TCI:

  - The source reference signal(s) in M TCIs provide QCL information at least for UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC

    - FFS: decide between “all”, “subset”, or “all or subset”

- For the separate UL TCI:

  - The source reference signal(s) in N TCIs provide a reference for determining common UL TX spatial filter(s) at least for dynamic-grant/configured-grant based PUSCH, all or subset of dedicated PUCCH resources in a CC

    - FFS: decide between “all”, “subset”, or “all or subset”

  - Optionally, this UL TX spatial filter can also apply to all SRS resources in resource set(s) configured for antenna switching/codebook-based/non-codebook-based UL transmissions

- FFS: Whether the UL TCI state is taken from a common/same or separate TCI state pool from DL TCI state

  - e.g., common/same pool in both RRC and MAC, separate pools in RRC and MAC, separate pools in RRC but single combined TCI pool in MAC, or common/same pool in RRC but separate pools in MAC

  - Note that TCI state pool for joint DL and UL beam indication is still FFS

- FFS: Whether Rel.17 supports TCI configured for single channel (e.g. PDSCH only, single CORESET)

- Note: This does not preclude the type of UE supporting only 1 beam tracking loop, i.e. UE reports value of 1 in UE FG 2-62.

{For later discussion} Proposal 1.C: On Rel.17 unified TCI framework, based on theRAN1#102-e agreement the following is supported for both joint DL/UL TCI and separate DL/UL TCI:

- For single-TRP scenarios:

  - The source reference signal(s) in one TCI provide common QCL information at least for UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC (i.e. M=1 in this case)

    - FFS: decide between “all”, “subset”, or “all or subset”

  - The source reference signal in one TCI provides a reference for determining common UL TX spatial filter at least for dynamic-grant/configured-grant based PUSCH, all or subset of dedicated PUCCH resources in a CC (i.e. N=1 in this case)

    - FFS: decide between “all”, “subset”, or “all or subset”

  - FFS: The support for M>1 and/or N>1 for single-TRP

- FFS: the support for mTRP

- Note: This does not preclude that the source reference signal(s) in one TCI can provide common QCL/spatial filter info for both DL and UL signals.

{For later discussion} Proposal 1.D:

- On Rel.17 unified TCI framework, based on the RAN1#102-e agreement the following is supported for both joint DL/UL TCI and separate DL/UL TCI:

  - The source reference signal(s) in M TCIs can also provide common QCL information for one or more CSI-RS resources for CSI and some CSI-RS resource for BM with repetition ‘ON’ , where the target CSI-RS resource(s) are determined by NW configuration

    - FFS: Support for some aperiodic CSI-RS resource for BM with repetition ‘OFF’

- FFS (RAN1#104-e): select a scheme to provide the QCL information or the UL TX spatial filter for other channels, signals, or CORESETs not included in the Rel.17 unified TCI framework

{For later discussion} Proposal 1.E: On Rel.17 unified TCI framework, for both joint DL/UL TCI and separate DL/UL TCI:

- Support the use of CSI-RS for tracking as source RS to determine a UL TX spatial filter

- [Support the use SRS for BM as a source RS to represent a DL RX spatial filter, configured together with either a CSI-RS for BM, SSB, or DL TRS as the QCL source (or spatial relation) for the SRS ]

Conclusion 1: On Rel.17 unified TCI framework, in RAN1#103-e, there is no consensus on supporting:

- that the source reference signal(s) in M TCIs can also provide common QCL information for CSI-RS

  - Issue 2 (L1/L2-centric inter-cell mobility)

Proposal 2.A: On Rel.17 enhancements to enable L1/L2-centric inter-cell mobility:

- The following use cases are assumed:

  - Network architecture:

    - NSA, i.e. LTE PCell and NR-PSCell

    - SA

  - Intra-band CA

    - FFS: If inter-band CA is also included

  - Intra- RAT (excluding inter-RAT)

  - Intra-frequency scenario:

    - The SSBs of non-serving cells have the same center frequency and SCS as the SSBs of the serving cell

    - An SSB of a non-serving cell is associated with a PCI different from the PCI of the serving cell

    - FFS: Support for inter-frequency scenario

  - FFS: Whether to support intra-DU only operation, or whether inter-DU is also allowed

    - FFS: If inter-DU operation is also included

- The following enhancement scope is assumed:

  - {For later discussion} No RRC reconfiguration signaling is needed during and after handover when a TCI associated with non-serving cell RS is indicated

    - A non-serving cell RS is an RS that is or has an SSB of a non-serving cell as direct or indirect QCL source

    - This implies no C-RNTI update during inter-cell mobility during and after handover

  - Facilitating measurement and reporting of non-serving RSs via incorporating non-serving cell info with some TCI(s), along with the necessary measurement and reporting scheme(s)

    - FFS: Detailed/exact method(s)

    - FFS: Whether this also implies the support of beam indication (TCI state update along with the necessary TCI state activation) for TCI(s) associated with non-serving cell RS(s)

    - FFS: Metric for the measurement and reporting, e.g. L1-RSRP or L3-RSRP or time- or spatial-domain-filtered L1-RSRP

    - FFS: Beam-level event-driven mechanism, using serving cell RS and/or non-serving cell RS

  - Facilitate serving cell to provide configurations for non-serving cell SSBs via RRC

    - FFS: details for the configurations, e.g. time/frequency location, transmission power, etc.

    - FFS: other information needed for inter-cell mobility

  - Note: In RAN1's understanding, non-serving cell SSB and non-serving cell RS can be part of the serving cell configuration

  - Issue 3 (beam indication signaling medium)

The following agreement was made during the first GTW.

Agreement On beam indication signaling medium to support joint or separate DL/UL beam indication in Rel.17 unified TCI framework: - Support L1-based beam indication using at least UE-specific (unicast) DCI to indicate joint or separate DL/UL beam indication from the active TCI states - The existing DCI formats 1_1 and 1_2 are reused for joint beam indication - FFS: If additional DCI format(s) are supported, e.g. existing DCI formats 0_0, 0_1, 0_2, 1_0 as well as new DCI format(s) dedicated for beam indication - Support a mechanism for UE to acknowledge successful decoding of beam indication - The ACK/NAK of the PDSCH scheduled by the DCI carrying the beam indication can be used as an ACK also for the DCI - FFS: Whether any additional specification support is needed - FFS beam indication for the TCI state assumption/update for the following cases: - The beam indication UE-specific DCI (i.e. the CORESETs with the DCI received by UE), the scheduled PDSCH by the DCI and the associated PUCCH for the acknowledgment of the beam indication DCI - Non-UE-specific CORESETs and PUSCH/PDSCH scheduled/activated and PUCCH transmission triggered by non-UE-specific CORESETs - Support activation of one or more TCI states via MAC CE analogous to Rel.15/16: - At least for the single activated TCI state, the activated TCI state is applied - The content for the MAC CE is determined based on the outcome of issue 1 - FFS: If supported, default TCI state when more than one TCI states are activated by MAC CE - Note: There is no implications on the support of single TRP or multi-TRP - Support a UE capability for the minimum beam indication delay - FFS: Whether to measure beam indication delay from DCI reception or from acknowledgment of DCI - FFS: The exact supported values e.g. {0.5ms, 2ms, 3ms} - FFS: Additional enhancement such as L1-based beam indication with group-common DCI - FFS: Whether the Rel.17 beam indication can also apply to beam indication for single channel (e.g. PDSCH only, single CORESET) or a subset of channels - FFS: Additional details on extending the support of L1-based beam indication when separate UL (from DL) common beam indication is configured Continue email discussion on the yellow part.

Proposal 3.A: On the three yellow parts of the issue 3 agreement in the 1st GTW session:

- Yellow 3.1: “for joint beam indication” text. This can be addressed as follows:

  - The existing DCI formats 1_1 and 1_2 are reused for joint DL/UL or separate DL/UL beam indication

    - FFS: If additional DCI format(s) are supported, e.g. existing DCI formats 0_0, 0_1, 0_2, 1_0 as well as new DCI format(s) dedicated for beam indication

    - FFS: support of DCI format 1_0 for joint DL/UL beam indication

    - FFS: support new DCI format(s) dedicated for beam indication for joint or separate DL/UL beam indication

    - FFS: support for reusing the existing UL-related DCI format(s) (e.g. 0_0, 0_1, 0_2) for joint or separate DL/UL beam indication

    - FFS: support for reusing DCI format 1_0, 1_1, and 1_2 for joint or separate DL/UL beam indication

- Yellow 3.2: vivo FFS

  - Conclusion: There is no consensus in including the two FFS points proposed by vivo in the agreement made in the 1st GTW session

- {For later discussion} Yellow 3.3: UE capability text, revised as follows (per inputs from interested companies, based on where we left off during the GTW discussion):

  - Application time of the beam indication: if beam indication is received, down-select from the following:

    - Alt1: the first slot that is at least X ms or Y symbols after the DCI with the joint or separate DL/UL beam indication

    - Alt2: the first slot that is at least X ms or Y symbols after the acknowledgment of the joint or separate DL/UL beam indication

  - FFS: When and how to apply the minimum beam indication delay

  - Support a UE capability for the minimum value of X or Y

    - FFS: the beam application time X or Y is configured by the gNB via higher-layer (RRC) signaling based the UE capability

    - FFS: the exact minimum values of X (e.g., 0.5ms, 2ms, 3ms) or Y supported by UE

  - Issue 4 (MP-UE)

Proposal 4.A: In Rel.17 enhancement for facilitating fast uplink panel selection, the following use cases are assumed:

- MPE mitigation

- UE power saving

- UL interference management

- Support different configurations across panels

- UL mTRP

Proposal 4.B: In Rel.17 enhancement on MP-UE to facilitate fast UL panel selection and MPE mitigation, UL Tx panel(s) are assumed to be a same set or subset of DL Rx panel(s)

s

Proposal 4.C: In Rel.17 enhancement for facilitating fast uplink panel selection, UE-initiated UL panel selection/activation are supported:

- FFS: Whether NW-initiated panel selection/activation is also supported

- FFS specification impact” – UE initiated panel selection/activation does not necessarily have specification impact

  - Issue 5 (MPE mitigation)

Proposal 5.A:  On UE reporting for MPE mitigation for Rel.17, investigate and, if needed, specify the following:

- Reporting of P-MPR report based on Rel.16 framework.

  - FFS: Maximum reported number of panels, e.g. single or multiple

- Reporting SSBRI(s)/CRI(s) and/or indication of panel selection for the purpose of indicating:

  - Alt1: alternative UE panel(s) or TX beam(s) for UL transmission

  - Alt2: feasible UE panel(s) or TX beam(s) for UL transmission taking the MPE effect into account

  - FFS: indication of panel selection details (e.g. explicit/implicit)

- Any additional reporting: down-select from the following in RAN1#104-e

  - Alt0: no additional reporting content

  - Alt1: P-MPR + L1-RSRP

  - Alt2: virtual PHR + L1-RSRP

  - Alt3: L1-RSRP/SINR with and without MPE effect

  - Alt4: virtual PHR

  - Alt5: P-MPR or virtual PHR + CRI/SSBRI

  - Other options are not precluded

  - Note that PHR including PH and Pcmax is calculated based on P-MPR and the L1-RSRP

  - Issue 6 (beam refinement/tracking)

{For later discussion} Proposal 6.A: Investigate and, if needed, specify at least the following enhancements for beam refinement/tracking in Rel.17:

- Beam measurement and reporting enhancement via RACH (e.g. RO for measurement and MSG3 for reporting)

- Improving efficiency (latency and/or overhead) of beam refinement assuming the unified TCI framework (issue 1):

  - Enabling joint DL TX and RX beam refinement/tracking (P2+P3)

  - Additional UE report to aid P1/P2/P3 related measurement/report configuration (triggering frequency or periodicity)

- Beam management with reduced DL signaling assuming the unified TCI framework (issue 1):

  - Dynamic beam update based on beam report (without beam indication)

  - Dynamic beam measurement and report triggered by beam indication (without CSI-RS/CSI triggering)

  - Configuring/indicating to UE multiple SSBs for beam tracking

  - Semi-static/pre-planned (RRC based) beam transition (for, e.g. isolated HST deployment)

  - Reducing activation delay of TCI states (e.g. via storing QCL properties of a subset of source RSs for a time period)